3GPP TSG-RAN WG4 meeting #98-e	R4-2101452

Electronic Meeting, 25 January – 5 February 2021

Agenda Item:	7.9.4.2.3

Source:	Ericsson

Title:	Simulation results of sDCI-based FDM/TDM transmission

Document for:	Information

# 1	Introduction

RAN4#97-e agreed with the simulation assumption of mDCI/sDCI-based SDM transmission schemes [1]. This contribution provides our alignment and impairment simulation results according to the simulation assumption.

# 2	Simulation results

## 2.1	Single-DCI based FDM Scheme A

Figure 1 shows our ideal simulation results of single-DCI based FDM Scheme A. Table 1 summarizes the SNR levels to achieve 70% of maximum throughput.

| Simulation cases | Duplex mode | BW / SCS | MCS / MCS Table | Propagation condition | Antenna configuration | Frequency offset / Time offset | Metric |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 5-1 | FDD | 10MHz / 15kHz | MCS19 (16QAM 0.5) MCS Table 3 | TDLA30-10 for both TRP1 and TRP2 | 2x2 ULA Low for each TRP | 200Hz / -0.5us | 70% of max Tput |
| 5-2 | FDD | 10MHz / 15kHz | MCS19 (16QAM 0.5) MCS Table 3 | TDLA30-10 for both TRP1 and TRP2 | 2x4 ULA Low for each TRP | 200Hz / -0.5us | 70% of max Tput |
| 6-1 | TDD | 40MHz / 30kHz | MCS19 (16QAM 0.5) MCS Table 3 | TDLA30-10 for both TRP1 and TRP2 | 2x2 ULA Low for each TRP | 300Hz / -0.25us | 70% of max Tput |
| 6-2 | TDD | 40MHz / 30kHz | MCS19 (16QAM 0.5) MCS Table 3 | TDLA30-10 for both TRP1 and TRP2 | 2x4 ULA Low for each TRP | 300Hz / -0.25us | 70% of max Tput |

| (a) FDD SCS=15kHz | (b) TDD SCS=30kHz |
| --- | --- |

Figure 1	Simulation results of Single-DCI based FSDM transmission.

Table 1	Summary of simulation results for single-DCI based FDM Scheme A (SNR to achieve 70% of maximum throughput (dB)).

|  | FDD SCS=15kHz 2Rx | FDD SCS=15kHz 4Rx | TDD SCS=30kHz 2Rx | TDD SCS=30kHz 4Rx |
| --- | --- | --- | --- | --- |
| Alignment results | 13.3 | 6.9 | 13.2 | 6.6 |
| Impairment results | 15.3 | 8.9 | 15.2 | 8.9 |

## 2.2	Single-DCI based Inter-slot TDM

Figure 2 shows our ideal simulation results of single-DCI based Inter-slot TDM. Table 2 summarizes the SNR levels to achieve 1% of PDSCH BLER.

| Simulation cases | Duplex mode | BW / SCS | MCS / MCS Table | Propagation condition | Antenna configuration | Frequency offset / Time offset | Metric |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 7-1 | FDD | 10MHz / 15kHz | MCS19 (16QAM 0.5) MCS Table 3 | TDLA30-10 for both TRP1 and TRP2 | 2x2 ULA Low for each TRP | 200Hz / 2us | 1% BLER |
| 7-2 | FDD | 10MHz / 15kHz | MCS19 (16QAM 0.5) MCS Table 3 | TDLA30-10 for both TRP1 and TRP2 | 2x4 ULA Low for each TRP | 200Hz / 2us | 1% BLER |
| 8-1 | TDD | 40MHz / 30kHz | MCS19 (16QAM 0.5) MCS Table 3 | TDLA30-10 for both TRP1 and TRP2 | 2x2 ULA Low for each TRP | 300Hz / 1us | 1% BLER |
| 8-2 | TDD | 40MHz / 30kHz | MCS19 (16QAM 0.5) MCS Table 3 | TDLA30-10 for both TRP1 and TRP2 | 2x4 ULA Low for each TRP | 300Hz / 1us | 1% BLER |

| (a) FDD SCS=15kHz | (b) TDD SCS=30kHz |
| --- | --- |

Figure 2	Simulation results of Single-DCI based SDM transmission.

Table 2	Summary of simulation results for single-DCI based inter-slot TDM (SNR to achieve 1% of PDSCH BLER (dB)).

|  | FDD SCS=15kHz 2Rx | FDD SCS=15kHz 4Rx | TDD SCS=30kHz 2Rx | TDD SCS=30kHz 4Rx |
| --- | --- | --- | --- | --- |
| Alignment results | -0.6 | -5.0 | -1.1 | -5.7 |
| Impairment results | 1.4 | -3.0 | 0.9 | -3.7 |

# 3	Summary

This contribution provided our alignment and impairment simulation results for PDSCH with multi-TRP transmission. We would like to consider these results for the final PDSCH demodulation requirements.

# References

- R4-2017530, “Simulation assumption for PDSCH requirement with single-DCI and Multi-DCI transmission schemes”, Ericsson, Intel.