3GPP TSG-RAN WG4 Meeting # 98-e                               R4-2101497

Electronic Meeting, 25th January– 5th Febuary, 2021

Source: 	Huawei, HiSilicon

Title: 	Simulation results on DL co-existence on 6.425-7.125GHz, 10.0-10.5 GHz for indoor hotspot

Agenda Item:	10.1.3.2

Document for:	Approval

Introduction

In RAN4#98e, a work plan for indoor simulation was approved in WF [1] that is listed in the following.

- If RAN accepts the SI extension for 1 meeting, following work plan has been proposed:

- Agree on antenna parameters and other simulation assumptions if any for indoor before Dec. 15th

- Alignment on UE Tx power and SINR cdf  for January 10th 2021.

- Run simulations and submit results to next RAN4#98e

There are two options for the BS antenna parameters for indoor hotspot based on the email discussion.

- omni antenna.

- AAS antenna with following parameters:

| Parameter | Small cell Indoor |
| --- | --- |
| Am (dB) | 30 |
| SLAv (dB) | 30 |
| j3dB (deg.) | 90 |
| q3dB (deg.) | 90 |
| GE,max (dBi) | 5.5 |
| LE (dB) | 2.0 |
| (M, N) | (4, 4) |
| dh (m) | 0.5 |
| dv (m) | 0.5 |
| Horizontal coverage range (deg.) | N/A |
| Vertical coverage range (deg.) | N/A |
| Conducted power (before ohmic loss) per antenna element (dBm) | 9 |
| Mechanical downtilt (deg.) | N/A (Note 4) |
| Note 1: MxN means there are M vertical and N horizontal elements Note 2: LE is included in GE,max Note 3: The vertical coverage range includes the mechanical downtilt. Note 4: Boresight direction is perpendicular to the ceiling. |  |

In this paper, we’d like to provide the adjacent channel downlink co-existence simulation results on both 6.425-7.125GHz and 10.0-10.5GHz for indoor hotspot using the option 2 about BS antenna parameters and other assumptions in TR 38.921.

Discussion

Simulation results on 6.425-7.125GHz for indoor hotspot

Based on the current specifications, we set 45dB ACLR for BS and 33dB ACS for UE as baseline. The corresponding ACIR is about 32.7dB. When we simulate the scenario, different delta-ACIR X values were considered in order to find the reasonable requirements. The CDF figures of SINR and throughput for Indoor hotspot are shown below.

Figure 1 The CDF figures of SINR and throughput for Indoor hotspot

The Indoor hotspot simulation results of the downlink throughput loss of the victim UE with different delta-ACIR values are shown at 7GHz centre frequency in table 1

Table 1 The Indoor hotspot simulation results of the downlink throughput loss at 7GHz centre frequency

| ACIR offset X [dB] | 0 | -2 | -4 | -6 |
| --- | --- | --- | --- | --- |
| Average throughput (7GHz) baseline | 350.29 | 349.35 | 350.48 | 349.49 |
| Average throughput (7GHz) | 349.75 | 348.29 | 349.22 | 347.2 |
| Average throughput loss (7GHz) | 0.15% | 0.30% | 0.36% | 0.66% |
| 5%-tile throughput (7GHz) baseline | 115.5 | 115.62 | 111.69 | 115.66 |
| 5%-tile throughput (7GHz) | 115.48 | 115.55 | 111.61 | 115.54 |
| 5%-tile throughput loss (7GHz) | 0.02% | 0.06% | 0.07% | 0.10% |

Based on the approved WF in last meeting, the targeted ACIR value in DL (indoor excluded) is 30.9 dB for 6.425-7.125 GHz. However, the DL ACIR for 6.425-7.125 GHz can be set less than 25.9dB for indoor scenario. It means that Indoor hotspot is not the restricted scenario for 6.425-7.125 GHz.

Observation 1: Since the DL ACIR for 6.425-7.125 GHz can be set as less than 25.9dB which is less than the agreed value 30.9dB in last meeting, indoor hotspot is not the restricted scenario for 6.425-7.125 GHz.

Simulation results on 10.0-10.5GHz for indoor hotspot

The CDF figures of SINR and throughput for Indoor hotspot on 10.0-10.5GHz are shown below.

Figure 2 The CDF figures of SINR and throughput for Indoor hotspot at 10GHz

The Indoor hotspot simulation results of the downlink throughput loss of the victim UE with different delta-ACIR values are shown at 10GHz centre frequency in table 2

Table 2 The Indoor hotspot simulation results of the downlink throughput loss at 10GHz centre frequency

| ACIR offset X [dB] | 0 | -2 | -4 | -6 |
| --- | --- | --- | --- | --- |
| Average throughput (10GHz) baseline | 352.24 | 350.25 | 353.68 | 347.18 |
| Average throughput (10GHz) | 351.78 | 349.51 | 352.55 | 345.55 |
| Average throughput loss (10GHz) | 0.13% | 0.21% | 0.32% | 0.47% |
| 5%-tile throughput (10GHz) baseline | 116.71 | 115.14 | 115.85 | 111.91 |
| 5%-tile throughput (10GHz) | 116.7 | 115.06 | 115.74 | 111.79 |
| 5%-tile throughput loss (10GHz) | 0.01% | 0.07% | 0.09% | 0.11% |

Based on the approved WF in last meeting, the targeted ACIR value in DL (indoor excluded) is 29.6 dB for 10.0-10.5 GHz. However, the DL ACIR for 10.0-10.5 GHz can be set less than 25.9dB for indoor scenario. It means that Indoor hotspot is not the restricted scenario for 10.0-10.5 GHz.

Observation 2: Since the DL ACIR for 10.0-10.5 GHz can be set as less than 25.9dB which is less than the agreed value 29.6dB in last meeting, indoor hotspot is not the restricted scenario for 10.0-10.5 GHz.

Analysis

Based on the analysis and simulation results, the indoor hotspot scenario is not the restricted scenario for both 6.425-7.125 GHz and 10.0-10.5 GHz. There is no need to change and review the agreed targeted DL ACIR in last meeting, based on the simulation results of indoor hotspot scenario.

Proposal: There is no need to change the agreed targeted DL ACIR in last meeting, based on the simulation results of indoor hotspot scenario.

Summary

Based on the simulation results and analysis, all the observations and proposals are listed below:

Observation 1: Since the DL ACIR for 6.425-7.125 GHz can be set as less than 25.9dB which is less than the agreed value 30.9dB in last meeting, indoor hotspot is not the restricted scenario for 6.425-7.125 GHz.

Observation 2: Since the DL ACIR for 10.0-10.5 GHz can be set as less than 25.9dB which is less than the agreed value 29.6dB in last meeting, indoor hotspot is not the restricted scenario for 10.0-10.5 GHz.

Proposal: There is no need to change the agreed targeted DL ACIR in last meeting, based on the simulation results of indoor hotspot scenario.

- References

- R4-2008927, WF on Coexistence Simulations results for 6.425-7.125 GHz and 10.0-10.5 GHz, Huawei, HiSilicon, CATT, Nokia, Ericsson, ZTE