3GPP TSG-RAN WG4 Meeting # 98-e	R4-2101499

# Electronic Meeting, 2-13 Nov., 2020

Agenda item:	10.1.2

Source: 	Huawei

Title: 	TP to TR 38.921: remaining BS parameters

Document for:	Approval

- Introduction

The WF on coexistence simulations results for 6.425-7.125 GHz and 10.0-10.5 GHz was approved in [1].

This paper provides a TP for TR 38.921 to capture the simulation results.

References

[1] R4-2016904, WF on Coexistence Simulations results for 6.425-7.125 GHz and 10.0-10.5 GHz, Huawei, HiSilicon, CATT, Nokia, Ericsson, ZTE

TP to TR 38.921

< START OF CHANGE>

4.3	Co-existence simulation results

DL Simulation results for 6.425-7.125GHz and 10.0-10.5 GHz are presented in Table 4.3-1.

Table 4.3-1: DL simulation results for 6.425-7.125GHz and 10.0-10.5 GHz

|  | ACIR (dB) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Company | 6.425-7.125GHz |  |  | 10.0-10.5GHz |  |  |
|  | Urban macro uncoord. | Indoor | Dense Urban | Urban Macro uncoord. | Indoor | Dense Urban |
| CATT | 29.5 |  |  | 28 |  |  |
| Nokia | 32.7 | 31.7 | 23.7 | 30.7 | 32.2 | 20.7 |
| Huawei | 30.7 | < 25.9 |  | 29.7 | < 25.9 |  |
| Ericsson | 30 |  |  | 29 |  |  |
| ZTE | 31.7 |  |  | 29.7 |  |  |
| Qualcomm | 30.9 |  |  | 30.5 |  |  |
| Average | 30.9 |  |  | 29.6 |  |  |
| Average after removing highest and lowest values | 30.9 |  |  | 29.5 |  |  |

UL Simulation results for 6.425-7.125GHz and 10.0-10.5 GHz are presented in Table 4.3-2.

Table 4.3-2: UL simulation results for 6.425-7.125GHz and 10.0-10.5 GHz

|  | ACIR (dB) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Company | 6.425-7.125GHz |  |  | 10.0-10.5GHz |  |  |
|  | Urban macro uncoord. | Indoor | Dense Urban | Urban Macro uncoord. | Indoor | Dense Urban |
| CATT | 26 |  |  | 26.2 |  |  |
| Nokia | 26.9 |  | 21 | 25.9 |  | Not conclusive |
| Huawei | 27.9 | < 23.9 |  | 25.9 | < 23.9 |  |
| Ericsson | 27 |  |  | 23 |  |  |
| ZTE | 27.9 |  |  | 24.9 |  |  |
| Qualcomm | 22 |  |  | 22 / 21.5(*) |  |  |
| Average value | 26.3 |  |  | 24.6 |  |  |
| Average after removing highest and lowest values | 27 |  |  | 24.9 |  |  |
| Note (*): 22 with 23dBm and 21.5 with 20dBm |  |  |  |  |  |  |

< END OF CHANGE>