3GPP TSG-RAN WG4 Meeting # 98-e                                                              R4-2101598

Electronic Meeting, 25th January– 5th Febuary, 2021

Source:	Huawei, HiSilicon

Title:	TP for TR 38.717-03-01 CA_n8A-n78A-n79A CA_n8A-n78(2A)-n79A

Agenda item:	9.9.2

Document for:	Approval

Background

This contribution provides text proposal on the NR CA band combination CA_n8A-n78A-n79A CA_n8A-n78(2A)-n79A as defined in [1].

Text Proposal

---Start of changes---

6.X	CA_n8-n78-n79

6.X.1	Operating bands for CA

Table 6.X.1-1: 3DL Inter-band CA operating bands

| NR CA Band | NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n8-n78-n79 | n8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

6.X.2	Channel bandwidths per operating band for CA

Table 6.X.2-1: Supported channel bandwidths per CA configuration for 3DL inter-band CA

| NR CA config | UL config | NR Band | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n8A-n78A-n79A | - | n8 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n78 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 |  | 80 | 90 | 100 |  |
|  |  | n79 |  |  |  |  |  |  | 40 | 50 | 60 |  | 80 |  | 100 |  |
| CA_n8A-n78(2A)-n79A | - | n8 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n78 | See CA_n78(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n79 |  |  |  |  |  |  | 40 | 50 | 60 |  | 80 |  | 100 |  |

6.X.3	Co-existence studies

Co-existence studies can be omitted because fallback combinations cover these requirements and analysis.

6.X.4	∆TIB,c and ∆RIB,c values

For three simultaneous DLs and one UL of Band n1, n8 and n79, the TIB,c and RIB,c values are shown in table 6.X.4-1 and table 6.X.4-2, respectively.

Table 6.X.4-1: ΔTIB,c for 3DL aggregation

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n8-n78-n79 | n8 | 0.6 |
|  | n78 | 0.8 |
|  | n79 | 0.8 |

Table 6.X.4-2: ΔRIB,c for 3DL aggregation

| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n8-n78-n79 | n8 | 0.2 |
|  | n78 | 0.5 |
|  | n79 | 0.5 |

6.X.5	REFSENS requirements

MSD studies can be omitted because the corresponding analysis and requirements are covered by fallback combinations.

---End of changes---

Reference

[1]	RP-202720, “Revised WID: Rel-17 NR inter-band CA for 3 bands DL with 1 band UL”, CATT