3GPP TSG-RAN WG4 Meeting #98-e	R4-2101683

Online, 25th Jan– 05th Feb, 2021

Title: 	Discussion on conditions for FR2 new FWA UE

Source: 	Huawei, HiSilicon

Agenda item:	9.24.3

Document for:	Discussion

Introduction

In RAN4#97e meeting, the condition requirements for FR2 new FWA UE have been introduced into TS38.133 and the minimum RP values for FR2 new FWA UE need to be studied.

Discussion

In RAN4 #97e meeting, it was agreed that FR2 new FWA UE is introduced as UE power class 5 in FR2. The minimum RP values used in condition requirements for UE power class 5 depend on how to group the bands for power class 5. According to the principles of band group in TS38.133, the frequency bands grouping is derived based on UE REFSENS requirements specified in TS38.101.

In RF session, the following agreements on REFSENS for FR2 new FWA UE have been achieved in [1].

- Agreements in GTW session: - -92.6dBm (n257) - -92.8dBm (n258)

The group for a band is determined based on the lowest REFSENS requirement normalized by the number of subcarriers among its supported SCSs for its supported bandwidths. Then, the normalized REFSENS of bands n257 and n258 for UE power class 5 can be shown as Table 1.

Table 1: REFSENS for UE power class 5

| Operating Band | SCS kHz | 50 | 100 | 200 | 400 | Duplex Mode | Normalized values | 50 | 100 | 200 | 400 | minimum Io |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | MHz | MHz | MHz | MHz |  |  | MHz | MHz | MHz | MHz | per SCS |
|  |  | (dBm) | (dBm) | (dBm) | (dBm) |  |  | (dBm) | (dBm) | (dBm) | (dBm) | (dBm) |
| n257 (PC1) | 60 | -97.5 | -94.5 | -91.5 | - | TDD | 60 | -126.5 | -126.5 | -126.5 | - | -126.5 |
|  | 120 | -97.5 | -94.5 | -91.5 | -88.5 |  | 120 | -123.3 | -123.5 | -123.5 | -123.5 | -123.5 |
| n257 (PC5) | 60 | -92.6 | - | - | - | TDD | 60 | -121.6 | - | - | - | -121.6 |
|  | 120 | -92.6 | - | - | - |  | 120 | -118.4 | - | - | - | -118.4 |
| n258 (PC5) | 60 | -92.8 | - | - | - | TDD | 60 | -121.8 | - | - | - | -121.8 |
|  | 120 | -92.8 | - | - | - |  | 120 | -118.6 | - | - | - | -118.6 |

Table 1 also provides the normalized REFSENS of band n257 for UE power class 1. Band n257 for UE power class 1 is specified as the group A. The normalized REFSENS difference between band n257 for UE power class 5 and group A is 4.9 dB, and the normalized REFSENS difference between band n258 for UE power class 5 and group A is 4.7 dB. The step between the neighbour groups is defined as 0.5 dB. 4.9 dB and 4.7 dB REFSENS differences can be quantized as 10 steps. Hence, bands n257 and n258 for UE power class 5 can be defined as the group K.

Proposal 1: Bands n257 and n258 for UE power class 5 can be defined as the group K.

According to the derivation of Minimum SSB_RP values for FR2 specified in TS38.133, the methodology to define the Minimum SSB_RP level for power class X (PC_X) and operating band Y (Band_Y) is defined as:

- For Rx Beam Peak angle of arrival For Intra-frequency: Minimum SSB_RP (PC_X, Band_Y) = -109.5 dBm/120kHz + Refsens PC_X, Band_Y, 50MHz – Refsens PC3, n260, 50MHz + Y PC_X – Y PC3 +∆MBP,n, For Inter-frequency: Minimum SSB_RP (PC_X, Band_Y) = -107.5 dBm/120kHz + Refsens PC_X, Band_Y, 50MHz – Refsens PC3, n260, 50MHz + Y PC_X – Y PC3 +∆MBP,n . - For Spherical coverage For Intra-frequency: Minimum SSB_RP (PC_X, Band_Y) = -96.9 dBm/120kHz + EIS spherical coverage PC_X, Band_Y, 50MHz – EIS spherical coverage PC3, n260, 50MHz + Z PC_X – Z PC3 +∆MBS,n For Inter-frequency: Minimum SSB_RP (PC_X, Band_Y) = -94.9 dBm/120kHz + EIS spherical coverage PC_X, Band_Y, 50MHz – EIS spherical coverage PC3, n260, 50MHz + Z PC_X – Z PC3 +∆MBS,n

In RAN4#95e meeting, the following agreements on spherical coverage for were achieved in [2].

- Japan FWA shall have 8 dB degradation in requirement in 85th %ile direction, from peak direction requirement - EIRP in 85%ile direction (CDF) is min peak EIRP – 8 dB - EIS in 85%ile direction (CCDF) is REFSENS + 8 dB

Under the condition of CBW=50MHz, the EIS spherical coverage for UE power class 5 is -84.6dBm for band n257 and -84.8dBm for band n258. According to TS38.101-2, the REFSENS value for UE power class 3 in Band n260 is -85.7dBm and the EIS spherical coverage for UE power class 3 in Band n260 is -73.1dBm. The values of ∆MBP,n and ∆MBS,n for UE power class 3 in Band n260 are 0.5dB and 0.4dB as specified in TS38.101-2 clause 6.2.1.3. In TS38.133, the values of Y and Z for UE power class 3 are defined as 7dB and 7dB respectively. Hence, the Minimum SSB_RP values for UE power class 5 can be calculated as Table 2.

Table 1: Minimum SSB_RP values for UE power class 5

| Case | Direction | Bands for PC5 | Refsens Diff with PC3 (dB) | YPC3 /ZPC3 (dB) | Minimum SSB_RP (dBm/120kHz) |
| --- | --- | --- | --- | --- | --- |
| Intra-freq | Rx Beam Peak | n257 | -6.9 | 7 | -123.4+YPC5 |
|  |  | n258 | -7.1 | 7 | -123.6+YPC5 |
|  | Spherical coverage | n257 | -11.5 | 7 | -115.4+ZPC5 |
|  |  | n258 | -11.7 | 7 | -115.6+ZPC5 |
| Inter-freq | Rx Beam Peak | n257 | -6.9 | 7 | -121.4+YPC5 |
|  |  | n258 | -7.1 | 7 | -121.6+YPC5 |
|  | Spherical coverage | n257 | -11.5 | 7 | -113.4+ZPC5 |
|  |  | n258 | -11.7 | 7 | -113.6+ZPC5 |

Proposal 2: The minimum SSB_RP values for UE power class 5 can be defined as Table 2.

Conclusions

This contribution provides the discussion on the clarification of antenna configurations for 4Rx capable UE. The following are provided:

Proposal 1: Bands n257 and n258 for UE power class 5 can be defined as the group K.

Proposal 2: The minimum SSB_RP values for UE power class 5 can be defined as Table 2.

Based on the proposals, the corresponding CR is provided in [3].

Reference

- R4-2016874, “WF on FR2 FWA RF requirements”, SoftBank Corp

- R4-2008921, “WF on FR2 FWA evaluations”, Qualcomm

- R4-2101684, “ CR on condition requirements for UE power class 5 in TS38.133”, Huawei, HiSilicon