3GPP TSG-RAN WG4 Meeting #98-e	R4-2101785

# Electronic Meeting, 25th Jan. – 5th Feb. 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 1669. | rev |  | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to 38.133 correction on CCSF for NR measurements for positioning |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos-Core |  |  |  |  |  | Date: |  |  | 2021-01-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The definition of NR PRS measurement for positioning with long periodicity has not been finalized. How to specify CCSF for NR PRS measurement for positioning with short periodicity has not been finalized. Requirements of CCSF for PRS-based measurements have not been specified. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Updated definition of NR PRS measurement for positioning with long periodicity - CCSF for PRS measurement with short periodicity is based on “NR PRS measurement on single positioning frequency layer” - CCSF requirements for SSB based measurement are reused for PRS based measurement |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | CCSF requirements for NR PRS measurement for positioning are incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.5.2.2, 9.1.5.2.3, 9.1.5.2.4, 9.1.5.2.5, 9.1.5.2.6, 9.1.5.2.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< Start of change #1 >

#### 9.1.5.2	Monitoring of multiple layers within gaps

The carrier-specific scaling factor CSSFwithin_gap,i for a measurement object i derived in this chapter is applied to following measurement types:

-	SSB-based intra-frequency measurement object with no measurement gap in clause 9.2.5, when all of the SMTC occasions of this intra-frequency measurement object are overlapped by the measurement gap.

-	SSB-based intra-frequency measurement object with measurement gap in clause 9.2.6.

--	CSI-RS based inter-frequency measurement in clause xxx, when CSI-RS resources for L3 measurement of this inter-frequency measurement object are overlapped by the measurement gap.

-	CSI-RS based inter-frequency measurement in clause xxx, when CSI-RS resources for L3 measurement of this inter-frequency measurement object are partially overlapped by the measurement gap.

-	SSB-based inter-frequency measurement object with measurement gap in clause 9.3.4.

-	Including inter-frequency measurement with no measurement gap, when all of the SMTC occasions of this inter-frequency measurement object are overlapped by the measurement gap, if UE supports interFrequencyMeas-NoGap-r16.

-	E-UTRA Inter-RAT measurement object in clauses 9.4.2 and 9.4.3.

-	NR PRS-based measurements for positioning in clause 9.9.

-	E-UTRA Inter-RAT RSTD and E-CID measurements in clauses 9.4.4 and 9.4.5.

-	NR Inter-RAT measurement object configured by the E-UTRAN PCell (TS 36.133 [15] clause 8.17.4).

-	E-UTRAN Inter-frequency measurement object configured by the E-UTRAN PCell (TS 36.133 [15] clause 8.17.3) and by the E-UTRAN PSCell (TS 36.133 [15] clause 8.19.3).

-	E-UTRAN Inter-frequency RSTD measurement configured by the E-UTRAN PCell (TS 36.133 [15] clause 8.17.15).

-	UTRA Inter-RAT measurement object configured by the E-UTRAN PCell (TS 36.133 [15] clauses 8.17.5 to 8.17.12).

-	GSM Inter-RAT measurements configured by the E-UTRAN PCell (TS 36.133 [15] clauses 8.17.13 and 8.17.14).

UE is expected to conduct the measurement of this measurement object i only within the measurement gaps.

If the higher layer signaling in TS 38.331 [2] of smtc2 is present and smtc1 is fully overlapping with measurement gaps and smtc2 is partially overlapping with measurement gaps, CSSFwithin_gap,i and requirements derived from CSSFoutside_gap,i are not specified.

Number of SSB layers should include SSB for mobility and that as associated SSB for CSI-RS mobility. the ssbfrequency is counted only once if the ssbfrequency for mobility and associated SSB are the same, or ssbfrequency and smtc in multiple MOs are the same.

Editor’s note:	FFS how to add the layer corresponding to the associated SSB for a MO with only CSI-RS measurement configured

##### 9.1.5.2.1	EN-DC mode: carrier-specific scaling factor for SSB and CSI-RS-based L3 measurements performed within gaps

The scaling value CSSFwithin_gap,i below has been derived without considering GSM inter-RAT carriers.

When one or more measurement objects are monitored within measurement gaps, the carrier specific scaling factor for a target measurement object with index i is designated as CSSFwithin_gap,i and is derived as described in this clause.

If measurement object i refers to an RSTD measurement with periodicity Tprs>160ms or with periodicity Tprs=160ms but prs-MutingInfo-r9 is configured, CSSFwithin_gap,i=1. Otherwise, the CSSFwithin_gap,i for other measurement objects (including RSTD measurement with periodicity Tprs=160ms) participate in the gap competition are derived as below.

For each measurement gap j not used for an RSTD measurement with periodicity Tprs>160ms or with periodicity Tprs=160ms but prs-MutingInfo-r9 is configured within an arbitrary 160ms period, count the total number of intra-frequency measurement objects and inter-frequency/inter-RAT measurement objects which are candidates to be measured within the gap j.

-	An NR measurement object with SSB measurement configured is a candidate to be measured in a gap if its SMTC duration is fully covered by the MGL excluding RF switching time. For intra-frequency NR carriers, if the higher layer in TS 38.331 [2] signaling of smtc2 is configured, the assumed periodicity of SMTC occasions corresponds to the value of higher layer parameter smtc2; otherwise the assumed periodicity of SMTC occasions corresponds to the value of higher layer parameter smtc1.

-	An NR measurement object with CSI-RS measurement configured is a candidate to be measured in a gap if the window confining all CSI-RS resources are fully covered by the MGL excluding RF switching time. -

-	An inter-RAT UTRA measurement object configured by E-UTRA PCell [15] is a candidate to be measured in all measurement gaps.

-	An inter-frequency E-UTRA measurement object configured by E-UTRA PCell [15] is a candidate to be measured in all measurement gaps.

-	For UEs which support and are configured with per FR gaps, the counting is done on a per FR basis, and for UEs which are configured with per UE gaps the counting is done on a per UE basis.

-	Mintra,i,j: Number of intra-frequency measurement objects, including both SSB and CSI-RS based,  which are candidates to be measured in gap j where the measurement object i is also a candidate. Otherwise Mintra,i,j  equals 0.

-	Minter,i,j : Number of NR inter-frequency layers or NR inter-RAT frequency layer, including both SSB and CSI-RS based, configured by E-UTRA PCell, EUTRA inter-frequency measurement objects configured by E-UTRA PCell, UTRA inter-RAT measurement objects configured by E-UTRA PCell which are candidates to be measured in gap j where the measurement object i is also a candidate. Otherwise Minter,i,j  equals 0.

-	Mtot,i,j = Mintra,i,j + Minter,i,j : Total number of intra-frequency, inter-frequency and inter-RAT frequency layers which are candidates to be measured in gap j where the measurement object i is also a candidate. Otherwise Mtot,i,j equals 0.

For each measurement gap j used for an RSTD measurement with periodicity Tprs>160ms or with periodicity Tprs=160ms but prs-MutingInfo-r9 is configured within an arbitrary 160ms period, Mintra,i,j = Minter,i,j = Mtot,i,j =0.

The carrier specific scaling factor CSSFwithin_gap,i is given by:

If measGapSharingScheme is equal sharing, CSSFwithin_gap,i= max(ceil(Ri×Mtot,i,j)), where j=0…(160/MGRP)-1

If measGapSharingScheme is not equal sharing and

-	measurement object i is an intra-frequency measurement object, CSSFwithin_gap,i is the maximum among

-	ceil(Ri×Kintra×Mintra,i,j) in gaps where Minter,i,j≠0, where j=0…(160/MGRP)-1

-	ceil(Ri×Mintra,i,j) in gaps where Minter,i,j=0, where j=0…(160/MGRP)-1

-	measurement object i is an inter-frequency or inter-RAT measurement object, CSSFwithin_gap,i is the maximum among

-	ceil(Ri×Kinter×Minter,i,j) in gaps where Mintra,i,j ≠0, where j=0…(160/MGRP)-1

-	ceil(Ri×Minter,i,j) in gaps where Mintra,i,j=0, where j=0…(160/MGRP)-1

Where Ri is the maximal ratio of the number of measurement gap where measurement object i is a candidate to be measured over the number of measurement gap where measurement object i is a candidate and not used for RSTD measurement with periodicity Tprs>160ms or with periodicity Tprs=160ms but prs-MutingInfo-r9 is configured within an arbitrary 1280ms period.

##### 9.1.5.2.2	SA mode: carrier-specific scaling factor for SSB and CSI-RS-based L3 measurements performed within gaps

When one or more measurement objects are monitored within measurement gaps, the carrier specific scaling factor for a target measurement object with index i is designated as CSSFwithin_gap,i and is derived as described in this clause.

If measurement object i refers to a long-periodicity measurement which is any of:

-	an E-UTRA RSTD measurement with periodicity Tprs>160ms or with periodicity Tprs=160ms but prs-MutingInfo-r9 is configured, or

-	an NR PRS measurement for positioning with periodicity Tprs>160ms if mutingOption1 is not configured, or with periodicity Tprs-muting >160ms if mutingOption1 is configured, where Tprs-muting = Tprs * X * dl-prs-MutingBitRepetitionFactor and X is the size of NR-MutingPattern-r16 configured in DL-PRS-MutingOption1-r16.

then CSSFwithin_gap,i=1. Otherwise, the CSSFwithin_gap,i for other measurement objects (including E-UTRA RSTD measurement with periodicity Tprs=160ms) participate in the gap competition and the CSSFwithin_gap,i are derived as below.

Table 9.1.5.2.2-1: void

For each measurement gap j not used for a long-periodicity measurement defined above, count the total number of intra-frequency measurement objects and inter-frequency/inter-RAT measurement objects and NR PRS measurement on single positioning frequency layer which are candidates to be measured within the gap j.

-	An NR measurement object with SSB measurement configured is a candidate to be measured in a gap if its SMTC duration is fully covered by the MGL excluding RF switching time. For intra-frequency NR measurement objects, if the higher layer in TS 38.331 [2] signaling of smtc2 is configured, the assumed periodicity of SMTC occasions corresponds to the value of higher layer parameter smtc2; otherwise the assumed periodicity of SMTC occasions corresponds to the value of higher layer parameter smtc1.

-	An NR measurement object with CSI-RS measurement configured is a candidate to be measured in a gap if the window confining all CSI-RS resources are fully covered by the MGL excluding RF switching time.

-	An inter-frequency SFTD measurement object, if to be measured with measurement gaps, is a candidate to be measured in all measurement gaps.

-	For UEs which support and are configured with per FR gaps, the counting is done on a per FR basis, and for UEs which are configured with per UE gaps the counting is done on a per UE basis. For UEs which support and are configured with per FR gaps, the CSSF requirements do not apply when NR PRS measurement in one FR gap collides with SSB/CSI-RS/PRS measurements in the other FR gap in time domain.

-	Mintra,i,j: Number of intra-frequency measurement objects, including both SSB and CSI-RS based, which are candidates to be measured in gap j where the measurement object i is also a candidate. Otherwise Mintra,i,j  equals 0.

-	Minter,i,j : Number of NR inter-frequency, EUTRA inter-RAT and UTRA inter-RAT frequency layers, including both SSB and CSI-RS based, NR PRS measurement on single positioning frequency layer, which are candidates to be measured in gap j where the measurement object i is also a candidate. Otherwise Minter,i,j  equals 0.

-	Mtot,i,j = Mintra,i,j + Minter,i,j : Total number of intra-frequency, inter-frequency and inter-RAT frequncy layers and NR PRS measurement on single positioning frequency layer, which are candidates to be measured in gap j where the measurement object i is also a candidate. Otherwise Mtot,i,j equals 0.

For each measurement gap j used for a long-periodicity measurement defined above, Mintra,i,j = Minter,i,j = Mtot,i,j =0. The carrier specific scaling factor CSSFwithin_gap,i is given by:

If measGapSharingScheme is equal sharing, CSSFwithin_gap,i= max(ceil(Ri×Mtot,i,j)), where j=0…(160/MGRP)-1

If measGapSharingScheme is not equal sharing and

-	measurement object i is an intra-frequency measurement object, CSSFwithin_gap,i is the maximum among

-	ceil(Ri×Kintra×Mintra,i,j) in gaps where Minter,i,j≠0, where j=0…(160/MGRP)-1

-	ceil(Ri×Mintra,i,j) in gaps where Minter,i,j=0, where j=0…(160/MGRP)-1

-	measurement object i is an inter-frequency or inter-RAT measurement object or NR PRS measurement on single positioning frequency layer, CSSFwithin_gap,i is the maximum among

-	ceil(Ri×Kinter×Minter,i,j) in gaps where Mintra,i,j ≠0, where j=0…(160/MGRP)-1

-	ceil(Ri×Minter,i,j) in gaps where Mintra,i,j=0, where j=0…(160/MGRP)-1

Where Ri is the maximal ratio of the number of measurement gap where measurement object i is a candidate to be measured over the number of measurement gap where measurement object i is a candidate and not used for a long-periodicity measurement defined above.

CSSFwithin_gap,k=1 during TDetect, E-UTRAN FDD specified in clause 9.4.4.1.2.2 and TDetect, E-UTRAN TDD specified in clause 9.4.4.2.2.2, where k is the carrier frequency where the UE is performing cell detection of the inter-RAT E-UTRA OTDOA assistance data reference cell when acquiring the subframe and slot timing of the cell according to clause 9.4.4. In this case, the UE cell identification and measurement periods derived based on CSSFwithin_gap,i in clauses 9.2.5.1, 9.2.5.2, 9.2.6.2, 9.2.6.3, 9.3.4, 9.3.5, 9.4.2.2, 9.4.2.3 and 9.10.2 may be extended for measurement objects of which the cell identification and measurement periods are overlapped with TDetect, E-UTRAN FDD and TDetect, E-UTRAN TDD.

##### 9.1.5.2.3	NE-DC: carrier-specific scaling factor for SSB-based and CSI-RS based L3 measurements performed within gaps

When one or more measurement objects are monitored within measurement gaps, the carrier specific scaling factor for a target measurement object with index i is designated as CSSFwithin_gap,i and is derived as described in this clause.

If measurement object i refers to a long-periodicty measurement which is any of:

-	an E-UTRA RSTD measurement with periodicity Tprs>160ms or with periodicity Tprs=160ms but prs-MutingInfo-r9 is configured, or

-	an NR PRS measurement for positioning with periodicity Tprs>160ms if mutingOption1 is not configured, or with periodicity Tprs-muting >160ms if mutingOption1 is configured, where Tprs-muting = Tprs * X * dl-prs-MutingBitRepetitionFactor and X is the size of NR-MutingPattern-r16 configured in DL-PRS-MutingOption1-r16.

then CSSFwithin_gap,i=1. Otherwise, the CSSFwithin_gap,i for other measurement objects (including E-UTRA RSTD measurement with periodicity Tprs=160ms) participate in the gap competition are derived as below.

For each measurement gap j not used for a long-periodicity measurement defined above, count the total number of intra-frequency measurement objects and inter-frequency/inter-RAT measurement objects and [TBD for NR positioning measurements] which are candidates to be measured within the gap j.

-	An NR measurement object with SSB measurement configured is a candidate to be measured in a gap if its SMTC duration is fully covered by the MGL excluding RF switching time. For intra-frequency NR measurement objects, if the higher layer in TS 38.331 [2] signaling of smtc2 is configured, the assumed periodicity of SMTC occasions corresponds to the value of higher layer parameter smtc2; otherwise the assumed periodicity of SMTC occasions corresponds to the value of higher layer parameter smtc1.

-	An NR measurement object with CSI-RS measurement configured is a candidate to be measured in a gap if the window confining all CSI-RS resources are fully covered by the MGL excluding RF switching time.

-	An inter-RAT measurement object is a candidate to be measured in all measurement gaps.

-	An inter-frequency E-UTRA measurement object is a candidate to be measured in all measurement gaps.

For UEs which support and are configured with per FR gaps, the counting is done on a per FR basis, and for UEs which are configured with per UE gaps the counting is done on a per UE basis. For UEs which support and are configured with per FR gaps, the CSSF requirements do not apply when NR PRS measurement in one FR gap collides with SSB/CSI-RS/PRS measurements in the other FR gap in time domain.

If the number of configured interfrequency and interRAT measuerement objects and NR PRS measurement on single positioning frequency layer is non-zero and the UE is configured with per UE gaps, or if the UE is configured with per FR gaps:

FR1 and FR2 intrafrequency measurement objects belong to group A

Interfrequency and interRAT measurement objects belong to group B

MgroupA,i,j: Sum of the number of FR1 intra-frequency measurement objects Mintra-FR1,i,j and the number of FR2 intra-frequency measurement objects Mintra-FR2,i,j , including both SSB and CSI-RS based, which are candidates to be measured in gap j where the measurement object i is also a candidate. Otherwise MgroupA,i,j  equals 0.

MgroupBi,j: Number of NR inter-frequency, EUTRA inter-RAT and UTRA inter-RAT measurement objects, including both SSB and CSI-RS based,  NR PRS measurement on single positioning frequency layer, which are candidates to be measured in gap j where the measurement object i is also a candidate. Otherwise MgroupB,i,j  equals 0.

If the number of configured inter-frequency and inter-RAT measuerement objects and NR PRS measurement on single positioning frequency layer is zero and the UE is configured with per UE gaps:

FR1 intrafrequency measurement objects belong to group A

FR2 intrafrequency measurement objects belong to group B

MgroupA,i,j: The number of FR1 intrafrequency measurement objects Mintra-FR1,i,j , including both SSB and CSI-RS based, which are candidates to be measured in gap j where the measurement object i is also a candidate. Otherwise MgroupA,i,j  equals 0.

MgroupBi,j : The number of FR2 intrafrequency measurement objects Mintra-FR2,i,j , including both SSB and CSI-RS based, which are candidates to be measured in gap j where the measurement object i is also a candidate. Otherwise MgroupB,i,j  equals 0.

Mtot,i,j = MgroupA,i,j + MgroupB,i,j : Total number of group A and group B measurement objects which are candidates to be measured in gap j where the measurement object i is also a candidate. Otherwise Mtot,i,j equals 0.

For each measurement gap j used for a long-periodicity measurement defined above, Mintra,i,j = Minter,i,j = Mtot,i,j =0. The carrier specific scaling factor CSSFwithin_gap,i is given by:

If measGapSharingScheme is equal sharing, CSSFwithin_gap,i= max(ceil(Ri×Mtot,i,j)), where j=0…(160/MGRP)-1

If measGapSharingScheme is not equal sharing and

-	measurement object i is a group A measurement object, CSSFwithin_gap,i is the maximum among

-	ceil(Ri×Kintra×MgroupA,i,j) in gaps where MgroupB,i,j≠0, where j=0…(160/MGRP)-1

-	ceil(Ri×MgroupA,i,j) in gaps where MgroupB,i,j=0, where j=0…(160/MGRP)-1

-	measurement object i is an group B measurement object, CSSFwithin_gap,i is the maximum among

-	ceil(Ri×Kinter×MgroupBi,j) in gaps where MgroupA,i,j ≠0, where j=0…(160/MGRP)-1

-	ceil(Ri×MgroupB,i,j) in gaps where MgroupA,i,j=0, where j=0…(160/MGRP)-1

Where Ri is the maximal ratio of the number of measurement gap where measurement object i is a candidate to be measured over the number of measurement gap where measurement object i is a candidate and not used for a long-periodicity measurement defined above.

##### 9.1.5.2.4	NR-DC: carrier-specific scaling factor for SSB-based and CSI-RS-based L3 measurements performed within gaps

When one or more measurement objects are monitored within measurement gaps, the carrier specific scaling factor for a target measurement object with index i is designated as CSSFwithin_gap,i and is derived as described in this clause.

If measurement object i refers to a long-periodicity measurement which is any of:

-	an E-UTRA RSTD measurement with periodicity Tprs>160ms or with periodicity Tprs=160ms but prs-MutingInfo-r9 is configured, or

-	an NR PRS measurement for positioning with periodicity Tprs>160ms if mutingOption1 is not configured, or with periodicity Tprs-muting >160ms if mutingOption1 is configured, where Tprs-muting = Tprs * X * dl-prs-MutingBitRepetitionFactor and X is the size of NR-MutingPattern-r16 configured in DL-PRS-MutingOption1-r16.

then CSSFwithin_gap,i=1. Otherwise, the CSSFwithin_gap,i for other measurement objects (including E-UTRA RSTD measurement with periodicity Tprs=160ms) participate in the gap competition and the CSSFwithin_gap,i are derived as below.

For each measurement gap j not used for an RSTD measurement with periodicity Tprs>160ms or with periodicity Tprs=160ms but prs-MutingInfo-r9 is configured within an arbitrary 160ms period, count the total number of intra-frequency measurement objects and inter-frequency/inter-RAT measurement objects and NR PRS measurement on single positioning frequency layer which are candidates to be measured within the gap j.

-	An NR measurement object with SSB measurement configured is a candidate to be measured in a gap if its SMTC duration is fully covered by the MGL excluding RF switching time. For intra-frequency NR measurement objects, if the higher layer in TS 38.331 [2] signaling of smtc2 is configured, the assumed periodicity of SMTC occasions corresponds to the value of higher layer parameter smtc2; otherwise the assumed periodicity of SMTC occasions corresponds to the value of higher layer parameter smtc1.

-	An NR measurement object with CSI-RS measurement configured is a candidate to be measured in a gap if the window confining all CSI-RS resources are fully covered by the MGL excluding RF switching time.

For UEs which support and are configured with per FR gaps, the counting is done on a per FR basis, and for UEs which are configured with per UE gaps the counting is done on a per UE basis. For UEs which support and are configured with per FR gaps, the CSSF requirements do not apply when NR PRS measurement in one FR gap collides with SSB/CSI-RS/PRS measurements in the other FR gap in time domain.

If the number of configured interfrequency and interRAT measuerement objects and NR PRS measurement on single positioning frequency layer is non-zero and the UE is configured with per UE gaps, or if the UE is configured with per FR gaps:

FR1 and FR2 intrafrequency measurement objects belong to group A

Interfrequency and interRAT measurement objects and NR PRS measurement on single positioning frequency layer belong to group B

MgroupA,i,j: Sum of the number of FR1 intra-frequency measurement objects Mintra-FR1,i,j and the number of FR2 intra-frequency measurement objects Mintra-FR2,i,j , including both SSB and CSI-RS based, which are candidates to be measured in gap j where the measurement object i is also a candidate. Otherwise MgroupA,i,j  equals 0.

MgroupBi,j : Number of NR inter-frequency, EUTRA inter-RAT and UTRA inter-RAT measurement objects and NR PRS measurement on single positioning frequency layer , including both SSB and CSI-RS based, which are candidates to be measured in gap j where the measurement object i is also a candidate. Otherwise MgroupB,i,j  equals 0.

If the number of configured interfrequency and interRAT measuerement objects and NR PRS measurement on single positioning frequency layer is zero and the UE is configured with per UE gaps:

FR1 intrafrequency measurement objects belong to group A

FR2 intrafrequency measurement objects belong to group B

MgroupA,i,j: The number of FR1 intrafrequency measurement objects Mintra-FR1,i,j , including both SSB and CSI-RS based, which are candidates to be measured in gap j where the measurement object i is also a candidate. Otherwise MgroupA,i,j  equals 0.

MgroupBi,j : The number of FR2 intrafrequency measurement objects Mintra-FR2,i,j , including both SSB and CSI-RS based, which are candidates to be measured in gap j where the measurement object i is also a candidate. Otherwise MgroupB,i,j  equals 0.

Mtot,i,j = MgroupA,i,j + MgroupB,i,j : Total number of group A and group B measurement objects which are candidates to be measured in gap j where the measurement object i is also a candidate. Otherwise Mtot,i,j equals 0.

For each measurement gap j used for a long-periodicity measurement defined above, Mintra,i,j = Minter,i,j = Mtot,i,j =0. The carrier specific scaling factor CSSFwithin_gap,i is given by:

If measGapSharingScheme is equal sharing, CSSFwithin_gap,i= max(ceil(Ri×Mtot,i,j)), where j=0…(160/MGRP)-1

If measGapSharingScheme is not equal sharing and

-	measurement object i is a group A measurement object, CSSFwithin_gap,i is the maximum among

-	ceil(Ri×Kintra×MgroupA,i,j) in gaps where MgroupB,i,j≠0, where j=0…(160/MGRP)-1

-	ceil(Ri×MgroupA,i,j) in gaps where MgroupB,i,j=0, where j=0…(160/MGRP)-1

-	measurement object i is an group B measurement object, CSSFwithin_gap,i is the maximum among

-	ceil(Ri×Kinter×MgroupBi,j) in gaps where MgroupA,i,j ≠0, where j=0…(160/MGRP)-1

-	ceil(Ri×MgroupB,i,j) in gaps where MgroupA,i,j=0, where j=0…(160/MGRP)-1

Ri is the maximal ratio of the number of measurement gap where measurement object i is a candidate to be measured over the number of measurement gap where measurement object i is a candidate and not used for a long-periodicity measurement defined above.

##### 9.1.5.2.5	SA mode: carrier-specific scaling factor for PRS-based measurements performed within gaps

The requirements in this clause apply for NR PRS-based measurements for positioning in clause 9.9.

The scaling value CSSFwithin_gap,i specified in clause 9.1.5.2.2 shall apply.

##### 9.1.5.2.6	NE-DC: carrier-specific scaling factor for PRS-based measurements performed within gaps

The requirements in this clause apply for NR PRS-based measurements for positioning in clause 9.9.

The scaling value CSSFwithin_gap,i specified in clause 9.1.5.2.3 shall apply.

##### 9.1.5.2.7	NR-DC: carrier-specific scaling factor for PRS-based measurements performed within gaps

The requirements in this clause apply for NR PRS-based measurements for positioning in clause 9.9.

The scaling value CSSFwithin_gap,i specified in clause 9.1.5.2.4 shall apply.

< End of change #1 >