3GPP TSG RAN WG4 Meeting #98-e	R4-2101960

E-meeting,25th Jan – 5th Feb, 2021

Agenda Item:	7.4.3.2.1

Source: 	ZTE Corporation

Title:	Discussion on IAB test configurations

Document for:	Approval

- Introduction

During RAN4#97e, WF for manufacturer declarations, test models and configurations including Rx FRC was approved for further study, therefore in this contribution, we want to share some initial considerations on test configurations perspective.

- Discussion

Firstly of all, since in-band/guard band NB-IoT is not supported for AAS based architecture, therefore NB-IoT should be removed from IAB-DU as IAB-DU only support IAB-DU 1-H and 1-O which is AAS based architecture.

Proposal 1: to remove in-band NB-IoT from test configurations for IAB-DU 1-H and 1-O.

Secondly, as discussed in the last RAN4 meeting, lots of IAB-DU and IAB-MT have the same requirements. In addition, it’s highly possible that IAB-DU and IAB-MT share the same hardware which means the same configuration could be applied for both IAB-DU and IAB-MT, therefore we propose to have the same test configurations for IAB-DU and IAB-MT which could be inherited from existing BS test configurations.

Proposal 2: to define the same test configurations for IAB-DU and IAB-MT.

One exception case is for IAB-MT, the minimum channel bandwidth for IAB-MT is 10MHz for FR1, therefore the following table should be updated with 10MHz for FDL_high – FDL_low <100 MHz.

Table 4.7.2-1: Signal to be used to build NR TCs

| Operating Band characteristics |  | FDL_high – FDL_low <100 MHz | FDL_high – FDL_low ≥ 100 MHz |
| --- | --- | --- | --- |
| TC signal | BWchannel | 5 MHz (Note 1) | 20 MHz (Note 1) |
| characteristics | Subcarrier spacing | Smallest supported subcarrier spacing |  |
| NOTE 1: If this channel bandwidth is not supported, the narrowest supported channel bandwidth shall be used. |  |  |  |

- Conclusion

In this contribution, we share some initial understandings on the IAB-MT test configurations and proposals are made as following:

Proposal 1: to remove in-band NB-IoT  from test configurations for IAB-DU 1-H and 1-O.

Proposal 2: to define the same test configurations for IAB-DU and IAB-MT.

- References

- R4-2017672,WF on manufacturer declarations, test models and configurations including Rx FRC, Ericsson, approved.