3GPP TSG-RAN WG4 Meeting #98-e	R4-2102041

Electronic Meeting, 25 January – 5 February 2021

Source:	Ericsson

Title:	TP for TR 37.717-41-11: Including band combinations DC_2A-7A-66A-71A_n2A

Agenda item:		9.6.2

Document for:	Approval

# 1. Introduction

A text proposal for TR 37.717-31-11 to add  DC_2A-7A-66A-71A_n2A.

# 2. Text Proposal

---Start of changes---

### 5.1.X	DC_2-7-66-71_n2

#### 5.1.X.1	Configuration for EN-DC

Based on values for DC_2-7-66_n71.

Table 5.1.X.1-1: Band combinations EN-DC (five bands)

| EN-DC band configuration | UL configuration(s) |
| --- | --- |
| DC_2A-7A-66A-71A_n2A | DC_7A_n2A DC_66A_n2A DC_71A_n2A |

#### 5.1.X.2	∆TIB and ∆RIB values

Table 5.1.X.2.-1: ΔTIB,c due to EN-DC (five bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2A-7A-66A-71A_n2A | 2 | 0.5 |
|  | 7 | 0.5 |
|  | 66 | 0.5 |
|  | 71 | 0.3 |
|  | n2 | 0.5 |

Table 5.1.X.2.-2: ΔRIB,c due to EN-DC (five bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_2A-7A-66A-71A_n2A | 2 | 0.3 |
|  | 7 | 0.5 |
|  | 66 | 0.5 |
|  | 71 | 0 |
|  | n2 | 0.3 |

#### 5.1.X.3	Reference sensitivity exceptions

Compared to its fallback modes, there are no additional MSD requirements for this band combination.

---End of changes---

# Reference

[1]		RP-202590, “Rel-17 WID on DC of 4 bands LTE inter-band CA (4DL1UL) and 1 NR band (1DL1U)” Nokia