3GPP TSG-RAN WG4 Meeting #98-e                                                       R4-2102118

Electronic meeting, 2021

Agenda item:	7.8.1.1.2

Source:	Ericsson

Title:	Simulation results on URLLC UE CQI reporting requirements for CQI table 3

Document for:	Information

# Introduction

In this paper, we present our simulation results on URLLC UE CQI reporting test for CQI table 3.

# Simulation Results

## Simulation configurations

Our simulations are performed based on the configurations proposed in WF[1].

Table 1 Simulation configurations for CQI table 3 tests

| Parameter |  |  | Unit | FDD | TDD |
| --- | --- | --- | --- | --- | --- |
| Bandwidth |  |  | MHz | 10 | 40 |
| Number of allocated PDSCH resource blocks |  |  |  | 52 | 106 |
| Subcarrier spacing |  |  | kHz | 15 | 30 |
| MCS table |  |  |  | Table 3 |  |
| PDSCH starting symbol/length |  |  |  | 2/12 |  |
| Number of PDSCH MIMO layers |  |  |  | 1 |  |
| PDSCH mapping type |  |  |  | Type A |  |
| DMRS type |  |  |  | Type 1 |  |
| DMRS duration |  |  |  | single-symbol DMRS |  |
| Number of additional DMRS |  |  |  | 1 |  |
| Slot pattern |  |  |  | N/A | 7D1S2U, S=6D: 4G: 4U |
| Propagation channel |  |  |  | AWGN |  |
| Antenna configuration |  |  |  | 1x2, ULA low​ 1x4, ULA low​ |  |
| Beamforming Model |  |  |  | As specified in Annex B.4.1 |  |
| ZP CSI-RS configuration | CSI-RS resource Type |  |  | Periodic |  |
|  | Number of CSI-RS ports (X) |  |  | 1A |  |
|  | CDM Type |  |  | No CDM |  |
|  | Density (ρ) |  |  | 1 |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  |  | Row 2,4 |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | 9 |  |
|  | CSI-RS periodicity and offset |  | slot | 5/1 | 10/1 |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  |  | Periodic |  |
|  | Number of CSI-RS ports (X) |  |  | 1 |  |
|  | CDM Type |  |  | No CDM |  |
|  | Density (ρ) |  |  | 3 |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1) |  |  | Row 1,(0,-) |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | 13 |  |
|  | NZP CSI-RS-timeConfig periodicity and offset |  | slot | 5/1 | 10/1 |
| CSI-IM configuration | CSI-IM resource Type |  |  | Periodic |  |
|  | CSI-IM RE pattern |  |  | 0 |  |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  |  | (4, 9) |  |
|  | CSI-IM timeConfig periodicity and offset |  | slot | 5/1 | 10/1 |
| ReportConfigType |  |  |  | Periodic |  |
| CQI-table |  |  |  | Table 3 |  |
| reportQuantity |  |  |  | cri-RI-PMI-CQI |  |
| timeRestrictionForChannelMeasurements |  |  |  | Not configured |  |
| timeRestrictionForInterferenceMeasurements |  |  |  | Not configured |  |
| cqi-FormatIndicator |  |  |  | Wideband |  |
| pmi-FormatIndicator |  |  |  | Wideband |  |
| Sub-band Size |  |  | RB | 8 | 16 |
| Csi-ReportingBand |  |  |  | 1111111 |  |
| CSI-Report periodicity and offset |  |  | slot | 5/0 | 10/9 |
| aperiodicTriggeringOffset |  |  |  | Not configured |  |
| Codebook configuration |  | Codebook Type |  | typeI-SinglePanel |  |
|  |  | Codebook Mode |  | 1 |  |
|  |  | (CodebookConfig-N1,CodebookConfig-N2) |  | Not configured |  |
|  |  | CodebookSubsetRestriction |  | 010000 |  |
|  |  | RI Restriction |  | 00000001 |  |
| Physical channel for CSI report |  |  |  | PUCCH |  |
| CQI/RI/PMI delay |  |  | ms | 8 | 9.5 |
| Maximum number of HARQ transmission |  |  |  | 1 |  |
| Target BLER |  |  |  | 10^-5 |  |
| PBCH |  |  |  | slot#0 per 20ms periodicity |  |
| PT-RS |  |  |  | Disabled |  |
| PDSCH is not scheduled on slots containing CSI-RS or slots which are not full DL |  |  |  |  |  |

## Simulation results

### The simulation results for different scenarios are summarized below.

Table 2 CQI reporting results for FDD with 1Tx2Rx antenna configuration

| SNR [dB] | Median CQI Index | BLER with following CQI | Percentage of Reported CQI Index in {median CQI -1, median CQ1 +1} (%) | BLER with median CQI-1 | BLER with median CQI | BLER with median CQI+1 |
| --- | --- | --- | --- | --- | --- | --- |
| -12 | 0 | 1.6427e-01 | 100 | 1.6427e-01 | 1.6427e-01 | 1.6427e-01 |
| -11.5 | 0 | 1.1720e-01 | 100 | 1.1720e-01 | 1.1720e-01 | 1.1720e-01 |
| -11 | 0 | 6.7733e-02 | 100 | 6.7733e-02 | 6.7733e-02 | 6.7733e-02 |
| -10.5 | 1 | 2.1600e-02 | 100 | 2.1600e-02 | 2.1600e-02 | 1.7067e-01 |
| -10 | 1 | 2.6144e-03 | 100 | 2.6144e-03 | 2.6144e-03 | 1.2573e-01 |
| -9.5 | 1 | 1.0831e-04 | 100 | 1.3239e-04 | 1.3289e-04 | 6.6533e-02 |
| -9 | 2 | 1.3333e-02 | 100 | 0 | 1.3333e-02 | 1.6400e-01 |
| -8.5 | 2 | 6.0332e-04 | 100 | 0 | 6.0332e-04 | 1.0653e-01 |
| -8 | 2 | 6.6211e-06 | 100 | 0 | 1.8328e-05 | 3.9867e-02 |
| -7.5 | 3 | 2.9565e-03 | 100 | 0 | 2.9565e-03 | 1.4733e-01 |
| -7 | 3 | 2.1519e-05 | 100 | 0 | 1.7719e-05 | 7.9867e-02 |
| -6.5 | 4 | 1.4933e-02 | 100 | 0 | 1.4933e-02 | 5.7853e-01 |
| -6 | 4 | 1.5981e-04 | 100 | 0 | 1.3289e-04 | 1.8187e-01 |
| -5.5 | 4 | 1.6563e-06 | 100 | 0 | 0 | 1.0200e-01 |
| -5 | 4 | 0 | 100 | 0 | 0 | 1.5733e-02 |
| -4.5 | 5 | 0 | 100 | 0 | 8.8594e-05 | 8.8920e-01 |
| -4 | 5 | 0 | 100 | 0 | 0 | 1.9853e-01 |
| -3.5 | 5 | 0 | 100 | 0 | 0 | 8.5067e-02 |
| -3 | 6 | 3.9619e-03 | 100 | 0 | 3.9619e-03 | 1.0000e+00 |
| -2.5 | 6 | 0 | 100 | 0 | 0 | 8.7747e-01 |
| -2 | 6 | 0 | 100 | 0 | 0 | 1.6333e-01 |
| -1.5 | 6 | 0 | 100 | 0 | 0 | 2.5067e-02 |
| -1 | 7 | 0 | 100 | 0 | 8.2474e-05 | 9.9987e-01 |
| -0.5 | 7 | 0 | 100 | 0 | 0 | 3.2000e-01 |
| 0 | 7 | 0 | 100 | 0 | 0 | 5.5867e-02 |
| 0.5 | 8 | 0 | 100 | 0 | 4.9938e-04 | 1.0000e+00 |
| 1 | 8 | 0 | 100 | 0 | 0 | 9.7827e-01 |
| 1.5 | 8 | 0 | 100 | 0 | 0 | 2.0120e-01 |
| 2 | 8 | 0 | 100 | 0 | 0 | 1.6800e-02 |
| 2.5 | 9 | 0 | 100 | 0 | 8.8594e-06 | 1.0000e+00 |
| 3 | 9 | 0 | 100 | 0 | 0 | 8.7107e-01 |
| 3.5 | 9 | 0 | 100 | 0 | 0 | 1.4120e-01 |
| 4 | 9 | 0 | 100 | 0 | 0 | 5.4321e-03 |
| 4.5 | 10 | 0 | 100 | 0 | 8.8594e-06 | 1.0000e+00 |
| 5 | 10 | 0 | 100 | 0 | 0 | 5.3067e-01 |
| 5.5 | 10 | 0 | 100 | 0 | 0 | 6.0267e-02 |
| 6 | 11 | 1.2477e-03 | 100 | 0 | 1.2477e-03 | 1.0000e+00 |
| 6.5 | 11 | 0 | 100 | 0 | 0 | 9.9227e-01 |
| 7 | 11 | 0 | 100 | 0 | 0 | 4.1773e-01 |
| 7.5 | 11 | 0 | 100 | 0 | 0 | 2.8000e-02 |
| 8 | 12 | 0 | 100 | 0 | 2.6578e-05 | 1.0000e+00 |
| 8.5 | 12 | 0 | 100 | 0 | 0 | 8.8147e-01 |
| 9 | 12 | 0 | 100 | 0 | 0 | 8.7867e-02 |
| 9.5 | 13 | 0 | 100 | 0 | 1.6833e-04 | 1.0000e+00 |
| 10 | 13 | 0 | 100 | 0 | 0 | 1.0000e+00 |
| 10.5 | 13 | 0 | 100 | 0 | 0 | 7.8520e-01 |
| 11 | 13 | 0 | 100 | 0 | 0 | 1.2347e-01 |
| 11.5 | 14 | 7.7333e-03 | 100 | 0 | 7.7333e-03 | 1.0000e+00 |
| 12 | 14 | 1.3242e-05 | 100 | 0 | 8.8594e-06 | 9.9733e-01 |
| 12.5 | 14 | 0 | 100 | 0 | 0 | 9.3360e-01 |
| 13 | 14 | 0 | 100 | 0 | 0 | 1.1987e-01 |
| 13.5 | 15 | 0 | 100 | 0 | 8.7179e-04 | 8.7179e-04 |
| 14 | 15 | 0 | 100 | 0 | 9.1638e-06 | 9.1638e-06 |
| 14.5 | 15 | 0 | 100 | 0 | 0 | 0 |
| 15 | 15 | 0 | 100 | 0 | 0 | 0 |

Table 3 CQI reporting results for FDD with 1Tx4Rx antenna configuration

| SNR [dB] | Median CQI Index | BLER with following CQI | Percentage of Reported CQI Index in {median CQI -1, median CQ1 +1} (%) | BLER with median CQI-1 | BLER with median CQI | BLER with median CQI+1 |
| --- | --- | --- | --- | --- | --- | --- |
| -12 | 2 | 4.8571e-03 | 100 | 5.8874e-04 | 4.8571e-03 | 3.1107e-01 |
| -11.5 | 2 | 2.6144e-03 | 100 | 8.2474e-05 | 2.6144e-03 | 2.2933e-02 |
| -11 | 2 | 1.1556e-03 | 100 | 0 | 1.1556e-03 | 6.9841e-03 |
| -10.5 | 3 | 4.4444e-03 | 100 | 1.1517e-04 | 4.4444e-03 | 1.7453e-01 |
| -10 | 3 | 2.2295e-03 | 100 | 0 | 2.2295e-03 | 1.1867e-02 |
| -9.5 | 4 | 7.6000e-03 | 100 | 2.3920e-04 | 7.6000e-03 | 9.9520e-01 |
| -9 | 4 | 3.6757e-03 | 100 | 0 | 3.6757e-03 | 4.8773e-01 |
| -8.5 | 4 | 7.5330e-04 | 100 | 0 | 7.5330e-04 | 1.7200e-02 |
| -8 | 4 | 1.9863e-05 | 100 | 0 | 0 | 7.6000e-03 |
| -7.5 | 5 | 3.4472e-03 | 100 | 0 | 3.4472e-03 | 9.9987e-01 |
| -7 | 5 | 5.2107e-04 | 100 | 0 | 5.2107e-04 | 5.9640e-01 |
| -6.5 | 5 | 0 | 100 | 0 | 0 | 1.2933e-02 |
| -6 | 6 | 6.3492e-03 | 100 | 0 | 6.3492e-03 | 1.0000e+00 |
| -5.5 | 6 | 1.5238e-03 | 100 | 0 | 1.5238e-03 | 9.9933e-01 |
| -5 | 6 | 1.4898e-05 | 100 | 0 | 1.8328e-05 | 3.4427e-01 |
| -4.5 | 6 | 0 | 100 | 0 | 0 | 8.8000e-03 |
| -4 | 7 | 2.2295e-03 | 100 | 0 | 2.2295e-03 | 1.0000e+00 |
| -3.5 | 7 | 5.7935e-05 | 100 | 0 | 6.4147e-05 | 8.7160e-01 |
| -3 | 7 | 0 | 100 | 0 | 0 | 2.8667e-02 |
| -2.5 | 8 | 2.8800e-03 | 100 | 0 | 2.8800e-03 | 1.0000e+00 |
| -2 | 8 | 5.1314e-05 | 100 | 0 | 6.2016e-05 | 1.0000e+00 |
| -1.5 | 8 | 0 | 100 | 0 | 0 | 3.8893e-01 |
| -1 | 8 | 0 | 100 | 0 | 0 | 9.8667e-03 |
| -0.5 | 9 | 6.1162e-04 | 100 | 0 | 6.1162e-04 | 1.0000e+00 |
| 0 | 9 | 0 | 100 | 0 | 0 | 9.9760e-01 |
| 0.5 | 9 | 0 | 100 | 0 | 0 | 1.4427e-01 |
| 1 | 9 | 0 | 100 | 0 | 0 | 3.5556e-03 |
| 1.5 | 10 | 0 | 100 | 0 | 1.1913e-04 | 1.0000e+00 |
| 2 | 10 | 0 | 100 | 0 | 0 | 9.5173e-01 |
| 2.5 | 10 | 0 | 100 | 0 | 0 | 6.2267e-02 |
| 3 | 11 | 0 | 100 | 0 | 7.1204e-04 | 1.0000e+00 |
| 3.5 | 11 | 0 | 100 | 0 | 8.8594e-06 | 9.9960e-01 |
| 4 | 11 | 0 | 100 | 0 | 0 | 8.2360e-01 |
| 4.5 | 11 | 0 | 100 | 0 | 0 | 7.7333e-03 |
| 5 | 12 | 0 | 100 | 0 | 1.1913e-04 | 1.0000e+00 |
| 5.5 | 12 | 0 | 100 | 0 | 0 | 9.8000e-01 |
| 6 | 12 | 0 | 100 | 0 | 0 | 6.1867e-02 |
| 6.5 | 13 | 0 | 100 | 0 | 8.1569e-04 | 1.0000e+00 |
| 7 | 13 | 0 | 100 | 0 | 0 | 1.0000e+00 |
| 7.5 | 13 | 0 | 100 | 0 | 0 | 9.9053e-01 |
| 8 | 13 | 0 | 100 | 0 | 0 | 8.9600e-02 |
| 8.5 | 14 | 6.9333e-03 | 100 | 0 | 6.9333e-03 | 1.0000e+00 |
| 9 | 14 | 5.1314e-05 | 100 | 0 | 4.4297e-05 | 1.0000e+00 |
| 9.5 | 14 | 0 | 100 | 0 | 0 | 9.3747e-01 |
| 10 | 14 | 0 | 100 | 0 | 0 | 1.5013e-01 |
| 10.5 | 15 | 0 | 100 | 0 | 1.1204e-03 | 1.1204e-03 |
| 11 | 15 | 0 | 100 | 0 | 8.8594e-06 | 8.8594e-06 |
| 11.5 | 15 | 0 | 100 | 0 | 0 | 0 |
| 12 | 15 | 0 | 100 | 0 | 0 | 0 |
| 12.5 | 15 | 0 | 100 | 0 | 0 | 0 |
| 13 | 15 | 0 | 100 | 0 | 0 | 0 |
| 13.5 | 15 | 0 | 100 | 0 | 0 | 0 |
| 14 | 15 | 0 | 100 | 0 | 0 | 0 |
| 14.5 | 15 | 0 | 100 | 0 | 0 | 0 |
| 15 | 15 | 0 | 100 | 0 | 0 | 0 |

Table 4 CQI reporting results for TDD with 1Tx2Rx antenna configuration

| SNR [dB] | Median CQI Index | BLER with following CQI | Percentage of Reported CQI Index in {median CQI -1, median CQ1 +1} (%) | BLER with median CQI-1 | BLER with median CQI | BLER with median CQI+1 |
| --- | --- | --- | --- | --- | --- | --- |
| -12 | 0 | 1.9111e-01 | 100 | 1.9111e-01 | 1.9111e-01 | 1.9111e-01 |
| -11.5 | 0 | 1.4617e-01 | 100 | 1.4617e-01 | 1.4617e-01 | 1.4617e-01 |
| -11 | 0 | 8.8153e-02 | 100 | 8.8153e-02 | 8.8153e-02 | 8.8153e-02 |
| -10.5 | 1 | 2.3345e-02 | 100 | 2.3345e-02 | 2.3345e-02 | 1.9495e-01 |
| -10 | 1 | 7.5418e-04 | 100 | 7.5418e-04 | 7.5418e-04 | 1.4599e-01 |
| -9.5 | 1 | 2.4694e-06 | 100 | 0 | 0 | 8.5017e-02 |
| -9 | 2 | 1.4808e-02 | 100 | 0 | 1.4808e-02 | 1.7073e-01 |
| -8.5 | 2 | 1.3600e-04 | 100 | 0 | 1.4565e-04 | 1.1620e-01 |
| -8 | 2 | 0 | 100 | 0 | 0 | 3.5192e-02 |
| -7.5 | 3 | 0 | 100 | 0 | 5.4406e-04 | 1.7787e-01 |
| -7 | 3 | 0 | 100 | 0 | 0 | 1.0697e-01 |
| -6.5 | 4 | 1.8641e-02 | 100 | 0 | 1.8641e-02 | 7.6045e-01 |
| -6 | 4 | 2.2225e-05 | 100 | 0 | 1.1576e-05 | 2.1098e-01 |
| -5.5 | 4 | 0 | 100 | 0 | 0 | 1.2056e-01 |
| -5 | 5 | 2.0209e-02 | 100 | 0 | 2.0209e-02 | 1.0000e+00 |
| -4.5 | 5 | 1.4816e-05 | 100 | 0 | 2.2407e-05 | 9.9007e-01 |
| -4 | 5 | 0 | 100 | 0 | 0 | 2.5679e-01 |
| -3.5 | 5 | 0 | 100 | 0 | 0 | 1.2178e-01 |
| -3 | 6 | 5.7323e-03 | 100 | 0 | 5.7323e-03 | 1.0000e+00 |
| -2.5 | 6 | 0 | 100 | 0 | 0 | 9.9721e-01 |
| -2 | 6 | 0 | 100 | 0 | 0 | 2.1620e-01 |
| -1.5 | 6 | 0 | 100 | 0 | 0 | 5.3833e-02 |
| -1 | 7 | 0 | 100 | 0 | 1.0418e-04 | 1.0000e+00 |
| -0.5 | 7 | 0 | 100 | 0 | 0 | 3.6812e-01 |
| 0 | 7 | 0 | 100 | 0 | 0 | 7.3345e-02 |
| 0.5 | 8 | 0 | 100 | 0 | 5.1537e-04 | 1.0000e+00 |
| 1 | 8 | 0 | 100 | 0 | 0 | 9.9895e-01 |
| 1.5 | 8 | 0 | 100 | 0 | 0 | 2.4303e-01 |
| 2 | 8 | 0 | 100 | 0 | 0 | 1.7944e-02 |
| 2.5 | 9 | 0 | 100 | 0 | 2.3152e-05 | 1.0000e+00 |
| 3 | 9 | 0 | 100 | 0 | 0 | 9.4669e-01 |
| 3.5 | 9 | 0 | 100 | 0 | 0 | 1.6341e-01 |
| 4 | 10 | 4.8244e-03 | 100 | 0 | 4.8244e-03 | 1.0000e+00 |
| 4.5 | 10 | 4.9740e-06 | 100 | 0 | 2.2407e-05 | 1.0000e+00 |
| 5 | 10 | 0 | 100 | 0 | 0 | 9.6446e-01 |
| 5.5 | 10 | 0 | 100 | 0 | 0 | 1.5331e-01 |
| 6 | 11 | 0 | 100 | 0 | 4.0650e-03 | 1.0000e+00 |
| 6.5 | 11 | 0 | 100 | 0 | 0 | 9.9930e-01 |
| 7 | 11 | 0 | 100 | 0 | 0 | 2.0453e-01 |
| 7.5 | 11 | 0 | 100 | 0 | 0 | 2.5249e-03 |
| 8 | 12 | 0 | 100 | 0 | 1.1204e-05 | 1.0000e+00 |
| 8.5 | 12 | 0 | 100 | 0 | 0 | 9.9512e-01 |
| 9 | 12 | 0 | 100 | 0 | 0 | 1.8571e-01 |
| 9.5 | 13 | 6.8442e-03 | 100 | 0 | 6.8442e-03 | 1.0000e+00 |
| 10 | 13 | 1.7409e-05 | 100 | 0 | 3.3611e-05 | 9.9948e-01 |
| 10.5 | 13 | 0 | 100 | 0 | 0 | 7.9686e-01 |
| 11 | 13 | 0 | 100 | 0 | 0 | 7.4913e-02 |
| 11.5 | 14 | 0 | 100 | 0 | 1.1204e-05 | 1.0000e+00 |
| 12 | 14 | 0 | 100 | 0 | 0 | 1.0000e+00 |
| 12.5 | 14 | 0 | 100 | 0 | 0 | 9.7247e-01 |
| 13 | 14 | 0 | 100 | 0 | 0 | 1.4077e-01 |
| 13.5 | 15 | 0 | 100 | 0 | 4.0333e-04 | 4.0333e-04 |
| 14 | 15 | 0 | 100 | 0 | 0 | 0 |
| 14.5 | 15 | 0 | 100 | 0 | 0 | 0 |
| 15 | 15 | 0 | 100 | 0 | 0 | 0 |

Table 5 CQI reporting results for TDD with 1Tx4Rx antenna configuration

| SNR [dB] | Median CQI Index | BLER with following CQI | Percentage of Reported CQI Index in {median CQI -1, median CQ1 +1} (%) | BLER with median CQI-1 | BLER with median CQI | BLER with median CQI+1 |
| --- | --- | --- | --- | --- | --- | --- |
| -12 | 2 | 1.3240e-02 | 100 | 4.3694e-04 | 1.3240e-02 | 2.5784e-01 |
| -11.5 | 2 | 1.0801e-02 | 100 | 1.1204e-05 | 1.0801e-02 | 1.9338e-02 |
| -11 | 2 | 2.6396e-03 | 100 | 0 | 2.6396e-03 | 1.4983e-02 |
| -10.5 | 3 | 1.0627e-02 | 100 | 1.2733e-04 | 1.0627e-02 | 2.4321e-01 |
| -10 | 3 | 3.6210e-03 | 100 | 0 | 3.6210e-03 | 1.4808e-02 |
| -9.5 | 4 | 1.0976e-02 | 100 | 2.1994e-04 | 1.0976e-02 | 1.0000e+00 |
| -9 | 4 | 7.6139e-03 | 100 | 0 | 7.6139e-03 | 7.0627e-01 |
| -8.5 | 4 | 1.5550e-03 | 100 | 0 | 1.5550e-03 | 2.1603e-02 |
| -8 | 5 | 1.1150e-02 | 100 | 0 | 1.1150e-02 | 1.0000e+00 |
| -7.5 | 5 | 6.2718e-03 | 100 | 0 | 6.2718e-03 | 1.0000e+00 |
| -7 | 5 | 7.7939e-04 | 100 | 0 | 7.7939e-04 | 8.8240e-01 |
| -6.5 | 5 | 0 | 100 | 0 | 0 | 2.6132e-02 |
| -6 | 6 | 1.2195e-02 | 100 | 0 | 1.2195e-02 | 1.0000e+00 |
| -5.5 | 6 | 3.1839e-03 | 100 | 0 | 3.1839e-03 | 1.0000e+00 |
| -5 | 6 | 3.2331e-05 | 100 | 0 | 2.2407e-05 | 7.1707e-01 |
| -4.5 | 6 | 0 | 100 | 0 | 0 | 1.9338e-02 |
| -4 | 7 | 8.8850e-03 | 100 | 0 | 8.8850e-03 | 1.0000e+00 |
| -3.5 | 7 | 1.5404e-04 | 100 | 0 | 1.6206e-04 | 9.8049e-01 |
| -3 | 7 | 0 | 100 | 0 | 0 | 3.6585e-02 |
| -2.5 | 8 | 7.1027e-03 | 100 | 0 | 7.1027e-03 | 1.0000e+00 |
| -2 | 8 | 6.9637e-05 | 100 | 0 | 1.1204e-04 | 1.0000e+00 |
| -1.5 | 8 | 0 | 100 | 0 | 0 | 5.0697e-01 |
| -1 | 8 | 0 | 100 | 0 | 0 | 1.4634e-02 |
| -0.5 | 9 | 0 | 100 | 0 | 1.4774e-03 | 1.0000e+00 |
| 0 | 9 | 0 | 100 | 0 | 0 | 9.9983e-01 |
| 0.5 | 9 | 0 | 100 | 0 | 0 | 1.4111e-01 |
| 1 | 10 | 9.4077e-03 | 100 | 0 | 9.4077e-03 | 1.0000e+00 |
| 1.5 | 10 | 1.8514e-04 | 100 | 0 | 1.2733e-04 | 1.0000e+00 |
| 2 | 10 | 0 | 100 | 0 | 0 | 1.0000e+00 |
| 2.5 | 10 | 0 | 100 | 0 | 0 | 9.3206e-02 |
| 3 | 11 | 5.5960e-03 | 100 | 0 | 5.5960e-03 | 1.0000e+00 |
| 3.5 | 11 | 1.0943e-04 | 100 | 0 | 1.2324e-04 | 1.0000e+00 |
| 4 | 11 | 0 | 100 | 0 | 0 | 3.9390e-01 |
| 4.5 | 11 | 0 | 100 | 0 | 0 | 8.8850e-03 |
| 5 | 12 | 0 | 100 | 0 | 4.6303e-05 | 1.0000e+00 |
| 5.5 | 12 | 0 | 100 | 0 | 0 | 1.0000e+00 |
| 6 | 12 | 0 | 100 | 0 | 0 | 1.4286e-01 |
| 6.5 | 13 | 0 | 100 | 0 | 7.3420e-03 | 1.0000e+00 |
| 7 | 13 | 0 | 100 | 0 | 1.3444e-04 | 1.0000e+00 |
| 7.5 | 13 | 0 | 100 | 0 | 0 | 9.5801e-01 |
| 8 | 13 | 0 | 100 | 0 | 0 | 1.9338e-02 |
| 8.5 | 14 | 0 | 100 | 0 | 4.2574e-04 | 1.0000e+00 |
| 9 | 14 | 0 | 100 | 0 | 0 | 1.0000e+00 |
| 9.5 | 14 | 0 | 100 | 0 | 0 | 9.9408e-01 |
| 10 | 14 | 0 | 100 | 0 | 0 | 1.0819e-01 |
| 10.5 | 15 | 1.8133e-03 | 100 | 0 | 1.8133e-03 | 1.8133e-03 |
| 11 | 15 | 0 | 100 | 0 | 0 | 0 |
| 11.5 | 15 | 0 | 100 | 0 | 0 | 0 |
| 12 | 15 | 0 | 100 | 0 | 0 | 0 |
| 12.5 | 15 | 0 | 100 | 0 | 0 | 0 |
| 13 | 15 | 0 | 100 | 0 | 0 | 0 |
| 13.5 | 15 | 0 | 100 | 0 | 0 | 0 |
| 14 | 15 | 0 | 100 | 0 | 0 | 0 |
| 14.5 | 15 | 0 | 100 | 0 | 0 | 0 |
| 15 | 15 | 0 | 100 | 0 | 0 | 0 |

# Conclusion

The paper presents simulation results for URLLC UE CQI reporting tests for CQI table 3.

# References

- R4- 2017674, “WF on ultra-low BLER requirements”, Ericsson