3GPP TSG-RAN WG4 Meeting #98-e	R4-2102170

# Electronic meeting, January 25th – February 5th, 2021

Source:		Ericsson

Title:			New band n67 – UE RF impacts

Agenda item:		9.28.1

Document for:		Discussion

# Introduction

In last RAN#90-e, a new WI has been approved to specify the new NR band n67, re-farmed from LTE band 67.

This contribution is discussing the core UE RF impacts when introducing this band.

# Discussion

## Presentation

Band n67 is a SDL band. It corresponds to the DL part of band n28.

It shall support following channel bandwidths:

- 5 MHz channel bandwidth and for 15 kHz subcarrier spacing.

- 10, 15 and 20 MHz channel bandwidth and for 15 kHz and 30 kHz subcarrier spacing.

## General

### Band definition

The band n67 shall be added to list of NR operating bands in FR1, table 5.2-1 as below:

| NR operating band | Uplink (UL) operating band BS receive / UE transmit FUL,low – FUL,high | Downlink (DL) operating band BS transmit / UE receive FDL,low – FDL,high | Duplex mode |
| --- | --- | --- | --- |
| n67 | N/A | 758 MHz – 803 MHz | SDL |

### BS channel bandwidth

Following channel BWs shall be added to table 5.3.5-1:

| NR band / SCS / BS channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR Band | SCS kHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
|  | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
| n67 | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |

### Channel arrangement

NR-ARFCN for n67 shall be added in table 5.4.2.3-1:

| NR operating band | ΔFRaster (kHz) | Uplink range of NREF (First – <Step size> – Last) | Downlink range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n67 | 100 | N/A | 151600 – <20> – 160600 |

And sync raster information shall be added to table 5.4.3.3-1:

| NR operating band | SS Block SCS | SS Block pattern (NOTE 1) | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n67 | 15 kHz | Case A | 1901 – <1> – 2002 |

## Tx requirements

As n67 is SDL band, there is no Tx requirement applicable for that band. And as E-UTRA band 67 was specified, this band was already considered in the coexistence spurious table 6.5.3.2-1.

No update is expected in the Tx requirements when introducing band n67.

## Rx requirements

### REFSENS

REFSENS limits for band n67 shall be added in table 7.3.2-1:

| Operating band / SCS / Channel bandwidth / Duplex-mode |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 40 MHz (dBm) | 50 MHz (dBm) | 60 MHz (dBm) | 70 MHz (dBm) | 80 MHz (dBm) | 90 MHz (dBm) | 100 MHz (dBm) | Duplex Mode |
| n67 | 15 | -98.5 | -95.5 | -93.5 | -90.8 |  |  |  |  |  |  |  |  |  | SDL |
|  | 30 |  | -95.6 | -93.6 | -91.0 |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

And associated RB allocation table 7.3.2-3:

| Operating band / SCS / Channel bandwidth / Duplex mode |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Duplex Mode |
| n67 | 15 | 25 | 251 | 251 | 251 |  |  |  |  |  |  |  |  |  | FDD |
|  | 30 |  | 101 | 101 | 101 |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

# Conclusion

In this contribution, we analyzed the UE RF impacts when introducing band n67.

Proposal: Agree with the listed changes in this contribution.

# References

- RP-202279, New WID on introduction of n67, Ericsson, Telefonica