3GPP TSG-RAN WG4 Meeting # 98-e	R4-2102288

# Electronic Meeting, , Jan. 25-Feb. 5, 2021

Source: 	Skyworks Solutions, Inc.

Title: 	Network and UE options for the support of irregular channel bandwidth

Agenda Item:	12.3.4.1 UE perspective [FS_NR_eff_BW_util]

Document for:	Approval

# Introduction

In the RAN#90 e-meeting, an update of the study [1] was approved to analyze the potential solution for supporting irregular channel bandwidths with 6MHz for n12 added. Further from [2], this contribution discusses the different cases from a Network and UE prospective and provides an analysis of potential solutions and their related constraints.

# Discussion

The updated Study Item [1] for irregular channel bandwidths (not a multiple of 5 MHz) lists the cases to be studied as noted in Table 1. Although 6 MHz channel bandwidth was added to band n12, since it was already discussed for band n29, it does not change the study in [2]. Form the last RAN4 meeting, a way forward was discussed but not agreed [3].

Table 1: Irregular channel bandwidths and associated bands for study

| Band (s) | Channel Bandwidth(s) |
| --- | --- |
| n5 | 7, 11, 12.5 MHz |
| n12 | 6, 12 MHz |
| n26 | 7 MHz |
| n28 | 13, 33 MHz |
| n29 | 6, 11 MHz |

In the objective section, two main options should be studied:

- Using the immediately higher existing channel bandwidth with reduced allocation

- Using overlapped immediately lower existing channel bandwidths to fully use the spectrum

- Overlapping may exist from:

  - Network prospective only: UE operates with a single existing channel but different UEs have different channel position that overlaps

  - UE prospective: UE operates with a two existing channel at different channel position that overlaps

  - BS may operate with the full bandwidth or in overlapping mode

## High Level Analysis of the Cases

One critical aspect of the solution in terms of achievable SU, is about the channel raster and SSB raster:

- Since all bands are legacy bands, they all use 100 kHz raster as can be seen from Table 5.4.2.3-1 in 38.101-1

- Except band n5 which uses both 15 kHz and 30 kHz SSB all the bands are based on 15 kHz SSB as can be seen from Table 5.4.3.3-1 in 38.101-1

Since SSB is 20 RB and 1.2MHz raster:

- 15 kHz SSB is 3.6 MHz and only one can fit in 6 or 7 MHz

- 30 kHz SSB is 7.2 MHz and cannot fit into 7 MHz bandwidth thus 30 kHz SSB is not applicable to band n5 at least for 7 MHz bandwidth

In Table 2 we summarize the current specification situation and potential limitations for the different cases.

Table 2: High level analysis of irregular channel bandwidths cases

| Band | ChBW [MHz] | Related NS | Bandwidth aspects |
| --- | --- | --- | --- |
| n5 | 7, 11, 12.5 | NS_100 | 30 kHz SSB not possible and only one 15kHz SSB for 7 MHz |
| n12 | 6, 12 | NS_06 | Only one 15kHz SSB for 6 MHz, Current maximum BW is 10MHz but DL only |
| n26 | 7 | NS_12/13/14/15, NS_100 | Only one 15kHz SSB for 7 MHz |
| n28 | 13, 33 | NS_17/18 | Current maximum BW is 30 MHz with limitation on positions as needed for dual duplexer implementation. 30 MHz has already 0.5 dB MPR as it is above the 3% fractional BW requirement |
| n29 | 6, 11 | N-A | Only one 15kHz SSB for 6 MHz, Current maximum BW is 10MHz but DL only |

Observation 1:

- All are low band so 60 KHz SCS is not needed

- All cases use 100kHz raster

- There are limitations in terms of SSB number, size and placement for 6 and 7 MHz bandwidths

- There are cases where the requested bandwidth is larger than the current largest specified BW, thus emissions can’t be guaranteed and there could be limitations on RF BW in UL.

- All FDD bands have specific regulatory requirements.

The use of the existing immediately lower UE channel bandwidth in overlapping from networks only should be the baseline operation for the UE and as proposed below, the only mode of operation in UL such that emissions can be guaranteed without additional test and no additional specification work. In the case where a reduced allocation of the immediately higher BW, or overlapping from UE point of view is used in DL, the asymmetric channel bandwidths operation could apply.

Proposal 1:

- 30 kHz SSB is not applicable to Band n5 irregular channel bandwidth (at least for 7 MHz)

- Only 30 MHz UE bandwidth is considered for 33 MHz in Band n28 with current position limitations (note 7 in 38.101-1 Table 5.3.5-1)

- For Band 29, current maximum BW is 10 MHz but since it is a DL only band and thus no regulatory emissions apply, support of 11 MHz can be studied

- To enable legacy UEs use of immediately lower channel bandwidth with overlap form Network only should be the baseline operation

- Use of the immediately higher bandwidth is not supported in UL to avoid specifying and testing NS related emissions, if needed asymmetric UL/DL UE operation can be used.

## Overlap from Network Only for UE

To clarify this mode of operation it must be clear that the separate channels cannot be operated by a single UE, but rather must be scheduled to separate UE groups as described in Figure 1 for the 7MHz example.

Figure 1: Description of UE and BS operation when using immediately lower existing channel bandwidth on UE side and overlapping from a network point of view (7 MHz example)

Figure 1 is valid for both UL and DL and describe a possible 7 MHz scenario for 15 kHz SCS, the UEs are scheduled in two groups (yellow and red at the bottom) with 5 MHz channel BW on each edge of the desired WB. At the top, the BS uses all RBs covering the two 5MHz channels, the overlap region (orange) can be dynamically scheduled to either UE group one or two. The non-overlapping region (yellow and red) can only be scheduled to one group.

Some of the constraints are also illustrated, like the fact that the BS guard band should be larger or equal to the UE guards band or the fact that the UE carriers should fall on the 100 kHz raster.

If the existing immediately lower UE channel bandwidth can be used in an overlapped manner to match the desired bandwidth, it must fulfill a number of constraints:

- To automatically fulfill emissions, the BS/Network BW guard bands should be higher or equal to the UE BW guard band

- The two channels should overlap in an FFT aligned manner for the lowest SCS

- Since all the proposed band are on a 100 kHz raster, each channel should be separated by a multiple of 100 kHz

- In the case where the BS SU cannot accommodate separate non-overlapping SSBs, the SSBs should be staggered in time or common

- In order to use a common SSB for the two UE channels:

  - The overlap should be at least 20 RB (3.6MHz for 15kHz SSB, 7.2MHz for 30kHz SSB)

  - The SSB should fit on the SSB raster grid (1.2MHz)

  - The distance between the two UE carriers should be a multiple of RBs

- For some cases where the two SSBs could be separate and with enough distance, it might be possible to operate the BS with fractional RBs to remove some constraints but at the expense of an in-band guard band. Since this most likely is not the generic case, the study of this option is not a priority

- If SSB cannot overlap partially, they either need to be the same for each UE CC, which requires at least 20 RB overlap and falling on 1.2MHz raster, or they need to be separate and UE CCs must fall on 1.2MHz raster and multiple of RB.

To provide some view on these constraints, Table 3 provides the least common multiplier (LCM) for some of the cases. To fulfill the constraint, the distance between the two UE carriers must be a multiple of that number.

Table 3: LCM value for alignment constraints

| alignment | LCM |
| --- | --- |
| 15 kHz SCS FFT and 100 kHz raster | 300 kHz |
| 30 kHz SCS FFT and 100 kHz raster | 300 kHz |
| 15 kHz RB and 100kHz raster | 900 kHz |
| 30 kHz RB and 100kHz raster | 1.8MHz |
| 30 kHz RB and 1.2 MHz SSB raster | 3.6 MHz |

Note that solutions based on 100 kHz raster and 30 kHz RB ensures that BS carrier is on 100 kHz raster. Furthermore, solutions based on SSB raster automatically support the 100 kHz raster and 30 kHz RB conditions.

As a reference, the optimum SU, based on same interpolation method used for 35 MHz and 45 MHz channel bandwidth cases is detailed in Table 4, provides:

- The SU in RBs for 15 and 30 kHz (except for 6 and 7 MHz where only 15 kHz can apply)

- The related SU in %

- The related positions of the UE carriers using the immediately lower existing channel bandwidth

- The resulting smallest guard band

Table 4: SU based on interpolation with resulting CC position and guard bands for the UE channels

Observations:

- 90% or above SU is achieved for all 15kHz cases

- In most cases, except for 7 MHz at 15 kHz the UE CCs and 12 MHz at 30 kHz, the UE CCs are not on the 100 kHz raster

- In some cases (yellow highlight), the resulting BS guard band is smaller than the immediately higher existing channel BW

- In other cases (red highlight), the resulting BS guard band is smaller than the immediately lower existing channel BW and thus the default UE operation cannot guarantee emissions.

The different solutions for each bandwidth using the 900 kHz or 1.8 MHz constraint related to 100 kHz channel raster and RB multiple have been studied and can be found in Table 5 where the same parameters for Table 4 are given, additionally the RB overlap between carriers (OV) is calculated on top of SU and both upper and lower GB are calculated.

Table 5: SU based on raster and RB constraints with resulting CC position and guard bands for the UE channels

Observations:

- For 6 MHz, SSB position needs more study:

  - 30 kHz RB solution results in same SU than 5MHz => not useful

  - 15 kHz RB solution:

    - Has exactly 20 RB overlap => need to fall exactly on 1.2MHz raster

    - Needs a 50 kHz GB shift to meet 100 kHz raster and results in upper GB too low to meet 10 MHz GB.

    - Meets 90% SU for 15 kHz

- For 7 MHz, need to find a solution for partially overlapping SSB in time:

  - 30 kHz RB solution works for both 15 and 30 kHz and is the same, but RB overlap is 15:

    - Common and non-overlapping 15 KHz SSB not feasible

    - Need to study the possibility of overlapping SSB in time and meeting 1.2MHz raster

    - > 90% SU for 15 kHz

- For 11 MHz, only 15 kHz SCS solution possible with some additional constraints:

  - 30 kHz SCS solution results in same SU than 10MHz => not useful

  - 15 kHz SCS solution:

    - Cannot work for 30kHz (half RB)

    - Needs a 50 kHz GB shift to meet 100 kHz raster and results in upper GB too low for to meet 10 MHz GB

    - > 90% SU for 15 kHz

- For 12 MHz, good overall 30 kHz solution:

  - 30 kHz SCS solution works for both 15 and 30 kHz and is close to the optimum interpolated result

  - With 19 30 kHz RB overlap 30kHz SSB does not work but can use a common 15 kHz SSB as the overlap is more than 1.2MHz larger than 20 RB.

  - > 90% SU for 15 kHz

- For 12.5 MHz, no better solution than 12 MHz => use 12 MHz:

  - 30 kHz SCS solution works for both 15 and 30 kHz and is the same than 12MHz

  - < 90% SU for 15 kHz

- For 13 MHz, two possible solutions depending on 30 kHz RB needed or not:

  - If 30 kHz SCS needed then solution is the same than 12 MHz => use 12 MHz

    - < 90% SU for 15 kHz

  - If 30 kHz SCS solution is not needed then:

    - SU is very close to optimum based on interpolation and > 90%

- For 33 MHz, two possible solutions depending on whether 30 kHz RB is needed or not:

  - If 30 kHz SCS solution is needed, valid for 15 and 30 kHz, has 18 RB less than optimum but still achieves > 90% SU for 15 and 30 kHz SCS

  - If 30 kHz SCS solution is needed, the 15kHz solution achieve a better SU but with only 5 more RBs

  - Note that 33MHz is not achievable for n28 anyhow due to duplexer restrictions

Given this first order analysis, solutions that can achieve >90% SU for 15 kHz and meet all constraints should be favored, thus the following proposal.

Proposal 2: The solutions in following Table are adopted for further study:

| Target BW | SCS | UE CH BW / RB / SU% | BS RB / SU % | SSB constraint | other |
| --- | --- | --- | --- | --- | --- |
| 6 MHz | 15 kHz only | 5 / 25 / 75% | 30 / 90% | 15 kHz common in exact 20RB overlap Need to be on SSB raster point | 50kHz GB shift |
| 7 MHz | 15 kHz and 30 kHz | 5 / 25 / 64% | 35 / 90% | 15kHz Partial overlap only, need to be staggered in time and SSB raster point | Lost BW due to SSB resources |
| 11 MHz | 15 kHz only | 10 / 52 / 85% | 57 / 93% | Common 15 kHz SSB | 50kHz GB shift |
| 12 MHz | 15 kHz and 30 kHz | 10 / 52 / 78% 10 / 24 / 72% | 62 / 93% 29 / 87% | Common 15 kHz SSB | none |
| 12.5 MHz | 15 kHz and 30 kHz | 10 / 52 / 75% 10 / 24 / 69% | 62 / 89% 29 / 84% | Use 12 MHz solution |  |
| 13 MHz | 15 kHz and 30 kHz | 10 / 52 / 72% 10 / 24 / 67% | 62 / 86% 29 / 80% | Use 12 MHz solution |  |
|  | 15 kHz only | 10 / 52 / 72% | 67 / 93% | Common 15 kHz SSB | 50kHz GB shift |
| 33 MHz | 15 kHz and 30 kHz | 30 / 160 / 87% 30 / 78 / 85% | 170 / 93% 83 / 91% | Common 15 kHz SSB | Not for n28 |
|  | 15 kHz only | 30 / 160 / 87% | 175 / 96% | Common 15 kHz SSB | 50kHz GB shift |

As described in the above proposal, there are further aspects to be studied especially for SSB design for 6 and 7 MHz cases. The 6 MHz case has a possibility for common SSB and not suffer from additional unused SSB resources for partially overlapped SSB staggered in time that is required for the 7 MHz case, provided an adequate SSB raster point can be found. Depending on the cases, SSB raster points are distant by Nx1.2 +/- 0.1 MHz so a detailed study of the BS and UE channels placement is needed within the band to know if there are solutions for the SSB placement.

The 6 MHz 15 kHz SCS and the 7 MHz 15 and 30 kHz SCS cases are illustrated in Figure 2:

- At the top of the figure, the different frequency constraints with 100 kHz carrier raster, 1.2 MHz SSB raster, LCM(15,100) kHz, LCM(30,100) kHz shifts are provided

- UE1, UE2 and BS RB design and related carrier position are illustrated:

  - 6 MHz with 15 kHz SCS RBs and common SSB RBs (orange)

  - 7 MHz with 15 kHz and 30 kHz SCS RBs with overlapped SSBs (yellow and red)

Figure 2: 6 and 7MHz UE and BS RB design

For 6 MHz, the common SSB (illustrated) is in the exact 20RB overlap and thus the center of the SSB should be aligned with the center of the BS RBs while the UE carriers need to be on the 100 kHz raster. In case there is no solution for this, two overlapping SSBs staggered in time like for 7 MHz should be studied and the overlap is 10 to 19 RBs

For 7 MHz, separate SSB are needed as the maximum overlap is 15 RB and thus SSBs need to be staggered in time, the minimum SSB overlap is 5RB (illustrated). This offers a few solutions to find the two SSB raster points but the distance needs to be a multiple of 180 kHz

Observation: The SSB design for 7 MHz and 6 MHz bandwidths needs further study with possible SSB raster points in the actual band and its position within the band to know if the solution is feasible.

## Use of Immediately Higher Existing UE Channel Bandwidth

As discussed in the above chapters, for UL we propose to only support the existing immediately lower channel bandwidth and in DL it should also be the default assumption. For a solution using a reduced allocation of the immediately higher existing EU channel bandwidth in UL, AMPR study would be required, especially for cases that are allocation based. Since all the FDD bands have an NS requirement, this is beyond the scope and effort agreeable to enable irregular channel bandwidth support and since the main goal is to enable the best spectrum usage and not the UL throughput, this should be acceptable and reflected in our proposal 2.

Optionally, the UE could use the existing immediately higher UE channel bandwidth with a reduced number of RB in DL, but still has to meet a number of constraints:

- Will need to be compatible with the design for the default UL/DL operation using existing immediately lower UE channel BW and reuse the SSB design

- Use the asymmetric UL/DL operation

- Carrier position needs to be on the 100kHz raster

- Since this is for DL only the BS guard band does not need to be larger than the guard band of the UE immediately higher channel BW

- The ACS and blocking performance should assume the full bandwidth and REFSENS is implicitly scaled with the number of RBs:

  - If adjacent channels are collocated this should not be an issue

  - If adjacent channels are not collocated and the existing immediately higher UE channel is much wider than the desired channel (it can only be 2 MHz wider on each side) near far effect may be an issue.

Proposal 3:

- Solution using the immediately higher existing UE channel bandwidth can be further studied for DL only:

  - It should be an optional UE capability

  - It should be compatible with the default UL/DL operation using immediately lower existing UE channel BW with overlap from network point of view

- No change to the EU specification should be assumed and ACS/blocking/REFSENS characteristics are those of the related existing UE channel bandwidth: It may not be feasible for all channel bandwidths and deployments

## Overlap from UE point of view

Overlap from the UE point of view uses a similar concept that described in Figure 1 but where one UE behaves like UE1+UE2. As such, all the same constrains apply and it should also use the same solutions so that both types of UEs can coexist.

From a hardware support point of view, it means that the UE must have 4 BB paths in RX and 2 BB paths in TX to support 2x2 DL MIMO and UL SISO with some means of multiplexing the two at BB or RF. Only in this case, the BB filtering will ensure emissions and blocking performance in UL and DL respectively.

It should be noted that all the bands discussed for irregular bandwidths are low bands where LB-LB inter band CA or DC are usually not implemented and 4x4 DL MIMO or 2x2 UL MIMO are not specified (nor really implementable from an antenna point of view). Thus, current UEs architecture do not support overlap from UE point of view for requested bands and this would induce extra cost for a very small BW increase.

We can anyhow discuss the DL and UL implementations separately.

For UL, there are multiple points at which recombination could occur:

- At BB: it is the easiest, the most precise and should not result into duplicated impairments but it is somewhat higher complexity to have two paths rather than implementing the BW itself.

- At TRX: this would require two Los and then impairments are doubled (2 LO, 2 Images) which makes emission unpredictable. Combination can be made rather precise (balanced)

- At RF: it is hard to recombine with precision, impairments are doubled and in low band, two PAs and/or two good antennas are questionable for a very small benefit in UL

Given that these solutions are complex or cannot reuse existing specification and would not prevent from having a specific test to guarantee emissions, overlap from UE prospective should not be supported in UL.

For DL, if the de-multiplexing happen at baseband it can be relatively straight forward if the transceiver uses the immediately higher channel bandwidth but depending on analog channel filter, AGC, ADC anti-aliasing may still have some degradation in blocking and ACS. Split at RF is an option but is not implemented in low bands in either the RF front-end or the transceiver.

Given that the performance may not be significantly better than using a bandwidth part of the immediately higher channel bandwidth and it requires additional hardware (at least for low bands requested here) we do not see that this approach should be prioritized. Especially, if the desired bandwidth is close (1-2MHz) away from the next higher channel bandwidth, its performance should be already acceptable.

Observation on overlap from UE point of view:

- This is not practical in UL due to the additional cost and little benefit. It does not prevent a specific emission test and MPR/A-MPR specification due to duplicated impairments in some cases.

- In DL, although there are less effort to enable, there is no guarantee that blocking and ACS are not degraded and it may not show significant benefits versus using the higher channel bandwidth at the expense of additional hardware

Proposal 4:

- Overlap from UE point of view is not supported in UL

- Unless it can be demonstrated that better DL performance is obtained versus using a BW part of the immediately higher BW as optional UE support, overlap from UE point of view should not be the priority to study

## How to Manage Irregular Bandwidth Cases in RAN4

As shown by the cumbersome analysis above, the use of irregular channel BW requires a detailed study of the constraints and in some cases, the exact band and channel position needs to be studied in order to find a suitable SSB design. It is therefore not possible to apply these irregular channel bandwidths blindly to other bands, nor is it desirable to extend the number of BW to be studied indefinitely. Finally, looking at the results related to the channel raster, bandwidths that are not an integer multiple of 1 MHz have little-to-no benefit compared to the cases that are an integer multiple of 1 MHz.

Proposal 5:

- Irregular channel bandwidths should be an integer multiple of 1 MHz

- Applying irregular channel bandwidths agreed in this SI to new bands should be done with agreement at plenary

- Adding new irregular channel bandwidths in this SI to existing or new bands should be done with agreement at plenary

# Conclusions

In this contribution, we provide an analysis of the requested irregular channel bandwidths with the current specification and RF characteristics of the related bands, the analysis of options that could be supported by the UE and their design constraints and based on the RF characteristics, bandwidth constraints and regulation requirements we make the following proposal.

Proposal 1:

- 30 kHz SSB is not applicable to Band n5 irregular channel bandwidth (at least for 7 MHz)

- Only 30 MHz UE bandwidth is considered for 33 MHz in Band n28 with current position limitations (note 7 in 38.101-1 Table 5.3.5-1)

- For Band 29, current maximum BW is 10 MHz but since it is a DL only band and thus no regulatory emissions apply, support of 11 MHz can be studied

- To enable legacy UEs use of immediately lower channel bandwidth with overlap form Network only should be the baseline operation

- Use of the immediately higher bandwidth is not supported in UL to avoid specifying and testing NS related emissions, if needed asymmetric UL/DL UE operation can be used.

Proposal 2: The solutions in following Table are adopted for further study:

| Target BW | SCS | UE CH BW / RB / SU% | BS RB / SU % | SSB constraint | other |
| --- | --- | --- | --- | --- | --- |
| 6 MHz | 15 kHz only | 5 / 25 / 75% | 30 / 90% | 15 kHz common in exact 20RB overlap Need to be on SSB raster point | 50kHz GB shift |
| 7 MHz | 15 kHz and 30 kHz | 5 / 25 / 64% | 35 / 90% | 15kHz Partial overlap only, need to be staggered in time and SSB raster point | Lost BW due to SSB resources |
| 11 MHz | 15 kHz only | 10 / 52 / 85% | 57 / 93% | Common 15 kHz SSB | 50kHz GB shift |
| 12 MHz | 15 kHz and 30 kHz | 10 / 52 / 78% 10 / 24 / 72% | 62 / 93% 29 / 87% | Common 15 kHz SSB | none |
| 12.5 MHz | 15 kHz and 30 kHz | 10 / 52 / 75% 10 / 24 / 69% | 62 / 89% 29 / 84% | Use 12 MHz solution |  |
| 13 MHz | 15 kHz and 30 kHz | 10 / 52 / 72% 10 / 24 / 67% | 62 / 86% 29 / 80% | Use 12 MHz solution |  |
|  | 15 kHz only | 10 / 52 / 72% | 67 / 93% | Common 15 kHz SSB | 50kHz GB shift |
| 33 MHz | 15 kHz and 30 kHz | 30 / 160 / 87% 30 / 78 / 85% | 170 / 93% 83 / 91% | Common 15 kHz SSB | Not for n28 |
|  | 15 kHz only | 30 / 160 / 87% | 175 / 96% | Common 15 kHz SSB | 50kHz GB shift |

Proposal 3:

- Solution using the immediately higher existing UE channel bandwidth can be further studied for DL only:

  - It should be an optional UE capability

  - It should be compatible with the default UL/DL operation using immediately lower existing UE channel BW with overlap from network point of view

- No change to the EU specification should be assumed and ACS/blocking/REFSENS characteristics are those of the related existing UE channel bandwidth: It may not be feasible for all channel bandwidths and deployments

Proposal 4:

- Overlap from UE point of view is not supported in UL

- Unless it can be demonstrated that better DL performance is obtained versus using a BW part of the immediately higher BW as optional UE support, overlap from UE point of view should not be the priority to study

Proposal 5:

- Irregular channel bandwidths should be an integer multiple of 1 MHz

- Applying irregular channel bandwidths agreed in this SI to new bands should be done with agreement at plenary

- Adding new irregular channel bandwidths in this SI to existing or new bands should be done with agreement at plenary

# References

[1]	RP-202339 Revised SID: Study on Efficient utilization of licensed spectrum that is not aligned with existing NR channel bandwidths	T-Mobile USA, Ericsson, RAN#90e

[2] R4-2014507 UE Support for Irregular Channel Bandwidths - Options and Constraints, Skyworks Solutions Inc., RAN4#97e

[3] R4-2017833 WF on Irregular channel bandwidths, Ericsson, RAN4#97e