3GPP TSG-WG4 Meeting #98-e	R4-2102376

# Electronic Meeting, Jan. 25-Feb. 5, 2021

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | 0682 | rev | - | Current version: | 15.12.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR for TS 38.101-3: correction CR for simultaneous Tx/Rx operation (R15) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2021-01-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Simultaneous Rx/Tx capability for TDD-TDD and TDD-FDD inter-band NR CA, SUL or inter-band EN-DC configurations should be a per band combination per band pair capability rather than a per BC capability. Two-band combination is the basis for reporting such a capability. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - In the applicability section, make it clear that the simultaneous Rx/Tx capability shall be reproted for combinations identified as mandatory in the spec as well as when the UE is capable of simultaneous Rx/Tx operation. Otherwise, if the capability is absent or not reported for capable of simultaneous Rx/Tx operation, it will have wrong restriction on the network scheduling. - Mandatory capability of simultaneous Rx/Tx also applies also for these carriers when applicable CA configuration is part of a higher order CA configuration since the capability should be a per band pair indicated capability. - To avoid misinterpretation of the spec, remove the capability indication for TDD-FDD band combinations and by default, the simultaneous Rx/Tx capability shall be reported for two-band TDD-FDD band combination unless it indicated in the spec simultaneous Rx/Tx operation is not supported. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The applicability of the simultaneous Rx/Tx capability for TDD-TDD and TDD-FDD inter-band NR CA, SUL or inter-band EN-DC configurations are not clear in the specification, which may lead wrongly scheduling restriction for the network if the capability is not reproted accurately. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2, 5.2A.2.1, 5.2C |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-1 |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## <Start of Change>

## 4.2	Applicability of minimum requirements

a)	In this specification the Minimum Requirements are specified as general requirements and additional requirements. Where the Requirement is specified as a general requirement, the requirement is mandated to be met in all scenarios

b)	For specific scenarios for which an additional requirement is specified, in addition to meeting the general requirement, the UE is mandated to meet the additional requirements.

c)	The spurious emissions power requirements are for the long-term average of the power. For the purpose of reducing measurement uncertainty it is acceptable to average the measured power over a period of time sufficient to reduce the uncertainty due to the statistical nature of the signal

d)	All the requirements for intra-band contiguous and non-contiguous CA apply under the assumption of the same slot format indicated by UL-DL-configuratin-common in the PCell and SCells for NR SA.

e)  For a terminal which supports TDD-TDD and TDD-FDD inter-band NR CA, SUL configurations, if the band combinations are identified as mandatory support of simultaneous Tx/Rx operation in the specification, or the terminal is capable of supporting simultaneous Tx/Rx operation, the capability of simultaneousRxTxInterBandCA or simultaneousRxTxSUL specified in [9] shall be reported. Unless otherwise indicated in the specification, all two-band TDD-FDD inter-band NR CA or SUL configurations shall report the simultaneousRxTx capability.

## <Next Change>

#### 5.2A.2.1	Inter-band CA (two bands)

Table 5.2A.2.1-1: Inter-band CA operating bands involving FR1 (two bands)

| NR CA Band | NR Band (Table 5.2-1) |
| --- | --- |
| CA_n3-n77 | n3, n77 |
| CA_n3-n78 | n3, n78 |
| CA_n3-n79 | n3, n79 |
| CA_n8-n75 | n8, n75 |
| CA n8-n78 | n8, n78 |
| CA_n8-n79 | n8, n79 |
| CA_n28-n752 | n28, n75 |
| CA_n28-n78 | n28, n78 |
| CA_n41-n78 | n41, n78 |
| CA_n75-n78 | n75, n78 |
| CA_n76-n78 | n76, n78 |
| CA_n77-n79 | n77, n79 |
| CA_n78-n793 | n78, n79 |
| NOTE 1: Void NOTE 2: The frequency range in band n28 is restricted for this band combination to 703-733 MHz for the UL and 758-788 MHz for the DL. NOTE 3: Simultaneous Rx/Tx capability does not apply for UEs supporting band n78 with a n77 implementation. |  |

## <Next Change>

## 5.2C	Operating band combination for SUL

NR operation is designed to operate in the operating band combination defined in Table 5.2C-1, where all operating bands are within FR1.

Table 5.2C-1: Operating band combination for SUL in FR1

| NR Band combination for SUL | NR Band (Table 5.2-1) |
| --- | --- |
| SUL_n78-n80 | n78, n80 |
| SUL_n78-n81 | n78, n81 |
| SUL_n78-n82 | n78, n82 |
| SUL_n78-n83 | n78, n83 |
| SUL_n78-n84 | n78, n84 |
| SUL_n78-n86 | n78, n86 |
| SUL_n79-n80 | n79, n80 |
| SUL_n79-n81 | n79, n81 |
| NOTE 1: If a UE is configured with both NR UL and NR SUL carriers in a cell, the switching time between NR UL carrier and NR SUL carrier is 0 us. NOTE 2: Void. NOTE 3: For UE supporting SUL band combination, UL MIMO is not configured on SUL carrier. |  |

## <End of Change>