3GPP TSG-WG4 Meeting #98-e	R4-2102381

# Electronic Meeting, Jan. 25-Feb. 5, 2021

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-3 | CR | 0482 | rev | - | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR for TS 38.101-3: NR V2X SL switching period |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL-Core |  |  |  |  |  | Date: |  |  | 2021-01-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The output power dynamics requirements for NR V2X is introduced in TS 38.101-3. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Specify the switching period requirement |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Requirements for NR V2X con-current opeation are not completed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3E |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-3 |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## <Start of Change>

6.3E	Output power dynamics for V2X operation in FR1

### 6.3E.1	General

The E-UTRA SL and NR SL switching time mask defines the observation period or switching period position between E-UTRA subframe and NR slot/mini-slot boundary. Both E-UTRA subframe and NR slot/mini-slot have ON power transmissions. The ON power is defined as the mean power over the symbol duration excluding any transient period. For E-UTRA subframe or NR slot/mini-slot having OFF power transmission, the general time mask for E-UTRA or NR shall apply.

For intra-band V2X operation,  the output power dynamics specified in clause 6.3.2G, 6.3.3G, 6.3.4G and 6.3.5G [4] shall be applied for E-UTRA SL transmission or the output power dynamics specified in clause 6.3E [2] shall be applied for NR SL transmission, respectively.

For the inter-band con-current NR V2X operation, output power dynamics requirement specified in subclause 6.3 of TS 36.101 [4] shall apply for the E-UTRA uplink in licensed band and the requirements specified in subclause 6.3E of TS 38.101-1 [2] shall apply for the sidelink in NR Band n47.

### 6.3E.2	Output power dynamics for intra-band V2X operation

For intra-band V2X operation bands UE specified in subclause 5.3E.1 and 5.3E.2, the UL switching time mask in Figure 6.3E.2-1 shall apply.

Figure 6.3E.2-1: Time mask for switching between NR V2X SL and E-UTRA V2X SL

Applicability of the priority is conditioned with the priorities of the two channels/signals are known to the UE T msec prior to the start of the earlier of the two transmissions, where T≤4 and is based on UE implementation. Otherwise, where to put the switching period is up to UE implementation.

## <End of Change>