3GPP TSG-RAN WG4 Meeting #98-e                                          R4-2102387

Electronic Meeting, Jan. 25-Feb. 5, 2021

Source: 	Huawei, Hisilicon

Title: 	On SUO for intra-band EN-DC

Agenda Item:	15.2

Document for:	Approval

# Introduction

Clarification on single uplink operation (SUO) for intra-band EN-DC was initiated in RAN#90e meeting [1]. Based on the discussion, RAN tasked RAN2/RAN4 to check if any specification clarification is needed to ensure there is no inter-operability issue between the UE side and network side.

This contribution provides further consideration on SUO for intra-band EN-DC.

# Discussion

Single UL operation (SUO) was initially introduced in RAN4 for the reason of MSD degradation caused by intermodulation products in dual uplink operation, and some conditions should be met for such claim as “single UL allowed” in RAN4 spec (Annex I of TS 38.101-3).

“…indicated by column single uplink allowed if the intermodulation products caused by the dual uplink operation do not interfere with its own primary downlink transmission channel bandwidth of PCell or PSCell. For intermodulation products falling into any secondary downlink channel bandwidth, UE single UL capability is not considered.”

For intra-band EN-DC, besides the potential IMD issue, most importantly, no RF requirements are defined for dual uplink operation for most of intra-band combinations. For these band combinations, along with “single UL allowed” indication, there is a note highlighting that “Only single switched UL is supported”. Such note is added to both intra-band contiguous and non-contiguous EN-DC for some configurations as cited below:

Table 5.5B.2-1: Intra-band contiguous EN-DC configurations

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | Single UL allowed |
| --- | --- | --- |
| DC_(n)5AA | DC_(n)5AA6 | Yes6 |
| DC_(n)12AA | DC_(n)12AA6 | Yes6 |
| DC_(n)38AA5 | DC_(n)38AA6 | Yes6 |
| DC_(n)41AA5 DC_(n)41AB5 DC_(n)41CA5 DC_(n)41DA5 | DC_(n)41AA | Yes3 |
| DC_(n)41AB5 DC_(n)41CA5 DC_(n)41DA5 | DC_41A_n41A | Yes3 |
| DC_(n)48AA5 | DC_(n)48AA6 | Yes6 |
| DC_(n)48CA5 | DC_(n)48AA6 DC_48A_n48A6 | Yes6 |
| DC_(n)48DA5 | DC_(n)48AA6 DC_48A_n48A6 | Yes6 |
| DC_(n)71AA2 | DC_(n)71AA | No4 |
| NOTE 1: Uplink EN-DC configurations are the configurations supported by the present release of specifications. NOTE 2: Requirements in this specification apply for NR SCS of 15 kHz only. NOTE 3: Single UL allowed due to potential emission issues, not self-interference. NOTE 4: For UE(s) supporting dynamic power sharing it is mandatory to do dual simultaneous UL. For UE(s) not supporting dynamic power sharing single UL is allowed. NOTE 5: The minimum requirements only apply for non-simultaneous Tx/Rx between all carriers. NOTE 6: Only single switched UL is supported |  |  |

Table 5.5B.3-1: Intra-band non-contiguous EN-DC configurations

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | Single UL allowed |
| --- | --- | --- |
| DC_2A_n2A | DC_2A_n2A5 | Yes5 |
| DC_3A_n3A | DC_3A_n3A2 | Yes2 |
| DC_5A_n5A | DC_5A_n5A5 | Yes5 |
| DC_7A_n7A6 | DC_7A_n7A5 | Yes5 |
| DC_41A_n41A3 DC_41C_n41A3 DC_41D_n41A3 | DC_41A_n41A | Yes4 |
| DC_48A_n48A3 | DC_48A_n48A5 | Yes5 |
| DC_48A_(n)48AA3 | DC_(n)48AA5 DC_48A_n48A5 | Yes5 |
| DC_48A-48A_n48A3 | DC_48A_n48A5 | Yes5 |
| DC_48C_n48A3 | DC_48A_n48A5 | Yes5 |
| DC_48D_n48A3 | DC_48A_n48A5 | Yes5 |
| DC_66A_n66A | DC_66A_n66A5 | Yes5 |
| NOTE 1: Uplink EN-DC configurations are the configurations supported by the present release of specifications. NOTE 2: Only single switched UL is supported in Rel.15 NOTE 3: The minimum requirements only apply for non-simultaneous Tx/Rx between all carriers. NOTE 4: Single UL allowed due to potential emission issues, not self-interference. NOTE 5: Only single switched UL is supported. NOTE 6: Requirements in this specification apply for NR SCS of 15 kHz only. |  |  |

From the specification, we have the following observations:

Observation 1: Besides DC_71_n71, DC_41_n41 and DC_3_n3, all other intra-band combinations can only support single switched UL

Observation 2: No dual uplink RF requirements are defined for EN-DC band combinations for which only single switched UL is supported

Observation 3: For some band combinations, e.g. DC_3_n3, only single switched UL is supported in Rel-15 while dual UL can be supported in Rel-16

It is worth noting that it is not the first time to discuss the SUO related issues. Actually there was an agreed WF in [3] on only single switched UL though the purpose of previous discussion is different the clarification intention raised in RAN meeting. However, the agreement of the WF is also helpful for the SUO clarification, i.e.

- The existing capability “singleUL-Transmission” can be reused for these EN-DC band combinations for which only single switched UL is supported.

According to the WF, we have the following observations:

- Optional capability “singleUL-Transmission” is reused to indicate supporting only single switched UL

- For UE supporting only single switched UL, NR could be dropped if dual UL scheduled by network

- For UE supporting only single switched UL, it may violate the regulatory requirements if dual UL is scheduled by network as no RF requirements are specified for these EN-DC combinations in RAN4

- For band combination which supports “only single switched UL” in early release, but dual UL in later release once the requirements for dual UL are specified, network cannot distinguish these two kinds of UE without capability indication

- Relying on specific note in RAN4 spec rather than available UE capability needs frequent update of NW software version once new intra-band EN-DC combinations are added into RAN4 spec

To avoid unexpected performance degradation and possible violation of regulatory requirements, we think that it is necessary to clarify in RAN2 specification that for intra-band EN-DC combinations which support only single switched UL, the capability singleUL-Transmission must be reported.

During the RAN discussion, three cases are utilized for better understanding the SUO issue, which are:

Case 1: the UE reports DC_2A_7A_66A_n66A (i.e. UL allowed in 2A and n66A), singleUL-Transmission is not required to be reported

Case 2: the UE reports DC_2A_7A_66A_n66A (i.e. UL allowed in 66A and n66A), singleUL-Transmission is required to be reported

Case 3: the UE reports DC 66A_n66A, singleUL-Transmission is required to be reported

These cases also relevant to another BCS issue discussed in [4]. However, in order to address the ambiguity of application of RF requirements for single switched UL, for the configurations which support intra-band EN-DC and Only single switched UL is supported, the UE shall report the singleUL-Transmission capability.

Proposal: It is proposed to send LS to RAN2 asking some clarification in RAN2 spec that that for intra-band EN-DC combinations which support only single switched UL, the capability singleUL-Transmission must be reported.

# Conclusion

Further consideration on clarification on SUO for intra-band EN-DC is provided in the contribution. To avoid unexpected performance degradation and possible violation of regulatory requirements, we think it is necessary to clarify in RAN2 specification that for intra-band EN-DC combinations which support only single switched UL, the capability singleUL-Transmission must be reported.

Proposal: It is proposed to send LS to RAN2 asking some clarification in RAN2 spec that that for intra-band EN-DC combinations which support only single switched UL, the capability singleUL-Transmission must be reported.

# References

[1] RP-202622, Clarification on single uplink operation (SUO) for intra-band EN-DC, Huawei, HiSilicon, Telus

[2] RP-202932, LS on single UL operation

[3] R4-2011934, WF on only single switched UL, Huawei, HiSilicon

[4] RP-202935, LS on BCS reporting and support for intra-band EN-DC band combinations