3GPP TSG-RAN WG4 Meeting #98-e	R4-2102586

Online, Jan 25th – Feb 3rd, 2021

Source: 	Qualcomm Incorporated

Title: 	Discussion on NR-U

Agenda item:	7.1.7.3

Document for:	Discussion on NR-U CQI Performance Tests

# Introduction

During the previous RAN4 meeting, there have been discussions about the need of introducing CSI Requirements for NR-U as collected in the WF [3].

This paper will present our views on this point and will address discussions that different companies in the email thread [5] during the previous meeting.

# DL Transmission Model for NR Unlicensed

Some companies expressed in their views in [5] that they see interest in introducing CQI requirements, and the WF in [6] suggests to discuss this option.

In our company’s view there are 2 items to be addressed if CQI requirements are to be introduced, please find them addressed in the next sections.

## Handling of LBT impact on CSI-RS in CQI Performance Tests

As discussed in the paper submitted by our company to this meeting regarding PDSCH Performance testing for NR-U, with the agreed LBT model in some cases scheduled periodic CSI-RS Transmission can not be transmitted due to LBT failure.

For a detailed explanation of these issue, please refer to [6] the description of the issue and our view on how RAN4 should discuss about these tests.

Proposal 1: Discuss tests definition for CQI Performances based on the expected UE behaviour and capabilities related to CSI-RS validation in case of LBT failure, in parallel with the discussion of the same issue for PDSCH Performances.

## TE Behaviour and Performance Metric

Independently from the previous issue, in case of missing CSI-RS occasions due to LBT failure, it’s our company’s view that there are no directions on the content of UE CSI reporting.

In order to define a functional test, it will have to be defined how the TE will behave upon receiving a UE reporting following a CSI-RS occasion that was not transmitted due to LBT failure.

Proposal 2: For NR-U CQI Performance Testing, TE is expected to disregard a UE reporting which comes following a scheduled CSI-RS transmission that did not happen due to LBT failure, and it is expected to continue scheduling according to the last valid reporting.

Proposal 3: For NR-U CQI Performance Testing, CQIs reported by the UE following a CSI-RS occasion that was not transmitted due to LBT failure will be ignored for the purposes of computing test pass/fail statistics.

# Conclusions

Proposal 1: Discuss tests definition for CQI Performances based on the expected UE behaviour and capabilities related to CSI-RS validation in case of LBT failure, in parallel with the discussion of the same issue for PDSCH Performances.

Proposal 2: For NR-U CQI Performance Testing, TE is expected to disregard a UE reporting which comes following a scheduled CSI-RS transmission that did not happen due to LBT failure, and it is expected to continue scheduling according to the last valid reporting.

Proposal 3: For NR-U CQI Performance Testing, CQIs reported by the UE following a CSI-RS occasion that was not transmitted due to LBT failure will be ignored for the purposes of computing test pass/fail statistics.

# References

[1] RP-191575, “Revised WID on NR-based Access to Unlicensed Spectrum”, 3GPP TSG RAN Meeting #84, Newport Beach, USA, June 3-6, 2019.

[2] “LTE; Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio transmission and reception;”, 3GPP TS 36.101 version 15.9.0 Release 15;

[3] R4-2017685, “Way Forward on NR-U UE demodulation requirements”, 3GPP TSG RAN WG4 Meeting #97-e, e-Meeting, November 2 - 13	2020.

[4] 3GPP TS 38.101-4, “User Equipment (UE) radio transmission and reception; Part 4: Performance requirements (Release 16)”

[5] R4-2017610, “Email discussion summary for [97e][315] NR_unlic_Demod_UE”, 3GPP TSG RAN WG4 Meeting #97-e, e-Meeting, November 2 - 13	2020.

[6] R4-2102585, “Discussion on NR-U PDSCH Demodulation Performance Tests”, 3GPP TSG RAN WG4 Meeting #97-e, e-Meeting, Jan 25 – Feb 3 2021.