3GPP TSG-RAN WG4 Meeting#98-e                	R4-2102865

Electronic Meeting, 2 January – 5 February 2021

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussions on Rel-16 NR eMIMO L1-SINR measurement Accuracy

Agenda item:	7.9.3.2

Document for:	Discussion

Introduction

During the RAN4#97-e meeting, a way forward on RRM requirements for NR eMIMO [1] was approved. There are still a number of open issues related to L1-SINR measurement accuracy.

This contribution provides our views of the open issues.

Discussion

Implementation margin of L1-SINR measurement accuracy

From [1], the open issue is as follows:

- FFS additional FR2 implementation margin in CMR + IMR scenario - Option 1: No obvious difference as it is SINR in CMR + IMR - Option 2: Consider RF margin [x] dB higher for FR2 than FR1 in CMR + IMR - FFS if any other implementation margin necessary to derive final requirement on the basis of simulation-based accuracy

In [1], the following agreement is captured:

Agreements:

Follow RAN1 assumption that UE uses same Rx beam for channel and interference measurements for both CMR only and CMR+IMR cases

Based on the agreement, it can be assumed that RF impairments are cancelled out.

Option 1: No obvious difference as it is SINR in CMR + IMR

Accuracy requirements of L1-SINR under extreme conditions

From [1], the open issue is:

- Option 1: 1dB higher than for normal condition (same logic as SS-SINR) - Option 2: 2dB higher than for normal condition - Option 3: other values

Adopting the same methodology as for SS-SINR seems reasonable.

Option 1: 1 dB higher than for normal condition

Relative accuracy of L1-SINR measurement requirement

From [1], the open issue is:

- FFS the necessity and definition of relative accuracy of L1-SINR measurement requirement - FFS the methodology for deriving relative accuracy of L1-SINR measurement requirement

Referring to Section 10.1.19 in TS 38.133, the relative accuracy definition of SSB based L1-RSRP is as follows:

The relative accuracy of SSB based L1-RSRP is defined as the L1-RSRP measured from one SSB compared to the largest measured value of L1-RSRP among all SSBs of the serving cell.

The relative accuracy definition of CSI-RS based L1-RSRP is:

The relative accuracy of CSI-RS based L1-RSRP is defined as the L1-RSRP measured from one CSI-RS compared to the largest measured value of L1-RSRP among all CSI-RS resources of the serving cell.

The same methodology as for L1-RSRP can be used to define relative accuracy of L1-SINR.

Relative accuracy of L1-SINR can be defined as follows:

- The relative accuracy of SSB based L1-SINR is defined as the L1-SINR measured from one SSB compared to the largest measured value of L1-SINR among all SSBs of the serving cell.

- The relative accuracy of CSI-RS based L1-SINR is defined as the L1-SINR measured from one CSI-RS compared to the largest measured value of L1-SINR among all CSI-RS resources of the serving cell.

Consider adopting the same methodology as L1-RSRP relative accuracy to derive L1-SINR relative accuracy requirements.

Conditions for L1-SINR measurements

At the RAN4 97-e meeting, a question was raised concerning the conditions for L1-SINR measurements. In general, the same methodology as for L1-RSRP can be used. The definition of the conditions for CMR only, CMR+NZP-IMR and CMR+ZP-IMR can be found in the draft CR [2].

Conclusion

The document has discussed the remaining open issues on L1-SINR measurements, and our conclusions are as follows:

- Option 1: No obvious difference as it is SINR in CMR + IMR

- Option 1: 1 dB higher than for normal condition

Relative accuracy of L1-SINR can be defined as follows:

- The relative accuracy of SSB based L1-SINR is defined as the L1-SINR measured from one SSB compared to the largest measured value of L1-SINR among all SSBs of the serving cell.

- The relative accuracy of CSI-RS based L1-SINR is defined as the L1-SINR measured from one CSI-RS compared to the largest measured value of L1-SINR among all CSI-RS resources of the serving cell.

Consider adopting the same methodology as L1-RSRP relative accuracy to derive L1-SINR relative accuracy requirements.

References

[1]   R4-2017375, WF on NR eMIMO RRM performance requirements, Samsung

[2]    R4-2102868, CR to TS 38.133: Adding conditions for L1-SINR reporting (Annex B.2), Nokia, Nokia Shanghai Bell