3GPP RAN WG4 Meeting #98-e		R4-2102885

Electronic meeting, 25th Jan – 5th Feb 2021

Agenda Item:	7.5.3.1.2

Source: 		Qualcomm Incorpor ated

Title: 			Discussion on Cell Configuration for EMR Test Case

Document for:	Discussion

# 1.	Introduction

We present our views on cell configurations for EMR test case.

# 2. 	Discussion

During the previous RAN4#97 e-meeting, 8 test cases were discussed aiming to introduce the detailed test cases in RAN4#98 e-meeting. The test cases are listed in the table.

| TC# | Serving Carrier [LTE, NR (FR1, FR2)] | Target Carrier [LTE, NR (FR1, FR2)] | Target Carrier type [mobility carrier or not] | Target Cell type [New cell, Detected cell] | Beam level Measurements [yes, no] | s-NonIntraSearch thresholds configured [yes, no] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | NR FR1 | NR FR1 | mobility | detected | Yes | Yes |
| 2 | NR FR1 | NR FR2 | not mobility | not detected | No | Yes |
| 3 | NR FR1 | LTE | mobility | detected | N/A | No |
| 4 | NR FR2 | NR FR1 | not mobility | not detected | Yes | No |
| 5 | NR FR2 | NR FR2 | mobility | detected | No | No |
| 6 | NR FR2 | LTE | mobility | detected | N/A | No |
| 7 | LTE | NR FR1 | mobility | detected | No | Yes |
| 8 | LTE | NR FR2 | not mobility | not detected | Yes | No |

Even though the set of listed test cases is an outcome of effort to minimize the number of test cases, we still can see some test cases which are expected to be not prevalent in real deployments.

- TC#3: We don’t expect LTE cell to be configured as SCell when there are NR serving cells in FR1.

- TC#4: We don’t expect NR FR1 cell to be configured as SCell while NR FR2 cell is PCell. Typically, it should be the other way around, which can be found in TC#2.

- TC#5: We don’t think EMR-based SCell fast configuration is a useful feature in FR2 + FR2 scenario.

- TC#6: We don’t expect NTE cell to be configured as SCell while NR FR2 cell is PCell. It should be the other way around, which can be found in TC#8.

With the above observation, we propose the following:

Proposal 1: RAN4 to not introduce test cases for the following combinations:

- PCell on NR RF1 and EMR target cell on LTE

- PCell on NR RF2 and EMR target cell on NR FR1

- PCell on NR RF2 and EMR target cell on NR FR2

- PCell on NR RF2 and EMR target cell on LTE

# 3.	Conclusion

We presented our views on cell configurations for EMR test case.

Proposal 1: RAN4 to not introduce test cases for the following combinations:

- PCell on NR RF1 and EMR target cell on LTE

- PCell on NR RF2 and EMR target cell on NR FR1

- PCell on NR RF2 and EMR target cell on NR FR2

- PCell on NR RF2 and EMR target cell on LTE