3GPP TSG-RAN WG4 #98-e		                              	  R4-2102927

Electronic Meeting, 25 January – 5 February, 2021

Source: 	Skyworks Solutions, Inc.

Title: 	35MHz 45MHz REFSENS

Agenda Item:	9.22.3 UE RF requirements [NR_FR1_35MHz_45MHz_BW-Core]

Document for:	Approval

# Introduction

This document presents 35MHz and 45MHz measurements and proposes associated REFSENS requirements.

# Discussion

## Background

At last RAN4 meeting #97e, the following agreements were captured in WF [1] and summarized in Table 1 below.

Table 1: WF agreements on REFSENS & AMPR

| Band | Channel Bandwidth (MHz) | REFSENS configuration |  | Comment |
| --- | --- | --- | --- | --- |
|  |  | Uplink (MHz) | Downlink (MHz) |  |
| n3 | 35 | 35 | 35 | Agreed Lcrb = 50 |
|  | 45 | 45 | 45 | Agreed Lcrb = 50 |
| n8 | 35 | 20 | 35 | Evaluation of worst-case and best-case MSD test points |
| n25 | 35 | 35 | 35 | Agreed Lcrb = 40 |
|  | 45 | 40 | 45 |  |
| n71 | 35 | 20 | 35 | Evaluation of worst-case and best-case MSD test points |

At last RAN plenary meeting, it was been agreed upon to study 35MHz CBW for n2 and 45MHz for n1. For n2, we re-use n25 WF assumptions, for n1, considering the large duplex distance, we assume symmetrical 45MHz Uplink (UL)/Downlink (DL) configuration.

We also wrap-up REFSENS requirements for n7 35MHz and n66 35MHz and 45MHz where no MSD occur.

## Power Amplifier Calibration and REFSENS Assumptions

For Power Amplifier (PA) transmitter (Tx) noise emission in receiver (Rx) band, we assume:

- PA calibration point is 20 MHz, 15 kHz, QPSK, DFT-S-OFDM, 100 RB at lower channel edge with 1 dB MPR;

- Post PA losses: 4dB;

- Power Class 3 (PC3) operation;

- Local Oscillator (LO) leakage: -28dBc;

- IQ Image rejection: -28dB;

- C-IM3: -60dBc, C-IM5: -70dBc

- We measure Tx noise in Rx band for 2 input noise floor test conditions at PA input: with direct connection from RF test equipment signal generator into PA, i.e. the PA noise PSD in Rx band is above thermal noise, and with close to thermal noise PSD in Rx band at PA input port by inserting an input band pass filter at PA input. Lowering the noise PSD in Rx band at PA input is important for the case when the measured Tx noise in Rx at PA output is very low. This has no impact in the case of moderate to high desensitization levels, such as 35MHz in n71 or 45MHz in band 25.

For bands where a difference is observed, we select a value which we believe is a fair representation of commercial performance.

For REFSENS estimations, we assume:

- 50 dB Tx to RX and Tx to Antenna duplexer rejection in Rx band, for all bands except in band n71 where 55dB is assumed,

- 10 dB Primary antenna to diversity antenna isolation;

- 4 dB Post PA loss;

- 4 dB Antenna to LNA front-end losses;

- 3 dB Antenna diversity gain;

- -1 dB SNR; and

- Previously agreed rules to compute correlated and uncorrelated MRC contributions.

### n1 REFSENS

Considering no MSD occurs for n1 50MHz REFSENS, we make the following proposal for n1 45MHz CBW.

Proposal 1: Adopt the following n1 two antenna port Reference Sensitivity QPSK and Uplink Configuration tables.

Table 7.3.2-1: Two Antenna Port Reference Sensitivity QPSK PREFSENS

| Operating Band | SCS kHz | 45 MHz (dBm) | Duplex Mode |
| --- | --- | --- | --- |
| n11 | 15 | -90.1 | FDD |
|  | 30 | -90.1 |  |
|  | 60 | -90.3 |  |
| NOTE 1: Four Rx antenna ports shall be the baseline for this operating band except for two Rx vehicular UE. NOTE 2: The transmitter shall be set to PUMAX as defined in clause 6.2.4. |  |  |  |

Table 7.3.2-3: Uplink Configuration for Reference Sensitivity

| Operating Band | SCS kHz | 45 MHz (dBm) | Duplex Mode |
| --- | --- | --- | --- |
| n11 | 15 | 1281 | FDD |
|  | 30 | 641 |  |
|  | 60 | 301 |  |
| NOTE 1: UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.3.2-1). |  |  |  |

### n3 REFSENS

The measured PA Tx noise level in Rx band for symmetrical 35MHz and 45MHz CBW is plotted in Figure 1 vs Lcrb.

Figure 1: n3 Tx noise level in Rx band vs Lcrb with (orange) and without PA input filter (blue). Left symmetrical UL/DL 35MHz, Right: symmetrical UL/DL 45MHz CBW.

At the agreed Lcrb=50, the Tx noise level is:

- For 35MHz CBW (Figure 1 – left): worst case -39dBm/33.855MHz (blue), best case -49 dBm/33.855MHz (orange). For the MSD analysis, we propose to adopt a midway point of -45dBm/33.855MHz, ie a Tx noise PSD in Rx band of -121dBm/Hz (orange).

- For 45MHz CBW (Figure 1 – right): worst case -35.7dBm/43.755MHz (blue), best case -40.2dBm/43.755MHz (orange). For the MSD analysis, we assume -38dBm/43.755MHz

The MSD analysis is presented in Table 2 and shows 0.7 dB receiver desense for 35MHz operation and 2.0 dB desense for 45MHz CBW.

Table 2: n3 35/45MHz Tx noise level in Rx band and MSD analysis.

Proposal 2: Adopt the following n3 two antenna port Reference Sensitivity QPSK and Uplink Configuration tables.

Table 7.3.2-1: Two Antenna Port Reference Sensitivity QPSK PREFSENS

| Operating Band | SCS kHz | 35 MHz (dBm) | 45 MHz (dBm) | Duplex Mode |
| --- | --- | --- | --- | --- |
| n31 | 15 | -87.5 | -85.1 | FDD |
|  | 30 | -87.6 | -85.2 |  |
|  | 60 | -87.8 | -85.3 |  |
| NOTE 1: Four Rx antenna ports shall be the baseline for this operating band except for two Rx vehicular UE. NOTE 2: The transmitter shall be set to PUMAX as defined in clause 6.2.4. |  |  |  |  |

Table 7.3.2-3: Uplink Configuration for Reference Sensitivity

| Operating Band | SCS kHz | 35 MHz (dBm) | 45 MHz (dBm) | Duplex Mode |
| --- | --- | --- | --- | --- |
| n31 | 15 | 501 | 501 | FDD |
|  | 30 | 241 | 241 |  |
|  | 60 | 101 | 101 |  |
| NOTE 1: UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.3.2-1). |  |  |  |  |

### n8 REFSENS

Here we reproduce here results presented at previous meeting from [2]. Figure 2 summarizes the n8 spectrum landscape for 35MHz DL CBW operation. According to WF [1], it is agreed to restrict n8 UL CBW to 20MHz, for which case A represents the REFSENS best-case scenario, and case B represents the REFSENS worst-case scenario. It was shown in [2] that case C leads to nearly 22dB receiver desense,

Figure 2: n8 35MHz DL CBW spectrum perspective for A) 20MHz uplink CBW at highest channel (top), B) 20MHz uplink CBW at lowest channel (middle), C) 35MHz uplink CBW (bottom).

Case A: 35MHz DL CBW MSD due to 20MHz UL CBW at highest channel

Tx noise level in 35MHz DL channel measured with 20MHz UL LCRB =25 is -36.0 dBm.

Case B: 35MHz DL CBW MSD due to 20MHz UL CBW at lowest channel

Figure 3 shows the measured 20MHz UL CBW Tx noise level in 35MHz DL CBW vs LCRB for the case when the UL channel is closest to the DL band.

Figure 3: 20MHz UL CBW Transmitter noise level in 35MHz DL CBW vs LCRB – Case B.

Tx noise level reaches a minimum for LCRB ranging from approximately 25 to 40 RB. We therefore propose to adopt LCRB =25 for MSD evaluation, value at which the transmitter noise level is -15.3 dBm.

The MSD analysis for case A and case B is summarized in Table 3.

Table 3: n8 35MHz MSD for case A, B.

Proposal 3: Adopt the following n8 two antenna port Reference Sensitivity requirements and test points.

Table 7.3.2-1: Two Antenna Port Reference Sensitivity QPSK PREFSENS for n8 35MHz CBW.

| Operating band / SCS / Channel bandwidth / Channel allocations / REFSENS/ Duplex mode |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | Channel bandwidth (DL) (MHz) | Channel bandwidth (UL) (MHz) | FC (DL) (MHz) | FC (UL) (MHz) | UL allocation (LCRB) | REFSENS (dBm) | Duplex Mode |
| n8 | 15 | 35 | 20 | 942.5 | 890.0 | 252 | -85.1 | FDD |
|  | 30 |  |  |  |  | 102 | -85.2 |  |
|  | 60 |  |  |  |  |  |  |  |
| n8 | 15 | 35 | 20 | 942.5 | 905.0 | 252 | -70.6 | FDD |
|  | 30 |  |  |  |  | 102 | -70.7 |  |
|  | 60 |  |  |  |  |  |  |  |
| NOTE 1: The transmitter shall be set to PUMAX as defined in clause 6.2.4 NOTE 2: UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.3.2-1) |  |  |  |  |  |  |  |  |

### n25/n2 REFSENS

For operation at 35MHz CBW, Tx noise in Rx band is measured assuming symmetrical 35MHz/35MHz UL/DL and is plotted in Figure 4 below. At the agreed Lcrb = 40RB, we measure -36.5dBm/33.855MHz when PA input noise PSD in Rx is that of RF transmit modulator (blue), and -45.3 dBm/33.855MHz when noise PSD in Rx band presented at PA input is close to thermal noise (orange). For MSD analysis we assume a Tx noise level of -40dBm/33.855MHz.

Figure 4: 35MHz symmetrical UL/DL n25/n2 Tx noise level in Rx band vs Lcrb with (orange) and without PA input filter (blue).

For operation at 45MHz CBW, two sets of measurements:

- Case A: best-case Tx noise in Rx, where a 40MHz UL carrier is modulated within the duplex-distance 45MHz channel at the highest frequency offset from the DL carrier,

- Case B: worst-case Tx noise in Rx, where a 40MHz UL carrier is modulated within the duplex-distance 45MHz channel at the lowest frequency offset from the DL carrier,

- Case C: symmetric 45MHz UL/DL configuration – case where DL carrier is partially overlapped by the upper ACLR regrowth – This case is measured for reference.

Case A, B and C are sketched in Figure 5 below.

Figure 5: n25/n2 asymmetric UL 40 MHz/ DL 45MHz measurement assumptions.

Tx noise levels for case A,B,C vs Lcrb, with thermal noise at PA input (orange curves) and with Tx modulator Tx noise PSD in Rx band at PA input (blue) are plotted in Figure 4.

Figure 4: Case A (Left-40MHz/45MHz UL/DL best case), Case B (Center-40MHz/45MHz UL/DL worst case), Case C (Right-45MHz/45MHz UL/DL) n25/n2 Tx noise level in Rx band vs Lcrb with (orange) and without PA input filter (blue).

At the agreed Lcrb=40, we assume the following Tx noise level in Rx band:

- Case A: -37dBm/43.755MHz

- Case B: -29dBm/43.755MHz

- Case C: -18dBm/43.755MHz

The n25 MSD analysis is shown in Table 2 and shows 2.2dB MSD for 40/45MHz Case A, 5.6dB for 40/45MHz Case B,13.6dB for 45/45MHz Case C and 1.6 dB for 35/35MHz.

Table 2: n25 35/45MHz Tx noise level in Rx band and MSD analysis.

The n2 MSD analysis is shown in Table 2 and shows 2.2dB MSD for 40/45MHz Case A, 5.6dB for 40/45MHz Case B,13.6dB for 45/45MHz Case C and 1.6 dB for 35/35MHz.

Table 2: n2 35/45MHz Tx noise level in Rx band and MSD analysis.

Proposal 4: Adopt the following n25 and n2 antenna port 35MHz Reference Sensitivity QPSK and Uplink Configuration tables.

Table 7.3.2-1: Two Antenna Port Reference Sensitivity QPSK PREFSENS

| Operating Band | SCS kHz | 35 MHz (dBm) | Duplex Mode |
| --- | --- | --- | --- |
| n21 | 15 | -87.1 | FDD |
|  | 30 | -87.2 |  |
|  | 60 | -87.3 |  |
| n251 | 15 | -86.1 | FDD |
|  | 30 | -86.2 |  |
|  | 60 | -86.3 |  |

Table 7.3.2-3: Uplink Configuration for Reference Sensitivity

| Operating Band | SCS kHz | 35 MHz (dBm) | Duplex Mode |
| --- | --- | --- | --- |
| n21 | 15 | 401 | FDD |
|  | 30 | 201 |  |
|  | 60 | 101 |  |
| n251 | 15 | 401 | FDD |
|  | 30 | 201 |  |
|  | 60 | 101 |  |

For 45MHz CBW operation, two REFSENS test points are proposed (best-case and worst-case) as:

| Operating band / SCS / Channel bandwidth / Channel allocations / REFSENS/ Duplex mode |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | Channel bandwidth (DL) (MHz) | Channel bandwidth (UL) (MHz) | FC (DL) (MHz) | FC (UL) (MHz) | UL allocation (LCRB) | REFSENS (dBm) | Duplex Mode |
| n2 | 15 | 45 | 40 | 1967.5 | 1885.0 | 401 | -85.4 | FDD |
|  | 30 |  |  |  |  | 201 | -85.5 |  |
|  | 60 |  |  |  |  | 101 | -85.6 |  |
| n2 | 15 | 45 | 40 | 1967.5 | 1890.0 | 401 | -81.6 | FDD |
|  | 30 |  |  |  |  | 201 | -81.7 |  |
|  | 60 |  |  |  |  | 101 | -81.8 |  |
| n25 | 15 | 45 | 40 | 1972.5 | 1890.0 | 401 | -84.4 | FDD |
|  | 30 |  |  |  |  | 201 | -85.5 |  |
|  | 60 |  |  |  |  | 101 | -85.1 |  |
| n25 | 15 | 45 | 40 | 1972.5 | 1895.0 | 401 | -81.0 | FDD |
|  | 30 |  |  |  |  | 201 | -81.1 |  |
|  | 60 |  |  |  |  | 101 | -81.2 |  |
| NOTE 1: The transmitter shall be set to PUMAX as defined in clause 6.2.4 NOTE 2: UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.3.2-1) |  |  |  |  |  |  |  |  |  |

### n7 REFSENS

Considering no MSD occurs for n7 30MHz and 40MHz REFSENS, we propose to adopt the following REFSENS requirements.

Proposal 5: n7 antenna port Reference Sensitivity QPSK and Uplink Configuration tables.

Table 7.3.2-1: Two Antenna Port Reference Sensitivity QPSK PREFSENS

| Operating Band | SCS kHz | 35 MHz (dBm) | Duplex Mode |
| --- | --- | --- | --- |
| n71 | 15 | -89.2 | FDD |
|  | 30 | -89.3 |  |
|  | 60 | -89.4 |  |
| NOTE 1: Four Rx antenna ports shall be the baseline for this operating band except for two Rx vehicular UE. NOTE 2: The transmitter shall be set to PUMAX as defined in clause 6.2.4. |  |  |  |

Table 7.3.2-3: Uplink Configuration for Reference Sensitivity

| Operating Band | SCS kHz | 35 MHz (dBm) | Duplex Mode |
| --- | --- | --- | --- |
| n71 | 15 | 451 | FDD |
|  | 30 | 201 |  |
|  | 60 | 101 |  |
| NOTE 1: 1Refers to the UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.3.2-1). |  |  |  |

### n66 REFSENS

Due to the large duplex distance, no MSD is expected for 35MHz and 45MHz CBW operation, we propose to adopt the following REFSENS requirements.

Proposal 6: n66 antenna port Reference Sensitivity QPSK and Uplink Configuration tables.

Table 7.3.2-1: Two Antenna Port Reference Sensitivity QPSK PREFSENS

| Operating Band | SCS kHz | 35 MHz (dBm) | 45 MHz (dBm) | Duplex Mode |
| --- | --- | --- | --- | --- |
| n661 | 15 | -90.7 | -89.6 | FDD |
|  | 30 | -90.8 | -89.7 |  |
|  | 60 | -91.0 | -89.8 |  |
| NOTE 1: Four Rx antenna ports shall be the baseline for this operating band except for two Rx vehicular UE. NOTE 2: The transmitter shall be set to PUMAX as defined in clause 6.2.4. |  |  |  |  |

Table 7.3.2-3: Uplink Configuration for Reference Sensitivity

| Operating Band | SCS kHz | 35 MHz (dBm) | 45 MHz (dBm) | Duplex Mode |
| --- | --- | --- | --- | --- |
| n661 | 15 | 1801 | 243 | FDD |
|  | 30 | 901 | 1001 |  |
|  | 60 | 401 | 501 |  |
| NOTE 1: UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.3.2-1). |  |  |  |  |

### n71 35MHz REFSENS

We reproduce here results presented at previous meeting from [3].

Figure 6 summarizes the n71 spectrum landscape for 35MHz downlink (DL) channel bandwidth (CBW) operation. According to WF [1], it is agreed to restrict n71 UL CBW to 20MHz, for which case A represents the REFSENS best-case scenario, and case B represents the REFSENS worst-case scenario. It was shown in [3] that case C leads to nearly 22dB receiver desense. It has also been shown that Case C would require 12-14dB A-MPR to meet band n29 protection limits.

Figure 6: n71 35MHz DL CBW spectrum perspective for A) 20MHz uplink CBW at highest channel (top), B) 20MHz uplink CBW at lowest channel (middle), C) 35MHz uplink CBW (bottom).

This paper presents measurement results to evaluate the impact of all cases A, B and C on maximum sensitivity degradation (MSD) and A-MPR. Broader perspective on specification impact can be found in [3].

Case A: 35MHz DL CBW MSD due to 20MHz UL CBW at highest channel

Tx noise level in 35MHz DL channel measured with 20MHz UL LCRB =25 is -36.8dBm.

Case B: 35MHz DL CBW MSD due to 20MHz UL CBW at lowest channel

Figure 6 shows measured 20MHz UL CBW Tx noise level in 35MHz DL CBW vs LCRB for the case when the UL channel is closest to the DL band.

Figure 6: 20MHz UL CBW Transmitter noise level in 35MHz DL CBW vs LCRB – Case B.

Tx noise level reaches a minimum for LCRB ranging from approximately 25 to 45 RB. We propose to adopt LCRB =25 for MSD evaluation, value at which the transmitter noise level -16.4 dBm, i.e. approximately 20dB higher than in Case A. This is expected as ACL shoulders overlap significantly the DL CBW.

The estimated MSD for each case is shown in Table 5 where no A-MPR is assumed, i.e., the PA total transmit power at 0dB back-off is 27dBm.

Table 5: n71 35MHz MSD for case A and B.

The MSD are approximately 2dB for Case A and 16.7dB for Case B.

Proposal 7: Adopt the following n71 two antenna port Reference Sensitivity requirements and test points.

Table 7.3.2-1: Two Antenna Port Reference Sensitivity QPSK PREFSENS for n71 35MHz CBW.

| Operating band / SCS / Channel bandwidth / Channel allocations / REFSENS/ Duplex mode |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | Channel bandwidth (DL) (MHz) | Channel bandwidth (UL) (MHz) | FC (DL) (MHz) | FC (UL) (MHz) | UL allocation (LCRB) | REFSENS (dBm) | Duplex Mode |
| n71 | 15 | 35 | 20 | 634.5 | 688.0 | 252 | -85.5 | FDD |
|  | 30 |  |  |  |  | 102 | -85.6 |  |
|  | 60 |  |  |  |  |  |  |  |
| n71 | 15 | 35 | 35 | 634.5 | 673.0 | 252 | -71.6 | FDD |
|  | 30 |  |  |  |  | 102 | -71.7 |  |
|  | 60 |  |  |  |  |  |  |  |
| NOTE 1: The transmitter shall be set to PUMAX as defined in clause 6.2.4 NOTE 2: UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.3.2-1) |  |  |  |  |  |  |  |  |

# Conclusion

In this contribution, we make proposals n1, n2, n25, n3, n7, n8, n66, n71 REFSENS requirements and associated test points.

Proposal 1: n1 two antenna port Reference Sensitivity QPSK and Uplink Configuration tables.

Table 7.3.2-1: Two Antenna Port Reference Sensitivity QPSK PREFSENS

| Operating Band | SCS kHz | 45 MHz (dBm) | Duplex Mode |
| --- | --- | --- | --- |
| n11 | 15 | -90.1 | FDD |
|  | 30 | -90.1 |  |
|  | 60 | -90.3 |  |
| NOTE 1: Four Rx antenna ports shall be the baseline for this operating band except for two Rx vehicular UE. NOTE 2: The transmitter shall be set to PUMAX as defined in clause 6.2.4. |  |  |  |

Table 7.3.2-3: Uplink Configuration for Reference Sensitivity

| Operating Band | SCS kHz | 45 MHz (dBm) | Duplex Mode |
| --- | --- | --- | --- |
| n11 | 15 | 1281 | FDD |
|  | 30 | 641 |  |
|  | 60 | 301 |  |
| NOTE 1: UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.3.2-1). |  |  |  |

Proposal 2: n3 two antenna port Reference Sensitivity QPSK and Uplink Configuration tables.

Table 7.3.2-1: Two Antenna Port Reference Sensitivity QPSK PREFSENS

| Operating Band | SCS kHz | 35 MHz (dBm) | 45 MHz (dBm) | Duplex Mode |
| --- | --- | --- | --- | --- |
| n31 | 15 | -87.5 | -85.1 | FDD |
|  | 30 | -87.6 | -85.2 |  |
|  | 60 | -87.8 | -85.3 |  |
| NOTE 1: Four Rx antenna ports shall be the baseline for this operating band except for two Rx vehicular UE. NOTE 2: The transmitter shall be set to PUMAX as defined in clause 6.2.4. |  |  |  |  |

Table 7.3.2-3: Uplink Configuration for Reference Sensitivity

| Operating Band | SCS kHz | 35 MHz (dBm) | 45 MHz (dBm) | Duplex Mode |
| --- | --- | --- | --- | --- |
| n31 | 15 | 501 | 501 | FDD |
|  | 30 | 241 | 241 |  |
|  | 60 | 101 | 101 |  |
| NOTE 1: UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.3.2-1). |  |  |  |  |

Proposal 3: n8 two antenna port Reference Sensitivity requirements and test points.

Table 7.3.2-1: Two Antenna Port Reference Sensitivity QPSK PREFSENS for n8 35MHz CBW.

| Operating band / SCS / Channel bandwidth / Channel allocations / REFSENS/ Duplex mode |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | Channel bandwidth (DL) (MHz) | Channel bandwidth (UL) (MHz) | FC (DL) (MHz) | FC (UL) (MHz) | UL allocation (LCRB) | REFSENS (dBm) | Duplex Mode |
| n8 | 15 | 35 | 20 | 942.5 | 890.0 | 252 | -85.1 | FDD |
|  | 30 |  |  |  |  | 102 | -85.2 |  |
|  | 60 |  |  |  |  |  |  |  |
| n8 | 15 | 35 | 20 | 942.5 | 905.0 | 252 | -70.6 | FDD |
|  | 30 |  |  |  |  | 102 | -70.7 |  |
|  | 60 |  |  |  |  |  |  |  |
| NOTE 1: The transmitter shall be set to PUMAX as defined in clause 6.2.4 NOTE 2: UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.3.2-1) |  |  |  |  |  |  |  |  |

Proposal 4: n25 and n2 antenna port Reference Sensitivity QPSK and Uplink Configuration tables.

Table 7.3.2-1: Two Antenna Port Reference Sensitivity QPSK PREFSENS

| Operating Band | SCS kHz | 35 MHz (dBm) | Duplex Mode |
| --- | --- | --- | --- |
| n21 | 15 | -87.1 | FDD |
|  | 30 | -87.2 |  |
|  | 60 | -87.3 |  |
| n251 | 15 | -86.1 | FDD |
|  | 30 | -86.2 |  |
|  | 60 | -86.3 |  |

Table 7.3.2-3: Uplink Configuration for Reference Sensitivity

| Operating Band | SCS kHz | 35 MHz (dBm) | Duplex Mode |
| --- | --- | --- | --- |
| n21 | 15 | 401 | FDD |
|  | 30 | 201 |  |
|  | 60 | 101 |  |
| n251 | 15 | 401 | FDD |
|  | 30 | 201 |  |
|  | 60 | 101 |  |

For 45MHz CBW operation, two REFSENS test points are proposed (best-case and worst-case) as:

| Operating band / SCS / Channel bandwidth / Channel allocations / REFSENS/ Duplex mode |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | Channel bandwidth (DL) (MHz) | Channel bandwidth (UL) (MHz) | FC (DL) (MHz) | FC (UL) (MHz) | UL allocation (LCRB) | REFSENS (dBm) | Duplex Mode |
| n2 | 15 | 45 | 40 | 1967.5 | 1885.0 | 401 | -85.4 | FDD |
|  | 30 |  |  |  |  | 201 | -85.5 |  |
|  | 60 |  |  |  |  | 101 | -85.6 |  |
| n2 | 15 | 45 | 40 | 1967.5 | 1890.0 | 401 | -81.6 | FDD |
|  | 30 |  |  |  |  | 201 | -81.7 |  |
|  | 60 |  |  |  |  | 101 | -81.8 |  |
| n25 | 15 | 45 | 40 | 1972.5 | 1890.0 | 401 | -84.4 | FDD |
|  | 30 |  |  |  |  | 201 | -85.5 |  |
|  | 60 |  |  |  |  | 101 | -85.1 |  |
| n25 | 15 | 45 | 40 | 1972.5 | 1895.0 | 401 | -81.0 | FDD |
|  | 30 |  |  |  |  | 201 | -81.1 |  |
|  | 60 |  |  |  |  | 101 | -81.2 |  |
| NOTE 1: The transmitter shall be set to PUMAX as defined in clause 6.2.4 NOTE 2: UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.3.2-1) |  |  |  |  |  |  |  |  |  |

Proposal 5: n7 two antenna port Reference Sensitivity requirements and test points.

Table 7.3.2-1: Two Antenna Port Reference Sensitivity QPSK PREFSENS

| Operating Band | SCS kHz | 35 MHz (dBm) | Duplex Mode |
| --- | --- | --- | --- |
| n71 | 15 | -89.2 | FDD |
|  | 30 | -89.3 |  |
|  | 60 | -89.4 |  |
| NOTE 1: Four Rx antenna ports shall be the baseline for this operating band except for two Rx vehicular UE. NOTE 2: The transmitter shall be set to PUMAX as defined in clause 6.2.4. |  |  |  |

Table 7.3.2-3: Uplink Configuration for Reference Sensitivity

| Operating Band | SCS kHz | 35 MHz (dBm) | Duplex Mode |
| --- | --- | --- | --- |
| n71 | 15 | 451 | FDD |
|  | 30 | 201 |  |
|  | 60 | 101 |  |
| NOTE 1: 1Refers to the UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.3.2-1). |  |  |  |

Proposal 6: n66 antenna port Reference Sensitivity QPSK and Uplink Configuration tables.

Table 7.3.2-1: Two Antenna Port Reference Sensitivity QPSK PREFSENS

| Operating Band | SCS kHz | 35 MHz (dBm) | 45 MHz (dBm) | Duplex Mode |
| --- | --- | --- | --- | --- |
| n661 | 15 | -90.7 | -89.6 | FDD |
|  | 30 | -90.8 | -89.7 |  |
|  | 60 | -91.0 | -89.8 |  |
| NOTE 1: Four Rx antenna ports shall be the baseline for this operating band except for two Rx vehicular UE. NOTE 2: The transmitter shall be set to PUMAX as defined in clause 6.2.4. |  |  |  |  |

Table 7.3.2-3: Uplink Configuration for Reference Sensitivity

| Operating Band | SCS kHz | 35 MHz (dBm) | 45 MHz (dBm) | Duplex Mode |
| --- | --- | --- | --- | --- |
| n661 | 15 | 1801 | 243 | FDD |
|  | 30 | 901 | 1001 |  |
|  | 60 | 401 | 501 |  |
| NOTE 1: UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.3.2-1). |  |  |  |  |

Proposal 7: Adopt the following n71 two antenna port Reference Sensitivity requirements and test points.

Table 7.3.2-1: Two Antenna Port Reference Sensitivity QPSK PREFSENS for n71 35MHz CBW.

| Operating band / SCS / Channel bandwidth / Channel allocations / REFSENS/ Duplex mode |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | Channel bandwidth (DL) (MHz) | Channel bandwidth (UL) (MHz) | FC (DL) (MHz) | FC (UL) (MHz) | UL allocation (LCRB) | REFSENS (dBm) | Duplex Mode |
| n71 | 15 | 35 | 20 | 634.5 | 688.0 | 252 | -85.5 | FDD |
|  | 30 |  |  |  |  | 102 | -85.6 |  |
|  | 60 |  |  |  |  |  |  |  |
| n71 | 15 | 35 | 35 | 634.5 | 673.0 | 252 | -71.6 | FDD |
|  | 30 |  |  |  |  | 102 | -71.7 |  |
|  | 60 |  |  |  |  |  |  |  |
| NOTE 1: The transmitter shall be set to PUMAX as defined in clause 6.2.4 NOTE 2: UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.3.2-1) |  |  |  |  |  |  |  |  |

# References

[1]	R4 2016864 WF on 35MHz and 45MHz REFSENS AMPR, Qualcomm Inc., Skyworks Solutions Inc., 3GPP TSG-RAN WG4 Meeting #97-e.

[2] 	R4-2016011 n8 35MHz AMPR and MSD Measurements, Skyworks Solutions Inc., RAN4#97e.

[3] 	R4-2016010 n71 35MHz AMPR and MSD Measurements, Skyworks Solutions, Inc., 3GPP TSG-RAN WG4 Meeting #97e.