3GPP TSG-RAN WG4 Meeting #98-e	R4-2102983

Electronic Meeting, Jan .25 – Feb. 5, 2021

Agenda item:			12.2.1

Source:	Huawei, HiSilicon

Title:	Email discussion summary for RAN4#98_#135_NR_RF_FR1_enh_Part_1

Document for:	Information

Introduction

This part includes contributions in agenda 12.2.1, 12.2.2.1 and 12.2.2.4.

Classify the contents into 3 topics:

- Topic #1: Work Plan

- Topic #2: UL MIMO configuration for SUL band configurations as in 12.2.2.1

- Topic #3: intra-band contiguous UL CA for FR1 power class 2 which is for agenda 12.2.2.4

- Topic #4: 4*4 DL MIMO for CA_n77(3A) and CA_n77(4A)

Candidate target of email discussion are as below:

- 1st round:

  - Make agreement on work plan

  - Reach consensus on enabling UL MIMO configuration for SUL

  - Reach consensus on intra-band contiguous UL CA PC2

  - Reach consensus on 4*4 DL MIMO for CA_n77(3A) and CA_n77(4A)

- 2nd round:

  - Approve on the CR for enabling UL MIMO configuration for SUL

  - Approve on the WF for intra-band contiguous UL CA HPUE

  - Approve on WF for 4*4 DL MIMO for CA_n77(3A) and CA_n77(4A)

Topic #1: Work plan for Rel-17 FR1 UE RF enhancement

Main technical topic overview. The structure can be done based on sub-agenda basis.

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2102627 | Huawei, HiSilicon | This paper provides additional Work Plan on Rel-17 FR1 UE RF enhancement according to the time budget agreed in RAN#90 meeting. |

Open issues summary

Sub-topic 1-1 Work plan for Rel-17 FR1 UE RF enhancement

Issue 1-1:  Work plan

- Proposals:

  - Agree on the work plan in R4-2102627

- Recommended WF

  - TBA

Companies views’ collection for 1st round

Open issues

| Sub-topic | Comments: (Company: …) |
| --- | --- |
| 1-1 | ZTE: The latest WID should be RP-202799, not RP-202088. Update the introduction part and reference before approval. |
|  | Huawei: Thanks for ZTE comments, we will revise accordingly. |
|  | Apple: For RAN4 #100-e meeting, #3 first bullet, should A-MPR requirement also need to be agreed for intra-band non-contiguous UL CA HPUE example band? |

CRs/TPs comments collection

Major close-to-finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

| Sub-topic#1 | Recommendations for 2nd round: |
| --- | --- |

Recommendations on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2102627 | To be revised |

Discussion on 2nd round (if applicable)

| T-doc number | Title | Comments |
| --- | --- | --- |

Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | Title | T-doc Status update recommendation |
| --- | --- | --- |

Topic #2: UL MIMO configuration for SUL band configurations

Main technical topic overview. The structure can be done based on sub-agenda basis.

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2100799 | CMCC | In RAN4#97E meeting, the draft CR: Introduce NR SUL bands n80 to UL-MIMO configuration (R4-2014735) was endorsed. This is an official CR to introduce SUL bands n80 to UL-MIMO configuration. - Section 5.2D: Introduce band n80 in the table of NR operating bands for UL-MIMO in Table 5.2D-1. - Section 6.2D: Introduce band n80 in UL MIMO configuration in Table 6.2D.1-1 - Section 5.2C: Remove Note 3 for SUL band combination in Table 5.2C-1 and Table 5.2C-2 (For UE supporting SUL band combination, UL MIMO is not configured on SUL carrier) |
| R4-2101854 | ZTE | Change Note 1 in Table 5.3C-1/2/3/4: NOTE 1: If a UE is configured with both NR UL and NR SUL carriers without enabling UL-MIMO in a cell, the switching time between NR UL carrier and NR SUL carrier is 0 us. |
| R4-2101855 | ZTE | Observation: 0µs switching time is not feasible with the UL-MIMO enabled for SUL Proposal: Revise Note 1 in Table 5.2C-1, 5.2C-2, 5.2C-3 and 5.2C-4 in TS 38.101-1 as: If a UE is configured with both NR UL and NR SUL carriers without enabling UL-MIMO in a cell, the switching time between NR UL carrier and NR SUL carrier is 0 us. |

Open issues summary

Sub-topic 2-1: 0µs switching time between SUL and NUL

Issue 2-1-1: Whether 0µs switching time is feasible between SUL and NUL when SUL is MIMO enabled?

- Proposals

  - Not feasible

- Recommended WF

  - TBA

Issue 2-1-2: Note 1 Table 5.2C-1, 5.2C-2, 5.2C-3 and 5.2C-4 in TS 38.101-1

- Proposals

  - Option 1:  change Note1 as in R4-2101854

  - Option 2:  No need to change the current contents of Note1

- Recommended WF

  - TBA

Companies views’ collection for 1st round

Open issues

| Sub-topic | Comments: (Company: …) |
| --- | --- |
| 2-1 | Issue 2-1-1: ZTE: Obviously not feasible. CMCC: We don’t think it is not feasible. The feature of dynamic Tx switching is for UE supporting maximum two concurrent transmission. If UE has high capability, 0us can be achieved based on UE implementation. OPPO: It is better to clarify the condition that 0us is applied. For the cases that only two Tx chain is implemented then 0us is not applicable for UL MIMO. For UEs with more Tx chain it is ok but apparently it is not current UE can do. Maybe capability is needed, if we want to make 0us for future UE but not for current UE? Huawei: We think the current requirement supporting UL MIMO with 0us is feasible. This is a general requirement which is used for case without UE capability as discussed in Rel-17. |
|  | Issue 2-1-2 ZTE: Changing Note 1 is necessary as in R4-2101854 CMCC: it is not necessary to change note 1 OPPO: OK with the idea but if we consider current UE and future possible more Tx chain UE, then note 1 may need to be improved. Huawei: The requirement in Note 1 is general requirement. After completion of defining UE capability, switching based on UE capability will be used, otherwise, the requirement in Note 1 should be used. No need to change Note 1. Apple: We are fine with option 1. |

CRs/TPs comments collection

Major close to finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2100799 | Title: CR on introducing NR SUL bands n80 to UL-MIMO configuration |
| R4-2101854 | Title: Draft CR to TS 38.101-1 on switching time between SUL and NUL |
|  | Nokia: Wouldn’t it clearer to say “configured with both NR SUL without enabling UL-MIMO and NR UL carriers in a cell”? CMCC: We don’t think change on Note 1 is needed. Huawei: No need to change Note 1. Apple: If CR is agreed. Is it necessary to evaluate the required switching time and add a time mask in the specifications for switching between MIMO SUL and NUL? |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

| Sub-topic | Status summary |
| --- | --- |
| 2-1 | Further discuss on feasibility of 0µs switching time between SUL and NUL when SUL is MIMO enabled based on CR R4-2101854. |

Suggestion on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | Comments collection |
| --- | --- |
| R4-2100799 | Title: CR on introducing NR SUL bands n80 to UL-MIMO configuration |
|  | To be agreed |
| R4-2101854 | Title: Draft CR to TS 38.101-1 on switching time between SUL and NUL |
|  | Return to |

Discussion on 2nd round (if applicable)

| T-doc number | Title | Comments |
| --- | --- | --- |

Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| T-doc number | Title | T-doc Status update recommendation |
| --- | --- | --- |

Topic #3: PC2 intra-band contiguous UL CA

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2100544 | Skyworks | Proposal 1 on HPUE architecture for class B/C contiguous UL CA: - A single TX with 200MHz PC2 PA (option1) is the baseline to develop MPR and A-MPR requirements - Two 100MHz PC2 PA (option 2) may reuse the same MPR values as already the case for PC3 but it should be verified why the worst case 2xPC3 PA 1RB+1RB MPR for non-contiguous UL CA is worse that the 1xPC3 PA related class C PA. - Additional requirement for two PC3 PAs architecture is FFS once single CC related requirements are finalized, potentially using a delta MPR compared to the option 1 baseline Proposal 2 on contiguous allocations PC2 class C UL CA MPR: - Contiguous allocation PC3 class C QPSK MPR is adopted for PC2 class C MPR - If specified, Class B PC2 MPR needs further evaluation Proposal 3 on contiguous allocations NS04 PC2 class C A-MPR: - NS04 A-MPR = MPR for outer class C PC2 - NS04 A-MPR = MPR+0.5dB for inner class C PC2 when RBstart ≤ 0.33*BWchannel_CA/0.18MHz - NS04 A-MPR = MPR for inner class C PC2 when RBstart > 0.33*BWchannel_CA/0.18MHz Proposal 4 on non-contiguous allocations MPR: - PC3 QPSK MPR is adopted for PC2 (1Tx) with additional back-off as in Table 6.2A.2.1-3 below (yellow highlight) Table 6.2A.2.1-3: non-contiguous RB allocation for Power Class 2 Modulation; MPR for bandwidth class B(dB); MPR for bandwidth class C(dB). inner; Outer11; Outer22; inner; Outer11; Outer22. DFT-s-OFDM; Pi/2 BPSK; 3; 6.5; 13; 3; 6.5; 13. QPSK; 3; 6.5; 3; 6.5. 16QAM; 3; 6.5; 3; 6.5. 64QAM; 4.5; 6.5; 5; 6.5. 256QAM; 6; 6.5; 6.5; 6.5. CP-OFDM; QPSK; 3; 6.5; 14; 3.5; 7; 14. 16QAM; 3; 7; 3.5; 7. 64QAM; 5; 7; 5; 7. 256QAM; 7.5; 7.5; 7.5; 7.5. NOTE 1: Outer 1 MPR for Pi/2 BPSK and QPSK is reduced by 2dB for aggregated allocation bandwidth > 10MHz NOTE 2: Outer 2 MPR is reduced by 4.5dB for aggregated allocation bandwidth > 10MHz. Proposal 5 on non-contiguous allocations NS04 A-MPR: - For channels and allocations where IM3 is within the -13dBm/MHz NS04 region, the PC2 MPR is sufficient - PC2 (1Tx) NS04 A-MPR for outer 1 and outer 2 with IM3 in -25dBm/MHz region is 15.5 for B<2.16 - All SEM limited allocations will see the back-off increase for PC2 vs PC3 but ACLR limited region will stay the same thus the following AMPR curve are proposed: AMPRIM3 to meet -25dBm/MHz MA = 15.5; 0 ≤ B < 2.16 14; 2.16 ≤ B < 3.24 13; 3.24 ≤ B < 5.04 11.5; 5.04 ≤ B < 10.08 10; 10.08 ≤ B < 16.56 8; 16.56 ≤ B < 21.96 6; 21.96 ≤ B |
| R4-2101103 | Xiaomi | Proposal 1: 2PA with PC3 shall be considered as reference for defining MPR requirements. Whether other set of MPR requirements are needed depends on the how much difference compared to 2PA with PC3 case. Proposal 2: the mechanism for single carrier for PC2 to meet SAR requirments can be reused for high power UE intra-band contiguous CA. |
| R4-2101160 | Qualcomm | Observation 1: Emissions are higher at FOOB edge of CA BW due to minimum CA guard band. Additional MPR is required due to added WOLA. Added MPR is only required for CA BW class B. Enough outer MPR for CA BW class C exists. Proposal 1: Increase MPR to 5.5dB for CA edge RB allocations for PC2 as shown in Table 2.1-1. The edge RB allocation is <= 2RBs. Proposal 2: Consolidate inner and outer 1 MPR for non-contiguous allocations for PC2 for CA BW class B. Increase inner and outer1 MPR for CA BW class C as shown in Table 2.2-1. Proposal 3: Use PC2 AMPR for CA_NS_04 for contiguous allocations as shown in table 2.3-1 when Fedge, low - BWChannel_CA < 2490.5 MHz. Proposal 4: For PC2, when Fedge, low - BWChannel_CA ≥ 2490.5 MHz, the AMPR = MPR for inner and outer 1 allocation, and AMPR= MPR – 1 for outer 2 allocations, else when Fedge, low - BWChannel_CA < 2490.5 MHz AMPR = A-MPRIM3 defined in Clause 6.2A.3.1.2.2.1 of 38.101-1, where AMPRIM3=MA, Where MA is defined as follows for both power class 3 and power class 2 MA = 13; 0 ≤ B < 2.16 11.5; 2.16 ≤ B < 3.24 10.5; 3.24 ≤ B < 5.04 9.5; 5.04 ≤ B < 10.08 8; 10.08 ≤ B < 16.56 7; 16.56 ≤ B < 21.96 6; 21.96 ≤ B |
| R4-2101755 | OPPO | Observation 1: Option2 (two 100MHz PC2 PAs) is better than Option3 (two 200MHz PC3 PA) from power consumption perspective without UL MIMO. Observation 2: Option2 (two 100MHz PC2 PAs) cannot achieve UL MIMO and UL CA simultaneously due to transceiver limitation. Proposal 1: From future proof perspective, propose to down select Option2 (two 100MHz PC2 PAs) to simplify the discussion. Proposal 2: Keep both Option1 (one 200MHz PC2 PA) and Option3 (two 200MHz PC3 PA) in this WI. Observation 3: “dualPA-Architecture” capability was defined in Rel-15 to distinguish UEs with one PA or two PA architecture, and maybe it can be reused here to distinguish Option1 and Option3 UE architectures. Proposal 3: Signaling like “dualPA-Architecture” capability can be considered to distinguish Option 1 and Option3 UE architectures and corresponding requirements. Observation 4: Optional features are defined and tested separately in RAN4/RAN5 and no combined requirements were defined in the past. Observation 5: UL MIMO+UL CA HPUE is out of this WI scope. Proposal 4: Do not define UL MIMO+UL CA HPUE requirements in this Rel-17 WI. |
| R4-2102184 | ZTE | Proposal 1. For 2PA architecture, the emission requirement is defined as the sum from both UE transmit antenna connectors. Proposal 2. No need to defined 2 sets of RF requirements. Proposal 3. A single TX PC2 PA 200MHz 1LO is the baseline to develop requirement. |
| R4-2102657 | Huawei | Observation 1: transparent TxD is enabled for UE configured with single SRS port(DCI0_0 or DCI0_1), and the emission requirements are defined based on the sum of the emissions from both UE transmit antenna connector. Observation 2: From current RAN1 and RAN2 spec, full power transmission is used for both single carrier and CA. It means transparent TxD is enabled for intra-band UL CA. Proposal 1: For intra-band contiguous UL CA declared with 2PA architecture, emission requirements should be defined based on the sum of the emissions from both UE transmit antenna connector. Proposal 2: Take “PC2 UL CA with one 26dBm PA 200MHz 1LO” as the reference RF architecture to define requirements, other architectures are not precluded. |

Open issues summary

Sub-topic 3-1 RF architecture

Issue 3-1-1: RF architecture baseline

- Proposals

  - Option 1:

    - A single TX PC2 PA is the baseline to develop MPR and A-MPR requirement;

    - Two 100MHz PC2 PA (option 2) may reuse the same MPR values as already the case for PC3 but it should be verified why the worst case 2xPC3 PA 1RB+1RB MPR for non-contiguous UL CA is worse that the 1xPC3 PA related class C PA

    - Additional requirement for two PC3 PAs architecture is FFS once single CC related requirements are finalized, potentially using a delta MPR compared to the option 1 baseline

  - Option 2:  Take “PC2 UL CA with one 26dBm PA 200MHz 1LO” as the reference RF architecture to define requirements, other architectures are not precluded. No need to defined 2 sets of RF requirements.

  - Option 3: 2PA with PC3 shall be considered as reference for defining MPR requirements. Whether other set of MPR requirements are needed depends on the how much difference compared to 2PA with PC3 case.

  - Option 4: down select Option2 (two 100MHz PC2 PAs) to simplify the discussion.

Keep both Option1 (one 200MHz PC2 PA) and Option3 (two 200MHz PC3 PA) in this WI.

- Recommended WF

  - TBA

Issue 3-1-2: signalling on RF architecture

- Proposals

  - Signaling like “dualPA-Architecture” capability can be considered to distinguish Option 1(one 200MHz PC2 PA) and Option3(two 200MHz PC3 PA)  UE architectures and corresponding requirements.

- Recommended WF

  - TBA

Sub-topic 3-2 RF requirements

Issue 3-2-1: MPR for contiguous allocation

- Proposals

  - Option 1: Contiguous allocation PC3 class C QPSK MPR is adopted for PC2 class C MPR; Class B PC2 MPR needs further evaluation(PC3 MPR requires and additional 1dB for PC2 inner and 0.5dB for outer for class B)

  - Option 2: Increase MPR to 5.5dB for CA edge RB allocations for PC2 as shown in Table 2.1-1. The edge RB allocation is <= 2RBs.

| Modulation |  | MPR for bandwidth class B(dB) |  |  | MPR for bandwidth class C(dB) |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | inner | outer | edge | inner | outer |
| DFT-s-OFDM | Pi/2 BPSK | 1.0 | 3.5 | [5.5] | 2.5 | 7 |
|  | QPSK | 1.0 | 3.5 | [5.5] | 2.5 | 7 |
|  | 16QAM | 1.5 | 3.5 | [5.5] | 2.5 | 7 |
|  | 64QAM | 3.0 | 4.0 | [5.5] | 5 | 7 |
|  | 256QAM | 5.5 | 6.0 |  | 7 | 7.5 |
| CP-OFDM | QPSK | 2.0 | 4.0 | [5.5] | 3.5 | 8 |
|  | 16QAM | 2.5 | 4.0 | [5.5] | 3.5 | 8 |
|  | 64QAM | 3.5 | 4.0 | [5.5] | 5 | 8 |
|  | 256QAM | 6.5 | 6.5 |  | 7 | 8 |

  - Option 3: R4-2102657 provides some initial simulation results: bandwidth class B, inner up to 2.6dB, outer up to 6.7dB; bandwidth class C, inner up to 4dB, outer up to 6.5dB.

- Recommended WF

  - TBA

Issue 3-2-2: MPR for non-contiguous allocation

- Proposals

  - Option 1:

PC3 QPSK MPR is adopted for PC2 (1Tx) with additional back-off as in Table 6.2A.2.1-3 below (yellow highlight)

Table 6.2A.2.1-3: non-contiguous RB allocation for Power Class 2

| Modulation |  | MPR for bandwidth class B(dB) |  |  | MPR for bandwidth class C(dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | inner | Outer11 | Outer22 | inner | Outer11 | Outer22 |
| DFT-s-OFDM | Pi/2 BPSK | 3 | 6.5 | 13 | 3 | 6.5 | 13 |
|  | QPSK | 3 | 6.5 |  | 3 | 6.5 |  |
|  | 16QAM | 3 | 6.5 |  | 3 | 6.5 |  |
|  | 64QAM | 4.5 | 6.5 |  | 5 | 6.5 |  |
|  | 256QAM | 6 | 6.5 |  | 6.5 | 6.5 |  |
| CP-OFDM | QPSK | 3 | 6.5 | 14 | 3.5 | 7 | 14 |
|  | 16QAM | 3 | 7 |  | 3.5 | 7 |  |
|  | 64QAM | 5 | 7 |  | 5 | 7 |  |
|  | 256QAM | 7.5 | 7.5 |  | 7.5 | 7.5 |  |
| NOTE 1: Outer 1 MPR for Pi/2 BPSK and QPSK is reduced by 2dB for aggregated allocation bandwidth > 10MHz NOTE 2: Outer 2 MPR is reduced by 4.5dB for aggregated allocation bandwidth > 10MHz |  |  |  |  |  |  |  |

  - Option 2:

Consolidate inner and outer 1 MPR for non-contiguous allocations for PC2 for CA BW class B. Increase inner and outer1 MPR for CA BW class C as shown in Table 2.2-1.

- Table 2.2-1: non-contiguous RB allocation for Power Class  2

| Modulation |  | MPR for bandwidth class B(dB) |  |  | MPR for bandwidth class C(dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | inner/ Outer11 | Outer22 | inner | Outer11 | Outer22 |
| DFT-s-OFDM | Pi/2 BPSK |  | 5.5 | 11.5 | 5.5 | 8.5 | 13 |
|  | QPSK |  | 5.5 |  | 5.5 |  |  |
|  | 16QAM |  | 5.5 |  | 5.5 |  |  |
|  | 64QAM |  | 6 |  | 5.5 |  |  |
|  | 256QAM |  | 6.5 |  | 6.5 |  |  |
| CP-OFDM | QPSK |  | 6.5 | 12 | 5.5 | 8.5 | 14 |
|  | 16QAM |  | 7 |  | 5.5 |  |  |
|  | 64QAM |  | 7 |  | 5.5 |  |  |
|  | 256QAM |  | 7.5 |  | 7.5 |  |  |
| NOTE 1: Outer 1 MPR for Pi/2 BPSK and QPSK is reduced by 2dB for aggregated allocation bandwidth > 10MHz NOTE 2: Outer 2 MPR is reduced by 4.5dB for aggregated allocation bandwidth > 10MHz |  |  |  |  |  |  |  |

  - Option 3: R4-2102657 provides some initial simulation results: for bandwidth class C, outer 1 up to 7.5dB, outer 2 up to 16.7dB.

- Recommended WF

  - TBA

Issue 3-2-3: AMPR for NS_04 contiguous allocation

- Proposals

  - Option 1:

    - NS04 A-MPR = MPR for outer class C PC2

    - NS04 A-MPR = MPR+0.5dB for inner class C PC2 when RBstart ≤ 0.33*BWchannel_CA/0.18MHz

    - NS04 A-MPR = MPR for inner class C PC2 when RBstart > 0.33*BWchannel_CA/0.18MHz

  - Option 2: Use PC2 AMPR for CA_NS_04 for contiguous allocations as shown in table 2.3-1 when Fedge, low - BWChannel_CA < 2490.5 MHz.

CA_NS_04 Contiguous RB allocation for Power Class 2

| Modulation |  | AMPR for bandwidth class B(dB) |  |  | AMPR for bandwidth class C(dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | inner | inner1 | outer | inner | inner1 | outer |
| DFT-s-OFDM | Pi/2 BPSK | 1.0 | 5.5 | 5 | 2.5 | 5.5 | 7 |
|  | QPSK | 1.0 | 6 | 5.5 | 2.5 | 6 | 7 |
|  | 16QAM | 1.5 | 6 | 6 | 2.5 | 6 | 7 |
|  | 64QAM | 3.0 | 6.5 | 6 | 5 | 6.5 | 7 |
|  | 256QAM | 5.5 | 8 | [7.5] | 7 | 8 | 7.5 |
| CP-OFDM | QPSK | 2.0 | 7.5 | 6.5 | 3.5 | 7.5 | 8 |
|  | 16QAM | 2.5 | 7.5 | 6.5 | 3.5 | 7.5 | 8 |
|  | 64QAM | 3.5 | 7.5 | 6.5 | 5 | 7.5 | 8 |
|  | 256QAM | 6.5 | 10 | [8] | 7 | 10 | 8 |
| Note 1: RBstart ≤ 0.33*BWchannel_CA/0.18MHz, LCRB ≤ 4MHz/0.18MHz |  |  |  |  |  |  |  |

- Recommended WF

  - TBA

Issue 3-2-4: AMPR for NS_04 non-contiguous allocation

- Proposals

  - Option 1:

- For channels and allocations where IM3 is within the -13dBm/MHz NS04 region, the PC2 MPR is sufficient

- PC2 (1Tx) NS04 A-MPR for outer 1 and outer 2 with IM3 in -25dBm/MHz region is 15.5 for B<2.16

- All SEM limited allocations will see the back-off increase for PC2 vs PC3 but ACLR limited region will stay the same thus the following AMPR curve are proposed: AMPRIM3 to meet -25dBm/MHz

MA = 		15.5; 	0 ≤ B < 2.16

14; 	2.16 ≤ B < 3.24

13;       3.24 ≤ B < 5.04

11.5; 	5.04 ≤ B < 10.08

10; 	10.08 ≤ B < 16.56

8;        16.56 ≤ B < 21.96

6; 	      21.96 ≤ B

  - Option 2:  For PC2, when Fedge, low - BWChannel_CA ≥ 2490.5 MHz, the AMPR = MPR for inner and outer 1 allocation, and AMPR= MPR – 1 for outer 2 allocations, else when Fedge, low - BWChannel_CA < 2490.5 MHz AMPR = A-MPRIM3 defined in Clause 6.2A.3.1.2.2.1 of 38.101-1, where AMPRIM3=MA, Where MA is defined as follows for both power class 3 and power class 2

MA = 	13; 	0 ≤ B < 2.16

11.5; 	2.16 ≤ B < 3.24

10.5;       3.24 ≤ B < 5.04

9.5; 	5.04 ≤ B < 10.08

8; 	10.08 ≤ B < 16.56

7;   16.56 ≤ B < 21.96

6; 	 21.96 ≤ B

- Recommended WF

  - TBA

Issue 3-2-5: emission requirements

- Proposals

  - For 2PA architecture, the emission requirement is defined as the sum from both UE transmit antenna connectors.

- Recommended WF

  - TBA

Issue 3-2-6: Whether CA UL MIMO requirement is needed?

- Proposals

  - Do not define UL MIMO+UL CA HPUE requirements in this Rel-17 WI

- Recommended WF

  - TBA

Issue 3-2-7: SAR Solution for TDD contiguous UL CA

- Proposals

- The mechanism for single carrier for PC2 to meet SAR requirements can be reused for high power UE intra-band contiguous CA.

Moderator note: in WF R4-2017827, already have agreements

  - Pcmax:  mechanism as for single carrier

  - Capability of MaxUplinkDutyCyUse the same power class fallback cle: Reuse the capability for single carrier case

Is there additional mechanism for SAR requirement?

- Recommended WF

  - TBA

Companies views’ collection for 1st round

Open issues

| Sub-topic | Comments (Company: …) |
| --- | --- |
| 3-1 | Issue 3-1-1 ZTE: Both Option 1 and Option 2. For the RF architecture, using a single TX PC2 PA 200MHz 1LO to develop requirements. It seems option 1 has the same meaning. For RF requirements except MPR/A-MPR, it is no need to define two sets of RF requirements such as SEM, SE.... For the MPR/A-MPR requirement, probably also no needs to define 2 sets of values but it is FFS. Xiaomi: We have no strong view on which RF architecture is baseline. But we think both single TX PC2 PA and two Tx PC3 shall be considered. Skyworks: both options 1 and 2 use single PC2 PA as the baseline option which is our preference. From our PC3 work we know it automatically covers the 2xPC2 PAs with 100MHz each so do not need specific requirement for this (but signaling may need to distinguish between 2xPC2 100MHz and 2xPC3 200MHz (but related to TxDiv discussion). We are not completely against 2xPC3 option but it is not consistent with PC1.5 being available in both n41 and n77/78. RAN4 should try to harmonize architecture across different cases. We need to clarify if 2xPC3 then has a different MPR table, signaling… Nokia: We believe that a single Tx PC2 PA with 200MHz is the most appropriate choice. Because UEs supporting these CAs as PC2 (high end devices) would also support UL MIMO. LGE: In option1. Why RAN4 need to consider two PC2 PA? we think baseline RF architecture is single PC2 PA and other RF architecture such as two PC3 PA can be considered. RAN4 need to exclude two PC2 PA RF architecture in here. So LGE prefer option2 for intra-band contiguous CA as baseline. Qualcomm: Agree with option 1 and option 2. For option 3, Delta MPR could be added if necessary, for 2 PC3 PAs. Single set of requirements regardless of architecture should be the goal for simplicity. OPPO: Prefer Option 4 if considering this UE might also support UL MIMO case, which is more future proof. And both 1PA 200MHz PC2 and 2 PC3 PA with 200MHz each can be used as reference architecture. Huawei: We support option 1. It means we use 1PA 26dBm 200MHz architecture to derive the MPR/AMPR requirement. For option 2, it is generally similar as option 1, the problem is: if we finally find MPR value for other architecture is higher, can we define additional requirement for other architecture? We think it should be allowed. Vivo：From flexibility point of view option 1 and option 2 is preferred. However, 2 PC3 PAs architecture may also beneficial and it is proposed to consider the requirements applicability to be included. Apple: Option 1 and Option 2 using single PC2 PA as the reference architecture to define only one set of RF requirements. Other architectures are UE’s implementation choice which still need to meet the same requirements. The testability and signaling aspects need to be considered when using dual PA which either split carriers or split power. For example, using two 100MHz PC2 PA, how composite ACLR can be verified in conductive tests? |
|  | Issue 3-1-2 ZTE: Agree with the proposal. Xiaomi: Suppprt the proposal Skyworks: this depends if we have two tyoes of two PA architecture and also depends on signaling decided for TxDiv. Too early to decide Nokia: It is better to focus on completing the 3-1-1 and establishing requirements. Then, we explain RAN2 how these requirements are interpreted to make them find out suitable signaling structures. DualPA-architecture singling itself originally just tells network if MPR is bigger(infinite) or smaller as far as we understand. LGE: We agree with Skyworks and Nokia since RAN4 do not need to define 2 set RF requirements. RAN4 can further discuss how to apply the RF requirements according to different RF architecture. Qualcomm: This will depend if actual performance is distinguishable between 1PA and 2PA type architectures. Signalling should be introduced if it is worth the effort to define the 2 sets of requirements. Agree with Nokia’s comment. OPPO: Agree with the proposal. Currently there are already signaling to differentiate UE architectures and requirements Huawei: seems the signaling is already there. But we agree it is too early to discuss on this. Vivo: support Qualcomm’s explanation and simplification is preferred while keeping the flexibility. Apple: Agree with Skyworks. |
| 3-2 | Issue 3-2-1 Skyworks: we are OK to discuss the addition of edge RBs like for PC2 single CC. LGE: It is premature to decide the detail MPR values. Currently we propose need more simulation results for PC2 intra-band contiguous CA both class B and class C. If RAN4 make consensus on the initial MSD values, then recoemmend to add [] in MPR values. Qualcomm: To LGE, we are fine with square brackets initially to the consensus of the 3 contributions so far and welcome other company contributions. Huawei: Some questions for Qualcomm: If PA is biased assumed with PC2 operation, whether PC3 can get a better linearity compared with PC2? Or worse? For CA, considering the RB is squeezed into the center, larger guard band can be expected, the distance to CBW edge is farther compared with single CC? Why CA guard band is reduced in your paper? Why added wola lead to higher MPR? Wola is supposed to optimize the OOBE? Why edge allocation is not needed for BW class C? We propose to align some values with bracket in 2nd round. Apple: Is Option 1 based on single PC2 PA? |
|  | Issue 3-2-2 Skyworks: we are ok to discuss compromise between the different input but we are not OK with merging inner and outer 1 as we still see a significant difference. We may amend AMPR values or allocation definition slightly if needed. LGE: we have same comment in issue 3-2-1 for non-contiguous RB allocation as follow It is premature to decide the detail MPR values. Currently we propose need more simulation results for PC2 intra-band contiguous CA both class B and class C for non-contiguous RB allocation. If RAN4 make consensus on the initial MSD values, then recoemmend to add [] in MPR values. Qualcomm: To LGE: same comment as 3-2-1. To Skyworks: We can try to bring more analysis to prove rise of inner MPR. Huawei: to Qualcomm, could you show us the architecture you assume for the MPR values? We also find that MPR of PC3 inner case is not enough for PC2, at least we agree to add more MPR for NC allocation inner part for PC2. For outer 1, we also see higher MPR for DFT-s-OFDM compared with PC3. Can we agree on: Increase inner and outer1 MPR for CA BW class B and C for QPSK? Delta value is FFS. |
|  | Issue 3-2-3 Skyworks: we can work with other contributors to coverge on NS04 values in round 2 LGE: It is premature to decide the detail MPR values. Currently we propose need more simulation results for PC2 intra-band contiguous CA both class B and class C for non-contiguous RB allocation. If RAN4 make consensus on the initial MSD values, then recoemmend to add [] in A-MPR values. Qualcomm: To LGE, we are fine with square brackets initially to the consensus of the 3 contributions so far and welcome other company contributions. Huawei: we don’t provide evaluation on AMPR in this meeting. In the current spec, AMPR value is highly related to MPR value, do we follow this approach for PC2? If yes, do we need to decide on MPR firstly? |
|  | Issue 3-2-4 Skyworks: we can work with other contributors to coverge on NS04 values in round 2 LGE: It is premature to decide the detail MPR values. Currently we propose need more simulation results for PC2 intra-band contiguous CA both class B and class C for non-contiguous RB allocation. If RAN4 make consensus on the initial MSD values, then recoemmend to add [] in A-MPR values. Qualcomm: To LGE, we are fine with square brackets initially to the consensus of the 3 contributions so far and welcome other company contributions. Huawei: we don’t provide evaluation on AMPR in this meeting. In the current spec, AMPR value is highly related to MPR value, do we follow this approach for PC2? If yes, do we need to decide on MPR firstly? |
|  | Issue 3-2-5 ZTE: Agree with the proposal. Xiaomi: Agree with the proposal. The sum emissions should comply with regulation requirements. Skyworks: if 2 antenna PAs are used the TxDiv and MIMO agreements on sum of emissions stands. LGE: Acceptable the proposal for 2PA/2Tx RF architecture. Qualcomm: Wording is always confusing here. The sum of the emissions at the 2 antenna ports must meet the general SEM, additional SEM, general spurious and additional spurious requirements. So, we seemed to be aligned. OPPO: Agree with the proposal. Huawei: we agree. To Qualcomm, the wording follows UL MIMO spec. Vivo: Agree Apple: Agree if 2PA is to split the power, but not to split the carriers. |
|  | Issue 3-2-6 ZTE:Agree with the proposal. China Telecom: To our understanding, not only UL MIMO+UL CA HPUE requirements, but also UL MIMO+UL CA PC3 requirements have not been specified yet. We intend to agree with OPPO that based on the current descriptions in the Rel-17 FR1 WID, UL MIMO+UL CA PC2/3 is not explicitly included in the WID, and we may need to modify the WID descriptions to include UL MIMO+UL CA PC2/3 requirements. Xiaomi: Agree with the proposal. Do not define UL MIMO+UL CA HPUE requirements in this Rel-17 WI Skyworks: Whether or not UL MIMO + UL CA is part of R17 is not really relevant: if one PC2 PA is used then UL MIMO is covered easily with an additional PC2 PA. if two PC3 PA are used then TxDiv is needed by default thus UL MIMO should be feasible. LGE: agreed with the proposal not to simultaneous support both UL-MIMO and CA. Qualcomm: If one PA architecture is used, UL-MIMO is achievable. This was mentioned in a few contributions. Whether or not to include this feature in release 17 depends on PC3 completion. OPPO: Agree with the proposal, even we consider UL MIMO for future proof but UL MIMO + UL CA is not part of the Rel-17 WI. And it should be considered separately. Huawei: currently TS 38.101-1 is missing CA UL MIMO RF requirement. We think it could be start from PC3, but PC2 can be also included. In our understanding, CA UL MIMO requirement may reuse most RF requirements of CA, but we need to make it clear whether a UE indicate support UL MIMO under intra-band CA can meet the RF requirements. Vivo: Agree Apple: Agree with Skyworks. |
|  | Issue 3-2-7: ZTE:Agree with the proposal. It was already agreed in last WF. Xiaomi: Agree with the proposal. It can be ignored since it was agreed in the last meeting. Skyworks: agree LGE: agree with the proposal. It was already agreed in last WF. Apple: Agree with the proposal. |

CRs/TPs comments collection

Major close to finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

| Sub-topic#3 | Status summary |
| --- | --- |
| 3-1 | Issue 3-1-1: - Option 1 or Option 2 with one PC2 PA as reference RF architecture: 8 - Both one 200MHz PC2 PA and 2 200MHz PC3 PA architectures should be considered: 2 - two 100MHz PC2 Pas, but also keep one 200MHz PC2 PA and two 200MHz PC3 PA in the WI: 1 Potential agreement: 1. Take one 200MHz PC2 PA/1LO as the reference RF architecture to develop RF requirements, other RF architectures are not precluded for implementation. 2. Delta MPR/AMPR requirements for two PC3 PAs architecture can be defined if necessary Issue 3-1-2: postpone the discussion, focus on establishing requirements currently |
| 3-2 | Issue 3-2-1: Further discussion on: - Do we have Edge RB allocation for PC2 contiguous CA? - If yes, it is only for Bandwidth class B or both bandwidth classes? - Contiguous allocation PC3 class C QPSK MPR is adopted for PC2 class C MPR? - Bandwidth class B need increase MPR for inner and outer? Issue 3-2-2: Further discussion on: - Merge inner and outer 1? - Inner MPR rise? - Outer1 MPR rise? - Outer 2 MPR: - For BW class B, MPR rise? - For BW class C, MPR rise? |
|  | Issue 3-2-3: Further discussion on: - AMPR value is highly related to MPR value, do we follow this approach for PC2? - For BW class C outer, AMPR= MPR? - For BW class B outer, AMPR=MPR? - when RBstart ≤ 0.33*BWchannel_CA/0.18MHz, Inner AMPR=MPR+0.5dB? ortherwise, AMPR=MPR? Encourage companies work together on value alignment, and bracket need to be added. Issue 3-2-4: Further discussion on: - AMPR value is highly related to MPR value, do we follow this approach for PC2? - For IM3 is within the -13dBm/MHz NS04 region, the PC2 MPR is sufficient? Seems aligns between 2 companies. - For AMPRIM3 to meet -25dBm/MHz, All SEM limited allocations will see the back-off increase for PC2 vs PC3 but ACLR limited region will stay the same? Encourage companies work together on value alignment, and bracket need to be added. |
|  | Issue 3-2-5: Potential agreement: For 2PA architecture, the emission requirement is defined as the sum from both UE transmit antenna connectors. Issue 3-2-6: further discuss on following options - Modify the WID descriptions to include UL MIMO+UL CA PC2/3 requirements. - Do not define UL MIMO+UL CA HPUE requirements in this Rel-17 WI Issue 3-2-7: this topic can be closed since already agreed in the last meeting. |

Suggestion on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| 1 | WF on RF architecture and requirements for intra-band UL NC CA | Huawei, HiSilicon |
| 2 | WF on MPR and AMPR for intra-band UL NC CA | Skyworks |

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |

Discussion on 2nd round (if applicable)

| T-doc number | Title | Comments |
| --- | --- | --- |

Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| T-doc number | Title | T-doc Status update recommendation |
| --- | --- | --- |

Topic #4: intra-band NC DL CA

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2102284 | Skyworks | Observation1: - The support of n77(3A) in Japan requires a UE architecture with 3 receive LO and 3 RF paths per DL antennas - The same 3 LO architecture covers n77(3A) in the US with reduced total frequency span - FFS if n77(4A) could also be supported with 3LO once operator spectrum allocation in the U.S. is understood and channel configuration and aggregated bandwidth adjusted accordingly (i.e. maximum 300 MHz?) Observation2: If this mandate is still valid for n77(3A) and n77(4A) it means that for single band support: - 3 LO and potentially 4 LO are needed - 12 and potentially up to 16 receive paths are needed - The request being for 3x100MHz and 4x100 MHz the total baseband bandwidth is up to 1.6GHz - There may be limited support for higher order inter-band combinations as a lot of RF and baseband resources are already in use for n77 - There is low probability that good 4x4 MIMO channel is available for the 3 or 4 CCs simultaneously Proposal: To enable larger ecosystem support for n77(3A/4A) combinations and/or additional bands in related inter-band DL CA: - 4x4 DL MIMO stays mandatory for n77 CA with 2CCs and all related higher order inter-band DL CA combinations - 4x4 DL MIMO is optional support with capability signaling for n77(3A) and n77(4A) DL CA (>2CC) and all related higher order inter-band DL CA combinations - Related per band and per band combinations capability signaling and RAN4 specification update is needed |
| R4-2100707 moderator Note: move to Agenda 12.2.1 | SoftBank | Proposal 1: CA_n77(3A) is release independence from Rel-16. Observation 1: The maximum throughput of CA_n77(3A) is lower than that of CA_n77(2A) if Option 3 is applied. Proposal 2: Option 1 is selected. - Option 1: 4x4 DL MIMO is mandatory support for n77(3A) DL CA and all related higher order inter-band DL CA combinations: - No signaling and specification update needed - Limited support for lower end phones but also potential limitation on how many additional bands are supported in inter-band DL CA - Option 3: 4x4 DL MIMO is optional support for n77(3A) DL CA and all related higher order inter-band DL CA combinations: - Signaling and specification update needed - Larger support for n77(3A)combination and/or additional bands in related inter-band DL CA. |
| R4-2100708 moderator Note: move to Agenda 12.2.1 | SoftBank | TP for TR 38.717-01-01: CA_3DL_n77(3A)_1UL_n77A |

Open issues summary

Sub-topic 4-1 n77(3A) and n77(4A) DL CA

Issue 4-1-1: Release independent definition for intra-band n77(3A) DL CA

- Proposals

  - Option 1: Rel-16

  - Option 2: Rel-17

- Recommended WF

  - TBA

Issue 4-1-2: 4*4 DL MIMO for n77(3A)

- Proposals

  - Option 1: 4x4 DL MIMO is optional support with capability signaling for n77(3A) DL CA and all related higher order inter-band DL CA combinations

  - Option 2: 4x4 DL MIMO is mandatory support for n77(3A) DL CA and all related higher order inter-band DL CA combinations

- Recommended WF

  - TBA

Issue 4-1-3: 4*4 DL MIMO for n77(4A)

- Proposals

  - 4x4 DL MIMO is optional support with capability signaling for n77(4A) DL CA and all related higher order inter-band DL CA combinations

- Recommended WF

  - TBA

Companies views’ collection for 1st round

Open issues

| Sub-topic | Comments (Company: …) |
| --- | --- |
| 4-1 | Issue 4-1-1 ZTE: Ok with Option 1. SoftBank: Option 1 is preferrable considering our deployment plan. Skyworks: R17 was agreed in last meeting way forward, it needs wider company support to change the agreement. To our knowledge R16 UEs do not support the 3 LO for this anyhow. We will need input from more UE/chipset companies. Qualcomm: Option 2. We support 3 DL CCs and 4x4 DL MIMO from release 17. OPPO: Option 2 is more feasible. Huawei: we believe it depends on operator’s request. Vivo: Option 2 Apple: Option 2 |
|  | Issue 4-1-2 SoftBank: Support Option 2. We understand the difficulty of supporting 4x4 MIMO to the multiple non-contiguous carriers considering the UE implementation. But if Option 1 is applied, the maximum throughput of CA_n77(3A) will be lower than that of CA_n77(2A) since CA_n77(3A) supports only 6 layers as a baseline even though CA_n77(2A) shall support 8 layers. It reduces the advantage of introducing CA_n77(3A) to the network. Skyworks: we think option 1 will provide a wider ecosystem and does not preclude 4x4 MIMO support by a large set of devices. With related signaling operators will be able to configure UEs in the best way from their capabilities Qualcomm: Option 1. OPPO: Option 1 is more feasible. Huawei: we support option 2. For inter-band CA with 4R Band, 4MIMO is mandatory to support, we don’t want to change the decision after 2 release generation. Vivo: option 1 Apple: Option 1 is preferred. |
|  | Issue 4-1-3 Skyworks: as discussed in our paper it is difficult to evaluate the number of LO and BW needed with the current status of FCC outcome but it would be consistent to use the same approach for n77(3A0 and n77(4A) Qualcomm: We need more time to check 4CCs and 4x4 DL MIMO for release 17. OPPO: Agree with the proposal. Huawei: We don’t support change 4MIMO mandatory agreement made in Rel-15. However, it is too early to conclude before FCC outcome. |

CRs/TPs comments collection

Major close to finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2100708 | Title: TP for TR 38.717-01-01: CA_3DL_n77(3A)_1UL_n77A |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

| Sub-topic#4 | Status summary |
| --- | --- |
| 4-1 | Issue 4-1-1: Release independent definition for intra-band n77(3A) DL CA Further discuss on whether change agreement to Rel-16? Issue 4-1-2: Option1: 5 Option2: 2 Further discuss on 4*4 DL MIMO for n77(3A) Issue 4-1-3: It depends on FCC outcome and approach of n77(3A) Recommend to focus on n77(3A) first |

Suggestion on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| 1 | WF on 4Rx requirement for CA_n77(3A) and CA_77(4A) | SoftBank |

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2100708 | To be agreed |

Discussion on 2nd round (if applicable)

| T-doc number | Title | Comments |
| --- | --- | --- |

Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| T-doc number | Title | T-doc Status update recommendation |
| --- | --- | --- |