3GPP TSG-RAN WG4 Meeting #98-e	R4-2102350

# Electronic Meeting, January 25 – February 05, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.133 | CR | 7040 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR 36.133 (A.8.16.106) Correction to test case for direct scell activation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_euCA-Perf |  |  |  |  |  | Date: |  |  | 2021-02-04 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Measurement gap configuration during time period T1 is missing for UEs that do not support inter-frequency measurements without measurement gaps. SCell is not configured until T2, hence during T1 the SCell to-be is just an inter-frequency neighbour cell. Header of Table A.8.16.106.1-2 is incorrect. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introducing the following: - Adding measurement gap pattern with ID #0 during T1 - Correcting header for Table A.8.16.106.1-2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Functionality pertaing to LTE Direct SCell activation cannot be verified for UEs that do not support inter-frequency measurements without gaps. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.8.16.106 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR 36.521-3 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Unchanged Sections Omitted

First Modification

### A.8.16.106	Direct Activation of Known SCell in Non-DRX with generic duplex modes

#### A.8.16.106.1	Test Purpose and Environment

The purpose of this test is to verify that the direct SCell activation delays are within the requirements stated in clause 7.7.18 for UE configured with a single downlink SCell, when the SCell is known by the UE at the time of direct activation.

The test parameters are given in Tables A.8.16.106.1-1 and cell-specific parameters in A.8.16.106.1-2 below. The test consists of three successive time periods, with duration of T1, T2 and T3, respectively. There are two carriers, each with one cell. Cell 1 and Cell 2 operate in either FDD or TDD duplex mode according to test configuration. Both cells have constant signal levels throughout the test. Before the test starts the UE is connected to Cell 1 (PCell) on radio channel 1 (PCC) but is not aware of Cell 2 (SCell1) on radio channel 2 (SCC1). The UE is only monitoring the PCC. The UE shall be continuously scheduled in the PCell throughout the whole test.

At the beginning of T1, the UE is configured to measure radio channel 2 and starts detecting the Cell 2 (SCell1) on radio channel 2 (SCC1). Measurement gap pattern with ID #0 as specified in Table 8.1.2.1-1 is configured to enable inter-frequency monitoring on SCC1. During T1 Cell 2 is detected and measured and measurement report is sent by the UE to the test equipment. Thereafter measurement gaps are deconfigured by the test equipment. Test equipment sends the RRCConnectionReconfiguration message for the activation of the SCell within Nact_known to ensure the configured SCell is known.

Time period T2 starts when UE receives the RRCConnectionReconfiguration message for the activation of the SCell, sent from the test equipment to the UE received in a subframe # denoted m at the UE antenna connector.

The E-UTRAN shall use RRCConnectionReconfiguration message with parameter sCellState set to activated for the SCell1 (Cell 2), which causes the SCell1 to become configured and activated on radio channel 2 (SCC1). The message is sent from the test equipment to the UE and is received in a subframe # denoted m at the UE antenna connector. The UE shall accomplish the activation of the SCell no later than subframe (m+ Ndirect).

Time period T3 starts at (m+ Ndirect), at which point UE shall be reporting a valid CQI for both PCell and SCell1.

During T3, the UE shall be continuously scheduled in the SCell1.

The test equipment verifies the activation time by counting the subframes from the time when the direct SCell activation is sent and until a CSI report with other than CQI index 0 is received.

The test equipment verifies the CSI report from the direct activated SCell after the activation procedure is completed contains CQI index other than 0.

Table A.8.16.106.1-1: General test parameters for known SCell direct activation case

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1, 2 | Two radio channels are used for this test |
| Active PCell |  | Cell 1 | Primary cell on RF channel number 1. |
| Deconfigured deactivated SCell |  | Cell 2 | Deconfigured deactivated secondary cell on RF channel number 2. |
| CP length |  | Normal |  |
| Gap Pattern Id |  | 0 | As specified in clause 8.1.2.1. |
| DRX |  | OFF | Continuous monitoring of primary cell |
| CQI/PMI periodicity and offset configuration index |  | 0 |  |
| Cell-individual offset for cells on RF channel number 1 | dB | 0 | Individual offset for cells on PCC. |
| Cell-individual offset for cells on RF channel number 2 | dB | 0 | Individual offset for cells on SCC1. |
| SCell measurement cycle (measCycleSCell) | ms | 320 |  |
| T1 | s | 1 | During this time the PCell shall be known and the SCell1 is detected, and UE shall report a valid CQI for PCell. |
| T2 | s | Ndirect | During this time the UE shall be configured with directly activated SCell1. |
| T3 | s | 1 | During this time the UE shall report a valid CQI for both PCell and SCell1. |

Table A.8.16.106.1-2: Cell specific test parameters for known SCell direct activation case

| Parameter | Unit |  | Cell 1 |  |  |  | Cell 2 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T3 |  | T1 | T2 | T3 |
| E-UTRA RF Channel Number |  |  | 1 |  |  |  | 2 |  |  |  |
| Duplex mode |  |  | FDD or TDD |  |  |  | FDD or TDD |  |  |  |
| TDD special subframe configuration |  |  | 6 |  |  |  | 6 |  |  |  |
| TDD uplink-downlink configuration |  |  | 1 |  |  |  | 1 |  |  |  |
| BWchannel | MHz |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |  |
| PDSCH parameters: DL Reference Measurement Channel |  |  | 5MHz: R.7 FDD 10MHz: R.3 FDD 20MHz: R.6 FDD 5MHz: R.4 TDD 10MHz: R.0 TDD 20MHz: R.3 TDD |  |  |  | 5MHz: R.7 FDD 10MHz: R.3 FDD 20MHz: R.6 FDD 5MHz: R.4 TDD 10MHz: R.0 TDD 20MHz: R.3 TDD |  |  |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  |  |
| OCNG Patterns |  |  | 5MHz: OP.20 FDD 10MHz: OP.10 FDD 20MHz: OP.17 FDD 5MHz: OP.9 TDD 10MHz: OP.1 TDD 20MHz: OP.7 TDD |  |  |  | 5MHz: OP.16 FDD 10MHz: OP.2 FDD 20MHz: OP.12 FDD 5MHz: OP.10 TDD 10MHz: OP.2 TDD 20MHz: OP.8 TDD |  |  |  |
| PBCH_RA | dB |  | 0 |  |  |  | 0 |  |  |  |
| PBCH_RB | dB |  |  |  |  |  |  |  |  |  |
| PSS_RA | dB |  |  |  |  |  |  |  |  |  |
| SSS_RA | dB |  |  |  |  |  |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |  |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |  |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |  |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |  |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |  |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |  |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |  |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |  |  |  |  |  |
| NocNote 2 | dBm/15 kHz |  | -104 |  |  |  | -104 |  |  |  |
| Ês/Noc | dB |  | 17 |  |  |  | 17 |  |  |  |
| Ês/Iot | dB |  | 17 |  |  |  | 17 |  |  |  |
| RSRP Note 3 | dBm/15 kHz |  | -87 |  |  |  | -87 |  |  |  |
| SCH_RP Note 3 | dBm/15 kHz |  | -87 |  |  |  | -87 |  |  |  |
| Io Note 3 | dBm/Ch BW |  | -59.13 +10log (NRB,c /50) |  |  |  | -59.13 +10log (NRB,c /50) |  |  |  |
| Propagation Condition |  |  | AWGN |  |  |  | AWGN |  |  |  |
| Antenna Configuration |  |  | 1x2 |  |  |  | 1x2 |  |  |  |
| Timing offset to Cell 1 | s |  | - |  |  |  | 0 |  |  |  |
| Time alignment error relative to cell 1 Note 5 | s |  | - |  |  |  | ≤ TAE |  |  |  |
| Note 1: OCNG shall be used such that all cells are fully allocated, and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Es/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T2. Note 5: Time alignment error (TAE) as specified in TS 36.104 [30] clause 6.5.3.1. The TAE value depends upon the type of carrier aggregation. |  |  |  |  |  |  |  |

#### A.8.16.106.2	Test Requirements

The UE shall accomplish the activation of the SCell no later than subframe m+Ndirect.

Time period T3 starts at (m+ Ndirect), at which point UE shall be reporting a valid CQI for both PCell and SCell1.

NOTE:	The direct SCell activation delay, Ndirect, can be expressed as: Ndirect = TRRC_Process [+T1] + Ttime_direct, where:

TRRC_Process is the RRC procedure delay = 20 ms which is the RRC procedure delay defined for SCell addition in clause 11.2 of TS 36.331 [2],

T1 is the delay from subframe m+ TRRC_Process until the transmission of RRCConnectionReconfigurationComplete message

Ttime_direct is the direct SCell activation delay. If the SCell is known, then Ttime_direct is 20 ms. If the SCell is unknown, then Ttime_direct is 30 ms provided the SCell can be successfully detected on the first attempt.

This gives a total of Ndirect = 20 + T1 + 20 ms = 40 + T1 ms for FDD and TDD.

During T3 the UE shall send CSI reports for SCell1 with non-zero CQI index and continue to send CSI reports for SCell 1 with non-zero CQI index until the end of T3. All of the above test requirements shall be fulfilled in order for the observed SCell1 direct activation delay to be counted as correct. The rate of correct observed SCell1 direct activation delay during repeated tests shall be at least 90%.

End of FIRST Modification

Unchanged Sections Omitted