| 3GPP TR 37.717-00-00 V0.3.0 (2021-02) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Networks; Band combinations for SA NR Supplementary uplink (SUL), NSA NR SUL, NSA NR SUL with UL sharing from the UE perspective (ULSUP) (Release 17) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2020, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

1	Scope	7

2	References	7

3	Definitions of terms, symbols and abbreviations	8

3.1	Terms	8

3.2	Symbols	8

3.3	Abbreviations	8

4	Background	8

4.1	TR Maintenance	9

5	SA NR SUL band combination: Specific Band Combination Part	9

5.1	SUL_n41A-n83A	9

5.1.1	Operating bands	9

5.1.2	Channel bandwidths per operating band	10

5.1.3	Maximum output power	11

5.1.4	Spurious emission band UE co-existence	11

5.1.5	MSD	11

5.1.6	∆TIB and ∆RIB values	11

5.2	SUL_n79A-n83A	11

5.2.1	Operating bands	11

5.2.2	Channel bandwidths per operating band	13

5.2.3	Maximum output power	14

5.2.4	Spurious emission band UE co-existence	14

5.2.5	MSD	14

5.2.6	∆TIB and ∆RIB values	14

5.2.7	Out-of-band blocking exceptions	14

5.Y	SUL_nX-nY/CA_nX_SUL_nY-nZ	53

5.Y.1	Operating bands	53

5.Y.2	Configuration	53

5.Y.3	Maximum output power	53

5.Y.4	Spurious emission band UE co-existence	53

5.Y.5	REFSENS requirements	53

5.Y.6	∆TIB and ∆RIB values	53

5.Y.7	Out-of-band blocking exception	53

6	NSA NR SUL band combination: Specific Band Combination Part	53

6.Y	DC_X_SUL_nY-nZ	53

6.Y.1	Operating bands	53

6.Y.2	Configuration	53

6.Y.3	Maximum output power	54

6.Y.4	Spurious emission band UE co-existence	54

6.Y.5	REFSENS requirements	54

6.Y.6	∆TIB and ∆RIB values	54

7	NSA NR SUL with UL sharing from ULSUP band combination: Specific Band Combination Part	54

7.1	DC_28_SUL_n41-n83	54

7.1.1	Operating bands	54

7.1.2	Configuration	54

7.1.3	Maximum output power	54

7.1.4	Spurious emission band UE co-existence	55

7.1.5	MSD	56

7.1.6	∆TIB and ∆RIB values	56

7.Y	DC_X_SUL_nY-nZ	56

7.Y.1	Operating bands	56

7.Y.2	Configuration	56

7.Y.3	Maximum output power	56

7.Y.4	Spurious emission band UE co-existence	56

7.Y.5	REFSENS requirements	56

7.Y.6	∆TIB and ∆RIB values	57

Annex A (informative): Change history	58

Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

1	Scope

The present document is a technical report for SA NR Supplementary uplink (SUL), NSA NR SUL and NSA NR SUL with UL sharing from the UE perspective (ULSUP) under Rel-17 time frame. The purpose is to gather the relevant background information and studies in order to address NR SUL for the Rel-17 bands/band combinations in Table 1-1 to Table 1-4.

Table 1-1: Release 17 SUL bands

| Band number | UL | DL | Duplex mode |
| --- | --- | --- | --- |
| n97 | 2300 MHz – 2400 MHz | N/A | SUL |
| n98 | 1880 MHz – 1920 MHz | N/A | SUL |

Table 1-2: Release 17 SA SUL band combinations

| SA SUL band combination | REL independent from |
| --- | --- |
| SUL_n41-n83 | Rel-15 |
| SUL_n79-n83 | Rel-15 |
| CA_n28_SUL_n41-n83 | Rel-15 |
| CA_n28_SUL_n79-n83 | Rel-15 |
| CA_n1_SUL_n78-n80 | Rel-15 |
| CA_n1_SUL_n78-n84 | Rel-15 |
| CA_n41_SUL_n79-n80 | Rel-15 |
| CA_n79_SUL_n41-n80 | Rel-15 |
| CA_n3_SUL_n78-n80 | Rel-15 |
| CA_n41_SUL_n79-n83 | Rel-15 |
| CA_n79_SUL_n41-n83 | Rel-15 |
| SUL_n79-n97 | Rel-15 |
| SUL_n41-n98 | Rel-15 |
| SUL_n79-n98 | Rel-15 |

Table 1-3: Release 17 NSA SUL band combinations

| NSA SUL band combination | REL independent from |
| --- | --- |

Table 1-4: Release 17 NSA SUL with ULSUP band combinations

| NSA SUL with ULSUP band combination | REL independent from |
| --- | --- |
| DC_28_SUL_n41-n83 | Rel-15 |

This TR contains a general part and specific band combination part. The actual requirements are added to the corresponding technical specifications.

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2] 	RP-201026, " New WID on Rel-17 Band combinations for SA NR Supplementary uplink (SUL), NSA NR SUL, NSA NR SUL with UL sharing from the UE perspective (ULSUP)", RAN#88

3	Definitions of terms, symbols and abbreviations

3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

example: text used to clarify abstract rules by applying them literally.

3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

ΔRIB,c	Allowed reference sensitivity relaxation due to support for inter-band CA operation, for serving cell c.

ΔTIB,c	Allowed maximum configured output power relaxation due to support for inter-band CA operation, for serving cell c.

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

<ABBREVIATION>	<Expansion>

BS				Base Station

BCS				Bandwidth Combination Set

CA				Carrier Aggregation

CC				Component Carriers

DC				Dual Connectivity

DL				Downlink

E-UTRA 				Evolved UMTS Terrestrial Radio Access

FDD				Frequency Division Duplex

MPR 				Allowed maximum power reduction

MSD				Maximum Sensitivity Degradation

REFSENS				Reference Sensitivity power level

SCS				Subcarrier spacing

SUL				Supplementary uplink

TDD				Time Division Duplex

UE				User Equipment

UL				Uplink

ULSUP				UL sharing from the UE perspective

4	Background

The present document is a technical report for SA NR SUL, NSA NR SUL and NSA NR SUL with UL sharing from ULSUP under Rel-17 time frame. It covers both the UE and BS side. The document is divided in different parts:

- 	Specific SA NR SUL part:  this part covers each band combination and its specific issues independently from each other (i.e. one subclause is defined per band combination)

- 	Specific NSA NR SUL part:  this part covers each band combination and its specific issues independently from each other (i.e. one subclause is defined per band combination)

- 	Specific NSA NR SUL with UL sharing from ULSUP part:  this part covers each band combination and its specific issues independently from each other (i.e. one subclause is defined per band combination)

4.1	TR Maintenance

A single company is responsible for introducing all approved TPs in the current TR, i.e. TR editor. However, it is the responsibility of the contact person of each band/band combination to ensure that the TPs related to the band/band combination have been implemented.

5	SA NR SUL band combination: Specific Band Combination Part

5.1	SUL_n41A-n83A

5.1.1	Operating bands

Table 5.1.1-1: SUL band combination

| NR Band combination for SUL | NR Band (Table 5.2-1) |
| --- | --- |
| SUL_n41-n832 | n41, n83 |
| NOTE 1: If a UE is configured with both NR UL and NR SUL carriers in a cell, the switching time between NR UL carrier and NR SUL carrier is 0 us. NOTE 2: For UE supporting SUL band combination simultaneous Rx/Tx capability is mandatory. |  |

5.1.2	Channel bandwidths per operating band

Table 5.1.2-1: Supported bandwidths per SUL band combination

| SUL configuration | NR Band | Subcarrier spacing (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SUL_n41A-n83A | n41 | 15 |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  | 0 |
|  |  | 30 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  | n83 | 15 | Yes | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |

5.1.3	Maximum output power

There is only single UL in uplink so this requirement is not applicable.

5.1.4	Spurious emission band UE co-existence

There is only single UL in uplink so this requirement is not applicable.

5.1.5	MSD

For SUL operation, the reference receive sensitivity (REFSENS) requirement for downlink bands shall be met for a supplementary uplink transmission bandwidth less than or equal to that in Table 5.1.5-1.

Table 5.1.5-1: Supplementary Uplink configuration for reference sensitivity

| NR Band / SCS of SUL band / Channel bandwidth of the DL band / NRB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DL band | SUL band | SCS of SUL band (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz |
| n41 | n83 | 15 |  | 100 | 100 | 100 |  | 100 | 100 | 100 | 100 | 100 | 100 | 100 |
| n41 | n83 | 30 |  | 50 | 50 | 50 |  | 50 | 50 | 50 | 50 | 50 | 50 | 50 |

For SUL_n41-n83, there is no harmonic or harmonic mixing product generated by Band n83 that may fall into the RX band of Band n41. Therefore, MSD due to harmonic or harmonic mixing is not needed.

5.1.6	∆TIB and ∆RIB values

For SUL_n41-n83, the ∆TIB,c and ∆RIB values are given in the tables below.

Table 5.1.6-1: ΔTIB,c

| SUL Band combination | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| SUL_n41-n83 | n41 | 0.3 |
|  | n83 | 0.3 |

Table 5.1.6-2: ΔRIB,c

| SUL Band combination | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| SUL_n41-n83 | n41 | 0 |

5.2	SUL_n79A-n83A

5.2.1	Operating bands

Table 5.2.1-1: SUL band combination

| NR Band combination for SUL | NR Band (Table 5.2-1) |
| --- | --- |
| SUL_n79-n832 | n79, n83 |
| NOTE 1: If a UE is configured with both NR UL and NR SUL carriers in a cell, the switching time between NR UL carrier and NR SUL carrier is 0 us. NOTE 2: For UE supporting SUL band combination simultaneous Rx/Tx capability is mandatory. |  |

5.2.2	Channel bandwidths per operating band

Table 5.2.2-1: Supported bandwidths per SUL band combination

| SUL configuration | NR Band | Subcarrier spacing (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SUL_n79A-n83A | n79 | 15 |  |  |  |  |  |  | Yes | Yes |  |  |  |  | 0 |
|  |  | 30 |  |  |  |  |  |  | Yes | Yes | Yes | Yes |  | Yes |  |
|  |  | 60 |  |  |  |  |  |  | Yes | Yes | Yes | Yes |  | Yes |  |
|  | n83 | 15 | Yes | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |

5.2.3	Maximum output power

There is only single UL in uplink so this requirement is not applicable.

5.2.4	Spurious emission band UE co-existence

There is only single UL in uplink so this requirement is not applicable.

5.2.5	MSD

For SUL operation, the reference receive sensitivity (REFSENS) requirement for downlink bands shall be met for a supplementary uplink transmission bandwidth less than or equal to that in Table 5.2.5-1.

Table 5.2.5-1: Supplementary Uplink configuration for reference sensitivity

| NR Band / SCS of SUL band / Channel bandwidth of the DL band / NRB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DL band | SUL band | SCS of SUL band (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz |
| n79 | n83 | 15 |  |  |  |  |  |  | 100 | 100 | 100 | 100 |  | 100 |
| n79 | n83 | 30 |  |  |  |  |  |  | 50 | 50 | 50 | 50 |  | 50 |

For SUL_n79-n83, there is no harmonic or harmonic mixing product generated by Band n83 that may fall into the RX band of Band n79. Therefore, MSD due to harmonic or harmonic mixing is not needed.

5.2.6	∆TIB and ∆RIB values

For SUL_n79-n83, the ∆TIB,c and ∆RIB values are given in the tables below.

Table 5.2.6-1: ΔTIB,c

| SUL Band combination | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| SUL_n79-n83 | n79 | 0.8 |
|  | n83 | 0.5 |

Table 5.2.6-2: ΔRIB,c

| SUL Band combination | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| SUL_n79-n83 | n79 | 0.5 |

5.2.7	Out-of-band blocking exceptions

For SUL_n79-n83, exceptions to the requirement specified in TS 38.101-1 Table 7.6C.3-2 are allowed since the intermodulation product of the SUL carrier and the CW interfering signal fully or partially overlaps with the DL carrier.

Table 5.2.7-1: SUL operating band combination with out-of-band exceptions allowed

NR Band combination for SUL

SUL_n79-n83

## 5.3	CA_n28A_SUL_n41A-n83A

### 5.3.1	Operating bands

Table 5.3.1-1: SUL band combination

| NR Band combination for SUL | NR Band (Table 5.2-1) |
| --- | --- |
| CA_n28_SUL_n41-n83 | n28, n41, n83 |
| NOTE 1: If a UE is configured with both NR UL and NR SUL carriers in a cell, the switching time between NR UL carrier and NR SUL carrier is 0 us. NOTE 2: For UE supporting SUL band combination simultaneous Rx/Tx capability is mandatory. |  |

### 5.3.2	Channel bandwidths per operating band

Table 5.3.2-1: Supported bandwidths per SUL band combination

| SUL band combination with CA | UL configuration | NR Band | Subcarrier spacing (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n28A_SUL_n41A-n83A | SUL_n41A-n83A | n28 | 15 | Yes | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n41 | 15 |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | n83 | 15 | Yes | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 5.3.3	Maximum output power

There is only single UL in uplink so this requirement is not applicable.

### 5.3.4	Spurious emission band UE co-existence

There is only single UL in uplink so this requirement is not applicable.

### 5.3.5	REFSENS requirements

For SUL operation with downlink CA, the reference receive sensitivity (REFSENS) requirement for downlink bands specified in clause 7.3A.2 from TS 38.101-1 shall be met when supplementary uplink configuration for reference sensitivity are specified as below.

Table 5.3.5-1: Supplementary uplink configuration for reference sensitivity

| NR Band / SCS of SUL band / Channel bandwidth of the DL band / NRB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DL band | SUL band | SCS of SUL band (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz |
| n28 | n83 | 15 | 100 | 100 | 100 | 100 |  | 100 |  |  |  |  |  |  |
|  |  | 30 |  | 50 | 50 | 50 |  | 50 |  |  |  |  |  |  |
| n41 | n83 | 15 |  | 100 | 100 | 100 |  | 100 | 100 | 100 | 100 | 100 | 100 | 100 |
|  |  | 30 |  | 50 | 50 | 50 |  | 50 | 50 | 50 | 50 | 50 | 50 | 50 |

The reference sensitivity exception between band n83 and n28 are specified below. The reference sensitivity exception between band n83 and n28 are specified below. The operation is limited to using equal or wider separation from n83 Tx to n28 Rx compared to n28 Tx-Rx separation. The reference sensitivity exception does not take into account the condition where band n83 channel bandwidths is wider than band n28. Otherwise, UE REFSENS performance will be degraded.

Table 5.3.5-2: Reference sensitivity exceptions due to cross band isolation

| UL band | DL band | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 40 MHz (dBm) | 50 MHz (dBm) | 60 MHz (dBm) | 80 MHz (dBm) | 90 MHz (dBm) | 100 MHz (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n83 | n28 | 0 | 0 | 0 | 0 |  | 0 |  |  |  |  |  |  |

Table 5.3.5-3: Uplink configuration for reference sensitivity exceptions due to cross band isolation

| UL band | DL band | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 40 MHz (dBm) | 50 MHz (dBm) | 60 MHz (dBm) | 80 MHz (dBm) | 90 MHz (dBm) | 100 MHz (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n831 | n28 | 25 | 25 | 25 | 25 |  | 25 |  |  |  |  |  |  |
| NOTE 1: The Tx-Rx carrier center frequency separation between SUL band and DL band is the same as the Tx-Rx carrier center frequency separation of DL band specified in table 5.4.4-1. The channel bandwidth of SUL band is the same as DL band. |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 5.3.6	∆TIB and ∆RIB values

For CA_n28_SUL_n41-n83, the TIB,c and RIB,c values are given in the tables below.

Table 5.3.6-1: ΔTIB,c

| SUL Band combination | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n28_SUL_n41-n83 | n28 | 0.3 |
|  | n41 | 0.3 |
|  | n83 | 0.3 |

Table 5.3.6-2: ΔRIB,c

| SUL Band combination | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n28_SUL_n41-n83 | n28 | 0.2 |
|  | n41 | 0 |

## 5.4	CA_n28A_SUL_n79A-n83A

### 5.4.1	Operating bands

Table 5.4.1-1: SUL band combination

| NR Band combination for SUL | NR Band (Table 5.2-1) |
| --- | --- |
| CA_n28-_SUL_n79-n83 | n28, n79, n83 |
| NOTE 1: If a UE is configured with both NR UL and NR SUL carriers in a cell, the switching time between NR UL carrier and NR SUL carrier is 0 us. NOTE 2: For UE supporting SUL band combination simultaneous Rx/Tx capability is mandatory. |  |

### 5.4.2	Channel bandwidths per operating band

Table 5.4.2-1: Supported bandwidths per SUL band combination

| SUL band combination with CA | UL configuration | NR Band | Subcarrier spacing (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n28A_SUL_n79A-n83A | SUL_n79A-n83A | n28 | 15 | Yes | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n79 | 15 |  |  |  |  |  |  | Yes | Yes |  |  |  |  |  |
|  |  |  | 30 |  |  |  |  |  |  | Yes | Yes | Yes | Yes |  | Yes |  |
|  |  |  | 60 |  |  |  |  |  |  | Yes | Yes | Yes | Yes |  | Yes |  |
|  |  | n83 | 15 | Yes | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 5.4.3	Maximum output power

There is only single UL in uplink so this requirement is not applicable.

### 5.4.4	Spurious emission band UE co-existence

There is only single UL in uplink so this requirement is not applicable.

### 5.4.5	REFSENS requirements

For SUL operation with downlink CA, the reference receive sensitivity (REFSENS) requirement for downlink bands specified in clause 7.3A.2 from TS 38.101-1 shall be met when supplementary uplink configuration for reference sensitivity are specified as below.

Table 5.4.5-1: Supplementary uplink configuration for reference sensitivity

| NR Band / SCS of SUL band / Channel bandwidth of the DL band / NRB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DL band | SUL band | SCS of SUL band (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz |
| n28 | n83 | 15 | 100 | 100 | 100 | 100 |  | 100 |  |  |  |  |  |  |
|  |  | 30 |  | 50 | 50 | 50 |  | 50 |  |  |  |  |  |  |
| n79 | n83 | 15 |  |  |  |  |  |  | 100 | 100 | 100 | 100 |  | 100 |
|  |  | 30 |  |  |  |  |  |  | 50 | 50 | 50 | 50 |  | 50 |

The reference sensitivity exception between band n83 and n28 are specified below. The reference sensitivity exception between band n83 and n28 are specified below. The operation is limited to using equal or wider separation from n83 Tx to n28 Rx compared to n28 Tx-Rx separation. The reference sensitivity exception does not take into account the condition where band n83 channel bandwidths is wider than band n28. Otherwise, UE REFSENS performance will be degraded.

Table 5.4.5-2: Reference sensitivity exceptions due to cross band isolation

| UL band | DL band | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 40 MHz (dBm) | 50 MHz (dBm) | 60 MHz (dBm) | 80 MHz (dBm) | 90 MHz (dBm) | 100 MHz (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n83 | n28 | 0 | 0 | 0 | 0 |  | 0 |  |  |  |  |  |  |

Table 5.4.5-3: Uplink configuration for reference sensitivity exceptions due to cross band isolation

| UL band | DL band | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 40 MHz (dBm) | 50 MHz (dBm) | 60 MHz (dBm) | 80 MHz (dBm) | 90 MHz (dBm) | 100 MHz (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n83 | n28 | 25 | 25 | 25 | 25 |  | 25 |  |  |  |  |  |  |
| NOTE 1: The Tx-Rx carrier center frequency separation between SUL band and DL band is the same as the Tx-Rx carrier center frequency separation of DL band specified in table 5.4.4-1. The channel bandwidth of SUL band is the same as DL band. |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 5.4.6	∆TIB and ∆RIB values

For CA_n28_SUL_n79-n83, the TIB,c and RIB,c values are given in the tables below.

Table 5.4.6-1: ΔTIB,c

| SUL Band combination | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n28_SUL_n79-n83 | n28 | 0.5 |
|  | n79 | 0.8 |
|  | n83 | 0.5 |

Table 5.4.6-2: ΔRIB,c

| SUL Band combination | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n28_SUL_n79-n83 | n28 | 0.2 |
|  | n79 | 0.5 |

## 5.5	CA_n1A_SUL_n78A-n80A

### 5.5.1	Operating bands

Table 5.5.1-1: SUL band combination

| NR Band combination for SUL | NR Band (Table 5.2-1) |
| --- | --- |
| CA_n1_SUL_n78-n80 | n1, n78, n80 |
| NOTE 1: If a UE is configured with both NR UL and NR SUL carriers in a cell, the switching time between NR UL carrier and NR SUL carrier is 0 us. NOTE 2: For UE supporting SUL band combination simultaneous Rx/Tx capability is mandatory. |  |

### 5.5.2	Channel bandwidths per operating band

Table 5.5.2-1: Supported bandwidths per SUL band combination

| SUL band combination with CA | UL configuration | NR Band | Subcarrier spacing (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n1A_SUL_n78A-n80A | SUL_n78A-n80A | n1 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | n80 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |

### 5.5.3	Maximum output power

There is only single UL in uplink so this requirement is not applicable.

### 5.5.4	Spurious emission band UE co-existence

There is only single UL in uplink so this requirement specified in clause 6.5.3.2 from 38.101-1 is applicable.

Table 5.5.4-1 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_n1_SUL_n78-n80.

Table 5.5.4-1: Impact of UL/DL Harmonic/Harmonic mixing

|  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band |  | Low Band Edge | High Band Edge | Low Band Edge | High Band Edge | Low Band Edge | High Band Edge | Low Band Edge | High Band Edge |
| n1 | DL frequency range | 2110 | 2170 | 4220 | 4340 | 6330 | 6510 | 8440 | 8680 |
| n78 | UL/DL frequency range | 3300 | 3800 | 6600 | 7600 | 9900 | 11400 | 13200 | 15200 |
| n80 | UL frequency range | 1710 | 1785 | 3420 | 3570 | 5130 | 5355 | 6840 | 7140 |

The 2nd harmonic of band n80 may fall into Rx band of n78. The MSD has been specified into the clause 7.3C.2 from 38.101-1.

### 5.5.5	REFSENS requirements

For SUL operation with CA, the reference receive sensitivity (REFSENS) requirement for downlink bands specified in clause 7.3A.2 from TS 38.101-1 shall be met when supplementary uplink configuration for reference sensitivity are specified as below.

Table 5.5.5-1: Supplementary uplink configuration for reference sensitivity

|  | NR Band / SCS of SUL band / Channel bandwidth of the DL band / NRB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DL band | SUL band | SCS of SUL band (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| n1 | n80 | 15 | 160 | 160 | 160 | 160 | 160 | 160 | 160 | 160 |  |  |  |  |  |
| n78 | n80 | 15 |  | 160 | 160 | 160 | 160 | 160 | 160 | 160 | 160 | 160 | 160 | 160 | 160 |

### 5.5.6	∆TIB and ∆RIB values

For CA_n1_SUL_n78-n80, the TIB,c and RIB,c values are given in the tables below.

Table 5.5.6-1: ΔTIB,c

| SUL Band combination | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n1_SUL_n78-n80 | n1 | 0.6 |
|  | n78 | 0.8 |
|  | n80 | 0.6 |

Table 5.5.6-2: ΔRIB,c

| SUL Band combination | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n1_SUL_n78-n80 | n1 | 0.2 |
|  | n78 | 0.5 |

## 5.6	CA_n1A_SUL_n78A-n84A

### 5.6.1	Operating bands

Table 5.6.1-1: SUL band combination

| NR Band combination for SUL | NR Band (Table 5.2-1) |
| --- | --- |
| CA_n1_SUL_n78-n84 | n1, n78, n84 |
| NOTE 1: If a UE is configured with both NR UL and NR SUL carriers in a cell, the switching time between NR UL carrier and NR SUL carrier is 0 us. NOTE 2: For UE supporting SUL band combination simultaneous Rx/Tx capability is mandatory. |  |

### 5.6.2	Channel bandwidths per operating band

Table 5.6.2-1: Supported bandwidths per SUL band combination

| SUL band combination with CA | UL configuration | NR Band | Subcarrier spacing (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n1A_SUL_n78A-n84A | SUL_n78A-n84A | n1 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | n84 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |

### 5.6.3	Maximum output power

There is only single UL in uplink so this requirement is not applicable.

### 5.6.4	Spurious emission band UE co-existence

There is only single UL in uplink so this requirement specified in clause 6.5.3.2 from 38.101-1 is applicable.

Table 5.6.4-1 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_n1_SUL_n78-n84.

Table 5.6.4-1: Impact of UL/DL Harmonic/Harmonic mixing

|  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band |  | Low Band Edge | High Band Edge | Low Band Edge | High Band Edge | Low Band Edge | High Band Edge | Low Band Edge | High Band Edge |
| n1 | DL frequency range | 2110 | 2170 | 4220 | 4340 | 6330 | 6510 | 8440 | 8680 |
| n78 | UL/DL frequency range | 3300 | 3800 | 6600 | 7600 | 9900 | 11400 | 13200 | 15200 |
| n84 | UL frequency range | 1920 | 1980 | 3840 | 3960 | 5760 | 5940 | 7680 | 7920 |

There is no harmonic/harmonic mixing issue for this band combination.

### 5.6.5	REFSENS requirements

For SUL operation with CA, the reference receive sensitivity (REFSENS) requirement for downlink bands specified in clause 7.3A.2 from TS 38.101-1 shall be met when supplementary uplink configuration for reference sensitivity are specified as below.

Table 5.6.5-1: Supplementary uplink configuration for reference sensitivity

|  | NR Band / SCS of SUL band / Channel bandwidth of the DL band / NRB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DL band | SUL band | SCS of SUL band (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| n1 | n841 | 15 | 25 | 50 | 75 | 100 | 128 | 128 | 128 | 128 |  |  |  |  |  |
| n78 | n84 | 15 |  | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 |
| NOTE 1: The Tx-Rx carrier center frequency separation between SUL band and DL band is the same as the Tx-Rx carrier center frequency separation of DL band specified in table 5.4.4-1 from TS 38.101-1. The channel bandwidth of SUL band is the same as DL band. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 5.6.6	∆TIB and ∆RIB values

For CA_n1_SUL_n78-n84, the TIB,c and RIB,c values are given in the tables below.

Table 5.6.6-1: ΔTIB,c

| SUL Band combination | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n1_SUL_n78-n84 | n1 | 0.6 |
|  | n78 | 0.8 |
|  | n84 | 0.6 |

Table 5.6.6-2: ΔRIB,c

| SUL Band combination | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n1_SUL_n78-n84 | n1 | 0.2 |
|  | n78 | 0.5 |

## 5.7	CA_n41A_SUL_n79A-n80A

### 5.7.1	Operating bands

Table 5.7.1-1: SUL band combination

| NR Band combination for SUL | NR Band (Table 5.2-1) |
| --- | --- |
| CA_n41_SUL_n79-n80 | n41, n79, n80 |
| NOTE 1: If a UE is configured with both NR UL and NR SUL carriers in a cell, the switching time between NR UL carrier and NR SUL carrier is 0 us. NOTE 2: For UE supporting SUL band combination simultaneous Rx/Tx capability is mandatory. |  |

### 5.7.2	Channel bandwidths per operating band

Table 5.7.2-1: Supported bandwidths per SUL band combination

| SUL band combination with CA | UL configuration | NR Band | Subcarrier spacing (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n41A_SUL_n79A-n80A | SUL_n79A-n80A | n41 | 15 |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  | n79 | 15 |  |  |  |  |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  |  |  |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |
|  |  |  | 60 |  |  |  |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |
|  |  | n80 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |

### 5.7.3	Maximum output power

There is only single UL in uplink so this requirement is not applicable.

### 5.7.4	Spurious emission band UE co-existence

There is only single UL in uplink so this requirement specified in clause 6.5.3.2 from 38.101-1 is applicable.

Table 5.7.4-1 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_n41_SUL_n79-n80.

Table 5.7.4-1: Impact of UL/DL Harmonic/Harmonic mixing

|  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band |  | Low Band Edge | High Band Edge | Low Band Edge | High Band Edge | Low Band Edge | High Band Edge | Low Band Edge | High Band Edge |
| n41 | DL frequency range | 2490 | 2690 | 4992 | 5380 | 7488 | 8070 | 9984 | 10760 |
| n79 | UL/DL frequency range | 4400 | 5000 | 8800 | 10000 | 13200 | 15000 | 17600 | 20000 |
| n80 | UL frequency range | 1710 | 1785 | 3420 | 3570 | 5130 | 5355 | 6840 | 7140 |

There is no harmonic/harmonic mixing issue for this combination. The MSD due to cross band isolation between band n80 and n41 has been specified into the 38.101-1

### 5.7.5	REFSENS requirements

For SUL operation with CA, the reference receive sensitivity (REFSENS) requirement for downlink bands specified in clause 7.3A.2 from TS 38.101-1 shall be met when supplementary uplink configuration for reference sensitivity are specified as below.

Table 5.7.5-1: Supplementary uplink configuration for reference sensitivity

|  | NR Band / SCS of SUL band / Channel bandwidth of the DL band / NRB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DL band | SUL band | SCS of SUL band (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| n41 | n80 | 15 |  | 160 | 160 | 160 |  | 160 | 160 | 160 | 160 |  | 160 | 160 | 160 |
| n79 | n80 | 15 |  |  |  |  |  |  | 160 | 160 | 160 | 160 | 160 | 160 | 160 |

### 5.7.6	∆TIB and ∆RIB values

For CA_n41_SUL_n79-n80, the TIB,c and RIB,c values are given in the tables below.

Table 5.7.6-1: ΔTIB,c

| SUL Band combination | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n41_SUL_n79-n80 | n41 | 0.31 |
|  |  | 0.82 |
|  | n79 | 0.8 |
|  | n80 | 0.3 |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2515-2690 MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496-2515 MHz. |  |  |

Table 5.7.6-2: ΔRIB,c

| SUL Band combination | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n41_SUL_n79-n80 | n41 | 0.5 |
|  | n79 | 0.5 |

## 5.8	CA_n79A_SUL_n41A-n80A

### 5.8.1	Operating bands

Table 5.8.1-1: SUL band combination

| NR Band combination for SUL | NR Band (Table 5.2-1) |
| --- | --- |
| CA_n79_SUL_n41-n80 | n41, n79, n80 |
| NOTE 1: If a UE is configured with both NR UL and NR SUL carriers in a cell, the switching time between NR UL carrier and NR SUL carrier is 0 us. NOTE 2: For UE supporting SUL band combination simultaneous Rx/Tx capability is mandatory. |  |

### 5.8.2	Channel bandwidths per operating band

Table 5.8.2-1: Supported bandwidths per SUL band combination

| SUL band combination with CA | UL configuration | NR Band | Subcarrier spacing (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n79A_SUL_n41A-n80A | SUL_n41A-n80A | n41 | 15 |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  | n79 | 15 |  |  |  |  |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  |  |  |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |
|  |  |  | 60 |  |  |  |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |
|  |  | n80 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |

### 5.8.3	Maximum output power

There is only single UL in uplink so this requirement is not applicable.

### 5.8.4	Spurious emission band UE co-existence

There is only single UL in uplink so this requirement specified in clause 6.5.3.2 from 38.101-1 is applicable.

Table 5.8.4-1 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_n79_SUL_n41-n80.

Table 5.8.4-1: Impact of UL/DL Harmonic/Harmonic mixing

|  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band |  | Low Band Edge | High Band Edge | Low Band Edge | High Band Edge | Low Band Edge | High Band Edge | Low Band Edge | High Band Edge |
| n41 | DL frequency range | 2490 | 2690 | 4992 | 5380 | 7488 | 8070 | 9984 | 10760 |
| n79 | UL/DL frequency range | 4400 | 5000 | 8800 | 10000 | 13200 | 15000 | 17600 | 20000 |
| n80 | UL frequency range | 1710 | 1785 | 3420 | 3570 | 5130 | 5355 | 6840 | 7140 |

There is no harmonic/harmonic mixing issue for this combination. The MSD due to cross band isolation between band n80 and n41 has been specified into the 38.101-1

### 5.8.5	REFSENS requirements

For SUL operation with CA, the reference receive sensitivity (REFSENS) requirement for downlink bands specified in clause 7.3A.2 from TS 38.101-1 shall be met when supplementary uplink configuration for reference sensitivity are specified as below.

Table 5.8.5-1: Supplementary uplink configuration for reference sensitivity

|  | NR Band / SCS of SUL band / Channel bandwidth of the DL band / NRB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DL band | SUL band | SCS of SUL band (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| n41 | n80 | 15 |  | 160 | 160 | 160 |  | 160 | 160 | 160 | 160 |  | 160 | 160 | 160 |
| n79 | n80 | 15 |  |  |  |  |  |  | 160 | 160 | 160 | 160 | 160 | 160 | 160 |

### 5.8.6	∆TIB and ∆RIB values

For CA_n79_SUL_n41-n80, the TIB,c and RIB,c values are given in the tables below.

Table 5.8.6-1: ΔTIB,c

| SUL Band combination | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n79_SUL_n41-n80 | n41 | 0.31 |
|  |  | 0.82 |
|  | n79 | 0.8 |
|  | n80 | 0.3 |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2515-2690 MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496-2515 MHz. |  |  |

Table 5.8.6-2: ΔRIB,c

| SUL Band combination | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n79_SUL_n41-n80 | n41 | 0.5 |
|  | n79 | 0.5 |

## 5.9	CA_n3A_SUL_n78A-n80A

### 5.9.1	Operating bands

Table 5.9.1-1: SUL band combination

| NR Band combination for SUL | NR Band (Table 5.2-1) |
| --- | --- |
| CA_n3_SUL_n78-n80 | n3, n78, n80 |
| NOTE 1: If a UE is configured with both NR UL and NR SUL carriers in a cell, the switching time between NR UL carrier and NR SUL carrier is 0 us. NOTE 2: For UE supporting SUL band combination simultaneous Rx/Tx capability is mandatory. |  |

### 5.9.2	Channel bandwidths per operating band

Table 5.9.2-1: Supported bandwidths per SUL band combination

| SUL band combination with CA | UL configuration | NR Band | Subcarrier spacing (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n3A_SUL_n78A-n80A | SUL_n78A-n80A | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | n80 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |

### 5.9.3	Maximum output power

There is only single UL in uplink so the requirement for each band in clause 6.2.1 from 38.101-1 is applicable.

### 5.9.4	Spurious emission band UE co-existence

There is only single UL in uplink so this requirement specified in clause 6.5.3.2 from 38.101-1 is applicable.

Table 5.9.4-1 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_n1_SUL_n78-n84.

Table 5.9.4-1: Impact of UL/DL Harmonic/Harmonic mixing

|  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band |  | Low Band Edge | High Band Edge | Low Band Edge | High Band Edge | Low Band Edge | High Band Edge | Low Band Edge | High Band Edge |
| n3 | DL frequency range | 1805 | 1880 | 3610 | 3760 | 5415 | 5640 | 7220 | 7520 |
| n78 | UL/DL frequency range | 3300 | 3800 | 6600 | 7600 | 9900 | 11400 | 13200 | 15200 |
| n80 | UL frequency range | 1710 | 1785 | 3420 | 3570 | 5130 | 5355 | 6840 | 7140 |

The 2nd harmonic interference of band n80 may fall into the Rx band n78. However, the corresponding MSD has been specified in the spec.

### 5.9.5	REFSENS requirements

For SUL operation with CA, the reference receive sensitivity (REFSENS) requirement for downlink bands specified in clause 7.3A.2 from TS 38.101-1 shall be met when supplementary uplink configuration for reference sensitivity are specified as below.

Table 5.9.5-1: Supplementary uplink configuration for reference sensitivity

|  | NR Band / SCS of SUL band / Channel bandwidth of the DL band / NRB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DL band | SUL band | SCS of SUL band (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| n3 | n801 | 15 | 25 | 50 | 50 | 50 | 50 | 50 | 50 |  |  |  |  |  |  |
| n78 | n80 | 15 |  | 160 | 160 | 160 | 160 | 160 | 160 | 160 | 160 | 160 | 160 | 160 | 160 |
| NOTE 1: The Tx-Rx carrier center frequency separation between SUL band and DL band is the same as the Tx-Rx carrier center frequency separation of DL band specified in table 5.4.4-1 from TS 38.101-1. The channel bandwidth of SUL band is the same as DL band. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

The harmonic exception between band n80 and n78 is shown below.

Table 5.9.5-2: Reference sensitivity for SUL operation (exceptions due to harmonic issue)

| NR Band / Channel bandwidth of the high band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
|  |  | dB | dB | dB | dB | dB | dB | dB | dB | dB | dB | dB | dB | dB |
| n80 | n781,2 |  | 23.9 | 22.1 | 20.9 | 20.1 | 19.3 | 17.9 | 16.8 | 16.0 | 15.5 | 14.8 | 14.3 | 13.8 |
|  | n783 |  | 1.1 | 0.8 | 0.3 |  |  |  |  |  |  |  |  |  |
| NOTE 1: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band and a range ∆FHD above and below the edge of this downlink transmission bandwidth. The value ∆FHD depends on the band combination: ∆FHD = 10 MHz for SUL_n78-n80, SUL_n78-n86. NOTE 2: The requirements should be verified for UL EARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 3: The requirements are only applicable to channel bandwidths no larger than 20 MHz and with a carrier frequency at MHz offset from in the victim (higher band) with , whereandare the channel bandwidths configured in the aggressor (lower) and victim (higher) bands in MHz, respectively. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 5.9.5-3: Supplementary uplink configuration (exceptions due to harmonic issue)

| NR Band / Channel bandwidth of the high band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (NRB) | 10 MHz (NRB) | 15 MHz (NRB) | 20 MHz (NRB) | 25 MHz (NRB) | 30 MHz (NRB) | 40 MHz (NRB) | 50 MHz (NRB) | 60 MHz (NRB) | 70 MHz (NRB) | 80 MHz (NRB) | 90 MHz (NRB) | 100 MHz (NRB) |
| n80 | n78 |  | 25 | 36 | 50 | 50 | 50 | 50 | 50 | 50 | 50 | 50 | 50 | 50 |
| NOTE 1: 15 kHz SCS is assumed for UL band. NOTE 2: The UL configuration applies regardless of the channel bandwidth of the low band NOTE 3: Unless stated otherwise, UL resource blocks shall be centered within the transmission bandwidth configuration for the channel bandwidth. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 5.9.6	∆TIB and ∆RIB values

For CA_n3_SUL_n78-n80, the TIB,c and RIB,c values are given in the tables below.

Table 5.9.6-1: ΔTIB,c

| SUL Band combination | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n3_SUL_n78-n80 | n3 | 0.6 |
|  | n78 | 0.8 |
|  | n80 | 0.6 |

Table 5.9.6-2: ΔRIB,c

| SUL Band combination | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n3_SUL_n78-n80 | n3 | 0.2 |
|  | n78 | 0.5 |

## 5.10	CA_n41A_SUL_n79A-n83A

### 5.10.1	Operating bands

Table 5.10.1-1: SUL band combination

| NR Band combination for SUL | NR Band (Table 5.2-1) |
| --- | --- |
| CA_n41_SUL_n79-n83 | n41, n79, n83 |
| NOTE 1: If a UE is configured with both NR UL and NR SUL carriers in a cell, the switching time between NR UL carrier and NR SUL carrier is 0 us. NOTE 2: For UE supporting SUL band combination simultaneous Rx/Tx capability is mandatory. |  |

### 5.10.2	Channel bandwidths per operating band

Table 5.10.2-1: Supported bandwidths per SUL band combination

| SUL band combination with CA | UL configuration | NR Band | Subcarrier spacing (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n41A_SUL_n79A-n83A | SUL_n79A-n83A | n41 | 15 |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  | n79 | 15 |  |  |  |  |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  |  |  |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |
|  |  |  | 60 |  |  |  |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |
|  |  | n83 | 15 | Yes | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |

### 5.10.3	Maximum output power

There is only single UL in uplink so the requirement for each band in clause 6.2.1 from 38.101-1 is applicable.

### 5.10.4	Spurious emission band UE co-existence

There is only single UL in uplink so this requirement specified in clause 6.5.3.2 from 38.101-1 is applicable.

Table 5.10.4-1 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_n41_SUL_n79-n83.

Table 5.10.4-1: Impact of UL/DL Harmonic/Harmonic mixing

|  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band |  | Low Band Edge | High Band Edge | Low Band Edge | High Band Edge | Low Band Edge | High Band Edge | Low Band Edge | High Band Edge |
| n41 | DL frequency range | 2490 | 2690 | 4992 | 5380 | 7488 | 8070 | 9984 | 10760 |
| n79 | UL/DL frequency range | 4400 | 5000 | 8800 | 10000 | 13200 | 15000 | 17600 | 20000 |
| n83 | UL frequency range | 703 | 748 | 1406 | 1496 | 2109 | 2244 | 2812 | 2992 |

There is no harmonic/harmonic mixing issue for this combination.

### 5.10.5	REFSENS requirements

For SUL operation with CA, the reference receive sensitivity (REFSENS) requirement for downlink bands specified in clause 7.3A.2 from TS 38.101-1 shall be met when supplementary uplink configuration for reference sensitivity are specified in 7.3C.2-1 from TS 38.101-1.

### 5.10.6	∆TIB and ∆RIB values

For CA_n41_SUL_n79-n83, the TIB,c and RIB,c values are given in the tables below.

Table 5.10.6-1: ΔTIB,c

| SUL Band combination | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n41_SUL_n79-n83 | n41 | 0.3 |
|  | n79 | 0.8 |
|  | n83 | 0.5 |

Table 5.10.6-2: ΔRIB,c

| SUL Band combination | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n41_SUL_n79-n83 | n41 | 0 |
|  | n79 | 0.5 |

## 5.11	CA_n79A_SUL_n41A-n83A

### 5.11.1	Operating bands

Table 5.11.1-1: SUL band combination

| NR Band combination for SUL | NR Band (Table 5.2-1) |
| --- | --- |
| CA_n79_SUL_n41-n83 | n41, n79, n83 |
| NOTE 1: If a UE is configured with both NR UL and NR SUL carriers in a cell, the switching time between NR UL carrier and NR SUL carrier is 0 us. NOTE 2: For UE supporting SUL band combination simultaneous Rx/Tx capability is mandatory. |  |

### 5.11.2	Channel bandwidths per operating band

Table 5.11.2-1: Supported bandwidths per SUL band combination

| SUL band combination with CA | UL configuration | NR Band | Subcarrier spacing (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n79A_SUL_n41A-n83A | SUL_n41A-n83A | n41 | 15 |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  | n79 | 15 |  |  |  |  |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  |  |  |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |
|  |  |  | 60 |  |  |  |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |
|  |  | n83 | 15 | Yes | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |

### 5.11.3	Maximum output power

There is only single UL in uplink so the requirement for each band in clause 6.2.1 from 38.101-1 is applicable.

### 5.11.4	Spurious emission band UE co-existence

There is only single UL in uplink so this requirement specified in clause 6.5.3.2 from 38.101-1 is applicable.

Table 5.11.4-1 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_n79_SUL_n41-n83.

Table 5.11.4-1: Impact of UL/DL Harmonic/Harmonic mixing

|  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band |  | Low Band Edge | High Band Edge | Low Band Edge | High Band Edge | Low Band Edge | High Band Edge | Low Band Edge | High Band Edge |
| n41 | UL/DL frequency range | 2496 | 2690 | 4992 | 5380 | 7488 | 8070 | 9984 | 10760 |
| n79 | DL frequency range | 4400 | 5000 | 8800 | 10000 | 13200 | 15000 | 17600 | 20000 |
| n83 | UL frequency range | 703 | 748 | 1406 | 1496 | 2109 | 2244 | 2812 | 2992 |

UL 2nd harmonic may fall into Rx of band n79.

### 5.11.5	REFSENS requirements

For SUL operation with CA, the reference receive sensitivity (REFSENS) requirement for downlink bands specified in clause 7.3A.2 from TS 38.101-1 shall be met when supplementary uplink configuration for reference sensitivity are specified in 7.3C.2-1 from TS 38.101-1.

UL 2nd harmonic between band n41 and n79 can follow current RAN4’s agreement of the fallback CA_n41-n79.

### 5.11.6	∆TIB and ∆RIB values

For CA_n79_SUL_n41-n83, the TIB,c and RIB,c values are given in the tables below.

Table 5.11.6-1: ΔTIB,c

| SUL Band combination | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n79_SUL_n41-n83 | n41 | 0.3 |
|  | n79 | 0.8 |
|  | n83 | 0.5 |

Table 5.11.6-2: ΔRIB,c

| SUL Band combination | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n79_SUL_n41-n83 | n41 | 0 |
|  | n79 | 0.5 |

## 5.12	SUL_n79A-n97A

### 5.12.1	Operating bands

Table 5.12.1-1: SUL band combination

| NR Band combination for SUL | NR Band (Table 5.2-1) |
| --- | --- |
| SUL_n79-n97 | n79, n97 |
| NOTE 1: If a UE is configured with both NR UL and NR SUL carriers in a cell, the switching time between NR UL carrier and NR SUL carrier is 0 us. NOTE 2: For UE supporting SUL band combination simultaneous Rx/Tx capability is mandatory. |  |

### 5.12.2	Channel bandwidths per operating band

Table 5.12.2-1: Supported bandwidths per SUL band combination

| SUL band combination with CA | UL configuration | NR Band | Subcarrier spacing (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SUL_n79A-n97A | SUL_n79A-n97A | n79 | 15 |  |  |  |  |  |  | Yes | Yes |  |  |  |  |  | 0 |
|  |  |  | 30 |  |  |  |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |
|  |  |  | 60 |  |  |  |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |
|  |  | n97 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  | Yes |  |  |  |

### 5.12.3	Maximum output power

There is only single UL in uplink so the requirement for each band in clause 6.2.1 from 38.101-1 is applicable.

### 5.12.4	Spurious emission band UE co-existence

There is only single UL in uplink so this requirement specified in clause 6.5.3.2 from 38.101-1 is applicable.

Table 5.12.4-1 summarizes frequency ranges where harmonics and/or harmonics mixing occur for SUL_n79-n97.

Table 5.12.4-1: Impact of UL/DL Harmonic/Harmonic mixing

|  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band |  | Low Band Edge | High Band Edge | Low Band Edge | High Band Edge | Low Band Edge | High Band Edge | Low Band Edge | High Band Edge |
| n79 | UL/DL frequency range | 4400 | 5000 | 8800 | 10000 | 13200 | 15000 | 17600 | 20000 |
| n97 | UL frequency range | 2300 | 2400 | 4600 | 4800 | 6900 | 7200 | 9200 | 9600 |

The 2nd harmonic interference of band n97 may fall into Rx band of n79.

### 5.12.5	REFSENS requirements

For SUL operation with CA, the reference receive sensitivity (REFSENS) requirement for downlink bands specified in clause 7.3A.2 from TS 38.101-1 shall be met when supplementary uplink configuration for reference sensitivity are specified as below.

Table 5.12.5-1: Supplementary uplink configuration for reference sensitivity

|  | NR Band / SCS of SUL band / Channel bandwidth of the DL band / NRB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DL band | SUL band | SCS of SUL band (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| n79 | n97 | 15 |  |  |  |  |  |  | 270 | 270 | 270 |  | 270 |  | 270 |

The MSD exception for SUL_n79-n97 are specified below.

Table 5.12.5-2: Reference sensitivity for SUL operation (exceptions due to harmonic issue)

| UL band |  | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n97 |  | n791,2 |  |  |  |  |  |  | 29.4 | 28.4 | 27.6 | 26.3 |  | 25.3 |
| NOTE 1: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band and a range ∆FHD above and below the edge of this downlink transmission bandwidth. The value ∆FHD depends on the band combination: ∆FHD = 10 MHz for SUL_n78-n80, SUL_n78-n86. NOTE 2: The requirements should be verified for UL EARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 5.12.5-3: Supplementary uplink configuration (exceptions due to harmonic issue)

| UL band | DL band | 5 MHz (NRB) | 10 MHz (NRB) | 15 MHz (NRB) | 20 MHz (NRB) | 25 MHz (NRB) | 30MHz (NRB) | 40 MHz (NRB) | 50 MHz (NRB) | 60 MHz (NRB) | 80 MHz (NRB) | 90 MHz (NRB) | 100 MHz (NRB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n97 | n79 |  |  |  |  |  |  | 100 | 135 | 160 | 216 |  | 270 |
| NOTE 1: 15 kHz SCS is assumed for UL band. NOTE 2: The UL configuration applies regardless of the channel bandwidth of the low band NOTE 3: Unless stated otherwise, UL resource blocks shall be centered within the transmission bandwidth configuration for the channel bandwidth. |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 5.12.6	∆TIB and ∆RIB values

For SUL_n79-n97, the TIB,c and RIB,c values are given in the tables below.

Table 5.12.6-1: ΔTIB,c

| SUL Band combination | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| SUL_n79-n97 | n79 | 0.8 |
|  | n97 | 0.3 |

Table 5.12.6-2: ΔRIB,c

| SUL Band combination | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| SUL_n79-n97 | n79 | 0.5 |

## 5.13	SUL_n41A-n98A

### 5.13.1	Operating bands

Table 5.13.1-1: SUL band combination

| NR Band combination for SUL | NR Band (Table 5.2-1) |
| --- | --- |
| SUL_n41-n982 | n41, n98 |
| NOTE 1: If a UE is configured with both NR UL and NR SUL carriers in a cell, the switching time between NR UL carrier and NR SUL carrier is 0 us. NOTE 2: For UE supporting SUL band combination simultaneous Rx/Tx capability is mandatory. |  |

### 5.13.2	Channel bandwidths per operating band

Table 5.13.2-1: Supported bandwidths per SUL band combination

| SUL band combination with CA | UL configuration | NR Band | Subcarrier spacing (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SUL_n41A-n98A | SUL_n41A-n98A | n41 | 15 |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  | n98 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |

### 5.13.3	Maximum output power

There is only single UL in uplink so the requirement for each band in clause 6.2.1 from 38.101-1 is applicable.

### 5.13.4	Spurious emission band UE co-existence

There is only single UL in uplink so this requirement specified in clause 6.5.3.2 from 38.101-1 is applicable.

Table 5.13.4-1 summarizes frequency ranges where harmonics and/or harmonics mixing occur for SUL_n41-n98.

Table 5.13.4-1: Impact of UL/DL Harmonic/Harmonic mixing

|  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band |  | Low Band Edge | High Band Edge | Low Band Edge | High Band Edge | Low Band Edge | High Band Edge | Low Band Edge | High Band Edge |
| n41 | UL/DL frequency range | 2496 | 2690 | 4992 | 5380 | 7488 | 8070 | 9984 | 10760 |
| n98 | UL frequency range | 1880 | 1920 | 3760 | 3840 | 5640 | 5760 | 7520 | 7680 |

There is no harmonic/harmonic mixing issue for this band combination.

### 5.13.5	REFSENS requirements

For SUL operation, the reference receive sensitivity (REFSENS) requirement for downlink bands specified in clause 7.3A.2 from TS 38.101-1 shall be met when supplementary uplink configuration for reference sensitivity are specified as below.

Table 5.13.5-1: Supplementary uplink configuration for reference sensitivity

|  | NR Band / SCS of SUL band / Channel bandwidth of the DL band / NRB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DL band | SUL band | SCS of SUL band (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| n41 | n98 | 15 |  | 216 | 216 | 216 |  | 216 | 216 | 216 | 216 |  | 216 | 216 | 216 |

### 5.13.6	∆TIB and ∆RIB values

For SUL_n41-n98, the TIB,c and RIB,c values are given in the tables below.

Table 5.13.6-1: ΔTIB,c

| SUL Band combination | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| SUL_n41-n98 | n41 | 0.5 |
|  | n98 | 0.5 |

Table 5.13.6-2: ΔRIB,c

| SUL Band combination | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| SUL_n41-n98 | n41 | 0.2 |

## 5.14	SUL_n79A-n98A

### 5.14.1	Operating bands

Table 5.14.1-1: SUL band combination

| NR Band combination for SUL | NR Band (Table 5.2-1) |
| --- | --- |
| SUL_n79-n98 | n79, n98 |
| NOTE 1: If a UE is configured with both NR UL and NR SUL carriers in a cell, the switching time between NR UL carrier and NR SUL carrier is 0 us. NOTE 2: For UE supporting SUL band combination simultaneous Rx/Tx capability is mandatory. |  |

### 5.14.2	Channel bandwidths per operating band

Table 5.14.2-1: Supported bandwidths per SUL band combination

| SUL band combination with CA | UL configuration | NR Band | Subcarrier spacing (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SUL_n79A-n98A | SUL_n79A-n98A | n79 | 15 |  |  |  |  |  |  | Yes | Yes |  |  |  |  |  | 0 |
|  |  |  | 30 |  |  |  |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |
|  |  |  | 60 |  |  |  |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |
|  |  | n98 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |

### 5.14.3	Maximum output power

There is only single UL in uplink so the requirement for each band in clause 6.2.1 from 38.101-1 is applicable.

### 5.14.4	Spurious emission band UE co-existence

There is only single UL in uplink so this requirement specified in clause 6.5.3.2 from 38.101-1 is applicable.

Table 5.14.4-1 summarizes frequency ranges where harmonics and/or harmonics mixing occur for SUL_n79-n98.

Table 5.14.4-1: Impact of UL/DL Harmonic/Harmonic mixing

|  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band |  | Low Band Edge | High Band Edge | Low Band Edge | High Band Edge | Low Band Edge | High Band Edge | Low Band Edge | High Band Edge |
| n79 | UL/DL frequency range | 4400 | 5000 | 8800 | 10000 | 13200 | 15000 | 17600 | 20000 |
| n98 | UL frequency range | 1880 | 1920 | 3760 | 3840 | 5640 | 5760 | 7520 | 7680 |

There is no harmonic or harmonic mixing issue.

### 5.14.5	REFSENS requirements

For SUL operation with CA, the reference receive sensitivity (REFSENS) requirement for downlink bands specified in clause 7.3A.2 from TS 38.101-1 shall be met when supplementary uplink configuration for reference sensitivity are specified as below.

Table 5.14.5-1: Supplementary uplink configuration for reference sensitivity

|  | NR Band / SCS of SUL band / Channel bandwidth of the DL band / NRB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DL band | SUL band | SCS of SUL band (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| n79 | n98 | 15 |  |  |  |  |  |  | 216 | 216 | 216 |  | 216 |  | 216 |

### 5.14.6	∆TIB and ∆RIB values

For SUL_n79-n98, the TIB,c and RIB,c values are given in the tables below.

Table 5.14.6-1: ΔTIB,c

| SUL Band combination | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| SUL_n79-n98 | n79 | 0.8 |
|  | n98 | 0.3 |

Table 5.14.6-2: ΔRIB,c

| SUL Band combination | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| SUL_n79-n98 | n79 | 0.5 |

5.Y	SUL_nX-nY/CA_nX_SUL_nY-nZ

5.Y.1	Operating bands

5.Y.2	Configuration

5.Y.3	Maximum output power

<Editor’s note: This requirement is only applicable when there is simultaneous transmission in the band combination.>

5.Y.4	Spurious emission band UE co-existence

<Editor’s note: This requirement is only applicable when there is simultaneous transmission in the band combination.>

5.Y.5	REFSENS requirements

5.Y.6	∆TIB and ∆RIB values

5.Y.7	Out-of-band blocking exception

<Editor’s note: This requirement is only applicable when there is Out-of-band blocking exception in the band combination.>

6	NSA NR SUL band combination: Specific Band Combination Part

6.Y	DC_X_SUL_nY-nZ

6.Y.1	Operating bands

6.Y.2	Configuration

6.Y.3	Maximum output power

6.Y.4	Spurious emission band UE co-existence

6.Y.5	REFSENS requirements

6.Y.6	∆TIB and ∆RIB values

7	NSA NR SUL with UL sharing from ULSUP band combination: Specific Band Combination Part

7.1	DC_28_SUL_n41-n83

7.1.1	Operating bands

Table 7.1.1-1: EN-DC band combination

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_28_SUL_n41-n832 | 1 | SUL_n41-n83 | N/A |
| NOTE 1: If a UE is configured with both NR UL and NR SUL carriers in a cell, the switching time between NR UL carrier and NR SUL carrier can be up to 140us and placed in SUL resources. NOTE 2: Applicable for UE supporting inter-band carrier aggregation with mandatory simultaneous Rx/Tx capability. |  |  |  |

7.1.2	Configuration

Table 7.1.2-1: Inter-band EN-DC configurations

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_28A_SUL_n41A-n83A | DC_28A_n41A DC_28A_n83A_ULSUP-TDM_n41 | 28A | SUL_n41A-n83A |

7.1.3	Maximum output power

Table 7.1.3-1: Maximum output power for inter-band EN-DC

| DC configuration | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_28A_n41A DC_28A_n83A_ULSUP-TDM_n41 | 23 | +2/-3 |

7.1.4	Spurious emission band UE co-existence

Table 7.1.4-1: Spurious emissions for inter-band EN-DC

| EN-DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_28_n41 DC_28_n83_ULSUP-TDM_n41 | E-UTRA Band 4, 10, 12, 13, 14, 17, 18, 19, 20, 26, 27, 29, 39, 42, 43, 50, 51, 52, 65, 66, 71, 73, 85 NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 1 | FDL_low | - | FDL_high | -50 | 1 | 9, 10 |
|  | E-UTRA Band 2, 3, 5, 8, 24, 25, 30, 31, 34, 40, 44, 48, 70, 72 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 11, 21, 74, 75, 76 | FDL_low | - | FDL_high | -50 | 1 | 9, 11 |
|  | Frequency range | 470 | - | 694 | -42 | 8 | 5, 17 |
|  | Frequency range | 470 | - | 710 | -26.2 | 6 | 14 |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x 180 kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 5: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 and Table 6.6.3.1A-1 from the edge of the channel bandwidth. NOTE 9: Applicable when the assigned E-UTRA carrier is confined within 718 MHz and 748 MHz and when the channel bandwidth used is 5 or 10 MHz. NOTE 10: As exceptions, measurements with a level up to the applicable requirement of -36 dBm/MHz is permitted for each assigned E-UTRA carrier used in the measurement due to 2nd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.6-1) for which the 2nd harmonic totally or partially overlaps the measurement bandwidth (MBW). NOTE 11: As exceptions, measurements with a level up to the applicable requirement of -38 dBm/MHz is permitted for each assigned E-UTRA carrier used in the measurement due to 3rd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.6-1) for which the 3rd harmonic totally or partially overlaps the measurement bandwidth (MBW). NOTE 14: This requirement is applicable for 5 and 10 MHz E-UTRA channel bandwidth allocated within 718-728MHz. For carriers of 10 MHz bandwidth, this requirement applies for an uplink transmission bandwidth less than or equal to 30 RB with RBstart > 1 and RBstart < 48. NOTE 17: This requirement is applicable in the case of a 10 MHz E-UTRA carrier confined within 703 MHz and 733 MHz, otherwise the requirement of -25 dBm with a measurement bandwidth of 8 MHz applies. |  |  |  |  |  |  |  |

7.1.5	MSD

For DC_28_SUL_n41-n83, there are neither harmonic and harmonic mixing products generated by Band 28/n83 that may fall into the RX band of Band n41 nor IMD products produced by Band 28 and n41 that impact the reference sensitivity of Band 28. Therefore, MSD requirements are not needed for this configuration.

7.1.6	∆TIB and ∆RIB values

For DC_28_SUL_n41-n83, the TIB,c and RIB values are given in the tables below.

Table 7.1.6-1: ΔTIB,c

| EN-DC Band combination | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_28_SUL_n41-n83 | 28 | 0.3 |
|  | n41 | 0.3 |
|  | n83 | 0.3 |

Table 7.1.6-2: ΔRIB,c

| EN-DC Band combination | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_28_SUL_n41-n83 | 28 | 0 |
|  | n41 | 0 |

7.Y	DC_X_SUL_nY-nZ

7.Y.1	Operating bands

7.Y.2	Configuration

7.Y.3	Maximum output power

7.Y.4	Spurious emission band UE co-existence

7.Y.5	REFSENS requirements

7.Y.6	∆TIB and ∆RIB values

Annex A (informative): Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2020-08 | 3GPP RAN4#96-e | R4-2001223 |  |  |  | Initial TR skeleton | 0.0.1 |
| 2020-08 | 3GPP RAN4#96-e | R4-2001224 |  |  |  | Implemented TP’s from RAN4#96e R4-2011668 TP to TR 37.717-00-00 SUL_n41A-n83A, Huawei, HiSilicon, Etisalat, CMCC R4-2011669 TP to TR 37.717-00-00 SUL_n79A-n83A, Huawei, CBN, CMCC, HiSilicon, ABS R4-2011671 TP to TR 37.717-00-00 DC_28A_SUL_n41A-n83A, Huawei, HiSilicon, Etisalat, CMCC R4-2011672 TP for TR 37.717-00-00 for CA_n28A-n41A_SUL_n41A-n83A, Huawei, HiSilicon R4-2011673 TP for TR 37.717-00-00 for CA_n28A-n79A_SUL_n79A-n83A, Huawei, HiSilicon | 0.1.0 |
| 2020-11 | 3GPP RAN4#97-e | R4-2014801 |  |  |  | Implemented TP’s from RAN4#97e R4-2015540 TP for TR 37.717-00-00 to correct the notation of SUL band, Huawei, HiSilicon R4-2016748 TP for TR 37.717-00-00 for CA_n1A_SUL_n78A-n80A, Huawei, HiSilicon R4-2016749 TP for TR 37.717-00-00 for CA_n1A_SUL_n78A-n84A, Huawei, HiSilicon R4-2016750 TP for TR 37.717-00-00 for CA_n41A_SUL_n79A-n80A, Huawei, HiSilicon R4-2016751 TP for TR 37.717-00-00 for CA_n79A_SUL_n41A-n80A, Huawei, HiSilicon | 0.2.0 |
| 2021-02 | 3GPP RAN4#98-e | R4-2100292 |  |  |  | Implemented TP’s from RAN4#98e R4-2103070 TP for TR 37.717-00-00 for CA_n3A_SUL_n78A-n80A, Huawei, HiSilicon R4-2103071 TP for TR 37.717-00-00 for CA_n41A_SUL_n79A-n83A, Huawei, HiSilicon R4-2103072 TP for TR 37.717-00-00 for CA_n79A_SUL_n41A-n83A, Huawei, HiSilicon R4-2103074 TP for TR 37.717-00-00 for SUL_n79A-n97A, Huawei, HiSilicon R4-2103075 TP for TR 37.717-00-00 for SUL_n41A-n98A, Huawei, HiSilicon R4-2101611 TP for TR 37.717-00-00 for SUL_n79A-n98A, Huawei, HiSilicon | 0.3.0 |