3GPP TR 38.717-02-01 V0.3.0 (2021-02)

Technical Report

3rd Generation Partnership Project;

Technical Specification Group Radio Access Network;

Rel-17 NR inter-band Carrier Aggregation/Dual connectivity for 2 bands DL with x bands UL (x=1, 2)

(Release 17)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.
This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2019, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	14

1	Scope		15

2	References		15

3	Definitions, symbols and abbreviations	15

3.1	Definitions		15

3.2	Symbols		16

3.3	Abbreviations		16

4	Background		16

4.1	TR Maintenance	16

5	NR Inter-Band Combination: General Part	16

6	Both bands within FR1 Carrier Aggregation: Specific Band Combination Part	17

6.1	 CA_n5-n25		17

6.1.1	Common for 1 band UL and 2 bands UL CA	17

6.1.1.1 Operating bands for CA	17

6.1.1.2	Channel bandwidths per operating band for CA	17

6.1.1.3	Co-existence studies	17

6.1.1.4	∆TIB and ∆RIB values	18

6.1.1.5	REFSENs requirements	18

6.1.1.6	OOB blocking exception requirements	18

6.1.2	 Specific for 2 bands UL CA	18

6.1.2.1	Maximum output power for inter-band CA	18

6.1.2.2	 UE co-existence	19

6.1.2.3	 REFSENS requirements	20

6.2	 CA_n71-n78		20

6.2.1	Common for 1 band UL and 2 bands UL CA	20

6.2.1.1 Operating bands for CA	20

6.2.1.2	Channel bandwidths per operating band for CA	20

6.2.1.3	Co-existence studies	21

6.2.1.4	∆TIB and ∆RIB values	21

6.2.1.5	REFSENs requirements	21

6.2.1.6	OOB blocking exception requirements	21

6.2.2	 Specific for 2 bands UL CA	21

6.2.2.1	Maximum output power for inter-band CA	21

6.2.2.2	 UE co-existence	21

6.2.2.3	 REFSENS requirements	22

6.3	 CA_n7-n66		22

6.3.1	Common for 1 band UL and 2 bands UL CA	22

6.3.1.1	Operating bands for CA	22

6.3.1.2	Channel bandwidths per operating band for CA	23

6.3.1.3	UE co-existence studies	23

6.3.1.4	∆TIB and ∆RIB values	23

6.3.1.5	REFSENS requirements	24

6.3.1.6	OOB blocking exception requirements	24

6.3.2	Specific for 2 bands UL CA	24

6.3.2.1	Maximum output power for inter-band CA	24

6.3.2.2	UE co-existence studies	24

6.3.2.3	REFSENS requirements	25

6.8	 CA_n25-38		26

6.4.1	 Common for 1 band UL and 2 bands UL CA	26

6.4.1.2	Channel bandwidths per operating band for CA	26

6.4.1.2	Channel bandwidths per operating band for CA	26

6.4.1.3	UE co-existence studies	26

6.4.1.4	∆TIB and ∆RIB values	27

6.4.1.5	REFSENS requirements	27

6.4.1.6	OOB blocking exception requirements	28

6.4.2	Specific for 2 bands UL CA	28

6.4.2.1	Maximum output power for inter-band CA	28

6.4.2.2	UE co-existence studies	28

6.4.2.3	REFSENS requirements	29

6.5	 CA_n1-n77		30

6.5.1	 Common for 1 band UL and 2 bands UL CA	30

6.5.1.1	  Operating bands for CA	30

6.5.1.2	  Channel bandwidths per operating band for CA	30

6.5.1.3	  UE co-existence studies	32

6.5.1.4	  ∆TIB and ∆RIB values	32

6.5.1.5	  REFSENS requirements	32

6.5.1.6	  OOB blocking exception requirements	32

6.5.2	 Specific for 2 bands UL CA	32

6.5.2.1	  Maximum output power for inter-band CA	32

6.5.2.2	  UE co-existence studies	32

6.5.2.3	  REFSENS requirements	34

6.6	CA_n77-n79		34

6.6.1	 Common for 1 band UL and 2 bands UL CA	34

6.6.1.1	  Operating bands for CA	34

6.6.1.2	  Channel bandwidths per operating band for CA	35

6.6.1.3	  UE co-existence studies	37

6.6.1.4	  ∆TIB and ∆RIB values	37

6.6.1.5	  REFSENS requirements	37

6.6.1.6	  OOB blocking exception requirements	37

6.6.2	 Specific for 2 bands UL CA	37

6.6.2.1	  Maximum output power for inter-band CA	37

6.6.2.2	  UE co-existence studies	37

6.7	CA_n78-n79		40

6.7.1	 Common for 1 band UL and 2 bands UL CA	40

6.7.1.1	 Operating bands for CA	40

6.7.1.2	 Channel bandwidths per operating band for CA	40

6.7.1.3	 UE co-existence studies	42

6.7.1.4	 ∆TIB and ∆RIB values	42

6.7.1.5	 REFSENS requirements	42

6.7.1.6	 OOB blocking exception requirements	43

6.7.2	 Specific for 2 bands UL CA	43

6.7.2.1	 Maximum output power for inter-band CA	43

6.7.2.2	 UE co-existence studies	43

6.7.2.3	 REFSENS requirements	45

6.8	CA_n25-n48		45

6.8.1	Common for 1 band UL and 2 bands UL CA	45

6.8.1.1	 Operating bands for CA	45

6.8.1.2	 Channel bandwidths per operating band for CA	45

6.8.1.3	 UE co-existence studies	45

6.8.1.4	 ∆TIB and ∆RIB values	46

6.8.1.5 	 REFSENS requirements	46

6.8.1.6 	 OOB blocking exception requirements	47

6.9	CA_n28-n79		47

6.9.1	Common for 1 band UL and 2 bands UL CA	47

6.9.1.1 Operating bands for CA	47

6.9.1.2	Channel bandwidths per operating band for CA	48

6.9.1.3	UE Co-existence studies	48

6.9.1.4	∆TIB and ∆RIB values	48

6.9.1.5	REFSENs requirements	49

6.9.1.6	OOB blocking exception requirements	49

6.9.2	 Specific for 2 bands UL CA	49

6.9.2.1	Maximum output power for inter-band CA	49

6.9.2.2	 UE co-existence studies	49

6.9.2.3	 REFSENS requirements	51

6.12	 CA_n28-n77		54

6.12.1	 Common for 1 band UL and 2 bands UL CA	54

6.12.1.1	 Operating bands for CA	54

6.12.1.2	 Channel bandwidths per operating band for CA	54

6.12.1.4	 ∆TIB and ∆RIB values	54

6.12.1.5	 REFSENS requirements	55

6.12.1.6	    OOB blocking exception requirements	55

6.13	 CA_n28-n78		55

6.13.1	 Common for 1 band UL and 2 bands UL CA	55

6.13.1.1	 Operating bands for CA	55

6.13.1.2	 Channel bandwidths per operating band for CA	55

6.13.1.4	 ∆TIB and ∆RIB values	56

6.13.1.5	 REFSENS requirements	56

6.13.1.6	    OOB blocking exception requirements	56

6.14.1.4	 ∆TIB and ∆RIB values	57

6.15.1.4	 ∆TIB and ∆RIB values	59

6.15.2	 Specific for 2 bands UL CA	60

6.15.2.1	Maximum output power for inter-band CA	60

6.15.2.2	 UE co-existence studies	60

6.15.2.3	 REFSENS requirements	61

6.16	CA_n41-n77		61

6.16.1	Common for 1 band UL and 2 bands UL CA	61

6.16.1.1 Operating bands for CA	61

6.16.1.2	Channel bandwidths per operating band for CA	62

6.16.1.3	Co-existence studies	62

6.16.1.4	∆TIB and ∆RIB values	63

6.16.1.5	REFSENs requirements	63

6.16.1.6	OOB blocking exception requirements	63

6.16.2	 Specific for 2 bands UL CA	63

6.16.2.1	Maximum output power for inter-band CA	63

6.16.2.2	 UE co-existence	63

6.16.2.3	 REFSENS requirements	65

6.17	 CA_n71-n77		66

6.17.1	Common for 1 band UL and 2 bands UL CA	66

6.17.1.1 Operating bands for CA	66

6.17.1.2	Channel bandwidths per operating band for CA	66

6.17.1.3	Co-existence studies	66

6.17.1.4	∆TIB and ∆RIB values	67

6.17.1.5	REFSENs requirements	67

6.17.1.6	OOB blocking exception requirements	67

6.17.2	 Specific for 2 bands UL CA	67

6.17.2.1	Maximum output power for inter-band CA	67

6.17.2.2	 UE co-existence	67

6.17.2.3	 REFSENS requirements	69

6.18	CA_n5-n48		69

6.18.1	Common for 1 band UL and 2 bands UL CA	69

6.18.1.1	 Operating bands for CA	69

6.18.1.2	 Channel bandwidths per operating band for CA	69

6.18.1.3	 UE co-existence studies	70

6.18.1.4	 ∆TIB and ∆RIB values	70

6.18.1.5	 REFSENS requirements	71

6.18.1.6	 OOB blocking exception requirements	71

6.18.2	 Specific for 2 bands UL CA	71

6.18.2.1	Maximum output power for inter-band CA	71

6.18.2.2	 UE co-existence	71

6.18.2.3  REFSENs requirements	72

6.19	 CA_n34-n79		72

6.19.1	 Common for 1 band UL and 2 bands UL CA	72

6.19.1.1	 Operating bands for CA	72

6.19.1.2	 Channel bandwidths per operating band for CA	73

6.19.1.3	 Co-existence studies	73

6.19.1.4	 ∆TIB and ∆RIB values	73

6.19.1.5	 REFSENS requirements	74

6.19.1.6	 OOB blocking exception requirements	74

6.19.2	 Specific for 2 bands UL CA	74

6.19.2.1	 UE co-existence studies	74

6.19.2.2	 REFSENS requirements	75

6.20	 CA_n5-n25		75

6.20.1	Common for 1 band UL and 2 bands UL CA	75

6.20.1.1	Operating bands for CA	75

6.20.1.2	Channel bandwidths per operating band for CA	76

6.20.1.3	UE co-existence studies	76

6.20.1.4	∆TIB and ∆RIB values	76

6.20.1.5	REFSENS requirements	77

6.20.1.6	OOB blocking exception requirements	77

6.20.2	Specific for 2 bands UL CA	77

6.20.2.1	Maximum output power for inter-band CA	77

6.20.2.2	UE co-existence studies	77

6.20.2.3	REFSENS requirements	78

6.21	 CA_n25-n77	78

6.21.1	Common for 1 band UL and 2 bands UL CA	78

6.21.1.1	Operating bands for CA	78

6.21.1.2	Channel bandwidths per operating band for CA	78

6.21.1.3	UE co-existence studies	79

6.21.1.4	∆TIB and ∆RIB values	79

6.21.1.5	REFSENS requirements	79

6.21.1.6	OOB blocking exception requirements	80

6.21.2	Specific for 2 bands UL CA	81

6.21.2.1	Maximum output power for inter-band CA	81

6.21.2.2	UE co-existence studies	81

6.21.2.3	REFSENS requirements	82

6.22	CA_n8-n28		82

6.22.1	Common for 1 band UL and 2 bands UL CA	82

6.22.1.1 Operating bands for CA	82

6.22.1.2	Channel bandwidths per operating band for CA	83

6.22.1.3	UE Co-existence studies	83

6.22.1.4	∆TIB and ∆RIB values	83

6.22.1.5	REFSENs requirements	84

6.22.1.6	OOB blocking exception requirements	84

6.23	CA_n3-n7		84

6.23.1	Common for 1 band UL and 2 bands UL CA	84

6.23.1.1 Operating bands for CA	84

6.23.1.2	Channel bandwidths per operating band for CA	84

6.23.1.3	UE Co-existence studies	84

6.23.1.4	∆TIB and ∆RIB values	85

6.23.1.5	REFSENs requirements	85

6.23.1.6	OOB blocking exception requirements	86

6.24.1.4	 ∆TIB and ∆RIB values	87

6.24.1.5 REFSENS requirements	87

6.24.1.6	OOB blocking exception requirements	87

6.24.2	Specific for 2 bands UL CA	87

6.24.2.1	Maximum output power for inter-band CA	87

6.24.2.2	UE co-existence studies	87

6.24.2.3	REFSENS requirements	88

6.25.1.4	 ∆TIB and ∆RIB values	90

6.25.2	Specific for 2 bands UL CA	91

6.25.2.1	Maximum output power for inter-band CA	91

6.25.2.2	UE co-existence studies	91

6.25.2.3	REFSENS requirements	92

6.26.1.4	 ∆TIB and ∆RIB values	93

6.26.1.5 REFSENS requirements	94

6.26.1.6	OOB blocking exception requirements	94

6.27.1.3	 UE co-existence studies	94

6.27.1.4	 ∆TIB and ∆RIB values	95

6.27.1.5 	 REFSENS requirements	95

6.27.2	Specific for 2 bands UL CA	96

6.27.2.1	 UE co-existence studies	96

6.27.2.2	 REFSENS requirements	98

6.28.1.4	 ∆TIB and ∆RIB values	99

6.28.1.5 REFSENS requirements	99

6.28.1.6	OOB blocking exception requirements	99

6.28.2	Specific for 2 bands UL CA	99

6.28.2.1	Maximum output power for inter-band CA	99

6.28.2.2	UE co-existence studies	100

6.28.2.3	REFSENS requirements	101

6.29.1.4	 ∆TIB and ∆RIB values	102

6.29.1.5 REFSENS requirements	102

6.29.1.6	OOB blocking exception requirements	102

6.29.2	Specific for 2 bands UL CA	102

6.29.2.1	Maximum output power for inter-band CA	102

6.29.2.2	UE co-existence studies	103

6.29.2.3	REFSENS requirements	104

6.30	CA_n8-n20		104

6.30.1	Common for 1 band UL and 2 bands UL CA	104

6.30.1.1 Operating bands for CA	104

6.30.1.2	Channel bandwidths per operating band for CA	104

6.30.1.3	UE Co-existence studies	104

6.30.1.4	∆TIB and ∆RIB values	105

6.30.1.5	REFSENs requirements	105

6.30.1.6	OOB blocking exception requirements	105

6.31	n7-n77		106

6.31.1	Common for 1 band UL and 2 bands UL CA	106

6.31.1.1	 Operating bands for CA	106

6.31.1.2	 Channel bandwidths per operating band for CA	106

6.31.1.3	 UE co-existence studies	106

6.31.1.4	 ∆TIB and ∆RIB values	106

6.31.1.5 	 REFSENS requirements	107

6.31.1.6 	 OOB blocking exception requirements	107

6.31.2	 Specific for 2 bands UL CA	107

6.31.2.1	Maximum output power for inter-band CA	107

6.31.2.2	 UE co-existence studies	107

6.31.2.3	 REFSENS requirements	108

7	Both bands within FR2 Carrier Aggregation: Specific Band Combination Part	109

8	FR1+FR2 Carrier Aggregation: Specific Band Combination Part	110

8.1	CA_n41-n257		111

8.1.1	Common for 1 band UL and 2 bands UL CA	111

8.1.1.1	Operating bands for CA	111

8.1.1.2	Channel bandwidths per operating band for CA	111

8.1.1.3	∆TIB and ∆RIB values	112

8.1.1.4	REFSENS requirements	112

9	2 bands Dual Connectivity: Specific Band Combination Part	112

9.X	DC_nX-nY		112

9.x.1	Operating bands for DC_nX-nY	112

9.x.2   Configurations for DC_nX-nY	113

9.x.3   Maximum output power for NR-DC	113

9.1	DC_n5-n48		113

9.1.1	Operating bands for DC_n5-n48	113

9.1.2   Configurations for DC_n5-n48	113

9.2	DC_n41-n257		113

9.2.1	Operating bands for DC_n41-n257	113

9.2.2   Configurations for DC_n41-n257	114

Annex A: Change history	115

Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

1	Scope

The present document is a technical report for power class 3 NR inter-band CA and DC for 2 bands DL with up to 2 bands UL under Rel-17 time frame. The purpose is to gather the relevant background information and studies in order to address NR inter-band CA and DC for 2 bands DL with up to 2 bands UL for the Rel-17 band combinations.

This TR contains a general part and band specific combination part. The actual requirements are added to the corresponding technical specifications.

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 38.101-1: "NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone".

[3]	3GPP TS 38.101-2: "NR; User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone".

[4]	3GPP TS 38.101-3: "NR; User Equipment (UE) radio transmission and reception; Part 3:  Range 1 and Range 2 Interworking operation with other radios".

3	Definitions, symbols and abbreviations

3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Aggregated Channel Bandwidth: The RF bandwidth in which a UE transmits and receives multiple contiguously aggregated carriers.

Carrier aggregation: Aggregation of two or more component carriers in order to support wider transmission bandwidths.

Inter-band carrier aggregation: Carrier aggregation of component carriers in different operating bands.

NOTE:	Carriers aggregated in each band can be contiguous or non-contiguous.

3.2	Symbols

For the purposes of the present document, the following symbols apply:

ΔRIB,c	Allowed reference sensitivity relaxation due to support for inter-band CA operation, for serving cell c.

ΔTIB,c	Allowed maximum configured output power relaxation due to support for inter-band CA

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

BS	Base Station

CA	Carrier Aggregation

DC	Dual Connectivity

DL	DownLink

FDD	Frequency Division Duplex

IMD	Inter-modulation

MSD	Maximum Sensitivity Deduction

SCS	Subcarrier spacing

TDD	Time Division Duplex

UE	User Equipment

UL	UpLink

4	Background

The present document is a technical report for NR inter-band CA and DC for 2 bands DL with up to 2 bands UL under Rel-17 time frame. It covers both the UE and BS side. The document is divided in two different parts:

- 	General part: this part covers BS and UE specific which is band combination independent.

- 	Specific band combination part:  this part covers each band combination and its specific issues independently from each other (i.e. one subclause is defined per band combination)

4.1	TR Maintenance

A single company is responsible for introducing all approved TPs in the current TR, i.e. TR editor. However, it is the responsibility of the contact person of each band combination to ensure that the TPs related to the band combination have been implemented.

5	NR Inter-Band Combination: General Part

The general issue for NR inter-band combination including:

1: common issues for both 1 band UL and 2 bands UL NR CA, including the impact of UL/DL harmonic/ harmonic mixing associated with REFSEN,  delta Tib and delta Rib, and OOB blocking exception, etc.

2: 2 bands UL NR CA specific issues, including MSD caused by IMD issue, etc.

3: No new issue for inter-band NR DC combination, the 2 bands UL NR CA specific issues shall be re-used.

6	Both bands within FR1 Carrier Aggregation: Specific Band Combination Part

## 6.1		CA_n5-n25

6.1.1	Common for 1 band UL and 2 bands UL CA

6.1.1.1 Operating bands for CA

Table 6.1.1-1: CA band combination of band n5 + n25

| NR CA Band Combination | NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n5-n25 | n5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | n25 | 1850 MHz | – | 1915 MHz | 1930 MHz | – | 1995 MHz | FDD |

6.1.1.2	Channel bandwidths per operating band for CA

Table 8.x.2-1: Supported bandwidths per CA band combination of band n2+n78

| NR CA configuration / Bandwidth combination set [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA configuration | UL configuration | NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Bandwidth combination set |
| CA_n5A-n25A | CA_n5A-n25A | n5 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n25 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
| CA_n5A-n25(2A) | CA_n5A-n25A | n5 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n25 | See CA_n25(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.1.1.3	Co-existence studies

Table 6.1.1.3-1/2 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_n5-n25.

Table 6.1.1.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n5 | 824 | 849 | 869 | 894 | 1648 | 1698 | 2472 | 2547 |
| n25 | 1850 | 1915 | 1930 | 1995 | 3700 | 3830 | 5550 | 5745 |

Based on above table, there is no harmonic issue for CA_n5-n25.

Table 6.1.1.3-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n5 | 824 | 849 | 869 | 894 | 1738 | 1788 | 2607 | 2682 |
| n25 | 1850 | 1915 | 1930 | 1995 | 3860 | 3990 | 5790 | 5985 |

Based on above table, there is no harmonics mixing issue for CA_n5-n25.

6.1.1.4	∆TIB and ∆RIB values

For CA_n5-n25, the TIB,c and RIB values are given in the tables below.

Table 6.1.1.4-1: ΔTIB,c

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n5-n25 | n5 | 0.3 |
|  | n25 | 0.3 |

Table 6.1.1.4-2: ΔRIB

| Inter-band CA Configuration | NR Band | ΔRIB [dB] |
| --- | --- | --- |
| CA_n5-n25 | n5 | 0 |
|  | n25 | 0 |

6.1.1.5	REFSENs requirements

There is no harmonic issue for the CA combination.

6.1.1.6	OOB blocking exception requirements

There is no OOB exception for the CA combination.

6.1.2		Specific for 2 bands UL CA

6.1.2.1	Maximum output power for inter-band CA

Table 6.1.2.1-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n5A-n25A | 23 | +2/-32 |
| NOTE 2: 2 refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB |  |  |

6.1.2.2		UE co-existence

Table 6.1.2.2-1 gives IMD interference analysis for CA_ n5-n25 with 2 ULs.

Table 6.1.2.2-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 824 | 849 | 1850 | 1915 |
| Two tone 2nd order IMD products | fy_low – fx_high | fy_high – fx_low | fx_low + fy_low | fx_high + fy_high |
| IMD frequency limits (MHz) | 1001 | 1091 | 2674 | 2764 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | 2*fy_low – fx_high | 2*fy_high – fx_low |
| IMD frequency limits (MHz) | 267 | 152 | 2851 | 3006 |
| Two-tone 3rd order IMD products | 2*fx_low + fy_low | 2*fx_high + fy_high | 2*fy_low + fx_low | 2*fy_high + fx_high |
| IMD frequency limits (MHz) | 3498 | 3613 | 4524 | 4679 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | 3*fy_low – fx_high | 3*fy_high – fx_low |
| IMD frequency limits (MHz) | 557 | 697 | 4701 | 4921 |
| Two-tone 4th order IMD products | 3*fx_low + fy_low | 3*fx_high + fy_high | 3*fy_low + fx_low | 3*fy_high + fx_high |
| IMD frequency limits (MHz) | 4322 | 4462 | 6374 | 6569 |
| Two-tone 4th order IMD products | 2*fy_low – 2*fx_high | 2*fy_high – 2*fx_low | 2*fx_low + 2*fy_low | 2*fx_high + 2*fy_high |
| IMD frequency limits (MHz) | 2002 | 2182 | 5348 | 5528 |
| Two-tone 5th order IMD products | ¦4*fx_low – fy_high¦ | ¦4*fx_high – fy_low¦ | 4*fy_low – fx_high | 4*fy_high – fx_low |
| IMD frequency limits (MHz) | 1381 | 1546 | 6551 | 6836 |
| Two-tone 5th order IMD products | 4*fx_low + fy_low | 4*fx_high + fy_high | 4*fy_low + fx_low | 4*fy_high + fx_high |
| IMD frequency limits (MHz) | 5146 | 5311 | 8224 | 8509 |
| Two-tone 5th order IMD products | ¦3*fx_low – 2*fy_high¦ | ¦3*fx_high – 2*fy_low¦ | 3*fy_low – 2*fx_high | 3*fy_high – 2*fx_low |
| IMD frequency limits (MHz) | 1358 | 1153 | 3852 | 4097 |
| Two-tone 5th order IMD products | 2*fx_low + 3*fy_low | 2*fx_high + 3*fy_high | 2*fy_low + 3*fx_low | 2*fy_high + 3*fx_high |
| IMD frequency limits (MHz) | 7198 | 7443 | 6172 | 6377 |

Based on the table 6.x.2.2-1, there is no IMD issue for CA_n5-n25.

Table 6.1.2.2-2 lists the protected bands required for the 2UL bands CA configuration.

Table 6.1.2.2-2: Protected bands for the 2UL bands CA configuration

| NR CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n5-n25 | E-UTRA Band 4, 5, 10, 12, 13, 14, 17, 24, 26, 28, 29, 30, 42, 48, 53, 66, 70, 71, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 41, 43 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 2, 25 | FDL_low | - | FDL_high | -50 | 1 | 4 |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x 180 kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 4: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.1.3.1-1 from the edge of the channel bandwidth. |  |  |  |  |  |  |  |

6.1.2.3		REFSENS requirements

There is no MSD issue for the CA combination.

## 6.2		CA_n71-n78

6.2.1	Common for 1 band UL and 2 bands UL CA

6.2.1.1 Operating bands for CA

Table 6.2.1-1: CA band combination of band n38 + n66

| NR CA Band Combination | NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n71-n78 | n71 | 663 MHz | – | 698 MHz | 617 MHz | – | 652 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

6.2.1.2	Channel bandwidths per operating band for CA

Table 8.x.2-1: Supported bandwidths per CA band combination of band n2+n78

| NR CA configuration / Bandwidth combination set [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA configuration | UL configuration | NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Bandwidth combination set |
| CA_n71A-n78A | CA_n71A-n78A | n71 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
| CA_n71A-n78(2A) | CA_n71A-n78A | n71 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | See CA_n78(2A) Bandwidth Combination Set 2 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.2.1.3	Co-existence studies

DC_71A_n78A is defined in Rel-16. Co-existence studies for DC_71A_n78A can be reused for CA_n71-n78.

6.2.1.4	∆TIB and ∆RIB values

For CA_n71-n78, the TIB,c and RIB values are given in the tables below.

Table 6.2.1.4-1: ΔTIB,c

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n71-n78 | n71 | 0.5 |
|  | n78 | 0.8 |

Table 6.2.1.4-2: ΔRIB

| Inter-band CA Configuration | NR Band | ΔRIB [dB] |
| --- | --- | --- |
| CA_n71-n78 | n71 | 0.2 |
|  | n78 | 0.5 |

6.2.1.5	REFSENs requirements

There is no harmonic issue for the CA combination.

6.2.1.6	OOB blocking exception requirements

Table 6.2.1.6-1: CA band combination with exceptions allowed

CA band combination

CA_n71-n78

6.2.2		Specific for 2 bands UL CA

6.2.2.1	Maximum output power for inter-band CA

Table 6.2.2.1-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n71A-n78A | 23 | +2/-32 |
| NOTE 2: 2 refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB |  |  |

6.2.2.2		UE co-existence

Table 6.2.2.2-2 lists the protected bands required for the 2UL bands CA configuration.

Table 6.2.2.2-2: Protected bands for the 2UL bands CA configuration

| NR CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n71-n78 | E-UTRA Band 5, 26 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 41 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.2.3.1-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x 180 kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. |  |  |  |  |  |  |  |

6.2.2.3		REFSENS requirements

DC_71A_n78A is defined in existing Rel-16 TS 38.101-3. Two-tone 5th order IMD products may fall into the own Rx Band of Band n71. The MSD value can be reused from DC_71A_n78A.

Table 6.2.2.3: 2DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n71A-n78A CA_n71A-n78(2A) | n71 | 681.5 | 5 | 25 | 635.5 | 5.5 | FDD | IMD5 |
|  | n78 | 3361.5 | 10 | 50 | 3582.5 | N/A | TDD | N/A |

6.3		CA_n7-n66

6.3.1	Common for 1 band UL and 2 bands UL CA

6.3.1.1	Operating bands for CA

Table 6.3.1.1-1: CA band combination of band n7+n66

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
| n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |

6.3.1.2	Channel bandwidths per operating band for CA

Table 6.3.1.2-1: Supported bandwidths per CA band combination of band n7+n66

| CA operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 80 | 90 | 100 | Bandwidth combination set |
| CA_n7A-n66A | CA_n7A-n66A | n7 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  | 1 |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | n66 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
| CA_n7A-n66(2A) | CA_n7A-n66A | n7 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | n66 | See CA_n66(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n7(2A)-n66 | CA_n7A-n66A | n7 | See CA_n7(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n66 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
| CA_n7(2A)-n66(2A) | CA_n7A-n66A | n7 | See CA_n7(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n66 | See CA_n66(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.3.1.3	UE co-existence studies

Table 6.3.1.3-1/2 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_n7-n66.

Table 6.3.1.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | nth Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n7 | 2500 | 2570 | 2620 | 2690 | 5000 | 5140 | 7500 | 7710 |  |  |
| n66 | 1710 | 1780 | 2110 | 2200 | 3420 | 3560 | 5130 | 5340 |  |  |

Table 6.3.1.3-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | mth Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n7 | 2500 | 2570 | 2620 | 2690 | 5240 | 5380 | 7860 | 8070 |  |  |
| n66 | 1710 | 1780 | 2110 | 2200 | 4220 | 4400 | 6330 | 6660 |  |  |

Based on above table, there is no harmonic issue for the band combination of n7 and n66.

Based on above table, there is no harmonic mixing issue for the band combination of n7 and n66.

6.3.1.4	∆TIB and ∆RIB values

For CA_n7-n66 , the TIB,c and RIB,c values are given in the tables below.

Table 6.3.1.4-1: ΔTIB,c

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n7-n66 | n7 | 0.5 |
|  | n66 | 0.5 |

Table 6.1.x.4-2: ΔRIB,c

| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n7-n66 | n7 | 0.5 |
|  | n66 | 0.5 |

6.3.1.5	REFSENS requirements

There are no specific REFSENS requirements for 1 band UL.

6.3.1.6	OOB blocking exception requirements

There is no OOB blocking exception for this CA band combination.

6.3.2	Specific for 2 bands UL CA

6.3.2.1	Maximum output power for inter-band CA

Table 6.3.2.2-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n7A-n66A | 23 | +2/-32 |
| NOTE 2: 2 refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB |  |  |

6.3.2.2	UE co-existence studies

Table 6.3.2.1-1 lists Band n7 +Band n66 2UL bands CA  2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.3.2.2-1: Band n7 and Band n66 UL IMD products

| UE UL carriers | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| UL frequencies (MHz) | 1710 | 1780 | 2500 | 2570 |
| 2nd order IMD products | f2_low – f1_high | f2_high – f1_low | f2_low + f1_low | f2_high + f1_high |
| IMD frequency limit (MHz) | 720 | 860 | 4210 | 4350 |
| 3rd order IMD products | 2*f1_low – f2_high | 2*f1_high – f2_low | 2*f2_low – f1_high | 2*f2_high – f1_low |
| IMD frequency limit (MHz) | 850 | 1060 | 3220 | 3430 |
| 3rd order IMD products | 2*f1_low + f2_low | 2*f1_high + f2_high | 2*f2_low + f1_low | 2*f2_high + f1_high |
| IMD frequency limit (MHz) | 5920 | 6130 | 6710 | 6920 |
| 4th order IMD products | 3*f1_low – f2_high | 3*f1_high – f2_low | 3*f2_low – f1_high | 3*f2_high – f1_low |
| IMD frequency limit (MHz) | 2560 | 2840 | 5720 | 6000 |
| 4th order IMD products | 3*f1_low + f2_low | 3*f1_high + f2_high | 3*f2_low + f1_low | 3*f2_high + f1_high |
| IMD frequency limit (MHz) | 7630 | 7910 | 9210 | 9490 |
| 4th order IMD products | 2*f1_low – 2*f2_high | 2*f1_high – 2*f2_low | 2*f1_low + 2*f2_low | 2*f1_high + 2*f2_high |
| IMD frequency limit (MHz) | -1720 | -1440 | 8420 | 8700 |
| 5th order IMD products | f1_low – 4*f2_high | f1_high – 4*f2_low | f2_low – 4*f1_high | f2_high – 4*f1_low |
| IMD frequency limit (MHz) | -8570 | -8220 | -4620 | -4270 |
| 5th order IMD products | f1_low + 4*f2_low | f1_high + 4*f2_high | f2_low + 4*f1_low | f2_high + 4*f1_high |
| IMD frequency limit (MHz) | 11710 | 12060 | 9340 | 9690 |
| 5th order IMD products | 2*f1_low – 3*f2_high | 2*f1_high - 3*f2_low | 2*f2_low – 3*f1_high | 2*f2_high – 3*f1_low |
| IMD frequency limit (MHz) | -4290 | -3940 | -340 | 10 |
| 5th order IMD products | 2*f1_low + 3*f2_low | 2*f1_high + 3*f2_high | 2*f2_low + 3*f1_low | 2*f2_high + 3*f1_high |
| IMD frequency limit (MHz) | 10920 | 11270 | 10130 | 10480 |

Based on the table above, the 4th order IMD may fall into Rx frequencies of band n7.

Table 6.3.2.2-2 lists the protected bands required for the 2UL bands CA configuration.

Table 6.3.2.2-2: Protected bands for the 2UL bands CA configuration

| UL NR CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n7-n66 | E-UTRA Band 2, 4, 5, 7, 10, 12, 13, 14, 17, 26, 27, 28, 29, 30, 43, 50, 51, 66, 74, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 42, 48 NR band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 2570 | - | 2575 | +1.6 | 5 | 15, 21, 26 |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 15, 21, 26 |
|  | Frequency range | 2595 | - | 2620 | -40 | 5 | 15, 21 |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.5.3.1-2 are permitted for each assigned NR carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x RBsize kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 15: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth. NOTE 21: This requirement is applicable for any channel bandwidths within the range 2500 - 2570 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 2560.5 - 2562.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 2552 - 2560 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. NOTE 26: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. |  |  |  |  |  |  |  |

6.3.2.3	REFSENS requirements

Table 6.3.2.3-1 lists the MSD required due to the 4th IMD for the dual uplink configuration The MSD value is reused from E-UTRA CA_4-7.

Table 6.3.2.3-1: MSD due to IMD issue

| Operating band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA Configuration | Operating band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n7A-n66A | n7 | 2535 | 5 | 25 | 2655 | 15 | FDD | IMD4 |
|  | n66 | 1730 | 5 | 25 | 2130 | N/A | TDD | N/A |

6.8		CA_n25-38

6.4.1		Common for 1 band UL and 2 bands UL CA

6.4.1.2	Channel bandwidths per operating band for CA

Table 6.4.1.1-1: CA band combination of band n25+n38

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n25 | 1850 MHz | – | 1915 MHz | 1930 MHz | – | 1995 MHz | FDD |
| n38 | 2570 MHz | – | 2620 MHz | 2570 MHz | – | 2620 MHz | TDD |

6.4.1.2	Channel bandwidths per operating band for CA

Table 6.4.1.2-1: Supported bandwidths per CA band combination of band n25+n38

| CA operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 80 | 90 | 100 | Bandwidth combination set |
| CA_n25A-n38A | CA_n25A-n38A | n25 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | n38 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
| CA_n25(2A)-n38A | CA_n25A-n38A | n25 | See CA_n25(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n38 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |

6.4.1.3	UE co-existence studies

Table 6.4.1.3-1 summarizes frequency ranges where harmonics occur for CA_n25-n38.

Table 6.4.1.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | nth Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n25 | 1850 | 1915 | 1930 | 1995 | 3700 | 3830 | 5550 | 5745 |  |  |
| n38 | 2570 | 2620 | 2570 | 2620 | 5140 | 5240 | 7710 | 7860 |  |  |

Based on above table, there is no harmonic issue for the band combination of n25 and n38.

Table 6.4.1.3-2 summarizes frequency ranges where harmonics mixing occur for CA_n25-n38.

Table 6.4.1.3-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | mth Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n25 | 1850 | 1915 | 1930 | 1995 | 3700 | 3830 | 5550 | 5745 |  |  |
| n38 | 2570 | 2620 | 2570 | 2620 | 5140 | 5240 | 7710 | 7860 |  |  |

Based on above table, there is no harmonic mixing issue for the band combination of n25 and n38.

6.4.1.4	∆TIB and ∆RIB values

For CA_n25-n38 , the ∆TIB,c and ∆RIB,c values are given according to the general framework for the high-high band combination.

Table 6.4.1.4-1: ΔTIB,c

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n25-n38 | n25 | 0.5 |
|  | n38 | 0.5 |

Table 6.4.1.4-2: ΔRIB,c

| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n25-n38 | n25 | 0 |
|  | n38 | 0 |

6.4.1.5	REFSENS requirements

Since the UE RF architecture for CA_n25-n38 is similar to CA_n25-n41, MSD due to cross band isolation defined for CA_n25-n41 is assumed.

Table 6.4.2.3-1: Reference sensitivity exceptions (MSD) due to cross band isolation for NR CA FR1

| NR Band / Channel bandwidth of the affected DL band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 70 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| n38 | n25 | 0.6 | 0.6 | 0.6 | 0.6 | 0.6 | 0.6 | 0.6 |  |  |  |  |  |  |

Table 6.4.2.3-2: Uplink configuration for reference sensitivity exceptions due to cross band isolation for NR CA FR1

| NR Band / SCS / Channel bandwidth of the affected DL band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | SCS of UL band (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| n38 | n25 | 15 | 160 | 160 | 160 | 160 | 160 | 160 | 160 |  |  |  |  |  |  |

6.4.1.6	OOB blocking exception requirements

There is no OOB blocking exception for this CA band combination.

6.4.2	Specific for 2 bands UL CA

6.4.2.1	Maximum output power for inter-band CA

Table 6.4.2.2-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_25A-n38A | 23 | +2/-32 |
| NOTE 2: 2 refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB |  |  |

6.4.2.2	UE co-existence studies

Table 6.4.2.2-1 lists Band n25 +Band n38 2UL bands CA  2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.4.2.2-1: Band n25 and Band n38 UL harmonics and IMD products

| UE UL carriers | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| UL frequencies (MHz) | 1850 | 1915 | 2570 | 2620 |
| 2nd order IMD products | f2_low – f1_high | f2_high – f1_low | f2_low + f1_low | f2_high + f1_high |
| IMD frequency limit (MHz) | 655 | 770 | 4420 | 4535 |
| 3rd order IMD products | 2*f1_low – f2_high | 2*f1_high – f2_low | 2*f2_low – f1_high | 2*f2_high – f1_low |
| IMD frequency limit (MHz) | 1080 | 1260 | 3225 | 3390 |
| 3rd order IMD products | 2*f1_low + f2_low | 2*f1_high + f2_high | 2*f2_low + f1_low | 2*f2_high + f1_high |
| IMD frequency limit (MHz) | 6270 | 6450 | 6990 | 7155 |
| 4th order IMD products | 3*f1_low – f2_high | 3*f1_high – f2_low | 3*f2_low – f1_high | 3*f2_high – f1_low |
| IMD frequency limit (MHz) | 2930 | 3175 | 5795 | 6010 |
| 4th order IMD products | 3*f1_low + f2_low | 3*f1_high + f2_high | 3*f2_low + f1_low | 3*f2_high + f1_high |
| IMD frequency limit (MHz) | 8120 | 8365 | 9560 | 9775 |
| 4th order IMD products | 2*f1_low – 2*f2_high | 2*f1_high – 2*f2_low | 2*f1_low + 2*f2_low | 2*f1_high + 2*f2_high |
| IMD frequency limit (MHz) | -1540 | -1310 | 8840 | 9070 |
| 5th order IMD products | f1_low – 4*f2_high | f1_high – 4*f2_low | f2_low – 4*f1_high | f2_high – 4*f1_low |
| IMD frequency limit (MHz) | -8630 | -8365 | -5090 | -4780 |
| 5th order IMD products | f1_low + 4*f2_low | f1_high + 4*f2_high | f2_low + 4*f1_low | f2_high + 4*f1_high |
| IMD frequency limit (MHz) | 12130 | 12395 | 9970 | 10280 |
| 5th order IMD products | 2*f1_low – 3*f2_high | 2*f1_high - 3*f2_low | 2*f2_low – 3*f1_high | 2*f2_high – 3*f1_low |
| IMD frequency limit (MHz) | -4160 | -3880 | -605 | -310 |
| 5th order IMD products | 2*f1_low + 3*f2_low | 2*f1_high + 3*f2_high | 2*f2_low + 3*f1_low | 2*f2_high + 3*f1_high |
| IMD frequency limit (MHz) | 11410 | 11690 | 10690 | 10985 |

Based on the table above, no IMD issue is identified for this band combination.

Table 6.4.2.2-2 lists the protected bands required for the 2UL bands CA configuration.

Table 6.4.2.2-2: Protected bands for the 2UL bands CA configuration

| UL NR CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n25-n38 | E-UTRA Band 4, 5, 10, 12, 13, 14, 17, 27, 29, 30, 42, 48, 66, 71, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | E-UTRA Band 25 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | E-UTRA Band 43, 48 NR band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 2620 | - | 2645 | -15.5 | 5 | 15, 21, 26 |
|  | Frequency range | 2645 | - | 2690 | -40 | 1 | 15, 21 |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.5.3.1-2 are permitted for each assigned NR carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x RBsize kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 15: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth. NOTE 21: This requirement is applicable for any channel bandwidths within the range 2500 - 2570 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 2560.5 - 2562.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 2552 - 2560 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. NOTE 26: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. |  |  |  |  |  |  |  |

6.4.2.3	REFSENS requirements

There is no specific REFSENS requirement for 2 band UL.

6.5		CA_n1-n77

6.5.1		Common for 1 band UL and 2 bands UL CA

6.5.1.1			Operating bands for CA

Table 6.5.1.1-1: CA band combination of band n1+n77

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
| n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

6.5.1.2			Channel bandwidths per operating band for CA

Table 6.5.1.2-1: Supported bandwidths per CA band combination of band n1+n77

| NR CA configuration | Uplink CA configuration | NR Band | SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n1A-n77A | CA_n1A-n77A | n1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |

6.5.1.3			UE co-existence studies

This section is skipped since CA_n1-n77 without 2UL CA has been already specified in TS 38.101-1, and impact of harmonic issue has been studied in TR 38.716-02-00.

6.5.1.4			∆TIB and ∆RIB values

For CA_n1-n77 , the ΔTIB,c and ΔRIB,c values are given in the tables below. The same values are shown as what are specified in TS 38.101-1.

Table 6.5.1.4-1: ΔTIB,c

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n1-n77 | n1 | 0.6 |
|  | n77 | 0.8 |

Table 6.5.1.4-2: ΔRIB,c

| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n1-n77 | n1 | 0.2 |
|  | n77 | 0.5 |

6.5.1.5			REFSENS requirements

This section is skipped since CA_n1-n77 without 2UL CA has been already specified in TS 38.101-1, and impact of harmonic issue has been studied in TR 38.716-02-00.

6.5.1.6			OOB blocking exception requirements

This section is skipped since CA_n1-n77 without 2UL CA has been already specified.

6.5.2		Specific for 2 bands UL CA

6.5.2.1			Maximum output power for inter-band CA

Table 6.5.2.1-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n1A-n77A | 23 | +2/-32 |
| NOTE 2: 2 refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB |  |  |

6.5.2.2			UE co-existence studies

Table 6.5.2.2-1 lists Band n1 +Band n77 2UL bands CA  2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.5.2.2-1: Band n1 and Band n77 UL IMD products

| UE UL carriers | fx_low | fx_high |  | fy_low | fy_high |  |
| --- | --- | --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ |  | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 1320 – 2280 |  |  | 5220 – 6180 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ |  | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |  |
| IMD frequency limits (MHz) | 360 – 660 |  |  | 4620 – 6480 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ |  | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 1920 – 1980 |  |  | 3300 – 4200 |  |  |
| Two-tone 3rd order IMD products | (fx_low – max BW fy) |  | (fx_high + max BW fy) | (fy_low – max BW fx) |  | (fy_high + max BW fx) |
| IMD frequency limits (MHz) | 1820 – 2080 |  |  | 3280 – 4220 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ |  | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ |  | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1560 – 2640 |  |  | 7920 – 10680 |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ |  | ¦2*fx_high –2* fy_low¦ |  |  |  |
| IMD frequency limits (MHz) | 2640 – 4560 |  |  |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ |  | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ |  | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 9060 – 10140 |  |  | 11820 – 14580 |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ |  | ¦2*fx_high +2* fy_high¦ |  |  |  |
| IMD frequency limits (MHz) | 10440 – 12360 |  |  |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ |  | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ |  | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 11220 – 14880 |  |  | 3480 – 4620 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ |  | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ |  | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 5940 – 8760 |  |  | 660 – 2640 |  |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ |  | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ |  | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15120 – 18780 |  |  | 10980 – 12120 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ |  | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ |  | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 13740 – 16560 |  |  | 12360 – 14340 |  |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. |  |  |  |  |  |  |

Based on above Table , it can be seen that

- 2nd order IMD products may fall into Rx frequencies of bands 1, 2, 3, 4, 9, 10, 11, 21, 23, 24, 25, 32, 33, 34, 35, 36, 37, 39, 45, 46, 47, 50, 51, 65, 66, 70, 74, 75, 76, n91, n92, n93, n94

- 3rd order IMD products may fall into Rx frequencies of bands  2, 3, 9, 22, 25, 31, 33, 34, 35, 36, 37, 39, 42, 43, 46, 47, 48, 49, 52, 70, 71, 72, 73, n77, n78, n79, n87, n88

- 4th order IMD products may fall into Rx frequencies of bands 1, 2, 3, 4, 7, 9, 10, 22, 23, 25, 30, 33, 34, 35, 36, 37, 38, 39, 40, 41, 42, 43, 48, 49, 52, 53, 65, 66, 69, 70, n77, n78, n79, n90

- 5th order IMD products may fall into Rx frequencies of bands 1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 17, 18, 19, 20, 21, 22, 23, 24, 25, 26, 27, 28, 29, 30, 32, 33, 34, 35, 36, 37, 38, 39, 40, 41, 42, 43, 44, 45, 48, 49, 50, 51, 53, 65, 66, 67, 68, 69, 70, 74, 75, 76, n77, n78, n79, 85, n90, n91, n92 ,n93, n94

Self-interference to own Rx frequencies are shown as red.

Table 6.5.2.2-2 lists the protected bands required for the 2UL bands CA configuration.

Table 6.5.2.2-2: Protected bands for the 2UL bands CA configuration

| UL NR CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n1-n77 | E-UTRA Band 1, 5, 7, 8, 11, 18, 19, 20, 21, 26, 27, 28, 40, 41, 65, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3, 34 | FDL_low | - | FDL_high | -50 | 1 | 4 |
|  | Frequency range | 1880 | - | 1895 | -40 | 1 | 4, 6 |
|  | Frequency range | 1895 | - | 1915 | -15.5 | 5 | 4, 6, 7 |
|  | Frequency range | 1915 | - | 1920 | +1.6 | 5 | 4, 6, 7 |
| NOTE 4: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth. NOTE 6: This requirement is applicable for any channel bandwidths within the range 1920 – 1980 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 1927.5 - 1929.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 1930 – 1938 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. NOTE 7: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. |  |  |  |  |  |  |  |

6.5.2.3			REFSENS requirements

MSD for harmonic issues is already covered by the study for fallback combinations of 1UL/2DL. Therefore, here we focus on MSD for IMD issues.

Table 6.5.2.3-1 lists the MSD required due to 2nd and 4th  IMD for the dual uplink configuration. We use same MSD as DC_1_n77 specified in TS 38.101-3. Note that no MSD for 3rd, 4th and 5th IMD to n77 is specified in TS 38.101-3, and then we follow the result.

Table 6.5.2.3-1: MSD due to IMD issue

| Operating band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA Configuration | Operating band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n1A-n77A | 1 | 1950 | 5 | 25 | 2140 | 29.8 | FDD | IMD24 |
|  |  |  |  |  |  | 32.5 5 |  |  |
|  | n77 | 4090 | 10 | 50 | 4090 | N/A | TDD | N/A |
| CA_n1A-n77A | 1 | 1950 | 5 | 25 | 2140 | 8.0 | FDD | IMD44 |
|  |  |  |  |  |  | 10.75 |  |  |
|  | n77 | 3710 | 10 | 50 | 3710 | N/A | TDD | N/A |
| NOTE 4: This band is subject to IMD5 also which MSD is not specified. NOTE 5: Applicable only if operation with 4 antenna ports is supported in the band with EN-DC configured. |  |  |  |  |  |  |  |  |

6.6	CA_n77-n79

6.6.1		Common for 1 band UL and 2 bands UL CA

6.6.1.1			Operating bands for CA

Table 6.6.1.1-1: CA band combination of band n77+n79

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |
| n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

6.6.1.2			Channel bandwidths per operating band for CA

Table 6.6.1.2-1: Supported bandwidths per CA band combination of band n77+n79

| NR CA configuration | Uplink CA configuration | NR Band | SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n77A-n79A | CA_n77A-n79A | n77 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  | n79 | 15 |  |  |  |  |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  |  |  |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |
|  |  |  | 60 |  |  |  |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |

6.6.1.3			UE co-existence studies

This section is skipped since CA_n77-n79 without 2UL CA has been already specified in TS 38.101-1, and impact of harmonic issue has been discussed in TR 37.865-01-01, where n77 and n79 is assumed as synchronous operation.

6.6.1.4			∆TIB and ∆RIB values

For CA_n77-n79 , the ΔTIB,c and ΔRIB,c values are given in the tables below. The same values are shown as what are specified in TS 38.101-1.

Table 6.6.1.4-1: ΔTIB,c

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n77-n79 | n77 | 0.5 |
|  | n79 | 0.5 |

Table 6.6.1.4-2: ΔRIB,c

| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n77-n79 | n77 | 0 |
|  | n79 | 0 |

6.6.1.5			REFSENS requirements

This section is skipped since CA_n77-n79 without 2UL CA has been already specified in TS 38.101-1, and impact of harmonic issue has been discussed in TR 37.865-01-01, where n77 and n79 is assumed as synchronous operation.

6.6.1.6			OOB blocking exception requirements

This section is skipped since CA_n77-n79 without 2UL CA has been already specified.

6.6.2		Specific for 2 bands UL CA

6.6.2.1			Maximum output power for inter-band CA

Table 6.6.2.1-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n77A-n79A | 23 | +2/-32 |
| NOTE 2: 2 refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB |  |  |

6.6.2.2			UE co-existence studies

Table 6.6.2.2-1 lists Band n77 +Band n79 2UL bands CA  2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.6.2.2-1: Band n77 and Band n79 UL IMD products

| UE UL carriers | fx_low | fx_high |  | fy_low | fy_high |  |
| --- | --- | --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ |  | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 200 – 1700 |  |  | 7700 – 9200 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ |  | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |  |
| IMD frequency limits (MHz) | 1600 – 4000 |  |  | 4600 – 6700 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ |  | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 11000 – 13400 |  |  | 12100 – 14200 |  |  |
| Two-tone 3rd order IMD products | (fx_low – max BW fy) |  | (fx_high + max BW fy) | (fy_low – max BW fx) |  | (fy_high + max BW fx) |
| IMD frequency limits (MHz) | 3200 – 4300 |  |  | 4300 – 5100 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ |  | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ |  | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 4900 – 8200 |  |  | 9000 – 11700 |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ |  | ¦2*fx_high –2* fy_low¦ |  |  |  |
| IMD frequency limits (MHz) | 400 – 3400 |  |  |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ |  | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ |  | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 14300 – 17600 |  |  | 16500 – 19200 |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ |  | ¦2*fx_high +2* fy_high¦ |  |  |  |
| IMD frequency limits (MHz) | 15400 – 18400 |  |  |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ |  | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ |  | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 13400 – 16700 |  |  | 8200 – 12400 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ |  | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ |  | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 4800 – 8400 |  |  | 100 – 3800 |  |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ |  | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ |  | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 20900 – 24200 |  |  | 17600 – 21800 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ |  | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ |  | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 19800 – 23400 |  |  | 18700 – 22600 |  |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. |  |  |  |  |  |  |

Based on above Table, it can be seen that

- 2nd order IMD products may fall into Rx frequencies of bands  5, 6, 8, 11, 12, 13, 14, 17, 18, 19, 20, 21, 24, 26, 27, 28, 29, 32, 44, 45, 50, 51, 67, 68, 71, 72, 73, 74, 75, 76, 85, n91, n92, n93, n94

- 3rd order IMD products may fall into Rx frequencies of bands 1, 2, 3, 4, 7, 9, 10, 22, 23, 25, 30, 34, 35, 36, 37, 38, 39, 40, 41, 42, 43, 46, 47, 48, 49, 52, 53, 65, 66, 69, 70, n77, n78, n79, n90

- 4th order IMD products may fall into Rx frequencies of bands 1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 17, 18, 19, 20, 21, 23, 24, 25, 26, 27, 28, 29, 30, 31, 32, 33, 34, 35, 36, 37, 38, 39, 40, 41, 44, 45, 46, 47, 50, 51, 52, 53, 65, 66, 67, 68, 69, 70, 71, 72, 73, 74, 75, 76, n77, n78, n79, 85, 87, 88, n90, n91, n92, n93, n94

- 5th order IMD products may fall into Rx frequencies of bands 1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 17, 18, 19, 20, 21, 22, 23, 24, 25, 26, 27, 28, 29, 30, 31, 32, 33, 34, 35, 36, 37, 38, 39, 40, 41, 42, 43, 44, 45, 46, 47, 48, 49, 50, 51, 52, 53, 65, 66, 67, 68, 69, 70, 71, 72, 73, 74, 75, 76, n77, n78, n79, 85, 87, 88, n90, n91, n92, n93, n94

Table 6.6.2.2-2 lists the protected bands required for the 2UL bands CA configuration.

Table 6.6.2.2-2: Protected bands for the 2UL bands CA configuration

| UL NR CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n77-n79 | E-UTRA Band 1, 3, 5, 8, 11, 18, 19, 21, 28, 34, 40, 41, 65, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| NOTE 3: Applicable when co-existence with PHS system operating in 1884.5 - 1915.7 MHz. |  |  |  |  |  |  |  |

6.6.2.3			REFSENS requirements

Considering both n77 and n79 are TDD bands and synchronous operation for CA_n77-n79 is assumed, no MSD needed for dual UL of CA_n77-n79.

6.7	CA_n78-n79

6.7.1		Common for 1 band UL and 2 bands UL CA

6.7.1.1		Operating bands for CA

Table 6.7.1.1-1: CA band combination of band n78+n79

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
| n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

6.7.1.2		Channel bandwidths per operating band for CA

Table 6.7.1.2-1: Supported bandwidths per CA band combination of band n78+n79

| NR CA configuration | Uplink CA configuration | NR Band | SCS (kHz) | 5 MHz |  |  | 10 MHz |  | 15 MHz |  | 20 MHz |  | 25 MHz |  | 30 MHz | 40 MHz |  |  | 50 MHz |  | 60 MHz |  | 70 MHz |  | 80 MHz |  | 90 MHz |  | 100 MHz | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n78A-n79AX | CA_n78A-n79AY | n78 | 15 |  |  |  | Yes |  | Yes |  | Yes |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  |  |  | Yes |  | Yes |  | Yes |  |  |  |  | Yes |  |  | Yes |  | Yes |  |  |  | Yes |  | Yes |  | Yes |  |
|  |  |  | 60 |  |  |  | Yes |  | Yes |  | Yes |  |  |  |  | Yes |  |  | Yes |  | Yes |  |  |  | Yes |  | Yes |  | Yes |  |
|  |  | n79 | 15 |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  | Yes |  |  |  | Yes |  |  |  | Yes |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  | Yes |  |  |  | Yes |  |  |  | Yes |  |
|  |  | n78 | 15 |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  | 1 |
|  |  |  | 30 |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes | Yes |  |  | Yes |  | Yes |  |  |  | Yes |  | Yes |  | Yes |  |
|  |  |  | 60 |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  | Yes | Yes |  |  | Yes |  | Yes |  |  |  | Yes |  | Yes |  | Yes |  |
|  |  | n79 | 15 |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  | Yes |  |  |  | Yes |  |  |  | Yes |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  | Yes |  |  |  | Yes |  |  |  | Yes |  |
| CA_ n78(2A)-n79A | CA_n78A-n79A | n78 | See CA_n78(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n79 | 15 |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes | Yes |  | Yes |  |  |  | Yes |  |  |  | Yes |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes | Yes |  | Yes |  |  |  | Yes |  |  |  | Yes |  |  |
| NOTE X: Simultaneous Rx/Tx capability does not apply for UEs supporting band n78 with a n77 implementation. NOTE Y: Simultaneous Rx/Tx capability does not apply to uplink CA transmission, and only apply to single uplink transmission. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.7.1.3		UE co-existence studies

This section is skipped since CA_n78-n79 without 2UL CA has been already specified in TS 38.101-1, and impact of harmonic issue has been discussed in TR 37.865-01-01.

Furthermore, ΔTIB and ΔRIB and MSD requirements have been specified for asynchronous operation in TS 38.101-1.

6.7.1.4		∆TIB and ∆RIB values

For CA_n77-n79 , the ΔTIB,c and ΔRIB,c values are given in the tables below. The same values are shown as what are specified in TS 38.101-1.

Table 6.7.1.4-1: ΔTIB,c

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n78-n79 | n78 | 0.5 |
|  |  | 1.58 |
|  | n79 | 0.5 |
|  |  | 1.58 |
| NOTE 8: The requirements only apply for UE supporting inter-band carrier aggregation with simultaneous Rx/Tx capability, and NR UL carrier frequencies are confined to 3700 MHz-3800MHz for n78 and 4400 MHz-4500MHz for n79. Simultaneous Rx/Tx capability does not apply for UEs supporting band n78 with a n77 implementation. |  |  |

Table 6.7.1.4-2: ΔRIB,c

| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n78-n79 | n78 | 0 |
|  | n79 | 0 |

6.7.1.5		REFSENS requirements

The MSD due to cross band isolation are specified below for n78 25MHz and 30MHz.

Table 6.7.1.5-1: Reference sensitivity exceptions (MSD) due to cross band isolation for NR CA FR1

| NR Band / Channel bandwidth of the affected DL band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 70 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| n781 | n79 |  |  |  |  |  |  | 2 | 2 | 2 |  | 2 |  | 2 |
| n79 | n781 |  | 2.6 | 2.6 | 2.6 | 2.6 | 2.6 | 2.6 | 2.6 | 2.6 |  | 2.6 | 2.6 | 2.6 |
| NOTE 1: The requirements only apply for UEs supporting inter-band carrier aggregation with simultaneous Rx/Tx capability. Simultaneous Rx/Tx capability does not apply for UEs supporting band n78 with a n77 implementation. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.7.1.5-2: Uplink configuration for reference sensitivity exceptions due to cross band isolation for NR CA FR1

| NR Band / SCS / Channel bandwidth of the affected DL band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | SCS of UL band (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| n783 | n79 | 30 |  |  |  |  |  | 270 | 270 | 270 | 270 |  | 270 |  | 270 |
| n79 | n783 | 30 |  | 270 | 270 | 270 | 270 | 270 | 270 | 270 | 270 |  | 270 | 270 | 270 |
| NOTE 1: The UL configuration applies regardless of the channel bandwidth of the UL band unless the UL resource blocks exceed that specified in Table 7.3.2-3 for the uplink bandwidth in which case the allocation according to Table 7.3.2-3 applies. NOTE 2: Refers to the UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth in Table 5.3.2-1. NOTE 3: The requirements only apply for UEs supporting inter-band carrier aggregation with simultaneous Rx/Tx capability. Simultaneous Rx/Tx capability does not apply for UEs supporting band n78 with a n77 implementation. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

This section is skipped since CA_n78-n79 without 2UL CA has been already specified in TS 38.101-1, and impact of harmonic issue has been discussed in TR 37.865-01-01.

Furthermore, ΔTIB and ΔRIB and MSD requirements have been specified for asynchronous operation in TS 38.101-1.

6.7.1.6		OOB blocking exception requirements

This section is skipped since CA_n78-n79 without 2UL CA has been already specified.

6.7.2		Specific for 2 bands UL CA

6.7.2.1		Maximum output power for inter-band CA

Table 6.7.2.1-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n78A-n79A | 23 | +2/-32 |
| NOTE 2: 2 refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB |  |  |

6.7.2.2		UE co-existence studies

Table 6.7.2.2-1 lists Band n78 +Band n79 2UL bands CA  2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.7.2.2-1: Band n78 and Band n79 UL IMD products

| UE UL carriers | fx_low | fx_high |  | fy_low | fy_high |  |
| --- | --- | --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ |  | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 600 – 1700 |  |  | 7700 – 8800 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ |  | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |  |
| IMD frequency limits (MHz) | 1600 – 3200 |  |  | 5000 – 6700 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ |  | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 11000 – 12600 |  |  | 12100 – 13800 |  |  |
| Two-tone 3rd order IMD products | (fx_low – max BW fy) |  | (fx_high + max BW fy) | (fy_low – max BW fx) |  | (fy_high + max BW fx) |
| IMD frequency limits (MHz) | 3200 – 3900 |  |  | 4300 – 5100 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ |  | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ |  | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 4900 – 7000 |  |  | 9400 – 11700 |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ |  | ¦2*fx_high –2* fy_low¦ |  |  |  |
| IMD frequency limits (MHz) | 1200 – 3400 |  |  |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ |  | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ |  | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 14300 – 16400 |  |  | 16500 – 18800 |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ |  | ¦2*fx_high +2* fy_high¦ |  |  |  |
| IMD frequency limits (MHz) | 15400 – 17600 |  |  |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ |  | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ |  | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 13800 – 16700 |  |  | 8200 – 10800 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ |  | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ |  | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 5600 – 8400 |  |  | 100 – 2600 |  |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ |  | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ |  | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 20900 – 23800 |  |  | 17600 – 20200 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ |  | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ |  | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 19800 – 22600 |  |  | 18700 – 21400 |  |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. |  |  |  |  |  |  |

Based on above Table, it can be seen that

- 2nd order IMD products may fall into Rx frequencies of bands  5, 6, 8, 11, 12, 13, 14, 17, 18, 19, 20, 21, 24, 26, 27, 28, 29, 32, 44, 45, 50, 51, 67, 68, 71, 74, 75, 76, 85, n91, n92, n93, n94

- 3rd order IMD products may fall into Rx frequencies of bands 1, 2, 3, 4, 7, 9, 10, 22, 23, 25, 30, 33, 34, 35, 36, 37, 38, 39, 40, 41, 42, 43, 46, 47, 48, 49, 52, 53, 65, 66, 69, 70, n77, n78, n79, n90

- 4th order IMD products may fall into Rx frequencies of bands 1, 2, 3, 4, 7, 9, 10, 11, 21, 23, 24, 25, 30, 32, 33, 34, 35, 36, 37, 38, 39, 40, 41, 45, 46, 47, 50, 51, 52, 53, 65, 66, 69, 70, 74, 75, 76, n77, n78, n79, n90, n91, n92, n93, n94

- 5th order IMD products may fall into Rx frequencies of bands 1, 2, 3, 4, 5, 6, 8, 9, 10, 11, 12, 13, 14, 17, 18, 19, 20, 21, 23, 24, 25, 26, 27, 28, 29, 30, 31, 32, 33, 34, 35, 36, 37, 38, 39, 40, 41, 44, 45, 46, 47, 50, 51, 53, 65, 66, 67, 68, 69, 70, 71, 72, 73, 74, 75, 76, 85, n87, n88, n90, n91, n92, n93, n94

Table 6.7.2.2-2: Protected bands for the 2UL bands CA configuration

| UL NR CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n78-n79 | E-UTRA Band 1, 3, 5, 8, 11, 18, 19, 21, 28, 34, 40, 41, 65, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| NOTE 3: Applicable when co-existence with PHS system operating in 1884.5 - 1915.7 MHz. |  |  |  |  |  |  |  |

6.7.2.3		REFSENS requirements

For harmonic, MSD studies can be omitted since MSD studies have been already conducted in 1UL/2DL NR CA fallback combinations.

For IMD, Considering both n78 and n79 are TDD bands and IMD occurs when UE transmit in both n77 and n79 and receive in neither n77 and n79,  no MSD are needed for dual UL of CA_n78-n79.

6.8	CA_n25-n48

6.8.1	Common for 1 band UL and 2 bands UL CA

6.8.1.1		Operating bands for CA

Table 6.8.1.1-1: CA band combination of band n25 and n48

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n25 | 1850 MHz | – | 1915 MHz | 1930 MHz | – | 1995 MHz | FDD |
| n48 | 3550 MHz | – | 3700 MHz | 3550 MHz | – | 3700 MHz | TDD |

6.8.1.2		Channel bandwidths per operating band for CA

Table 6.8.1.2-1: Supported bandwidths per CA band combination of band n25 and n48

| NR CA configuration | UL | NR Band | SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | BCS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n25A-n48A | - | n25 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n48 | 15 | Yes | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
| CA_n25A-n48(2A) | - | n25 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n48 | See CA_n48(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n25A-n48C | - | n25 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n48 | See CA_n48C Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.8.1.3		UE co-existence studies

Table 6.8.1.3-1 lists up to 7th harmonics for n25A-n48A which shows that there are 2nd harmonics issues.

Table 6.8.1.3-1: Impact of UL/DL Harmonic

|  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  | 6th Harmonic |  | 7th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n25 | 1850 | 1915 | 3700 | 3830 | 5550 | 5745 | 7400 | 7660 | 9250 | 9575 | 11100 | 11490 | 12950 | 13405 |
| n48 | 3550 | 3700 | 7100 | 7400 | 10650 | 11100 | 14200 | 14800 | 17750 | 18500 | 21300 | 22200 | 24850 | 25900 |

Table 6.8.1.3-2 list harmonic mixing issue for the 2DL bands CA with 1 UL. As can be seen there are no harmonic mixing issues.

Table 6.8.1.3-2 Harmonic mixing for 2DLs/1UL

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n25 | 1850 | 1915 | 1930 | 1995 | 3860 | 3990 | 5790 | 5985 | 7720 | 7980 |
| n48 | 3550 | 3700 | 3550 | 3700 | 7400 | 10650 | 11100 | 14200 | 14800 | 7400 |

6.8.1.4		∆TIB and ∆RIB values

For CA_n25-n48, the TIB,c and RIB,c values are derived from CA_2-48 and CA_n2-n48 and are given in the tables below.

Table 6.8.1.4-1: ΔTIB,c

| NR CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n25-n48 | n25 | 0.6 |
|  | n48 | 0.8 |

Table 6.8.1.4-2: ΔRIB,c

| NR CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n25-n48 | n25 | 0.2 |
|  | n48 | 0.5 |

6.8.1.5 		REFSENS requirements

As can be seen in the co-existence studies in 6.8.1.3 there are 2nd harmonics issues from n25 UL into the n48 DL. MSD values are same as for CA_n2A-n48A and need to be inserted into the 38.101-1 tables below.

CA_n25-n48 need to be added in NOTE 1 of Table 7.3A.4-1.

Table 7.3A.4-1: Reference sensitivity exceptions due to UL harmonic for NR CA FR1

| MSD due to harmonic exception for the DL band |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz |
|  |  | dB | dB | dB | dB | dB | dB | dB | dB | dB | dB | dB | dB |
| n25 | n481, 2 | 27.1 | 23.9 | 22.1 | 20.9 |  |  | 17.9 | 16.912 | 16.112 | 14.812 | 14.312 | 13.812 |
|  | n483 | 1.9 | 1.1 | 0.8 | 0.3 |  |  |  |  |  |  |  |  |
| NOTE 1: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band and a range ∆FHD above and below the edge of this downlink transmission bandwidth. The value ∆FHD depends on the band combination: ∆FHD = 10 MHz for CA_n1-n77, CA_n2-n78, CA_n3-n77, CA_n3-n78, CA_n2-n48, CA_n25-n48, CA_n25-n78, CA_n48-n66, CA_n66-n78. NOTE 2: The requirements should be verified for UL NR-ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 3: The requirements are only applicable to channel bandwidths no larger than 20 MHz and with a carrier frequency at MHz offset from in the victim (higher band) with , whereandare the channel bandwidths configured in the aggressor (lower) and victim (higher) bands in MHz, respectively. NOTE 12: For these bandwidths, the minimum requirements are restricted to operation when carrier is configured as a downlink carrier part of CA configuration. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3A.4-2: Uplink configuration for reference sensitivity exceptions due to UL harmonic interference for NR CA, FR1

| NR Band / Channel bandwidth of the high band |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz |
| n25 | n48 | 25 | 50 | 50 | 50 |  |  | 50 | 50 | 50 | 50 | 50 | 50 |

6.8.1.6 		OOB blocking exception requirements

No need to specify OOB exception requirement for CA_n25-n48 since it belongs to H-H configuration-

Table 6.8.1.6-1: CA band combination with exceptions allowed

CA band combination

No exceptions

6.9	CA_n28-n79

6.9.1	Common for 1 band UL and 2 bands UL CA

6.9.1.1 Operating bands for CA

Table 6.9.1.1-1: CA band combination CA_n28A-n79A

| NR CA Band Combination | NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n28-n79 | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

6.9.1.2	Channel bandwidths per operating band for CA

Table 6.9.1.2-1: Supported bandwidths per CA band combination CA_n28A-n79A

|  | CA operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration |  | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Bandwidth combination set |
| CA_n28A-n79A |  | CA_n28A-n79A | n28 | 15 | Yes | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  | 0 |
|  |  |  |  | 30 |  | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n79 | 15 |  |  |  |  |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  |  |  | 30 |  |  |  |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |
|  |  |  |  | 60 |  |  |  |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |

6.9.1.3	UE Co-existence studies

Table 6.9.1.3-1/2 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_n28-n79.

Table 6.9.1.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n28 | 703 | 748 | 758 | 803 | 1406 | 1496 | 2109 | 2244 | 2812 | 2992 |
| n79 | 4400 | 5000 | 4400 | 5000 | 8800 | 10000 | 13200 | 15000 | 17600 | 20000 |

Based on above table, there is no harmonic interference.

Table 6.9.1.3-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n28 | 703 | 748 | 758 | 803 | 1516 | 1606 | 2274 | 2409 | 3032 | 3212 |
| n79 | 4400 | 5000 | 4400 | 5000 | 8800 | 10000 | 13200 | 15000 | 17600 | 20000 |

Based on above table, there is no harmonic mixing issue.

6.9.1.4	∆TIB and ∆RIB values

For CA_n28A-n79A, the TIB,c and RIB are given in the tables below.

Table 6.9.1.4-1: ΔTIB,c

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n28-n79 | n28 | 0.5 |
|  | n79 | 0.8 |

Table 6.9.1.4-2: ΔRIB

| Inter-band CA Configuration | NR Band | ΔRIB [dB] |
| --- | --- | --- |
| CA_n28-n79 | n28 | 0.2 |
|  | n79 | 0.5 |

6.9.1.5	REFSENs requirements

There is no MSD exception requirement.

6.9.1.6	OOB blocking exception requirements

Since band n28 is a low band and n79 is a wide band, the OOBB exception is needed.

Table 6.9.1.6-1: CA band combination with exceptions allowed

CA band combination

CA_n28-n79

6.9.2		Specific for 2 bands UL CA

6.9.2.1	Maximum output power for inter-band CA

Table 6.9.2.1-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n28A-n79A | 23 | +2/-32 |
| NOTE 2: 2 refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB |  |  |

6.9.2.2		UE co-existence studies

Table 6.9.2.2-1 gives IMD interference analysis for CA_n28-n79 with 2 ULs.

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 703 | 748 | 4400 | 5000 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3652 | 4297 | 5103 | 5748 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3594 | 2904 | 8052 | 9297 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5806 | 6496 | 9503 | 10748 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 2891 | 2156 | 12452 | 14297 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 6509 | 7244 | 13903 | 15748 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 8594 | 7304 | 10206 | 11496 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 19297 | 16852 | 1408 | 2188 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 13594 | 11704 | 6556 | 7891 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 18303 | 20748 | 7212 | 7992 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 14606 | 16496 | 10909 | 12244 |

For band combination CA_n28-n79, no IMD interference will fall into Rx.

Table 6.9.2.2-2 lists the protected bands required for the 2UL bands CA configuration.

Table 6.9.2.2-2: Protected bands for the 2UL bands CA configuration

| NR CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected Band | Frequency range (Mhz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n28-n79 | E-UTRA Band 3, 5, 8, 18, 19, 34, 39, 40, 41, | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 1, 42, 65, 74 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 10 |
|  | Frequency range | 470 | - | 694 | -42 | 8 | 4, 14 |
|  | Frequency range | 470 | - | 710 | -26.2 | 6 | 13 |
|  | Frequency range | 662 | - | 694 | -26.2 | 6 | 4 |
|  | Frequency range | 758 | - | 773 | -32 | 1 | 4 |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 10, 11 |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.5.3.1-2 are permitted for each assigned NR carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x 180kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 3: Applicable when co-existence with PHS system operating in 1884.5 -1915.7 MHz NOTE 4: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth. NOTE 10: This requirement applies when the NR carrier is confined within 2545 - 2575 MHz or 2595 – 2645vMHz and the channel bandwidth is 10 or 20 MHz. NOTE 11: Applicable when the assigned NR carrier is confined within 718 MHz and 748 MHz and when the channel bandwidth used is 5 or 10 MHz. NOTE 13: This requirement is applicable for 5 and 10 MHz NR channel bandwidth allocated within 718 - 728 MHz. For carriers of 10 MHz bandwidth, this requirement applies for an uplink transmission bandwidth less than or equal to 30 RB with RBstart > 1 and Rbstart < 48. NOTE 14: This requirement is applicable in the case of a 10 MHz NR carrier confined within 703 MHz and 733 MHz, otherwise the requirement of -25 dBm with a measurement bandwidth of 8 MHz applies. |  |  |  |  |  |  |  |

6.9.2.3		REFSENS requirements

There is no additional MSD requirements for two UL CA_n28A-n79A

## 6.10		CA_n3-n77

### 6.10.1		Common for 1 band UL and 2 bands UL CA

#### 6.10.1.1		Operating bands for CA

Table 6.10.1.1-1: CA band combination of band n3+n77

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
| n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

#### 6.10.1.2		Channel bandwidths per operating band for CA

Table 6.10.1.2-1: Supported bandwidths per CA band combination of band n3+n77

| CA operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 80 | 90 | 100 | Bandwidth combination set |
| CA_n3A-n77(2A) | CA_n77(2A) | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |

#### 6.10.1.3		Co-existence studies

The studies for 1 band UL for the CA band combination of band n3 + n77 have been already completed and captured into TR 37.865-01-01, there is no IMD issue for band n3 + n77.

The IMD5 of n77(2A) may fall into band n3 DL when the two UL carriers are at the same time assigned within the frequency range of 3300MHz-3400MHz and the frequency range of 4000MHz-4200MHz separately.

#### 6.10.1.4		∆TIB and ∆RIB values

For CA_n3-n77, the TIB,c and RIB,c values are already specified in TR37.865-01-01.

#### 6.10.1.5		REFSENS requirements

Table 6.10.1.5-1 shows the MSD requirement for CA_n3-n77(2A) with UL CA_n77(2A).Table 6.10.1.5-1: 2DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n3A-n77(2A) | n3 | N/A | N/A | N/A | N/A | N/A6 | FDD | IMD5 |
|  | n77 | N/A | N/A | N/A | N/A | N/A | TDD | N/A |
| NOTE 6: Considering the spectrum holdings of the operator for CA_n77(2A) (when one uplink sub block is assigned within 3300-3400MHz, the other uplink sub block is not assigned within 4000-4200MHz or vice versa), no IMD5 result will fall in Rx frequency range of band n3. Therefore, no MSD requirement apply for this CA configuration when two uplink sub blocks are assigned within CA_77(2A). |  |  |  |  |  |  |  |  |

#### 6.10.1.6	    OOB blocking exception requirements

Table 6.10.1.6-1: CA band combination with exceptions allowed

CA band combination

No exception

## 6.11		CA_n3-n78

### 6.11.1		Common for 1 band UL and 2 bands UL CA

#### 6.11.1.1		Operating bands for CA

Table 6.11.1.1-1: CA band combination of band n3+n78

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
| n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

#### 6.11.1.2		Channel bandwidths per operating band for CA

Table 6.11.1.2-1: Supported bandwidths per CA band combination of band n3+n78

| CA operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 80 | 90 | 100 | Bandwidth combination set |
| CA_n3A-n78(2A) | CA_n78(2A) | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | n78 | See CA_n78(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |

#### 6.11.1.3		Co-existence studies

The studies for 1 band UL for the CA band combination of band n3 + n78 have been already completed and captured into TR 37.865-01-01, there is no IMD issue for band n3 + n78.

#### 6.11.1.4		∆TIB and ∆RIB values

For CA_n3-n78, the TIB,c and RIB,c values are already specified in TR37.865-01-01.

#### 6.11.1.5		REFSENS requirements

No additional MSD requirement need to be defined for this dual connectivity configuration..

#### 6.11.1.6	    OOB blocking exception requirements

Table 6.11.1.6-1: CA band combination with exceptions allowed

CA band combination

No exception

6.12		CA_n28-n77

6.12.1		Common for 1 band UL and 2 bands UL CA

6.12.1.1		Operating bands for CA

Table 6.12.1.1-1: CA band combination of band n28+n77

| NR CA Band | NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n28-n77 | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

6.12.1.2		Channel bandwidths per operating band for CA

Table 6.12.1.2-1: Supported bandwidths per CA band combination of band n28+n77

| NR CA configuration | Uplink CA configuration | NR Band | SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n28A-n77(2A) | CA_n77(2A)3 | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77(2A) in Table 5.5A.2-1 in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |

#### 6.12.1.3		Co-existence studies

The studies for 1 band UL for the CA band combination of band n28 + n78 have been already completed and captured into TR 38.716-02-00.

The IMD2 of n77(2A) may fall into band n28 DL when the two UL carriers are at the same time assigned within the frequency range of 3300MHz-3400MHz and the frequency range of 4000MHz-4200MHz separately.

6.12.1.4		∆TIB and ∆RIB values

For CA_n28-n77, the TIB,c and RIB,c values are already specified in TR38.716-02-00.

6.12.1.5		REFSENS requirements

Table 6.12.1.5-1 shows the MSD requirement for CA_n28-n77(2A) with UL CA_n77(2A).

Table 6.12.1.5-1: 2DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n28A-n77(2A) | n28 | N/A | N/A | N/A | N/A | N/A7 | FDD | IMD2 |
|  | n77 | N/A | N/A | N/A | N/A | N/A | TDD | N/A |
| NOTE 7: Considering the spectrum holdings of the operator for CA_n77(2A) (when one uplink sub block is assigned within 3300-3400MHz, the other uplink sub block is not assigned within 4000-4200MHz or vice versa), no IMD2 result will fall in Rx frequency range of band n28. Therefore, no MSD requirement apply for this CA configuration when two uplink sub blocks are assigned within CA_77(2A).. |

6.12.1.6	    OOB blocking exception requirements

Table 6.12.1.6-1: CA band combination with exceptions allowed

CA band combination

CA_n28-n77

6.13		CA_n28-n78

6.13.1		Common for 1 band UL and 2 bands UL CA

6.13.1.1		Operating bands for CA

Table 6.13.1.1-1: CA band combination of band n28+n78

| NR CA Band | NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n28-n78 | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

6.13.1.2		Channel bandwidths per operating band for CA

Table 6.13.1.2-1: Supported bandwidths per CA band combination of band n28+n78

| NR CA configuration | Uplink CA configuration | NR Band | SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n28A-n78(2A) | CA_n78(2A) | n28 | 15 | Yes | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | See CA_n78(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |

#### 6.13.1.3		Co-existence studies

The studies for 1 band UL for the CA band combination of band n28 + n78 have been already completed and captured into TR 37.865-01-01, there is no IMD issue for band n28 + n78.

6.13.1.4		∆TIB and ∆RIB values

For CA_n28-n78, the TIB,c and RIB,c values are already specified in TR37.865-01-01.

6.13.1.5		REFSENS requirements

No additional MSD requirement need to be defined for this band combs configuration.

#### 6.13.1.6	    OOB blocking exception requirements

Table 6.13.1.6-1: CA band combination with exceptions allowed

CA band combination

CA_n28-n78

## 6.14	CA_n41-n78

### 6.14.1		Common for 1 band UL and 2 bands UL CA

#### 6.14.1.1		Operating bands for CA

Table 6.14.1.1-1: CA band combination of band n41+n78

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
| n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

#### 6.14.1.2		Channel bandwidths per operating band for CA

Table 6.14.1.2-1: Supported bandwidths per CA band combination of band n41+n78

|  | CA operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration |  | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Bandwidth combination set |
| CA_n41A-n78(2A) |  | - | n41 | 15 |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |  | 0 |
|  |  |  |  | 30 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  | n78 | See CA_n78(2A) Bandwidth Combination Set 2 in Table 5.5A.2-1 in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

#### 6.14.1.3		Co-existence studies

Table 6.14.1.3-1 summarizes frequency ranges where harmonics occur for CA_n41-n78.

Table 6.14.1.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | nth Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n41 | 2496 | 2690 | 2496 | 2690 | 4992 | 5380 | 7488 | 8077 |  |  |
| n78 | 3300 | 3800 | 3300 | 3800 | 6600 | 7600 | 9900 | 11400 |  |  |

Based on above table, there is no harmonic issue for the band combination of n41 and n78.

Table 6.14.1.3-2 summarizes frequency ranges where harmonics mixing occur for CA_n41-n78.

Table 6.14.1.3-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | mth Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n41 | 2496 | 2690 | 2496 | 2690 | 4992 | 5380 | 7488 | 8077 |  |  |
| n78 | 3300 | 3800 | 3300 | 3800 | 6600 | 7600 | 9900 | 11400 |  |  |

Based on above table, there is no harmonic mixing issue for the band combination of n41 and n78.

6.14.1.4		∆TIB and ∆RIB values

For CA_n41-n78 , the ∆TIB,c and ∆RIB,c values have been defined in TS 38.101-1.

#### 6.14.1.5		REFSENS requirements

The MSD requirements due to cross band isolation defined as Table 6.14.1.5-1 and Table 6.14.1.5-2:

Table 6.14.1.5-1: Reference sensitivity exceptions (MSD) due to cross band isolation for NR CA FR1

| NR Band / Channel bandwidth of the affected DL band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 70 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| n41 | n78 |  | 8.3 | 8.3 | 8.3 | 7.3 | 6.5 | 6.3 | 5.3 | 4.5 | 4.3 | 4.0 | 3.9 | 3.8 |
| n78 | n411 |  | 4.5 | 4.5 | 4.5 |  | 4.5 | 4.5 | 4.5 | 4.5 |  | 4.5 | 4.5 | 4.5 |
| NOTE 1: Applicable only when harmonic mixing MSD for this combination is not applied. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.14.1.5-2: Uplink configuration for reference sensitivity exceptions due to cross band isolation for NR CA FR1

| NR Band / SCS / Channel bandwidth of the affected DL band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | SCS of UL band (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| n41 | n78 | 15 |  | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 |
| n78 | n41 | 30 |  | 270 | 270 | 270 |  | 270 | 270 | 270 | 270 |  | 270 | 270 | 270 |

#### 6.14.1.6	    OOB blocking exception requirements

Table 6.14.1.6-1: CA band combination with exceptions allowed

CA band combination

No exception

## 6.15	CA_n2-n66

### 6.15.1		Common for 1 band UL and 2 bands UL CA

#### 6.15.1.1		Operating bands for CA

Table 6.15.1.1-1: CA band combination of band n2+n66

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
| n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |

#### 6.15.1.2		Channel bandwidths per operating band for CA

Table 6.15.1.2-1: Supported bandwidths per CA band combination of band n2+n66

| CA operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 80 | 90 | 100 | Bandwidth combination set |
| CA_n2A-n66A | CA_n2A-n66A | n2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 1 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n66 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |

#### 6.15.1.3		Co-existence studies

Table 6.15.1.3-1 summarizes frequency ranges where harmonics occur for CA_n2-n66.

Table 6.15.1.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | nth Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n2 | 1850 | 1910 | 1930 | 1990 | 3700 | 3820 | 5550 | 5730 |  |  |
| n66 | 1710 | 1780 | 2110 | 2200 | 3420 | 3560 | 5130 | 5340 |  |  |

Based on above table, there is no harmonic issue for the band combination of n2 and n66.

Table 6.15.1.3-2 summarizes frequency ranges where harmonics mixing occur for CA_n2-n66.

Table 6.15.1.3-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | mth Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n2 | 1850 | 1910 | 1930 | 1990 | 3860 | 3980 | 5790 | 5970 |  |  |
| n66 | 1710 | 1780 | 2110 | 2200 | 4220 | 4400 | 6330 | 6660 |  |  |

Based on above table, there is no harmonic mixing issue for the band combination of n2 and n66.

6.15.1.4		∆TIB and ∆RIB values

For CA_n2-n66 , the ∆TIB,c and ∆RIB,c values are given according to DC_2_n66 relaxation values.

Table 6.15.1.4-1: ΔTIB,c

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n2-n66 | n2 | 0.5 |
|  | n66 | 0.5 |

Table 6.15.1.4-2: ΔRIB,c

| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n2-n66 | n2 | 0.3 |
|  | n66 | 0.3 |

#### 6.15.1.5		REFSENS requirements

#### No additional MSD requirement need to be defined for this band combs.6.15.1.6	    OOB blocking exception requirements

Table 6.15.1.6-1: CA band combination with exceptions allowed

CA band combination

No exception

6.15.2		Specific for 2 bands UL CA

6.15.2.1	Maximum output power for inter-band CA

Table 6.15.2.1-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n2A-n66A | 23 | +2/-32 |
| NOTE 2: 2 refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB |  |  |

6.15.2.2		UE co-existence studies

Table 6.15.2.2-1 lists Band n2 +Band n66 2UL bands CA  2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.15.2.2-1: Band n2 and Band n66 UL harmonics and IMD products

| UE UL carriers | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| UL frequencies (MHz) | 1850 | 1910 | 1710 | 1780 |
| 2nd order IMD products | f2_low – f1_high | f2_high – f1_low | f2_low + f1_low | f2_high + f1_high |
| IMD frequency limit (MHz) | 70 | 200 | 3560 | 3690 |
| 3rd order IMD products | 2*f1_low – f2_high | 2*f1_high – f2_low | 2*f2_low – f1_high | 2*f2_high – f1_low |
| IMD frequency limit (MHz) | 1920 | 2110 | 1510 | 1710 |
| 3rd order IMD products | 2*f1_low + f2_low | 2*f1_high + f2_high | 2*f2_low + f1_low | 2*f2_high + f1_high |
| IMD frequency limit (MHz) | 5410 | 5600 | 5270 | 5470 |
| 4th order IMD products | 3*f1_low – f2_high | 3*f1_high – f2_low | 3*f2_low – f1_high | 3*f2_high – f1_low |
| IMD frequency limit (MHz) | 3770 | 4020 | 3220 | 3490 |
| 4th order IMD products | 3*f1_low + f2_low | 3*f1_high + f2_high | 3*f2_low + f1_low | 3*f2_high + f1_high |
| IMD frequency limit (MHz) | 7260 | 7510 | 6980 | 7250 |
| 4th order IMD products | 2*f1_low – 2*f2_high | 2*f1_high – 2*f2_low | 2*f1_low + 2*f2_low | 2*f1_high + 2*f2_high |
| IMD frequency limit (MHz) | 140 | 400 | 7120 | 7380 |
| 5th order IMD products | f1_low – 4*f2_high | f1_high – 4*f2_low | f2_low – 4*f1_high | f2_high – 4*f1_low |
| IMD frequency limit (MHz) | 4930 | 5270 | 5620 | 5930 |
| 5th order IMD products | f1_low + 4*f2_low | f1_high + 4*f2_high | f2_low + 4*f1_low | f2_high + 4*f1_high |
| IMD frequency limit (MHz) | 8690 | 9030 | 9110 | 9420 |
| 5th order IMD products | 2*f1_low – 3*f2_high | 2*f1_high - 3*f2_low | 2*f2_low – 3*f1_high | 2*f2_high – 3*f1_low |
| IMD frequency limit (MHz) | 1310 | 1640 | 1990 | 2310 |
| 5th order IMD products | 2*f1_low + 3*f2_low | 2*f1_high + 3*f2_high | 2*f2_low + 3*f1_low | 2*f2_high + 3*f1_high |
| IMD frequency limit (MHz) | 8830 | 9160 | 8970 | 9290 |

Based on the table above:

- The 3rd order IMD may fall into Rx frequencies of band n2.

- The 5th order IMD may fall into Rx frequencies of band n66

Table 6.15.2.2-2 lists the protected bands required for the 2UL bands CA configuration.

Table 6.15.2.2-2: Protected bands for the 2UL bands CA configuration

| UL NR CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n2-n66 | E-UTRA Band 4, 5, 10, 12, 13, 14, 17, 24, 26, 27, 28, 29, 30, 41, 50, 51, 66, 70, 71, 74, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2, 25 | FDL_low | - | FDL_high | -50 | 1 | 4 |
|  | E-UTRA Band 42, 48, NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.5.3.1-2 are permitted for each assigned NR carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x 180kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 4: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth. |  |  |  |  |  |  |  |

6.15.2.3		REFSENS requirements

Table 6.15.2.3-1 lists the MSD required due to the 3rd and 5th IMD for the dual uplink configuration The MSD value is reused from DC_2_n66.

Table 6.15.2.3-1: MSD due to IMD issue

| Operating band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CA Configuration | Operating band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) |  |
| CA_n2A-n66A | n2 | 1855 | 5 | 25 | 1935 | 20 | IMD3 |
|  | n66 | 1775 | 5 | 25 | 2175 | N/A | N/A |
|  | n2 | 1883.3 | 5 | 25 | 1963.3 | N/A | N/A |
|  | n66 | 1750 | 5 | 25 | 2150 | 4 | IMD5 |

6.16	CA_n41-n77

6.16.1	Common for 1 band UL and 2 bands UL CA

6.16.1.1 Operating bands for CA

Table 6.16.1.1-1: CA band combination of band n41 + n77

| NR CA Band Combination | NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n41-n771 | n41 | 2495 MHz | – | 2690 MHz | 2495 MHz | – | 2690 MHz | TDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |
| NOTE 1: Applicable for UE supporting inter-band carrier aggregation with mandatory simultaneous Rx/Tx capability. |  |  |  |  |  |  |  |  |

6.16.1.2	Channel bandwidths per operating band for CA

Table 6.16.1.2-1: Supported bandwidths per CA band combination of band n41+n77

| NR CA configuration / Bandwidth combination set [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA configuration | UL configuration | NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Bandwidth combination set |
| CA_n41A-n77A | CA_n41A-n77A | n41 | 15 |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
| CA_n41(2A)-n77A | CA_n41A-n77A | n41 | See CA_n41(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
| CA_n41C-n77A | CA_n41A-n77A CA_n41C | n41 | See CA_n41C Bandwidth Combination Set 0 in Table 5.5A.1-1 in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
| CA_n41A-n77(2A) | CA_n41A-n77A | n41 | 15 |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.16.1.3	Co-existence studies

Table 6.16.1.3-1/2 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_n41-n77.

Table 6.16.1.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n41 | 2496 | 2690 | 2496 | 2690 | 4992 | 5380 | 7488 | 8070 | 9984 | 10760 |
| n77 | 3300 | 4200 | 3300 | 4200 | 6600 | 8400 | 9900 | 12600 | 13200 | 16800 |

Based on above table no harmonics occur.

Table 6.16.1.3-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n41 | 2496 | 2690 | 2496 | 2690 | 4992 | 5380 | 7488 | 8070 | 9984 | 10760 |
| n77 | 3300 | 4200 | 3300 | 4200 | 6600 | 8400 | 9900 | 12600 | 13200 | 16800 |

Based on above table 2nd UL harmonic of n77 is same as n41 DL 3rd harmonic.

6.16.1.4	∆TIB and ∆RIB values

For CA_n41-n77, it is proposed to re-use the TIB,c and RIB values from CA_n41-n78.

Table 6.16.1.4-1: ΔTIB,c

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n41-n771 | n41 | 0.3 |
|  | n77 | 0.8 |
| NOTE 1: The requirements only apply when the sub-frame and Tx-Rx timings are synchronized between the component carriers. In the absence of synchronization, the requirements are not within scope of these specifications. |  |  |

Table 6.16.1.4-2: ΔRIB

| Inter-band CA Configuration | NR Band | ΔRIB [dB] |
| --- | --- | --- |
| CA_n41-n771 | n77 | 0.5 |
| NOTE 1: The requirements only apply when the sub-frame and Tx-Rx timings are synchronized between the component carriers. In the absence of synchronization, the requirements are not within scope of these specifications. |  |  |

6.16.1.5	REFSENs requirements

No additional requirements are necessary.

6.16.1.6	OOB blocking exception requirements

There is no OOB exception for the CA combination.

6.16.2		Specific for 2 bands UL CA

6.16.2.1	Maximum output power for inter-band CA

Table 6.16.2.1-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n41A-n77A | 23 | +2/-32 |
| NOTE 2: 2 refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB |  |  |

6.16.2.2		UE co-existence

Table 6.16.2.2-1 gives IMD interference analysis for CA_ n41-n77 with 2 ULs.

Table 6.16.2.2-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 2496 | 2690 | 3300 | 4200 |
| Two tone 2nd order IMD products | ¦fy_low - fx_high¦ | ¦fy_high - fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 610 | 1704 | 5796 | 6890 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 792 | 2080 | 3910 | 5904 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 8292 | 9580 | 9096 | 11090 |
| Two-tone 4th order IMD products | ¦3*fx_low - fy_high¦ | ¦3*fx_high - fy_low¦ | ¦3*fy_low - fx_high¦ | ¦3*fy_high - fx_low¦ |
| IMD frequency limits (MHz) | 3288 | 4770 | 7210 | 10104 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 10788 | 12270 | 12396 | 15290 |
| Two-tone 4th order IMD products | ¦2*fx_low - 2*fy_high¦ | ¦2*fx_high - 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 3408 | 1220 | 11592 | 13780 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 14304 | 10510 | 7460 | 5784 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15696 | 19490 | 13284 | 14960 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 7608 | 4520 | 1470 | 912 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 14892 | 17980 | 14088 | 16470 |

Based on the table 6.16.2.2-1, there is 3rd and 4th order IMD are an issue for n77, however IMD3 cannot interfere own downlink channel even it hits the band.

Table 6.16.2.2-2 lists the protected bands required for the 2UL bands CA configuration.

Table 6.16.2.2-2: Protected bands for the 2UL bands CA configuration

| UL NR CA Configuration | Spurious emission |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band |  | Frequency range (MHz) |  |  |  | Maximum Level (dBm) |  |  | MBW (MHz) | NOTE |  |
| CA_n41-n77 |  | E-UTRA Band 1, 2, 3, 4, 5, 8, 10, 11, 12, 13, 14, 17, 18, 19, 20, 21, 24, 25, 26, 27, 28, 29, 30, 34, 39, 40, 44, 45, 50, 51, 53, 65, 66, 70, 71, 73, 74, 85, |  | FDL_low | - | FDL_high |  | -50 | 1 |  |  |  |
|  |  | Frequency range |  | 1884.5 |  | 1915.7 |  | -41 | 0.3 |  |  | 3 |
| NOTE 1: FDL_low and FDL_high refer to each frequency band specified in Table 5.2-1 in TS 38.101-1 or Table 5.5-1 in TS 36.1601 NOTE 3: Applicable when co-existence with PHS system operating in 1884.5 - 1915.7 MHz. |  |  |  |  |  |  |  |  |  |  |  |  |

6.16.2.3		REFSENS requirements

There is MSD defined for CA_41A-78A due to cross band isolation, it is proposed to define same requirement for CA_41A-77A.

Table 7.3A.6-1: Reference sensitivity exceptions (MSD) due to cross band isolation for NR CA FR1

| NR Band / Channel bandwidth of the affected DL band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 70 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| n41 | n77 |  | 8.3 | 8.3 | 8.3 | 7.3 | 6.5 | 6.3 | 5.3 | 4.5 | 4.3 | 4.0 | 3.9 | 3.8 |

Table 7.3A.6.2: Uplink configuration for reference sensitivity exceptions due to cross band isolation for NR CA FR1

| NR Band / SCS / Channel bandwidth of the affected DL band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | SCS of UL band (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| n41 | n77 | 15 |  | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 |

There is also harmonic receiver mixing issue that needs MSD.

Table 7.3A.4-4: Reference sensitivity exceptions due to harmonic mixing for CA in NR FR1

| NR Band / Channel bandwidth of the affected DL band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz(dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 70 MHz(dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| n77 | 412 |  | 10.4 | 10.4 | 10.4 |  | 10.4 | 10.4 | 10.4 | 10.4 |  | 10.4 | 10.4 | 10.4 |
| NOTE 2: The requirements should be verified for UL NR-ARFCN of the aggressor (high) band (superscript HB) such that in MHz and with carrier frequency in the victim (lower) band in MHz and the channel bandwidth configured in the higher band. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3A.4-4a: Uplink configuration for reference sensitivity exceptions due to receiver harmonic mixing for CA in NR FR1

| NR Band / SCS / Channel bandwidth of the affected DL band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| n77 | 41 | 15 |  | 25 | 36 | 50 |  | 50 | 50 | 50 | 50 |  | 50 | 50 | 50 |
| NOTE 1: The UL configuration applies regardless of the channel bandwidth of the UL band unless the UL resource blocks exceed that specified in Table 7.3.2-3 for the uplink bandwidth in which case the allocation according to Table 7.3.2-3 applies. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.17		CA_n71-n77

6.17.1	Common for 1 band UL and 2 bands UL CA

6.17.1.1 Operating bands for CA

Table 6.17.1-1: CA band combination of band n71 + n77

| NR CA Band Combination | NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n71-n77 | n71 | 663 MHz | – | 698 MHz | 617 MHz | – | 652 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

6.17.1.2	Channel bandwidths per operating band for CA

Table 6.17.2-1: Supported bandwidths per CA band combination of band n71+n77

| NR CA configuration / Bandwidth combination set [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA configuration | UL configuration | NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Bandwidth combination set |
| CA_n71A-n77A | CA_n71A-n77A | n71 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

6.17.1.3	Co-existence studies

Table 6.17.1.3-1/2 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_n71-n77.

Table 6.17.1.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| 71 | 663 | 698 | 617 | 652 | 1326 | 1396 | 1989 | 2094 | 2652 | 2792 | 3315 | 3490 |
| 77 | 3300 | 4200 | 3300 | 4200 | 6600 | 8400 | 9900 | 12600 | 13200 | 16800 | 16500 | 21000 |
| Note: The combination is subject to LB 5th order harmonic impact to n77 but does not happen in the US |  |  |  |  |  |  |  |  |  |  |  |  |

Based on above table no harmonics occur.

Table 6.17.1.3-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| 71 | 663 | 698 | 617 | 652 | 1234 | 1304 | 1851 | 1956 | 2468 | 2608 |
| 77 | 3300 | 4200 | 3300 | 4200 | 6600 | 8400 | 9900 | 12600 | 13200 | 16800 |

Based on above table no harmonic mixing occur.

6.17.1.4	∆TIB and ∆RIB values

For CA_n71-n77, it is proposed to re-use the TIB,c and RIB values from CA_n71-n78.

Table 6.17.1.4-1: ΔTIB,c

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n71-n77 | n71 | 0.5 |
|  | n77 | 0.8 |

Table 6.17.1.4-2: ΔRIB

| Inter-band CA Configuration | NR Band | ΔRIB [dB] |
| --- | --- | --- |
| CA_n71-n77 | n71 | 0.2 |
|  | n77 | 0.5 |

6.17.1.5	REFSENs requirements

No additional requirements are necessary.

6.17.1.6	OOB blocking exception requirements

Table 6.17.1.6-1: CA band combination with exceptions allowed

CA band combination

CA_n71-n77

6.17.2		Specific for 2 bands UL CA

6.17.2.1	Maximum output power for inter-band CA

Table 6.17.2.1-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n71A-n77A | 23 | +2/-32 |
| NOTE 2: 2 refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB |  |  |

6.17.2.2		UE co-existence

Table 6.17.2.2-1 gives IMD interference analysis for CA_ n71-n77 with 2 ULs.

Table 6.17.2.2-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 663 | 698 | 3300 | 4200 |
| Two tone 2nd order IMD products | ¦fy_low - fx_high¦ | ¦fy_high - fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2602 | 3537 | 3963 | 4898 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2874 | 1904 | 5902 | 7737 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4626 | 5596 | 7263 | 9098 |
| Two-tone 4th order IMD products | ¦3*fx_low - fy_high¦ | ¦3*fx_high - fy_low¦ | ¦3*fy_low - fx_high¦ | ¦3*fy_high - fx_low¦ |
| IMD frequency limits (MHz) | 2211 | 1206 | 9202 | 11937 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5289 | 6294 | 10563 | 13298 |
| Two-tone 4th order IMD products | ¦2*fx_low - 2*fy_high¦ | ¦2*fx_high - 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 7074 | 5204 | 7926 | 9796 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 16137 | 12502 | 508 | 1548 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 13863 | 17498 | 5952 | 6992 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 11274 | 8504 | 4506 | 6411 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11226 | 13996 | 8589 | 10494 |

Based on the table 6.17.2.2-1, there is 2nd and 5th order IMD are an issue for n77, however IMD2 cannot interfere own downlink channel even it hits the band.

Table 6.17.2.2-2 lists the protected bands required for the 2UL bands CA configuration.

Table 6.17.2.2-2: Protected bands for the 2UL bands CA configuration

| UL NR CA Configuration | Spurious emission |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band |  | Frequency range (MHz) |  |  |  | Maximum Level (dBm) |  |  | MBW (MHz) | NOTE |  |
| CA_n71-n77 |  | E-UTRA Band 1, 3, 4, 5, 7, 8, 10, 11, 12, 13, 14, 17, 18, 19, 20, 21, 24, 26, 27, 28, 29, 30, 34, 39, 40, 44, 45, 50, 51, 53, 65, 66, 71, 73, 74, 85, |  | FDL_low | - | FDL_high |  | -50 | 1 |  |  |  |
|  |  | Frequency range |  | 1884.5 |  | 1915.7 |  | -41 | 0.3 |  |  | 3 |
|  |  | E-UTRA Band 2, 25, 41, 70 |  | FDL_low | - | FDL_high |  | -50 | 1 |  |  | 2 |
|  |  | E-UTRA Band 29 |  | FDL_low | - | FDL_high |  | -38 | 1 |  |  | 4 |
|  |  | E-UTRA Band 71 |  | FDL_low | - | FDL_high |  | -50 | 1 |  |  | 4 |
| NOTE 1: FDL_low and FDL_high refer to each frequency band specified in Table 5.2-1 in TS 38.101-1 or Table 5.5-1 in TS 36.101 NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.5.3.1-2 are permitted for each assigned NR carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x RBsize kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 3: Applicable when co-existence with PHS system operating in 1884.5 - 1915.7 MHz. NOTE 4: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth. |  |  |  |  |  |  |  |  |  |  |  |  |

6.17.2.3		REFSENS requirements

It is proposed to use IMD5 test point in Table 6.17.2.3-1 which is based on similar CA_n28A-n77A.

Table 6.17.2.3-1

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n71A-n77A | n71 | 671 | 5 | 25 | 625 | 5.5 | FDD | IMD5 |
|  | n77 | 3300 | 10 | 50 | 3300 | N/A | TDD | N/A |

6.18	CA_n5-n48

6.18.1	Common for 1 band UL and 2 bands UL CA

6.18.1.1		Operating bands for CA

Table 6.18.1.1-1: CA band combination of band n5+n48

| NR CA Band | NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n5-n48 | n5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | n48 | 3550 MHz | – | 3700 MHz | 3550 MHz | – | 3700 MHz | TDD |

6.18.1.2		Channel bandwidths per operating band for CA

Table 6.18.1.2-1: Supported bandwidths per CA configuration of band n5+n48

|  |  | CA operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | UL CA Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | BCS |
| CA_n5A-n48A | CA_n5A-n48A | n5 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n48 | 15 | Yes5 | Yes | Yes | Yes |  |  | Yes | Yes6 |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes6 | Yes6 |  | Yes6 | Yes6,4 | Yes6 |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes6 | Yes6 |  | Yes6 | Yes6,4 | Yes6 |  |
| CA_n5A-n48(2A) | CA_n5A-n48A | n5 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n48 | See CA_48(2A) in Table 5.5A.2-1 in 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n5A-n48C | CA_n5A-n48A | n5 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n48 | See CA_n48C in Table 5.5A.1-1 in 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NOTE 4: This UE channel bandwidth is optional in this release of the specification. NOTE 5: For this bandwidth, the minimum requirements are restricted to operation when carrier is configured as an SCell part of DC or CA configuration. NOTE 6: For this bandwidth, the minimum requirements are restricted to operation when carrier is configured as an downlink SCell part of CA configuration. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.18.1.3		UE co-existence studies

Table 6.18.1.3-1 lists up to 5th harmonics for CA_n5A-n48A. As can be seen there are no harmonic issue.

Table 6.18.1.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n5 | 824 | 849 | 869 | 894 | 1648 | 1698 | 2472 | 2547 | 3296 | 3396 | 4120 | 4245 |
| n48 | 3550 | 3700 | 3550 | 3700 | 7100 | 7400 | 10650 | 11100 | 14800 | 14800 | 17750 | 18500 |

Table 6.18.1.3-2 list harmonic mixing issue for the 2DL bands CA with 1 UL. As can be seen there are no harmonic mixing issues.

Table 6.18.1.3-2 Harmonic mixing for 2DLs/1UL

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n5 | 824 | 849 | 869 | 894 | 1738 | 1788 | 2607 | 2682 | 3476 | 3576 | 4345 | 4245 |
| n48 | 3550 | 3700 | 3550 | 3700 | 7100 | 7400 | 10650 | 11100 | 14200 | 14800 | 17750 | 18500 |

Based on study above, there is no harmonic issue for this band combination.

6.18.1.4		∆TIB and ∆RIB values

For CA_n5-n48, the ΔTIB,c and ΔRIB,c values are given in the tables below.

Table 6.18.1.4-1: ΔTIB,c

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n5-n48 | n5 | 0.3 |
|  | n48 | 0.3 |

Table 6.18.1.4-2: ΔRIB,c

| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n5-n48 | n5 | 0 |
|  | n48 | 0 |

6.18.1.5		REFSENS requirements

As can be seen in the co-existence studies in 6.18.1.3 there are no harmonics issues.

6.18.1.6		OOB blocking exception requirements

There is no OOB exception for this CA combination.

6.18.2		Specific for 2 bands UL CA

6.18.2.1	Maximum output power for inter-band CA

Table 6.18.2.1-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n5A-n48A | 23 | +2/-32 |
| NOTE 2: 2 refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB |  |  |

6.18.2.2		UE co-existence

Table 6.18.2.2-1 lists Band n5+Band n48 2UL CA 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.18.2.2-1: Band n5 and Band n48 2 UL bands IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 824 | 849 | 3550 | 3700 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1648 | 1698 | 7100 | 7400 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2472 | 2547 | 10650 | 11100 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4*fy_low | 4*fy_high |
| 4th harmonics frequency limits (MHz) | 3296 | 3396 | 14200 | 14800 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 4120 | 4245 | 17750 | 18500 |
| Two tone 2nd order IMD products | fy_low – fx_high | fy_high – fx_low | fx_low + fy_low | fx_high + fy_high |
| IMD frequency limits (MHz) | 2701 | 2876 | 4374 | 4549 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | 2*fy_low – fx_high | 2*fy_high – fx_low |
| IMD frequency limits (MHz) | 2052 | 1852 | 6251 | 6576 |
| Two-tone 3rd order IMD products | 2*fx_low + fy_low | 2*fx_high + fy_high | 2*fy_low + fx_low | 2*fy_high + fx_high |
| IMD frequency limits (MHz) | 5198 | 5398 | 7924 | 8249 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | 3*fy_low – fx_high | 3*fy_high – fx_low |
| IMD frequency limits (MHz) | 1228 | 1003 | 9801 | 10276 |
| Two-tone 4th order IMD products | 3*fx_low + fy_low | 3*fx_high + fy_high | 3*fy_low + fx_low | 3*fy_high + fx_high |
| IMD frequency limits (MHz) | 6022 | 6247 | 11474 | 11949 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | 2*fx_low + 2*fy_low | 2*fx_high + 2*fy_high |
| IMD frequency limits (MHz) | 5752 | 5402 | 8748 | 9098 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 13976 | 13351 | 154 | 404 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15024 | 15649 | 6846 | 7096 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 9452 | 8952 | 4553 | 4928 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 12298 | 12798 | 9572 | 9947 |

Based on the table 6.18.2.2-1, there is no IMD issue for CA_n5-n48.

Table 6.18.2.2-2: Protected bands for the 2UL bands CA configuration

| NR CA Configuration | Spurious emission |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n5A-n48A | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 24, 25, 26, 29, 30, 65, 66, 70, 71, 73 |  | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 41 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 11, 21 |  | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range |  | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.5.3.1-2 are permitted for each assigned NR carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x RBsize kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 8: Applicable when co-existence with PHS system operating in 1884.5 - 1915.7 MHz. |  |  |  |  |  |  |  |  |

6.18.2.3  REFSENs requirements

Based on above coexistence study, there is no MSD issue for the CA combination.

  - 6.19		CA_n34-n79

    - 6.19.1		Common for 1 band UL and 2 bands UL CA

      - 6.19.1.1		Operating bands for CA

Table 6.19.1.1-1: CA band combination of band n34+n79

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n34 | 2010 MHz | – | 2025 MHz | 2010 MHz | – | 2025 MHz | FDD |
| n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |
| NOTE 1: Applicable for UE supporting inter-band carrier aggregation with mandatory simultaneous Rx/Tx capability. |  |  |  |  |  |  |  |

      - 6.19.1.2		Channel bandwidths per operating band for CA

Table 6.19.2-1: Supported bandwidths per CA band combination of band n34+n79

| CA operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 80 | 90 | 100 | Bandwidth combination set |
| CA_n34A-n79A | CA_n34A-n79A | n34 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n79 | 15 |  |  |  |  |  |  | Yes | Yes |  |  |  |  |  |
|  |  |  | 30 |  |  |  |  |  |  | Yes | Yes | Yes | Yes |  | Yes |  |
|  |  |  | 60 |  |  |  |  |  |  | Yes | Yes | Yes | Yes |  | Yes |  |

      - 6.19.1.3		Co-existence studies

Table 6.19.1.3-1 summarizes frequency ranges where harmonics occur for CA_n34-n79.

Table 6.19.1.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | nth Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n34 | 2010 | 2025 | 2010 | 2025 | 4020 | 4050 | 6030 | 6075 |  |  |
| n79 | 4400 | 5000 | 4400 | 5000 | 8800 | 10000 | 13200 | 15000 |  |  |

Based on above table, there is no harmonic issue for the band combination of n34 and n79.

Table 6.19.1.3-2 summarizes frequency ranges where harmonics mixing occur for CA_n34-n79.

Table 6.19.1.3-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | mth Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n34 | 2010 | 2025 | 2010 | 2025 | 4020 | 4050 | 6030 | 6075 |  |  |
| n79 | 4400 | 5000 | 4400 | 5000 | 8800 | 10000 | 13200 | 15000 |  |  |

Based on above table, there is no harmonic mixing issue for the band combination of n34 and n79.

      - 6.19.1.4		∆TIB and ∆RIB values

For CA_n34-n79 , the ∆TIB,c and ∆RIB,c values are given in the tables below.

Table 6.1.1.4-1: ΔTIB,c

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n34-n79 | n34 | 0.3 |
|  | n79 | 0.8 |

Table 6.1.1.4-2: ΔRIB,c

| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n34-n79 | n34 | 0 |
|  | n79 | 0.5 |

      - 6.19.1.5		REFSENS requirements

No specific REFSENS requirements need to be specified.

      - 6.19.1.6		OOB blocking exception requirements

No OOB blocking exception requirements for CA_n34-n79 specified.

    - 6.19.2		Specific for 2 bands UL CA

      - 6.19.2.1		UE co-existence studies

Table 6.19.2.1-1 lists Band n34 +Band n79 2UL CA  2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.19.2.1-1: Band n34 and Band n79 UL IMD products

| DL frequency | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
|  | 2010 | 2025 | 4400 | 5000 |
| 2nd order IMD products | ¦fy_low-fx_high¦ | ¦fy_high-fx_low¦ | ¦fy_low+fx_low¦ | ¦fy_high+fx_high¦ |
| IMD frequency limits (MHz) | 2375 | 2990 | 6410 | 7025 |
| Two-tone 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 980 | 350 | 6775 | 7990 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 8420 | 9050 | 10810 | 12025 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 5980 | 4750 | 12820 | 14050 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1030 | 1675 | 11175 | 12990 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 10430 | 11075 | 15210 | 17010 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 17990 | 15575 | 3700 | 3040 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 19610 | 22025 | 12440 | 13100 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 10980 | 9150 | 2725 | 3970 |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. |  |  |  |  |

Based on Table 6.19.2.1-1, although 4th order IMD may also fall into own Rx of band n79, both n34 and n79 are TDD band which means the UL/DL will not simultaneous work at the same time. Therefore, no need to consider the IMD issue.

Table 6.19.2.1-2 lists the protected bands required for the 2UL bands CA configuration.

Table 6.19.2.1-2: Protected bands for the 2UL bands CA configuration

| NR CA combination | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n34-n79 | E-UTRA Band 1, 3, 8, 11, 18, 19, 21, 28, 39, 40, 41, 42, 65, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |
| NOTE 8: Applicable when co-existence with PHS system operating in 1884.5 - 1915.7 MHz. |  |  |  |  |  |  |  |

      - 6.19.2.2		REFSENS requirements

According to the co-existent analysis in table 6.19.2.1-1,  no IMD issues are expected for this CA configuration with dual uplink carrier.

6.20		CA_n5-n25

6.20.1	Common for 1 band UL and 2 bands UL CA

6.20.1.1	Operating bands for CA

Table 6.20.1.1-1: CA band combination of band n5+n25

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
| n25 | 1850 MHz | – | 1915 MHz | 1930 MHz | – | 1995 MHz | FDD |

6.20.1.2	Channel bandwidths per operating band for CA

Table 6.20.1.2-1: Supported bandwidths per CA band combination of band n5+n25

| CA operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 80 | 90 | 100 | Bandwidth combination set |
| CA_n5A-n25A | CA_n5A-n25A | n5 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n25 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |

6.20.1.3	UE co-existence studies

Table 6.20.1.3-1/2 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_n5-n25.

Table 6.20.1.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | nth Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n5 | 824 | 849 | 869 | 894 | 1648 | 1698 | 2472 | 2547 |  |  |
| n25 | 1850 | 1915 | 1930 | 1995 | 3700 | 3830 | 5550 | 5745 |  |  |

Table 6.20.1.3-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | mth Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n5 | 824 | 849 | 869 | 894 | 1738 | 1788 | 2607 | 2682 |  |  |
| n25 | 1850 | 1915 | 1930 | 1995 | 3860 | 3990 | 5790 | 5985 |  |  |

Based on above table, there is no harmonic relations for n5+n25.

6.20.1.4	∆TIB and ∆RIB values

For CA_n5-n25, the TIB,c and RIB,c values are reused from the similar combination E-UTR CA_2-5 and are given in the tables below.

Table 6.20.1.4-1: ΔTIB,c

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n5-n25 | n5 | 0.3 |
|  | n25 | 0.3 |

Table 6.20.1.4-2: ΔRIB,c

| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n5-n25 | n5 | 0 |
|  | n25 | 0 |

6.20.1.5	REFSENS requirements

There is no REFSENS exception for 1UL/2DL of for CA_n5-n25.

6.20.1.6	OOB blocking exception requirements

There is no OOB blocking exception for this CA band combination.

6.20.2	Specific for 2 bands UL CA

6.20.2.1	Maximum output power for inter-band CA

Table 6.20.2.2-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n5A-n25A | 23 | +2/-32 |
| NOTE 2: 2 refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB |  |  |

6.20.2.2	UE co-existence studies

Table 6.20.2.1-1 lists Band n5 +Band n25 2UL bands CA  2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

| UE UL carriers | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| UL frequencies (MHz) | 824 | 849 | 1850 | 1915 |
| 2nd order IMD products | f2_low – f1_high | f2_high – f1_low | f2_low + f1_low | f2_high + f1_high |
| IMD frequency limit (MHz) | 1001 | 1091 | 2674 | 2764 |
| 3rd order IMD products | 2*f1_low – f2_high | 2*f1_high – f2_low | 2*f2_low – f1_high | 2*f2_high – f1_low |
| IMD frequency limit (MHz) | 267 | 152 | 2851 | 3006 |
| 3rd order IMD products | 2*f1_low + f2_low | 2*f1_high + f2_high | 2*f2_low + f1_low | 2*f2_high + f1_high |
| IMD frequency limit (MHz) | 3498 | 3613 | 4524 | 4679 |
| 4th order IMD products | 3*f1_low – f2_high | 3*f1_high – f2_low | 3*f2_low – f1_high | 3*f2_high – f1_low |
| IMD frequency limit (MHz) | 557 | 697 | 4701 | 4921 |
| 4th order IMD products | 3*f1_low + f2_low | 3*f1_high + f2_high | 3*f2_low + f1_low | 3*f2_high + f1_high |
| IMD frequency limit (MHz) | 4322 | 4462 | 6374 | 6594 |
| 4th order IMD products | 2*f1_low – 2*f2_high | 2*f1_high – 2*f2_low | 2*f1_low + 2*f2_low | 2*f1_high + 2*f2_high |
| IMD frequency limit (MHz) | 2182 | 2002 | 5348 | 5528 |
| 5th order IMD products | f1_low – 4*f2_high | f1_high – 4*f2_low | f2_low – 4*f1_high | f2_high – 4*f1_low |
| IMD frequency limit (MHz) | 6836 | 6551 | 1546 | 1381 |
| 5th order IMD products | f1_low + 4*f2_low | f1_high + 4*f2_high | f2_low + 4*f1_low | f2_high + 4*f1_high |
| IMD frequency limit (MHz) | 8224 | 8509 | 5146 | 5311 |
| 5th order IMD products | 2*f1_low – 3*f2_high | 2*f1_high - 3*f2_low | 2*f2_low – 3*f1_high | 2*f2_high – 3*f1_low |
| IMD frequency limit (MHz) | 4097 | 3852 | 1153 | 1358 |
| 5th order IMD products | 2*f1_low + 3*f2_low | 2*f1_high + 3*f2_high | 2*f2_low + 3*f1_low | 2*f2_high + 3*f1_high |
| IMD frequency limit (MHz) | 7198 | 7443 | 6172 | 6377 |

Based on the table above, there is no IMD issue for 2UL n5+n25.

Table 6.20.2.2-2 lists the protected bands required for the 2UL bands CA configuration.

Table 6.20.2.2-2: Protected bands for the 2UL bands CA configuration

| UL NR CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n5-n25 | E-UTRA Band 2, 4, 5, 10, 12, 13, 14, 17, 25, 26, 28, 29, 30, 42, 48, 50, 51, 53, 66, 70, 71, 74, 85, | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 41, 43, NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| NOTE 1: FDL_low and FDL_high refer to each frequency band specified in Table 5.2-1 in TS 38.101-1 or Table 5.5-1 in TS 36.101 NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.5.3.1-2 are permitted for each assigned NR carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x 180kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. |  |  |  |  |  |  |  |

6.20.2.3	REFSENS requirements

There is no REFSENS exception for 2UL/2DL of for CA_n5-n25.

6.21		CA_n25-n77

6.21.1	Common for 1 band UL and 2 bands UL CA

6.21.1.1	Operating bands for CA

Table 6.21.1.1-1: CA band combination of band n25+n77

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n25 | 1850 MHz | – | 1915 MHz | 1930 MHz | – | 1995 MHz | FDD |
| n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

6.21.1.2	Channel bandwidths per operating band for CA

Table 6.21.1.2-1: Supported bandwidths per CA band combination of band n25+n77

|  | CA operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration |  | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Bandwidth combination set |
| CA_n25A-n77A |  | CA_n25A-n77A | n25 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n77 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

6.21.1.3	UE co-existence studies

Table 6.21.1.3-1/2 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_n25-n77.

Table 6.21.1.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | nth Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n25 | 1850 | 1915 | 1930 | 1995 | 3700 | 3830 | 5550 | 5745 |  |  |
| n77 | 3300 | 4200 | 3300 | 4200 | 6600 | 8400 | 9900 | 12600 |  |  |

Table 6.21.1.3-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | mth Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n25 | 1850 | 1915 | 1930 | 1995 | 3860 | 3990 | 5790 | 5985 |  |  |
| n77 | 3300 | 4200 | 3300 | 4200 | 6600 | 8400 | 9900 | 12600 |  |  |

Based on above table, the 2nd harmonic of band n25 uplink may fall into band n77.

Based on above table, the 2nd harmonic of band n25 downlink may fall into band 77 thus there can be harmonic mixing issue for the band combination of n25 and n77.

6.21.1.4	∆TIB and ∆RIB values

For CA_n25-n77 , the TIB,c and RIB,c values are reused from the similar combination CA_n2-n77 and are given in the tables below.

Table 6.21.1.4-1: ΔTIB,c

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n25-n77 | n25 | 0.6 |
|  | n77 | 0.8 |

Table 6.21.1.4-2: ΔRIB,c

| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n25-n77 | n25 | 0.2 |
|  | n77 | 0.5 |

6.21.1.5	REFSENS requirements

The MSD due to the harmonic issue is the same as the one for CA_n2-n77, which is reused for CA_n25-n77.

Table 6.21.1.5-1: Reference sensitivity exceptions due to UL harmonic for NR CA FR1

| MSD due to harmonic exception for the DL band |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz |
|  |  | dB | dB | dB | dB | dB | dB | dB | dB | dB | dB | dB | dB |
| n25 | n771, 2 |  | 23.9 | 22.1 | 20.9 | 19.8 | 19.0 | 17.9 | 16.8 | 16.0 | 15.5 | 14.8 | 14.3 |
|  | n773 |  | 1.1 | 0.8 | 0.3 | 0.1 |  |  |  |  |  |  |  |
| NOTE 1: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band and a range ∆FHD above and below the edge of this downlink transmission bandwidth. The value ∆FHD depends on the band combination: ∆FHD = 10 MHz for CA_n1-n77, CA_n2-n78, CA_n3-n77, CA_n3-n78, CA_n2-n48, CA_n25-n77, CA_n25-n78, CA_n48-n66, CA_n66-n78. NOTE 2: The requirements should be verified for UL NR-ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 3: The requirements are only applicable to channel bandwidths no larger than 20 MHz and with a carrier frequency at MHz offset from in the victim (higher band) with , whereandare the channel |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.21.1.5-2: Uplink configuration for reference sensitivity exceptions due to UL harmonic interference for NR CA, FR1

| NR Band / Channel bandwidth of the high band |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz |
| n2 | n77 |  | 25 | 36 | 50 | 50 | 50 | 50 | 50 | 50 | 50 | 50 | 50 |

The MSD due to the harmonic mixing issue is the same as the one for CA_n2-n77, which is reused for CA_n25-n77.

Table 6.21.1.5-3: Reference sensitivity exceptions due to harmonic mixing for CA in NR FR1

| NR Band / Channel bandwidth of the affected DL band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz(dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 70 MHz(dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| n77 | n25 | 6.7 | 5.0 | 4.0 | 3.7 |  |  |  |  |  |  |  |  |  |

Table 6.21.1.5-4: Uplink configuration for reference sensitivity exceptions due to receiver harmonic mixing for CA in NR

| NR Band / SCS / Channel bandwidth of the affected DL band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| n77 | n25 | 15 | 25 | 50 | 75 | 100 |  |  |  |  |  |  |  |  |  |

6.21.1.6	OOB blocking exception requirements

There is no OOB blocking exception for this CA band combination.

6.21.2	Specific for 2 bands UL CA

6.21.2.1	Maximum output power for inter-band CA

Table 6.21.2.2-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n25A-n77A | 23 | +2/-32 |
| NOTE 2: 2 refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB |  |  |

6.21.2.2	UE co-existence studies

Table 6.3.2.1-1 lists Band n25 +Band n77 2UL bands CA  2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.21.2.2-1: Band n25 and Band n77 UL IMD products

| UE UL carriers | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| UL frequencies (MHz) | 1850 | 1915 | 3300 | 4200 |
| 2nd order IMD products | f2_low – f1_high | f2_high – f1_low | f2_low + f1_low | f2_high + f1_high |
| IMD frequency limit (MHz) | 1385 | 2350 | 5150 | 6115 |
| 3rd order IMD products | 2*f1_low – f2_high | 2*f1_high – f2_low | 2*f2_low – f1_high | 2*f2_high – f1_low |
| IMD frequency limit (MHz) | 500 | 530 | 4685 | 6550 |
| 3rd order IMD products | 2*f1_low + f2_low | 2*f1_high + f2_high | 2*f2_low + f1_low | 2*f2_high + f1_high |
| IMD frequency limit (MHz) | 7000 | 8030 | 8450 | 10315 |
| 4th order IMD products | 3*f1_low – f2_high | 3*f1_high – f2_low | 3*f2_low – f1_high | 3*f2_high – f1_low |
| IMD frequency limit (MHz) | 1350 | 2445 | 7985 | 10750 |
| 4th order IMD products | 3*f1_low + f2_low | 3*f1_high + f2_high | 3*f2_low + f1_low | 3*f2_high + f1_high |
| IMD frequency limit (MHz) | 8850 | 9945 | 11750 | 14515 |
| 4th order IMD products | 2*f1_low – 2*f2_high | 2*f1_high – 2*f2_low | 2*f1_low + 2*f2_low | 2*f1_high + 2*f2_high |
| IMD frequency limit (MHz) | 4700 | 2770 | 10300 | 12230 |
| 5th order IMD products | f1_low – 4*f2_high | f1_high – 4*f2_low | f2_low – 4*f1_high | f2_high – 4*f1_low |
| IMD frequency limit (MHz) | 14950 | 11285 | 4360 | 3200 |
| 5th order IMD products | f1_low + 4*f2_low | f1_high + 4*f2_high | f2_low + 4*f1_low | f2_high + 4*f1_high |
| IMD frequency limit (MHz) | 15050 | 18715 | 10700 | 11860 |
| 5th order IMD products | 2*f1_low – 3*f2_high | 2*f1_high - 3*f2_low | 2*f2_low – 3*f1_high | 2*f2_high – 3*f1_low |
| IMD frequency limit (MHz) | 8900 | 6070 | 855 | 2850 |
| 5th order IMD products | 2*f1_low + 3*f2_low | 2*f1_high + 3*f2_high | 2*f2_low + 3*f1_low | 2*f2_high + 3*f1_high |
| IMD frequency limit (MHz) | 13600 | 16430 | 12150 | 14145 |

Based on the table above, the 2nd and 4th order IMDs may fall into Rx frequencies of band n25.

Further, the 4th order and 5th order IMDs may fall into Rx frequencies of band n77.

Table 6.3.2.2-2 lists the protected bands required for the 2UL bands CA configuration (the same as CA_n2-n77).

Table 6.21.2.2-2: Protected bands for the 2UL bands CA configuration

| UL NR CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n25-n77 | E-UTRA Band 4, 5, 12, 13, 14, 17, 26, 29, 30, 41, 65, 66, 70, 71 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2, 25 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| NOTE 1: FDL_low and FDL_high refer to each frequency band specified in Table 5.2-1 in TS 38.101-1 or Table 5.5-1 in TS 36.101 NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.5.3.1-2 are permitted for each assigned NR carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x RBsize kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. |  |  |  |  |  |  |  |

6.21.2.3	REFSENS requirements

Table 6.21.2.3-1 lists the MSD required due to the 2nd, 3rd, and 5th IMD for the dual uplink configuration The MSD value is reused from CA_n2-n77.

Table 6.21.2.3-1: 2DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n25A-n77A | n25 | 1855 | 5 | 25 | 1935 | 26 | FDD | IMD2 |
|  | n77 | 3790 | 10 | 50 | 3790 | N/A | TDD | N/A |
|  | n25 | 1885 | 5 | 25 | 1965 | 8.0 | FDD | IMD4 |
|  | n77 | 3690 | 10 | 50 | 3690 | N/A | TDD | N/A |
|  | n25 | 1885 | 5 | 25 | 1965 | 5 | FDD | IMD5 |
|  | n77 | 3790 | 10 | 50 | 3790 | N/A | TDD | N/A |

6.22	CA_n8-n28

6.22.1	Common for 1 band UL and 2 bands UL CA

6.22.1.1 Operating bands for CA

Table 6.22.1.1-1: CA band combination CA_n8A-n28A

| NR CA Band Combination | NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n8-n28 | n8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |

6.22.1.2	Channel bandwidths per operating band for CA

Table 6.22.1.2-1: Supported bandwidths per CA band combination CA_n8A-n28A

|  | CA operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration |  | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Bandwidth combination set |
| CA_n8A-n28A |  | - | n8 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n28 | 15 | Yes | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.22.1.3	UE Co-existence studies

Table 6.22.1.3-1/2 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_n8-n28.

Table 6.22.1.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n8 | 880 | 915 | 925 | 960 | 1760 | 1830 | 2640 | 2745 | 3520 | 3660 |
| n28 | 703 | 748 | 758 | 803 | 1406 | 1496 | 2109 | 2244 | 2812 | 2992 |

Based on above table, there is no harmonic interference.

Table 6.22.1.3-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n8 | 880 | 915 | 925 | 960 | 1850 | 1920 | 2775 | 2880 | 3700 | 3840 |
| n28 | 703 | 748 | 758 | 803 | 1516 | 1606 | 2274 | 2409 | 3032 | 3212 |

Based on above table, there is no harmonic mixing issue.

6.22.1.4	∆TIB and ∆RIB values

For CA_n8A-n28A, the TIB,c and RIB are given in the tables below reusing the values of DC_8_n28.

Table 6.22.1.4-1: ΔTIB,c

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n8-n28 | n8 | 0.6 |
|  | n28 | 0.5 |

Table 6.22.1.4-2: ΔRIB

| Inter-band CA Configuration | NR Band | ΔRIB [dB] |
| --- | --- | --- |
| CA_n8-n28 | n8 | 0.2 |
|  | n28 | 0.2 |

6.22.1.5	REFSENs requirements

There is no MSD exception requirement for CA_n8-n28.

6.22.1.6	OOB blocking exception requirements

There is no OOB blocking exception requirement for CA_n8-n28.

6.23	CA_n3-n7

6.23.1	Common for 1 band UL and 2 bands UL CA

6.23.1.1 Operating bands for CA

Table 6.23.1.1-1: CA band combination CA_n3A-n7A

| NR CA Band Combination | NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n3-n7 | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |

6.23.1.2	Channel bandwidths per operating band for CA

Table 6.23.1.2-1: Supported bandwidths per CA band combination CA_n3A-n7A

|  | CA operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration |  | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Bandwidth combination set |
| CA_n3A-n7A |  | - | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  | 1 |
|  |  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | n7 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |

6.23.1.3	UE Co-existence studies

Table 6.23.1.3-1/2 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_n3-n7.

Table 6.23.1.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n3 | 1710 | 1785 | 1805 | 1880 | 3420 | 3570 | 5130 | 5355 | 6840 | 7140 |
| n7 | 2500 | 2570 | 2620 | 2690 | 5000 | 5140 | 7500 | 7710 | 10000 | 10280 |

Based on above table, there is no harmonic interference.

Table 6.23.1.3-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n3 | 1710 | 1785 | 1805 | 1880 | 3610 | 3760 | 5415 | 5640 | 7220 | 7520 |
| n7 | 2500 | 2570 | 2620 | 2690 | 5240 | 5380 | 7860 | 8070 | 10480 | 10760 |

Based on above table, there is no harmonic mixing issue.

6.23.1.4	∆TIB and ∆RIB values

For CA_n3A-n7A, the TIB,c and RIB are given in the tables below reusing the values of DC_3_n7.

Table 6.23.1.4-1: ΔTIB,c

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n3-n7 | n3 | 0.5 |
|  | n7 | 0.5 |

Table 6.23.1.4-2: ΔRIB

| Inter-band CA Configuration | NR Band | ΔRIB [dB] |
| --- | --- | --- |
| CA_n3-n7 | n3 | 0 |
|  | n7 | 0 |

6.23.1.5	REFSENs requirements

The MSD due to cross band isolation for CA_n3-n7 is shown below, referring to DC_3_n41.

Table 6.23.1.5-1: Reference sensitivity exceptions (MSD) due to cross band isolation for PC3 EN-DC in NR FR1

| E-UTRA or NR Band / Channel bandwidth of the affected DL band / MSD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 70 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| n7 | n3 | 0.6 | 0.6 | 0.6 | 0.6 | 0.6 | 0.6 | 0.6 |  |  |  |  |  |  |

Table 6.23.1.5-2: Uplink configuration for reference sensitivity exceptions due to cross band isolation for EN-DC in NR FR1

| E-UTRA or NR Band / SCS / Channel bandwidth of the affected DL band / UL RB allocation of the agressor band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | SCS of UL band (kHz) | 5 MHz (LCRB) | 10 MHz (LCRB) | 15 MHz (LCRB) | 20 MHz (LCRB) | 25 MHz (LCRB) | 30 MHz (LCRB) | 40 MHz (LCRB) | 50 MHz (LCRB) | 60 MHz (LCRB) | 70 MHz (LCRB) | 80 MHz (LCRB) | 90 MHz (LCRB) | 100 MHz (LCRB) |
| n7 | n3 | 15 | 270 | 270 | 270 | 270 | 270 | 270 | 270 |  |  |  |  |  |  |

6.23.1.6	OOB blocking exception requirements

There is no OOB blocking exception requirement for CA_n3-n7.

## 6.24	CA_n3-n18

### 6.24.1		Common for 1 band UL and 2 bands UL CA

#### 6.24.1.1		Operating bands for CA

Table 6.24.1.1-1: CA band combination of band n3+n18

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
| n18 | 815 MHz | – | 830 MHz | 860 MHz | – | 875 MHz | FDD |

#### 6.24.1.2		Channel bandwidths per operating band for CA

Table 6.24.1.2-1: Supported bandwidths per CA band combination of band n3+n18

| NR CA Configuration | UL Configuration | NR Band | Channel bandwidth (MHz) (NOTE 3) |  |  |  |  |  |  |  |  |  |  |  |  | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n3A-n18A | CA_n3A-n18A | n3 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  | 0 |
|  |  | n18 | 5 | 10 | 15 |  |  |  |  |  |  |  |  |  |  |  |

#### 6.24.1.3		Co-existence studies

Table 6.24.1.3-1/2 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_n3-n18.

Table 6.24.1.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n3 | 1710 | 1785 | 1805 | 1880 | 3420 | 3570 | 5130 | 5355 | 6840 | 7140 |
| n18 | 815 | 830 | 860 | 875 | 1630 | 1660 | 2445 | 2490 | 3260 | 3320 |

Based on above table, there is no harmonic issue for the band combination of n3 and n18.

Table 6.24.1.3-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n3 | 1710 | 1785 | 1805 | 1880 | 3420 | 3570 | 5130 | 5355 | 6840 | 7140 |
| n18 | 815 | 830 | 860 | 875 | 1720 | 1750 | 2580 | 2625 | 3440 | 3500 |

Based on above table, there is no harmonic mixing issue for the band combination of n3 and n18.

6.24.1.4		∆TIB and ∆RIB values

For CA_n3-n18 , the ∆TIB,c and ∆RIB,c values are given in the tables below which refer to TS 36.101 CA_3-18 relaxation values.

Table 6.24.1.4-1: ΔTIB,c

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n3-n18 | n3 | 0.3 |
|  | n18 | 0.3 |

Table 6.24.1.4-2: ΔRIB,c

| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n3-n18 | n3 | 0 |
|  | n18 | 0 |

6.24.1.5 REFSENS requirements

There are no specific REFSENS requirements for 1 band UL.

6.24.1.6	OOB blocking exception requirements

There is no OOB blocking exception for this CA band combination.

6.24.2	Specific for 2 bands UL CA

6.24.2.1	Maximum output power for inter-band CA

Table 6.24.2.2-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n3A-n18A | 23 | +2/-32 |
| NOTE 2: 2 refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB |  |  |

6.24.2.2	UE co-existence studies

Table 6.24.2.1-1 lists Band n3 +Band n18 2UL bands CA  2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.24.2.2-1: Band n3 and Band n18 UL IMD products

| UE UL carriers | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| UL frequencies (MHz) | 815 | 830 | 1710 | 1785 |
| 2nd order IMD products | ¦f2_low – f1_high¦ | ¦f2_high – f1_low¦ | ¦f2_low + f1_low¦ | ¦f2_high + f1_high¦ |
| IMD frequency limit (MHz) | 880 | 970 | 2525 | 2615 |
| 3rd order IMD products | ¦f2_low – 2*f1_high¦ | ¦f2_high – 2*f1_low¦ | ¦2*f2_low – f1_high¦ | ¦2*f2_high – f1_low¦ |
| IMD frequency limit (MHz) | 50 | 155 | 2590 | 2755 |
| 3rd order IMD products | ¦2*f1_low + f2_low¦ | ¦2*f1_high + f2_high¦ | ¦2*f2_low + f1_low¦ | ¦2*f2_high + f1_high¦ |
| IMD frequency limit (MHz) | 3340 | 3445 | 4235 | 4400 |
| 4th order IMD products | ¦3*f1_low – f2_high¦ | ¦3*f1_high – f2_low¦ | ¦3*f2_low – f1_high¦ | ¦3*f2_high –f1_low¦ |
| IMD frequency limit (MHz) | 660 | 780 | 4300 | 4540 |
| 4th order IMD products | ¦3*f1_low +f2_low¦ | ¦3*f1_high + f2_high¦ | ¦3*f2_low+f1_low¦ | ¦3*f2_high +f1_high¦ |
| IMD frequency limit (MHz) | 4155 | 4275 | 5945 | 6185 |
| 4th order IMD products | ¦2*f1_low –2*f2_high¦ | ¦2*f1_high –2*f2_low¦ | ¦2*f1_low +2*f2_low¦ | ¦2*f1_high +2*f2_high¦ |
| IMD frequency limit (MHz) | 1760 | 1940 | 5050 | 5230 |
| 5th order IMD products | ¦f1_low –4*f2_high¦ | ¦f1_high –4*f2_low¦ | ¦f2_low –4*f1_high¦ | ¦f2_high –4*f1_low¦ |
| IMD frequency limit (MHz) | 6010 | 6325 | 1475 | 1610 |
| 5th order IMD products | ¦f1_low +4*f2_low¦ | ¦f1_high +4*f2_high¦ | ¦f2_low+4*f1_low¦ | ¦f2_high +4*f1_high¦ |
| IMD frequency limit (MHz) | 7655 | 7970 | 4970 | 5105 |
| 5th order IMD products | ¦2*f1_low –3*f2_high¦ | ¦2*f1_high –3*f2_low¦ | ¦2*f2_low -3*f1_high¦ | ¦2*f2_high -3*f1_low¦ |
| IMD frequency limit (MHz) | 3470 | 3725 | 930 | 1125 |
| 5th order IMD products | ¦2*f1_low +3*f2_low¦ | ¦2*f1_high +3*f2_high¦ | ¦2*f2_low+3*f1_low¦ | ¦2*f2_high +3*f1_high¦ |
| IMD frequency limit (MHz) | 6760 | 7015 | 5865 | 6060 |

Based on the table above, the 4th order IMD may fall into Rx frequencies of band n3.

Table 6.24.2.2-2 lists the protected bands required for the 2UL bands CA configuration.

Table 6.24.2.2-2: Protected bands for the 2UL bands CA configuration

| UL NR CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n3-n18 | E-UTRA Band 1, 3, 11, 21, 28, 34, 65 NR Band n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.5.3.1-2 are permitted for each assigned NR carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x RBsize kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 3: Applicable when co-existence with PHS system operating in 1884.5 -1915.7 MHz |  |  |  |  |  |  |  |

6.24.2.3	REFSENS requirements

Table 6.24.2.3-1 lists the MSD required due to the 4th IMD for the dual uplink configuration The MSD value is reused from TS 38.101-3 DC_18A_n3A.

Table 6.24.2.3-1: MSD due to IMD issue

| Operating band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA Configuration | Operating band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n3A-n18A | n18 | 818 | 5 | 25 | 863 | N/A | FDD | N/A |
|  | n3 | 1731 | 5 | 25 | 1826 | 4 | FDD | IMD4 |

## 6.25	CA_n18-n41

### 6.25.1		Common for 1 band UL and 2 bands UL CA

#### 6.25.1.1		Operating bands for CA

Table 6.25.1.1-1: CA band combination of band n18+n41

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n18 | 815 MHz | – | 830 MHz | 860 MHz | – | 875 MHz | FDD |
| n411 | 2496 MHz |  | 2690 MHz | 2496 MHz |  | 2690 MHz | TDD |

NOTE 1: 	The frequency range in band n41 is restricted for this band combination to 2595 – 2645 MHz.

#### 6.25.1.2		Channel bandwidths per operating band for CA

Table 6.25.1.2-1: Supported bandwidths per CA band combination of band n18+n41

| NR CA Configuration | UL Configuration | NR Band | SCS [kHz] | Channel bandwidth (MHz) (NOTE 3) |  |  |  |  |  |  |  |  |  |  |  |  | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n18A-n41A | CA_n18A-n41A | n18 | 15 | 5 | 10 | 15 |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n41 | 15 |  | 10 | 15 | 20 |  | 30 | 40 | 50 | 60 |  | 80 | 90 | 100 |  |

#### 6.25.1.3		Co-existence studies

Table 6.25.1.3-1/2 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA _ n18-n41.

Table 6.25.1.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n18 | 815 | 830 | 860 | 875 | 1630 | 1660 | 2445 | 2490 | 3260 | 3320 |
| n41 | 2595 | 2645 | 2595 | 2645 | 5190 | 5290 | 7785 | 7935 | 10380 | 10580 |

Based on above table, there is no harmonic issue for the band combination of n18 and n41.

Table 6.25.1.3-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n18 |  | 815 |  | 830 |  | 860 |  | 875 |  | 1720 |  | 1750 |  | 2580 |  | 2625 |  | 3440 |  | 3500 |  |
| n41 |  | 2595 |  | 2645 |  | 2595 |  | 2645 |  | 5190 |  | 5290 |  | 7785 |  | 7935 |  | 10380 |  | 10580 |  |

Based on above table, there is n18 3rd harmonic mixing issue to n41.

6.25.1.4		∆TIB and ∆RIB values

For CA_n18-n41 , the ∆TIB,c and ∆RIB,c values are given in the tables below which refer to TS 36.101 CA_8-41 relaxation values.

Table 6.25.1.4-1: ΔTIB,c

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n18-n41 | n18 | 0.3 |
|  | n41 | 0.3 |

Table 6.25.1.4-2: ΔRIB,c

| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n18-n41 | n18 | 0 |
|  | n41 | 0 |

#### 6.25.1.5		REFSENS requirements

#### IMD2 may fall in n18 but due to spectrum holding considerations, this IMD2 is not applicable.

#### The MSD for n18 3rd harmonic mixing issue to n41 can refer to 36.101 CA_20-41 value.

Table 6.25.1.5-1: Reference sensitivity exceptions due to harmonic mixing for CA in NR FR1

| NR Band / Channel bandwidth of the affected DL band |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| n413,4 | n18 | [24.3] | [24.3] | [22.5] |  |  |  |  |  |  |  |  |
| NOTE 3: These requirements apply when there is at least one individual RE within the downlink transmission bandwidth of the victim (lower) band for which the 3rd harmonic is within the uplink transmission bandwidth or the uplink adjacent channel's transmission bandwidth of an aggressor (higher) band. NOTE 4: The requirements should be verified for UL NR-ARFCN of the aggressor (higher) band (superscript HB) such that in MHz and with the carrier frequency in the victim (lower) band and the channel bandwidth configured in the higher band. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.25.1.5-2: Uplink configuration for reference sensitivity exceptions due to receiver harmonic mixing for CA in NR FR1

| NR Band / SCS / Channel bandwidth of the affected DL band |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz |
| n41 | n18 | 15 | 25 | 50 | 75 |  |  |  |  |  |  |  |  |
| NOTE 1: The UL configuration applies regardless of the channel bandwidth of the UL band unless the UL resource blocks exceed that specified in Table 7.3.2-3 for the uplink bandwidth in which case the allocation according to Table 7.3.2-3 applies. |  |  |  |  |  |  |  |  |  |  |  |  |  |

#### 6.25.1.6	    OOB blocking exception requirements

There is no OOB blocking exception for this CA band combination.

6.25.2	Specific for 2 bands UL CA

6.25.2.1	Maximum output power for inter-band CA

Table 6.25.2.2-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n18A-n41A | 23 | +2/-32 |
| NOTE 2: 2 refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB |  |  |

6.25.2.2	UE co-existence studies

Table 6.25.2.2-1 lists Band n18 +Band n41 2UL bands CA  2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.25.2.2-1: Band n18 and Band n41 UL harmonics and IMD products

| UE UL carriers | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| UL frequencies (MHz) | 815 | 830 | 2595 | 2645 |
| 2nd order IMD products | ¦f2_low – f1_high¦ | ¦f2_high – f1_low¦ | ¦f2_low + f1_low¦ | ¦f2_high + f1_high¦ |
| IMD frequency limit (MHz) | 1765 | 1830 | 3410 | 3475 |
| 3rd order IMD products | ¦f2_low – 2*f1_high¦ | ¦f2_high – 2*f1_low¦ | ¦2*f2_low – f1_high¦ | ¦2*f2_high – f1_low¦ |
| IMD frequency limit (MHz) | 935 | 1015 | 4360 | 4475 |
| 3rd order IMD products | ¦2*f1_low + f2_low¦ | ¦2*f1_high + f2_high¦ | ¦2*f2_low + f1_low¦ | ¦2*f2_high + f1_high¦ |
| IMD frequency limit (MHz) | 4225 | 4305 | 6005 | 6120 |
| 4th order IMD products | ¦3*f1_low – f2_high¦ | ¦3*f1_high – f2_low¦ | ¦3*f2_low – f1_high¦ | ¦3*f2_high –f1_low¦ |
| IMD frequency limit (MHz) | 105 | 200 | 6955 | 7120 |
| 4th order IMD products | ¦3*f1_low +f2_low¦ | ¦3*f1_high + f2_high¦ | ¦3*f2_low+f1_low¦ | ¦3*f2_high +f1_high¦ |
| IMD frequency limit (MHz) | 5040 | 5135 | 8600 | 8765 |
| 4th order IMD products | ¦2*f1_low –2*f2_high¦ | ¦2*f1_high –2*f2_low¦ | ¦2*f1_low +2*f2_low¦ | ¦2*f1_high +2*f2_high¦ |
| IMD frequency limit (MHz) | 3530 | 3660 | 6820 | 6950 |
| 5th order IMD products | ¦f1_low –4*f2_high¦ | ¦f1_high –4*f2_low¦ | ¦f2_low –4*f1_high¦ | ¦f2_high –4*f1_low¦ |
| IMD frequency limit (MHz) | 9550 | 9765 | 615 | 725 |
| 5th order IMD products | ¦f1_low +4*f2_low¦ | ¦f1_high +4*f2_high¦ | ¦f2_low+4*f1_low¦ | ¦f2_high +4*f1_high¦ |
| IMD frequency limit (MHz) | 11195 | 11410 | 5855 | 5965 |
| 5th order IMD products | ¦2*f1_low –3*f2_high¦ | ¦2*f1_high –3*f2_low¦ | ¦2*f2_low -3*f1_high¦ | ¦2*f2_high -3*f1_low¦ |
| IMD frequency limit (MHz) | 6125 | 6305 | 2700 | 2845 |
| 5th order IMD products | ¦2*f1_low +3*f2_low¦ | ¦2*f1_high +3*f2_high¦ | ¦2*f2_low+3*f1_low¦ | ¦2*f2_high +3*f1_high¦ |
| IMD frequency limit (MHz) | 9415 | 9595 | 7635 | 7780 |

Based on the table above, there is no IMD issue for this band combination.

Table 6.25.2.2-2: Protected bands for the 2UL bands CA configuration

| UL NR CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n18-n41 | E-UTRA Band 1, 3, 34, 42, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2, 25 | FDL_low | - | FDL_high | -50 | 1 | 4 |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 758 | - | 799 | -50 | 1 |  |
|  | Frequency range | 799 | - | 803 | -40 | 1 |  |
|  | Frequency range | 860 | - | 890 | -40 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.5.3.1-2 are permitted for each assigned NR carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x 180kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 3: Applicable when co-existence with PHS system operating in 1884.5 -1915.7 MHz NOTE 4: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth. |  |  |  |  |  |  |  |

6.25.2.3	REFSENS requirements

No additional MSD requirement need to be defined for this band combs since no IMD issue as analysis.

## 6.26	CA_n25-n29

### 6.26.1		Common for 1 band UL and 2 bands UL CA

#### 6.26.1.1		Operating bands for CA

Table 6.26.1.1-1: CA band combination of band n25+n29

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n25 | 1850 MHz | – | 1915 MHz | 1930 MHz | – | 1995 MHz | FDD |
| n29 | N/A |  |  | 717 MHz | – | 728 MHz | SDL |

#### 6.26.1.2		Channel bandwidths per operating band for CA

Table 6.26.1.2-1: Supported bandwidths per CA band combination of band n25+n29

|  | CA operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration |  | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Bandwidth combination set |
| CA_n25A-n29A |  | - | n25 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  | 0 |
|  |  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | n29 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

#### 6.26.1.3		Co-existence studies

Table 6.26.1.3-1/2 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_n25-n29.

Table 6.26.1.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n25 | 1850 | 1915 | 1930 | 1995 | 3700 | 3830 | 5550 | 5745 | 7400 | 7660 |
| n29 | 0 | 0 | 717 | 728 | 0 | 0 | 0 | 0 | 0 | 0 |

Based on above table, no harmonic for CA_n25-n29

Table 6.26.1.3-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n25 | 1850 | 1915 | 1930 | 1995 | 3860 | 3990 | 5790 | 5985 | 7720 | 7980 |
| n29 | 0 | 0 | 717 | 728 | 1434 | 1456 | 2151 | 2184 | 2868 | 2912 |

Based on above table, no harmonic for CA_n25-n29.

6.26.1.4		∆TIB and ∆RIB values

For CA_n25-n29 , the ΔTIB,c and ΔRIB,c values are given in the tables below. The values same as CA_n29-n70 what are specified in TS 38.101-1.

Table 6.26.1.4-1: ΔTIB,c

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n25-n29 | n25 | 0.3 |
|  | n29 | 0 |

Table 6.26.1.4-2: ΔRIB,c

| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n25-n29 | n25 | 0 |
|  | n29 | 0 |

6.26.1.5 REFSENS requirements

There are no specific REFSENS requirements for 1 band UL.

6.26.1.6	OOB blocking exception requirements

There is no OOB blocking exception for this CA band combination.

## 6.27	CA_n25-n71

### 6.27.1		Common for 1 band UL and 2 bands UL CA

#### 6.27.1.1		Operating bands for CA

Table 6.27.1.1-1: CA band combination of band n25+n71

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n25 | 1850 MHz | – | 1915 MHz | 1930 MHz | – | 1995 MHz | FDD |
| n71 | 663 MHz | – | 698 MHz | 617 MHz | – | 652 MHz | FDD |

#### 6.27.1.2		Channel bandwidths per operating band for CA

Table 6.27.1.2-1: Supported bandwidths per CA band combination of band n25+n71

| NR CA Configuration | UL Configuration | NR Band | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n25A-n71A | CA_n25A-n71A | n25 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  | 1 |
|  |  | n71 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |

6.27.1.3		UE co-existence studies

Table 6.27.1.3-1 lists up to 7th harmonics for n25A-n71A. As can be seen, 3rd harmonic from n71 UL might fall into n25 DL.

Table 6.27.1.3-1: Impact of UL/DL Harmonic

|  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  | 6th Harmonic |  | 7th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n25 | 1850 | 1915 | 3700 | 3830 | 5550 | 5745 | 7400 | 7660 | 9250 | 9575 | 11100 | 11490 | 12950 | 13405 |
| n71 | 663 | 698 | 1326 | 1396 | 1989 | 2094 | 2652 | 2792 | 3315 | 3490 | 3978 | 4188 | 4641 | 4886 |

Table 6.27.1.3-2 list harmonic mixing issue for the 2DL bands CA with 1 UL. As can be seen, 3rd harmonic mixing from n71 DL might affect n25 UL.

Table 6.27.1.3-2 Harmonic mixing for 2DLs/1UL

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n25 | 1850 | 1915 | 1930 | 1995 | 3860 | 3990 | 5790 | 5985 | 7720 | 7980 |
| n71 | 663 | 698 | 617 | 652 | 1234 | 1304 | 1851 | 1956 | 2468 | 2608 |

6.27.1.4		∆TIB and ∆RIB values

For CA_n25A-n71, the TIB,c and RIB,c values are derived from LTE CA_4-17 which use a trap filter.

Table 6.27.1.4-1: ΔTIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n25-n71 | 25 | 0.3 |
|  | n71 | 0.6 |

Table 6.27.1.4-2: ΔRIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n25-n71 | 25 | 0 |
|  | n71 | 0.3 |

6.27.1.5 		REFSENS requirements

Due to identified harmonic issues MSD is derived from CA_4-17 and need to be defined in 38.101-1 as defined below.

Table 6.27.1.5-1: Reference sensitivity exceptions due to UL harmonic for NR CA FR1

| MSD due to harmonic exception for the DL band |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz |
|  |  | dB | dB | dB | dB | dB | dB | dB | dB | dB | dB | dB | dB |
| n71 | n256 | 10 | 7.5 | 6 | 5.1 | 4.1 | 3.0 | 2.1 |  |  |  |  |  |
| NOTE 6: These requirements apply when the lower edge frequency of the 10 MHz, 15 MHz, or 20 MHz uplink channel in Band 71 is located at or below 668 MHz and the downlink channel in Band n25 is located with its upper edge at 1995 MHz. NOTE 7: These requirements apply when there is at least one individual RE within the downlink transmission bandwidth of the victim (lower) band for which the 3rd harmonic is within the uplink transmission bandwidth or the uplink adjacent channel’s transmission bandwidth of an aggressor (higher) band. NOTE 8: The requirements should be verified for UL EARFCN of the aggressor (higher) band (superscript HB) such that in MHz and with the carrier frequency in the victim (lower) band and the channel bandwidth configured in the higher band. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.27.1.5-2: Uplink configuration for reference sensitivity exceptions due to UL harmonic interference for NR CA, FR1

| NR Band / Channel bandwidth of the high band |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz |
| n71 | n25 | 83 | 83 | 83 | 83 | 83 | 83 | 83 |  |  |  |  |  |
| NOTE 3: These requirements apply when the lower edge frequency of the uplink channel in Band n71 is located at or below 668 MHz and the downlink channel in Band n25 is located with its upper edge at 1995 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.27.1.5-3: Reference sensitivity exceptions due to harmonic mixing for CA in NR FR1

| NR Band / Channel bandwidth of the affected DL band |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| n25 | n713,4 | 26.5 | 23.3 | 20.9 | 15.3 |  |  |  |  |  |  |  |
| NOTE 3: These requirements apply when there is at least one individual RE within the downlink transmission bandwidth of the victim (lower) band for which the 3rd harmonic is within the uplink transmission bandwidth or the uplink adjacent channel's transmission bandwidth of an aggressor (higher) band. NOTE 4: The requirements should be verified for UL NR-ARFCN of the aggressor (higher) band (superscript HB) such that in MHz and with the carrier frequency in the victim (lower) band and the channel bandwidth configured in the higher band. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.27.1.5-4: Uplink configuration for reference sensitivity exceptions due to receiver harmonic mixing for CA in NR FR1

| NR Band / SCS / Channel bandwidth of the affected DL band |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz |
| n25 | n71 | 15 | 25 | 50 | 75 | 100 |  |  |  |  |  |  |  |
| NOTE 1: The UL configuration applies regardless of the channel bandwidth of the UL band unless the UL resource blocks exceed that specified in Table 7.3.2-3 for the uplink bandwidth in which case the allocation according to Table 7.3.2-3 applies. |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.27.2	Specific for 2 bands UL CA

6.27.2.1		UE co-existence studies

Table 6.27.2.1-1 lists Band n25 + Band n71 2UL bands CA  2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.27.2.1-1: Band n25 and Band n71 UL IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_high – fx_low¦ | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1252 | 1152 | 2513 | 2613 |
| 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 589 | 454 | 3002 | 3167 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3176 | 3311 | 4363 | 4528 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 2504 | 2304 | 5026 | 5226 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 74 | 244 | 4852 | 5082 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 3839 | 4009 | 6213 | 6443 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6997 | 6702 | 942 | 737 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 8063 | 8358 | 4502 | 4707 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 4419 | 4154 | 1606 | 1841 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6876 | 7141 | 5689 | 5924 |

Based on Table 6.27.2.1-1 there are no IMD issues affecting own Rx frequencies.

Table 6.27.2.1-2 lists the protected bands required for the 2UL bands CA configuration as to be used in Table 6.5A.3.2.3-1 of TS 38.101-1.

Table 6.27.2.1-2: Protected bands for the 2UL bands CA configuration

| UL NR CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n25-n71 | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 26, 30, 48, 53, 66, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 41, 70 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | NR Band n71 | FDL_low | - | FDL_high | -50 | 1 | 4 |
|  | E-UTRA Band 29 | FDL_low | - | FDL_high | -38 | 1 | 4 |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.5.3.1-2 are permitted for each assigned NR carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x 180kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 4: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth. |  |  |  |  |  |  |  |

6.27.2.2		REFSENS requirements

Based on the co-existence studies for CA_n25-n71 there are no further MSD needed to be defined.

## 6.28	CA_n13-n25

### 6.28.1		Common for 1 band UL and 2 bands UL CA

#### 6.28.1.1		Operating bands for CA

Table 6.28.1.1-1: CA band combination of band n13+n25

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n13 | 777 MHz | – | 787 MHz | 746 MHz | – | 756 MHz | FDD |
| n25 | 1850 MHz | – | 1915 MHz | 1930 MHz | – | 1995 MHz | FDD |

#### 6.28.1.2		Channel bandwidths per operating band for CA

Table 6.28.1.2-1: Supported bandwidths per CA band combination of band n13+n25

| NR CA Configuration | UL Configuration | NR Band | Channel bandwidth (MHz) (NOTE 3) |  |  |  |  |  |  |  |  |  |  |  |  | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n13A-n25A | CA_n13A-n25A | n13 | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n25 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |

#### 6.28.1.3		Co-existence studies

Table 6.28.1.3-1/2 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_n13-n25.

Table 6.28.1.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | nth Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n13 | 777 | 787 | 746 | 756 | 1554 | 1574 | 2331 | 2361 |  |  |
| n25 | 1850 | 1915 | 1930 | 1995 | 3700 | 3830 | 5550 | 5745 |  |  |

Table 6.28.1.3-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | mth Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n13 | 777 | 787 | 746 | 756 | 1492 | 1512 | 2238 | 2268 |  |  |
| n25 | 1850 | 1915 | 1930 | 1995 | 3860 | 3990 | 5790 | 5985 |  |  |

Based on above table, there is no harmonic and harmonic mixing issue for the band combination of n13 and n25.

6.28.1.4		∆TIB and ∆RIB values

For CA_n13-n25 , the ΔTIB,c and ΔRIB,c values are given in the tables below. The same values are shown as what are specified in TR 38.101-3 DC_13_n2.

Table 6.28.1.4-1: ΔTIB,c

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n13-n25 | n13 | 0.3 |
|  | n25 | 0.3 |

Table 6.28.1.4-2: ΔRIB,c

| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n13-n25 | n13 | 0 |
|  | n25 | 0 |

6.28.1.5 REFSENS requirements

There is neither harmonic issue nor harmonics mixing issue for the band combination of n13 and n25.

6.28.1.6	OOB blocking exception requirements

There is no OOB blocking exception for this CA band combination.

6.28.2	Specific for 2 bands UL CA

6.28.2.1	Maximum output power for inter-band CA

Table 6.28.2.2-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n13A-n25A | 23 | +2/-32 |
| NOTE 2: 2 refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB |  |  |

6.28.2.2	UE co-existence studies

Table 6.28.2.2-1 lists band n13+band n25 2UL CA 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.28.2.2-1: Band n13 and Band n25 UL IMD products

| UE UL carriers | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| UL frequencies (MHz) | 777 | 787 | 1850 | 1915 |
| 2nd order IMD products | ¦f2_low – f1_high¦ | ¦f2_high – f1_low¦ | ¦f2_low + f1_low¦ | ¦f2_high + f1_high¦ |
| IMD frequency limit (MHz) | 1063 | 1138 | 2627 | 2702 |
| 3rd order IMD products | ¦f2_low – 2*f1_high¦ | ¦f2_high – 2*f1_low¦ | ¦2*f2_low – f1_high¦ | ¦2*f2_high – f1_low¦ |
| IMD frequency limit (MHz) | 276 | 361 | 2913 | 3053 |
| 3rd order IMD products | ¦2*f1_low + f2_low¦ | ¦2*f1_high + f2_high¦ | ¦2*f2_low + f1_low¦ | ¦2*f2_high + f1_high¦ |
| IMD frequency limit (MHz) | 3404 | 3489 | 4477 | 4617 |
| 4th order IMD products | ¦3*f1_low – f2_high¦ | ¦3*f1_high – f2_low¦ | ¦3*f2_low – f1_high¦ | ¦3*f2_high –f1_low¦ |
| IMD frequency limit (MHz) | 416 | 511 | 4763 | 4968 |
| 4th order IMD products | ¦3*f1_low +f2_low¦ | ¦3*f1_high + f2_high¦ | ¦3*f2_low+f1_low¦ | ¦3*f2_high +f1_high¦ |
| IMD frequency limit (MHz) | 4181 | 4276 | 6327 | 6532 |
| 4th order IMD products | ¦2*f1_low –2*f2_high¦ | ¦2*f1_high –2*f2_low¦ | ¦2*f1_low +2*f2_low¦ | ¦2*f1_high +2*f2_high¦ |
| IMD frequency limit (MHz) | 2126 | 2276 | 5254 | 5404 |
| 5th order IMD products | ¦f1_low –4*f2_high¦ | ¦f1_high –4*f2_low¦ | ¦f2_low –4*f1_high¦ | ¦f2_high –4*f1_low¦ |
| IMD frequency limit (MHz) | 6613 | 6883 | 1193 | 1298 |
| 5th order IMD products | ¦f1_low +4*f2_low¦ | ¦f1_high +4*f2_high¦ | ¦f2_low+4*f1_low¦ | ¦f2_high +4*f1_high¦ |
| IMD frequency limit (MHz) | 8177 | 8447 | 4958 | 5063 |
| 5th order IMD products | ¦2*f1_low –3*f2_high¦ | ¦2*f1_high –3*f2_low¦ | ¦2*f2_low -3*f1_high¦ | ¦2*f2_high -3*f1_low¦ |
| IMD frequency limit (MHz) | 3976 | 4191 | 1339 | 1499 |
| 5th order IMD products | ¦2*f1_low +3*f2_low¦ | ¦2*f1_high +3*f2_high¦ | ¦2*f2_low+3*f1_low¦ | ¦2*f2_high +3*f1_high¦ |
| IMD frequency limit (MHz) | 7104 | 7319 | 6031 | 6191 |

Based on the table above, there is no IMD for the band combination of n13 and n25.

Table 6.28.2.2-2 lists the protected bands required for the 2UL bands CA configuration refer to DC_13_n2 in 38.101-3.

Table 6.28.2.2-2: Protected bands for the 2UL bands CA configuration

| UL NR CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n13-n25 | E-UTRA Band 4, 5,12,13,17, 26, 29, 41, 48, 66, 70, 71 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2,14, 25 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | E-UTRA Band 30 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 769 | - | 775 | -35 | 0.00625 | 5 |
|  | Frequency range | 799 | - | 805 | -35 | 0.00625 | 5 |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.5.3.1-2 are permitted for each assigned NR carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x 180kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 5: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 and Table 6.6.3.1A-1 from the edge of the channel bandwidth. |  |  |  |  |  |  |  |

6.28.2.3	REFSENS requirements

There is no MSD issue due to IMD for this combination.

## 6.29	CA_n13-n66

### 6.29.1		Common for 1 band UL and 2 bands UL CA

#### 6.29.1.1		Operating bands for CA

Table 6.29.1.1-1: CA band combination of band n13+n66

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n13 | 777 MHz | – | 787 MHz | 746 MHz | – | 756 MHz | FDD |
| n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |

#### 6.29.1.2		Channel bandwidths per operating band for CA

Table 6.29.1.2-1: Supported bandwidths per CA band combination of band n13+n66

| NR CA Configuration | UL Configuration | NR Band | Channel bandwidth (MHz) (NOTE 3) |  |  |  |  |  |  |  |  |  |  |  |  | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n13A-n66A | CA_n13A-n66A | n13 | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n66 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |

#### 6.29.1.3		Co-existence studies

Table 6.29.1.3-1/2 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_n13-n66.

Table 6.29.1.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | nth Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n13 | 777 | 787 | 746 | 756 | 1554 | 1574 | 2331 | 2361 |  |  |
| n66 | 1710 | 1780 | 2110 | 2200 | 3420 | 3560 | 5130 | 5340 |  |  |

Table 6.29.1.3-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | mth Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n13 | 777 | 787 | 746 | 756 | 1492 | 1512 | 2238 | 2268 |  |  |
| n66 | 1710 | 1780 | 2110 | 2200 | 4220 | 4400 | 6330 | 6600 |  |  |

Based on above table, there is no harmonic and harmonic mixing issue for the band combination of n13 and n66.

6.29.1.4		∆TIB and ∆RIB values

For CA_n13-n66 , the ΔTIB,c and ΔRIB,c values are given in the tables below. The same values are shown as what are specified in TR 38.101-3 DC_13_n66.

Table 6.29.1.4-1: ΔTIB,c

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n13-n66 | n13 | 0.3 |
|  | n66 | 0.3 |

Table 6.29.1.4-2: ΔRIB,c

| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n13-n66 | n13 | 0 |
|  | n66 | 0 |

6.29.1.5 REFSENS requirements

There is neither harmonic issue nor harmonics mixing issue for the band combination of n13 and n66.

6.29.1.6	OOB blocking exception requirements

There is no OOB blocking exception for this CA band combination.

6.29.2	Specific for 2 bands UL CA

6.29.2.1	Maximum output power for inter-band CA

Table 6.29.2.2-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n13A-n66A | 23 | +2/-32 |
| NOTE 2: 2 refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB |  |  |

6.29.2.2	UE co-existence studies

Table 6.29.2.2-1 lists band n13+band n66 2UL CA 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.29.2.2-1: Band n13 and Band n66 UL IMD products

| UE UL carriers | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| UL frequencies (MHz) | 777 | 787 | 1710 | 1780 |
| 2nd order IMD products | ¦f2_low – f1_high¦ | ¦f2_high – f1_low¦ | ¦f2_low + f1_low¦ | ¦f2_high + f1_high¦ |
| IMD frequency limit (MHz) | 923 | 1003 | 2487 | 2567 |
| 3rd order IMD products | ¦f2_low – 2*f1_high¦ | ¦f2_high – 2*f1_low¦ | ¦2*f2_low – f1_high¦ | ¦2*f2_high – f1_low¦ |
| IMD frequency limit (MHz) | 136 | 226 | 2633 | 2783 |
| 3rd order IMD products | ¦2*f1_low + f2_low¦ | ¦2*f1_high + f2_high¦ | ¦2*f2_low + f1_low¦ | ¦2*f2_high + f1_high¦ |
| IMD frequency limit (MHz) | 3264 | 3354 | 4197 | 4347 |
| 4th order IMD products | ¦3*f1_low – f2_high¦ | ¦3*f1_high – f2_low¦ | ¦3*f2_low – f1_high¦ | ¦3*f2_high –f1_low¦ |
| IMD frequency limit (MHz) | 551 | 651 | 4343 | 4563 |
| 4th order IMD products | ¦3*f1_low +f2_low¦ | ¦3*f1_high + f2_high¦ | ¦3*f2_low+f1_low¦ | ¦3*f2_high +f1_high¦ |
| IMD frequency limit (MHz) | 4041 | 4141 | 5907 | 6127 |
| 4th order IMD products | ¦2*f1_low –2*f2_high¦ | ¦2*f1_high –2*f2_low¦ | ¦2*f1_low +2*f2_low¦ | ¦2*f1_high +2*f2_high¦ |
| IMD frequency limit (MHz) | 1846 | 2006 | 4974 | 5134 |
| 5th order IMD products | ¦f1_low –4*f2_high¦ | ¦f1_high –4*f2_low¦ | ¦f2_low –4*f1_high¦ | ¦f2_high –4*f1_low¦ |
| IMD frequency limit (MHz) | 6053 | 6343 | 1328 | 1438 |
| 5th order IMD products | ¦f1_low +4*f2_low¦ | ¦f1_high +4*f2_high¦ | ¦f2_low+4*f1_low¦ | ¦f2_high +4*f1_high¦ |
| IMD frequency limit (MHz) | 7617 | 7907 | 4818 | 4928 |
| 5th order IMD products | ¦2*f1_low –3*f2_high¦ | ¦2*f1_high –3*f2_low¦ | ¦2*f2_low -3*f1_high¦ | ¦2*f2_high -3*f1_low¦ |
| IMD frequency limit (MHz) | 3556 | 3786 | 1059 | 1229 |
| 5th order IMD products | ¦2*f1_low +3*f2_low¦ | ¦2*f1_high +3*f2_high¦ | ¦2*f2_low+3*f1_low¦ | ¦2*f2_high +3*f1_high¦ |
| IMD frequency limit (MHz) | 6684 | 6914 | 5751 | 5921 |

Based on the table above, there is no IMD for the band combination of n13 and n66.

Table 6.29.2.2-2 lists the protected bands required for the 2UL bands CA configuration refer to DC_13_n66 in 38.101-3.

Table 6.29.2.2-2: Protected bands for the 2UL bands CA configuration

| UL NR CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n13-n66 | Bands 2, 4, 5, 7, 12, 13, 17, 25, 26, 27, 29, 41, 53, 66, 70, 71, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 14 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | E-UTRA Band 24, 30, 46, 48, NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 769 | - | 775 | -35 | 0.00625 | 5 |
|  | Frequency range | 799 | - | 805 | -35 | 0.00625 | 5 |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.5.3.1-2 are permitted for each assigned NR carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x 180kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 5: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 and Table 6.6.3.1A-1 from the edge of the channel bandwidth. |  |  |  |  |  |  |  |

6.29.2.3	REFSENS requirements

There is no MSD issue due to IMD for this combination.

6.30	CA_n8-n20

6.30.1	Common for 1 band UL and 2 bands UL CA

6.30.1.1 Operating bands for CA

Table 6.30.1.1-1: CA band combination CA_n8A-n20A

| NR CA Band Combination | NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n8-n20 | n8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n20 | 832 MHz | – | 862 MHz | 791 MHz | – | 821 MHz | FDD |

6.30.1.2	Channel bandwidths per operating band for CA

Table 6.30.1.2-1: Supported bandwidths per CA band combination CA_n8A-n20A

| NR CA configuration | Uplink CA configuration | NR Band | Channel bandwidth (MHz) (NOTE 3) |  |  |  |  |  |  |  |  |  |  |  |  | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n8A-n20A | - | n8 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n20 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |

6.30.1.3	UE Co-existence studies

Table 6.30.1.3-1/2 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_n8-n20.

Table 6.30.1.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n8 | 880 | 915 | 925 | 960 | 1760 | 1830 | 2640 | 2745 | 3520 | 3660 |
| n20 | 832 | 862 | 791 | 821 | 1664 | 1724 | 2496 | 2586 | 3328 | 3448 |

Based on above table, there is no harmonic interference.

Table 6.30.1.3-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n8 | 880 | 915 | 925 | 960 | 1850 | 1920 | 2775 | 2880 | 3700 | 3840 |
| n20 | 832 | 862 | 791 | 821 | 1582 | 1642 | 2373 | 2463 | 3146 | 3284 |

Based on above table, there is no harmonic mixing issue.

6.30.1.4	∆TIB and ∆RIB values

For CA_n8A-n20A, the TIB,c and RIB are given in the tables below reusing the values of DC_8_n20.

Table 6.30.1.4-1: ΔTIB,c

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n8-n20 | n8 | 0.4 |
|  | n20 | 0.4 |

Table 6.30.1.4-2: ΔRIB

| Inter-band CA Configuration | NR Band | ΔRIB [dB] |
| --- | --- | --- |
| CA_n8-n20 | n8 | 0 |
|  | n20 | 0 |

6.30.1.5	REFSENs requirements

There is no MSD exception requirement for CA_n8-n20.

6.30.1.6	OOB blocking exception requirements

There is no MSD exception requirement for CA_n8-n20.

6.31	n7-n77

6.31.1	Common for 1 band UL and 2 bands UL CA

6.31.1.1		Operating bands for CA

Table 6.31.1.1-1: CA band combination of band n7 and n77

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
| n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

6.31.1.2		Channel bandwidths per operating band for CA

Table 6.31.1.2-1: Supported bandwidths per CA band combination of band n7 and n77

| NR CA configuration | UL | NR Band | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | BCS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n7A-n77A | CA_n7A-n77A | n7 | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 |  |  |  |  |  | 0 |
|  |  | n77 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n7(2A)-n77A | CA_n7A-n77A | n7 | See CA_n7(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n77 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n7A-n77(2A) | CA_n7A-n77A | n7 | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 |  |  |  |  |  | 0 |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n7(2A)-n77(2A) | CA_n7A-n77A | n7 | See CA_n7(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.31.1.3		UE co-existence studies

Table 6.31.1.3-1 lists up to 7th harmonics for n7A-n77A which shows that there are no harmonics issues.

Table 6.31.1.3-1: Impact of UL/DL Harmonic

|  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  | 6th Harmonic |  | 7th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n7 | 2500 | 2570 | 5000 | 5140 | 7500 | 7710 | 10000 | 10280 | 12500 | 12850 | 15000 | 15420 | 17500 | 17990 |
| n77 | 3300 | 4200 | 6600 | 8400 | 9900 | 12600 | 13200 | 16800 | 16500 | 21000 | 19800 | 25200 | 23100 | 29400 |

Table 6.31.1.3-2 list harmonic mixing issue for the 2DL bands CA with 1 UL. As can be seen there are no harmonic mixing issues.

Table 6.31.1.3-2 Harmonic mixing for 2DLs/1UL

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n7 | 2500 | 2570 | 2620 | 2690 | 5240 | 5380 | 7860 | 8070 | 10480 | 10760 |
| n77 | 3300 | 4200 | 3300 | 4200 | 6600 | 8400 | 9900 | 12600 | 13200 | 16800 |

6.31.1.4		∆TIB and ∆RIB values

For CA_n7-n77, the TIB,c and RIB,c values are same as for DC_7_n77 and are given in the tables below.

Table 6.31.1.4-1: ΔTIB,c

| NR CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n7-n77 | n7 | 0.5 |
|  | n77 | 0.8 |

Table 6.31.1.4-2: ΔRIB,c

| NR CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n7-n77 | n7 | 0 |
|  | n77 | 0.5 |

6.31.1.5 		REFSENS requirements

There is no harmonic issue for the CA combination.

6.31.1.6 		OOB blocking exception requirements

No need to specify OOB exception requirement for CA_n7-n77.

Table 6.31.1.6-1: CA band combination with exceptions allowed

CA band combination

No exceptions

6.31.2		Specific for 2 bands UL CA

6.31.2.1	Maximum output power for inter-band CA

Table 6.31.2.1-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n7A-n77A | 23 | +2/-32 |
| NOTE 2: 2 refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB |  |  |

6.31.2.2		UE co-existence studies

Table 6.31.2.2-1 lists Band n7 +Band n77 2UL bands CA  2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.31.2.2-1: Band n7 and Band n77 UL harmonics and IMD products

| UE UL carriers | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| UL frequencies (MHz) | 2500 | 2570 | 3300 | 4200 |
| 2nd order IMD products | ¦fy_high – fx_low¦ | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1700 | 730 | 5800 | 6770 |
| 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 800 | 1840 | 4030 | 5900 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 8300 | 9340 | 9100 | 10970 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 3400 | 1460 | 11600 | 13540 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 3300 | 4410 | 7330 | 10100 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 10800 | 11910 | 12400 | 15170 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 14300 | 10630 | 6980 | 5800 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15700 | 19370 | 13300 | 14480 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 7600 | 4760 | 1110 | 900 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 14900 | 17740 | 14100 | 16110 |

Based on the table above:

- The 4th order IMD may fall into Rx frequencies of band n7.

- The 3rd and 4th order IMD may fall into Rx frequencies of band n77

Table 6.31.2.2-2 lists the protected bands required for the 2UL bands CA configuration.

Table 6.31.2.2-2: Protected bands for the 2UL bands CA configuration

| UL NR CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n7-n77 | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 11, 18, 19, 20, 21, 26, 27, 28, 31, 32, 33, 34, 40, 50, 51, 65, 66, 67, 68, 72, 74, 75, 76 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 2570 | - | 2575 | +1.6 | 5 | 4, 6, 7 |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 4, 6, 7 |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 4, 6 |
| NOTE 4: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth. NOTE 5: Void. NOTE 6: This requirement is applicable for any channel bandwidths within the range 1920 – 1980 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 1927.5 - 1929.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 1930 – 1938 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. NOTE 7: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. |  |  |  |  |  |  |  |

6.31.2.3		REFSENS requirements

Table 6.31.2.3-1 lists the MSD required. Same values as for DC_7A_n77A are used.

Table 6.31.2.3-1: MSD due to IMD issue

| Operating band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CA Configuration | Operating band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) |  |
| CA_n7A-n77A | n7 | 2540 | 5 | 25 | 2660 | 7.1 | IMD4 |
|  | n77 | 3870 | 10 | 50 | 3870 | N/A | N/A |

7	Both bands within FR2 Carrier Aggregation: Specific Band Combination Part

< Editor's note: This section is reserved for future used>

8	FR1+FR2 Carrier Aggregation: Specific Band Combination Part

8.1	CA_n41-n257

8.1.1	Common for 1 band UL and 2 bands UL CA

8.1.1.1	Operating bands for CA

Table 8.1.1.1-1: CA band combination of band n41+n257

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
| n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

8.1.1.2	Channel bandwidths per operating band for CA

Table 8.1.1.2-1: Supported bandwidths per CA band combination of band n41+n257

< Editor's note: see Table 5.5A.1-1 in TS38.101-3 for the table format>

| NR CA configuration | Uplink CA configuration | NR Band | Channel bandwidth (MHz) (NOTE 3) |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | 200 | 400 |  |
| CA_n41A-n257A | CA_n41A-n257A | n41 |  | 10 | 15 | 20 |  | 30 | 40 | 50 | 60 |  | 80 | 90 | 100 |  |  | 0 |
|  |  | n257 |  |  |  |  |  |  |  | 50 |  |  |  |  | 100 | 200 | 400 |  |
| CA_n41A-n257G | CA_n41A-n257A, CA_n41A-n257G | n41 |  | 10 | 15 | 20 |  | 30 | 40 | 50 | 60 |  | 80 | 90 | 100 |  |  | 0 |
|  |  | n257 | See CA_n257G Bandwidth Combination Set 0 in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n41A-n257H | CA_n41A-n257A, CA_n41A-n257G, CA_n41A-n257H | n41 |  | 10 | 15 | 20 |  | 30 | 40 | 50 | 60 |  | 80 | 90 | 100 |  |  | 0 |
|  |  | n257 | See CA_n257H Bandwidth Combination Set 0 in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n41A-n257I | CA_n41A-n257A, CA_n41A-n257G, CA_n41A-n257H, CA_n41A-n257I | n41 |  | 10 | 15 | 20 |  | 30 | 40 | 50 | 60 |  | 80 | 90 | 100 |  |  | 0 |
|  |  | n257 | See CA_n257I Bandwidth Combination Set 0 in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

8.1.1.3	∆TIB and ∆RIB values

Assuming separate antenna architecture between FR1 and FR2, TIB,c and RIB values for this CA configuration are given in the tables below.

Table 8.1.1.3-1: ΔTIB,c

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n41-n257 | n41 | 0 |
|  | n257 | 0 |

Table 8.1.1.4-2: ΔRIB,c

| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n41-n257 | n41 | 0 |
|  | n257 | 0 |

8.1.1.4	REFSENS requirements

No IMD issues are expected in this CA configuration as these are TDD bands. These TDD bands do not transmit and receive simultaneously

9	2 bands Dual Connectivity: Specific Band Combination Part

9.X	DC_nX-nY

< Editor's note: The texts for NR DC can only be added associated with the texts for the corresponding inter-band 2 bands UL CA above, which means contribution only adding  pure NR DC texts is not allowed.>

9.x.1	Operating bands for DC_nX-nY

Table 9.X.1-1: Inter-band NR DC operating bands within FR1

| NR DC Band | NR Band |
| --- | --- |
| DC_nX-nY | nX, nY |

Table 9.X.1-2: Inter-band NR DC operating bands for FR1+FR2

| NR DC Band | NR Band |
| --- | --- |
| DC_nX-nY | nX, nY |

9.x.2   Configurations for DC_nX-nY

Table 9.x.2-1: Inter-band NR DC configurations within FR1

| NR DC configuration | Uplink NR DC configuration |
| --- | --- |
| DC_nXA-nYA | DC_nXA-nYA |

Table 9.x.2-2: Inter-band NR DC configurations for FR1+FR2

| NR DC configuration | Uplink NR DC configuration |
| --- | --- |
| DC_nXA-nYA | DC_nXA-nYA |

9.x.3   Maximum output power for NR-DC

< Editor's note: Text will be added, the examples is given as follows. More information can be referred to Table 6.2B.1.3-1 in TS38.101-1. It shall be noted that no need to specify the UE power class for FR1+FR2 NR DC combination. For FR2-FR2 NR DC combination, it is FFS.>

Table 9.x.3-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_nXA-nYA | 23 | +x/-y1 |
| NOTE 1: refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB |  |  |

9.1	DC_n5-n48

9.1.1	Operating bands for DC_n5-n48

Table 9.1.1-1: Inter-band NR DC operating bands

| NR DC Band | NR Band |
| --- | --- |
| DC_n5-n48 | N5, n48 |

9.1.2   Configurations for DC_n5-n48

Table 9.1.2-1: Inter-band NR DC configurations

| NR DC configuration | Uplink NR DC configuration |
| --- | --- |
| DC_n5A-n48A | DC_n5A-n48A |
| DC_n5A-n48(2A) | DC_n5A-n48A |
| DC_n5A-n48C | DC_n5A-n48A |

9.2	DC_n41-n257

9.2.1	Operating bands for DC_n41-n257

Table 9.2.1-2: Inter-band NR DC operating bands for FR1+FR2

| NR DC Band | NR Band |
| --- | --- |
| DC_n41-n257 | n41, n257 |

9.2.2   Configurations for DC_n41-n257

Table 9.2.2-2: Inter-band NR DC configurations for FR1+FR2

| NR DC configuration | Uplink NR DC configuration |
| --- | --- |
| DC_n41A-n257A DC_n41A-n257G DC_n41A-n257H DC_n41A-n257I | DC_n41A-n257A DC_n41A-n257G DC_n41A-n257H DC_n41A-n257I |

Annex A:
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2020-08 | RAN4#96-e | R4-2011626 |  |  |  | TR skeleton | 0.0.1 |
| 2020-08 | RAN4#96-e | R4-2010792 |  |  |  | The following approved TP in RAN4 #96-e meeting are included: - R4-2011637 TP for TR 38.717-02-01: CA_n5-n25, Huawei, HiSilicon, Bell Mobility, Telus - R4-2011638 TP for TR 38.717-02-01: CA_n71-n78, Huawei, HiSilicon, Bell Mobility, Telus - R4-2011639 TP to TR 38.717-02-01: CA_n7-n66, Nokia, Bell Mobility - R4-2011640 TP to TR 38.717-02-00: CA_n25-n38, Nokia, Bell Mobility - R4-2010539 TP for CA 2DL2UL n1-n77 for TR 38.717-02-01, NTT DOCOMO INC. - R4-2010560 TP for CA 2DL2UL n77-n79 for TR 38.717-02-01, NTT DOCOMO INC. - R4-2011643 TP for CA 2DL2UL n78-n79 for TR 38.717-02-01, NTT DOCOMO INC. - R4-2011645 TP for TR 38.717-02-00 to include CA_n25A-n48A, CA_n25A-n48(2A), CA_n25A-n48C, Ericsson, T-Mobile US - R4-2011646 TP for TR 38.717-02-01: CA_n28A-n79A, Huawei, HiSilicon,CBN - R4-2011628 TP for TR 38.717-02-01 CA_n3-n77(2A) _UL_n77(2A),Samsung, KDDI - R4-2011629 TP for TR 38.717-02-01 CA_n3-n78(2A) _UL_n78(2A), Samsung, KDDI - R4-2011630 TP for TR 38.717-02-01 CA_n28-n77(2A) _UL_n77(2A), Samsung, KDDI - R4-2011631 TP for TR 38.717-02-01 CA_n28-n78(2A) _UL_n78(2A), Samsung, KDDI | 0.1.0 |
| 2020-11 | RAN4#97-e | R4-2015184 |  |  |  | Including the approved TP in RAN4 #97-e meeting. - R4-2016680 TP for TR 38.717-02-01 CA_n41-n78 - R4-2014131 TP for TR 38.717-02-01 CA_n2-n66 - R4-2016682 TP for TR 38.717-02-01: CA_n41-n77 - R4-2016683 TP for TR 38.717-02-01: CA_n71A-n77A - R4-2016684 TP for TR 37.717-02-01: CA_n5-n48 - R4-2016686 TP to TR 38.717-02-01: CA_n5-n25 - R4-2016687 TP to TR 38.717-02-01: CA_n25-n77 - R4-2015082 TP to TR 38.717-02-01 to correct CA_n7(2A)-n66 BCS - R4-2016689 TP for TR 38.717-02-01: to add UL configuration for CA_n78A-n79A and CA_n78(2A)-n79A_BCS0 - R4-2016690 TP for TR 38.717-02-01: CA_n8A-n28A - R4-2016691 TP for TR 38.717-02-01: CA_n3A-n7A | V0.2.0 |
| 2021-02 | RAN4#98-e | R4-2102304 |  |  |  | Including the approved TP in RAN4 #98-e meeting. - R4-2102210 TP for TR38.717-02-01_Removal of the sub-clauses under clause 7 ZTE Corporation - R4-2103037 TP for TR 38.717-02-01: CA_n3-n18 Samsung, KDDI - R4-2103038 TP for TR 38.717-02-01: CA_n18-n41 Samsung, KDDI - R4-2103039 TP for TR 38.717-02-01: DC_n3-n41 Samsung, KDDI - R4-2100973 TP for TR 38.717-02-01: CA_n25-n29 Samsung, TELUS, Bell mobility - R4-2103042 TP for TR 38.717-02-01: CA_n13-n25 Samsung, TELUS, Bell mobility - R4-2103043 TP for TR 38.717-02-01: CA_n13-n66 Samsung, TELUS, Bell mobility - R4-2101599 TP for TR 38.717-02-01: CA_n8A-n20A Huawei, HiSilicon - R4-2103047 TP for TR 38.717-02-01 to include CA_n7-n77 Ericsson, Bell Mobility - R4-2103049 TP for TR 38.717-02-01: CA_n41-n257 Samsung, KDDI | V0.3.0 |