3GPP TSG-WG RAN4 Meeting #98-e	R4-2102453

# Electronic meeting, 25th January – 5th February, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.104 | CR | 4929 | rev | - | Current version: | 11.17.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to 36.104: Correction to Band 24 requirements (Rel-11) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_B24_mod-Core |  |  |  |  |  | Date: |  |  | 2021-02-01 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-11 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | There are two regulatory updates related to BS operation in Band 24 which need to be reflected in 36.104: - Regulations limits the downlink power to 9.8 dBW/MHz and limits transmission between 1526 – 1536 MHz - OOBE emission limits have been modified. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Band 24 downlink transmission limitation is added in Table 5.5-1, FCC Order DA has been updated in Clause 6.6.3. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Band 24 requirements would be not aligned with the latest regulation updates. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5, 6.6.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 36.141 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Isolated impact analysis: since there is no Band 24 product, proposed changes are from Rel’10. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 5.5	Operating bands

E-UTRA is designed to operate in the operating bands defined in Table 5.5-1.

Table 5.5-1 E-UTRA frequency bands

| E-UTRA Operating Band |  |  | Uplink (UL) operating band BS receive UE transmit |  |  |  | Downlink (DL) operating band BS transmit UE receive |  |  |  |  |  |  | Duplex Mode |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | FUL_low – FUL_high |  |  |  | FDL_low – FDL_high |  |  |  |  |  |  |  |  |
| 1 |  |  | 1920 MHz | – | 1980 MHz |  |  | 2110 MHz |  | – |  | 2170 MHz |  | FDD |  |
| 2 |  |  | 1850 MHz | – | 1910 MHz |  |  | 1930 MHz |  | – |  | 1990 MHz |  | FDD |  |
| 3 |  |  | 1710 MHz | – | 1785 MHz |  |  | 1805 MHz |  | – |  | 1880 MHz |  | FDD |  |
| 4 |  |  | 1710 MHz | – | 1755 MHz |  |  | 2110 MHz |  | – |  | 2155 MHz |  | FDD |  |
| 5 |  |  | 824 MHz | – | 849 MHz |  |  | 869 MHz |  | – |  | 894MHz |  | FDD |  |
| 61 |  |  | 830 MHz | – | 840 MHz |  |  | 875 MHz |  | – |  | 885 MHz |  | FDD |  |
| 7 |  |  | 2500 MHz | – | 2570 MHz |  |  | 2620 MHz |  | – |  | 2690 MHz |  | FDD |  |
| 8 |  |  | 880 MHz | – | 915 MHz |  |  | 925 MHz |  | – |  | 960 MHz |  | FDD |  |
| 9 |  |  | 1749.9 MHz | – | 1784.9 MHz |  |  | 1844.9 MHz |  | – |  | 1879.9 MHz |  | FDD |  |
| 10 |  |  | 1710 MHz | – | 1770 MHz |  |  | 2110 MHz |  | – |  | 2170 MHz |  | FDD |  |
| 11 |  |  | 1427.9 MHz | – | 1447.9 MHz |  |  | 1475.9 MHz |  | – |  | 1495.9 MHz |  | FDD |  |
| 12 |  |  | 699 MHz | – | 716 MHz |  |  | 729 MHz |  | – |  | 746 MHz |  | FDD |  |
| 13 |  |  | 777 MHz | – | 787 MHz |  |  | 746 MHz |  | – |  | 756 MHz |  | FDD |  |
| 14 |  |  | 788 MHz | – | 798 MHz |  |  | 758 MHz |  | – |  | 768 MHz |  | FDD |  |
| 15 |  |  | Reserved |  |  |  |  | Reserved |  |  |  |  |  | FDD |  |
| 16 |  |  | Reserved |  |  |  |  | Reserved |  |  |  |  |  | FDD |  |
| 17 |  |  | 704 MHz | – | 716 MHz |  |  | 734 MHz |  | – |  | 746 MHz |  | FDD |  |
| 18 |  |  | 815 MHz | – | 830 MHz |  |  | 860 MHz |  | – |  | 875 MHz |  | FDD |  |
| 19 |  |  | 830 MHz | – | 845 MHz |  |  | 875 MHz |  | – |  | 890 MHz |  | FDD |  |
| 20 |  |  | 832 MHz | – | 862 MHz |  |  | 791 MHz |  | – |  | 821 MHz |  | FDD |  |
| 21 |  |  | 1447.9 MHz | – | 1462.9 MHz |  |  | 1495.9 MHz |  | – |  | 1510.9 MHz |  | FDD |  |
| 22 |  |  | 3410 MHz | – | 3490 MHz |  |  | 3510 MHz |  | – |  | 3590 MHz |  | FDD |  |
| 23 |  |  | 2000 MHz | – | 2020 MHz |  |  | 2180 MHz |  | – |  | 2200 MHz |  | FDD |  |
| 249 |  |  | 1626.5 MHz | – | 1660.5 MHz |  |  | 1525 MHz |  | – |  | 1559 MHz |  | FDD |  |
| 25 |  |  | 1850 MHz | – | 1915 MHz |  |  | 1930 MHz |  | – |  | 1995 MHz |  | FDD |  |
| 26 |  |  | 814 MHz | – | 849 MHz |  |  | 859 MHz |  | – |  | 894 MHz |  | FDD |  |
| 27 |  |  | 807 MHz | – | 824 MHz |  |  | 852 MHz |  | – |  | 869 MHz |  | FDD |  |
| 28 |  |  | 703 MHz | – | 748 MHz |  |  | 758 MHz |  | – |  | 803 MHz |  | FDD |  |
| 29 |  | N/A |  |  |  |  |  | 717 MHz |  | – |  | 728 MHz |  |  | FDD2 |  |
| ... |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 33 |  |  | 1900 MHz | – | 1920 MHz |  |  | 1900 MHz |  | – |  | 1920 MHz |  | TDD |  |
| 34 |  |  | 2010 MHz | – | 2025 MHz |  |  | 2010 MHz |  | – |  | 2025 MHz |  | TDD |  |
| 35 |  |  | 1850 MHz | – | 1910 MHz |  |  | 1850 MHz |  | – |  | 1910 MHz |  | TDD |  |
| 36 |  |  | 1930 MHz | – | 1990 MHz |  |  | 1930 MHz |  | – |  | 1990 MHz |  | TDD |  |
| 37 |  |  | 1910 MHz | – | 1930 MHz |  |  | 1910 MHz |  | – |  | 1930 MHz |  | TDD |  |
| 38 |  |  | 2570 MHz | – | 2620 MHz |  |  | 2570 MHz |  | – |  | 2620 MHz |  | TDD |  |
| 39 |  |  | 1880 MHz | – | 1920 MHz |  |  | 1880 MHz |  | – |  | 1920 MHz |  | TDD |  |
| 40 |  |  | 2300 MHz | – | 2400 MHz |  |  | 2300 MHz |  | – |  | 2400 MHz |  | TDD |  |
| 41 |  |  | 2496 MHz | – | 2690 MHz |  |  | 2496 MHz |  | – |  | 2690 MHz |  | TDD |  |
| 42 |  |  | 3400 MHz | – | 3600 MHz |  |  | 3400 MHz |  | – |  | 3600 MHz |  | TDD |  |
| 43 |  |  | 3600 MHz | – | 3800 MHz |  |  | 3600 MHz |  | – |  | 3800 MHz |  | TDD |  |
| 44 |  |  | 703 MHz | – | 803 MHz |  |  | 703 MHz |  | – |  | 803 MHz |  | TDD |  |
| Note 1: Band 6 is not applicable. Note 2: Restricted to E-UTRA operation when carrier aggregation is configured. The downlink operating band is paired with the uplink operating band (external) of the carrier aggregation configuration that is supporting the configured Pcell. NOTE 3: Void NOTE 4: Void NOTE 5: Void NOTE 6: Void NOTE 7: Void NOTE 8: Void NOTE 9: DL operation is restricted to 1526-1536 MHz frequency range. UL operation is restricted to 1627.5 – 1637.5 MHz and 1646.5 – 1656.5 MHz per FCC Order DA 20-48. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

---------------------------------------------------------- NEXT SECTION ---------------------------------------------------------

#### 6.6.3.3	Additional requirements

These requirements may be applied for the protection of other systems operating inside or near each supported E-UTRA BS downlink operating band. The limits may apply as an optional protection of such systems that are deployed in the same geographical area as the E-UTRA BS, or they may be set by local or regional regulation as a mandatory requirement for an E-UTRA operating band. It is in some cases not stated in the present document whether a requirement is mandatory or under what exact circumstances that a limit applies, since this is set by local or regional regulation. An overview of regional requirements in the present document is given in subclause 4.3.

In certain regions the following requirement may apply. For E-UTRA BS operating in Bands 5, 26, 27 or 28, emissions shall not exceed the maximum levels specified in Tables 6.6.3.3-1.

Table 6.6.3.3-1: Additional operating band unwanted emission limits for E-UTRA bands <1GHz

| Channel bandwidth | Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Minimum requirement | Measurement bandwidth (Note 5) |
| --- | --- | --- | --- | --- |
| 1.4 MHz | 0 MHz f < 1 MHz | 0.005 MHz f_offset < 0.995 MHz | -14 dBm | 10 kHz |
| 3 MHz | 0 MHz f < 1 MHz | 0.015 MHz f_offset < 0.985 MHz | -13 dBm | 30 kHz |
| 5 MHz | 0 MHz f < 1 MHz | 0.015 MHz f_offset < 0.985 MHz | -15 dBm | 30 kHz |
| 10 MHz | 0 MHz f < 1 MHz | 0.05 MHz f_offset < 0.95 MHz | -13 dBm | 100 kHz |
| 15 MHz | 0 MHz f < 1 MHz | 0.05 MHz f_offset < 0.95 MHz | -13 dBm | 100 kHz |
| 20 MHz | 0 MHz f < 1 MHz | 0.05 MHz f_offset < 0.95 MHz | -13 dBm | 100 kHz |
| All | 1 MHz f < fmax | 1.05 MHz f_offset < f_offsetmax | -13 dBm | 100 kHz |

In certain regions the following requirement may apply. For E-UTRA BS operating in Bands 2, 4, 10, 23, 25, 35, 36, 41, emissions shall not exceed the maximum levels specified in Table 6.6.3.3-2.

Table 6.6.3.3-2: Additional operating band unwanted emission limits for E-UTRA bands>1GHz

| Channel bandwidth | Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Minimum requirement | Measurement bandwidth (Note 5) |
| --- | --- | --- | --- | --- |
| 1.4 MHz | 0 MHz f < 1 MHz | 0.005 MHz f_offset < 0.995 MHz | -14 dBm | 10 kHz |
| 3 MHz | 0 MHz f < 1 MHz | 0.015 MHz f_offset < 0.985 MHz | -13 dBm | 30 kHz |
| 5 MHz | 0 MHz f < 1 MHz | 0.015 MHz f_offset < 0.985 MHz | -15 dBm | 30 kHz |
| 10 MHz | 0 MHz f < 1 MHz | 0.05 MHz f_offset < 0.95 MHz | -13 dBm | 100 kHz |
| 15 MHz | 0 MHz f < 1 MHz | 0.05 MHz f_offset < 0.95 MHz | -15 dBm | 100 kHz |
| 20 MHz | 0 MHz f < 1 MHz | 0.05 MHz f_offset < 0.95 MHz | -16 dBm | 100 kHz |
| All | 1 MHz f < fmax | 1.5 MHz f_offset < f_offsetmax | -13 dBm | 1 MHz |

In certain regions the following requirement may apply. For E-UTRA BS operating in Bands 12, 13, 14, 17, 29 emissions shall not exceed the maximum levels specified in Table 6.6.3.3-3.

Table 6.6.3.3-3: Additional operating band unwanted emission limits for E-UTRA (bands 12, 13, 14, 17 and 29)

| Channel bandwidth | Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Minimum requirement | Measurement bandwidth (Note 5) |
| --- | --- | --- | --- | --- |
| All | 0 MHz f < 100 kHz | 0.015 MHz f_offset < 0.085 MHz | -13 dBm | 30 kHz |
| All | 100 kHz f < fmax | 150 kHz f_offset < f_offsetmax | -13 dBm | 100 kHz |

In certain regions, the following requirements may apply to an E-UTRA TDD BS operating in the same geographic area and in the same operating band as another E-UTRA TDD system without synchronisation. For this case the emissions shall not exceed -52 dBm/MHz in each supported downlink operating band except in:

-	The frequency range from 10 MHz below the lower channel edge to the frequency 10 MHz above the upper channel edge of each supported band.

In certain regions the following requirement may apply for protection of DTT. For E-UTRA BS operating in Band 20, the level of emissions in the band 470-790 MHz, measured in an 8MHz filter bandwidth on centre frequencies Ffilter according to Table 6.6.3.3-4, shall not exceed the  maximum emission level PEM,N declared by the manufacturer.  This requirement applies in the frequency range 470-790 MHz even though part of the range falls in the spurious domain.

Table 6.6.3.3-4: Declared emissions levels for protection of DTT

| Filter centre frequency, Ffilter | Measurement bandwidth | Declared emission level [dBm] |
| --- | --- | --- |
| Ffilter = 8*N + 306 (MHz); 21 ≤ N ≤ 60 | 8 MHz | PEM,N |

Note: 	The regional requirement is defined in terms of EIRP (effective isotropic radiated power), which is dependent on both the BS emissions at the antenna connector and the deployment (including antenna gain and feeder loss). The requirement defined above provides the characteristics of the basestation needed to verify compliance with the regional requirement. Compliance with the regional requirement can be determined using the method outlined in Annex G.

In certain regions the following requirement may apply for the protection of systems operating in frequency bands adjacent to band 1 as defined in clause 5.5, in geographic areas in which both an adjacent band service E-UTRA are deployed.

The power of any spurious emission shall not exceed:

Table 6.6.3.3-5: Emissions limits for protection of adjacent band services

| Operating Band | Frequency range | Maximum Level | Measurement Bandwidth |
| --- | --- | --- | --- |
| 1 | 2100-2105 MHz | -30 + 3.4 (f - 2100 MHz) dBm | 1 MHz |
|  | 2175-2180 MHz | -30 + 3.4 (2180 MHz - f) dBm | 1 MHz |

In regions where FCC regulation applies, requirements for protection of GPS according to FCC Order DA 20-48 applies for operation in Band 24. The following normative requirement covers the base station, to be used together with other information about the site installation to verify compliance with the requirement in FCC Order DA 20-48. The requirement applies to BS operating in Band 24 to ensure that appropriate interference protection is provided to the 1541 – 1650 MHz band. This requirement applies to the frequency range 1541-1650 MHz, even though part of this range falls within the spurious domain.

The level of emissions in the 1541 – 1650 MHz band, measured in measurement bandwidth according to Table 6.6.3.3-6 shall not exceed the maximum emission levels PEM,B24,a, PEM,B24,b, PEM,B24,c, PEM,B24,d, PEM,B24,e and PEM,B24,f declared by the manufacturer.

Table 6.6.3.3-6: Declared emissions levels for protection of the 1541-1650 MHz band

| Operating Band | Frequency range | Declared emission level (dBW) (Measurement bandwidth = 1 MHz) | Declared emission level (dBW) of discrete emissions of less than 700 Hz bandwidth (Measurement bandwidth = 1 kHz) | Declared emission level (dBW) of discrete emissions of less than 2 kHz bandwidth (Measurement bandwidth = 1 kHz) |
| --- | --- | --- | --- | --- |
| 24 | 1541 - 1559 MHz | PEM,B24,a |  | PEM,B24,f |
|  | 1559 - 1610 MHz | PEM,B24,b | PEM,B24,d |  |
|  | 1610 - 1650 MHz | PEM,B24,c | PEM,B24,e |  |

Note:	The regional requirements in FCC Order DA 20-48 are defined in terms of EIRP (effective isotropic radiated power), which is dependent on both the BS emissions at the antenna connector and the deployment (including antenna gain and feeder loss). The EIRP level is calculated using: PEIRP = PE + Gant where PE denotes the BS unwanted emission level at the antenna connector, Gant equals the BS antenna gain minus feeder loss. The requirement defined above provides the characteristics of the base station needed to verify compliance with the regional requirement.