3GPP TSG-WG RAN4 Meeting #98-e	R4-2102478

# Electronic meeting, 25th January – 5th February, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.141 | CR | 0967 | rev | - | Current version: | 12.13.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to 37.141: Correction to Band 24 requirements (Rel-12) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_B24_mod-Perf |  |  |  |  |  | Date: |  |  | 2021-02-01 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-12 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | There are two regulatory updates related to BS operation in Band 24 which need to be reflected in 37.141: - Regulations limits the downlink power to 9.8 dBW/MHz and limits transmission between 1526 – 1536 MHz - OOBE emission limits have been modified. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Band 24 downlink transmission limitation is added in Table 4.4-1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Band 24 requirements would be not aligned with the latest regulation updates. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Isolated impact analysis: since there is no Band 24 product, proposed changes are from Rel’10. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 4.4	Operating bands and band categories

MSR requirements are applicable for band definitions and band numbering as defined in the specifications TS 45.005 [6], TS25.104 [3], TS 25.105 [4] and TS 36.104 [5]. For the purpose of defining the BS requirements, the operating bands are divided into three band categories as follows:

-	Band Category 1 (BC1): Bands for E-UTRA FDD and UTRA FDD operation

-	Band Category 2 (BC2): Bands for E-UTRA FDD, UTRA FDD and GSM/EDGE operation

-	Band Category 3 (BC3): Bands for E-UTRA TDD and UTRA TDD operation

NOTE:	For UTRA TDD, requirements in the present document cover the 1.28 Mcps UTRA TDD option.

The paired and unpaired bands for the three Band Categories are shown in Table 4.4-1 and 4.4-2, together with the corresponding E-UTRA, UTRA and GSM/EDGE band designations. In the present specification, the operating band of an MSR Base Stations is designated using the E-UTRA band number according to the tables.

# Table 4.4-1: Paired bands in E-UTRA, UTRA and GSM/EDGE.

| MSR and E-UTRA Band number | UTRA Band number | GSM/EDGE Band designation | Uplink (UL) BS receive UE transmit |  |  |  |  | Downlink (DL) BS transmit UE receive |  |  |  |  |  | Band category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | I | - | 1920 MHz | – |  | 1980 MHz |  | 2110 MHz |  | – |  | 2170 MHz |  | 1 |
| 2 | II | PCS 1900 | 1850 MHz | – |  | 1910 MHz |  | 1930 MHz |  | – |  | 1990 MHz |  | 2 |
| 3 | III | DCS 1800 | 1710 MHz | – |  | 1785 MHz |  | 1805 MHz |  | – |  | 1880 MHz |  | 2 |
| 4 | IV | - | 1710 MHz | – |  | 1755 MHz |  | 2110 MHz |  | – |  | 2155 MHz |  | 1 |
| 5 | V | GSM 850 | 824 MHz | – |  | 849 MHz |  | 869 MHz |  | – |  | 894 MHz |  | 2 |
| 6 (NOTE 1) | VI | - | 830 MHz | – |  | 840 MHz |  | 875 MHz |  | – |  | 885 MHz |  | 1 (NOTE 1) |
| 7 | VII | - | 2500 MHz | – |  | 2570 MHz |  | 2620 MHz |  | – |  | 2690 MHz |  | 1 |
| 8 | VIII | E-GSM | 880 MHz | – |  | 915 MHz |  | 925 MHz |  | – |  | 960 MHz |  | 2 |
| 9 | IX | - | 1749.9 MHz | – |  | 1784.9 MHz |  | 1844.9 MHz |  | – |  | 1879.9 MHz |  | 1 |
| 10 | X | - | 1710 MHz | – |  | 1770 MHz |  | 2110 MHz |  | – |  | 2170 MHz |  | 1 |
| 11 | XI | - | 1427.9 MHz | – |  | 1447.9 MHz |  | 1475.9 MHz |  | – |  | 1495.9 MHz |  | 1 |
| 12 | XII | - | 699 MHz | – |  | 716 MHz |  | 729 MHz |  | – |  | 746 MHz |  | 1 |
| 13 | XIII | - | 777 MHz | – |  | 787 MHz |  | 746 MHz |  | – |  | 756 MHz |  | 1 |
| 14 | XIV | - | 788 MHz | – |  | 798 MHz |  | 758 MHz |  | – |  | 768 MHz |  | 1 |
| 15 | XV | - | Reserved |  |  |  |  | Reserved |  |  |  |  |  |  |
| 16 | XVI | - | Reserved |  |  |  |  | Reserved |  |  |  |  |  |  |
| 17 | - | - | 704 MHz | – |  | 716 MHz |  | 734 MHz |  | – |  | 746 MHz |  | 1 (NOTE 2) |
| 18 | - | - | 815 MHz | – |  | 830 MHz |  | 860 MHz |  | – |  | 875 MHz |  | 1 (NOTE 2) |
| 19 | XIX | - | 830 MHz | – |  | 845 MHz |  | 875 MHz |  | – |  | 890 MHz |  | 1 |
| 20 | XX | - | 832 MHz | – |  | 862 MHz |  | 791 MHz |  | – |  | 821 MHz |  | 1 |
| 21 | XXI | - | 1447.9 MHz | – |  | 1462.9 MHz |  | 1495.9 MHz |  | – |  | 1510.9 MHz |  | 1 |
| 22 | XXII | - | 3410 MHz | – |  | 3490 MHz |  | 3510 MHz |  | – |  | 3590 MHz |  | 1 |
| 23 | - | - | 2000 MHz | – |  | 2020 MHz |  | 2180 MHz |  | – |  | 2200 MHz |  | 1 (NOTE 2) |
| 24 (NOTE 10) | - | - | 1626.5 MHz | – |  | 1660.5 MHz |  | 1525 MHz |  | – |  | 1559 MHz |  | 1 (NOTE 2) |
| 25 | XXV | - | 1850 MHz | – |  | 1915 MHz |  | 1930 MHz |  | – |  | 1995 MHz |  | 1 |
| 26 | XXVI | - | 814 MHz | – |  | 849 MHz |  | 859 MHz |  | – |  | 894 MHz |  | 1 |
| 27 | - | - | 807 MHz | – |  | 824 MHz |  | 852 MHz |  | – |  | 869 MHz |  | 1 (NOTE 2) |
| 28 | - | - | 703 MHz | – |  | 748 MHz |  | 758 MHz |  | – |  | 803 MHz |  | 1 (NOTE 2) |
| 29 | - | - | N/A |  |  |  |  | 717 MHz |  | – |  | 728 MHz |  | 1 (NOTE 2, NOTE 3) |
| 30 | - | - | 2305 MHz | – |  | 2315 MHz |  | 2350 MHz |  | – |  | 2360 MHz |  | 1 (NOTE 2) |
| 31 | - | - | 452.5 MHz | – |  | 457.5 MHz |  | 462.5 MHz |  | – |  | 467.5 MHz |  | 1 (NOTE 2) |
| 32 (NOTE 3) | XXXII (NOTE 4) | - |  |  | N/A |  |  |  | 1452 MHz |  | – |  | 1496 MHz | 1 |
| NOTE 1: The band is for UTRA only. NOTE 2: The band is for E-UTRA only. NOTE 3: Resticted to E-UTRA operation when carrier aggregation is configured. The downlink operating band is paired with the uplink operating band (external) of the carrier aggregation configuration that is supporting the configured Pcell. NOTE 4: Restricted to UTRA operation when dual band is configured (e.g., DB-DC-HSDPA or dual band 4C-HSDPA). The down link frequenc(ies) of this band are paired with the uplink frequenc(ies) of the other FDD band (external) of the dual band configuration. NOTE 5: Void NOTE 6: Void NOTE 7: Void NOTE 8: Void NOTE 9: Void NOTE 10: DL operation is restricted to 1526-1536 MHz frequency range. UL operation is restricted to 1627.5 – 1637.5 MHz and 1646.5 – 1656.5 MHz per FCC Order DA 20-48. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |