3GPP TSG-RAN4 Meeting #98-e	R4-2102792

# Electronic Meeting, Jan 25 – February 5, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 1773 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on CGI reading requirements 38.133 R17 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, MediaTek |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RRM_Enh-Core |  |  |  |  |  | Date: |  |  | 2020-12-30 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - In R4-2017363 it is agreed to add 30ms LTE power up time for CGI reading delay of LTE target cell. - For interruption due to SIB1 decoding, the number of interruptions should be the total SIB decoding delay (which is based on acutal SIB1 scheduling from the NW) divided by default SIB periodicity. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - For NR-LTE inter-RAT CGI reading, add the reporting delay requirements and include the 30ms LTE power up time in this reporting delay. - Update the SIB1 interruption requirements such that the number of interruptions is the total SIB decoding delay divided by default SIB periodicity. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | CGI reading requirements are incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.4.7, 8.2.1.2.16, 8.2.2.2.14, 8.2.3.2.14, 8.2.4.2.11 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

9.4.7	NR – E-UTRAN measurements with autonomous gaps

9.4.7.1	CGI identification of an E-UTRA cell with autonomous gaps

The requirements in this clause apply when the UE is configured with standalone NR, NE-DC or NR-DC. The UE shall identify and report the CGI when requested by an NR PCell for the purpose ‘reportCGI’. The UE may make autonomous gaps in downlink reception and uplink transmission for receiving MIB and SIB1 message according to clause 5.5.3.1 in TS 38.331 [2]. If autonomous gaps are used for measurement with the purpose of ‘reportCGI’, regardless of whether SCell(s) are configured or not, the UE shall be able to identify a new CGI of E-UTRA cell within Tidentify_CGI, E-UTAN= 150 ms. This is the maximum allowed time for the UE to identify a new CGI of an E-UTRA cell, provided that the E-UTRA cell has been already identified by the UE.

A cell shall be considered identifiable following conditions are fulfilled:

-	RSRP related side conditions given in Clause 9.1 in [15] are fulfilled for a corresponding Band,

-	SCH_RP and SCH Ês/Iot according to Annex B.2.2 in [15] for a corresponding Band

The MIB of an E-UTRA cell whose CGI is identified shall be considered decodable by the UE provided the PBCH demodulation requirements are met according to [25].

The requirement for identifying a new CGI of an E-UTRA cell within Tidentify_CGI, E-UTRAN is applicable when no DRX is used as well as when any of the DRX cycles specified in TS 38.331 [2] is used.

9.4.7.2	CGI reporting delay

The E-UTRA CGI reporting delay is defined as the time between a command that will trigger an E-UTRA CGI report and the point when the UE starts to transmit the measurement report over the air interface. This requirement assumes that the measurement report is not delayed by other RRC signalling on the DCCH. This measurement reporting delay excludes a delay uncertainty of 2 x TTIDCCH resulting when inserting the measurement report to the TTI of the uplink DCCH. This measurement reporting delay excludes any delay caused by lack of UL resources for UE to send the measurement report.

The CGI reporting delay shall be less than Tidentify_CGI, E-UTAN plus RRC procedure delay defined in clause 12 in TS 38.331 [2], and an additional 30ms margin.

<End of Change 1>

<Start of Change 2>

8.2.1.2.16	 Interruptions when identifying CGI of an NR cell with autonomous gaps

When a UE is identifying CGI of an NR cell with autonomous gaps, the UE is allowed interruptions on PSCell or any activated SCell:

-	with up to K1 interruptions with interrupted slots up to interruption length X1 specified in Table 8.2.1.2.16-1 for each interruption during MIB decoding time period TMIB (ms) specified in clause 9.11.

-	with up to L1 interruptions with interrupted slots up to interruption length Y1 specified in Table 8.2.1.2.16-1 during SIB1 decoding time period TSIB1 (ms) specified in clause 9.11 for SSB and CORESET for RMSI scheduling multiplexing patterns 1.

-	with up to L2 interruptions with interrupted slots up to interruption length Y2 specified in Table 8.2.1.2.16-1 during SIB1 decoding time period TSIB1 (ms) specified in clause 9.11 for SSB and CORESET for RMSI scheduling multiplexing patterns 2 and 3.

Where:

-	K1 = 6 for the target cell carrier frequency on FR1 and K1 = 25 for the target cell carrier frequency on FR2, and

-	L1 = TSIB1/20, and

-	L2 = TSIB1/TSMTC, where TSMTC is the periodicity of the SMTC occasion configured for the target cell carrier.

Table 8.2.1.2.16-1: Interruption length X1, Y1 and Y2 during measurements with autonomous gaps

|  | NR Slot length (ms) of victim cell | Interruption length X1 (slots) | Interruption length Y1 (slots) | Interruption length Y2 (slots) |
| --- | --- | --- | --- | --- |
| 0 | 1 | 6 | 7 | 6 |
| 1 | 0.5 | 12 | 13 | 10 |
| 2 | 0.25 | 24 | 25 | 19 |
| 3 | 0.125 | 48 | 49 | 37 |

<End of Change 2>

<Start of Change 3>

8.2.2.2.14	 Interruptions when identifying CGI of an NR cell with autonomous gaps

When a UE is identifying CGI of an NR cell with autonomous gaps, the UE is allowed interruptions on PCell or any activated SCell:

-	with up to K1 interruptions with interrupted slots up to interruption length X1 specified in Table 8.2.2.2.14-1 for each interruption during MIB decoding time period TMIB (ms) specified in clause 9.11.

-	with up to L1 interruptions with interrupted slots up to interruption length Y1 specified in Table 8.2.2.2.14-1 during SIB1 decoding time period TSIB1 (ms) specified in clause 9.11 for SSB and CORESET for RMSI scheduling multiplexing patterns 1.

-	with up to L2 interruptions with interrupted slots up to interruption length Y2 specified in Table 8.2.2.2.14-1 during SIB1 decoding time period TSIB1 (ms) specified in clause 9.11 for SSB and CORESET for RMSI scheduling multiplexing patterns 2 and 3.

Where:

-	K1 = 6 for the target cell carrier frequency on FR1 and K1 = 25 for the target cell carrier frequency on FR2, and

-	L1 = TSIB1/20, and

-	L2 = TSIB1/TSMTC, where TSMTC is the periodicity of the SMTC occasion configured for the target cell carrier.

Table 8.2.2.2.14-1: Interruption length X1, Y1 and Y2 during measurements with autonomous gaps

|  | NR Slot length (ms) of victim cell | Interruption length X1 (slots) | Interruption length Y1 (slots) | Interruption length Y2 (slots) |
| --- | --- | --- | --- | --- |
| 0 | 1 | 6 | 7 | 6 |
| 1 | 0.5 | 12 | 13 | 10 |
| 2 | 0.25 | 24 | 25 | 19 |
| 3 | 0.125 | 48 | 49 | 37 |

<End of Change 3>

<Start of Change 4>

8.2.3.2.14	 Interruptions when identifying CGI of an NR cell with autonomous gaps

When a UE is identifying CGI of an NR cell with autonomous gaps, the UE is allowed interruptions on PCell or any activated SCell:

-	with up to K1 interruptions with interrupted slots up to interruption length X1 specified in Table 8.2.3.2.14-1 for each interruption during MIB decoding time period TMIB (ms) specified in clause 9.11.

-	with up to L1 interruptions with interrupted slots up to interruption length Y1 specified in Table 8.2.3.2.14-1 during SIB1 decoding time period TSIB1 (ms) specified in clause 9.11 for SSB and CORESET for RMSI scheduling multiplexing patterns 1.

-	with up to L2 interruptions with interrupted slots up to interruption length Y2 specified in Table 8.2.3.2.14-1 during SIB1 decoding time period TSIB1 (ms) specified in clause 9.11 for SSB and CORESET for RMSI scheduling multiplexing patterns 2 and 3.

Where:

-	K1 = 6 for the target cell carrier frequency on FR1 and K1 = 25 for the target cell carrier frequency on FR2, and

-	L1 = TSIB1/20, and

-	L2 = TSIB1/TSMTC, where TSMTC is the periodicity of the SMTC occasion configured for the target cell carrier.

Table 8.2.3.2.14-1: Interruption length X1, Y1 and Y2 during measurements with autonomous gaps

|  | NR Slot length (ms) of victim cell | Interruption length X1 (slots) | Interruption length Y1 (slots) | Interruption length Y2 (slots) |
| --- | --- | --- | --- | --- |
| 0 | 1 | 6 | 7 | 6 |
| 1 | 0.5 | 12 | 13 | 10 |
| 2 | 0.25 | 24 | 25 | 19 |
| 3 | 0.125 | 48 | 49 | 37 |

<End of Change 4>

<Start of Change 5>

8.2.4.2.11	 Interruptions when identifying CGI of an NR cell with autonomous gaps

When a UE is identifying CGI of an NR cell with autonomous gaps, the UE is allowed interruptions on PCell, PSCell or any activated SCell:

-	with up to K1 interruptions with interrupted slots up to interruption length X1 specified in Table 8.2.4.2.11-1 for each interruption during MIB decoding time period TMIB (ms) specified in clause 9.11.

-	with up to L1 interruptions with interrupted slots up to interruption length Y1 specified in Table 8.2.4.2.11-1 during SIB1 decoding time period TSIB1 (ms) specified in clause 9.11 for SSB and CORESET for RMSI scheduling multiplexing patterns 1.

-	with up to L2 interruptions with interrupted slots up to interruption length Y2 specified in Table 8.2.4.2.11-1 during SIB1 decoding time period TSIB1 (ms) specified in clause 9.11 for SSB and CORESET for RMSI scheduling multiplexing patterns 2 and 3.

Where:

-	K1 = 6 for the target cell carrier frequency on FR1 and K1 = 25 for the target cell carrier frequency on FR2, and

-	L1 = TSIB1/20, and

-	L2 = TSIB1/TSMTC, where TSMTC is the periodicity of the SMTC occasion configured for the target cell carrier.

Table 8.2.4.2.11-1: Interruption length X1, Y1 and Y2 during measurements with autonomous gaps

|  | NR Slot length (ms) of victim cell | Interruption length X1 (slots) | Interruption length Y1 (slots) | Interruption length Y2 (slots) |
| --- | --- | --- | --- | --- |
| 0 | 1 | 6 | 7 | 6 |
| 1 | 0.5 | 12 | 13 | 10 |
| 2 | 0.25 | 24 | 25 | 19 |
| 3 | 0.125 | 48 | 49 | 37 |

<End of Change 5>