3GPP TSG-WG RAN4 Meeting #98-e	R4-2103077

# Online, 25th January – 5th February, 2021

Source: 				Nokia, Nokia Shanghai Bell, [Bell Mobility]

Title: 				TP to TR 38.717-03-01 CA_n5-n25-n77

Agenda Item: 	9.9.2 [NR_CA_R17_3BDL_1BUL]

Document for:	Approval

# Introduction

In this contribution, a text proposal to complete 3DL/1UL NR CA configuration CA_n5A-n25A-n77A is provided.

# TP to TR 38.717-03-01

## 6.x	CA_n5-n25-n77

### 6.x.1	Operating bands for CA

Table 6.X.1-1: 3DL Inter-band CA operating bands

| NR CA Band | NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n5-n25-n77 | n5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | n25 | 1850 MHz | – | 1915 MHz | 1930 MHz | – | 1995 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

### 6.X.2	Channel bandwidths per operating band for CA

Table 6.X.2-1: Supported channel bandwidths per CA configuration for 3DL inter-band CA

| NR CA Configuration | UL Config | NR Band | Channel bandwidth (MHz) |  |  |  |  |  |  |  |  |  |  |  |  | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n5A-n25A-n77A | - | n5 | 5 | 10 | 15 | 10 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n25 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
|  |  | n77 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |

### 6.X.3	Co-existence studies

Table 6.X.3-1 summarizes frequency ranges where harmonics occur due to 3DL bands CA with 1 UL. The 4th and 5th harmonics of band n5 uplink may fall into band n77 downlink. The 2nd harmonic of band n25 may fall into band n77 downlink.

Table 6.X.3-1: Harmonic Interference for 3DLs/1UL

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n5 | 824 | 849 | 869 | 894 | 1648 | 1698 | 2472 | 2547 | 3296 | 3396 | 4120 | 4245 |
| n25 | 1850 | 1915 | 1930 | 1995 | 3700 | 3830 | 5550 | 5745 | - | - | - | - |
| n77 | 3300 | 4200 | 3300 | 4200 | 6600 | 8400 | 9900 | 12600 | - | - | - | - |

Table 6.X.3-2 gives harmonic mixing issue for CA with Band n5, n25 and n77. Theare is a harmonic mixing relation for the 4th harmonic of n5 downlink and n77 downlink. Theare is a harmonic mixing relation for the 2nd harmonic of n25 downlink and n77 downlink.

Table 6.X.3-2 Harmonic mixing for 3DLs/1UL

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n5 | 824 | 849 | 869 | 894 | 1738 | 1788 | 2607 | 2682 | 3476 | 3576 | 4345 | 4470 |
| n25 | 1850 | 1915 | 1930 | 1995 | 3860 | 3990 | 5790 | 5985 | - | - | - | - |
| n77 | 3300 | 4200 | 3300 | 4200 | 6600 | 8400 | 9900 | 12600 | - | - | - | - |

For single uplink, the UE coexistence is already considered for these bands in TS 38.101-1 [3].

### 6.X.4	∆TIB,c and ∆RIB,c values

For three simultaneous DLs and one UL of Band n5, n25 and n77, the TIB,c and RIB,c values are shown in table 6.X.4-1 and table 6.X.4-2, respectively. The requirement is reused from the similar combination, DC_2-5_n77.

Table 6.X.4-1: ΔTIB,c for 3DL aggregation

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n5-n25-n77 | n5 | 0.6 |
|  | n25 | 0.6 |
|  | n77 | 0.8 |

Table 6.X.4-2: ΔRIB,c for 3DL aggregation

| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n5-n25-n77 | n5 | 0.2 |
|  | n25 | 0.2 |
|  | n77 | 0.5 |

### 6.X.5	REFSENS requirements

MSD due to the above harmonic issues are specifed in 2DL/1UL fallback CAs, CA_n5-n77 and CA_n25-n77.