3GPP TSG-RAN WG4 Meeting #98-e                                                            R4-2103087

Online, 25th Jan - 5th Feb, 2021

Source:	Samsung, KDDI

Title:	TP for TR 38.717-03-02: CA_n3-n18-n41

Agenda item:			9.11.2

Document for:	Approval

# 1. Introduction

This contribution is a text proposal for TR 38.717-03-02 to include CA_n3A-n18A-n41(2A) according to the request in [1].

2. Reference

- RP-202201, Revised WID on Rel-17 NR Inter-band Carrier Aggregation/Dual Connectivity for 3 bands DL with 2 bands UL.

# 3. Text Proposal

<Start of Text Proposal>

5.1.x	CA_n3-n18-n41

5.1.x.1	Operating bands for CA

Table 5.1.x.1-1: 3DL Inter-band CA operating bands

| NR CA Band | NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n3-n18-n41 | n3 | 1710MHz | – | 1780MHz | 1805MHz | – | 1880MHz | FDD |
|  | n18 | 815MHz | – | 830MHz | 860MHz | – | 875MHz | FDD |
|  | n41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |

5.1.x.2	Channel bandwidths per operating band for CA

Table 5.1.x.2-1: Supported channel bandwidths per CA configuration for 3DL inter-band CA

| NR CA Configuration | Uplink CA configuration | NR Band | Channel bandwidth (MHz) (NOTE 3) |  |  |  |  |  |  |  |  |  |  |  |  | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n3A-n18A-n41A | CA_n3A-n41A CA_n3A-n18A CA_n18A-n41A | n3 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  | 0 |
|  |  | n18 | 5 | 10 | 15 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n41 |  | 10 | 15 | 20 |  | 30 | 40 | 50 | 60 |  | 80 | 90 | 100 |  |

5.1.x.3	Co-existence studies

Co-existence studies of CA_n3-n18-n41 with 2UL are already covered in the constituent fall-back modes, it can get that:

- IMD2 of band n18 UL and band n41 UL falling to band n3 DL

- IMD2 and IMD3 of band n3 UL and band n41 UL falling to band n18 DL.

- IMD2 and IMD3 of band n3 UL and band n18 L falling to band n41 DL.

5.1.x.4	REFSENS requirements

Table 5.1.x.4-1 shows the required MSD levels for the CA configuration, its value can refer to the value of DC_18A-41A_n3A in TS 38.101-3 and CA_3A-5A-7A in TS 36.101 and update with considering n41 changed from 5 to 10MHz.

Table 5.1.x.4-1: 3DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n3A-n18A-n41A | n18 | 820 | 5 | 25 | 865 | N/A | FDD | N/A |
|  | n3 | 1720 | 5 | 25 | 1815 | N/A | FDD | N/A |
|  | n41 | 2540 | 10 | 50 | 2540 | [N/A]1 | TDD | IMD2 |
|  | n18 | 820 | 5 | 25 | 865 | N/A | FDD | N/A |
|  | n3 | 1725 | 5 | 25 | 1820 | N/A | FDD | N/A |
|  | n41 | 2630 | 10 | 50 | 2630 | 16.0 | TDD | IMD3 |
|  | n18 | 820 | 5 | 25 | 865 | 28.9 | FDD | IMD2 |
|  | n3 | 1765 | 5 | 25 | 1860 | N/A | FDD | N/A |
|  | n41 | 2630 | 10 | 50 | 2630 | N/A | TDD | N/A |
|  | n18 | 830 | 5 | 25 | 875 | [19.0] | FDD | IMD3 |
|  | n3 | 1725 | 5 | 25 | 1820 | N/A | FDD | N/A |
|  | n41 | 2670 | 5 | 25 | 2670 | N/A | TDD | N/A |
|  | n3 | 1755 | 5 | 25 | 1850 | 28.8 | FDD | IMD2 |
|  | n41 | 2670 | 10 | 50 | 2670 | N/A | TDD | N/A |
|  | n18 | 820 | 5 | 25 | 865 | N/A | FDD | N/A |

NOTE 1: [N/A] is explained that an IMD2 may fall in band n41, but due to operator's spectrum holding, the MSD is not applicable.

<End of Text Proposal>