3GPP TSG-RAN WG4 Meeting #98-e                                                            R4-2103089

Online, 25th Jan - 5th Feb, 2021

Source:	Samsung, KDDI

Title:	TP for TR 38.717-03-02: CA_n3-n41-n78

Agenda item:			9.11.2

Document for:	Approval

# 1. Introduction

This contribution is a text proposal for TR 38.717-03-02 to include CA_n3A-n41A-n78A and CA_n3A-n41A-n78(2A) according to the request in [1].

2. Reference

- RP-202201, Revised WID on Rel-17 NR Inter-band Carrier Aggregation/Dual Connectivity for 3 bands DL with 2 bands UL.

# 3. Text Proposal

<Start of Text Proposal>

5.1.x	CA_n3-n41-n78

5.1.x.1	Operating bands for CA

Table 5.1.x.1-1: 3DL Inter-band CA operating bands

| NR CA Band | NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n3-n41-n78 | n3 | 1710MHz | – | 1780MHz | 1805MHz | – | 1880MHz | FDD |
|  | n41 | 2496MHz | – | 2690MHz | 2496MHz | – | 2690MHz | TDD |
|  | n78 | 3300MHz | – | 3800MHz | 3300MHz | – | 3800MHz | TDD |

5.1.x.2	Channel bandwidths per operating band for CA

Table 5.1.x.2-1: Supported channel bandwidths per CA configuration for 3DL inter-band CA

| NR CA Configuration | Uplink CA configuration | NR Band | Channel bandwidth (MHz) (NOTE 3) |  |  |  |  |  |  |  |  |  |  |  |  | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n3A-n41A-n78A | CA_n3A-n41A CA_n3A-n78A CA_n41A-n78A | n3 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  | 0 |
|  |  | n41 |  | 10 | 15 | 20 |  | 30 | 40 | 50 | 60 |  | 80 | 90 | 100 |  |
|  |  | n78 |  | 10 | 15 | 20 |  |  | 40 | 50 | 60 |  | 80 | 90 | 100 |  |
| CA_n3A-n41A-n78(2A) | CA_n3A-n41A CA_n3A-n78A CA_n41A-n78A | n3 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  | 0 |
|  |  | n41 |  | 10 | 15 | 20 |  | 30 | 40 | 50 | 60 |  | 80 | 90 | 100 |  |
|  |  | n78 | See CA_n78(2A) Bandwidth Combination Set 2 in Table 5.5A.2-1 in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |

5.1.x.3	Co-existence studies

Co-existence studies of CA_n3-n41-n78 with 2UL are already covered in the constituent fall-back modes, it can get that:

- IMD3 and IMD 4of band n41 UL and band n78 UL falling to band n3 DL, while for IMD4 it doesn’t need MSD results for lower order IMD.

- IMD2, IMD3 and IMD5 of band n3 UL and band n41 UL falling to band n78 DL, while there was no IMD2 impact to n78 and for IMD5 it doesn’t need MSD results for lower order IMD.

5.1.x.4	REFSENS requirements

Table 5.1.x.4-1 shows the required MSD levels for the CA configuration, its value can reuse the value of DC_3A-n41A_n78A and DC_3A-41A_n78A in TS 38.101-3.

Table 5.1.x.4-1: 3DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n3A-n41A-n78A CA_n3A-n41A-n78(2A) | n3 | 1730 | 5 | 25 | 1825 | N/A | FDD | N/A |
|  | n41 | 2560 | 10 | 50 | 2560 | N/A | TDD | N/A |
|  | n78 | 3390 | 10 | 50 | 3390 | 16.4 | TDD | IMD3 |
|  | n3 | 1745 | 5 | 25 | 1840 | 16.4 | TDD | IMD3 |
|  | n41 | 2620 | 5 | 25 | 2620 | N/A | FDD | N/A |
|  | n78 | 3400 | 10 | 52 | 3400 | N/A | TDD | N/A |

<End of Text Proposal>