TSG-RAN Working Group 4 (Radio) meeting #98-E	R4-2103109

Electronic Meeting, January 25th to February 5th 2021

Source:	Ericsson

Title:	TP to TR 38.921: Addition of in-door antenna parameters and correction to model in subclause 8.1

Agenda item:	10.1.4

Document for:	Approval

# Introduction

At the last RAN4 meeting (RAN4#97-E) antenna parameters for indoor AAS base stations was discussed. Currently, parameter sets for Macro suburban, Macro urban and Micro urban is defined for the frequency range 6.425 to 10.500 GHz. In this contribution we present antenna parameters relevant for an In-door deployment scenario. In conjunction suggestions for some minor corrections is presented.

At the end of this contribution a text proposal is attached for approval. The text proposal consists of two parts; Addition of antenna parameters for in-door deployment scenario and correction of antenna parameter definition in subclause 8.1.

This is a revision of R4-2101182.

# Discussion

For the frequency range in mind it is reasonable to assume a dual polarized 4x4 URA (Uniform Rectangular Array) antenna with 0.5l element separation is representable for an in-door AAS base station. Currently, the array antenna model and corresponding description in TR 38.921, subclause 8.1 does not state explicitly how the element separation is selected for a specific design. Since the element separation is given in terms of wavelengths, it could be incorrectly understood that the element separation varies as function of frequency. The element separation is fixed for a specific product implementation. Typically, the design element separation is determined to guarantee grating lobe free operation for a specific steering range. Consequently, the element separation is set after the highest supported frequency. For in-door deployment of AAS base station several deployment scenarios can be considered, e.g. wall mounted, roof mounted, etc. For the study regarding extension of NR into 6 to 10 GHz it has been suggested to focus on the roof mounted case, where the array antenna panel in the roof with the normal pointing downwards towards the floor.

For the roof mounted case, the horizontal coverage ranges are not relevant since the coverage is not limited to a specific sector. The same reasoning applies for the vertical coverage range, where no limitation is applied. Therefore, those parameters are not relevant for the in-door AAS base station deployment scenario. To clarify the placement of the base station a note is required in the parameter table.

In Table 2-1, a parameter set relevant for in-door AAS base station operating within the frequency range 6 to 10 GHz have been added to previously defined parameter sets.

Table 2-1: AAS base station array antenna parameters relevant for 6 to 10 GHz

| Parameter | Macro Sub-urban | Macro Urban | Micro Urban | Small cell Indoor |
| --- | --- | --- | --- | --- |
| Am (dB) | 30 | 30 | 30 | 30 |
| SLAv (dB) | 30 | 30 | 30 | 30 |
| 3dB (deg.) | 90 | 90 | 90 | 90 |
| 3dB (deg.) | 65 | 90 | 90 | 90 |
| GE,max (dBi) | 6.4 | 5.5 | 5.5 | 5.5 |
| LE (dB) | 2.0 | 2.0 | 2.0 | 2.0 |
| (M, N) | (8, 16) | (8, 16) | (8, 8) | (4, 4) |
| dh (m) | 0.5 | 0.5 | 0.5 | 0.5 |
| dv (m) | 0.7 | 0.5 | 0.5 | 0.5 |
| Horizontal coverage range (deg.) | +/- 60 | +/- 60 | +/- 60 | N/A |
| Vertical coverage range (deg.) | 90 to 100 | 90 to 120 | 90 to 120 | N/A |
| Conducted power (before ohmic loss) per antenna element (dBm) | 22 | 22 | 16 | 9 |
| Mechanical downtilt (deg.) | 6 | 10 | N/A | N/A |

In conjunction of the introduction of indoor parameters, some general error corrections were fixed. The updates are summarized as:

- In Table 8.1.1-1, update the definition of M and N to match the equations given in table 8.1.1-2.

- In Table 8.1.2-1, update the parameter with respect to M and N definitions.

# Conclusion

At the end of this contribution a text proposal to TR 38.921, subclause 8.1 is attached for approval. The text proposal adds missing in-door AAS base station parameters and corrects errors in the antenna model.

# References

[1]	R4-2011830, “TP to TR 38.921: Addition of BS antenna model and parameters in subclause 4.2.3 and subclause 8.1”, Ericsson

[2] 	R4-2011932, “LS on Parameters of terrestrial component of IMT for sharing and compatibility studies in preparation for WRC-23 (6.425 to 10.5 GHz)”, Ericsson

[3]	R4-2014979, “TP to TR 38.921: Correction to antenna parameter table in clause 3 and sub-clause 8.1”, Ericsson

TEXT PROPOSAL to subclause 8.1:

## 8.1	BS antenna characteristics

### 8.1.1		Array antenna model

In Table 8.1.1-1, the parameters used by the parameterized array antenna model are described.

Table 8.1.1-1: Parameters of the parameterized array antenna model

| Parameter | Symbol | Unit |
| --- | --- | --- |
| Front to back ratio | Am | dB |
| Side lobe suppression | SLAv | dB |
| Horizontal HPBW | 3dB | Degrees |
| Vertical HPBW | 3dB | Degrees |
| Array element peak gain | GE,max | dBi |
| Number of radiating elements rows and columns | (M, N) | Integer |
| Horizontal element separation | dh | m |
| Vertical element separation | dv | m |
| Electrical down-tilt angle | etilt | Degrees |
| Electrical scan angle | escan | Degrees |

The parameterized antenna model is built around array antenna model where the element factor, array factor and linear phase progressing is characterized as described by equations in Table 8.1.1-2.

Table 8.1.1-2: Array antenna model details

| Description | Equation | Unit |
| --- | --- | --- |
| Peak normalized element radiation pattern | Aθ,φ=-min--min12φφ3dB2,Am-min12θ-90θ3dB2,SLAv ,Am | dB |
| Peak gain normalized element radiation pattern | AEθ,φ=GE,max+Aθ,φ | dBi |
| Composite array radiation pattern | AAθ,φ=AEθ,φ+10log10m=1Mn=1Nwm,nvm,n2 , where vm,n=expj2πnm-1dvλcosθ+mn-1dhλsinθsinφ wm,n=1MNexpj2πnm-1dvλsinθetilt-mn-1dhλcosθetiltsinφescan | dBi |

### 8.1.2		Array antenna parameters

In Table 8.1.2-1, base station array antenna parameters for different deployment scenarios is listed. Element parameters have been selected to produce correct element peak gain determined by calculating the directivity from a given geometry including beam widths. The element directivity can be calculated based on the pattern described by Table 8.1.2-1 in dBi as:

GE,max=DE,max-LE	(8.1.2-1)

, where the peak directivity DE,max is calculated from given values on 3dB, 3dB, dh and dv as:

DE,max=10log104πAlinθ,φmax-ππ0πAlinθ,φsinθdθdφ	(8.1.2-2)

, where Alin(,) is defined in linear scale as:

Alinθ,φ=10Aθ,φ10	(8.1.2-3)

Table 8.1.2-1: BS array antenna parameters

| Parameter | Macro Sub-urban | Macro Urban | Micro Urban | Small cell indoor |
| --- | --- | --- | --- | --- |
| Am (dB) | 30 | 30 | 30 | 30 |
| SLAv (dB) | 30 | 30 | 30 | 30 |
| 3dB (deg.) | 90 | 90 | 90 | 90 |
| 3dB (deg.) | 65 | 90 | 90 | 90 |
| GE,max (dBi) | 6.4 | 5.5 | 5.5 | 5.5 |
| LE (dB) | 2.0 | 2.0 | 2.0 | 2.0 |
| (M, N) | (16, 8) | (16, 8) | (8,8) | (4, 4) |
| dh (m) | 0.5 | 0.5 | 0.5 | 0.5 |
| dv (m) | 0.7 | 0.5 | 0.5 | 0.5 |
| Horizontal coverage range (deg.) | +/- 60 | +/- 60 | +/- 60 | N/A |
| Vertical coverage range (deg.) | 90 to 100 | 90 to 120 | 90 to 120 | N/A |
| Conducted power (before ohmic loss) per antenna element (dBm) | 22 (Note 4) | 22 (Note 4) | 16 (Note 5) | 9 (Note 6) |
| Mechanical downtilt (deg.) | 6 | 10 | N/A | N/A (Note 7) |
| Note 1: MxN means there are M vertical and N horizontal elements Note 2: LE is included in GE,max Note 3: The vertical coverage range includes the mechanical downtilt. Note 4: The conducted power per element assumes 16x8x2 elements (i.e. power per H/V polarized element). Note 5: The conducted power per element assumes 8x8x2 elements (i.e. power per H/V polarized element). Note 6: The conducted power per element assumes 4x4x2 elements (i.e. power per H/V polarized element). Note 7: Boresight direction is perpendicular to the ceiling. |  |  |  |  |

## 8.2	UE antenna characteristics

The outcome of the RAN4 study for collecting technical background information relevant for the frequency range 7 to 24 GHz indicated that the frequency range 7.25-[10-13] GHz would have “FR1 like” requirements, and as such we can assume that in the 10-10.5GHz range this applies. The UE will most likely therefore have a conducted interface with an assumed isotropic radiation pattern antenna and no beam forming.