3GPP TSG-RAN WG4 Meeting # 98-e 	R4-2103170

# Electronic Meeting, 25th January – 5th February, 2021

Source:		Verizon, Ericsson, MediaTek, LGE

Title:			TP for TR38.xxx for PC2 CA_n5A-n77A

Agenda item:		9.19.2

Document for:		Approval

# Introduction

This contribution is a text proposal to introduce PC2 2DL/2UL CA_n5-n77 according to the request in [1]. And, the associated requirements in this TP are kept on the same of RAN4 agreed proposals [2].

# Reference

[1] RP-202373, Revised basket WID: High power UE for NR inter-band Carrier Aggregation with 2 bands downlink and x bands uplink (x =1,2)

[2] R4-1900534, Power class 2 UE for FDD-TDD EN-DC, MediaTek

# Text Proposal

<Start of Text Proposal>

## 5.x 	CA_n5-n77

### 5.x.1	Configurations

Table 5.x.1-1: NR CA configurations and bandwidth combinations sets for supporting power class 2

| NR CA configuration | Uplink CA configuration | NR Band | SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n5A-n77A CA_n5A-n77(2A) | CA_n5A-n77A | n5 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes4 | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes4 | Yes |  |
| NOTE 4: This UE channel bandwidth is optional in this release of the specification |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 5.x.2	Maximum output power

| Uplink CA configuration | Power class 2 cases for CA_n5A-n77A | CA power class | Carrier n5 power class | Carrier n77 power class |
| --- | --- | --- | --- | --- |
| CA_n5A-n77A | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

The tolerance +2/-3 dB is applied. Also when the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB.

### 5.x.3	REFSENS requirements

According to the PC3 CA_n5A-n77A study, the 4th and 5th IMD products from dual uplink of band 5 and n77 may fall into band 5 Rx frequency range. Thus additional MSD should be considered to mitigate the impact of the interference for PC2 CA_n5A-n77A combination.

#### 5.x.3.1	Power class 2 Case A

The additional MSD due to intermodulation for PC2 CA_n5A-n77A are defined in table 5.x.3.1-1.

Table 5.x.3.1-1: MSD test points for PCell due to dual uplink operation for PC2 NR CA in NR FR1 (two bands)

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD for PC2 (dB) | Duplex mode | Source of IMD |
| CA_n5A-n77A | n5 | 844 | 5 | 25 | 889 | 18.6 | FDD | IMD44 |
|  | n77 | 3421 | 10 | 50 | 3421 | N/A | TDD | N/A |
| NOTE 4: This band is subject to IMD5 also which MSD is not specified. |  |  |  |  |  |  |  |  |

#### 5.x.3.2	Power class 2 Case B

The additional MSD due to intermodulation for PC2 Case B CA_n5A-n77A are same as the Case A defined in table 5.x.3.1-1.

#### 5.x.3.3	OOB blocking exception requirements

Since band n5 is a low band and n77 is a wide band, the OOBB exception is needed.

Table 5.x.4-1: NR DC band combination with exceptions allowed

CA band combination

CA_n5-n77

### 5.x.4		∆TIB and ∆RIB values

For the ∆TIB,c and ∆RIB,c values, same PC3 CA_n5A-n77A requirements are applied for PC2 CA_n5A-n77A.

<End of Text Proposal>