3GPP TSG-RAN WG4 Meeting # 98-e 	R4-2103176

# Electronic Meeting, 25th January – 5th February, 2021

Source:		Verizon, Ericsson, MediaTek, LGE

Title:			TP for TR 37.826 for DC_5_n77

Agenda item:		9.20.2

Document for:		Approval

# Introduction

This contribution is a text proposal for TR 37.826 to include DC_5_n77 according to the request in [1]. And, the associated requirements in this TP are kept on the same of RAN4 agreed proposals [2].

# Reference

[1] RP-20xxxx, Revised WID on High power UE (power class 2) for EN-DC with 1 LTE band + 1 NR TDD band

[2] R4-1900534, Power class 2 UE for FDD-TDD EN-DC, MediaTek

# Text Proposal

<Start of Text Proposal>

## 6.x	DC_5A_n77A

### 6.x.1	Transmitter Characteristics

#### 6.x.1.1	Maximum Output Power

Table 6.x.1.1-1: Maximum output power for inter-band EN-DC (two bands)

| EN-DC combination | Power class 2 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_5A_n77A | 266 | +2/-3 |
| NOTE 6: The UE supports PC3 within E-UTRA cell group, and supports either PC3 or PC2 within NR cell group. Power class support within each individual cell group is signalled separately by the UE. |  |  |

#### 6.x.1.2	Co-existence study

According to the PC3 DC_5A_n77A study, the 4th and 5th IMD products from dual uplink of band 5 and n77 may fall into the band 5 Rx frequency range. Thus, additional MSD for IMD 4 and 5 should be considered to mitigate the impact of the interference for PC2 DC_5A_n77A combination.

### 6.x.2	Receiver Characteristics

#### 6.x.2.1	MSD test points for intermodulation interference due to dual uplink operation for PC2 EN-DC in NR FR1 involving two bands

#### 6.x.2.1.1	Power class 2 Case A

The additional MSD due to intermodulation for PC2 Case A DC_5A_n77A are defined in table 6.x.2.1-1.

Table 6.x.2.1-1: MSD test points for PCell due to dual uplink operation for PC2 EN-DC in NR FR1 (two bands)

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR EN-DC Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD for PC2 (dB) | Duplex mode | Source of IMD |
| DC_5A_n77A | 5 | 844 | 5 | 25 | 889 | 18.60 | FDD | IMD44 |
|  | n77 | 3421 | 10 | 50 | 3421 | N/A | TDD | N/A |
| NOTE 4: This band is subject to IMD5 also which MSD is not specified. |  |  |  |  |  |  |  |  |

#### 6.x.2.1.2	Power class 2 Case B

The additional MSD due to intermodulation for PC2 Case B DC_5A_n77A are same as the Case A defined in table 6.x.2.1-1.

#### 6.x.2.2		OOB blocking exception requirements

Since band 5 is a low band and n77 is a wide band, the OOBB exception is needed.

Table 6.x.2.4-1: EN-DC band combination with exceptions allowed

EN-DC band combination

DC_5_n77

<End of Text Proposal>