3GPP TSG-RAN WG4 Meeting #98e 										   				     R4-2103256

Electronic Meeting, Jan 25th – Feb 5th, 2021

Agenda item:			12.2.2

Source:	Nokia, Nokia Shanghai Bell

Title:	TP to TR 38.808: BS antenna array

Document for:	Approval

# Introduction

The study item on supporting NR from 52.6 GHz to 71 GHz [1] was approved at RAN#86. Before that, 3GPP RAN studied requirements for NR beyond 52.6GHz up to 114.25GHz, potential use cases and deployment scenarios, and NR system design requirements and considerations on top of regulatory requirements [2].

BS antenna arrays were discussed in following contributions submitted to RAN4#98-e:

- R4-2101183	Ericsson

- R4-2102007	Nokia, Nokia Shanghai Bell

- R4-2102570	Huawei

This text proposal merges the proposed content to be captured into TR 38.808.

# Text proposal to TR 38.808 V1.0.0

<Start of text proposal>

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 38.913: "Study on Scenarios and Requirements for Next Generation Access Technologies"

[2]	3GPP TR 38.807: "Study on requirements for NR beyond 52.6 GHz".

[3]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[4]	ETSI EN 302 567 v2.1.20: "Multiple-Gigabit/s radio equipment operating in the 60 GHz band; Harmonised Standard covering the essential requirements of article 3.2 of Directive 2014/53/EU".

[5]	R1-2007549 "Further discussion on B52 numerology" FUTUREWEI.

[6]	R1-2007558 "Discussion on physical layer impacts for NR beyond 52.6 GHz" Lenovo, Motorola Mobility.

[7]	R1-2007604 "PHY design in 52.6-71 GHz using NR waveform" Huawei, HiSilicon.

[8]	R1-2007642 "Physical layer design for NR 52.6-71GHz" Beijing Xiaomi Software Tech.

[9]	R1-2007652 "Discussion on required changes to NR using existing DL/UL NR waveform" vivo.

[10]	R1-2007785 "Consideration on required changes to NR using existing NR waveform" Fujitsu.

[11]	R1-2007790 "Consideration on supporting above 52.6GHz in NR" InterDigital, Inc.

[12]	R1-2007847 "System Analysis of NR opration in 52.6 to 71 GHz" CATT.

[13]	R1-2007883 "Required changes to NR using existing DL/UL NR waveform" TCL Communication Ltd.

[14]	R1-2007926 "Required changes to NR using existing DL/UL NR waveform" Nokia, Nokia Shanghai Bell.

[15]	R1-2007929 "On phase noise compensation for NR from 52.6GHz to 71GHz" Mitsubishi Electric RCE.

[16]	R1-2009379 "Discussion on Required Changes to NR in 52.6 – 71 GHz" Intel Corporation.

[17]	R1-2007965 "On the required changes to NR for above 52.6GHz" ZTE, Sanechips.

[18]	R1-2007982 "On NR operations in 52.6 to 71 GHz" Ericsson.

[19]	R1-2009653 "Consideration on required physical layer changes to support NR above 52.6 GH"	LG Electronics.

[20]	R1-2008076 "Discussion on required changes to NR using existing DL/UL NR waveform in 52.6GHz ~ 71GHz" CMCC.

[21]	R1-2008082 "Study on the numerology to support 52.6 GHz to 71GHz" NEC.

[22]	R1-2008872 "Design aspects for extending NR to up to 71 GHz" Samsung.

[23]	R1-2008250 "Discusson on required changes to NR using DL/UL NR waveform" OPPO.

[24]	R1-2008353 "Considerations on required changes to NR from 52.6 GHz to 71 GHz" Sony.

[25]	R1-2008457 "A Discussion on Physical Layer Design for NR above 52.6GHz" Apple.

[26]	R1-2008493 "Discussions on required changes on supporting NR from 52.6GHz to 71 GHz" CAICT.

[27]	R1-2008501 "On required changes to NR using existing DL/UL NR waveform for operation in 60GHz band" MediaTek Inc.

[28]	R1-2008516 "On NR operation between 52.6 GHz and 71 GHz" Convida Wireless.

[29]	R1-2009062 "Evaluation Methodology and Required Changes on NR from 52.6 to 71 GHz" NTT DOCOMO, INC.

[30]	R1-2008615 "NR using existing DL-UL NR waveform to support operation between 52p6 GHz and 71 GHz" Qualcomm Incorporated.

[31]	R1-2008726 "Discussion on physical layer aspects for NR beyond 52.6GHz" WILUS Inc.

[32]	R1-2008769 "Waveform considerations for NR above 52.6 GHz" Charter Communications.

[33]	R1-2007550 "On channel access modes in 60GHz" FUTUREWEI.

[34]	R1-2007559 "Discussion on channel access for NR beyond 52.6 GHz" Lenovo, Motorola Mobility.

[35]	R1-2008976 "Channel access mechanism for 60 GHz unlicensed operation" Huawei, HiSilicon.

[36]	R1-2007643 "Channel access mechanism for NR on 52.6-71 GHz" Beijing Xiaomi Software Tech.

[37]	R1-2007653 "Discussion on channel access mechanism" vivo.

[38]	R1-2007791 "On Channel access mechanisms" InterDigital, Inc.

[39]	R1-2007848 "Channel Access Mechanism in support of NR operation in 52.6 to 71 GHz" CATT.

[40]	R1-2007884 "Channel access mechanism" TCL Communication Ltd.

[41]	R1-2007918 "Channel access mechanisms for NR from 52.6-71GHz" AT&T.

[42]	R1-2009312 "Design of NR channel access mechanisms for 60 GHz unlicensed band" Nokia, Nokia Shanghai Bell.

[43]	R1-2009380 "Channel Access Procedure for NR in 52.6 - 71 GHz" Intel Corporation.

[44]	R1-2007966 "On the channel access mechanism for above 52.6GHz" ZTE, Sanechips.

[45]	R1-2007983 "Channel Access Mechanism" Ericsson.

[46]	R1-2008046 "Considerations on channel access mechanism to support NR above 52.6 GHz" LG Electronics.

[47]	R1-2008091 "Discussion on channel access mechanism for above 52.6GHz" Spreadtrum Communications.

[48]	R1-2008157 "Channel access mechanism for 60 GHz unlicensed spectrum" Samsung.

[49]	R1-2008251 "Discussion on channel access" OPPO.

[50]	R1-2008354 "Channel access mechanism for 60 GHz unlicensed spectrum" Sony.

[51]	R1-2008458 "Views on Channel Access Mechanisms for Unlicensed Access above 52.6 GHz" Apple.

[52]	R1-2008494 "Discussions on channel access mechanism on supporting NR from 52.6GHz to 71 GHz" CAICT.

[53]	R1-2008517 "On Channel Access Mechanism and Interference Handling for Supporting NR from 52.6 GHz to 71 GHz" Convida Wireless.

[54]	R1-2008548 "Channel Access Mechanism for NR in 60 GHz unlicensed spectrum" NTT DOCOMO, INC.

[55]	R1-2008563 "Discussion on channel access mechanism" ITRI.

[56]	R1-2009362 "Channel access mechanism for NR in 52p6 to 71GHz band" Qualcomm Incorporated.

[57]	R1-2008717 "Discussion on channel access mechanism for 52.6 to 71GHz unlicensed ban"	Potevio

[58]	R1-2008770 "Further aspects of channel access mechanisms" Charter Communications.

[59]	R1-2007560 "Additional evaluations for NR beyond 52.6GHz" Lenovo, Motorola Mobility.

[60]	R1-2007654 "Evaluation on different numerologies for NR using existing DL/UL NR waveform" vivo.

[61]	R1-2007792 "Evaluation results for above 52.6 GHz" InterDigital, Inc.

[62]	R1-2007928 "Simulation Results for NR from 52.6 GHz to 71 GHz" Nokia, Nokia Shanghai Bell.

[63]	R1-2007943 "Considerations on performance evaluation for NR in 52.6-71GHz" Intel Corporation.

[64]	R1-2009450 "Simulation results for NR above 52.6GHz" ZTE, Sanechips.

[65]	R1-2007984 "Evaluation results for NR in 52.6 - 71 GHz" Ericsson.

[66]	R1-2008047 "Considerations on phase noise compensation to support NR above 52.6 GHz" LG Electronics.

[67]	R1-2008873 "Evaluation results for extending NR to up to 71 GHz" Samsung.

[68]	R1-2009615 "Discussion on other aspects" OPPO.

[69]	R1-2008459 "Evaluation results for Physical Layer Design for NR above 52.6GHz" Apple.

[70]	R1-2008549 "Potential Enhancements for NR on 52.6 to 71 GHz" NTT DOCOMO, INC.

[71]	R1-2009157 "Performance evaluations for NR above 52.6 GHz" Charter Communications.

[72]	R1-2009610 "Link level and System level evaluation for NR system operating in 52.6GHz to 71GHz" Huawei, HiSilicon.

[73]	3GPP TR 38.803: "Study on new radio access technology; Radio Frequency (RF) and co-existence aspects".

[74]	Hua Wang, Fei Wang, Sensen Li, Tzu-Yuan Huang, Amr S. Ahmed, Naga Sasikanth Mannem, Jeongseok Lee, Edgar Garay, David Munzer, Christopher Snyder, Sanghoon Lee, Huy Thong Nguyen, and Michael Edward Duffy Smith, "Power Amplifiers Performance Survey 2000-Present," [Online]. Available: https://gems.ece.gatech.edu/PA_survey.html

[75]	ETSI TR 101 854: "Fixed Radio Systems; Point-to-point equipment; Derivation of receiver interference parameters useful for planning fixed service point-to-point systems operating different equipment classes and/or capacities"

[76]	Kambiz Hadipour, Andrea Ghilioni1, Andrea Mazzanti1, Matteo Bassi1, Francesco Svelto, “A 40GHz to 67GHz Bandwidth 23dB Gain 5.8dB Maximum NF mm-Wave LNA in 28nm CMOS”, 2015 IEEE Radio Frequency Integrated Circuits Symposium

[77]	Domenico Pepe, Domenico Zito, “32 dB Gain 28 nm Bulk CMOS W-Band LNA”, IEEE Microwave and Wireless Components Letters, Vol. 25, No. 1, January 2015

[78]	Domenico Pepe1, Domenico Zito, “72 GHz CMOS LNA with Transformer-based Input Integrated Matching”, IEEE 2015

[79]	Hossein Noori, Miles Sanner, Naveen Yanduru, “A 0.8 dB NF, 4.6 dBm IIP3, 1.8 - 2.2 GHz, Low-Power LNA in 130 nm RF SOI CMOS Technology”, IEEE 2015

[80]	Joost Melai, Peter Magnée, Ivo Pouwel, Pieter Weijs, Ihor Brunets, Rob van Dalen, Anurag Vohra, Luuk Tiemeijer, Ralf Pijper, Hans Tuinhout, Nicole Wils, Nicolae Cazana, “QUBiC generation 9, a new BiCMOS process optimized for mmWave applications”, IEEE 2015

[81]	Cristina Andrei, Olof Bengtsson, Ralf Doerner, Serguei A. Chevtchenko, Wolfgang Heinrich, Matthias Rudolph, “Dynamic behaviour of a Low-Noise Amplifier GaN MMIC under input power overdrive”, Proceedings of the 45th European Microwave Conference

# 3	Definitions of terms, symbols and abbreviations

<Unchanged sections omitted>

### 4.2.5	BS aspects

#### 4.2.5.1	BS antenna arrays

More detailed antenna array modelling may be further considered in work item if needed.

BS antenna array configurations were evaluated with respect to target output power. It is assumed that each antenna array element has its own power amplifier. The loss associated with the transmission between the power amplifier and element is included in the element loss factor. The EIRP can be calculated as shown in Table 4.2.5.1-1. The examples in table 4.2.5.1-1 are provided for 8x16, 16x16 and 32x32 element arrays, and the same principle can be used to evaluate the array sizes required to reach 40 dBm and 60 dBm EIRP, which were set for targets for system evaluations according to annex A.2. The result is that reaching 40 dBm EIRP per polarization requires at least an 4x8 element array, whereas 60 dBm EIRP per polarization requires at least 16x16 element array.

It should be noted that the estimates are very conservative, given that the power amplifier study [74] as captured in clause 4.2.4 outlines the saturated power for CMOS technology between 22 and 23 dBm for 52.6 – 71 GHz frequency range. Therefore, plenty of design margin for possible front-end losses have been included.

Table 4.2.5.1-1: Example antenna arrays

|  | Unit | Antenna array examples |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Carrier frequency | GHz | 70.0 | 70 |  | 70 |
| Array size (MxN) |  | 8x16 | 16x16 |  | 32x32 |
| System Gain & Coverage Analysis |  |  |  |  |  |
| Total Conducted Power | dBm | 28.0 | 31 |  | 37 |
| Tx Arrayed Antenna Gain | dBi | 24.8 | 27.8 |  | 33.8 |
| EIRP per polarization (Note) | dBm | 52.8 | 58.8 |  | 70.8 |
| Number of antenna polarizations |  | 2 | 2 |  | 2 |
| Number of streams / polarizations |  | 1 | 1 |  | 1 |
| Transmitter Array Information (per polarization) |  |  |  |  |  |
| number of columns |  | 16 | 16 | 32 |  |
| number of rows |  | 8 | 16 | 32 |  |
| Number of antenna elements |  | 128 | 256 | 1024 |  |
| Element spacing | lambda | 0.5 | 0.5 | 0.5 |  |
| Array size, X dimension | mm | 34.3 | 34.3 | 68.5 |  |
| Array size, Y dimension | mm | 17.1 | 34.3 | 68.5 |  |
| Element directivity | dBi | 6.0 | 6 | 6 |  |
| Array factor | dB | 21.0 | 24 | 30 |  |
| Antenna directive gain | dBi | 27.0 | 30 | 36 |  |
| Element loss factor | % | 60 % | 60% | 60% |  |
| Element loss factor | dB | -2.2 | -2.2 | -2.2 |  |
| Tx Array Antenna Gain | dBi | 24.8 | 27.8 | 33.8 |  |
| Transmitter Power Amplifier Info (per polarization) |  |  |  |  |  |
| PA OP1dB | dBm | 16 | 16 |  | 16 |
| Backoff, i.e. modulated signal PAPR | dB | 9.0 | 9 |  | 9 |
| Average conducted power per PA | dBm | 7 | 7 |  | 7 |
| Power into each sub-array or antenna element | dBm | 7 | 7 |  | 7 |
| Number of Tx sub-array PAs or antenna elements |  | 128 | 256 |  | 1024 |
| Multi element or sub-array power gain | dB | 21 | 24 |  | 30 |
| Total Conducted Power per polarization | dBm | 28.0 | 31 |  | 37 |
| Note: The EIRP calculation is a simplified example and may not include all aspects of real implementations, as not identified during the study phase. |  |  |  |  |  |

Total radiated power (TRP) is calculated in dBm as: TRP=P+3-LE, where P is the total conducted power per polarization in dBm, LE is the element loss factor in dB and 3 is a factor accounting for two polarizations.

The BS antenna array examples in Table 4.2.5.1-1 are not restricted to the BS antenna array configurations and antenna array patterns in annex A.2 (i.e. System level simulations assumptions), which were defined for the purpose of performance evaluations in RAN1 and are not intended to restrict implementation freedom. The EIRP evaluations are based on an assumption that the full array is used to form a single beam. It could be possible to use the 32x32 array as four 16x16 sub-arrays, each forming a separate beam and then EIRP of each beam would be equivalent to the 16x16 array example. The examples are not intended to form a baseline for possible co-existence evaluations.

The array sizes are similar to FR2 array in 24.25 – 52.6 GHz frequency range, and therefore also the beamwidths are similar. Therefore, no need was found to start further studies on antenna array modelling and or other antenna evaluations. If there is a need, BS antenna models from TR 38.803 [73] can be re-used.

#### 4.2.5.2	Noise figure

Considering recent state-of-the-art LNA-only noise figure publications in [76 to 81], figure 4.2.5.2-1 presents the LNA-only noise figure for different semi-conductor technologies over frequency based on published data.

Figure 4.2.5.2-1: LNA-only NF trend over frequency

It can be seen that when LNA-only noise figures are compared to the typical noise figures in Table 4.2.5.2-1, there is a significant difference. This difference implies significant pre- and post-LNA losses from routing and other sources.

Referring to ETSI technical report TR 101 854 [75] on fixed radio point-to-point systems, one can find information on typical NF and associated industrial margins (IMF) values across wide range of frequencies. The IMF considers various performance variations of system elements over e.g. temperature extremes, voltage variations, or aging by capturing production spread of RF circuits.

For the 52.6 – 71 GHz range, the above NF values are in range of 10 – 13 dB.

Table 4.2.5.2-1: Typical Noise Figures (NF) and associated Industrial Margins (IMF)

| Frequency band (GHz) | Typical Noise Figure (NF) (dB) | Industrial margin (IMF) (dB) |
| --- | --- | --- |
| 48 - 50 | ~9 | +3 |
| 52 - 55 | ~10 | +3 |
| 71 - 76 | ~13 | +4 |
| 81 - 86 | ~13 | +4 |

NOTE:	The above ETSI originated Industrial Margins (IMF) shall not be confused with the Implementation Margins (IM) used in process of RAN4 conformance requirements derivation.

Referring to the system level evaluation assumptions in annex A, the BS NF was assumed as 7 dB, while UE NF was assumed as 10 dB (13 dB optionally).

Link level and system level simulation assumptions in annex A are assuming UE mobility (i.e. 3km/h), while the above referred ETRI TR treats about the fixed point-to-point applications. Still one can observe good alignment of NF values among the ETSI TR and those in this 52.6-71 GHz performance evaluation. This can be interpreted that the RF technology to be used for both static and mobile applications in 52.6-71 GHz frequency range will have similar and comparable capabilities, making the ETSI TR 101 854 [75] a valid reference for this technical report.

##### 4.2.5.3	Output power and ACLR dependencies

To initially investigate the dependencies between output power, linearity and PAE, the empirical measurements of a 28 nm FD-SOI CMOS PA from a research project was used to model the behavior at 70 GHz proxy frequency where the non-linear characteristics is kept with the output power scaled as -20 dB/decade while PAE scaled as ~-5dB /decade. CP-OFDM waveform at 400 MHz carrier bandwidth was used.

For 70 GHz proxy frequency, the example of relationship between ACLR and output power is presented in Figure 4.2.5.3-1. It should be noted that the output power is related to PA output (including package losses) and does not consider losses such as routing, switch losses etc. implying that the achievable power before the radiating elements could be lower.

As shown in Figure 4.2.5.3-1, the achievable output power decreases with increased ACLR. Figure 4.2.5.3-1 depicts the example of relation between ACLR and PAE. The examples in these figures should be considered as illustrative of the general trends, while the absolute numbers should not be derived from those examples, given that they are based on a single measurement of one component.

Figure 4.2.5.3-1: One example of ACLR versus output power (left) and PAE versus ACLR (right)

### 4.2.6	UE aspects

<End of text proposal>