3GPP TSG-RAN WG4 Meeting # 98-e 												R4-2103326

Electronic Meeting, Nov. 2020

Agenda item:			12.3.2.1.2, 12.3.2.1.3, 12.3.2.2.1, 12.3.2.2.2

Source:	Moderator (Qualcomm Incorporated)

Title:	Email discussion summary for [98e][139] NR_RF_FR2_req_enh2_Part_2

Document for:	Information

# Introduction

The topics for discussion are based on the agenda items highlighted below.

    - RF core requirements	[NR_RF_FR2_req_enh2-Core]

      - Inter-band DL CA enhancements	[NR_RF_FR2_req_enh2-Core]

        - Applicability of CBM/IBM for different CA configurations	[NR_RF_FR2_req_enh2-Core]

        - UE requirements for CA configurations CA_n258A-n260A and CA_n257A-n259A based on IBM	[NR_RF_FR2_req_enh2-Core]

        - UE requirements for CA configurations within the same frequency group based on CBM	[NR_RF_FR2_req_enh2-Core]

      - Inter-band UL CA	[NR_RF_FR2_req_enh2-Core]

        - UE requirements for CA configuration CA_n257A-n259A based on IBM	[NR_RF_FR2_req_enh2-Core]

# Topic #1:	Applicability of CBM/IBM for different DL CA configurations

Agenda item 11.3.2.1.1

## Companies’ contributions summary

| T-doc number | Title | Company | Proposals / Observations |
| --- | --- | --- | --- |
| R4-2100143 | Specification differences between IBM and CBM | Nokia, Nokia Shanghai Bell | Moved to topic #3, agenda item 11.3.2.1.3 |
| R4-2100142 | IBM RF requirements for CA configurations within same frequency group | Nokia, Nokia Shanghai Bell | Moved from topic #3, agenda item 11.3.2.1.3 Proposal 1: Current maximum input level 7.4A.3, ACS 7.5A.3 and in-band blocking 7.6A.2.3 requirements can be re-used for CA configurations where bands are within same frequency group and/or deployment assumptions which are non-collocated. Proposal 2: Rel-16 reference sensitivity requirement including power imbalance aspect can be reused for the CA configurations within same frequency group in Rel-17. ΔRIB can be discussed case by case when new CA configurations are introduced which is a RAN4 custom. Proposal 3:Rel-16 EIS spherical coverage requirement can be reused for the CA configurations within same frequency group in Rel-17. ΔRIB,S,n can be discussed case by case when new CA configurations are introduced which is a RAN4 custom. |
| R4-2100570 | Views on Applicability of CBM/IBM for different CA configurations | Sony, Ericsson | Observation 1: Since co-located is a possible deployment scenario for the same and different frequency groups, CBM can support CA configurations within the same frequency group and in different frequency groups. Observation 2: It is not feasible to limit the IBM/CBM applicability for a particular band combination in the specification since it would limit deployment flexibility (collocation or non-collocation). Observation 3: Supporting CBM UEs with different frequency groups can speed up inter-band DL CA deployment and save network resources since it does not require advanced phase shift networks on the UEs. Observation 4: Supporting IBM UE within the same band group can significantly improve network deployment flexibility. Proposal 1: Define the IBM/CBM as a UE capability for each band combination. Proposal 2: The network shall be able to configure a UE with a supported band combination according to its advertised capabilities, including the BM capability (which must be indicated for each supported band combination) in accordance with standard capability indication. Proposal 3: RAN4 concludes that CBM UEs are applicable for DL inter-band CA with both the same and different frequency groups, at least for the collocated scenarios. Observation 5: For the collocated scenario, there are options for the UE to safely switch beams before scheduling restrictions would be needed, e.g. available time in UL and DL if carriers are not always fully scheduled and during the DL-UL switch. Observation 6: A beam switch could be performed safely within the DL-UL guard (if properly performed) also for an MRTD = 3 us for the collocated scenario. |
| R4-2100598 | Applicability for CBM and IBM for FR2 inter-band CA band combos | Qualcomm Incorporated | Proposal 1: IBM based requirements will be specified for every requested band combination in TS 38.101-2. Proposal 2: IBM based requirements are band combination dependent but same requirement structure is applied for all band combinations Proposal 3: CBM based requirements are band combination dependent but same requirement structure that follows intra-band CA is applied for all band combinations |
| R4-2100636 | Discussion on applicability of CBM and IBM for different CA configurations | LG Electronics | For CBM applicability Proposal 1: CBM is applicable to inter-band CA within same frequency group rather than with difference frequency group. Proposal 2: CBM is applicable to co-located deployment. For IBM applicability Proposal 3: For inter-band CA with different frequency group, IBM is applicable as baseline. Proposal 4: For inter-band CA within same frequency group, both IBM and CBM can be applicable and UE can support either IBM or CBM. For applicability of CBM/IBM capability Proposal 5: Consider maximum frequency separation which CBM UE can support per band combination as applicability of CBM capability. For frequency separation class Proposal 6: Introduce new UE capability for frequency separation class per band combination for inter-band CA based on CBM. |
| R4-2101284 | Frequency separation class consideration for inter-band CA based on CBM | Intel Corporation | Observation 1: CBM is more applicable to collocated gNB deployment scenario. Proposal: Introduce a new optional frequency separation class capability signaling for CBM UE, such frequency separation class covers UE’s entire captured bandwidth including the gap between the bands. |
| R4-2101373 | Frequency separation class for inter-band CA within the same frequency group based on CBM | Xiaomi | Proposal 1: inter-band CA within the same frequency group could reuse the same frequency separation class with intra-band non-contiguous CA for uplink and downlink operation. Proposal 2: UE capability for the frequency separation class of inter-band CA should be introduced per band combination. Proposal 3: inter-band CA within the same frequency group with the limitation of frequency separation can support CBM applicability. |
| R4-2101756 | Discussion on Rel-17 FR2 CBM IBM | OPPO | 2.1 CBM/IBM applicability Observation 1: The only left over uncertain BM type is CBM for different freq group case. Observation 2: There is performance degradation caused by beam squint for UE with CBM but whether this is severe enough to exclude CBM is FFS. Observation 3: CBM can be supported by part of the freq ranges if not the whole band combination. Observation 4: It is more flexible for UE implementation and NW scheduling based on UE BM capabilities. Proposal 1: Including CBM for inter freq group band combinations, and it depends on UE capability to indicate which kind of BM types are supported. Proposal 2: Take equal priority for CBM/IBM and no need to define which one is baseline. 2.2 BM capability reporting Observation 5: The applicable collocated or non-collocated scenario shall be clear to NW when UE reports CBM capability. Observation 6: CBM capability itself only indicates UE has common transceiver and antenna for Pcell and Scell bands and no information about performance if scheduled under non-collocated scenario. Observation 7: Through implicit requirement definition assumptions or explicit NW deployment scenario indications, the applicability of CBM can be clear. Proposal 3: To facilitate NW scheduling, it is proposed to clearly define the NW deployment scenarios that the reported CBM capability can be applied. Observation 8: Freq range based CBM capability reporting is more flexible and maximum UE performance, meanwhile the complexity of NW scheduling and specification definition would be comparably high. Proposal 4: It is proposed to define CBM UE capability based on the freq range approach. |
| R4-2102182 | Discussion on FR2 Inter-band DL CA enhancements | ZTE Corporation | Observation: Current TS38.104 BS OTA TAE requirement cannot satisfy that assumption for the co-located deployment for FR2 inter-band CA. Proposal 1. To compliance to CBM UE capability for inter-band DL CA, both TS38.133 and TS38.104 (OTA TAE requirements) need to be discussed/updated accordingly. Proposal 2. It should not restrict the CBM only support CA configurations within same frequency group in the specification. Proposal 3. IBM should be the baseline for all inter-band DL CA combinations. Proposal 4. The same IBM requirements apply to all CA configurations for the same band combination. However, the IBM requirements may not the same between L-L/H-H and L-H band combination. |
| R4-2102511 | Discussion on the CBM/IBM applicability of Rel-17 FR2 inter-band CA | Google Inc. | Proposal 1: The CBM can only support CA configuration within same frequency group. Proposal 2: IBM should be the baseline for those inter band CA combinations which are recognized as non-CBM band combinations. Proposal 3: If proposal 1 is not acceptable, it is proposed to determine the specific Fs,inter value to differentiate CBM from IBM for all inter-band CA combinations in same frequency group. Proposal 4: Do not introduce the new frequency separation class per band per receiving chain for inter-band CA based on CBM or IBM. |
| R4-2102676 | on CBM and IBM for FR2 inter-band DL CA | Huawei, HiSilicon | Observation1: Without performance degradation allowance, “BCs within the same freq. group based on CBM” is not applicable. Proposal 1: RAN4 also study applicability on “BCs within the same freq. group based on CBM”, 2 options are provided: - Option 1: Accept demodulation performance degradation for L+L/H+H band combinations with CBM type, and make clarification into RAN4 spec. - Option 2: Confirm “BCs within the same freq. group based on CBM” is not applicable. Proposal 2: RAN4 agrees to clearly define CBM/IBM in TS 38.101-2, it can be defined as: Independent beam management (IBM): UE select a suitable DL beam for CCs across bands based on DL measurements on both Bands. Common beam management (CBM): UE select a suitable DL beam for CCs across bands based on DL measurements only on one of the Bands. Proposal 3: BCs in different freq. group with CBM type is applicable, and there is demodulation performance degradation. Observation 2: CA configuration with CBM type can support non-collocated deployment considering half power beam width of gNB and UE. Proposal 4: Clarify in RAN4 spec that CBM type can support non-collocated deployment with possible demodulation performance degradation. Proposal 5: IBM/CBM is indicated per band combination as an UE capability, there is no limitation on feasible BM type for each band configuration. Proposal 6: Separation class extends to be indicated per band combination for L+L and H+H CA combinations with CBM type. Observation 3: If separation class per RF chain can be introduced, DL-only separation class UE capability is enabled or removed |

## Open issues summary

### Sub-topic 1-1

CBM definition shall be introduced in TS38.101-2:

- UE select a suitable DL beam for CCs across bands based on DL measurements only on one of the Bands

### Sub-topic 1-2

IBM definition shall be introduced in TS38.101-2:

- UE select a suitable DL beam for CCs across bands based on DL measurements on both Bands

### Sub-topic 1-3

IBM inter-CA requirement framework established for n260+n261 shall be applied to any requested CA band pair (parameter values discussed separately)

### Sub-topic 1-4

UEs shall be allowed to declare support for any band pair with CBM:

- With no restrictions

- Only if RAN4 spec acknowledges demodulation performance degradation

- Some restrictions (frequency group, co-location of gNB)

### Sub-topic 1-5

Discussion on Fs_inter as a capability declaration per CBM band pair.

(1-5-1:) Motivation for Fs_inter

(1-5-2:) Fs_inter should be introduced to allow an additional limitation on UE capability

(1-5-3:) Fs_inter is optional

(1-5-4:) Fs_inter only applies to band pairs in same band group only

## Companies views’ collection for 1st round

### Open issues

| Issue | Options | Company Comments |
| --- | --- | --- |
| 1-1: CBM definition | not necessary to be introduced in spec |  |
|  | UE select a suitable DL beam for CCs across bands based on DL measurements only on one of the Bands | Qualcomm: We support introduction of definition. Wording improvement suggestion below. LG Electronics : Generally Ok to introduce definition ‘CBM UE’. Need more discussion on wording. ZTE: support to introduce the definition. Open to the wordings. Xiaomi: support introduce the definition of ‘CBM’, the wording need further discuss. Vivo: Generally fine, but suggest to refine wording, e.g. using “DL Rx beam” instead of “DL beam”. OPPO: Ok to introduce definition ‘CBM. Wording needs refinement. Samsung: support to introduce, but better to define in definition section (clause 3.1) for easy alignment among specs Sony: we agree to introduce the definition of CBM/IBM in the specification. Ericsson: a definition of the CBM in the RAN4 specification could be introduced for the purpose of defining minimum requirements, DL beams based on UE measurements on one band. The UE behavior is not specified in the RAN4 specifications. Nokia: Support addition of definition, wording proposal: A UE that supports inter-band CA with CBM selects its RX beam(s) for the DL beam(s) provided by the gNB for all CCs in all bands based on DL measurements made in the only band configured with the reference signal. Apple: Ok to introduce a definition for CBM. The exact wording needs further discussion. Verizon: We support introduction of definition. |
|  | Other definition (please include) | Qualcomm: minor wording changes to proposal: A UE that supports inter-band CA with CBM selects its DL beam(s) for all CCs in all bands based on DL measurements made in the only band configured with the reference signal. Xiaomi: whether it implies CBM just applies to co-located deployment. Vivo：Consider ”DL Rx Beam” rather than “DL beam”. Huawei: Further refinement based on QC’s: A UE that supports inter-band CA with CBM selects its DL Rx beam(s) for all CCs in all bands based on DL measurements made in the only band configured with the reference signal for beam management. |
| 1-2: IBM definition | not necessary to be introduced in spec |  |
|  | UE select a suitable DL beam for CCs across bands based on DL measurements on both Bands | MediaTek: A clarification question based on n261+n260 example: Does the proposed text means UE selects one n261 beam based on n261 DL measurements, and also selects one n260 beam based on n260 DL measurement. Qualcomm: We support introduction of definition. Wording improvement suggestion below. LG Electronics : Generally Ok to introduce definition ‘IBM UE’. Need more discussion on wording. ZTE: support to introduce the definition. Open to the wordings. Xiaomi: support introduce the definition of ‘IBM’, the wording need further discuss. vivo: ok with this definition. Minor adjustment may be needed similar to previous one. OPPO: Ok to introduce definition ‘IBM. Wording needs refinement. Samsung: support to introduce, but better to define in definition section (clause 3.1) for easy alignment among specs Sony: we agree to introduce the definition of CBM/IBM in the specification. Ericsson: a definition of the IBM in the RAN4 specification could be introduced for the purpose of defining minimum requirements, DL beams based on UE measurements on both bands. The UE behavior is not specified in the RAN4 specifications. Nokia: Support addition of definition, wording proposal A UE that supports inter-band CA with IBM selects its RX beam(s) for the DL beam(s) provided by the gNB for all CCs in each band based on DL measurements made in that band. DOCOMO: We support introduction of definition. Also, we support wording revised by Qualcomm. Apple: Ok to introduce a definition for IBM. The exact wording needs further discussion. Verizon: We support introduction of definition. |
|  | Other definition (please include) | Qualcomm: minor wording changes to proposal: - A UE that supports inter-band CA with IBM selects its DL beam(s) for all CCs in each band based on DL measurements made in that band. - Huawei: A UE that supports inter-band CA with IBM selects its DL Rx beam(s) for all CCs across bands based on DL measurements on both Bands. I think it seems better? |
| 1-3: IBM inter-CA requirement framework established for n260+n261 shall be applied to any requested CA band pair (parameter values discussed separately) | Agree | Qualcomm: agree OPPO: Ok with IBM at least. Xiaomi: support IBM inter-band CA could reuse the same framework with that established for n260+n261 vivo: discussion for per band combination is needed. Huawei: Agree. Applicable for both IBM and CBM. Sony: support to define the framework according to n260+n261. Ericsson: acceptable if this is the requirement framework (actual requirements vary). Nokia Agree Verizon: Agree! |
|  | Disagree | LG Electronics : Some requested CA band pair can be applicable CBM UE. Need more discussion for CBM. ZTE: At least, n260+n261 IBM requirement framework can be applied any CA band pair with IBM capability. For CBM capability combination, it may premature to adopt the IBM requirement framework. (seems it is discussing in topic #3 whether or not intra-band DL CA framework is applied for CBM requirement... ) Samsung: agree to reuse the framework for other IBM across different frequency group, further discussion is needed to apply for IBM within same frequency group. Apple: Proposal needs clarification. The IBM inter-band framework cannot apply to any requested CA band pair, for example to CBM inter-band CA. |
| 1-4: UEs shall be allowed to declare support for any band pair with CBM: | With no restrictions | Ericsson: there should be no restrictions in the CBM/IBM indication even though RAN4 only specifies requirements for specific test case. CBM can be feasible for collocation across “frequency ranges” in the field. Configuration is up to the network. The network should not have to look in RAN4 tables to find out if there is a requirement in order to figure out the UE capability. Nokia: No restrictions up to UE vendor to decide. Of course requirements need to be developed such are relaxation of REFSENS as a function of frequency separation. Verizon: No restriction! |
|  | Only if RAN4 spec acknowledges demodulation performance degradation | Huawei: generally there should be no restriction on indicating supporting of CBM. But for CBM, different freq group or same freq group, there is performance degradation, we need to state this in RAN4 spec. but there is no restriction on signaling. |
|  | Frequency group restriction | Qualcomm: RAN4 proposals to treat CBM inter-CA like extended intra-band is based on the reasoning that UEs can support close-by bands by extending the bandwidths of the signals it can receive. If CBM across band groups must be pursued, the justification to treat inter-CA CBM like intra-CA goes away. So, we may have to come up with a new set of requirements for UEs that support CA across different band groups using CBM. Example of problem with extending CBM to band pairs that are not touching or overlapping: Beam squint can become significant for 28+43 (17 GHz span) CBM. This means that even for co-located deployments, CBM UEs will feature mis-aimed beams and result in system interference. This problem can become worse for UEs with higher gain beams like PC1. ZTE: If there are new bands introduced in future, how to group this bands? Grouped it in existing frequency group or new frequency group? Vivo: There is possible considerable performance degradation (R4-2102714) for different frequency group restriction. This problem can be discussed with Fs,inter. Samsung: agree with Qualcomm that CBM shall be treated more like intra-band CA, so there should be some restriction on bands’ frequency DOCOMO: UEs shall be allowed to declare support for same freq. group with CBM. If UEs support different freq. group with CBM, we should make it clear that there is an advantage to use even if the performance might be is worse than IBM. Verizon: We agree to specify frequency group(s) based on the agreed degradation level(s). |
|  | other restriction | MediaTek: Only if the UE meet CBM relative requirements. What’s CBM relative requirement is FFS. LG Electronics: CBM needs to be applicable to only co-located deployment. ZTE: At least, CBM should be restricted in co-located scenario. Xiaomi: prefer to CBM only apply to co-located deployment. vivo: the deployment scenario restriction is needed. Our simulation shows CBM degradation is huge even using the wide Rx beam under non-co-located deployment. In our understanding, CBM can only work under co-located deployment. OPPO: Ok with this assumption. Regarding the deployments should be clear whether this UE can support or not to NW. Sony: support that UEs shall be allowed to declare support for any band pair with CBM, as long as it meets the CBM requirements if there would be any. Apple: CBM should be limited to co-located scenario within the same frequency group. |
| 1-5-1: Fs_inter motivation | Gain drop may be considered as not acceptable for large frequency separation | Qualcomm: We appreciate this problem, but this is already addressed in Refsens relaxations in intra-band, and is proposed to be introduced for CBM inter-CA LG Electronics : Support ZTE: Agree. Xiaomi: support introduce Fs_inter for CBM inter-band CA, based on Fs_inter, we don’t need distinguish whether the pair bands within same frequency group or between different frequency groups, only if they can meet the frequency separation for CBM, it can be applied for CBM. OPPO: ok with the concept of Fs_inter, how to connect this concept with the gain drop can be discussed further. Huawei: what is the difference between Fs_inter and frequency separation span? Nokia: Can be handled with REFSENS relaxation. Introduction of Fs is a burden for Network. Apple: We agree that for the CBM inter-band CA additional gain drop is expected, and therefore further relaxation needs to be studied. |
|  | UE cannot be physically configured for CCs separated by more than FS_inter |  |
|  | Other | vivo: both of above. Furthermore, introduce Fs,inter can expand the applicability of CBM, It may also helpful for operators to plan the use of spectrum Sony: we recommend focusing on the applicability of CBM first and then start to discuss whether to introduce the frequency separation class. Ericsson: CBM/IBM specification should be completed first, then restrictions, if any, can be introduced. Verizon: agree with Ericsson! |
| 1-5-2: Fs_inter should be introduced to allow an additional limitation on UE capability | Agree | LG Electronics : vivo:simulation shows even for same freq. group, the CBM performance degradation may considerable for some case. |
|  | Disagree | Qualcomm: Refsens relaxations can capture performance degradation gain drop due to large frequency separation, so there has to be some other motivation for Fs_inter Is the intent to allow less or no relaxation for UEs that only support partial inter-band capability? ZTE: We have same view with QC. It can be reflected in gain drop, i.e. reference degradation. Also, if Fs_inter is too large to cause large gain drop, does it mean the CBM capability cannot be used? It seems like the FR1-FR1 NR CA combination with large MSD values. Xiaomi: what is the additional limitation on UE capability? OPPO: Need to clarifiy whether this is the freq gap for gain drop already defined in RAN4. Huawei: how gNB use this UE capability? Nokia: Can be handled with REFSENS relaxation. Introduction of Fs is a burden for Network. Apple: We think further relaxation is required but we don’t think that an additional capability is necessary. |
| 1-5-3: Fs_inter optionality | Optional | OPPO: This can be further discussed once it is agreed to be introduced. For now our understanding is that it is optional. |
|  | Mandatory | LG Electonics Xiaomi vivo: if we try to introduce this parameter to restrict the CBM, it should be mandatory. |
| 1-5-4: Fs_inter applicability | CBM band pairs in same band group only |  |
|  | Any CBM band pairs but not IBM pairs | LG Electronics : Support Xiaomi vivo: In our understanding, Fs,inter is to ensure acceptable performance degradation for CBM. OPPO |
|  | Any band pair |  |

### CRs/TPs comments collection

N/A.

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

| Issue | Options | Status Summary |
| --- | --- | --- |
| 1-1: CBM definition | There is agreement that the standard would benefit from introduction of a definition. This definition seems to have good support: A UE that supports inter-band CA with CBM selects its DL Rx beam(s) for all CCs in all bands based on DL measurements made in the only band configured with the reference signal for beam management. | Capture in CBM WF after 2nd round discussion |
| 1-2: IBM definition | There is agreement that the standard would benefit from introduction of a definition. Two options to create definition - A UE that supports inter-band CA with IBM selects its DL beam(s) for all CCs in each band based on DL measurements made in that band. - A UE that supports inter-band CA with IBM selects its DL Rx beam(s) for all CCs across bands based on DL measurements on both Bands. | Capture in IBM WF after 2nd round discussion |
| 1-3: IBM inter-CA requirement framework established for n260+n261 shall be applied to any requested CA band pair (parameter values discussed separately) | Agree: Qualcomm, Oppo, Xiaomi, Vivo, Huawei, Sony, Nokia, Verizon, ZTE, Samsung (different frequency group only) Disagree: Samsung (same frequency group only) | IBM WF discussion required |
| 1-4: UEs shall be allowed to declare support for any band pair with CBM: | With no restrictions: Ericsson, Nokia, Verizon, Huawei (*) * Only if RAN4 spec acknowledges demodulation performance degradation | CBM WF discussion required |
|  | Frequency group restriction: Qualcomm, ZTE, Vivo, Samsung, Docomo, Verizon, Apple |  |
|  | Restriction on network (co-location): LGE, ZTE, Vivo, Sony, Apple Unspecified: MediaTek |  |
| 1-5: Fs_inter (updated for round 2, highlighted company believes question was misleading and have asked to capture revised stand) | Support introduction of Fs_inter for any band pair: Mandatory: Vivo, LGE, Xiaomi, Huawei Optional: Oppo | There does not seem to consensus to introduce Fs_inter before CBM requirements are put in place, with more companies against than for. No further action this meeting. |
|  | Fs_inter not needed (see issue 3-3): Qualcomm, Nokia, ZTE, Huawei, Apple Not needed immediately: Sony, Ericsson, Verizon |  |

Summary imported from [138] with revised 2nd round objectives:

| Issue | Status summary |
| --- | --- |
| Issue 2-1-1: Beam management for CA within same frequency group | Tentative agreements: RAN4 will not label CBM or IBM as a default BM method for any band combination. Used BM is based on UE capability. This issue is not discussed anymore in RAN4. Recommendations for 2nd round: Capture agreement in IBM WF. |
| Issue 2-1-2: Beam management for CA across different frequency groups | Recommendations for 2nd round: continue in WF discussions |

Recommendations on WF/LS assignment

(none)

### CRs/TPs

(none)

## Discussion on 2nd round (if applicable)

| Issue | Options | Company Comments |
| --- | --- | --- |
| 1-4: UEs shall be allowed to declare support for any band pair with CBM: | Re: Frequency group restriction ZTE: If there are new bands introduced in future, how to group this bands? Grouped it in existing frequency group or new frequency group? | MediaTek: In our understanding, current frequency group only on behalf of existed band. i.e. 28GHz group = n257, n258, n261 39GHz group = n259, n260 For any new bands introduced in the future, RAN4 shall discuss them as new situations. Hence, RAN4 may group it in existing frequency group or new frequency group based on technical judgement at that time. ZTE: We are fine with the current frequency group, but we are not sure what the technical judgement is. We thought there existed a simple way to judge, such as 28GHz group is in the scope <f1, and 39GHz group is in the scope of [f2, f3] where f1, f2, f3 are FFS. vivo: This issue was also discussed in previous meetings. In our understanding, there is actually no clear boundary for different freq. groups. Although introducing this concept now may help the discussion, considering the introduction of new bands in the future, this vague definition may make the problem more complicated. Samsung: frequency group is used for convenience of discussion, there will be no explicit group to be defined with dedicated bands. In our understanding, how to judge same/different frequency group depends on detailed band combination case by case. Sony: As we have commented in the 1st round, we think UE should be allowed to declare support for any band pair with CBM, regardless the frequency group. Therefore, we can avoid the ambiguity and forward compatibility issue of frequency group if we adopt this proposal. Ericsson: the UE shall be able to declare CBM for any supported band combination regardless of the ‘frequency group’, it is up to the network to configure the UE according to its advertised capabilities for the particular deployment scenario. The network should not have to look in RAN4 specification to find what the UE can support for a band combination. Google: Similar view to Samsung. The rule to differentiate frequency group for the new bands can be discussed case by case. |
| 1-5-1: Fs_inter motivation | Re: Gain drop may be considered as not acceptable for large frequency separation Huawei: what is the difference between Fs_inter and frequency separation span? | LG Electronics : Our understanding is that Fs_inter is frequency separation class and frequency separation span is difference between lower edge of lowest CC and upper edge of highest CC in FR2 inter-band CA. Fs_inter is to indicate the maximum frequency span. It is similar to FR2 intra-band NC CA in Rel-16. MediaTek: In our view, Fs_inter_CBM is “frequency span limitation for inter-band CA based on CBM”. (# We add “_CBM” to make the applicability clearer.) We think the role of “Fs_inter_CBM” is to define the largest frequency span that the UE can support. For example, if the UE can support Fs_inter_CBM = 4GHz, and also can support n257+n258. And then, the active CC frequency span of n257+n258 shall be NOT wider than 4GHz. For example, 25.5-29.5GHz and 24.5-28.5GHz are supported, but 24.25-29.5 GHz (> 4GHz) is not supported. Furthermore, extra REFSENS relaxation based on different Fs_inter_CBM for specific band combinations shall be defined. Qualcomm: What is intended meaning of ‘frequency separation span’? Perhaps you mean ‘Configured DL spectrum’? (see below). If so, Fs_inter would be a capability and per Rel-16 ‘The configured downlink spectrum is defined as the frequency band from the lowest edge of the lowest CC to the upper edge of the highest CC of all UL and DL configured CCs.’ The relaxations were not intended to be capability based, but configuration based, for continuity with intra-band CA. vivo: similar to MTK’ view. We propose this parameter to extend the applicability of CBM when it is difficult for CBM to completely cover two bands, considering that the frequency span of FR2 band is very large. At the same time, this parameter can also facilitate us to judge whether a specific band pair and RB allocation can support CBM. Samsung: we have no strong view but would like to confirm our understanding: so if Fs_inter is introduced as UE capability, and EIS relaxation is depending on configured DL spectrum, then there will be many frequency in lower part of lower band and higher part of higher band actually does not support inter-band CA. it seems like different situation as intra-band CA. Huawei: If Fs,inter means the separation class UE capability, why in the 1st round discussion, we say the motivation for Fs,inter is for refsens relaxation, refsens relaxation is based on configured separation span. They are totally different concept, we don’t understand why we discuss Fs,inter(UE capability) and configured separation in one issue. Nokia: We would not like to introduce new UE capability for this. Instead we can just update the Table 7.3A.2.2-1: EIS Relaxation for CA operation to cover larger separation cases. Google: Similar view to Samsung. |
| 1-5-2: Fs_inter should be introduced to allow an additional limitation on UE capability | Qualcomm: Refsens relaxations can capture performance degradation gain drop due to large frequency separation, so there has to be some other motivation for Fs_inter Is the intent to allow less or no relaxation for UEs that only support partial inter-band capability? | LG Electronics : In Rel-16, 9 Frequency separation classes (FSCs) were specified from 800MHz to 2400MHz in Table 5.3A.4-2 for intra-band NC CA to indicate the maximum frequency span between lower edge of lowest component carrier and upper edge of highest component carrier. And, 3 values of EIS relaxation were also specified from 800MHz to 2400MHz in Table 7.3A.2.2-1 for the intra-band NC CA. For example, same EIS relaxation of 0.5dB applies to UE capable of FSC of 2 and UE capable of FSC of 3. However, In aspect of UE implementation, some UEs can be implemented with FSC of 2 but not FSC of 3. It means FSC is needed to distinguish the UE capability regardless of same EIS requirement. Same principle needs to be applied to CBM UE in FR2 inter-band CA. Because, in aspect of UE implementation, the CBM UE can be similar to FR2 intra-band NC CA. Table 5.3A.4-2: Frequency separation classes for non-contiguous intra-band operation Frequency separation class; Max. allowed frequency separation (Fs). I; 800 MHz. II; 1200 MHz. III; 1400 MHz. IV; 1000 MHz. V; 1600 MHz. VI; 1800 MHz. VII; 2000 MHz. VIII; 2200 MHz. IX; 2400 MHz. NOTE 1: Fs values larger than 1400 MHz apply only to downlink frequency separation.. Table 7.3A.2.2-1: EIS Relaxation for CA operation Configured DL spectrum (MHz); (dB). ≤ 800; 0.0. > 800 and ≤ 1400; 0.5. > 1400 and ≤ 2400; 1.5. MediaTek: We think “Fs_inter_CBM” is to indicate what frequency span CAN be configured, and the corresponding relaxation shall be further defined, such as EIS relaxation. (# We add “_CBM” to make the applicability clearer.) Qualcomm: The example refsens relaxation is available not as a function of capability, but as function of configuration (if CCs are configured far apart). So we do not see the argument that FSC is needed to ‘to distinguish the UE capability regardless of same EIS requirement’. How does having Fs_inter as a new capability help?. ZTE: The CBM itself is the UE capability, when UE report the CBM capability for a band combination, then the requirements including REFSEN relaxations needs to be defined. We are not sure why it is needed to define additional capability(Fs_inter)? if the REFSEN degradation dramatically, then UE has choice not to report CBM. vivo: Fs,inter limits the MAX span between two CCs in a specific band combination under CBM. If the span exceeds this distance, it is considered that the performance degradation of CBM or the complexity of UE implementation is unacceptable. Other requirements can be defined based on this. We can also understand the concerns that this restriction might seems redundant in the sense that it is also related to per-band pair support of CBM. However, Fs,inter can be more flexible in that certain RB allocation combination within a band pair may also be evaluated. Since FR2 band can be so wide, this might be benficial. Huawei: To QC, the capability is used to inform the gNB how large span could be configured to the UE, the similar as we introduce separation class for intra-band NC CA. it has no relation with refsens relaxation. As you commented, refsens relax is defined with configured span, but Fs,inter is about UE capability that maximum span the UE supports. Motivation on Fs,inter discussion in 1st round is misleading. Sony: We share a similar understanding as ZTE, we don’t see a need to define additional capability Fs_inter here. However, we can define the requirement of CBM with a performance relaxation, where the performance relaxation can be derived based a maximum allowed frequency separation. But again, it does not require to introduce any additional capability upon CBM. A device can meet such CBM requirement on any band pair should be allowed to declare CBM capability. Ericsson: we see no need for a separate incapability for this case. |
|  | Xiaomi: what is the additional limitation on UE capability? | LG Electronics : It is to allow maximum frequency span between aggregated CCs for CBM UE. For example, UE needs to inform CBM UE capability and additional FSC(Fs_inter) as well. Qualcomm: The definition of inter-band CA capability (between bands 1 and 2) means the UE can be configured with a CC anywhere in band 1 along another CC anywhere in band 2. Fs_inter is an incapability that tells the network the UE cannot support inter-band CA, but can support ‘partial inter-band’ CA Xiaomi: I see, thanks, I misunderstood this issue as first. Vivo: Reference to previous answer. Huawei: people show misunderstood on this Fs,inter discussion in the 1st round, not only us. Could moderator capture something as the outcome of this meeting? Nokia: We would not like to introduce new UE capability for this. Instead we can just update the Table 7.3A.2.2-1: EIS Relaxation for CA operation to cover larger separation cases. |
|  | Huawei: how gNB use this UE capability? | LG Electronics : gNB can use this UE capability for CBM UE supporting FR2 inter-band CA as FR2 intra-band NC CA in Rel-16. Qualcomm: Fs_inter is technically an incapability or limitation. It has to ensure all desired CCs can fall inside Fs_inter prior to configuring the UE accordingly. Vivo: gNB can do evaluation between two CCs, and different results may apply for the same band pair since the band can be very wide. Huawei: We already understand the abbreviation Fs,inter. And our position is support to introduce separation class UE capability for inter-band CA within the same frequency group. |

## Summary on 2nd round (if applicable)

This section contains no CRs, TPs or WFs. Below is moderator’s summary of 2nd round discussion that may be used by companies for future submissions.

| Issue | Options | Company Comments |
| --- | --- | --- |
| 1-4: UEs shall be allowed to declare support for any band pair with CBM: | Re: Frequency group restriction ZTE: If there are new bands introduced in future, how to group this bands? Grouped it in existing frequency group or new frequency group? | No consensus on whether there should be limitation on which band pairs can be supported with CBM Some companies believed it would be judged case by case (2/7), others believed the topic moot because there ought not to be any restriction (2/7) and some believed band groups as defined today will continue to serve, even if only in informal capacity (2/7). One company thought that band groups could make things more complicated in the future. |
| 1-5-1: Fs_inter motivation | Re: Gain drop may be considered as not acceptable for large frequency separation Huawei: what is the difference between Fs_inter and frequency separation span? | Common view was that Fs_inter is a capability and ‘frequency separation span’ was a configuration. 5 companies believed that one was capability, and one was configuration. 1 company believed REFSENS relaxations were by capability while 3 believed it was by configuration. Finally the company that asked for clarification disapproved of the moderator: ‘why in the 1st round discussion, we say the motivation for Fs,inter is for refsens relaxation, refsens relaxation is based on configured separation span. They are totally different concept’ Note: Contribution R4-2101284 identified gain droop as motivation for Fs_inter. 1st round discussion was framed to get more opinions on gain droop as a motivation for Fs_inter. |
| 1-5-2: Fs_inter should be introduced to allow an additional limitation on UE capability | Qualcomm: Refsens relaxations can capture performance degradation gain drop due to large frequency separation, so there has to be some other motivation for Fs_inter Is the intent to allow less or no relaxation for UEs that only support partial inter-band capability? | There was no consensus that Fs_inter should be introduced. 3 companies believed Fs_inter is required or beneficial in addition to REFSENS relaxations. Some believed relaxations could be a function of Fs_inter also. 1 company mentioned Fs_inter not related to REFSENS relaxations 5 companies believed Fs_inter is NOT required to be defined. Some suggested REFSENS relaxations could suffice. |
|  | Xiaomi: what is the additional limitation on UE capability? | 4 companies agreed that Fs_inter was similar to FS in intra-band 1 company believes moderator caused widespread confusion: Huawei: people show misunderstood on this Fs,inter discussion in the 1st round, not only us. Could moderator capture something as the outcome of this meeting? |
|  | Huawei: how gNB use this UE capability? | General agreement that gNB evaluates UE capability with desired inter-band configuration. |

# Topic #2:	UE requirements for DL CA configurations CA_n258A-n260A and CA_n257A-n259A based on IBM

Agenda item 11.3.2.1.2

## Companies’ contributions summary

| T-doc number | Title | Company | Proposals / Observations |
| --- | --- | --- | --- |
| R4-2100620 | RF specifications for DLCA n260A_n258A and n259A_n257A based on IBM | Qualcomm Incorporated | Proposal 1: Apply same ΔRIB,P,n and ΔRIB,S,n for CA_ n260A_n258A as for CA_ n260A_n261A for power class 3 Proposal 2: Apply same ΔRIB,P,n and ΔRIB,S,n for CA_ n259A_n257A as for CA_ n260A_n261A for power class 3 Proposal 3: Use same maximum input level, ACS and in-band blocking requirements for CA_ n260A_n258A as for CA_ n260A_n261A. Proposal 4: Use same maximum input level, ACS and in-band blocking requirements for CA_ n259A_n257A as for CA_ n260A_n261A. |
| R4-2100695 | Proposals on PC3 RIB of CA_n258A-n260A and CA_n257A-n259A | MediaTek Beijing Inc. | Proposal1: ΔRIB difference shall at least reflect UE multi-band relaxation factor difference. Proposal2: PC3 ΔRIB of CA_n258A-n260A and CA_n257A-n259A based on IBM shall be not less than below values: For CA_n258A-n260A: NR CA bands; NR band; ΔRIB,P,n (dB). CA_n258-n260; n258; 4.1. n260; 4.0. NR CA bands; NR band; ΔRIB,S,n (dB). CA_n258-n260; n258; 4.2. n260; 3.2. For CA_n257A-n259A: NR CA bands; NR band; ΔRIB,P,n (dB). CA_n257-n259; n257; 4.2. n259; 4.0. NR CA bands; NR band; ΔRIB,S,n (dB). CA_n257-n259; n257; 4.2. n259; 3.2. |
| R4-2100747 | FR2 inter-band CA for different frequency band groups with IBM | Nokia, Nokia Shanghai Bell | Observation 1: The IBM requirement for n260+n261 is the baseline for other FR2 band combinations between the low and high frequency band group based on IBM. Proposal 1: The relaxation values of FR2 inter-band CA between the low band group (n257, n258 or n261) and high band group (n259 and n260) for IBM requirement are the same as the ones for CA_n260-n261, i.e., 3.5 dB for all these bands. Proposal 2: For the FR2 inter-band DL CA, maximum input level, ACS, and in-band blocking requirement are band combination agnostic and shall be reused for any new FR2 inter-band combinations. Proposal 3: CA band combination in Proposal 1 shall be treated in the Rel-17 CA basket work item by following the standard block approval process. Observation 2: 47 GHz band can be a new frequency group other than 28 GHz (low band) and 39 GHz (high band) groups. |
| R4-2101199 | Band specific requirements for FR2 DL Inter-band CA of n257n259 with IBM | NTT DOCOMO INC. | Proposal 1: For ΔRIB,P,n and ΔRIB,S,n, the same relaxation values with n260+n261 should be applied to n257+n259. Observation 1: Inter-band CA including intra-band contiguous CA with maximum number of bands is 2 bands and maximum number of CCs is 12 CCs have been requested by several companies and already captured in a related WID [4] Observation 2: Rel-16 core specification (clause 7.3A.2.3 in TS 38.101-2 v16.6.0) seems to supports inter-band CA including intra-band CA. “For the combination of intra-band and inter-band carrier aggregation, the intra-band CA relaxation, ΔRIB, is also applied according to the clause 7.3A.2.1 and 7.3A.2.2.” Proposal 2: Apply release independent approach from Rel-16 to inter-band CA including intra-band contiguous CA with maximum number of bands is 2 bands and maximum number of CCs is 12 CCs. |
| R4-2102189 | Discussion on UE requirements for CA configurations based on IBM | ZTE Corporation | Proposal 1. Same RF Requirements for CA_n260A-n261A can be applied for CA_n258A-n260A and CA_n257A-n259A based on IBM. Proposal 2. General framework would be necessary for FR2 inter-band CA band combination. |
| R4-2102607 | CA_n258A-n260A and CA_n257A-n259A based on IBM | Apple | Observation 1: The RF requirements for CA_n260A-n261A may potentially be applicable to CA_n258A-n260A. Observation 2: The RF requirements for CA_n260A-n261A may not be applicable to CA_n257A-n259A which could be subjected to further performance degradation. |

## Open issues summary

### Sub-topic 2-1

(2-1-1:) Agree/disagree: IBM inter-band CA requirements shall be specified not per band pair, but per CA between band groups, for example: L+H, L+L, etc

(2-1-2:) Agree/disagree: The relaxation values of FR2 inter-band CA between the low band group (n257, n258 or n261) and high band group (n259 and n260) for IBM requirement are the same as the ones for CA_n260-n261, i.e., 3.5 dB for all these bands.

### Sub-topic 2-2

Agree/disagree: For the FR2 inter-band DL CA, maximum input level, ACS, and in-band blocking requirement are band combination agnostic and shall be reused for any new FR2 inter-band combinations.

### Sub-topic 2-3

Agree/disagree: Apply release independent approach from Rel-16 to inter-band CA including intra-band contiguous CA with maximum number of bands is 2 bands and maximum number of CCs is 12 CCs.

## Companies views’ collection for 1st round

### Open issues

| Issue | Options | Company Comments |
| --- | --- | --- |
| 2-1-1: IBM inter-band CA requirements shall be specified not per band pair, but per CA between band groups, for example: L+H, L+L, etc | Agree |  |
|  | Disagree | MediaTek: We shall discuss it per band pair, because of so many link budget component are frequency dependent. Qualcomm: we are open to this idea as a ‘default’ specification or safety net for cases when specific band pairs have not been evaluated and specified, but the combination is included in the standard. When requirements for a band pair are explicitly specified, it would supersede the proposed default specification. LG Electronics : RAN4 already agreed not to specify frequency band group. So, the requirements should be specified per band pair which needs to be indicated with IBM or CBM. ZTE: The requirements should be defined per band combination. However, we think general framework/requirements would be beneficial for the limited band combination, otherwise it should be needed discussed case by case which is not effective to our understanding. Also if there are some exceptions, separate discussion will be needed Xiaomi: support define the IBM requirements for inter-band CA case by case. vivo: Not all requirements can be treated like this, discussion on per band combination maybe better. OPPO: Should be per band pair rather than per freq group. Huawei: we always have general requirement and also BC specific RF requirement. We would like to know: what is the difference between CBM and IBM general RF requirement? Samsung: it has been agreed that frequency group term will not exist in specification Sony: disagree Ericsson: not agreed. Nokia: It may be that there will be some “rules” how requirements are done like we have for example for LTE CA dTib/dRib but requirements itself will made for individual CA configurations. There is also agreement not to define frequency group in RAN4. DOCOMO: We agree to reuse the requirement except for relaxation. While we agree that the relaxation values for n260+261 are reused for these for n257+259, however, it may be better to study the value for each band combination at this stage though it may be easy in the specification procedure to make the relaxation values common per CA between band groups. Apple: We support to specify IBM requirements for different band groups only, but the IBM requirements should be discussed case-by-case. |
| 2-1-2: The relaxation values of FR2 inter-band CA between the low band group (n257, n258 or n261) and high band group (n259 and n260) for IBM requirement are the same as the ones for CA_n260-n261, i.e., 3.5 dB for all these bands | Agree | Qualcomm: Our analysis shows 3.5dB can be supported for all the listed band combos, for both peak and spherical ZTE: Agree Sony : support Ericsson: agreed. Nokia: Agree DOCOMO: We support applying the same values. Apple: Ok to consider 3.5 dB relaxation for the combinations n257-n259 and n258-n260. We would like to note that the 3.5 dB relaxation will not be assumed for low band + high bands in general, and that RAN4 needs to study the relaxation for the CA case-by-case. |
|  | Disagree | MediaTek: Disagree. We have detailed proposals for different band combinations in R4-2100695. LG Electronics : The relaxation values can be different depending on separation between aggregated bands. OPPO: Relaxation should be based on the framework used in CA_n260-n261 rather than whether it is enough or not. Huawei: relaxation requriemnt is BC specific requirement, it should be discussed per Band pair |
| 2-2: For the FR2 inter-band DL CA, maximum input level, ACS, and in-band blocking requirement are band combination agnostic and shall be reused for any new FR2 inter-band combinations | Agree | Qualcomm: agree ZTE: Agree Xiaomi: agree vivo: yes. Sony : agree Ericsson : at least for the relative requirements Nokia:Agree Apple: Ok Verizon: agree! |
|  | Disagree |  |
| 2-3: Apply release independent approach from Rel-16 to inter-band CA including intra-band contiguous CA with maximum number of bands is 2 bands and maximum number of CCs is 12 CCs | Agree | Qualcomm: agree ZTE: No strong view. A question for clarification, does it include the UL number, i.e. 1 band UL or 2 bands UL, or both? Xiaomi: We agree maximum number of DL bands is 2 bands, but why need we limit the maximum number of CCs is 12 CCs? Yes, current band combs request in basket is maximum number of CCs is 12 CCs, i.e.,CA_n260M-n261A and CA_n257I-n259M. But maybe CA_n260M-n261M will be requested in next meeting. Sony: agree with release with independent approach. However, not sure about 12 CCs. Ericsson: in general, release independence possible if signaling allows this. Where does the number of CCs come from? If the CA BW classes exist in the earlier release, a configuration can be release independent. Nokia: Agree for DL. DOCOMO: We support this proposal. Verizon: Agree! |
|  | Disagree | Huawei: why 12CCs? Does it mean the aggregated bandwidth is 1200MHz? or 2400MHz? |

### CRs/TPs comments collection

Major close-to-finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2101199 | MediaTek: We shall achieve consensus on value firstly. |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

| Issue | Options | Status Summary |
| --- | --- | --- |
| 2-1-1: IBM inter-band CA requirements shall be specified not per band pair, but per CA between band groups, for example: L+H, L+L, etc | Strong consensus to retain separate requirements by band pair | Can capture in IBM WF as agreeable: ‘IBM inter-band CA requirements shall be specified per band pair’ |
| 2-1-2: The relaxation values of FR2 inter-band CA between the low band group (n257, n258 or n261) and high band group (n259 and n260) for IBM requirement are the same as the ones for CA_n260-n261, i.e., 3.5 dB for all these bands | Agree: Qualcomm, ZTE, Sony, Ericsson, Nokia, Docomo, Apple Disagree: Mediatek General preference to treat by band-pair (no specific proposal): LGE, Oppo, Huawei | IBM WF discussion required |
| 2-2: For the FR2 inter-band DL CA, maximum input level, ACS, and in-band blocking requirement are band combination agnostic and shall be reused for any new FR2 inter-band combinations |  | Capture in IBM WF as agreeable |
| 2-3: Apply release independent approach from Rel-16 to inter-band CA including intra-band contiguous CA with maximum number of bands is 2 bands and maximum number of CCs is 12 CCs |  | IBM WF discussion required There is consensus on release independence from 16 for inter-band CA combos including intra-band contiguous CA with maximum number of bands is 2 bands No consensus on 12 CC limitation |

Summary imported from [138] with revised 2nd round objectives:

|  | Status summary |
| --- | --- |
| Sub-topic 2-4: Dual polarization antenna | Recommendations for 2nd round: Discuss if RAN4 can agree not to introduce new requirements for IBM architecture with CC per polarization under the IBM WF |

Recommendations on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| R4-2103111 | WF on FR2 UEs that support inter-band DL CA with IBM | Sony |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

## Discussion on 2nd round (if applicable)

| Issue | Options | Company Comments |
| --- | --- | --- |
| 2-1-1: IBM inter-band CA requirements shall be specified not per band pair, but per CA between band groups, for example: L+H, L+L, etc | Re: Disagree Huawei: we always have general requirement and also BC specific RF requirement. We would like to know: what is the difference between CBM and IBM general RF requirement? | Qualcomm: Not sure comment justifies company stand of ‘disagree’. CBM requirement framework is being discussed in WF. |
| 2-3: Apply release independent approach from Rel-16 to inter-band CA including intra-band contiguous CA with maximum number of bands is 2 bands and maximum number of CCs is 12 CCs | Re: Agree ZTE: No strong view. A question for clarification, does it include the UL number, i.e. 1 band UL or 2 bands UL, or both? | MediaTek: In our view, UL and DL shall be defined, separately, even though the number can be the same. DOCOMO: Thank you for your comments. As MediaTek says, we also think that the number of DL and UL bands should be defined separately. Therefore, we only showed max bands of DL is 2 bands, i.e. UL is not 2 bands, in our contribution. Qualcomm: It may be too early to decide if UL part will be release independent. ZTE: Thanks for the reply. In our view, for a complete inter-band combination, UL and DL should be defined together. It seems it is only for 1UL/2DL FR2-FR2 inter-band CA(2 bands for DL and 1 band for UL) according to companies’ reply, correct? And for 1 band UL, how many CCs should be supported? |
|  | Re: Agree Xiaomi: We agree maximum number of DL bands is 2 bands, but why need we limit the maximum number of CCs is 12 CCs? Yes, current band combs request in basket is maximum number of CCs is 12 CCs, i.e.,CA_n260M-n261A and CA_n257I-n259M. But maybe CA_n260M-n261M will be requested in next meeting. | MediaTek: We think define max DL number as 2 bands is reasonable. But about max CC number, we think the number shall be FFS. Moreover, shall we define the max CC number clearly? Let’s use max is 12 CC as an example, shall we further define it in detailed, such as: - 6 (Band_A) + 6 (band_B) - 4 (Band_A) + 8 (Band_B) - 4 (Band_A) + (4+4) (Band_B, two CC sub-groups) DOCOMO: Thank you for your comments. The reasons why we proposed to limit the max number of CCs were: - The max number of CCs among the FR2 inter band combination included in the WID (R4-2015057) until November was 12CCs. - In TS 38.307, since Table 5.2.2-1 defines the number of CCs for inter-band CA within FR1 for release independent, we followed the same format. Therefore, we have no strong view about max number of CCs as long as it is more than or equal to 12CCs And we have no intention to exclude the possibility of future extension when more than 12CCs are proposed in the future. Qualcomm: We prefer to not have a 12cc limitation after this discussion, thank you. Xiaomi: In fact, there has a new request for CA_n257I-n259L with 13CCs in WID (R4-2015057) that coming from DOCOMO, maybe next meeting, some operator will request CA_n257K-n259L, and we prefer to not have number of CCs limit, too. ZTE: As xUL/2DL NR CA basket rapporteur, we don’t know how to treat the requested combinations(if any) whose DL CC number larger than 12 if there are no guidance. So we tend to agree not limitation on the max. DL CC number. |
|  | Re: Disagree Huawei: why 12CCs? Does it mean the aggregated bandwidth is 1200MHz? or 2400MHz? | DOCOMO: Thank you for your comment. Please see above (DCM's comment for Xiaomi's question) for the reason for 12CCs. The aggregated bandwidth is 1200MHz if we mention it since CA BW classes we described in the table are within fallback group 3. But, we are also fine with defining more than 1200MHz as aggregated bandwidth if someone request. |

## Summary on 2nd round (if applicable)

WF assigned in this section.

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| R4-2103111 | Agreeable |

Summary of round 2 discussion

| Issue | Options | Company Comments |
| --- | --- | --- |
| 2-1-1: IBM inter-band CA requirements shall be specified not per band pair, but per CA between band groups, for example: L+H, L+L, etc | Re: Disagree Huawei: we always have general requirement and also BC specific RF requirement. We would like to know: what is the difference between CBM and IBM general RF requirement? | One company pointed out that the comment offered was not relevant to decision to disagree with question asked. |
| 2-3: Apply release independent approach from Rel-16 to inter-band CA including intra-band contiguous CA with maximum number of bands is 2 bands and maximum number of CCs is 12 CCs | Re: Agree ZTE: No strong view. A question for clarification, does it include the UL number, i.e. 1 band UL or 2 bands UL, or both? | Disagreement on whether UL and DL should be defined together. One company pointed out it is too early to determine if UL part can be release independent. |
|  | Re: Agree Xiaomi: We agree maximum number of DL bands is 2 bands, but why need we limit the maximum number of CCs is 12 CCs? Yes, current band combs request in basket is maximum number of CCs is 12 CCs, i.e.,CA_n260M-n261A and CA_n257I-n259M. But maybe CA_n260M-n261M will be requested in next meeting. | While there was consensus based on participants in second round discussion, in the parallel discussion in WF this issue is still open: Further discuss maximum number of CCs for such combinations is needed. - Option 1: no need to limit the maximum number of CCs - Option 2: maximum number of CCs is FFS, but at least 12 |
|  | Re: Disagree Huawei: why 12CCs? Does it mean the aggregated bandwidth is 1200MHz? or 2400MHz? | Proponent replied it need not be limited to 1200 MHz. |

# Topic #3:	UE requirements for DL CA configurations within the same frequency group based on CBM

Agenda item 11.3.2.1.3.

## Companies’ contributions summary

| T-doc number | Title | Company | Proposals / Observations |
| --- | --- | --- | --- |
| R4-2100142 | IBM RF requirements for CA configurations within same frequency group | Nokia, Nokia Shanghai Bell | Moved to topic #1, agenda item 11.3.2.1.1 |
| R4-2100143 | Specification differences between IBM and CBM | Nokia, Nokia Shanghai Bell | Moved from topic #1, agenda item 11.3.2.1.1 Observation 1: Current maximum input level 7.4A.3, ACS 7.5A.3 and in-band blocking 7.6A.2.3 requirements can be re-used for CBM UE. Observation 2: Rel-16 reference sensitivity requirement including power imbalance may not be suitable for setting requirements for CBM UE. It is likely that the power imbalance needs to be reduced compared to requirements for IBM UE. Observation 3: In REN4#97 most of the companies were in an opinion that EIS spherical coverage requirement is not necessary for CBM UE. Observation 4: It was agreed in RAN4#97 that for FR2 inter-band CA define at least peak EIS requirement for CBM band pair for inter-band DL CA Observation 5: It is likely that frequency separation class type of requirement is needed for CBM UE similarly as for non-contiguous intraband CA. |
| R4-2100621 | CBM requirements for DLCA band combinations from the same frequency group | Qualcomm Incorporated | Proposal 1: Specify CBM requirements for all requested DL CA band combinations for bands from the same frequency groups Proposal 2: Do not specify EIS spherical coverage requirements for DL CA band combinations using CBM from the same frequency group Proposal 3: CBM based requirements are band combination dependent but same requirement structure that follows intra-band CA is applied for all band combinations [5] Proposal 4: For DL CA band combinations from the same group using CBM, specify same requirements for maximum level input, ACS and in-band blocking as that for intra-band contiguous CA scenarios |
| R4-2102608 | Inter-band DL CA within same frequency group based on CBM | Apple | Observation 1: The nature of FR2 inter-band DL CA within the same frequency group based on CBM is quite similar to intra-band DL CA from both UE implementation and beam management perspective. Observation 2: The RF transceiver gain flatness and beam squinting effect typically deteriorate further with increasing carrier frequency separation. Observation 3: The peak EIS (REFSENS) performance degradation may be different between PCell carrier and SCell carrier depending on how UE processes the DL beam measurement. However, the requirement is not necessary to differentiate PCell and SCell and a single relaxation value can be defined for both carriers. Proposal 1: For inter-band CA within the same frequency group based on CBM, the spherical coverage (common spherical coverage) requirement as defined for inter-band DL CA within different frequency groups based on IBM is deemed unnecessary. Proposal 2: Requirements for inter-band DL CA within the same frequency group based on CBM would not be defined till there is a real demand for certain band combinations. |

## Open issues summary

### Sub-topic 3-1

(3-1-1:) Agree/disagree: Specify CBM requirements for all requested DL CA band combinations for bands from the same frequency groups.

(3-1-2:) Agree/disagree: Requirements for inter-band DL CA within the same frequency group based on CBM would not be defined till there is a real demand for certain band combinations.

### Sub-topic 3-2

Agree/disagree: Do not specify EIS spherical coverage requirements for DL CA band combinations using CBM from the same frequency group

### Sub-topic 3-3

(3-3-1:) Agree/disagree: CBM based REFSENS relaxations structure that follows intra-band CA is applied for all CBM band combinations.

(3-3-2:) Agree/disagree: CBM based REFSENS relaxations values are band combination dependent.

### Sub-topic 3-4

Agree/disagree: For DL CA band combinations from the same group using CBM, specify same requirements for maximum level input, ACS and in-band blocking as that for intra-band contiguous CA scenarios

## Companies views’ collection for 1st round

### Open issues

| Issue | Options | Company Comments |
| --- | --- | --- |
| 3-1-1: Specify CBM requirements for all requested DL CA band combinations for bands from the same frequency groups | Agree | Qualcomm: Agree. Work should be limited to explicitly requested combos ZTE: Agree. We think general framework/requirements would be beneficial for the limited band combination. Otherwise it should be needed discussed case by case which is not effective to our understanding. Also if there are some exceptions, separate discussion will be needed Samsung: general okay with this approach Sony: agree Ericsson: agreed. Nokia: There needs to be operator request in place and then requirements can be defined for all requested CA configurations within same frequency group. But this agreement does not mean that CBM requirements for combinations between different frequency groups are precluded forever. They may come in REL17 or later. Apple: CBM should be limited to co-located scenario within the same frequency group. Verizon: If the meaning of same frequency groups does not restrict to support different frequencies (e.g., HH or LL), we agree on this! |
|  | Disagree | MediaTek: There are so many possible band combinations, we shall have band combination demand list firstly before do solid discussion. LG Electronics : Depending on feasibility for CBM within different frequency groups. Xiaomi: in above definition for CBM, it doesn’t limit the frequency group. This concept is similar to Fs,inter, if there is new decision for Fs,inter, this can be reconsidered. OPPO: Should be band combination based. Huawei: IBM is also applied for BC in same freq group. We think both IBM and CBM should be defined for each Band combinations, and it is UE freedom to signal CBM or IBM. Additionally, CBM requirements should be accompany with statement of performance degradation allowance. |
| 3-1-2: Requirements for inter-band DL CA within the same frequency group based on CBM would not be defined till there is a real demand for certain band combinations | Agree | MediaTek: (similar comment) Support to have band combination demand list firstly. Qualcomm: Agree. Work should be limited to explicitly requested combos LG Electronics : Support ZTE: Agree. One question: in our understanding, the band combinations will be requested in Rel-17 xUL/2DL NR CA/DC basket WID. But usually, when companies request the band combination, no CBM/IBM capability information are included. Does it mean if the constitute bands belong to the same frequency group, then CBM can be applied to this combination? Vivo: Agree OPPO: If there is no inter-band DL CA demand then both IBM/CBM will not be considered. Otherwise, it should be considered for the combination. Samsung: the WID says “Define UE requirements for inter-band CA within the same freq. group (e.g. 28GHz + 28GHz) for common beam management (CBM) based on requested band combinations.” Sony: fine with the proposal Ericsson: any difference from other band combinations once a specification framework is in place? Nokia: Agree Apple: Agree Verizon: Assume we should have a clear definition first. |
|  | Disagree | Huawei: not agree. General requirement is always defined before specific requirement is defined. |
| 3-2: Do not specify EIS spherical coverage requirements for DL CA band combinations using CBM from the same frequency group | Agree | Qualcomm: Agree LG Electronics : Support. Xiaomi: agree vivo: generally OK OPPO: OK Samsung: agree Sony: fine with the proposal Ericsson: agreed. Nokia: Agree |
|  | Disagree | Huawei: not OK. Even within the same freq group, how we ensure the UE meet the common spherical coverage requirement? The frequency span can be up to 6GHz. |
| 3-3-1: CBM based REFSENS relaxations structure that follows intra-band CA is applied for all CBM band combinations | Agree | Qualcomm: Agree (this proposal only works for CBM between overlapping or touching bands) LG Electronics : Support Xiaomi: agree vivo: yes, same structure can be used for all band combination, but the value may need be evaluated case by case. OPPO: OK Nokia: Agree but should be also discussed later if it can be extended for combinations with different frequency group Apple: Will the REFSENS relaxation structure consider the wider frequency separation for inter-band CA compared to intra-band CA? |
|  | Disagree | MediaTek: (Similar comment) To have band combination demand list firstly, “all” is terrible. Huawei: we would like to know the difference compared with IBM relaxation on peak EIS and spherical EIS. Seems there is no difference, why we highlight it follow intra-band? Samsung: though this approach may work, we share similar view with MTK that requirement discussion should be based on requested band combination before talking all band combos |
| 3-3-2: CBM based REFSENS relaxations values are band combination dependent | Agree | MediaTek: Agree. Link budget component is frequency dependency. Qualcomm: Agree LG Electronics : Support ZTE: Agree. Requirements should be defined as band combination dependent Xiaomi: agree vivo: ok with this, the performance of CBM is related to frequency closely, It is safer to confirm the value case by case. OPPO: OK Nokia: More clarification needed what this means as in 3-3-1 intraband relaxation structure is mentioned which is band agnostic. If this proposal means similar way than what RAN4 did in REL16 DL CA interband CA for IBM UE then we disagree as we prefer 3-3-1 approach. Apple:Yes |
|  | Disagree | Huawei: we would like to know the difference compared with IBM relaxation on peak EIS and spherical EIS. Seems there is no difference, why we highlight it follow intra-band? Samsung: though it may work, consider this issue with issue 3-3-1 together, do we definitely need a large table for this? Relaxation per band combos per frequency separation? We think we need focus on request band combo firstly. |
| 3-4: For DL CA band combinations from the same group using CBM, specify same requirements for maximum level input, ACS and in-band blocking as that for intra-band contiguous CA scenarios | Agree | Qualcomm: Agree Xiaomi: agree Nokia: Agree |
|  | Disagree |  |

### CRs/TPs comments collection

N/A.

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

| Issue | Options | Status Summary |
| --- | --- | --- |
| 3-1: Requirements for inter-band DL CA within the same frequency group based on CBM would not be defined till there is a real demand for certain band combinations | Agree: Qualcomm, MediaTek, LGE, ZTE, Vivo, Oppo, Samsung, Sony, Ericsson, Nokia, Apple, Verizon Other comment: Huawei | Consensus possible CBM WF discussion required: Can Huawei confirm that their comment ‘General requirement is always defined before specific requirement is defined. ‘ pertains to establishing CBM requirement framework first before filling in band-pair-specific values? |
| 3-2: Do not specify EIS spherical coverage requirements for DL CA band combinations using CBM from the same frequency group | Agree: Qualcomm, LGE, Xiaomi, Vivo, Oppo, Samsung, Sony, Ericsson, Nokia Disagree: Huawei | CBM WF discussion required Fundamental assumption has been questioned: ‘Even within the same freq group, how we ensure the UE meet the common spherical coverage requirement? The frequency span can be up to 6GHz.’ |
| 3-3-1: CBM based REFSENS relaxations structure that follows intra-band CA is applied for all CBM band combinations | Agree: Qualcomm, LGE, Xiaomi, vivo, Oppo, Nokia, Apple, Mediatek*, Samsung* (*) Conditional on agreement of 3-1 Disagree: Huawei | Parallel discussion in [138] captured in table below. CBM WF discussion required |
| 3-3-2: CBM based REFSENS relaxations values are band combination dependent | Agree: MediaTek, Qualcomm, LGE, ZTE, Xiaomi, Vivo, Oppo, Nokia, Apple, Samsung* (*) Conditional on agreement of 3-1 Disagree: Huawei | CBM WF discussion required Disagreeing company wants to know: ‘we would like to know the difference compared with IBM relaxation on peak EIS and spherical EIS. Seems there is no difference, why we highlight it follow intra-band?’ |
| 3-4: For DL CA band combinations from the same group using CBM, specify same requirements for maximum level input, ACS and in-band blocking as that for intra-band contiguous CA scenarios | Consensus | Capture in CBM WF |

Summary imported from [138] with revised 2nd round objectives:

|  |  | Status summary |  |
| --- | --- | --- | --- |
| Issue 2-3-2: CBM UE and REFSENS for CA within same frequency group |  | Tentative agreements: Define REFSENS relaxation for CBM UE which is a function of frequency span between the CCs. Recommendations for 2nd round: Agreement to be included into CBM WF. |  |
| Issue 2-2-1: Non-collocated/collocated deployments | Recommendations for 2nd round: Continue discussion whether there is a need to limit CBM UEs from non-collocated deployments under the CBM WF |

Suggestion on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| R4-2103112 | WF on FR2 UEs that support inter-band DL CA with CBM | Samsung |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

## Discussion on 2nd round (if applicable)

| Issue | Options | Company Comments |
| --- | --- | --- |
| 3-1-2: Requirements for inter-band DL CA within the same frequency group based on CBM would not be defined till there is a real demand for certain band combinations | Re: agree ZTE: Agree. One question: in our understanding, the band combinations will be requested in Rel-17 xUL/2DL NR CA/DC basket WID. But usually, when companies request the band combination, no CBM/IBM capability information are included. Does it mean if the constitute bands belong to the same frequency group, then CBM can be applied to this combination? | MediaTek: We still prefer to ask the proponent to clarify the demand in advanced. Although we understand ZTE’s technical judgement. Qualcomm: In our view, it is agreed that any band combination can be supported with IBM. The original intent of CBM was to allow inter-band support with the same RF chain. Per this original intent, our judgment is yes, if the constitute bands belong to the same frequency group, then CBM can be applied to this combination. Discussion is on going however. vivo: agree. Samsung: agree, and yes to ZTE’s question Huawei: disagree. CBM or IBM is UE capability. Even BC within the same frequency group is introduced, it doesn’t mean which type is applied. If UE indicates IBM, then IBM applies, if UE indicates CBM, then CBM applies. |
|  | Re: (agree?) Samsung: the WID says “Define UE requirements for inter-band CA within the same freq. group (e.g. 28GHz + 28GHz) for common beam management (CBM) based on requested band combinations.” | MediaTek: Yes, “based on requested band combinations” is important, rather than estimate all possibilities. Samsung: sorry for making confusion in 1st round comment. Our position is “agree”. |
|  | Re: Disagree Huawei: not agree. General requirement is always defined before specific requirement is defined. | MediaTek: It’s not bad to have a general “framework” if possible. However, for exact requirement value, it shall be band-specific. Qualcomm: General framework would offer good guidance on feasibility. Framework may differ based on if CBM is extension of intra-band performance (‘inter-CA within same frequency group’), or if CBM is L+H. Huawei: for example, maximum output power, is this a per BC RF requirement? It is obviously general requirement is always there. |
| 3-2: Do not specify EIS spherical coverage requirements for DL CA band combinations using CBM from the same frequency group | Re: Disagree Huawei: not OK. Even within the same freq group, how we ensure the UE meet the common spherical coverage requirement? The frequency span can be up to 6GHz. Huawei: not OK. Even within the same freq group, how we ensure the UE meet the common spherical coverage requirement? The frequency span can be up to 6GHz. | Qualcomm: Do you have a framework proposal? We are ok to discuss common spherical coverage requirement for CBM inter-Ca between bands in the same group however. Huawei: common spherical coverage requirement applied to IBM UE could also applies for CBM UE. Without the requirement, how we ensure UE reach the RF requirement on the sphere not only on the peak dierection. |
| 3-3-1: CBM based REFSENS relaxations structure that follows intra-band CA is applied for all CBM band combinations | Re; Agree Apple: Will the REFSENS relaxation structure consider the wider frequency separation for inter-band CA compared to intra-band CA? | LG Electronics : Need to consider the wider frequency separation compared to intra-band CA. MediaTek: If wider frequency span is defined for specific band combination based on CBM , higher relaxation value is expected. vivo: structure is ok, but the details for the wider span can be discussed further. |
|  | Re: Disagree Huawei: we would like to know the difference compared with IBM relaxation on peak EIS and spherical EIS. Seems there is no difference, why we highlight it follow intra-band? | LG Electronics : In Rel-16, peak EIS and spherical EIS for FR2 inter-band CA were specified based on IBM. So. It does not make sense to apply Rel-16 requirements to CBM UE. Qualcomm: The mechanisms are different. In IBM we have two separate beam correspondence processes, so there are contributors from mismatch in those processes, as well as due to having multiple RF chains on at the same time. Huawei: To QC, how we ensure CBM is with single RF chain as intra-band CA does? |
| 3-3-2: CBM based REFSENS relaxations values are band combination dependent | Agree Nokia: More clarification needed what this means as in 3-3-1 intraband relaxation structure is mentioned which is band agnostic. If this proposal means similar way than what RAN4 did in REL16 DL CA interband CA for IBM UE then we disagree as we prefer 3-3-1 approach. | MediaTek: No matter how to show the final values in requirement document, technically, we shall consider different specific band combinations separately in discussion stage Qualcomm: It is true that the intra-band relaxations are band agnostic but note that the widest configured DL spectrum is only 2.4 GHz. Going by UE ability, maintaining good NF over say 6 MHz BW at 40 GHz is a different challenge compared to doing the same at 26 GHz. This translates to different sensitivity degradations depending on whether the ‘configured DL spectrum’ is centered at 26 GHz or 40 GHz. FFS if relaxations can be the same for all center frequencies. vivo: In our understanding, the relaxation structure should be defined per band combination. In addition, even same frequency span for different band combination may need different relaxation. Samsung: 3-3-1 approach seems simpler, also agree with Qualcomm for further study. One possible way is to go with 3-3-1 approach with relaxation values defined based on worst case, or depending on center frequency Huawei: we also think CBM use intra approach need further clarified and more time to check. It seems A framework is better than the details currently. |
|  | Re: Disagree Huawei: we would like to know the difference compared with IBM relaxation on peak EIS and spherical EIS. Seems there is no difference, why we highlight it follow intra-band? | LG Electronics : In Rel-16, REFSENS relacation for FR2 inter-band CA was specified based on IBM. So. It does not make sense to apply Rel-16 requirements to CBM UE. Qualcomm: The mechanisms are different. In IBM we have two separate beam correspondence processes, so there are contributors from mismatch in those processes, as well as due to having multiple RF chains on at the same time. Huawei: To QC, how we ensure CBM is with single RF chain as intra-band CA does? |
|  | Re: Disagree Samsung: though it may work, consider this issue with issue 3-3-1 together, do we definitely need a large table for this? Relaxation per band combos per frequency separation? We think we need focus on request band combo firstly. | MediaTek: We echo Samsung’s comment that “we need to focus on request band combo firstly”. Qualcomm: This discussion is more about framework than values. We don’t want to preclude a different set of relaxations for say 24+28G compare to say 39+43G (both combinations are hypothetical currently) vivo: This is also one of the reasons why we want to introduce Fs,inter to avoid defining requirements for unnecessary freq. spans and reduce the size of the table. Samsung: agree with Qualcomm’s consideration, but that does not mean the framework has to be band combination dependent. We are open for further study. |
| (continue discussion per request from one company despite no comment from them in first round) 3-4: For DL CA band combinations from the same group using CBM, specify same requirements for maximum level input, ACS and in-band blocking as that for intra-band contiguous CA scenarios | Agree | Qualcomm: Agree Xiaomi: agree Nokia: Agree Samsung: Agree. And “intra-band contiguous CA scenarios” should be changed to “intra-band CA scenarios” per Xiaomi’s comment in reflector. “intra-band CA scenarios” is the original proposal in contribution. |
|  | Disagree |  |

## Summary on 2nd round (if applicable)

WF assigned in this section:

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| R4-2103112 | to be revised, slated for treatment during online (GTW) session on 2/5 |

| Issue | Options | Company Comments |
| --- | --- | --- |
| 3-1-2: Requirements for inter-band DL CA within the same frequency group based on CBM would not be defined till there is a real demand for certain band combinations | Re: agree ZTE: Agree. One question: in our understanding, the band combinations will be requested in Rel-17 xUL/2DL NR CA/DC basket WID. But usually, when companies request the band combination, no CBM/IBM capability information are included. Does it mean if the constitute bands belong to the same frequency group, then CBM can be applied to this combination? | There seems to be common understanding that it is up to UE declaration. 1 company replied it depends entirely on what the UE chooses to declare for that combination. Another confirmed IBM was ok, and CBM under discussion. 2 companies agreed CBM can apply to bands in the same frequency group. |
|  | Re: Disagree Huawei: not agree. General requirement is always defined before specific requirement is defined. | Undetermined outcome |
| 3-2: Do not specify EIS spherical coverage requirements for DL CA band combinations using CBM from the same frequency group | Re: Disagree Huawei: not OK. Even within the same freq group, how we ensure the UE meet the common spherical coverage requirement? The frequency span can be up to 6GHz. Huawei: not OK. Even within the same freq group, how we ensure the UE meet the common spherical coverage requirement? The frequency span can be up to 6GHz. | Commenting company suggests ‘common spherical coverage requirement applied to IBM UE could also applies for CBM UE ‘ No detailed proposal was provided in this meeting however. |
| 3-3-1: CBM based REFSENS relaxations structure that follows intra-band CA is applied for all CBM band combinations | Re; Agree Apple: Will the REFSENS relaxation structure consider the wider frequency separation for inter-band CA compared to intra-band CA? | Common understanding that ‘REFSENS relaxation structure (would) consider the wider frequency separation for inter-band CA’ |
|  | Re: Disagree Huawei: we would like to know the difference compared with IBM relaxation on peak EIS and spherical EIS. Seems there is no difference, why we highlight it follow intra-band? | One company points out it is presumptuous to assume single band RF chain will behave like intra-band. |
| 3-3-2: CBM based REFSENS relaxations values are band combination dependent | Agree Nokia: More clarification needed what this means as in 3-3-1 intraband relaxation structure is mentioned which is band agnostic. If this proposal means similar way than what RAN4 did in REL16 DL CA interband CA for IBM UE then we disagree as we prefer 3-3-1 approach. | FFS |
|  | Re: Disagree Samsung: though it may work, consider this issue with issue 3-3-1 together, do we definitely need a large table for this? Relaxation per band combos per frequency separation? We think we need focus on request band combo firstly. | FFS |
| (continue discussion per request from one company despite no comment from them in first round) 3-4: For DL CA band combinations from the same group using CBM, specify same requirements for maximum level input, ACS and in-band blocking as that for intra-band contiguous CA scenarios | Agree | Consensus among companies (should not be likened to *contiguous* intraband) |
|  | Disagree |  |

# Topic #4:	UE requirements for UL CA configuration CA_n257A-n259A based on IBM

Agenda item 11.3.2.2.1.

## Companies’ contributions summary

| T-doc number | Title | Company | Proposals / Observations |
| --- | --- | --- | --- |
| R4-2100697 | Views on inter-band UL CA power class definition | MediaTek Beijing Inc. | Proposal1: For min requirement of “simultaneous uplink in multiple bands”, “per UE” concept shall be applied. Proposal2: Clarify whether apply power sharing mechanism for FR2 inter-band UL CA firstly, before discuss the details about based on “EIRP” or “TRP. |
| R4-2101202 | UE requirements for FR2 UL Inter-band CA | NTT DOCOMO INC. | Proposal: RAN4 should study advantage and feasibility of per bands-based maximum output power requirements for FR2 UL inter-band CA with different band group with IBE, at least from the following perspective: Compare with advantages of following options: -Option 1(Per band): Max TRP and max EIRP is 20dBm and 40dBm for each band, respectively -Option 2(Per UE): Max TRP and max EIRP is 23dBm and 43dBm in total -Option 3(Per band): Max TRP and max EIRP is 23dBm and 43dBm for each band, respectively Evaluate following perspectives: -UL performance gain of option 3(23d+23dBm transmission). -How much it has performance gain while there would be MPE issues -Effect on increasing the amount of interference caused by simultaneous 23 +23dBm transmission. -Confirm no MSD issues under assumption FR2 bands are synchronous and non-simultaneous Rx/Tx |
| R4-2101283 | Consideration on inter-band UL CA RF requirements | Intel Corporation | Proposal 1: Max TRP and max EIRP should be per UE for inter-band UL CA. Proposal 2: Emission requirements should be per UE for inter-band UL CA. Proposal 3: P-MPR should be per UE and PHR should be per CC. Proposal 4: MPR/A-MPR should be per band. Proposal 5: For inter-band UL CA the power class is defined in the same way as intra-band CA when only one band active at a time. Each band should meet the corresponding power class requirements for that band. Proposal 6: EIRP power sharing is feasible. For two bands with minimum peak EIRP x dBm for band 1 and y dBm for band 2, and x>y, if band 1 has x’ dBm and band 2 has y’ dBm actual Tx power in their corresponding beam peak directions, then the following inequality holds in principle and equality holds in full power mode. 10x'/10+10y'/10*10(x-y)/10≤ 10x/10 |
| R4-2101374 | The MOP and Tx requirements for inter-band UL CA in FR2 | Xiaomi | (will be discussed in [138] per proponent request) |

## Open issues summary

### Sub-topic 4-1

Open discussion: Specify advantages and disadvantages of per UE limit for TRP and EIRP, and need for power sharing

### Sub-topic 4-2

Open discussion: Specify advantages and disadvantages of per band limit for TRP and EIRP, and need for power sharing

### Sub-topic 4-3

Agree/disagree: P-MPR should be per UE and PHR should be per CC

### Sub-topic 4-4

Agree/disagree: MPR/A-MPR should be per band

### Sub-topic 4-5

Agree/disagree: EIRP sharing is feasible. For two bands with minimum peak EIRP x dBm for band 1 and y dBm for band 2, and x>y, if band 1 has x’ dBm and band 2 has y’ dBm actual Tx power in their corresponding beam peak directions, then the following inequality holds in principle and equality holds in full power mode.

10x'/10+10y'/10*10(x-y)/10≤  10x/10

## Companies views’ collection for 1st round

### Open issues

| Issue | Advantage - Company Comments | Disadvantage - Company Comments |
| --- | --- | --- |
| 4-1-1: Adopt per-UE limit for TRP | Intel: Regulatory is a limit applicable to a device and UE has to meet the limit no matter what band is. Huawei: it depends on regulation requirement on TRP. No limitation in FCC for TRP? | Qualcomm: - Coexistence studies were performed per band. Per UE is not consistent with co-ex studies FR2 is already UL limited. Per UE specification would only facilitate load balancing OPPO: It relates to how regulation requires, and our understanding is that it is per-band. Nokia Regulatory limit is applied per band. Per UE limitation is about meeting MPE, which is a different issue. |
| 4-1-2: Adopt per-UE limit for EIRP | MedkaTek: For min peak EIRP: (1) Align LTE/FR1 power class concept (2) Similar power consumption and thermal level for specific power class, no matter it is under single-band operation or inter-band UL CA operation. Intel: Regulatory is a limit applicable to a device and UE has to meet the limit no matter what band is. vivo: max EIRP is much better aligned with regulatory if defined toapplied per UE. Huawei: currently per UE is safe. But we are open to discuss on detail of regulation requirement and definition of EIRP. Nokia: agree per band | Qualcomm: EIRP cannot be ‘shared’ effectively due to dependence on observation location. Vivo: EIRP cannot be added from different direction directly, this is the key problem. Current EIRP definition is not that clear for UE have multiple radiated direction. OPPO: It relates to how regulation requires, and our understanding is that it is per-band. Nokia Regulatory limit is applied per band. Per UE limitation is about meeting MPE, which is a different issue. Given such a very low link budget for FR2, we should not reuse the FR1 power concept. |
| 4-2-1: Adopt per-band limit for TRP | Qualcomm: - Configured power becomes very simple due to no power sharing Improves UL throughput Xiaomi: agree per Band vivo: max TRP is applied per band is ok, for restricting the co-channel interference. OPPO: It relates to how regulation requires, and our understanding is that it is per-band. Nokia: agree per band DOCOMO: Power control requirements may be simplified. Also, UE test may not be complicated. |  |
| 4-2-2: Adopt per-band limit for EIRP | Qualcomm: - No need to deal with power sharing in EIRP domain Improves UL throughput Xiaomi: agree per Band vivo: min peak EIRP can be applied per band to ensure the UL coverage individually. OPPO: It relates to how regulation requires, and our understanding is that it is per-band. DOCOMO: Power control requirements may be simplified. Also, UE test may not be complicated. |  |

| Issue | Options | Company Comments |
| --- | --- | --- |
| 4-3: P-MPR should be per UE and PHR should be per CC | Agree | Intel: P-MPR is used to meet MPE and MPE is measured with the unit of power density. Since the power is over entire frequency range, we think P-MPR is per-UE. vivo: However, this should be further clarified after per-UE EIRP can be clarified. Huawei: PMPR is per UE. PHR is per CC defined by RAN1. |
|  | Disagree | Qualcomm: P-MPR that a UE applies may be band-specific, because the participating bands could be supported by different antenna modules. OPPO: PMPR is connected with UE implementation. And more specific discussion is needed. |
| 4-4: MPR/A-MPR should be per band | Agree | Intel: MPR/A-MPR may follow FR1 inter-band CA with two band UL where MPR/A-MPR is per-band. |
|  | Disagree | Qualcomm: While we agree with the concept, framework for simultaneous UL I FFS. LG Electronics : Need more discussion(per UE) Xiaomi: it will depend on whether EIRP is per band or per UE OPPO: Could be discussed further for certain band combination and maybe connected with UE implementations. Huawei: need to further study. Nokia: Need more analysis. |
| 4-5: Use this power sharing inequality: For two bands with minimum peak EIRP x dBm for band 1 and y dBm for band 2, and x>y, if band 1 has x’ dBm and band 2 has y’ dBm actual Tx power in their corresponding beam peak directions, then the following inequality holds in principle and equality holds in full power mode. 10x'/10+10y'/10*10(x-y)/10≤ 10x/10 | Agree | Intel: The inequality works with our proposal 5 where for inter-band UL CA, PC is defined in the same way as intra-band CA when only one band active at a time. Therefore, at a given time, only one UL band is active, i.e., x dBm for band 1. If a new min. peak EIRP x’ dBm ( x > x’) to be set for band 1, then there is room for band 2 at another time slot, i.e., y’ dBm for band 2. The inequality says the relationship between two bands across two different time instances. In theory, EIRP power sharing feasible. |
|  | Disagree | MediaTek: For UE power sharing mechanism, we think no this hard limitation is better. Qualcomm: How is this inequality defined? For each grid point? Only at the peaks? Also, the inequality fails for x’=x and y’=y. We do not see how EIRP sharing is not practical. LG Electronics : It is based on one company’s proposal. We would like to discuss more options in next meeting. vivo：Disagree Huawei: We would like to discuss more options in next meeting. Nokia: Need more analysis. DOCOMO: Thank you for the proposal, but we think we need to further sturdy if power sharing is needed or not before agreeing it. We understood that the scaling factor is introduced to address the difference of min peak EIRP between bands while max TRP is same. Question is that though this inequality apply to min peak EIRP, are the actual transmission power, i.e., x’ and y’, limited by upper bound? Verizon: FFS |

### CRs/TPs comments collection

N/A.

## Summary for 1st round

| Issue | Per-band | Per-UE | Status Summary |
| --- | --- | --- | --- |
| Max. TRP | Qualcomm, Xiaomi, Vivo, Oppo, Nokia Docomo See result of parallel discussion in [138] captured below | Intel, Huawei (*) * depends on whether ‘No limitation in FCC for TRP?’ See result of parallel discussion in [138] captured below | Further discussion in UL CA WF |
| Max. EIRP | Qualcomm, Xiaomi, Vivo, Oppo, Nokia, Docomo | Vivo, Intel, Huawei (*) * depends on regulators | Further discussion in UL CA WF |
| Min. peak EIRP | Vivo See result of parallel discussion in [138] captured below | MediaTek See result of parallel discussion in [138] captured below | Further discussion in UL CA WF |

.

| Issue | Options | Status Summary |
| --- | --- | --- |
| 4-3: P-MPR should be per UE and PHR should be per CC |  | Key issue for WF discussion: Is this valid if configured power is per-band?: PMPR is per UE. PHR is per CC defined by RAN1. |
| 4-4: MPR/A-MPR should be per band | Agree: Intel Disagree: Qualcomm, LGE, Xiaomi, Oppo, Huawei, Nokia | Proposal has no backing outside of proponent. No further action taken. |
| 4-5: Use this power sharing inequality: For two bands with minimum peak EIRP x dBm for band 1 and y dBm for band 2, and x>y, if band 1 has x’ dBm and band 2 has y’ dBm actual Tx power in their corresponding beam peak directions, then the following inequality holds in principle and equality holds in full power mode. 10x'/10+10y'/10*10(x-y)/10≤ 10x/10 | Agree: intel Disagree: MediaTek, Qualcomm, LGE, Vivo, Huawei, Nokia, Docomo, Verizon | Proposal has no backing outside of proponent. No further action taken. |

Summary imported from [138] with revised 2nd round objectives:

| Issue |  | Status summary |  |
| --- | --- | --- | --- |
| Issue 3-2-1: max. TRP Per band definition for non-overlapping bands for TRP was majority view |  | Recommendations for 2nd round: Continue discussion in UL CA WF with the assumption of per band for non-overlapping bands definition and address the questions on aggregated max TPR and how to handle non-overlapping bands keeping in mind that TRP is regulatory requirement in some countries |  |
| Issue 3-1-2: Min peak EIRP | Recommendations for 2nd round: Continue discussion in UL CA WF with the assumption of per band for non-overlapping bands definition and discuss the UE total power consumption issue, aggregated min peak EIRP definition and how to handle non-overlapping bands. |

Suggestion on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| R4-2103113 | WF on FR2 UEs that support inter-band UL CA | MediaTek |

### CRs/TPs

(none)

## Discussion on 2nd round (if applicable)

(Captured in WF)

## Summary on 2nd round (if applicable)

WF assigned in this section.

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| R4-2103113 | Agreeable |