3GPP TSG-RAN WG4 Meeting # 98-e 														   R4-2103467

Electronic Meeting, 25 Jan - 5 Feb, 2021

Agenda item:			9.24.2, 9.24.3

Source:	Huawei, HiSilicon

Title:	Email discussion summary for [98e][228] NR_FR2_FWA_Bn257_Bn258_RRM

Document for:	Information

Introduction

A new WI for FR2 FWA UE with maximum TRP of 23dBm for band n257 and n258 was approved in RAN#87-e meeting.  This work item is to introduce the requirements on FWA UE, which maintains the max EIRP of 43dBm and max TRP of 23dBm upper power limitation, and to study and specify corresponding RF requirements for such kind of UE type. The RF part and RRM/Demod part are planned to be completed by #96 and #98, respectively.

This email discussion is to address the open issues in RRM requirements part.

Topic #1: RRM performance part

Main technical topic overview. The structure can be done based on sub-agenda basis.

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2101683 | Huawei, HiSilicon | Discussion on conditions for FR2 new FWA UE |
| R4-2101684 | Huawei, HiSilicon | CR on condition requirements for UE power class 5 in TS38.133 |

Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

Issue 1-1: Band grouping of bands n257 and n258 for UE power class 5

- Proposals

  - Option 1 (Huawei):

    - Bands n257 and n258 for UE power class 5 can be defined as the group K.

- Recommended WF

  - Companies’ views are collected in 1st round discussion

Issue 1-2: Minimum SSB_RP values of bands n257 and n258 for UE power class 5

- Proposals

  - Option 1 (Huawei):

    - The minimum SSB_RP values for UE power class 5 can be defined based on Table 2.

Table 2: Minimum SSB_RP values for UE power class 5

| Case | Direction | Bands for PC5 | Refsens Diff with PC3 (dB) | YPC3 /ZPC3 (dB) | Minimum SSB_RP (dBm/120kHz) |
| --- | --- | --- | --- | --- | --- |
| Intra-freq | Rx Beam Peak | n257 | -6.9 | 7 | -123.4+YPC5 |
|  |  | n258 | -7.1 | 7 | -123.6+YPC5 |
|  | Spherical coverage | n257 | -11.5 | 7 | -115.4+ZPC5 |
|  |  | n258 | -11.7 | 7 | -115.6+ZPC5 |
| Inter-freq | Rx Beam Peak | n257 | -6.9 | 7 | -121.4+YPC5 |
|  |  | n258 | -7.1 | 7 | -121.6+YPC5 |
|  | Spherical coverage | n257 | -11.5 | 7 | -113.4+ZPC5 |
|  |  | n258 | -11.7 | 7 | -113.6+ZPC5 |

- Recommended WF

  - Companies’ views are collected in 1st round discussion

Companies views’ collection for 1st round

Open issues

| Company | Comments |
| --- | --- |
| Huawei | Issue 1-1: Support option 1. Issue 1-2: Support option 1. |

CRs/TPs comments collection

Major close-to-finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2101684 | Samsung: (1) The revision in Table 3.5.2-1, the superscript for n257 should be 5, instead of 2. (2) For Table B.2.4.2-2, I think the requirement for spherical coverage for PC4 seems to be supposed to be 115.4 and 115.6. |
|  | Qualcomm: In Table 3.5.3-1, the superscript for n257 should be 5, not 2. It seems this is the same comment as Samsung, however, there is a typo in the Table number. |
|  | Huawei: we can revise this CR based on companies’ comments. |
| YYY | Company A |
|  | Company B |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Issue#1-1 | Tentative agreements: - Bands n257 and n258 for UE power class 5 can be defined as the group K Candidate options: N/A Recommendations for 2nd round: N/A if tentative agreement is acceptable. |
| Issue#1-2 | Tentative agreements: - The minimum SSB_RP values for UE power class 5 can be defined based on Table 2. Table 2: Minimum SSB_RP values for UE power class 5 Case; Direction; Bands for PC5; Refsens Diff with PC3 (dB); YPC3 /ZPC3 (dB); Minimum SSB_RP (dBm/120kHz). Intra-freq; Rx Beam Peak; n257; -6.9; 7; -123.4+YPC5. n258; -7.1; 7; -123.6+YPC5. Spherical coverage; n257; -11.5; 7; -115.4+ZPC5. n258; -11.7; 7; -115.6+ZPC5. Inter-freq; Rx Beam Peak; n257; -6.9; 7; -121.4+YPC5. n258; -7.1; 7; -121.6+YPC5. Spherical coverage; n257; -11.5; 7; -113.4+ZPC5. n258; -11.7; 7; -113.6+ZPC5. Candidate options: N/A Recommendations for 2nd round: N/A if tentative agreement is acceptable. |

Recommendations on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| #1 | Way forward on FR2 new FWA UE RRM performance requirements | Huawei |

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2101684 | to be revised |

Discussion on 2nd round (if applicable)

Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| XXX | Based on 2nd round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |