3GPP TSG RAN WG4 Meeting #98-e		R4-2103518

Electronic Meeting, January 25th – February 5th, 2021

Agenda item:       7.1.6

Source: 	Nokia, Nokia Shanghai Bell

Title: 	WF on general test configurations for NR-U RRM performance requirements

Document for:     Discussion and Decision

# 1	Introduction

This document contains the agreements and discussions in RAN4#98-e in the email thread [98e][206] NR_unlic_RRM_2 for the topics concerning general RRM performance test configurations. The summary containing the complete discussion on that topic is found on the document [2].

Background information: in RAN4#97-e meeting, NR-U RRM performance requirements we first discussed, with the agreements captured in the WF [1].

The following colour code is used to highlight the agreements in this meeting:

Agreements in the GTW session

Agreements in the 1st round

Agreements in the 2nd round

# 2	General configuration of the RRM tests

SCS for data and SSB

- Configure the same SCS for data and SSB.

Cell configuration SCS and channel bandwidth on carrier frequency with CCA

- RAN4 define NR-U RRM test cases with SCS=30kHz for both SSB and data transmission and 40 MHz bandwidth.

Applicability of NR FDD test configurations

- NR FDD test configurations do not apply to the configuration of NR-U cells, but may apply to the configuration of NR cells in NR-U test cases.

E-UTRA, NR and NR-U configurations

- Configuration for cells without CCA in NR-U test cases:

- NR cells without CCA:

  - NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode

  - NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode

  - NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode

- LTE cells without CCA:

  - LTE FDD

  - LTE TDD

PRACH test configuration

- For handover and RRC re-establishment cases, RAN4 to assume PRACH configuration 1 and 2 as baseline for NR-U tests, as specified in Annex A.3.8.2 in TS 38.133.

- For the random access test case: RAN4 to discuss the PRACH configuration after the core requirements are defined.

- RAN4 to discuss defining a new test configuration with the new PRACH sequences introduced in NR Rel-16.

SSB test configuration and PBCH DMRS

- Number of SSB indexes per SS-burst NSSBQCL:

  - Configure PBCH DMRS sequence index with NSSBQCL = 1 with 1 SSB per slot and DRS transmission window 1 ms.

  - Configure PBCH DMRS sequence index with NSSBQCL = 2 with 2 SSBs per slot and DRS transmission window 1 ms.

- SSB configuration table:

- 4 SSB configurations are defined

- Further details are discussed in CR R4-2103521 or its revisions.

RMCs for PDSCH

- Define new RMC for PDSCH for slots with RMSI under CCA

  - SCS=30kHz

  - Reuse the same configuration as RMC for PDSCH for slots with RMSI (i.e., Type A, 24PRB, MCS4, dmrs-TypeA-Position=2, dmrs-Type=1, dmrs-AdditonalPositions=2, maxLength=1, Antenna port index: 1000, and Number of PDSCH DMRS CDM group(s) without data: 1, etc.).

- Still open

  - Proposal 3: Define new RMC for PDSCH for slots without RMSI under CCA

    - SCS=30kHz

    - Reusing SR.2.1 TDD (i.e., Type A, 24PRB, MCS4, dmrs-TypeA-Position=2, dmrs-Type=1, dmrs-AdditonalPositions=2, maxLength=1, Antenna port index: 1000, and Number of PDSCH DMRS CDM group(s) without data: 2, etc.)

CORESET configuration

- Define new RMC for CORESET for RMSI scheduling under CCA to transmit Type0-PDCCH in the discovery burst.

  - SCS/PRB: [30]kHz, [48]PRB

  - Configuration of PDCCH monitoring occasions for RMSI CORESET: [Index 0]

  - Note 7: [index 4 in Table 13-4A] in TS 38.213 [3].

RMC transmission burst

- For NR-U RRM tests, RMC is transmitted during the RMC transmission burst:

- The length of the RMC transmission burst in slots is defined as N. The RMC burst transmission format is determined according to the steps below:

  - Select N randomly from a given set of the number of slots S1 = {[1,3,5,8]} with equal probability as the total length of RMC burst transmission format.

  - A uniform random variable from 0 to 1 is generated. If the random variable is less than PCCA_DL, a burst of N fully occupied slots is transmitted. Otherwise, the RMC transmission burst is muted and the muting duration is the same as the number N of slots for determined burst format.

- RMC transmission burst is scheduled outside DBT window.

- RAN4 discuss further the number of slots in S1.

TDD UL/DL configuration

- NR-U RRM tests does not configure tdd-UL-DL-ConfigurationCommon using RRC configuration. DL scheduling is configured by DCI 1_1 slot by slot.

Existing configuration to be reused

- Considering the existing configuration from NR tests that might be reused, the following proposals FFS:

  - Proposal 1: Reuse the existing OCNG patterns in A.3.2.1 for NR-U RRM tests..

  - Proposal 2: Reuse the existing TCI state configuration in A.3.16.2 for NR-U RRM tests.

  - Proposal 3: Reuse the existing CSI-RS configurations in A.3.13.2 and A.3.17.1.2 for NR-U RRM tests. If necessary RAN4 define new CSI-RS configuration e.g. transmitted inside DBT window.

Antenna configurations

- Define new subclause for antenna configurations with unlicensed bands. For 4Rx UE, apply the same applicability rule as Rel-15 RRM test.

# 3	Applicability rules

Regarding UEs capable of dynamic channel access mode or semi-static channel access mode, the following options are open:

- Option 1:

  - The test configurations for dynamic channel access mode applies when:

    - UE is capable of only dynamic channel access mode, and

    - UE is capable of both dynamic channel access mode and semi-static channel access mode.

  - The test configurations for semi-static access mode applies when:

    - UE is capable of only semi-static channel access mode.

- Option 2:

  - The test configurations for dynamic channel access mode applies at least when:

    - UE is capable of only dynamic channel access mode, and

    - NW indicates dynamic channel occupancy.

    - FFS: for UE capable of semi-static access mode only or both semi-static and dynamic channel access modes

  - The test configurations for semi-static access mode applies at least when:

    - UE is capable of only semi-static channel access mode, and

    - NW indicates semi-static channel occupancy.

    - FFS: for UE capable of dynamic access mode only or both dynamic and semi-static access modes

- Option 3:

  - A UE that signals both FBE and LBE capability need to test only LBE test cases.

  - A UE that signals FBE only capability is subject to tests only with FBE configuration.

  - A UE that signals LBE only capability is subject to tests only with LBE configuration.

# References

- R4-2017089, “WF on NR-U RRM performance,” Nokia, Nokia Shanghai Bell, RAN4#97-e.

- R4-2103688, “Email discussion summary for [98e][206] NR_unlic_RRM_2”, Nokia, Nokia Shanghai Bell, RAN4#98-e.