3GPP TSG-RAN WG4 Meeting#98-e												R4-2103692

Electronic Meeting, 25th Jan 2021 - 5th Feb 2021

Agenda item:			7.5.3.1

Source:	Moderator (Nokia, Nokia Shanghai Bell)

Title:	Email discussion summary for [98e][210] LTE_NR_DC_CA_RRM_1

Document for:	Information

# Introduction

Briefly introduce background, the scope of this email discussion and provide some guidelines for email discussion if necessary.

## Background and scope

This T-doc will be used to guide and summarize the email discussion for the topic of Rel-16 LTE_NR_DC_CA_RRM_1 RRM requirements (AI 7.5.3.1), with the email thread identifier “[98e][210] LTE_NR_DC_CA_RRM_1”.

The scope of this email discussion are Rel-16 NR UE RRM requirements, and in particular the agenda items:

7.5.2	RRM core requirements maintenance (38.133/36.133)

7.5.2.1	Early Measurement reporting

7.5.3	RRM perf. requirements (38.133)

7.5.3.1	Early Measurement reporting

7.5.3.1.1	Accuracy requirements

7.5.3.1.2	Test cases

Based on the Agenda, the discussion has two topics: topic #1 RRM core requirements maintenance and topic #2 RRM perf. requirements.

Topic #1 will discuss the proposals in the R4-2102744 discussion paper in 3 different sub-topics. Additionally, the CRs related to R4-2102744 proposals are for commenting. One additional correction CR for Idle Mode CA/DC Measurements for Inactive mode is for commenting

Topic #2 will discuss the proposals in the two discussion papers R42102264 (Sub-topic 2-1) and R4-2102885 (Sub-topic 2-2). Additionally, 7 CR’s are available for commenting.

# Topic #1: RRM core requirements maintenance

Main technical topic overview. The structure can be done based on sub-agenda basis.

1 discussion paper has been submitted for RAN4#98. 3 proposals have been made each of which will be treated under Topic #1.

As only 1 company has submitted proposals for the aspect the general recommended WF for each sub-topic, is to discuss each proposal in this meeting and see if agreement can be reached.

Additionally, 3 CRs have been submitted.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2102744 | Huawei, HiSilicon | Tdoc Title: Discussion on remaining issues in EMR requirements Proposal 1: SSB index reading time for FR2 EMR is based on 5 samples (X=5). Proposal 2: RAN4 not to change the condition for detected cell requirements Proposal 3a: Update definition of NEUTRA_carrier in clause 4.2.2.5 of 38.133 Proposal 3b: Update definition of NNR_carrier in clause 4.2.2.5.6 of 36.133 |

## Open issues summary and views’ collection for 1st round

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

Interested companies are expected to add their views directly under the respective issues in a dialogue-like form, i.e., identical to how the chair would record views during a f2f meeting.

Please add further table rows as required and do not change previous comments of your company or other companies. Answering to questions from other companies is encouraged.

3 open issues have been listed for discussion in this meeting based on TDoc R4-2102744. The issues are treated one by one in the following 3 sup-topics.

### Sub-topic 1-1

Sub-topic description: SSB index reading time for FR2 EMR

Open issues and candidate options before e-meeting: SSB index reading time for FR2 EMR is left for further discussion in RAN4#97 meeting with two options [3; 5]. RAN4 need to select either of the options for SSB index reading for FR2, when Index reading has been requested for early measurement reporting in idle mode for inter-frequency carriers.

In Tdoc R4-2102744 proposal is allow UE 5 samples.

Issue 1-1-1: SSB index reading time for FR2 EMR

- Proposals

  - Option 1: SSB index reading time for FR2 EMR = 3

  - Option 2: SSB index reading time for FR2 EMR = 5

- Recommended WF

  - Discuss and agree (proposal from 1 company only)

| Company | Comments |
| --- | --- |
| Qualcomm | Support option 2. We share the same supporting argument provided in R4-2102744 “It can be seen that the FR2 requirements for connected mode are defined based on 5 samples (MSSB_index_inter is defined as same as Mpss/sss_sync_w/o_gaps and Mmeas_period_w/o_gaps for intra-frequency, both are based on 5 samples). Technically, this is also reasonable, since SSB index reading in FR2 requires not only DMRS sequence matching but also PBCH decoding, while in FR1 only the former is needed.” |
| Huawei | Support option 2. We provided reasons in our paper R4-2102744 as mentioned by QC above. |
| Nokia | Our preference here would be 3 to shorten the delay for Index reading. Additionally, N2 is agreed as 3 for FR3 and having aligned requirements will simplify the specification. Assuming 3 samples and the N1 scaling factor for shortest DRX will lead to 24 samples. The connected mode Index reading for an inter-frequency carrier is defined as MSSB_Index_inter which is 24 samples (for e.g. power class 2 UE). In connected mode a UE specific implementation freedom is given regarding Rx beams for measurements and hence, instead of defining samples and beams RAN4 selected just to define the number of samples. Now the question is how to capture such similar approach for idle. How to define the number of samples needed for Index reading without defining the how the does the sampling? |
| Ericsson | Even though Option 1 is preferred, Option 2 can be acceptable, given the existing connected mode requirements. |
| Apple | Support option 2. |
| MTK | Support option 2. |

### Sub-topic 1-2

Sub-topic description: Conditions for detected cell

Open issues and candidate options before e-meeting: In the RAN4#97 meeting it was discussed whether a clarification/change to the cell detected conditions when transitioning from connected to idle mode would be needed.

Issue 1-2-1: Conditions for detected cell

- Proposals

  - Option 1: Clarification/change to the cell detected conditions when transitioning from connected to idle mode is needed.

  - Option 2: Clarification/change to the cell detected conditions when transitioning from connected to idle mode is not needed.

- Recommended WF

  - Discuss and agree (proposal from 1 company only)

| Company | Comments |
| --- | --- |
| Qualcomm | Support Option 2. |
| Huawei | Support option 2. We provided reasons in section 2.2 in our paper R4-2102744. If the cell remains detectable only during the state transition but not during when UE is in idle mode, it means UE needs to search for cell again to measure the cell for EMR. |
| Nokia | As discussed in the last meeting the current text is not clear and our proposal was to clarify the wording: ‘The detected cell and SSBs remains detectable until UE reconnect to the network and transmits the early measurement report’ It is unclear how the UE should predict that a cell remains detectable in the future and hence if it is detectable when the UE has to transmit the EMR. Our proposal is to clarify that the cell remain detectable while entering idle mode. This together with the general condition: ‘shall remain detected after UE has entered Idle mode and during Idle mode, provided that the following conditions are met’ To clarify our proposal, we suggest: A cell which is detected cell in Connected mode prior to connection release, shall remain detected after UE has entered Idle mode and during Idle mode, provided that the following conditions are met: - The UE has been provided with a list of cells and/or carrier frequencies for early measurement reporting by dedicated RRC signaling and - The detected cell is among the list of cells or on a carrier frequency provided for early measurement reporting, and - The UE is provided with a valid timer T331 by dedicated RRC signaling, and - The detected cell and SSBs remains detectable until UE reconnect to the network and transmits the early measurement report, and - The carrier frequency of the detected cell and the carrier frequency of the serving cell are among the supported band combination of the UE. This would mean that the cell remains detectable if when entering idle mode and during idle the detected remains detectable. The cell status should as such not change depending on whether the network requests the EMR or not at connection setup (as it is not known if this will happen in the future). |
| Ericsson | Support Option 2. |
| Apple | Support option 2. |
| MTK | Support option 2. |

### Sub-topic 1-3

Sub-topic description: idle mode measurements for EMR in a cell

Open issues and candidate options before e-meeting: Early measurement reporting feature is introduced in Rel-16. Rel-16 also introduces HST for FR1. RAN4 has defined requirements for idle mode measurement requirements for FR1 HST for cell which indicates FR1 HST. Question has been raised how to address idle mode measurements in cells additionally indicating FR1 HST. Discussion relates to both 36.133 and 38.133.

Issue 1-3-1: Clarification regarding NEUTRA_carrier, while T331 is running

- Proposals

  - Option 1: Clarify that NEUTRA_carrier, while T331 is running, excludes configured E-UTRA carriers indicated to meet high speed requirements.

  - Option 2: other.

- Recommended WF

  - Discuss and agree (proposal from 1 company only)

| Company | Comments |
| --- | --- |
| Qualcomm | Option 1. |
| Huawei | Support option 1. We provided reasons in section 2.3 in our paper R4-2102744. With current wording, the carriers indicated for HST is counted twice in overall measurement period, so they should be excluded from the parameter NEUTRA_carrier. |
| Nokia | We are fine to clarify the HST scenario also for EMR. However, please clarify the reasoning for excluding the HST carriers as it is not clear. Currently EMR does not distinguish carriers (mobility or not) it seems that the thing to discuss with regard to table 1 in R4-2102744 is whether a UE is configured with a mix EMR and HST carriers. |
| Ericsson | Option 1 is acceptable |
| Apple | Support option 1. |
| MTK | Support option 1. |

### CRs/TPs comments collection

Major close-to-finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2102252 | Correction to Idle Mode CA/DC Measurements for Inactive mode, Nokia, Nokia Shanghai Bell |
|  | Company A |
|  | Company B |
| R4-2102745 | CR on EMR requirement maintenance in 38.133, Huawei, HiSilicon |
|  | Nokia: Parts of the changes are ok. Some are under discussion under the open Issues above. |
|  | Ericsson: depends on the outcome of the discussion above |
| R4-2102747 | CR on EMR requirements in 36.133, Huawei, HiSilicon |
|  | Nokia: changes are under discussion. Just for clarification – the sections 9.11.1A and 9.11.2B are referring to 36.133 (latest version 8 does not have such sections)? |
|  | Ericsson: depends on the outcome of the discussion above |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#1-1, Issue 1-1-1 | agreement in GTW: Agreement: SSB index reading time for FR2 EMR = 5 Candidate options: none Recommendations for 2nd round: Issue is closed |

|  | Status summary |
| --- | --- |
| Sub-topic#1-2, Issue 1-2-1 | agreements in GTW: Agreement: Clarification/change to the cell detected conditions when transitioning from connected to idle mode is not needed Candidate options: none Recommendations for 2nd round: Issue is closed |

|  | Status summary |
| --- | --- |
| Sub-topic#1-3, Issue 1-3-1 | agreements in GTW: Agreement: Clarify that NEUTRA_carrier, while T331 is running, excludes configured E-UTRA carriers indicated to meet high speed requirements. Candidate options: none Recommendations for 2nd round: Issue is closed |

Recommendations on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| #1 |  |  |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2102252 | No comments during 1st round -> agreeable. |
| R4-2102745 | Based on agreements during 1st round -> Return to |
| R4-2102747 | After discussion this needs to be revised |

## Discussion on 2nd round (if applicable)

Moderator: No open issues from Topic #1 RRM core requirements maintenance for 2nd round discussion.

### CRs/TPs comments collection

Major close-to-finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2102745 | CR on EMR requirement maintenance in 38.133, Huawei, HiSilicon |
|  | Company A |
|  | Company B |
| R4-2103727 (was R4-2102747) | CR on EMR requirements in 36.133, Huawei, HiSilicon |
|  | Company A |
|  | Company B |

## Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| R4-2102745 | agreeable. no comments received during 2nd round |
| R4-2103727 (was R4-2102747) | agreeable. no comments received during 2nd round |

# Topic #2: RRM performance requirements

Main technical topic overview. The structure can be done based on sub-agenda basis.

For RAN4#98 there are 2 aspects to discuss under the performance agenda:

- Accuracy requirements

- Test cases

4 CRs have been submitted for agenda 7.5.3.1.1, accuracy requirements.

2 discussion documents and 3 CR’s have been submitted for agenda 7.5.3.1.2, test cases.

Interested companies are expected to add their views directly under the respective issues in a dialogue-like form, i.e., identical to how the chair would record views during a f2f meeting.

Please add further table rows as required and do not change previous comments of your company or other companies. Answering to questions from other companies is encouraged.

3 open issues have been listed for discussion in this meeting based on TDoc R4-2102744. The issues are treated one by one in the following 3 sup-topics.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2102264 | Nokia, Nokia Shanghai Bell | Tdoc Title: Measurement Performance Requirements test for MR-DC Proposal 1: Define measurement performance test cases for SS-RSRP and SS-RSRQ measurement accuracies for LTE-NR_DC_CA. Proposal 2: Define intra-frequency measurement performance test cases for SS-RSRP and SS-RSRQ. Proposal 3: Define inter-frequency measurement performance test cases for SS-RSRP and SS-RSRQ. Proposal 4: Define inter-RAT measurement performance test cases for SS-RSRP and SS-RSRQ. |

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2102885 | Qualcomm Incorporated | Tdoc Title: Discussion on Cell Configuration for EMR Test Case Proposal 1: RAN4 to not introduce test cases for the following combinations: - PCell on NR RF1 and EMR target cell on LTE - PCell on NR RF2 and EMR target cell on NR FR1 - PCell on NR RF2 and EMR target cell on NR FR2 - PCell on NR RF2 and EMR target cell on LTE |

## Open issues summary and views’ collection for 1st round

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

Interested companies are expected to add their views directly under the respective issues in a dialogue-like form, i.e., identical to how the chair would record views during a f2f meeting.

Please add further table rows as required and do not change previous comments of your company or other companies. Answering to questions from other companies is encouraged.

A number of open issues have been listed for discussion in this meeting based on TDocs R4-2102264 and R4-2102885. The issues are treated one by one in the following sup-topics.

### Sub-topic 2-1

Sub-topic description: Measurement Performance Requirements test for MR-DC

Open issues and candidate options before e-meeting: it is proposed to introduce Measurement Performance Requirements test for MR-DC for Rel-16. It is proposed to introduce such test cases for the newly defined measurement requirement.

Issue 2-1-1: Measurement performance test cases for SS-RSRP and SS-RSRQ measurement accuracies for LTE-NR_DC_CA

- Proposals

  - Option 1: Define measurement performance test cases for SS-RSRP and SS-RSRQ measurement accuracies for LTE-NR_DC_CA

  - Option 2: Other

- Recommended WF

  - Discuss and agree (proposal from 1 company only)

| Company | Comments |
| --- | --- |
| Qualcomm | We don’t expect LTE cell to be configured as SCell when there are NR serving cells in FR1. We believe it should be the other way around. However, if we really need at least one test case to verify requirements of EMR on LTE, we can consider this option (FR1 NR PCell + LTE EMR). |
| Huawei | It seems option 1 is not for the Issue 2-1-1 based on our understanding of R4-2102264. Could Nokia please double check? We are in principle fine to introduce some accuracy test cases for EMR, but we need to discuss the TC list in more detail. |
| Nokia | Option 1 is supposed to be: Define measurement performance test cases for SS-RSRP and SS-RSRQ measurement accuracies for LTE-NR_DC_CA Support option 1 defining such requirements. |
| Ericsson | The updated option 1 is fine. |
| Apple | In our view such test is not so necessary, since measurement accuracy has been verified in other tests in both connected (accuracy test) and idle mode (cell reselection test). Even though there is some difference from functionality perspective, baseband performance is more or less the same. If majority strongly insists on defining some test, we suggest to verify both delay and accuracy in the same test.. |

Issue 2-1-2: Define intra-frequency measurement performance test cases for SS-RSRP and SS-RSRQ

- Proposals

  - Option 1: Define intra-frequency measurement performance test cases for SS-RSRP and SS-RSRQ

  - Option 2: Other

- Recommended WF

  - Discuss and agree (proposal from 1 company only)

| Company | Comments |
| --- | --- |
| Qualcomm | No objection to Option 1. |
| Huawei | We do not agree with option 1, since EMR is not configured for the serving frequency layer. |
| Nokia | Support option 1. This should be measurement performance test cases for SS-RSRP and SS-RSRQ for serving cell. |
| Ericsson | Agree with Option 1. |
| Apple | In our view this test is unnecessary. |

Issue 2-1-3: Define inter-frequency measurement performance test cases for SS-RSRP and SS-RSRQ

- Proposals

  - Option 1: Define inter-frequency measurement performance test cases for SS-RSRP and SS-RSRQ

  - Option 2: Other

- Recommended WF

  - Discuss and agree (proposal from 1 company only)

| Company | Comments |
| --- | --- |
| Qualcomm | No objection to Option 1. |
| Huawei | We are fine with option 1, and we suggest to follow the TC list for the EMR delay as discussed on email reflector before the meeting. |
| Nokia | Support option 1 |
| Ericsson | Agree with option 1 |
| Apple | In our view this test is unnecessary. |
| MTK | Support option 1 |

Issue 2-1-4: Define inter-RAT measurement performance test cases for SS-RSRP and SS-RSRQ

- Proposals

  - Option 1: Define inter-RAT measurement performance test cases for SS-RSRP and SS-RSRQ

  - Option 2: Other

- Recommended WF

  - Discuss and agree (proposal from 1 company only)

| Company | Comments |
| --- | --- |
| Qualcomm | Option 1 for potentially Issue 2-1-1 (FR1 NR PCell + LTE EMR). |
| Huawei | We are fine with option 1, and we suggest to follow the TC list for the EMR delay as discussed on email reflector before the meeting. |
| Nokia | Support option 1 |
| Ericsson | Agree on option 1 |
| Apple | Prefer not to have such test. |
| MTK | Support option 1 |

### Sub-topic 2-2

Sub-topic description: Discussion on Cell Configuration for EMR Test Case

Open issues and candidate options before e-meeting: it is proposed to reduce the test case scenarios for EMR based on needed configurations.

Issue 2-2-1: PCell on NR RF1 and EMR target cell on LTE

- Proposals

  - Option 1: RAN4 do not introduce test case for PCell on NR RF1 and EMR target cell on LTE

  - Option 2: Other

- Recommended WF

  - Discuss and agree (proposal from 1 company only)

| Company | Comments |
| --- | --- |
| Qualcomm | Proposed Option 1, but okay to introduce it if at least one test case verifying requirements of EMR on LTE needs to be introduced. |
| Huawei | The NR – LTE EMR measurement is for possible NE-DC setup, so the scenario is still possible, and we suggest to keep the test for verifying NR – LTE EMR performance. |
| Nokia | We support option 2. The use case here is NE-DC and hence we do not see this irrelevant. Agreed WF from RAN4#97 (R4-2017358): - Issue 4-11-2: Which target carrier scenarios to consider. - Agreement: - define test cases including with NR FR1 target carrier, with an NR FR2 target carrier and an LTE target carrier. |
| Ericsson | Ok to define the TC. |
| Apple | Fine to define the test to verify corresponding functionality. |
| MTK | Support option 2 |

Issue 2-2-2: PCell on NR RF2 and EMR target cell on NR FR1

- Proposals

  - Option 1: RAN4 do not introduce test case for PCell on NR RF2 and EMR target cell on NR FR1

  - Option 2: Other

- Recommended WF

  - Discuss and agree (proposal from 1 company only)

| Company | Comments |
| --- | --- |
| Qualcomm | Option 1. We don’t expect NR FR1 cell to be configured as SCell while NR FR2 cell is PCell. Typically, it should be the other way around. |
| Huawei | Support option 1. This is also aligned with the TC list provided by the moderator on email reflector before the meeting. |
| Nokia | Support option 1. The latest proposed list of test cases on the email reflector does not include this option. Additionally, in last meeting the agreed WF on test cases (R4-2017358) states: - Issue 4-11-1: Which serving carrier scenarios to consider - Agreement: Define test cases for the following conditions - NR FR1 carrier - LTE carrier |
| Ericsson | Option 1 is fine. |
| Apple | Support option 1. FR2 PCell with FR1 SCell is not a typical deployment. |
| MTK | Support option 1. It is not a typical case that NR FR2 cell is PCell |

Issue 2-2-3: PCell on NR RF2 and EMR target cell on NR FR2

- Proposals

  - Option 1: RAN4 do not introduce test case for Pcell on NR RF2 and EMR target cell on NR FR2

  - Option 2: Other

- Recommended WF

  - Discuss and agree (proposal from 1 company only)

| Company | Comments |
| --- | --- |
| Qualcomm | Option 1. We don’t think EMR-based Scell fast configuration is a useful feature in FR2 + FR2 scenario. A useful scenario for EMR on FR2 is FR1 Pcell + FR2 EMR. |
| Huawei | Support option 1. This is also aligned with the TC list provided by the moderator on email reflector before the meeting. |
| Nokia | Support option 1. See our comment for Issue 2-2-2. |
| Ericsson | Ok with Option 1. This is still a possible scenario but perhaps not a common one. |
| Apple | Support option 1. |
| MTK | Support option 1. It is not a typical case that NR FR2 cell is PCell |

Issue 2-2-4: PCell on NR RF2 and EMR target cell on LTE

- Proposals

  - Option 1: RAN4 do not introduce test case for Pcell on NR FR2 and EMR target cell on LTE

  - Option 2: Other

- Recommended WF

  - Discuss and agree (proposal from 1 company only)

| Company | Comments |
| --- | --- |
| Qualcomm | Option 1. We don’t expect NTE cell to be configured as Scell while NR FR2 cell is Pcell. It should be the other way around. |
| Huawei | Support option 1. This is also aligned with the TC list provided by the moderator on email reflector before the meeting. |
| Nokia | Support option 1. See our comment for Issue 2-2-2. |
| Ericsson | Option 1 is Ok. |
| Apple | Support option 1. FR2 PCell with LTE PSCell is not typical. |
| MTK | Support option 1. It is not a typical case that NR FR2 cell is PCell |

### CRs/TPs comments collection

Major close-to-finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2102262 | Draft Big CR: Introduction of Rel-16 MR-DC EMR RRM performance requirements (TS 38.133), Nokia, Nokia Shanghai Bell |
|  | Qualcomm: typo (Aboslute) in two places in the original version and another two places too. |
|  | Huawei: Is it same as R4-2017359 endorsed in last meeting? |
| R4-2102263 | Draft CR for Removal of brackets for idle mode CA measurement accuracy requirements, Nokia, Nokia Shanghai Bell |
|  | Qualcomm: the same typo as above |
|  | Huawei: We are fine to remove the [], but the side condition should be changed to -4dB to align with other Idle mode requirements (we made this change in our draftCR R4-2102751). |
| R4-2102751 | draftCR on accuracy requirements for EMR 38.133, Huawei, HiSilicon |
|  | Nokia: Changes are agreeable. We would prefer to check the SSB condition (changed from -6 to -4 and not -4.5) |
|  | Company B |
| R4-2102752 | CR to introduce accuracy requirements for EMR 36.133, Huawei, HiSilicon |
|  | Nokia: changes are agreeable. |
|  | Company B |
| R4-2100232 | Test case for EMR with both PCell and target cell in FR1, Apple |
|  | Nokia: Looks in general fine. Test purpose and environment: Is the assumption that in connected mode during T1 the UE is not configured to perform inter-F measurements? At some point during idle mode the cell 2 should be powered on. T3 is before T331 expires. Table 1-4: can we have numbers in []. Test requirements: the report shall include cell 2. |
|  | Apple: no strong view if UE should be configured with inter-f measurement during T1. Cell 2 is powered on at the beginning of T2. T3 is 10s, while T331 is 300s. Ok to put numbers in [] in table 1-4. |
| R4-2102261 | Draft CR for Idle Mode measurements of inter-frequency CA candidate cells for early reporting, Nokia, Nokia Shanghai Bell |
|  | Company A |
|  | Company B |
| R4-2102754 | draftCR to introduce TC4 for EMR, Huawei, HiSilicon |
|  | Nokia: In general, it looks fine. Some editorial: is there wrong table reference (E-UTRAN cell)? Clarify you why ‘(either higher, equal or lower priority)’ is included in the test requirements? Reference to clause 4.2.2 should it be 4.2.2.5.6? T331 is missing? Additionally, we should use a new cell in idle mode. If the UE would be configured with EN-DC and connection released the new cell (not the former PSCell) would be powered on while in idle mode. This way it is tested that UE perform the idle mode measurements. |
|  | Company B |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#2-1, Issue 2-1-1 | agreements in GTW: Agreement: - Define measurement performance test cases for SS-RSRP and SS-RSRQ measurement accuracies for LTE-NR_DC_CA - Measurement period and accuracy shall be verified in the same test. Candidate options: none Recommendations for 2nd round: Issue is closed |

|  | Status summary |
| --- | --- |
| Sub-topic#2-1, Issue 2-1-2 | Tentative agreement: Define intra-frequency measurement performance test cases for SS-RSRP and SS-RSRQ. Discuss how the serving cell measurement performance testing as a part of 2-1-1 test cases Candidate options: Recommendations for 2nd round: Discuss how serving cell measurement accuracy shall be verified in the same test as MR-DC core test. |

|  | Status summary |
| --- | --- |
| Sub-topic#2-1, Issue 2-1-3 | Tentative agreement: Define inter-frequency measurement performance test cases for SS-RSRP and SS-RSRQ. discuss how inter-frequency measurement performance as part of 2-1-1. Candidate options: Recommendations for 2nd round: Discuss how inter-frequency measurement performance shall be verified in the same test as MR-DC core test. |

|  | Status summary |
| --- | --- |
| Sub-topic#2-1, Issue 2-1-4 | Tentative agreement: Define inter-RAT measurement performance test cases for SS-RSRP and SS-RSRQ. discuss how inter-frequency measurement performance as part of 2-1-1. Candidate options: Recommendations for 2nd round: Discuss how inter-RAT measurement performance shall be verified in the same test as MR-DC core test. |

|  | Status summary |
| --- | --- |
| Sub-topic#2-2, Issue 2-2-1 | During GTW: Conclusion: previous agreements can be kept - Issue 4-11-2: Which target carrier scenarios to consider. - Agreement: - define test cases including with NR FR1 target carrier, with an NR FR2 target carrier and an LTE target carrier. Candidate options: Recommendations for 2nd round: Issue is closed |

|  | Status summary |
| --- | --- |
| Sub-topic#2-2, Issue 2-2-2 | agreement during GTW: Agreement: do not introduce test case for PCell on NR FR2 and EMR target cell on NR FR1 Candidate options: Recommendations for 2nd round: Issue is closed |

|  | Status summary |
| --- | --- |
| Sub-topic#2-2, Issue 2-2-3 | agreement during GTW: Agreement: do not introduce test case for PCell on NR FR2 and EMR target cell on NR FR2 Candidate options: Recommendations for 2nd round: Issue is closed |

|  | Status summary |
| --- | --- |
| Sub-topic#2-2, Issue 2-2-4 | agreement during GTW: Agreement: do not introduce test case for PCell on NR FR2 and EMR target cell on LTE Candidate options: Recommendations for 2nd round: Issue is closed |

Recommendations on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| R4-2103547 | WF on Test cases for MR-DC Idle mode CA measurements | Nokia, Nokia Shanghai Bell |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2102262 | To be revised (to capture agreed Draft CR agreements) |
| R4-2102263 | To be revised |
| R4-2102751 | Return to |
| R4-2102752 | agreeable |
| R4-2100232 | To be revised |
| R4-2102261 | agreeable (no comments received in 1st round) |
| R4-2102754 | To be revised |

## Discussion on 2nd round (if applicable)

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

During 1st round discussions following is agreed:

- Define measurement performance test cases for SS-RSRP and SS-RSRQ measurement accuracies for LTE-NR_DC_CA

  - Measurement period and accuracy shall be verified in the same test.

It was left open for 2nd round discussion how measurement performance for SS-RSRP and SS-RSRQ accuracies shall be verified in the same test as MR-DC core test.

### Sub-topic 2-4

Sub-topic description: How to include measurement performance for SS-RSRP and SS-RSRQ accuracies in the test cases.

Open issues and candidate options: Currently measurement performance requirements are tested in specific test cases. Similar measurement performance needs to be included in the core test cases to ensure that measurement performance is tested.

Issue 2-4-1: Measurement performance for SS-RSRP and SS-RSRQ

- Proposals

  - Option 1: Include the needed test settings in the test case to enable that measurement performance requirements can be tested against agreed accuracies.

  - Option 2: Other

| Company | Comments |
| --- | --- |
| Nokia | Option 1. Including the needed parameter setting to enable proper testing of the measurement performance would be needed. The reported value by the UE would then need to be within the set accuracy as defined in 36.133 and 38.133. Hence, the test cases would need to enable testing that (SS-)RSRP/Q measurement accuracy fulfil the requirements in the related sections (e.g. like 10.1.9B in 38.133). |
| Huawei | Option 1 is not very clear to us. We understand that in the test requirements, the accuracy should be also mentioned, e.g. the reported EMR results should fulfil the accuracy requirements in 10.1.9B in 38.133. We are not sure if this is same as the “needed test settings” in option 1. If not, we may need some clarification on the “needed test settings” in option 1. |

### Sub-topic 2-5

Sub-topic description: Test coverage.

Open issues and candidate options: Currently measurement performance requirements are defined for serving cell, NR inter-frequency SS-RSRP/SS-RSRQ; LTE inter-RAT RSRP/RSRQ and NR inter-RAT SS-RSRP/SS-RSRQ. Which measurement performance to test?

Issue 2-5-1: Measurement performance test coverage.

- Proposals

  - Option 1: Follow the test case list for serving cell and measured target cell. Hence, include measurement performance in all test cases and cover serving cell measurement performance, NR inter-frequency, NR inter-RAT and LTE inter-RAT measurement performance.

  - Option 2: Other

| Company | Comments |
| --- | --- |
| Nokia | Option 1. We are fine to follow the current TC list as it would then cover the NR inter-frequency in FR1 and FR2, LTE inter-RAT, NR inter-RAT in FR1 and FR2. However, it would need to be clarified how to enable measurement performance test for the cases where beam level reporting is used in the test while the UE does not support beam level measurements in idle mode. |
| Huawei | Support option 1. |
| MediaTek | Support option 1. |

### Sub-topic 2-6

Sub-topic description: Test coverage.

Open issues and candidate options: Currently measurement performance requirements are defined for serving cell, NR inter-frequency SS-RSRP/SS-RSRQ; LTE inter-RAT RSRP/RSRQ and NR inter-RAT SS-RSRP/SS-RSRQ. Which measurement performance to test?

Issue 2-6-1: Measurement performance test of RSRP and RSRQ.

- Proposals

  - Option 1: Test measurement performance of (SS-)RSRP and (SS-)RSRQ in each test case.

  - Option 2: Other

| Company | Comments |
| --- | --- |
| Nokia | Option 1. Both (SS-)RSRP and (SS-)RSRQ measurement performance should be tested in each test. |
| Huawei | We are fine with option 1. |
| MediaTek | Option 1. SS-RSRP and SS-RSRQ shall be tested in each test case |

## Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| R4-2103548 (from R4-2012262) | Email approval. |
| R4-2103549 (was R4-2102263) | Endorsed. No comments received in 2nd round. |
| R4-2102751 | Endorsed. No comments received in 2nd round. |
| R4-2103550 (was R4-2100232) | Endorsed. No comments received in 2nd round. |
| R4-2103551 (was R4-2102754) | Endorsed. No comments received in 2nd round. |
| R4-2103547 | based on the 2nd round discussion the WF is recommended for agreement |