3GPP TSG-RAN WG4 Meeting # 98-e 												R4-2103716

Electronic Meeting, 25 Jan. - 5 Feb., 2021

Agenda item:			11.6.2

Source:	Moderator (CMCC)

Title:	Email discussion summary for [98e][235] NR_HST_FR1_enh_RRM

Document for:	Information

Introduction

This email discussion focuses on RRM for Rel-17 NR FR1 HST, and in particular the agenda items:

11.6.2 RRM core requirements

11.6.2.1 UE RRM core requirements for CA scenario

The targets of email discussion for 1st round and 2nd round are:

- 1st round: focus on discussing the open issues and strive to minimize the open issues

- 2nd round: according to 1st round discussion, discuss left open issues for 2nd round, and strive to minimize the open issues.

- Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2100224 | Apple | Proposal 1: similar enhancement on PCell measurement in R16 HST can be used as baseline on enhancement on active/deactivated SCell in R17 HST enhancement in FR1. Proposal 2: even though in existing TS38.133 R16 HST enhancement applies to both PCell and active SCell, existing requirements have to be updated in R17 since “highSpeedMeasFlag-r16” shall only be used to enhance the measurement on PCC and another new IE is expected to be introduced to enhance the measurement on all the SCCs. Proposal 3: RAN4 can consider defining requirements for SCell activation with UE specific RS (such as TRS) for fine time tracking and AGC adjustment. Observation 1: direct SCell activation can be used in HST to facilitate SCell activation procedure. However, no standard effort is needed. |
| R4-2100475 | CATT | Proposal 1: The current table 9.2.5.2-3 cannot work for high speed. Enhanced intrafrequency cell identification and measurement period for SCC should be defined for HST CA scenario. Proposal 2: PSS/SSS detection for deactivated SCell may need to be enhanced depends on the deployment scenario. Proposal 3: RAN4 need to clarify the deployment for CA. PCell and SCell have the same coverage or not? |
| R4-2100633 | Qualcomm, Inc. | Proposal 1: Deactivated Scell measurement period HST enhancement is specified as the following table: DRX cycle; T SSB_measurement_period_intra. No DRX; 5 x measCycleSCell x CSSFintra. DRX cycle≤ 320ms; Max(800ms, 5 x max(measCycleSCell, DRX cycle)) x CSSFintra. DRX cycle> 320ms; 3 x max(measCycleSCell, DRX cycle) x CSSFintra. Deactivated Scell PSS/SSS detection time HST enhancement is specified as the following table: DRX cycle; TPSS/SSS_sync_intra. No DRX; 5 x measCycleSCell x CSSFintra. With DRX; Max(800ms, 5 x max(measCycleSCell, DRX cycle)) x CSSFintra. Proposal 2: Pcell enhancement should apply to activated Scell. No change in spec is needed. Proposal 3: No enhancement is needed for Scell activation delay. |
| R4-2100857 | CMCC | Proposal 1: for FR1 CA scenario, similar as that in Rel-16 NR HST WI, both measurement without MG and measurement with MG need to be considered. For activated Scell, intra-frequency measurement without measurement gap Proposal 2: For intra-frequency measurement without MG, it is necessary to enhance the requirements of PSS/SSS detection, time index detection and measurement period for activated SCell(s). Proposal 3: For intra-frequency measurement without MG, the enhancement of scaling factor M2 specified in Rel-16 NR HST WI can be reused for PSS/SSS detection, time index detection for activated SCell(s), and other enhancement can be further studied. Proposal 4: For intra-frequency measurement without MG, the enhancement on the number of samples and scaling factor M2 specified in Rel-16 NR HST WI can be reused for measurement period for activated SCell(s), and other enhancement can be further studied. For activated Scell, intra-frequency measurement with measurement gap Proposal 5: For intra-frequency measurement with MG, it is necessary to enhance the requirements of PSS/SSS detection, time index detection and measurement period for activated SCell(s). Proposal 6: For intra-frequency measurement with MG, the enhancement of scaling factor M2 specified in Rel-16 NR HST WI can be reused for PSS/SSS detection, time index detection for activated SCell(s), and other enhancement can be further studied. Proposal 7: For intra-frequency measurement with MG, the enhancement on the number of samples and scaling factor M2 specified in Rel-16 NR HST WI can be reused for measurement period for activated SCell(s), and other enhancement can be further studied. For deactivated SCell, intra-frequency measurement without measurement gap Proposal 8: For intra-frequency measurement without MG, it is necessary to enhance the requirements of PSS/SSS detection, time index detection and measurement period for deactivated SCell(s). Proposal 9: For intra-frequency measurement without MG, the enhancement of scaling factor M2 specified in Rel-16 NR HST WI can be reused for the scaling factor of 1.5 in the requirements of PSS/SSS detection requirements and time index detection requirements for deactivated SCell(s), and other enhancement can be further studied. Proposal 10: For intra-frequency measurement without MG, the enhancement on the number of samples and scaling factor M2 specified in Rel-16 NR HST WI can be reused for measurement period for deactivated SCell(s), and other enhancement can be further studied. For deactivated SCell, intra-frequency measurement with measurement gap Observation 1: for intra-frequency measurement with measurement gap, current TS38.133 do not specify the PSS/SSS detection requirements, time index detection requirements, and measurement period requirements for deactivated SCell. Proposal 11: considering that current TS38.133 do not specify the related requirements for deactivated SCell for intra-frequency measurement with measurement gap, it is proposed to focus on the discussion of enhancement on PSS/SSS detection, time index detection and measurement period for deactivated SCell(s) for intra-frequency measurement without MG, and the enhancement requirements can be reused for intra-frequency measurement with MG. SCell activation/deactivation delay requirement Proposal 12: whether the SCell activation/deactivation delay requirement need to be enhanced to support HST can be further studied. |
| R4-2101140 | Ericsson | - Proposal 1: RAN4 assume a new signaling for highspeed SCell is configurable to trigger SCell RRM enhancement for NR high speed train. - Proposal 2: For intrafrequency measurements without measurement gaps, TPSS/SSS_sync_intra, TSSB_time_index_intra and TSSB_measurement_period_intra with deactivated Scell are specified in below tables: Table 9.2.5.1-7: Time period for PSS/SSS detection when high speed SCell is configured (deactivated SCell) (FR1) DRX cycle; TPSS/SSS_sync_intra. No DRX; 5 x measCycleSCell x CSSFintra. DRX cycle≤ 320ms; 5 x max(measCycleSCell, 1.5x DRX cycle) x CSSFintra. DRX cycle> 320ms; 5 x max(measCycleSCell, DRX cycle) x CSSFintra. Table 9.2.5.1-8: Time period for time index detection when high speed SCell is configured (deactivated SCell) (FR1) DRX cycle; TSSB_time_index_intra. No DRX; 3 x measCycleSCell x CSSFintra. DRX cycle≤ 320ms; 3 x max(measCycleSCell, 1.5x DRX cycle) x CSSFintra. DRX cycle> 320ms; 3 x max(measCycleSCell, DRX cycle) x CSSFintra. Table 9.2.5.2-6: T SSB_measurement_period_intra when high speed SCell is configured (deactivated SCell) (FR1) DRX cycle; T SSB_measurement_period_intra. No DRX Note 2; max(200ms, ceil( 5 x Kp) x max(measCycleSCell, ,SMTC period)Note 1. DRX cycle≤ 160ms; max(200ms, ceil(5 x M2 Note 2 x Kp) x max(SMTC, measCycleSCell, period,DRX cycle)). 160ms < DRX cycle≤ 320ms; ceil(4 x M2 Note 2 x Kp) x max(SMTC period, measCycleSCell, DRX cycle). DRX cycle>320ms; ceil( Y Note 3 x Kp ) x max(measCycleSCell, DRX cycle). NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: M2 = 1.5 if SMTC periodicity > 40 ms, otherwise M2=1 NOTE 3: Y=3 when SMTC <= 40ms, Y=5 when SMTC > 40ms. - Proposal 3: For intrafrequency measurements without measurement gaps, TPSS/SSS_sync_intra, TSSB_time_index_intra with activated Scell are specified in below tables. Table 9.2.5.1-9: Time period for PSS/SSS detection, when high speed SCell is configured (activated SCell) (FR1) DRX cycle; TPSS/SSS_sync_intra. No DRX; max( 600ms, ceil( 5 x Kp) x SMTC period )Note 1 x CSSFintra. DRX cycle≤ 320ms; max( 600ms, ceil(M2 Note 2x 5 x Kp) x max(SMTC period,DRX cycle)) x CSSFintra. DRX cycle>320ms; ceil(5 x Kp) x DRX cycle x CSSFintra. NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: When RRM enhancement when high speed is not configured, M2 = 1.5; When high speed SCell is configured, M2 = 1.5 if SMTC periodicity > 40 ms;,otherwise M2=1.. Table 9.2.5.1-10: Time period for time index detection, when high speed SCell is configured (activated SCell) (FR1) DRX cycle; TSSB_time_index_intra. No DRX; max(120ms, ceil( 3 x Kp ) x SMTC period)Note 1 x CSSFintra. DRX cycle≤ 320ms; max(120ms, ceil (M2 Note 2 x 3 x Kp) x max(SMTC period,DRX cycle)) x CSSFintra. DRX cycle>320ms; Ceil(3 x Kp) x DRX cycle x CSSFintra. NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: When RRM enhancement when high speed is not configured, M2 = 1.5; When high speed SCell is configured, M2 = 1.5 if SMTC periodicity > 40 ms;,otherwise M2=1. - Proposal 4: For Intrafrequency measurements without measurement gaps, T SSB_measurement_period_intra with activated SCell uses Table 9.2.5.2-5. - Proposal 5: For Intrafrequency measurements with measurement gaps, TPSS/SSS_sync_intra, TSSB_time_index_intra with activated SCell are specified in below tables. Table 9.2.6.2-4: Time period for PSS/SSS detection, when high speed SCell is configured (activated SCell) (FR1) DRX cycle; TPSS/SSS_sync_intra. No DRX; max(600ms, 5 x max(MGRP, SMTC period)) x CSSFintra. DRX cycle≤ 320ms; max(600ms, ceil(M2Note 1x 5) x max(MGRP, SMTC period,DRX cycle)) x CSSFintra. DRX cycle>320ms; 5 x max(MGRP, DRX cycle) x CSSFintra. NOTE 1: When RRM enhancement when high speed is not configured, M2 = 1.5; When high speed SCell is configured, M2 = 1.5 if SMTC periodicity > 40 ms, otherwise M2=1.. Table 9.2.6.2-5: Time period for time index detection when high speed SCell is configured (activated SCell) (Frequency range FR1) DRX cycle; TSSB_time_index_intra. No DRX; max(120ms, 3 x max(MGRP, SMTC period)) x CSSFintra. DRX cycle≤ 320ms; max(120ms, ceil(M2Note 1x 3) x max(MGRP, SMTC period,DRX cycle) x CSSFintra). DRX cycle>320ms; 3 x max(MGRP, DRX cycle) x CSSFintra. NOTE 1: When RRM enhancement when high speed is not configured, M2 = 1.5; When high speed SCell is configured, M2 = 1.5 if SMTC periodicity > 40 ms, otherwise M2=1.. - Proposal 6: For Intrafrequency measurements with measurement gaps, T SSB_measurement_period_intra with activated SCell uses Table 9.2.6.3-3. - Proposal 7: SCell activation and deactivation delay keeps fulfilling Chapter8.3.2.3 in TS 38.133 SCell Activation and Deactivation Delay. - Open issue 1: In high speed scenario, SCell could need shorter delay, before channel changes too much. In the other hand, longer cooling down of SCell state may bring benefit to UE power saving. |
| R4-2101141 | Ericsson | CR on SCell RRM enhancement for NR high speed train scenario in FR1 |
| R4-2101173 | MediaTek inc. | Observation 1: The requirements of IDLE mode, INACTIVE mode, RLM and PSCell addition/change and conditional PSCell change are for SpCell and out of scope. Observation 2: The requirements of CONNECTED mode and CBW change do not include the multiple CCs scenario. Observation 3: The inter-frequency measurement, inter-RAT measurement and CSI-RS L3 measurement are not in the work item description of HST with CA in Rel-17. Proposal 1: No need to discuss and define the requirements for IDLE mode, INACTIVE mode, RLM and PSCell addition/change, conditional PSCell change, CONNECTED mode, CBW change, inter-frequency measurement, inter-RAT measurement and CSI-RS L3 measurement in HSR with CA in Rel-17. Observation 4: The worst cases of the multiple CCs delay requirement of active BWP switch, TCI-state switch and spatial relation switch delay are shorter than 1s. Observation 5: The Rel-16 HST requirements of the measurement gap, measurement capability and L1-RSRP measurement reuse the Rel-15 NR requirements. Proposal 2: The existing requirements of active BWP switch, TCI-state switch, spatial relation switch delay, measurement gap, measurement capability and L1-RSRP measurement in Rel-16 could be reused in HST with CA in Rel-17. Proposal 3: The requirements of timing, interruption, SCell activation/deactivation, UL carrier RRC reconfiguration delay, link recovery (BFD/CBD), CSSF, Intra-frequency measurement, L1-RSRP measurement and measurement accuracy for L1-SINR measurement shall be further discussed in the work item of HST with CA. |
| R4-2101707 | Huawei, HiSilicon | Proposal 1: Scaling factor Kp shall be added for deactivated SCell measurement requirements without gap in R15. Proposal 2: The measurement requirements for deactivated SCell with gap shall be added in R15. Proposal 3: It is proposed to reuse PSS/SSS detection, time index detection and measurement period in R16 HST for activated SCell in R17 HST. Proposal 4: When the issues on legacy deactivated SCell is fixed, the enhanced requirements on deactivated SCell in HST scenario can follow the similar rule as activated SCell. Proposal 5: It is recommended to discuss the necessity of inter-frequency measurement enhancement under high speed scenario. Proposal6: Legacy SCell activation delay can be reused in high speed scenario. |
| R4-2102863 | Nokia, Nokia Shanghai Bell | For intra-frequency cell identification with no DRX, the deactivated SCell requirements (TPSS/SSS_sync_intra, TSSB_time_index_intra and T SSB_measurement_period_intra) are reused for Rel-17 NR HST. For intra-frequency cell identification with DRX, the TPSS/SSS_sync_intra and TSSB_time_index_intra requirements for deactivated SCells are reused for Rel-17 NR HST. For intra-frequency cell identification with DRX, the T SSB_measurement_period_intra requirements for deactivated SCells are defined as follows: DRX cycle; T SSB_measurement_period_intra. No DRX; 5 x measCycleSCell x CSSFintra. DRX cycle ≤ 160ms; 5 x max(measCycleSCell, 1.5xDRX cycle) x CSSFintra. 160ms < DRX cycle ≤ 320ms; 4 x max(measCycleSCell, 1.5xDRX cycle) x CSSFintra. DRX cycle > 320ms; 5 x max(measCycleSCell, DRX cycle) x CSSFintra. For activated SCells under HST scenarios, the measurement requirement enhanced for Rel-16 HST PCells can be adopted for SCells. |

Topic #1: PSS/SSS detection, time index detection, and measurement period for activated SCell

Main technical topic overview. The structure can be done based on sub-agenda basis.

Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

Sub-topic 1-1: requirements for activated Scell for the scenario of intra-frequency measurement without measurement gap

Issue 1-1-1: whether PSS/SSS detection, time index detection, and measurement period for activated Scell for the scenario of intra-frequency measurement without measurement gap need to be enhanced

- Proposals

  - Option 1 (Apple, CATT, CMCC, HW, Nokia, Ericsson): Yes

- Recommended WF

  - 6 companies discuss issue 1-1-1, all the companies propose that PSS/SSS detection, time index detection, and measurement period for activated Scell for the scenario of intra-frequency measurement without measurement gap need to be enhanced

  - Moderator would like to check whether following option can be acceptable:

    - PSS/SSS detection, time index detection, and measurement period for activated Scell for the scenario of intra-frequency measurement without measurement gap need to be enhanced

Issue 1-1-2: enhancement on PSS/SSS detection for activated Scell, intra-frequency measurement without measurement gap

- Proposals

  - Option 1a (HW, Nokia, Apple): reuse PSS/SSS detection in R16 HST for activated SCell in R17 HST

  - Option 1b (CMCC): the enhancement of scaling factor specified in Rel-16 NR HST WI can be reused for PSS/SSS detection for activated SCell(s), and other enhancement can be further studied.

  - Option 1c (Qualcomm): Pcell enhancement should apply to activated Scell. No change in spec is needed

  - Option 2 (Ericsson):

| DRX cycle | TPSS/SSS_sync_intra |
| --- | --- |
| No DRX | max( 600ms, ceil( 5 x Kp) x SMTC period )Note 1 x CSSFintra |
| DRX cycle≤ 320ms | max( 600ms, ceil(M2 Note 2x 5 x Kp) x max(SMTC period,DRX cycle)) x CSSFintra |
| DRX cycle>320ms | ceil(5 x Kp) x DRX cycle x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: When RRM enhancement when high speed is not configured, M2 = 1.5; When high speed SCell is configured, M2 = 1.5 if SMTC periodicity > 40 ms;,otherwise M2=1. |  |

- Recommended WF

  - 6 companies discuss issue 1-1, 5 companies propose that PSS/SSS detection requirements specified in R16 HST can be reused for activated SCell in R17 HST. 1 company proposes a different enhancement.

  - Moderator would like to check whether following option can be acceptable:

    - For intra-frequency measurement without measurement gap, PSS/SSS detection requirements specified in R16 HST can be reused for activated SCell in R17 HST. Further enhancement can be further studied

Issue 1-1-3: enhancement on time index detection for activated Scell, intra-frequency measurement without measurement gap

- Proposals

  - Option 1a (Apple, HW, Nokia): reuse time index detection in R16 HST for activated SCell in R17 HST

  - Option 1b (CMCC): the enhancement of scaling factor specified in Rel-16 NR HST WI can be reused for time index detection for activated SCell(s), and other enhancement can be further studied.

  - Option 1c (Qualcomm, ): Pcell enhancement should apply to activated Scell. No change in spec is needed

  - Option 2 (Ericsson):

| DRX cycle | TSSB_time_index_intra |
| --- | --- |
| No DRX | max(120ms, ceil( 3 x Kp ) x SMTC period)Note 1 x CSSFintra |
| DRX cycle≤ 320ms | max(120ms, ceil (M2 Note 2 x 3 x Kp) x max(SMTC period,DRX cycle)) x CSSFintra |
| DRX cycle>320ms | Ceil(3 x Kp) x DRX cycle x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: When RRM enhancement when high speed is not configured, M2 = 1.5; When high speed SCell is configured, M2 = 1.5 if SMTC periodicity > 40 ms;,otherwise M2=1 |  |

- Recommended WF

  - 6 companies discuss issue 1-1, 5 companies propose that time index detection requirements specified in R16 HST can be reused for activated SCell in R17 HST. 1 company proposes a different enhancement.

  - Moderator would like to check whether following option can be acceptable:

    - For intra-frequency measurement without measurement gap, time index detection requirements specified in R16 HST can be reused for activated SCell in R17 HST. Further enhancement can be further studied

Issue 1-1-4: enhancement on measurement period for activated Scell, intra-frequency measurement without measurement gap

- Proposals

  - Option 1a (Apple, HW, Ericsson, Nokia): reuse measurement period in R16 HST for activated SCell in R17 HST

  - Option 1b (CMCC): the enhancement on the number of samples and scaling factor specified in Rel-16 NR HST WI can be reused for measurement period for activated SCell(s), and other enhancement can be further studied.

  - Option 1c (Qualcomm, ): Pcell enhancement should apply to activated Scell. No change in spec is needed

- Recommended WF

  - 6 companies discuss issue 1-1, all companies propose that measurement period requirements specified in R16 HST can be reused for activated SCell in R17 HST. 1 company proposes that on top of reusing Rel-16 HST requirements, further enhancement can be further discussed.

  - Moderator would like to check whether following option can be acceptable:

    - For intra-frequency measurement without measurement gap, measurement period requirements specified in R16 HST can be reused for activated SCell in R17 HST. Further enhancement can be further studied

Sub-topic 1-2: requirements for activated Scell for the scenario of intra-frequency measurement with measurement gap

Issue 1-2-1: whether PSS/SSS detection, time index detection, and measurement period for activated Scell for the scenario of intra-frequency measurement with measurement gap need to be enhanced

- Proposals

  - Option 1 (Apple, CATT, CMCC, HW, Nokia, Ericsson): Yes

- Recommended WF

  - 6 companies discuss issue 1-2-1, all the companies propose that PSS/SSS detection, time index detection, and measurement period for activated Scell for the scenario of intra-frequency measurement with measurement gap need to be enhanced

  - Moderator would like to check whether following option can be acceptable:

    - PSS/SSS detection, time index detection, and measurement period for activated Scell for the scenario of intra-frequency measurement with measurement gap need to be enhanced

Issue 1-2-2: enhancement on PSS/SSS detection for activated Scell, intra-frequency measurement with measurement gap

- Proposals

  - Option 1a (Apple, HW, Nokia,): reuse PSS/SSS detection in R16 HST for activated SCell in R17 HST

  - Option 1b (CMCC): the enhancement of scaling factor specified in Rel-16 NR HST WI can be reused for PSS/SSS detection for activated SCell(s), and other enhancement can be further studied.

  - Option 1c (Qualcomm, ): Pcell enhancement should apply to activated Scell. No change in spec is needed

  - Option 2 (Ericsson):

| DRX cycle | TPSS/SSS_sync_intra |
| --- | --- |
| No DRX | max(600ms, 5 x max(MGRP, SMTC period)) x CSSFintra |
| DRX cycle≤ 320ms | max(600ms, ceil(M2Note 1x 5) x max(MGRP, SMTC period,DRX cycle)) x CSSFintra |
| DRX cycle>320ms | 5 x max(MGRP, DRX cycle) x CSSFintra |
| NOTE 1: When RRM enhancement when high speed is not configured, M2 = 1.5; When high speed SCell is configured, M2 = 1.5 if SMTC periodicity > 40 ms, otherwise M2=1. |  |

- Recommended WF

  - 6 companies discuss issue 1-1, 5 companies propose that PSS/SSS detection requirements specified in R16 HST can be reused for activated SCell in R17 HST. 1 company proposes a different enhancement.

  - Moderator would like to check whether following option can be acceptable:

    - For intra-frequency measurement with measurement gap, PSS/SSS detection requirements specified in R16 HST can be reused for activated SCell in R17 HST. Further enhancement can be further studied

Issue 1-2-3: enhancement on time index detection for activated Scell, intra-frequency measurement with measurement gap

- Proposals

  - Option 1a (Apple, HW, Nokia): reuse time index detection in R16 HST for activated SCell in R17 HST

  - Option 1b (CMCC): the enhancement of scaling factor specified in Rel-16 NR HST WI can be reused for time index detection for activated SCell(s), and other enhancement can be further studied.

  - Option 1c (Qualcomm): Pcell enhancement should apply to activated Scell. No change in spec is needed

  - Option 2 (Ericsson):

| DRX cycle | TSSB_time_index_intra |
| --- | --- |
| No DRX | max(120ms, 3 x max(MGRP, SMTC period)) x CSSFintra |
| DRX cycle≤ 320ms | max(120ms, ceil(M2Note 1x 3) x max(MGRP, SMTC period,DRX cycle) x CSSFintra) |
| DRX cycle>320ms | 3 x max(MGRP, DRX cycle) x CSSFintra |
| NOTE 1: When RRM enhancement when high speed is not configured, M2 = 1.5; When high speed SCell is configured, M2 = 1.5 if SMTC periodicity > 40 ms, otherwise M2=1. |  |

- Recommended WF

  - 6 companies discuss issue 1-1, 5 companies propose that time index detection requirements specified in R16 HST can be reused for activated SCell in R17 HST. 1 company proposes a different enhancement.

  - Moderator would like to check whether following option can be acceptable:

    - For intra-frequency measurement with measurement gap, time index detection requirements specified in R16 HST can be reused for activated SCell in R17 HST. Further enhancement can be further studied

Issue 1-2-4: enhancement on measurement period for activated Scell, intra-frequency measurement with measurement gap

- Proposals

  - Option 1a (Apple, HW, Ericsson, Nokia): reuse measurement period in R16 HST for activated SCell in R17 HST

  - Option 1b (CMCC): the enhancement on the number of samples and scaling factor specified in Rel-16 NR HST WI can be reused for measurement period for activated SCell(s), and other enhancement can be further studied.

  - Option 1c (Qualcomm,): Pcell enhancement should apply to activated Scell. No change in spec is needed

- Recommended WF

  - 6 companies discuss issue 1-1, all companies propose that measurement period requirements specified in R16 HST can be reused for activated SCell in R17 HST. 1 company proposes that on top of reusing Rel-16 HST requirements, further enhancement can be further discussed.

  - Moderator would like to check whether following option can be acceptable:

    - For intra-frequency measurement with measurement gap, measurement period requirements specified in R16 HST can be reused for activated SCell in R17 HST. Further enhancement can be further studied

Companies views’ collection for 1st round

Open issues

| Company | Comments |
| --- | --- |
| Apple | issue 1-1-1: whether PSS/SSS detection, time index detection, and measurement period for activated Scell for the scenario of intra-frequency measurement without measurement gap need to be enhanced support option 1. Issue 1-1-2: enhancement on PSS/SSS detection for activated Scell, intra-frequency measurement without measurement gap Support option 1a, i.e. to reuse enhanced PSS/SSS detection requirement on PCC. We would like to comment option 1c. We believe in R16 spec measurement on activated SCell was mistakenly enhanced, since it is not in scope of R16 HST. We have also submitted a CR to fix that in R16 spec. Comments on option 2: we don’t think CSSF can be removed. Could Ericsson clarify why? Couldn’t find any analysis in R4-2101140. Issue 1-1-3: enhancement on time index detection for activated Scell, intra-frequency measurement without measurement gap Similar with 1-1-2. Issue 1-1-4: enhancement on measurement period for activated Scell, intra-frequency measurement without measurement gap Similar with 1-1-2. Issue 1-2-1: whether PSS/SSS detection, time index detection, and measurement period for activated Scell for the scenario of intra-frequency measurement with measurement gap need to be enhanced Support option 1. Issue 1-2-2: enhancement on PSS/SSS detection for activated Scell, intra-frequency measurement with measurement gap Similar with 1-1-2. Issue 1-2-3: enhancement on time index detection for activated Scell, intra-frequency measurement with measurement gap Similar with 1-1-2. Issue 1-2-4: enhancement on measurement period for activated Scell, intra-frequency measurement with measurement gap Similar with 1-1-2. |
| Huawei | Sub topic 1-1: Issue 1-1-1: agree with the recommended WF. Issue 1-1-2: agree with the recommended WF. For option2, cell identification requirements are still restricted by 2 searcher in HST scenario. We don’t think CSSFintra should be removed. Issue 1-1-3: agree with the recommended WF. Issue 1-1-4: agree with the recommended WF. Same comments for option 2. Sub topic 1-2: Issue 1-2-1: agree with the recommended WF. Issue 1-2-2: agree with the recommended WF. Issue 1-2-3: agree with the recommended WF. Issue 1-2-4: agree with the recommended WF. |
| Ericsson | Sub topic 1-1: Issue 1-1-1: agree with the recommended WF. Issue 1-1-2: agree with the recommended WF. For option2, CSSFintra removing is typo error, which should be kept. And same for else parts. Issue 1-1-3: agree with the recommended WF. Issue 1-1-4: agree with the recommended WF. Sub topic 1-2: Issue 1-2-1: agree with the recommended WF. Issue 1-2-2: agree with the recommended WF. Issue 1-2-3: agree with the recommended WF. Issue 1-2-4: agree with the recommended WF. |
| Nokia, Nokia Shanghai Bell | Sub topic 1-1: Issue 1-1-1: The recommended WF is OK. Issue 1-1-2: A question for clarification on the recommended WF: Could CMCC elaborate on what the other enhancements mean? CMCC: firstly, there is option 2 proposed by company. To move forward, we propose the recommended WF taking all the proposals from companies into account. On the other hand, considering this is the first meeting, except reusing the enhancement specified in Rel-16, it is better to allow company to propose and discuss further enhancement if there is. Issue 1-1-3: The same comment as for Issue 1-1-2. Issue 1-1-4: The same comment as for Issue 1-1-2. Sub topic 1-2: Issue 1-2-1: The recommended WF is OK. Issue 1-2-2: A question for clarification on the recommended WF: Could CMCC elaborate on what the other enhancements mean? CMCC: please see my above response. Issue 1-2-3: The same comment as for Issue 1-2-2. Issue 1-2-4: The same comment as for Issue 1-2-2. |
| MediaTek | Issue 1-1-1: support option 1 Issue 1-1-2: support option 1a. Issue 1-1-3: support option 1a Issue 1-1-4: support option 1a Issue 1-2-1: support option 1 Issue 1-2-2: support option 1a Issue 1-2-3: support option 1a Issue 1-2-4: support option 1a |
| QC | Agree with recommended WF for all issues in topic #1. |
| CMCC | Sub-topic 1-1: Issue 1-1-1: OK with the recommended WF. Issue 1-1-2: OK with the recommended WF. Issue 1-1-3: OK with the recommended WF. Issue 1-1-4: OK with the recommended WF. Sub-topic 1-2: Issue 1-2-1: OK with the recommended WF. Issue 1-2-2: OK with the recommended WF. Issue 1-2-3: OK with the recommended WF. Issue 1-2-4: OK with the recommended WF. |
| CATT | Sub topic 1-1: Issue 1-1-1: Agree with the recommended WF Issue 1-1-2: The similar scaling factor specified in Rel-16 HST for PCell can be extended to Scells. In our understanding, PCell and SCells have the same coverage. If so, the requirements of enhancement for PCell can be reused to SCell. Issue 1-1-3: Similar with 1-1-2 Issue 1-1-4: Similar with 1-1-2 Sub topic 1-2: Issue 1-2-1: Agree with the recommended WF Issue 1-2-2: Similar with 1-1-2 Issue 1-2-3: Similar with 1-1-2 Issue 1-2-4: Similar with 1-1-2 |

CRs/TPs comments collection

Major close-to-finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
|  | Company A |
|  | Company B |
|  | Company A |
|  | Company B |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#1-1 | Issue 1-1-1: whether PSS/SSS detection, time index detection, and measurement period for activated Scell for the scenario of intra-frequency measurement without measurement gap need to be enhanced Tentative agreements: PSS/SSS detection, time index detection, and measurement period for activated Scell for the scenario of intra-frequency measurement without measurement gap need to be enhanced. Issue 1-1-2: enhancement on PSS/SSS detection for activated Scell, intra-frequency measurement without measurement gap Tentative agreements: For intra-frequency measurement without measurement gap, PSS/SSS detection requirements specified in R16 HST can be reused for activated SCell in R17 FR1 HST. Further enhancement can be further studied. Issue 1-1-3: enhancement on time index detection for activated Scell, intra-frequency measurement without measurement gap Tentative agreements: For intra-frequency measurement without measurement gap, time index detection requirements specified in R16 HST can be reused for activated SCell in R17 FR1 HST. Further enhancement can be further studied. Issue 1-1-4: enhancement on measurement period for activated Scell, intra-frequency measurement without measurement gap Tentative agreements: For intra-frequency measurement without measurement gap, measurement period requirements specified in R16 HST can be reused for activated SCell in R17 FR1 HST. Further enhancement can be further studied. |
| Sub-topic#1-2 | Issue 1-2-1: whether PSS/SSS detection, time index detection, and measurement period for activated Scell for the scenario of intra-frequency measurement with measurement gap need to be enhanced Tentative agreements: PSS/SSS detection, time index detection, and measurement period for activated Scell for the scenario of intra-frequency measurement with measurement gap need to be enhanced. Issue 1-2-2: enhancement on PSS/SSS detection for activated Scell, intra-frequency measurement with measurement gap Tentative agreements: For intra-frequency measurement with measurement gap, PSS/SSS detection requirements specified in R16 HST can be reused for activated SCell in R17 FR1 HST. Further enhancement can be further studied. Issue 1-2-3: enhancement on time index detection for activated Scell, intra-frequency measurement with measurement gap Tentative agreements: For intra-frequency measurement with measurement gap, time index detection requirements specified in R16 HST can be reused for activated SCell in R17 FR1 HST. Further enhancement can be further studied. Issue 1-2-4: enhancement on measurement period for activated Scell, intra-frequency measurement with measurement gap Tentative agreements: For intra-frequency measurement with measurement gap, measurement period requirements specified in R16 HST can be reused for activated SCell in R17 FR1 HST. Further enhancement can be further studied. |

Recommendations on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| #1 | WF on RRM for FR1 HST A single WF will be used to cover all the topics. | CMCC |

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

Discussion on 2nd round (if applicable)

Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| R4-2103678 WF on RRM for FR1 HST | agreeable |

Topic #2: PSS/SSS detection, time index detection, and measurement period for deactivated SCell

Main technical topic overview. The structure can be done based on sub-agenda basis.

Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

Sub-topic 2-1: requirements for deactivated Scell for the scenario of intra-frequency measurement without measurement gap

Issue 2-1-1: whether PSS/SSS detection, time index detection, and measurement period for deactivated Scell for the scenario of intra-frequency measurement without measurement gap need to be enhanced

- Proposals

  - Option 1 (Apple, CATT, CMCC, Ericsson): Yes

- Recommended WF

  - 4 companies discuss issue 2-1-1, all the companies propose that PSS/SSS detection, time index detection, and measurement period for deactivated Scell for the scenario of intra-frequency measurement without measurement gap need to be enhanced

  - Moderator would like to check whether following option can be acceptable:

    - PSS/SSS detection, time index detection, and measurement period for deactivated Scell for the scenario of intra-frequency measurement without measurement gap need to be enhanced

Issue 2-1-2: enhancement on PSS/SSS detection for deactivated Scell, intra-frequency measurement without measurement gap

- Proposals

  - Option 1a (Apple): similar enhancement on PCell measurement in R16 HST can be used as baseline on enhancement on activated Scell

  - Option 1b (CMCC,): the enhancement of scaling factor M2 specified in Rel-16 NR HST WI can be reused for the scaling factor of 1.5 in the requirements of PSS/SSS detection for deactivated SCell(s), and other enhancement can be further studied.

  - Option 2 (Qualcomm,):

| DRX cycle | TPSS/SSS_sync_intra |
| --- | --- |
| No DRX | 5 x measCycleSCell x CSSFintra |
| With DRX | Max(800ms, 5 x max(measCycleSCell, DRX cycle)) x CSSFintra |

  - Option 3 (Ericsson):

| DRX cycle | TPSS/SSS_sync_intra |
| --- | --- |
| No DRX | 5 x measCycleSCell x CSSFintra |
| DRX cycle≤ 320ms | 5 x max(measCycleSCell, 1.5x DRX cycle) x CSSFintra |
| DRX cycle> 320ms | 5 x max(measCycleSCell, DRX cycle) x CSSFintra |

- Recommended WF

  - More discussion is needed

Issue 2-1-3: enhancement on time index detection for deactivated Scell, intra-frequency measurement without measurement gap

- Proposals

  - Option 1a (Apple): similar enhancement on PCell measurement in R16 HST can be used as baseline on enhancement on activated Scell

  - Option 1b (CMCC): the enhancement of scaling factor M2 specified in Rel-16 NR HST WI can be reused for the scaling factor of 1.5 in the requirements of time index detection for deactivated SCell(s), and other enhancement can be further studied.

  - Option 3 (Ericsson):

| DRX cycle | TSSB_time_index_intra |
| --- | --- |
| No DRX | 3 x measCycleSCell x CSSFintra |
| DRX cycle≤ 320ms | 3 x max(measCycleSCell, 1.5x DRX cycle) x CSSFintra |
| DRX cycle> 320ms | 3 x max(measCycleSCell, DRX cycle) x CSSFintra |

- Recommended WF

  - More discussion is needed

Issue 2-1-4: enhancement on measurement period for deactivated Scell, intra-frequency measurement without measurement gap

- Proposals

  - Option 1a (Apple): similar enhancement on PCell measurement in R16 HST can be used as baseline on enhancement on activated Scell

  - Option 1b (CMCC): the enhancement on the number of samples and scaling factor specified in Rel-16 NR HST WI can be reused for measurement period for deactivated SCell(s), and other enhancement can be further studied.

  - Option 2 (Qualcomm):

| DRX cycle | T SSB_measurement_period_intra |
| --- | --- |
| No DRX | 5 x measCycleSCell x CSSFintra |
| DRX cycle≤ 320ms | Max(800ms, 5 x max(measCycleSCell, DRX cycle)) x CSSFintra |
| DRX cycle> 320ms | 3 x max(measCycleSCell, DRX cycle) x CSSFintra |

  - Option 3 (Ericsson):

| DRX cycle | T SSB_measurement_period_intra |
| --- | --- |
| No DRX Note 2 | max(200ms, ceil( 5 x Kp) x max(measCycleSCell, ,SMTC period)Note 1 |
| DRX cycle≤ 160ms | max(200ms, ceil(5 x M2 Note 2 x Kp) x max(SMTC, measCycleSCell, period,DRX cycle)) |
| 160ms < DRX cycle≤ 320ms | ceil(4 x M2 Note 2 x Kp) x max(SMTC period, measCycleSCell, DRX cycle) |
| DRX cycle>320ms | ceil( Y Note 3 x Kp ) x max(measCycleSCell, DRX cycle) |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: M2 = 1.5 if SMTC periodicity > 40 ms, otherwise M2=1 NOTE 3: Y=3 when SMTC <= 40ms, Y=5 when SMTC > 40ms |  |

  - Option 4 (Nokia):

| DRX cycle | T SSB_measurement_period_intra |
| --- | --- |
| No DRX | 5 x measCycleSCell x CSSFintra |
| DRX cycle ≤ 160ms | 5 x max(measCycleSCell, 1.5xDRX cycle) x CSSFintra |
| 160ms < DRX cycle ≤ 320ms | 4 x max(measCycleSCell, 1.5xDRX cycle) x CSSFintra |
| DRX cycle > 320ms | 5 x max(measCycleSCell, DRX cycle) x CSSFintra |

- Recommended WF

  - More discussion is needed

Sub-topic 2-2: requirements for deactivated Scell for the scenario of intra-frequency measurement with measurement gap

Sub-topic description

Open issues and candidate options before e-meeting:

Background: R4-2101707 discuss the issue on cell identification for deactivated SCell for non-HST scenario in R15, and have the observation that in R15 there is no cell identification requirements specified for deactivated SCells with measurement gap.

Issue 2-2-1: if the issue on cell identification requirements for SCell for non-HST scenario in R15 exists (as described in R4-2101707), for Rel-17 FR1 HST, how to specify the PSS/SSS detection, time index detection, and measurement period for deactivated SCell for the scenario of intra-f measurement with MG to support HST.

- Proposals

  - Option 1 (HW): When the issues on legacy deactivated SCell for non-HST scenario is fixed, the enhanced requirements on deactivated SCell in HST scenario can follow the similar rule as activated SCell.

  - Option 2 (CMCC): considering that for non-HST scenario, current TS38.133 do not specify the cell identification requirements for deactivated SCell for intra-frequency measurement with measurement gap, for Rel-17 FR1 HST, it is proposed to focus on the discussion of enhancement on cell identification requirements for deactivated SCell(s) for intra-frequency measurement without MG firstly. The enhanced requirements specified for deactivated SCell(s) for intra-f without MG can be reused for deactivated SCell(s) for intra-f with MG.

- Recommended WF

  - Companies are encouraged to provide views on this issue

Companies views’ collection for 1st round

Open issues

| Company | Comments |
| --- | --- |
| Apple | Issue 2-1-1: whether PSS/SSS detection, time index detection, and measurement period for deactivated Scell for the scenario of intra-frequency measurement without measurement gap need to be enhanced Support option 1. Issue 2-1-2: enhancement on PSS/SSS detection for deactivated Scell, intra-frequency measurement without measurement gap We prefer option 1a or 1b. Regarding option 2, we suggest keeping M2. One example is SMTC= measCycleSCell=160ms, wherein scaling factor 1.5 is needed. If the train is moving with 500km/h in (relatively) small cell e.g. 700m ISD, probably network won’t configure large SMTC/DRX cycle and UE would follow 5 samples as proposed in option 2. However, in current practice not all the HST can reach 500km/h. When the train is moving not that fast or when very large ISD is deployed, we think network can still have a chance to reduce RS overhead by configuring longer SMTC or DRX. Issue 2-1-3: enhancement on time index detection for deactivated Scell, intra-frequency measurement without measurement gap Support option 1a or 1b. regrading option 3, we have similar comment as in issue 1-1-2. CSSF should not be removed. Issue 2-1-4: enhancement on measurement period for deactivated Scell, intra-frequency measurement without measurement gap Prefer option 1a or 1b. Comments on option 2: similar with 2-1-2. Comments on option 3: similarly, Kp should not be removed. Comments on option 4 when DRX cycle > 320ms: we are also fine with 5 samples. |
| Huawei | Sub-topic 2-1 Issue 2-1-1: support the recommended WF. Issue 2-1-2: agree with option 1a or 1b. In addition, we found an issue in the existing R15 deactivated scell measurement requirements, no Kp is scaled. Although UE performs measurements on deactivated SCell without gap, the SMTC occasion may be partial overlapped with gap as well. For the partial overlapping case, it is reasonable to scale Kp. One can argue that as measCycleSCell is larger or equal to 160ms, there is other opportunity for measurement on deactivated SCell during measCycleSCell. However the motivation of introducing measCycleSCell is to slowdown the measurement on deactivated SCC compared with activated cells. The measurement delay on deactivated SCell without scaling Kp may be smaller than that of activated cells, which is contradictory with the motivation. We think if the issue is fixed, the corresponding requirements in HST shall consider Kp as well. Issue 2-1-3: support 1a or 1b. We need further check the exact values in option 2. Concerns are on removing CSSFintra for option 3. Issue 2-1-4: need further check the exact values in option 2 and option 4. For option 3, in case Kp is removed, the measurement delay on deactivated SCell may be short than activated SCell. It is not logical. Sub-topic 2-2 Issue 2-2-1: can agree with option 2. |
| Ericsson | Sub-topic 2-1 Issue 2-1-1: agree with the recommended WF. Issue 2-1-2: Agree with 1a or 1b. It’s good to keep M2. Issue 2-1-3: Agree with 1a or 1b. It’s good to keep M2. Issue 2-1-4: Agree with 1a or 1b. Sub-topic 2-2 Issue 2-2-1: agree with option2. |
| Nokia, Nokia Shanghai Bell | Sub topic 2-1: Issue 2-1-1: The recommended WF is OK. Issue 2-1-2: As a suggestion, the discussion can be split into “Case 1: ‘No DRX” and “Case 2: With DRX”. Case 1: No DRX We are OK with TPSS/SSS_sync_intra = 5 x measCycleSCell x CSSFintra Case 2: With DRX We are OK to keep or remove the scaling factor (1.5) depending on DRX and measCycleSCell parameters. If it is removed, then there is only one level of granularity for this case. Could Qualcomm elaborate on how 800 ms is computed in Option 2? Issue 2-1-3: We suggest to follow the same approach as in Issue 2-1-2. Issue 2-1-4: The discussion can be split into “Case 1: ‘No DRX” and “Case 2: With DRX”: Case 1: No DRX It seems T SSB_measurement_period_intra = 5 x measCycleSCell x CSSFintra is OK. Case 2: With DRX The enhancement in Option 4 uses the same methodology as for Rel-16 HST. Both the measurement sample number and scaling factor should be adapted according to DRX and measCycleSCell; these are FFS as further discussions are necessary. Sub topic 2-2: Issue 2-2-1: A question for clarification on Option 2: Does Option 2 mean that the enhanced requirements for deactivated SCell for intra-frequency with measurement gaps will be specified irrespective of whether the corresponding requirements are defined for non-HST or not? |
| MediaTek | Issue 2-1-1: support option 1 Issue 2-1-2: support option 1a, we can follow the same logic as Rel-16 HST Issue 2-1-3: support option 1a, we can follow the same logic as Rel-16 HST Issue 2-1-4: more discussion is needed for factor ,e.g., number of sample, Kp in option 2 and option 4 To Nokia, what is the intention to put the factor 1.5 with DRX cycle rather than number of samples? Issue 2-2-1: support option 2 |
| QC | Issue 2-1-1: support recommended WF Issue 2-1-2: We would like to address above comments on option 2 Reason to remove M2: The relaxation factor (1.5 or M2) is added to address the case that SMTC period is much larger than DRx on duration when DRx cycle is long (>=320ms). In this case, UE either have to wake up during DRx off time to measure, or extend on duration to significantly large portion of DRx cycle, which defeat the purpose of DRx. However, for deactivated Scell, measCycleScell guarantees that measurement happens at most once per 160ms, hence for shorter DRx cycle, UE only needs to extend on duration once several DRx cycle. However, we agree that even taking the above analysis into consideration, relaxation factor can’t completely be covered by measCycleScell, but since deactivated Scell measurement requirement is slower than neighboring cell measurement, we prefer to reduce the implementation complexity by removing different scaling factor and addition rows we added in neighboring cell requirement enhancement. Therefore, we can keep implementation complexity low as well as speed up the deactivated Scell measurement. This explanation applies to issue 2-1-3, 2-1-4. After checking the math, 800ms is not needed in our proposal, it was to guarantee that with DRx cycle >= no DRx cycle, but it can hold without 800ms. We support Huawei’s view on Kp Issue 2-1-3: We missed the SSB index proposal, added here: DRX cycle; TSSB_time_index_intra. No DRX; 3 x measCycleSCell x CSSFintra. With DRX; 3 x max(measCycleSCell, DRX cycle)) x CSSFintra. Issue 2-1-4: Support option 2. For issue 2-1-2,3,4 proposal from Ericsson: if non-HST has both Kp and CSSF, they shouldn’t be removed in HST. Issue 2-2-1 Deactivated Scell intra-frequency with gap may not be a valid scenario, since gap is required for intra-frequency measurement only when SSB is outside of active BWP, but deactivated Scell doesn’t have an “active BWP”, therefore this scenario is not valid. |
| CMCC | Sub-topic 2-1: Issue 2-1-1: OK with the recommended WF. Issue 2-1-2: our basic consideration is that the enhancement specified in Rel-16 HST can be used as baseline to specify the requirements for CA. As for option 2, M2 is removed but the number of samples for 160ms DRX cycle is increased from 4 to 5, we are open to have further discussion. Issue 2-1-4: similar comment as in Issue 2-1-2. Sub-topic 2-2: QC pointed out that deactivated Scell intra-frequency with gap may be not needed, since there is no active BWP for deactivated BWP. We think this observation make sense, and we will have further check. |
| CATT | Sub-topic 2-1: Issue 2-1-1: Agree with the recommended WF. Issue 2-1-2: prefer to option 1a/1b. Keep M2. Issue 2-1-3: similar with 2-1-2 Issue 2-1-4: similar with 2-1-2 |

CRs/TPs comments collection

Major close to finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
|  | Company A |
|  | Company B |
|  | Company A |
|  | Company B |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#2-1 | Issue 2-1-1: whether PSS/SSS detection, time index detection, and measurement period for deactivated Scell for the scenario of intra-frequency measurement without measurement gap need to be enhanced Tentative agreements: PSS/SSS detection, time index detection, and measurement period for deactivated Scell for the scenario of intra-frequency measurement without measurement gap need to be enhanced. Issue 2-1-2: enhancement on PSS/SSS detection for deactivated Scell, intra-frequency measurement without measurement gap Candidate options: - Option 1a (Apple, Huawei, Ericsson, MTK, CATT): similar enhancement on PCell measurement in R16 HST can be used as baseline on enhancement on activated Scell - Option 1b (CMCC, Apple, Huawei, Ericsson, CATT): the enhancement of scaling factor M2 specified in Rel-16 NR HST WI can be reused for the scaling factor of 1.5 in the requirements of PSS/SSS detection for deactivated SCell(s), and other enhancement can be further studied. - Option 2 (Qualcomm,): DRX cycle; TPSS/SSS_sync_intra. No DRX; 5 x measCycleSCell x CSSFintra. With DRX; Max(800ms, 5 x max(measCycleSCell, DRX cycle)) x CSSFintra. Recommendations for 2nd round: Option 1a and option 1b are similar, both suggest to reusing the enhancement approach used in Rel-16 HST WI. Based on above observation, moderator would like to merge option 1a and option 1b as following: Option 1: similar enhancement to PCell measurement in R16 HST can be used as baseline for PSS/SSS detection enhancement on deactivated Scell. For 2nd round discussion, companies are encouraged to provide comments on following options: - Option 1: similar enhancement to PCell measurement in R16 HST can be used as baseline for PSS/SSS detection enhancement on deactivated Scell. - Option 2: DRX cycle; TPSS/SSS_sync_intra. No DRX; 5 x measCycleSCell x CSSFintra. With DRX; Max(800ms, 5 x max(measCycleSCell, DRX cycle)) x CSSFintra. Issue 2-1-3: enhancement on time index detection for deactivated Scell, intra-frequency measurement without measurement gap Candidate options: - Option 1a (Apple, Huawei, Ericsson, MTK, CATT): similar enhancement on PCell measurement in R16 HST can be used as baseline on enhancement on activated Scell - Option 1b (CMCC, Apple, Huawei, Ericsson, CATT): the enhancement of scaling factor M2 specified in Rel-16 NR HST WI can be reused for the scaling factor of 1.5 in the requirements of time index detection for deactivated SCell(s), and other enhancement can be further studied. - Option 2 (Ericsson): DRX cycle; TSSB_time_index_intra. No DRX; 3 x measCycleSCell x CSSFintra. DRX cycle≤ 320ms; 3 x max(measCycleSCell, 1.5x DRX cycle) x CSSFintra. DRX cycle> 320ms; 3 x max(measCycleSCell, DRX cycle) x CSSFintra. - Option 3 (QC): DRX cycle; TSSB_time_index_intra. No DRX; 3 x measCycleSCell x CSSFintra. With DRX; 3 x max(measCycleSCell, DRX cycle)) x CSSFintra. Recommendations for 2nd round: Option 1a and option 1b are similar, both suggest to reusing the enhancement approach used in Rel-16 HST WI. Based on above observation, moderator would like to merge option 1a and option 1b as following: Option 1: similar enhancement to PCell measurement in R16 HST can be used as baseline for time index detection enhancement on deactivated Scell. For 2nd round discussion, companies are encouraged to provide comments on following options: - Option 1: similar enhancement to PCell measurement in R16 HST can be used as baseline for time index detection enhancement on deactivated Scell. - Option 2: DRX cycle; TSSB_time_index_intra. No DRX; 3 x measCycleSCell x CSSFintra. With DRX; 3 x max(measCycleSCell, DRX cycle)) x CSSFintra. Issue 2-1-4: enhancement on measurement period for deactivated Scell, intra-frequency measurement without measurement gap Candidate options: - Option 1a (Apple, Ericsson, CATT): similar enhancement on PCell measurement in R16 HST can be used as baseline on enhancement on activated Scell - Option 1b (CMCC, Apple, Ericsson, CATT): the enhancement on the number of samples and scaling factor specified in Rel-16 NR HST WI can be reused for measurement period for deactivated SCell(s), and other enhancement can be further studied. - Option 2 (Qualcomm): DRX cycle; T SSB_measurement_period_intra. No DRX; 5 x measCycleSCell x CSSFintra. DRX cycle≤ 320ms; Max(800ms, 5 x max(measCycleSCell, DRX cycle)) x CSSFintra. DRX cycle> 320ms; 3 x max(measCycleSCell, DRX cycle) x CSSFintra. - Option 3 (Ericsson): DRX cycle; T SSB_measurement_period_intra. No DRX Note 2; max(200ms, ceil( 5 x Kp) x max(measCycleSCell, ,SMTC period)Note 1. DRX cycle≤ 160ms; max(200ms, ceil(5 x M2 Note 2 x Kp) x max(SMTC, measCycleSCell, period,DRX cycle)). 160ms < DRX cycle≤ 320ms; ceil(4 x M2 Note 2 x Kp) x max(SMTC period, measCycleSCell, DRX cycle). DRX cycle>320ms; ceil( Y Note 3 x Kp ) x max(measCycleSCell, DRX cycle). NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: M2 = 1.5 if SMTC periodicity > 40 ms, otherwise M2=1 NOTE 3: Y=3 when SMTC <= 40ms, Y=5 when SMTC > 40ms. - Option 4 (Nokia): DRX cycle; T SSB_measurement_period_intra. No DRX; 5 x measCycleSCell x CSSFintra. DRX cycle ≤ 160ms; 5 x max(measCycleSCell, 1.5xDRX cycle) x CSSFintra. 160ms < DRX cycle ≤ 320ms; 4 x max(measCycleSCell, 1.5xDRX cycle) x CSSFintra. DRX cycle > 320ms; 5 x max(measCycleSCell, DRX cycle) x CSSFintra. Recommendations for 2nd round: Option 1a and option 1b are similar, both suggest to reusing the enhancement approach used in Rel-16 HST WI. Based on above observation, moderator would like to merge option 1a and option 1b as following: Option 1: similar enhancement to PCell measurement in R16 HST can be used as baseline for measurement period enhancement on deactivated Scell. For 2nd round discussion, companies are encouraged to provide comments on following options: - Option 1: similar enhancement to PCell measurement in R16 HST can be used as baseline for measurement period enhancement on deactivated Scell. - Option 2 (Qualcomm): DRX cycle; T SSB_measurement_period_intra. No DRX; 5 x measCycleSCell x CSSFintra. DRX cycle≤ 320ms; Max(800ms, 5 x max(measCycleSCell, DRX cycle)) x CSSFintra. DRX cycle> 320ms; 3 x max(measCycleSCell, DRX cycle) x CSSFintra. |
| Sub-topic#2-2 | Issue 2-2-1: if the issue on cell identification requirements for SCell for non-HST scenario in R15 exists (as described in R4-2101707), for Rel-17 FR1 HST, how to specify the PSS/SSS detection, time index detection, and measurement period for deactivated SCell for the scenario of intra-f measurement with MG to support HST. Candidate options: - Option 1 (HW): When the issues on legacy deactivated SCell for non-HST scenario is fixed, the enhanced requirements on deactivated SCell in HST scenario can follow the similar rule as activated SCell. - Option 2 (CMCC, Huawei, Ericsson, MediaTek): considering that for non-HST scenario, current TS38.133 do not specify the cell identification requirements for deactivated SCell for intra-frequency measurement with measurement gap, for Rel-17 FR1 HST, it is proposed to focus on the discussion of enhancement on cell identification requirements for deactivated SCell(s) for intra-frequency measurement without MG firstly. The enhanced requirements specified for deactivated SCell(s) for intra-f without MG can be reused for deactivated SCell(s) for intra-f with MG. - Option 3 (QC): Deactivated Scell intra-frequency with gap may not be a valid scenario, since gap is required for intra-frequency measurement only when SSB is outside of active BWP, but deactivated Scell doesn’t have an “active BWP”, therefore this scenario is not valid Recommendations for 2nd round: Background: The issue on cell identification requirements for deactivated SCell with MG for non-HST scenario is discussed in Rel-15 maintenance AI. Based on the 1st round discussion of email thread #201, companies share the same understanding that deactivated SCell measurement does not belong to intra-frequency measurement with MG, since deactivated SCell does not have active BWP. In the 2nd round discussion, based on above background, companies are encouraged to check whether following suggestion is OK. - Following the same approach in non-HST scenario, for deactivated SCell, only consider the case without measurement gap. |

Suggestion on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| #1 | WF on RRM for FR1 HST A single WF will be used to cover all the topics. | CMCC |

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

Discussion on 2nd round (if applicable)

Sub-topic 2-1: requirements for deactivated Scell for the scenario of intra-frequency measurement without measurement gap

Issue 2-1-1: enhancement on PSS/SSS detection for deactivated Scell, intra-frequency measurement without measurement gap

- Option 1: similar enhancement to PCell measurement in R16 HST can be used as baseline for PSS/SSS detection enhancement on deactivated Scell.

- Option 2:

| DRX cycle | TPSS/SSS_sync_intra |
| --- | --- |
| No DRX | 5 x measCycleSCell x CSSFintra |
| With DRX | Max(800ms, 5 x max(measCycleSCell, DRX cycle)) x CSSFintra |

Recommended WF

- Companies are encouraged to provide comments

| Company | Comments |
| --- | --- |
| Apple | Support option 1. We gave example in 1st round that sometimes scaling factor is still needed. On the other hand, currently most of the HST cannot reach 500km/h. NW can still signal the flag if the train is moving at somewhat below 500km/h, such as 300km/h. For this case we think NW may benefit form relatively large SMTC periodicity (compared with 500km/h) from overhead perspective. If the train is moving at 500km/h, probably NW won’t configure large SMTC. For this case, option 1 and option 2 would be the same. |
| Nokia, Nokia Shanghai Bell | As commented in the first round, it is not clear how 800 ms is computed in Option 2. The removal of the scaling factor 1.5 (in Option 2) seems reasonable but further analysis is necessary. Hence, Option 1 is preferred. |
| QC | We can support option 1, but would like to add a note to address Huawei’s comment on Kp: follow Kp usage for non-HST. |
| MediaTek | Support option 1 |
| CATT | Support option 1 |
| Ericsson | Support option1. But we would like to see further analysis of option2. |
| Huawei | Support option1 and thanks QC to add the note about Kp |

Issue 2-1-2: enhancement on time index detection for deactivated Scell, intra-frequency measurement without measurement gap

- Option 1: similar enhancement to PCell measurement in R16 HST can be used as baseline for time index detection enhancement on deactivated Scell.

- Option 2:

| DRX cycle | TSSB_time_index_intra |
| --- | --- |
| No DRX | 3 x measCycleSCell x CSSFintra |
| With DRX | 3 x max(measCycleSCell, DRX cycle)) x CSSFintra |

Recommended WF

- Companies are encouraged to provide comments

| Company | Comments |
| --- | --- |
| Apple | Support option 1. Similar to issue 2-1-1 above. |
| Nokia, Nokia Shanghai Bell | Option 1 for the same reason provided in Issue 2-1-1. |
| QC | We can support option 1 |
| MediaTek | Support option1 |
| CATT | Option 1 same as Issue 2-1-1 |
| Ericsson | Support option1 |
| Huawei | Support option 1, similar comments to Kp |

Issue 2-1-3: enhancement on measurement period for deactivated Scell, intra-frequency measurement without measurement gap

- Option 1: similar enhancement to PCell measurement in R16 HST can be used as baseline for measurement period enhancement on deactivated Scell.

- Option 2 (Qualcomm):

| DRX cycle | T SSB_measurement_period_intra |
| --- | --- |
| No DRX | 5 x measCycleSCell x CSSFintra |
| DRX cycle≤ 320ms | Max(800ms, 5 x max(measCycleSCell, DRX cycle)) x CSSFintra |
| DRX cycle> 320ms | 3 x max(measCycleSCell, DRX cycle) x CSSFintra |

Recommended WF

- Companies are encouraged to provide comments

| Company | Comments |
| --- | --- |
| Apple | Support option 1. Similar to issue 2-1-1 above. |
| Nokia, Nokia Shanghai Bell | Option 1 for the same reason provided in Issue 2-1-1. |
| QC | We understand companies’ concern on option 2. But reducing complexity is crucial for UE implementation either, hence we would like to see if the following option is acceptable. The difference between this option and option 1 is in 160ms < DRX cycle≤ 320ms, scaling factor is increased by one. Pcell measurement should run faster because it is detectable only when UE is closer to cell edge, hence measurement should run fast enough to determine handover, especially for high speed scenario. However, deactivated cell can be identified and measured anywhere with serving cell coverage, hence slightly relaxed requirement to reduce UE implementation complexity is preferred. DRX cycle; T SSB_measurement_period_intra. No DRX; 5 x measCycleSCell x CSSFintra. DRX cycle≤ 320ms; ceil(5 x M2 x Kp) x max(measCycleSCell, DRX cycle)) x CSSFintra. DRX cycle> 320ms; ceil( Y x Kp ) x max(measCycleSCell, DRX cycle) x CSSFintra. M2 = 1.5 if SMTC periodicity > 40 ms, otherwise M2=1 Y=3 when SMTC <= 40ms, Y=5 when SMTC > 40ms. |
| MediaTek | Support option 1 |
| Ericsson | Support option 2. But we are open to further analysis of slight relaxation in option2, same as issue 2-1.1. |
| Huawei | Support option 1, similar comments to Kp. We also understand the motivation of Qualcomm, it can be further discussed. |

Sub-topic 2-2: requirements for deactivated Scell for the scenario of intra-frequency measurement with measurement gap

Issue 2-2-1: if the issue on cell identification requirements for SCell for non-HST scenario in R15 exists (as described in R4-2101707), for Rel-17 FR1 HST, how to specify the PSS/SSS detection, time index detection, and measurement period for deactivated Scell for the scenario of intra-f measurement with MG to support HST.

Background: The issue on cell identification requirements for deactivated Scell with MG for non-HST scenario is discussed in Rel-15 maintenance AI. Based on the 1st round discussion of email thread #201, companies share the same understanding that deactivated Scell measurement does not belong to intra-frequency measurement with MG, since deactivated Scell does not have active BWP.  In the GTW discussion of email thread #201, following agreement was reached:

Not to introduce the intra-frequency de-activated Scell measurement requirement with MG.

Recommended WF

- companies are encouraged to check whether following suggestion is OK.

  - Following the same approach in non-HST scenario, for deactivated Scell, no need to consider the case of intra-frequency measurement with measurement gap.

| Company | Comments |
| --- | --- |
| Apple | Support recommended WF. |
| Nokia, Nokia Shanghai Bell | The recommended WF is OK. |
| QC | Support recommended WF. |
| MediaTek | Support recommended WF. |
| CMCC | OK with recommended WF. |
| CATT | Support recommended WF |
| Ericsson | Support recommended WF |
| Huawei | Support recommended WF |

Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| XXX | Based on 2nd round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

Topic #3: SCell activation/deactivation delay requirement

Main technical topic overview. The structure can be done based on sub-agenda basis.

Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

Sub-topic 3-1: SCell activation/deactivation delay requirement

Sub-topic description:

Open issues and candidate options before e-meeting:

Issue 3-1-1: whether SCell activation delay requirement need to be enhanced for HST

- Proposals

  - Option 1 (Apple): Yes

  - Option 2 (Qualcomm, HW, Ericsson): No, legacy SCell activation delay can be reused in high speed scenario

  - Option 3 (CMCC): FFS

- Recommended WF

  - More discussion is needed

Issue 3-1-2: if the answer to Issue 3-1-1 is Yes, how to enhance SCell activation/deactivation delay requirement for HST

- Proposals

  - Option 1 (Apple): RAN4 can consider defining requirements for SCell activation with UE specific RS (such as TRS) for fine time tracking and AGC adjustment

- Recommended WF

  - More discussion is needed

Issue 3-1-3: whether SCell deactivation delay requirement need to be enhanced for HST

- Proposals

  - Option 1 (Ericsson): No, legacy SCell deactivation delay can be reused in high speed scenario

- Recommended WF

  - Companies are encouraged to provide views on this issue

Companies views’ collection for 1st round

Open issues

| Company | Comments |
| --- | --- |
| Apple | Issue 3-1-1: whether SCell activation delay requirement need to be enhanced for HST As UE vendor, we are perfectly fine with option 2. Actually, in our contribution we didn’t imply delay requirement “need” to be enhanced. Compared with measurement delay, activation delay is not the bottle neck. Nevertheless, we do mention three candidate approaches to facilitate SCell activation procedure since it is in the WID scope. However, none of the three options include any update on SCell activation delay requirements. Issue 3-1-2: if the answer to Issue 3-1-1 is Yes, how to enhance SCell activation/deactivation delay requirement for HST This is being discussed in R17 MR-DC. we don’t expect to spend any time in HST WI to discuss this issue. The reason we mention this is because in HST WID scope SCell activation delay enhancement is in the scope. Issue 3-1-3: whether SCell deactivation delay requirement need to be enhanced for HST Seems 3-1-3 is same as 3-1-1. |
| Huawei | Issue 3-1-1: support option 2. UE behaviours for SCell activation including PSS/SSS detection, AGC adjustment and fine time tracking are indispensable. Moreover there is almost no margin room for each step. In high speed scenario, in order to expedite SCell activation, only known SCell is triggered to be activated. Issue 3-1-3: we think this issue has the same meaning with issue 3-1-1. |
| Ericsson | Sub-topic 3-1 Issue 3-1-1: support option 2. Issue 3-1-3: same as Issue 3-1-1 |
| Nokia, Nokia Shnaghai Bell | Sub topic 3-1: Issue 3-1-1: Option 2 is OK. Issue 3-1-3: See Issue 3-1-1. |
| MediaTek | Issue 3-1-1: support option 2 Issue 3-1-3: support option 1 |
| QC | Issue 3-1-1: support option 2 Issue 3-1-3: support option 1 |
| CMCC | Issue 3-1-1: we would like to have further check. Issue 3-1-3: Ok with option 1 |
| CATT | Issue 3-1-1: support option 2 Issue 3-1-3: support option 1 |

CRs/TPs comments collection

Major close to finalize Wis and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going Wis, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| YYY | Company A |
|  | Company B |
| YYY | Company A |
|  | Company B |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#3-1 | Issue 3-1-1: whether SCell activation delay requirement need to be enhanced for HST Candidate options: - Option 1 (Qualcomm, HW, Ericsson, Apple, Nokia, MTK, CATT): No, legacy SCell activation delay can be reused in high speed scenario - Option 2 (CMCC): FFS Recommendations for 2nd round: Companies are suggested to have further check whether option 1 is acceptable. Issue 3-1-2: if the answer to Issue 3-1-1 is Yes, how to enhance SCell activation/deactivation delay requirement for HST Recommendations for 2nd round: Pending on the discussion on Issue 3-1-1. Can be deprioritized. For the 2nd discussion, suggest to focus on the discussion on issue 3-1-1. Issue 3-1-3: whether SCell deactivation delay requirement need to be enhanced for HST Tentative agreements: Legacy SCell deactivation delay can be reused in high speed train scenario. |

Suggestion on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| #1 | WF on RRM for FR1 HST A single WF will be used to cover all the topics. | CMCC |

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

Discussion on 2nd round (if applicable)

Issue 3-1-1: whether SCell activation delay requirement need to be enhanced for HST

- Option 1: No, legacy SCell activation delay can be reused in high speed scenario

- Option 2: FFS

Recommended WF

- Companies are encouraged to have further check whether option 1 is acceptable

| Company | Comments |
| --- | --- |
| Apple | Support option 1. Activation procedure itself is not the bottleneck for CA operation in HST. |
| Nokia, Nokia Shanghai Bell | Option 1 because the non-HST requirements are sufficient for CA under HST scenarios. |
| QC | Support option 1. |
| MediaTek | Support option 1. |
| CMCC | Need to further check. We prefer to keep it open for this meeting |
| Ericsson | Support option 1 |
| Huawei | Support option 1 |

Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| XXX | Based on 2nd round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

Topic #4: other

Main technical topic overview. The structure can be done based on sub-agenda basis.

Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

Sub-topic 4-1: inter-frequency measurement

Issue 4-1-1: whether to discuss inter-frequency measurement enhancement under high speed scenario

- Proposals

  - Option 1 (HW, Apple): It is recommended to discuss the necessity of inter-frequency measurement enhancement under high speed scenario

  - Option 2 (MTK): No

- Recommended WF

  - Companies are encouraged to provide views on this issue

Sub-topic 4-2: other requirements

Issue 4-2-1: except the requirements related with Scell activation/deactivation, other requirements to be discussed in work item of HST with CA

- Proposals

  - Option 1 (MTK): The requirements of timing, interruption, UL carrier RRC reconfiguration delay, link recovery (BFD/CBD), CSSF, Intra-frequency measurement, L1-RSRP measurement and measurement accuracy for L1-SINR measurement shall be further discussed in the work item of HST with CA.

- Recommended WF

  - Companies are encouraged to provide views on this issue

Companies views’ collection for 1st round

Open issues

| Company | Comments |
| --- | --- |
| Apple | Issue 4-1-1: whether to discuss inter-frequency measurement enhancement under high speed scenario We also support option 1, as mentioned in our discussion paper R4-2100224 proposal 4. Before adding a target cell as a candidate Scell, typically network would configure inter-frequency measurement on target CC. Thus, timely inter-frequency measurement is needed. Otherwise network may blindly add Scell without having measurement report to save time. Issue 4-2-1: except the requirements related with Scell activation/deactivation, other requirements to be discussed in work item of HST with CA Even though problems are not obviously identified, we think option 1 can be a good starting point. More study is needed to before we conclude that enhancement is needed. |
| Huawei | Issue 4-1-1: support option 1. Whether CA configuration is applied depends on the service load and network deployment. The possible scenario is that UE is configured with single carrier due to light load and afterwards multiple carriers are configured due to increased service load. The typical use case of Scell addition is that the candidate Scells were measured. In addition, the Pcell/Scell change may also need the measurement result on inter-frequency carrier. However in R16 HST scenario, no inter-frequency measurement is not considered. To better support transition between CA and single carrier and Pcell/Scell change, the inter-frequency measurement under high speed scenario may need to be considered. Issue 4-2-1: what’s enhancement to timing? In our understanding, the practical network deployments are considered in HST scenario as well. Also we are not clear about what’s interruption reduction in HST. |
| Ericsson | Issue 4-1-1: support option1. Measurement on Scell frequency is necessary to CA. For SCell, the inter-frequency measurement is reasonable to achieve expected offload effect. Issue 4-2-1: Is the focus of the question differentiation between HST and HST CA? |
| Nokia, Nokia Shanghai Bell | Sub topic 4-1: Issue 4-1-1: Option 1 because inter-frequency measurement enhancements are needed to support CA under HST. Sub topic 4-2: Issue 4-2-1: Should “intra-frequency measurement” be removed from the list in Option 1 because it is already discussed in sub-topics 1 and 2? We can consider each of the requirements on a case-by-case basis to determine if any of these needs to be enhanced. |
| MediaTek | Issue 4-1-1: We support option 2. In our understanding, the deactivated SCell measurement is more preferred than inter-frequency measurement. Because the measurement gap is needed for UE to perform inter-frequency measurement, e.g. 6ms interruption per 40 ms, and it would be an overhead (some resources may overlapped with measurement gap) for UE in HST scenario. On the other hand, measurement gap is not needed for UE to measure the deactivated SCell and the interruption of deactivated SCell measurement is sparse in time domain. Thus, the overhead is smaller than inter-frequency measurement. Issue 4-2-1: Our intention is to propose some related SCell requirements here to study whether the requirement shall be enhanced for Rel-17 or not. - For timing, we think the MRTD/MTTD requirement may be different if we consider non co-located deployment. - For link recovery, we need a confirmation on whether to consider eMIMO which introduce SCell BFD with the evaluation period scaled by number of the band. - For the CSSF, we wonder if a upper limit is needed to avoid that the measurement period is too long. - For L1-SINR measurement accuracy, issue follow the same story as SS-SINR in which we discussed an upper limit for SNR side condition. For other requirements, we have no strong view. All comment are welcome. |
| QC | Issue 4-1-1: We are open to discuss the necessity of inter-frequency measurement enhancement Issue 4-2-1: for the issues MediaTek raised above, we don’t see the connection of first two to HST, and we don’t see the feasibility of CSSF enhancement given the same hardware constraint as non-HST scenario. L1-SINR requirement applicability rule in fact is not restricted to Pcell from the current wording in 38.133, we also don’t see any change is needed. |
| CMCC | Issue 4-1-1: We are open to have further discussion. |

CRs/TPs comments collection

Major close to finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| YYY | Company A |
|  | Company B |
| YYY | Company A |
|  | Company B |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#4-1 | Issue 4-1-1: whether to discuss inter-frequency measurement enhancement under high speed scenario Candidate options: - Option 1 (HW, Apple, Ericsson, Nokia): It is recommended to discuss the necessity of inter-frequency measurement enhancement under high speed train scenario - Option 2 (MTK): No - Option 3 (QC, CMCC) open to discuss Recommendations for 2nd round: Further discussion is needed. |
| Sub-topic#4-2 | Issue 4-2-1: except the requirements related with Scell activation/deactivation, other requirements to be discussed in work item of HST with CA Candidate options: - Option 1 (MTK): The requirements of timing, interruption, UL carrier RRC reconfiguration delay, link recovery (BFD/CBD), CSSF, Intra-frequency measurement, L1-RSRP measurement and measurement accuracy for L1-SINR measurement shall be further discussed in the work item of HST with CA. In the 1st round discussion, 1 company think option 1 could be a good starting point to have further check. While other companies have some questions for option 1. More discussion is needed. Recommendations for 2nd round: Further discussion is needed. |

Suggestion on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| #1 | WF on RRM for FR1 HST A single WF will be used to cover all the topics. | CMCC |

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

Discussion on 2nd round (if applicable)

Issue 4-1-1: whether to discuss inter-frequency measurement enhancement under high speed scenario

- Option 1: Yes

- Option 2: No

Recommended WF

- Companies are encouraged to provide comments

| Company | Comments |
| --- | --- |
| Apple | Prefer option 1. Since this is the first meeting, we are open for further study. |
| Nokia, Nokia Shanghai Bell | Option 1. |
| QC | Support option 1, open to further study. |
| MediaTek | More discussion is needed. As we mentioned in first round, we think the UE can perform measurement starting from RRC receiving, i.e., skip the procedure of the inter-frequency measurement. |
| CMCC | OK with option 1, open to further study. |
| CATT | It depends on deployment scenario. It is possible. Therefore, we support option 1 to do further study |
| Ericsson | Support option 1, open to further study. |
| Huawei | Support option1. The same comments in 1st round. Whether CA configuration is applied depends on the service load and network deployment. The possible scenario is that UE is configured with single carrier due to light load and afterwards multiple carriers are configured due to increased service load. The typical use case of Scell addition is that the candidate Scells were measured. In addition, the Pcell/Scell change may also need the measurement result on inter-frequency carrier. However in R16 HST scenario, no inter-frequency measurement is not considered. To better support transition between CA and single carrier and Pcell/Scell change, the inter-frequency measurement under high speed scenario may need to be considered. |

Issue 4-2-1: except the requirements related with Scell activation/deactivation, other requirements to be discussed on whether to be enhanced to support HST with CA

- Option 1: The requirements of timing, interruption, UL carrier RRC reconfiguration delay, link recovery (BFD/CBD), CSSF, L1-RSRP measurement and measurement accuracy for L1-SINR measurement shall be further discussed in the work item of HST with CA.

Recommended WF

- Companies are encouraged to provide comments

| Company | Comments |
| --- | --- |
| Apple | Open to further discussion. |
| Nokia, Nokia Shanghai Bell | L1-RSRP measurement and measurement accuracy were discussed in Rel-16 HST. As the deployment scenario and the maximum speed of trains are the same, the enhanced requirement can be reused. For the other requirements, we are open for discussion. |
| QC | Our comments in the first round hasn’t been addressed by proponents of studying these requirements, would like to have further clarification before we agree with all the items listed in option 1. Repeat the comment with some modification here: - Interruption, UL carrier RRC reconfiguration delay: we don’t see why this has to be enhanced under high speed scenario - link recovery (BFD/CBD): this has been discussed in R16, anything change by taking CA into consideration? - CSSF: we don’t see how this can be enhanced in high speed, given that the hardware constraint is the same as non-HST - L1-RSRP measurement, measurement accuracy for L1-SINR measurement: this is agreed in R16, any change needed? |
| MediaTek | Our intention is to provide an overview to discuss whether the HST have impact on the existing requirement. We agree some existing requirements can be reused for Rel-17 HST. So far, we can agree with the requirement of interruption, UL carrier RRC reconfiguration delay, L1-RSRP measurement, timing can reuse the Rel-16 requirement. - For link recovery, SCell BFD/CBD with the evaluation period scaled by number of the band may increase with the number of bands without SpCell. For example, according to the Table 8.5.3.2-1, if P=2, MBFD=10, PBFD=3 and TDRX=160ms, the TEvaluate_BFD_CSI-RS =14.4 s Table 8.5.3.2-1: Evaluation period TEvaluate_BFD_CSI-RS for FR1 Configuration; TEvaluate_BFD_CSI-RS (ms). no DRX; Max(50, [MBFD P PBFD] TCSI-RS). DRX cycle ≤ 320ms; Max(50, [1.5 × MBFD P PBFD] Max(TDRX, TCSI-RS)). DRX cycle > 320ms; [MBFD P PBFD] TDRX. Note: TCSI-RS is the periodicity of CSI-RS resource in the set . TDRX is the DRX cycle length.. Thus, if RAN4 agreed this is not applicable to Rel-17 HST, we think RAN4 needs to add one note to capture this case. - According to Table 9.1.5.1.1-1 in TS 38.133, the measurement requirement is scaled by CSSF as follows. We suggest RAN4 can define an upper limit for CSSF in Rel-17 HST. Otherwise, the measurement time would be too long. - For L1-SINR measurement accuracy, in Rel-16, we introduce an upper bound for SSB SINR Es/Iot as following Table because of the inter carrier interference. Thus, follow similar logic, RAN4 need to further study whether an upper bound shall be introduced for L1-SINR. . |
| CATT | Open to further study |
| Ericsson | Open to further study |
| Huawei | Need some clarifications. E.g., what’s enhancement to timing? In our understanding, the practical network deployments are considered in HST scenario as well. Also we are not clear about what’s interruption reduction in HST. |

Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| XXX | Based on 2nd round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |