3GPP TSG-RAN WG4 Meeting #98-e 	R4-2103737

# Electronic Meeting, January 25  February 5, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | DraftCR | rev | 1 | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | TC5 and TC6: UE Rx-Tx time difference measurement requirements for FR1 and FR2 in SA |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos-Perf |  |  |  |  |  | Date: |  |  | 2021-01-15 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Test cases for UE Rx-Tx time difference measurements are missing |  |  |  |  |  |  |  |  |
| Summary of change: |  | Test cases for UE Rx-Tx time difference measurements are added |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test cases for UE Rx-Tx time difference measurements are missing |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New clauses: A.6.6.9, A.6.6.9.1, A.6.6.9.1.1, A.6.6.9.1.2, A.6.6.9.2, A.6.6.9.2.1, A.6.6.9.2.2; A.7.6.7, A.7.6.7.1, A.7.6.7.1.1, A.7.6.7.1.2, A.7.6.7.2, A.7.6.7.2.1, A.7.6.7.2.2. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revised from R4-2102555. |  |  |  |  |  |  |  |  |

--- change 1 ---

### A.6.6.9 UE Rx-Tx time difference measurements

#### A.6.6.9.1 UE Rx-Tx time difference measurement period for PRS on a serving frequency

##### A.6.6.9.1.1	Test purpose and environment

The purpose of the test is to verify that the UE Rx-Tx measurement meets the requirements specified in clause 9.9.4.5 in an environment with fading propagation conditions.

The supported test configurations in listed in Table A.6.6.9.1.1-1. The test parameters are as given in Tables TBD.

Table A.6.6.9.1.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations. |  |

Editor’s note: Table with common parameters is TBD.

Editor’s note: Table with cell-specific parameters is TBD.

Editor’s note: Table with SRS configuration is TBD.

Editor’s note: Table with DRX configuration is TBD.

##### A.6.6.9.1.2	Test requirements

The UE Rx-Tx time difference measurement time fulfils the requirements specified in clause 9.9.4.5.

The rate of the correct events for each neighbour cell observed during repeated tests shall be at least 90%, where the reported UE Rx-Tx measurement for each correct event shall be within the UE Rx-Tx reporting range specified in clause 10.1.25.3.1.

#### A.6.6.9.2 UE Rx-Tx time difference measurement period for PRS on multiple frequencies

##### A.6.6.9.2.1	Test purpose and environment

The purpose of the test is to verify that the UE Rx-Tx measurement meets the requirements specified in clause 9.9.4.5 in an environment with fading propagation conditions.

The supported test configurations in listed in Table A.6.6.9.1.1-1. The test parameters are as given in Tables TBD.

Table A.6.6.9.1.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations. |  |

Editor’s note: Table with common parameters is TBD.

Editor’s note: Table with cell-specific parameters is TBD.

Editor’s note: Table with SRS configuration is TBD.

Editor’s note: Table with DRX configuration is TBD.

##### A.6.6.9.2.2	Test requirements

The UE Rx-Tx time difference measurement time fulfils the requirements specified in clause 9.9.4.5.

The rate of the correct events for each neighbour cell observed during repeated tests shall be at least 90%, where the reported UE Rx-Tx measurement for each correct event shall be within the UE Rx-Tx reporting range specified in clause 10.1.25.3.1.

--- change 2 ---

### A.7.6.7 UE Rx-Tx time difference measurements

#### A.7.6.7.1 UE Rx-Tx time difference measurement period for PRS on a serving frequency

##### A.7.6.7.1.1	Test purpose and environment

The purpose of the test is to verify that the UE Rx-Tx measurement meets the requirements specified in clause 9.9.4.5 in an environment with fading propagation conditions.

The supported test configurations in listed in Table A.7.6.7.1.1-1. The test parameters are as given in Tables TBD.

Table A.7.6.7.1.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |

Editor’s note: Table with common parameters is TBD.

Editor’s note: Table with cell-specific parameters is TBD.

Editor’s note: Table with SRS configuration is TBD.

Editor’s note: Table with DRX configuration is TBD.

##### A.7.6.7.1.2	Test requirements

The UE Rx-Tx time difference measurement time fulfils the requirements specified in clause 9.9.4.5.

The rate of the correct events for each neighbour cell observed during repeated tests shall be at least 90%, where the reported UE Rx-Tx measurement for each correct event shall be within the UE Rx-Tx reporting range specified in clause 10.1.25.3.1.

#### A.7.6.7.2 UE Rx-Tx time difference measurement period for PRS on multiple frequencies

##### A.7.6.7.2.1	Test purpose and environment

The purpose of the test is to verify that the UE Rx-Tx measurement meets the requirements specified in clause 9.9.4.5 in an environment with fading propagation conditions.

The supported test configurations in listed in Table A.7.6.7.2.1-1. The test parameters are as given in Tables TBD.

Table A.7.6.7.2.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |

Editor’s note: Table with common parameters is TBD.

Editor’s note: Table with cell-specific parameters is TBD.

Editor’s note: Table with SRS configuration is TBD.

Editor’s note: Table with DRX configuration is TBD.

##### A.7.6.7.2.2	Test requirements

The UE Rx-Tx time difference measurement time fulfils the requirements specified in clause 9.9.4.5.

The rate of the correct events for each neighbour cell observed during repeated tests shall be at least 90%, where the reported UE Rx-Tx measurement for each correct event shall be within the UE Rx-Tx reporting range specified in clause 10.1.25.3.1.