3GPP TSG-RAN WG4 Meeting #98-e	R4-2103753

Electronic Meeting, January 25th – February 5th, 2021

Agenda item:			4.9, 5.4

Source:	Moderator (Intel Corporation)

Title:	Email discussion summary for [98e][314] Demod_R15_Maintenance

Document for:	Information

# Introduction

The scope of this email thread is:

- Rel-15 NR maintenance – UE demodulation and CSI requirements (AI 4.9.1 and 4.9.2)

- Rel-15 NR maintenance – BS demodulation requirements (AI 4.9.3)

- Note: There are no tdocs submitted in this meeting for LTE maintenance (up to Rel15) (AI 5.4)

Email discussion targets for the 1st round and 2nd round

- 1st round:

  - Collect comments for NR CRs.

- 2nd round:

  - Collect comments for revised NR CRs from the 1st round.

# Topic #1: Rel-15 NR maintenance - UE demodulation and CSI requirements

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2101441 | Ericsson | Rel-15 CR with the following changes for TS 38.101-4: - Changed 8.375ms -> 1.375ms CSI reporting delay - Changed maximum throughput for FDD case (Add one digit) - Changed the channel bits for slots including TRS to 50880 (= (12*12-18-6)*106*4), where it is assumed 12 OFDM symbols, 18 DMRS symbols, 6 TRS symbols |
| R4-2101442 | Ericsson | Rel-16 Cat A CR of R4-2101441 |
| R4-2102824 | Qualcomm Incorporated | Rel-15 CR with the following changes for TS 38.101-4: - EPRE ratio between LTE CRS and NR SSS is defined. - In OCNG pattern, CORESETs is removed from note 2. |
| R4-2102828 | Qualcomm Incorporated | Rel-16 Cat A CR of R4-2102828 |
| R4-2101945 | Intel Corporation | Rel-15 CR with the following changes for TS 38.101-4: - Add notes for all RI requiremnets with reference to table with CSI FRCS - Added FR1 FRCs for Rank 3 and Rank 4 cases - Corrected “Available RE-s” for FR2 FRC, because PT-RS overhead was not taken into account. |
| R4-2101946 | Intel Corporation | Rel-16 Cat A CR of R4-2101945 |
| R4-2102870 | Apple | Rel-15 CR with the following changes for TS 38.101-4: - Updated test parameters with Aperiodic reporting with PDCCH config for DCI format 0_1. |
| R4-2102871 | Apple | Rel-16 Cat A CR of R4-2102870 |

## Open issues summary

N/A

## Companies views’ collection for 1st round

### Open issues

N/A

### CRs comments collection

| CR number | Comments collection |
| --- | --- |
| R4-2101441 | Qualcomm: Changing the delay is not correct. For Periodic CSI reporting, delays are much larger. 4ms delay on UE side and then assuming same delay on gNB side, 8.375ms is correct. |
|  | Intel: Changes for Table A.3.2.1.1-8. It is not clear why we need 4 digits after point for this case. All NR FRCs are defined with 3 digits after point. |
|  | Apple: The CQI test in AWGN is not run in follow CQI mode, so this change would not have any impact to test performance in our understanding. Same comment as Intel on number of digits after decimal point. |
|  | Ericsson2: Thank you very much for your comments. We can keep ‘CQI/RI/PMI delay’ and ‘Max. Throughput averaged over 2 frames’ as is. Then this CR changes only channel bits in Table A.3.2.2.2-10. |
| R4-2102824 | Intel: Changes for OCNG: We prefer to keep CORESET in the note. Existing requirements are defined for TDM of CORESET and PDSCH. Therefore, we agree that part of the note with CORESET information is not applicable for existing requirements. Same time, based on our understanding, FDM of CORESET and PDSCH is possible form Phy design point of view and may be considered in future. |
|  | Huawei: We share the similar view with Intel as for OCNG updates; For EPRE ratio between LTE CRS and NR SSS, it is better to capture it in Annex C.3.1 to keep all downlink physical channel setup together. |
|  | Qualcomm: We are ok with Intel and Huawei’s comments. We can revise it to only capture EPRE Ratio in Annex C.3.1 |
| R4-2101945 | Qualcomm: Table A.4-3 has same title as Table A.4-2. May be we should add that this new table is for Rank3 and Rank4 in the title. |
|  | Intel: Suggestion from QC is fine for us. We can call Table A.4-2 as “Mapping of CQI Index to Information Bit payload (CQI table 2, Rank 1 and 2)” and Table A.4-3 as “Mapping of CQI Index to Information Bit payload (CQI table 2, Rank 3 and 4)”. |
|  | Ericsson: We are ok to add this note. On top of that, could you also remove ‘]’ from csi-ReportingBand for Test 2 in Table 8.4.2.2-1? |
|  | Intel: To Ericsson: We will remove ‘]’ in the revised version. |
| R4-2102870 | Qualcomm: Any reason to choose AL4 instead of AL8 for UL DCI? Please also add FR2 fading CQI reporting which is aperiodic. |
|  | Intel: - Based on our understanding, information about slots for PDCCH monitoring and TCI state is missing for PDCCH 0_1. - Also, probably, it is better to define PDCCH 0_1 in General table (i.e. Table 6.1.2-1 and Table 6.1.2-1) as “PDCCH configuration #2” and add note that this configuration is for tests with Aperiodic CSI report (we also can list all test cases). |
|  | Huawei: Based on our understanding, during the testing, TE has specific UL scheduling formats for HARQ feedback and aperiodic triggering, etc., it is not necessary to explicitly define it in RAN4 specification if no special reasons to use other DCI formats for UL scheduling for testing. |
|  | Ericsson: We have same view as Huawei. For example, some CQI tests in LTE (TS36.101) also use aperiodic CSI reporting with PUSCH, but TS36.101 does not specify the PDCCH configuration for CSI request. The detailed configuration is specified in RAN5 specification, for example, TS36.321-1 9.3.1.1.1.4.2: SS schedules the UL transmission every 5 ms to carry the PUSCH CQI feedback via PDCCH DCI format 0 with CQI request bit set to 1 and I_MCS=29 and N_PRB allocated to be less or equal to 4.. Ericsson2: We understand it is good to specify PDCCH to trigger aperiodic CSI-RS. We prefer the suggestion by Intel, put this configuration in the General table. |
|  | Anritsu: We also prefer the Aggregation level to be set to 8, unless there is a strong reason to use 4. To help searches I guess we should specify “Aggregation level” as the abbreviation AL isn’t defined in 38.101-4, even if all the experts know.. |
|  | Apple: To Ericsson and HW: Apart for DCI 0-1 configuring aperiodic reporting, it also triggers Aperiodic CSI-RS used for CSI computation, hence we think it is necessary to specify this. After further check AL 8 would actually be the correct configuration. We would not be able to configure AL 4 for DCI 0-1 and AL8 for DCI 1-1 based on UESS configuration rules. To Intel: we considered adding it in common parameters table and is a much simpler change, but following the existing format that only common parameters are specified in 6.1.2-1 and any additional parameters in specific test parameter tables, we updated every table. But we are fine with your suggestion if all can companies agree to it. To Anritsu : We will match the existing PDCCH configuration in terms or wording. |

## Summary for 1st round

### Open issues

Status summary

### CRs

| CR number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2101441 | To be revised |
| R4-2102824 | To be revised |
| R4-2101945 | To be revised |
| R4-2102870 | To be revised |

## Discussion on 2nd round (if applicable)

## Summary on 2nd round (if applicable)

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| XXX | Based on 2nd round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

# Topic #2: Rel-15 NR maintenance - BS demodulation requirements

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2100548 | Nokia, Nokia Shanghai Bell | Rel-15 CR with the following changes for TS 38.141-1: - Include the sentence “In tests performed with signal generators a synchronization signal may be provided from the BS to the signal generator, to enable correct timing of the wanted signal” from the radiated test specificaiton also in the conducted test specification. |
| R4-2100549 | Nokia, Nokia Shanghai Bell | Rel-16 Cat A CR of R4-2100548 |
| R4-2100550 | Nokia, Nokia Shanghai Bell | Rel-17 Cat A CR of R4-2100548 |
| R4-2100551 | Nokia, Nokia Shanghai Bell | Rel-15 CR with the following changes for TS 38.141-1: - Modify the first sentence of the various “Applicability of requirements for different channel bandwidths” clauses, to clarify that test requirements apply for all supported CBWs. |
| R4-2100552 | Nokia, Nokia Shanghai Bell | Rel-16 Cat A CR of R4-2100551 |
| R4-2100553 | Nokia, Nokia Shanghai Bell | Rel-17 Cat A CR of R4-2100551 |
| R4-2100554 | Nokia, Nokia Shanghai Bell | Rel-15 CR with the following changes for TS 38.141-2: - Modify the first sentence of the various “Applicability of requirements for different channel bandwidths” clauses, to clarify that test requirements apply for all supported CBWs. |
| R4-2100555 | Nokia, Nokia Shanghai Bell | Rel-16 Cat A CR of R4-2100554 |
| R4-2100556 | Nokia, Nokia Shanghai Bell | Rel-17 Cat A CR of R4-2100554 |
| R4-2100557 | Nokia, Nokia Shanghai Bell | Paper contains the explanation and motivation of CRs (R4-2100548, R4-2100551 and R4-2100554) introducing BS demodulation specification text changes pertaining to the applicability of requirements for different channel bandwidths and synchronization signals in conducted test setups. |

## Open issues summary

N/A

## Companies views’ collection for 1st round

### Open issues

N/A

### CRs comments collection

| CR number | Comments collection |
| --- | --- |
| R4-2100548 | Ericsson: Since it is now quite late to introduce changes to rel-15 and the change is to add some informative text, we would like to ask the proponent to clarify whether the change is really needed in rel-15. Regarding the change itself, it could be made more generic as follows: In tests performed with signal generators a synchronization signal may be provided between the BS and the signal generator, to enable correct timing of the wanted signal Ericsson2: To Nokia, in general, we should aim to minimize Rel-15 changes, but we’re OK to do this change. But if possible, we prefer more general formulation in both TS38.141-1/141-2. To moderator, if Nokia is fine, we would like ask CR for TS38.141-2 also. [Nokia]: We would be fine with the second CR. It can be attributed to Ericsson, if they like. Otherwise we can do it; assuming the Moderator agrees in general. |
|  | [Nokia]: In response to Ericsson’s comment. In our discussion paper motivating and explaining the CR [R4-2100557], we focus on the observations from the IAB WI, that the synchronization signal in the test setup (in particular its existence) seems to have been disregarded in many cases. As such it is recommended to clarify this part in 38.141-1 Additionally, the corresponding paragraph already exists in 38.141-2 and seems difficult to explain why it was omitted from 38.141-1, as this could be seen to potentially require two different test setups (w.r.t. synchronization) between conducted and radiated testing. It does not seem beneficial to us to only fix this issue from Rel-16 onwards, as long as changes to Rel-15 are still possible. In principal, we would be in favor of adding the generalized version of the synchronization signal text in 38.141-1, and to generalize the text already captured in 38.141-2. We did not propose it, to not run into issues with modifying existing agreed text. We would seek the guidance of the moderator, on the proposal to generalize the paragraph as indicated by Ericsson, in the understanding that a secondary CR for 38.141-2 adaptation would be necessary either this meeting or the next. |
|  | ZTE: We are fine with aligning 38.141-1 and 38.141-2 with regards to the synch signal between BS and signal generator. However, does this synchronization signal need to be standardized? Or it is already commonly known to both DUT and TE? If not, we may need to clarify what is the exact meaning of “the synchronization signal” under this context, e.g., does it refer to a “baseband clock” like signal? In which transmission form / frequency? [Nokia]: We don’t think that the exact implementation of the proprietary synchronization signal should be specified or even mentioned in the specification. Just specifying that it can be present (like it is done in 38.141-2 already) is necessary. Hopefully this will remove future discussion the likes we’ve seen in IAB… |
| R4-2100551 | Ericsson: In our view the original text is reasonably clear, but the change improves the clarity. The change seems relatively minor though and we would ask the proponent to clarify whether a Rel-15 change is needed at this late stage. Ericsson2: In general, we should aim to minimize Rel-15 changes. Since Nokia have produced a document and explained the need, we are OK on this occasion though. |
|  | [Nokia]: In response to Ericsson’s comment. In our discussion paper motivating and explaining the CR [R4-2100557], we have hinted at our observation of misinterpretation of the applicability rule by professionals. As such we see it as good practice to try and prevent future misunderstandings. In particular, for this part of the specification, which is currently poised for re-use in many other Rel-16 WIs. This also explains our efforts at such a late stage of Rel-15. Hopefully the inclusion of the detailed discussion paper to explain our reasoning, shows our intent to not just submit another CR to have one more CR at the end of Rel-15. |
|  | ZTE: we are ok to have the proposed clarification change. |
| R4-2100554 | Ericsson: In our view the original text is reasonably clear, but the change improves the clarity. The change seems relatively minor though and we would ask the proponent to clarify whether a Rel-15 change is needed at this late stage. Ericsson2: In general, we should aim to minimize Rel-15 changes. Since Nokia have produced a document and explained the need, we are OK on this occasion though. |
|  | [Nokia]: See comment above for R4-2100551. The difference is only 38.141-1 vs 38.141-2. |
|  | ZTE: we are ok to have the proposed clarification change. |

## Summary for 1st round

### Open issues

N/A

### CRs

| CR number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2100548 | To be revised |
| R4-2100551 | To be agreed |
| R4-2100552 | To be agreed (Rel-16 Cat A CR of R4-2100551) |
| R4-2100553 | To be agreed (Rel-17 Cat A CR of R4-2100551) |
| R4-2100554 | To be revised (Note: context is agreeable and need to fix cover sheet issue) |
| R4-2100555 | To be agreed (Rel-16 Cat A CR of R4-2100554) |
| R4-2100556 | To be agreed (Rel-17 Cat A CR of R4-2100554) |

New tdoc request

- Rel-15 Cat F CR for 38.141-2 with the following title: “CR for 38.141-2: BS demodulation synchronization in test setup”, company - Nokia, Nokia Shanghai Bell

- Rel-16 Cat A CR for 38.141-2 with the following title: “CR for 38.141-2: BS demodulation synchronization in test setup”, company - Nokia, Nokia Shanghai Bell

- Rel-17 Cat A CR for 38.141-2 with the following title: “CR for 38.141-2: BS demodulation synchronization in test setup”, company - Nokia, Nokia Shanghai Bell

## Discussion on 2nd round (if applicable)

## Summary on 2nd round (if applicable)

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| XXX | Based on 2nd round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |