3GPP TSG-RAN WG4 Meeting # 98-e	R4-2103792

# Electronic Meeting, 25 Jan - 05 Feb 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.145-2 | CR | 0280 | rev | 1 | Current version: | 15.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to TS 37.145-2: Corrections to conformance requirements, Rel-15 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15, AASenh_BS_LTE_UTRA-Perf, |  |  |  |  |  | Date: |  |  | 25-01-2021 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | During TFES drafting of the harmonized standard for AAS (EN 301 908 part 23) which is based on the AAS conformance specification the following errors in 37.145-2 were identified. These need to be corrected so part 23 and 37.145-2 are aligned. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The following errors have been corrected: 4.7 align with 38.141-2 and TFES (text from 37.105) 6.7.5.5.2-2b -4 corrected to +4 (-7+9(scaling)-2(TT)) 6.7.5.5.2-2c - missing table added for 1<f<3 GHz (table heading aligned with R5-2103887) 7.5.5.1.3 reference to table 7.4.5.1.3-1 should be 7.5.5.1.3-1, reference to 7.2.5.3 is to UTRA TDD sensitivity correct to general reference to sensitivity clause. Clause 7.5.5.3, reference to table 10.3.4-1,2,3 is a table in 37.105 should be Table 7.3.5.3-1,2,3 Clause 7.5.3.2 UTRA requirements for medium range and local area BS are missing Clause 7.5.3.3 E-UTRA requirements for medium range and local area BS are missing 7.2.5.5, 7.3.5.4, 7.4.5.3 and 7.9.5.2 NR specific requiremenyts have 4.2 to 6Ghz listed in table but there are no 4.2 to 6GHz MU or TT defined and no bands above 4.2 in AAS spec. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The requirements are not correct, drafting and approval of the harmonized standard is affected. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.7, 6.7.5.5.2, 7.2.5.5, 7.3.5.2, 7.3.5.3, 7.3.5.4, 7.4.5.3, 7.5.5.1.3, 7.5.5.3, 7.9.5.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | . |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

------------------------------ Modified section ------------------------------

## 4.7	Requirements for AAS BS capable of multi-band operation

For AAS BS capable of operation in multiple operating bands, the RF requirements in clause 6 and 7 apply separately to each supported operating band unless otherwise stated.

A hybrid AAS BS may be capable of supporting operation in multiple operating bands with one of the following implementations of TAB connectors in the transceiver array boundary:

-	All TAB connectors are single band TAB connectors.

-	Different sets of single band TAB connectors support different operating bands, but each TAB connector supports only operation in one single operating band.

-	Sets of single band TAB connectors support operation in multiple operating bands with some single band TAB connectors supporting more than one operating band.

-	All TAB connectors are multiband TAB connectors.

-	A combination of single band sets and multi-band sets of TAB connectors provides support of the hybrid AAS BS capability of operation in multiple operating bands.

Unless otherwise stated all requirements specified for an operating band apply only to the set of TAB connectors supporting that operating band.

In certain requirements it is explicitly stated that specific additions or exclusions to the requirement apply at multi-band TAB connectors as detailed in the requirement subclause. When referencing the NR specification 3GPP TS 38.104 [33] for a BS type 1-H the multi-band connector term is equivalent to a multi-band TAB connector in this specification.

In the case of an operating band being supported only by single band TAB connectors in a TAB connector TX min cell group or a TAB connector RX min cell group, single band requirements apply to that set of TAB connectors.

NOTE:	Each supported operating band needs to be operated separately during conformance testing on single band TAB connectors.

For a band supported by a TAB connector where the transmitted carriers are not processed in active RF components together with carriers in any other band, TX single band requirements shall apply. For a band supported by a TAB connector where the received carriers are not processed in active RF components together with carriers in any other band, RX single band requirements shall apply.

In the case of an operating band being supported only by multi-band TAB connectors supporting the same operating band combination in a TAB connector TX min cell group or a TAB connector RX min cell group, multi-band requirements apply to that set of TAB connectors.

The case of an operating band being supported by both multi-band TAB connectors and single band TAB connectors in a TAB connector TX min cell group or a TAB connector RX min cell group is not covered by the present release of this specification.

The case of an operating band being supported by multi-band TAB connectors which are not all supporting the same operating band combination in a TAB connector TX min cell group or a TAB connector RX min cell group is not covered by the present release of this specification.

An OTA AAS BS may be capable of supporting operation in multiple operating bands with one of the following implementations at the radiated interface boundary:

-	All RIBs are single band RIBs.

-	All RIBs are multiband RIBs.

-	A combination of single band RIBs and multi-band RIBs provides support of the OTA AAS BS capability of operation in multiple operating bands.

In certain requirements it is explicitly stated that specific additions or exclusions to the requirement apply at multi-band RIBs as detailed in the requirement subclause.

NOTE:	Each supported operating band needs to be operated separately during conformance testing for single RIBs.

For multi-band TAB connectors and multi-band RIBs supporting the bands for TDD, the RF requirements in the present specification assume no simultaneous uplink and downlink occur between the bands.

The RF requirements for multi-band TAB connectors and multi-band RIBs supporting bands for both FDD and TDD are not covered by the present release of this specification.

A RIB may operate multi-RAT where the individual RATs are operated in different RAT specific bands that partially or fully overlap; ΔfOBUE and ΔfOOB are according to the combined frequency range occupied by the overlapping bands.

------------------------------ Next modified section ------------------------------

Table 6.7.5.5.2-2a: Wide Area operating band unwanted emission mask (UEM) for BS supporting NR and not supporting UTRA in BC1 and BC3 bands below 1 GHz

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Minimum requirement (Note 1, 2) | Measurement bandwidth (Note 7) |
| --- | --- | --- | --- |
| 0 MHz f < 5 MHz | 0.05 MHz f_offset < 5.05 MHz | 3.8 dBm – 7/5(f_offset/MHz-0.05)dB | 100 kHz |
| 5 MHz f < min(10 MHz, fmax) | 5.05 MHz f_offset < min(10.05 MHz, f_offsetmax) | -3.2 dBm | 100 kHz |
| 10 MHz f fmax | 10.05 MHz f_offset < f_offsetmax | -7 dBm (Note 5) | 100 kHz |
| NOTE 1: For AAS BS supporting non-contiguous spectrum operation within any operating band, the minimum requirement within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap, where the contribution from the far-end sub-block or RF Bandwidth shall be scaled according to the measurement bandwidth of the near-end sub-block or RF Bandwidth. Exception is f ≥ 10 MHz from both adjacent sub blocks on each side of the sub-block gap, where the minimum requirement within sub-block gaps shall be -7dBm/100 kHz. NOTE 2: For AAS BS supporting multi-band operation with Inter RF Bandwidth gap < 2×ΔfOBUE the minimum requirement within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or RF Bandwidth on each side of the Inter RF Bandwidth gap, where the contribution from the far-end sub-block or RF Bandwidth shall be scaled according to the measurement bandwidth of the near-end sub-block or RF Bandwidth. |  |  |  |

Table 6.7.5.5.2-2b: Wide Area operating band unwanted emission mask (UEM) for BS supporting NR (except operation in Band n1) and not supporting UTRA in BC1 and BC3 bands > 3 GHz

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Minimum requirement (Note 1, 2) | Measurement bandwidth (Note 7) |
| --- | --- | --- | --- |
| 0 MHz f < 5 MHz | 0.05 MHz f_offset < 5.05 MHz | 4 dBm – 7/5(f_offset/MHz-0.05)dB | 100 kHz |
| 5 MHz f < min(10 MHz, fmax) | 5.05 MHz f_offset < min(10.05 MHz, f_offsetmax) | -3 dBm | 100 kHz |
| 10 MHz f fmax | 10.5 MHz f_offset < f_offsetmax | -6 dBm (Note 5) | 1 MHz |
| NOTE 1: For AAS BS supporting non-contiguous spectrum operation within any operating band, the minimum requirement within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap, where the contribution from the far-end sub-block shall be scaled according to the measurement bandwidth of the near-end sub-block. Exception is f ≥ 10 MHz from both adjacent sub blocks on each side of the sub-block gap, where the minimum requirement within sub-block gaps shall be -6dBm/1 MHz. NOTE 2: For AAS BS supporting multi-band operation with Inter RF Bandwidth gap < 2×ΔfOBUE the minimum requirement within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or RF Bandwidth on each side of the Inter RF Bandwidth gap, where the contribution from the far-end sub-block or RF Bandwidth shall be scaled according to the measurement bandwidth of the near-end sub-block or RF Bandwidth. |  |  |  |

Table 6.7.5.5.2-2c: Wide Area operating band unwanted emission mask (UEM) in BC1 and BC3 bands ≥ 1GHz and  3 GHz applicable for: BS supporting NR, not supporting NR operation in Band n1 and not supporting UTRA

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Minimum requirement (Note 1, 2) | Measurement bandwidth (Note 7) |
| --- | --- | --- | --- |
| 0 MHz f < 5 MHz | 0.05 MHz f_offset < 5.05 MHz | 3.8 dBm – 7/5(f_offset/MHz-0.05)dB | 100 kHz |
| 5 MHz f < min(10 MHz, fmax) | 5.05 MHz f_offset < min(10.05 MHz, f_offsetmax) | -3.2 dBm | 100 kHz |
| 10 MHz f fmax | 10.5 MHz f_offset < f_offsetmax | -6 dBm (Note 5) | 1 MHz |
| NOTE 1: For AAS BS supporting non-contiguous spectrum operation within any operating band, the minimum requirement within sub-block gaps is calculated as a cumulative sum of contributions from adjacent sub blocks on each side of the sub block gap, where the contribution from the far-end sub-block shall be scaled according to the measurement bandwidth of the near-end sub-block. Exception is f ≥ 10 MHz from both adjacent sub blocks on each side of the sub-block gap, where the minimum requirement within sub-block gaps shall be -6dBm/1 MHz. NOTE 2: For AAS BS supporting multi-band operation with Inter RF Bandwidth gap < 2×ΔfOBUE the minimum requirement within the Inter RF Bandwidth gaps is calculated as a cumulative sum of contributions from adjacent sub-blocks or RF Bandwidth on each side of the Inter RF Bandwidth gap, where the contribution from the far-end sub-block or RF Bandwidth shall be scaled according to the measurement bandwidth of the near-end sub-block or RF Bandwidth. |  |  |  |

------------------------------ Next modified section ------------------------------

#### 7.2.5.5	NR Test Requirements

For each measured carrier, the throughput measured in step 9 of clause 7.2.4.2 shall be ≥ 95 % of the maximum throughput of the reference measurement channel as specified in TS 38.104 [33] annex A.1 with parameters specified in table 7.2.5.5-1.

Table 7.2.5.5-1: EIS levels

| BS channel bandwidth [MHz] | Sub-carrier spacing [kHz] | Reference | EIS level [dBm] |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | measurement channel | f ≤ 3.0 GHz | 3.0 GHz < f ≤ 4.2 GHz |  |
| 5, 10, 15 | 15 | G- FR1-A1-1 in clause A.1 in [33] | Declared minimum EIS + 1.3 | Declared minimum EIS + 1.4 |  |
| 10, 15 | 30 | G- FR1-A1-2 in clause A.1 in [33] | Declared minimum EIS + 1.3 | Declared minimum EIS + 1.4 |  |
| 10, 15 | 60 | G- FR1-A1-3 in clause A.1 in [33] | Declared minimum EIS + 1.3 | Declared minimum EIS + 1.4 |  |
| 20, 25, 30, 40, 50 | 15 | G- FR1-A1-4 in clause A.1 in [33] | Declared minimum EIS + 1.3 | Declared minimum EIS + 1.4 |  |
| 20, 25, 30, 40, 50, 60, 70, 80, 90, 10 | 30 | G- FR1-A1-5 in clause A.1 in [33] | Declared minimum EIS + 1.3 | Declared minimum EIS + 1.4 |  |
| 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 60 | G- FR1-A1-6 in clause A.1 in [33] | Declared minimum EIS + 1.3 | Declared minimum EIS + 1.4 |  |

------------------------------ Next modified section ------------------------------

#### 7.3.5.2	UTRA FDD Test Requirements

The BER measurement result in step 9 of clause 7.3.4.2 shall not be greater than the limit specified in table 7.3.5.2-1.

Table 7.3.5.2-1: EISREFSENS levels

| BS Class |  | Reference measurement channel |  |  |  | Reference |  | EISREFSENS (dBm) |  |  | BER |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | measurement channel data rate |  | f ≤ 3.0 GHz |  | 3.0 GHz < f ≤ 4.2 GHz |  |
| Wide Area BS |  | 12.2kbps DPCH with reference measurement channel defined in annex A in TS 25.141 [10] (PN-9 data sequence or longer) |  |  |  | 12.2 kbps |  | -119.7 - ΔOTAREFSENS |  | -119.6 - ΔOTAREFSENS | BER shall not exceed 0.001 |
| Medium Range BS | 12.2kbps DPCH with reference measurement channel defined in annex A in TS 25.141 [10] (PN-9 data sequence or longer) |  | 12.2 kbps | -109.7 - ΔOTAREFSENS | -109.6 - ΔOTAREFSENS |  | BER shall not exceed 0.001 |  |
| Local Area BS | 12.2kbps DPCH with reference measurement channel defined in annex A in TS 25.141 [10] (PN-9 data sequence or longer) | 12.2 kbps | -105.7 - ΔOTAREFSENS | -105.6 - ΔOTAREFSENS | BER shall not exceed 0.001 |

------------------------------ Next modified section ------------------------------

#### 7.3.5.3	E-UTRA Test Requirements

For each measured E-UTRA carrier, the throughput measured in step 9 of clause 7.3.4.2 shall be ≥ 95 % of the maximum throughput of the reference measurement channel as specified in TS 36.141 [12] annex A.1 with parameters specified in table 7.3.5.3-1.

Table 7.3.5.3-1: E-UTRA Wide area AAS BS EISREFSENS levels

| E-UTRA channel bandwidth (MHz) | Reference measurement channel | EISREFSENS (dBm) |  |
| --- | --- | --- | --- |
|  |  | f ≤ 3.0 GHz | 3.0 GHz < f ≤ 4.2 GHz |
| 1.4 | FRC A1-1 in annex A.1 [12] | -105.5 - ΔOTAREFSENS | -105.4 - ΔOTAREFSENS |
| 3 | FRC A1-2 in annex A.1 [12] | -101.7 - ΔOTAREFSENS | -101.6 - ΔOTAREFSENS |
| 5 | FRC A1-3 in annex A.1 [12] | -100.2 - ΔOTAREFSENS | -100.1 - ΔOTAREFSENS |
| 10 |  |  |  |
| 15 | FRC A1-3 in annex A.1 [12] (Note) | -100.2 - ΔOTAREFSENS | -100.1 - ΔOTAREFSENS |
| 20 |  |  |  |
| NOTE: EISREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for each consecutive application of a single instance of FRC A1-3 in [11] mapped to disjoint frequency ranges with a width of 25 Resource Blocks each. |  |  |  |

Table 7.3.5.3-2: E-UTRA Local area AAS BS EISREFSENS levels

| E-UTRA channel bandwidth (MHz) | Reference measurement channel | EISREFSENS (dBm) |  |
| --- | --- | --- | --- |
|  |  | f ≤ 3.0 GHz | 3.0 GHz < f ≤ 4.2 GHz |
| 1.4 | FRC A1-1 in annex A.1 [12] | -97.5 - ΔOTAREFSENS | -97.4 - ΔOTAREFSENS |
| 3 | FRC A1-2 in annex A.1 [12] | -93.7 - ΔOTAREFSENS | -93.6 - ΔOTAREFSENS |
| 5 | FRC A1-3 in annex A.1 [12] | -92.2 - ΔOTAREFSENS | -92.1 - ΔOTAREFSENS |
| 10 |  |  |  |
| 15 | FRC A1-3 in annex A.1 [12] (Note) | -92.2 - ΔOTAREFSENS | -92.1 - ΔOTAREFSENS |
| 20 |  |  |  |
| NOTE: EISREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for each consecutive application of a single instance of FRC A1-3 in [11] mapped to disjoint frequency ranges with a width of 25 Resource Blocks each. |  |  |  |

Table 7.3.5.3-3: E-UTRA Medium range AAS BS EISREFSENS levels

| E-UTRA channel bandwidth (MHz) | Reference measurement channel | EISREFSENS (dBm) |  |
| --- | --- | --- | --- |
|  |  | f ≤ 3.0 GHz | 3.0 GHz < f ≤ 4.2 GHz |
| 1.4 | FRC A1-1 in annex A.1 [12] | -100.5 - ΔOTAREFSENS | -100.4 - ΔOTAREFSENS |
| 3 | FRC A1-2 in annex A.1 [12] | -96.7 - ΔOTAREFSENS | -96.6 - ΔOTAREFSENS |
| 5 | FRC A1-3 in annex A.1 [12] | -95.2 - ΔOTAREFSENS | -95.1 - ΔOTAREFSENS |
| 10 | FRC A1-3 in annex A.1 [12] (NOTE 1) FRC A1-8 in Annex A.1 [12] (NOTE 2) | -95.2 - ΔOTAREFSENS -97.9 - ΔOTAREFSENS | -95.1 - ΔOTAREFSENS -97.8 - ΔOTAREFSENS |
| 15 | FRC A1-3 in annex A.1 [12] (Note) | -95.2 - ΔOTAREFSENS | -95.1 - ΔOTAREFSENS |
| 20 | FRC A1-3 in annex A.1 [12] (NOTE 1) FRC A1-8 in Annex A.1 [12] (NOTE 2) | -95.2 - ΔOTAREFSENS -97.9 - ΔOTAREFSENS | -95.2 - ΔOTAREFSENS -97.9 - ΔOTAREFSENS |
| NOTE 1: EISREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for each consecutive application of a single instance of FRC A1-3 in [11] mapped to disjoint frequency ranges with a width of 25 Resource Blocks each. This reference measurement channel is not applied for Band 46. NOTE 2: EISREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for each single interlace of FRC A1-8 and A1-9. This reference measurement channel is only applied for Band 46. |  |  |  |

------------------------------ Next modified section ------------------------------

#### 7.3.5.4	NR Test Requirements

For each measured carrier, the throughput measured in step 9 of clause 7.3.4.2 shall be ≥ 95 % of the maximum throughput of the reference measurement channel as specified in annex TS 38.104 [33] A.1 with parameters specified in tables 7.3.5.4-1 to 7.3.5.4-3.

Table 7.3.5.4-1: Wide Area BS EISREFSENS levels

| BS channel bandwidth (MHz) | Sub-carrier spacing (kHz) | Reference measurement | EISREFSENS (dBm) |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | channel | f ≤ 3.0 GHz | 3.0 GHz < f ≤ 4.2 GHz |  |
| 5, 10, 15 | 15 | G-FR1-A1-1 | -100.4 – ΔOTAREFSENS | -100.3 – ΔOTAREFSENS |  |
| 10, 15 | 30 | G-FR1-A1-2 | -100.5 – ΔOTAREFSENS | -100.4 – ΔOTAREFSENS |  |
| 10, 15, 25, 30 | 60 | G-FR1-A1-3 | -97.6 – ΔOTAREFSENS | -97.5 – ΔOTAREFSENS |  |
| 20, 25, 30, 40, 50 | 15 | G-FR1-A1-4 | -94 – ΔOTAREFSENS | -93.9 – ΔOTAREFSENS |  |
| 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 30 | G-FR1-A1-5 | -94.3 – ΔOTAREFSENS | -94.2 – ΔOTAREFSENS |  |
| 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 60 | G-FR1-A1-6 | -94.4 – ΔOTAREFSENS | -94.3 – ΔOTAREFSENS |  |
| NOTE: PREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for each consecutive application of a single instance of the reference measurement channel mapped to disjoint frequency ranges with a width corresponding to the number of resource blocks of the reference measurement channel each, except for one instance that might overlap one other instance to cover the full BS channel bandwidth. |  |  |  |  |  |

Table 7.3.5.4-2: Medium Range BS EISREFSENS levels

| BS channel bandwidth (MHz) | Sub-carrier spacing (kHz) | Reference measurement | EISREFSENS (dBm) |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | channel | f ≤ 3.0 GHz | 3.0 GHz < f ≤ 4.2 GHz |  |
| 5, 10, 15 | 15 | G-FR1-A1-1 | -95.4 – ΔOTAREFSENS | -95.3 – ΔOTAREFSENS |  |
| 10, 15 | 30 | G-FR1-A1-2 | -95.5 – ΔOTAREFSENS | -95.4 – ΔOTAREFSENS |  |
| 10, 15 | 60 | G-FR1-A1-3 | -92.6 – ΔOTAREFSENS | -92.5 – ΔOTAREFSENS |  |
| 20, 25, 30, 40, 50, 100 | 15 | G-FR1-A1-4 | -89 – ΔOTAREFSENS | -88.9 – ΔOTAREFSENS |  |
| 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 30 | G-FR1-A1-5 | -89.3 – ΔOTAREFSENS | -89.2 – ΔOTAREFSENS |  |
| 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 60 | G-FR1-A1-6 | -89.4 – ΔOTAREFSENS | -89.3 – ΔOTAREFSENS |  |
| NOTE: PREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for each consecutive application of a single instance of the reference measurement channel mapped to disjoint frequency ranges with a width corresponding to the number of resource blocks of the reference measurement channel each, except for one instance that might overlap one other instance to cover the full BS channel bandwidth. |  |  |  |  |  |

Table 7.3.5.4-3: Local Area BS EISREFSENS levels

| BS channel bandwidth (MHz) | Sub-carrier spacing (kHz) | Reference measurement | EISREFSENS (dBm) |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | channel | f ≤ 3.0 GHz | 3.0 GHz < f ≤ 4.2 GHz |  |
| 5, 10, 15 | 15 | G-FR1-A1-1 | -92.4 – ΔOTAREFSENS | -92.3 – ΔOTAREFSENS |  |
| 10, 15 | 30 | G-FR1-A1-2 | -92.5 – ΔOTAREFSENS | -92.4 – ΔOTAREFSENS |  |
| 10, 15 | 60 | G-FR1-A1-3 | -89.6 – ΔOTAREFSENS | -89.5 – ΔOTAREFSENS |  |
| 20, 25, 30, 40, 50 | 15 | G-FR1-A1-4 | -86 – ΔOTAREFSENS | -85.9 – ΔOTAREFSENS |  |
| 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 30 | G-FR1-A1-5 | -86.3 – ΔOTAREFSENS | -86.2 – ΔOTAREFSENS |  |
| 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 60 | G-FR1-A1-6 | -86.4 – ΔOTAREFSENS | -86.3 – ΔOTAREFSENS |  |
| NOTE: PREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for each consecutive application of a single instance of the reference measurement channel mapped to disjoint frequency ranges with a width corresponding to the number of resource blocks of the reference measurement channel each, except for one instance that might overlap one other instance to cover the full BS channel bandwidth. |  |  |  |  |  |

------------------------------ Next modified section ------------------------------

#### 7.4.5.3	NR operation

For each measured carrier, the throughput measured in step 6 of clause 7.4.4.2 shall be ≥ 95 % of the maximum throughput of the reference measurement channel as specified in TS 38.104 [33] annex A.2 with parameters specified in tables 7.4.5.3-1 to 7.4.5.3-3.

Table 7.4.5.3-1: Wide Area BS dynamic range

| BS channel bandwidth | Subcarrier spacing (kHz) | Reference measurement channel | Wanted signal mean power (dBm) |  |  | Interfering signal mean power | Type of interfering signal |
| --- | --- | --- | --- | --- | --- | --- | --- |
| (MHz) |  |  | f ≤ 3.0 GHz | 3.0 GHz < f ≤ 4.2 GHz |  | (dBm) / BWConfig |  |
| 5 | 15 | G-FR1-A2-1 | -70.4 – ΔOTAREFSENS | -70.4 – ΔOTAREFSENS |  | -82.5 – ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-2 | -71.1 – ΔOTAREFSENS | -71.1 – ΔOTAREFSENS |  |  |  |
| 10 | 15 | G-FR1-A2-1 | -70.4 – ΔOTAREFSENS | -70.4 – ΔOTAREFSENS |  | -79.3 – ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-2 | -71.1 – ΔOTAREFSENS | -71.1 – ΔOTAREFSENS |  |  |  |
|  | 60 | G-FR1-A2-3 | -68.1 – ΔOTAREFSENS | -68.1 – ΔOTAREFSENS |  |  |  |
| 15 | 15 | G-FR1-A2-1 | -70.4 – ΔOTAREFSENS | -70.4 – ΔOTAREFSENS |  | -77.5 – ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-2 | -71.1 – ΔOTAREFSENS | -71.1 – ΔOTAREFSENS |  |  |  |
|  | 60 | G-FR1-A2-3 | -68.1 – ΔOTAREFSENS | -68.1 – ΔOTAREFSENS |  |  |  |
| 20 | 15 | G-FR1-A2-4 | -64.2 – ΔOTAREFSENS | -64.2 – ΔOTAREFSENS |  | -76.2 – ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-5 | -64.2 – ΔOTAREFSENS | -64.2 – ΔOTAREFSENS |  |  |  |
|  | 60 | G-FR1-A2-6 | -64.5 – ΔOTAREFSENS | -64.5 – ΔOTAREFSENS |  |  |  |
| 25 | 15 | G-FR1-A2-4 | -64.2 – ΔOTAREFSENS | -64.2 – ΔOTAREFSENS |  | -75.2 – ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-5 | -64.2 – ΔOTAREFSENS | -64.2 – ΔOTAREFSENS |  |  |  |
|  | 60 | G-FR1-A2-6 | -64.5 – ΔOTAREFSENS | -64.5 – ΔOTAREFSENS |  |  |  |
| 30 | 15 | G-FR1-A2-4 | -64.2 – ΔOTAREFSENS | -64.2 – ΔOTAREFSENS |  | -74.4 – ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-5 | -64.2 – ΔOTAREFSENS | -64.2 – ΔOTAREFSENS |  |  |  |
|  | 60 | G-FR1-A2-6 | -64.5 – ΔOTAREFSENS | -64.5 – ΔOTAREFSENS |  |  |  |
| 40 | 15 | G-FR1-A2-4 | -64.2 – ΔOTAREFSENS | -64.2 – ΔOTAREFSENS |  | -73.1 – ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-5 | -64.2 – ΔOTAREFSENS | -64.2 – ΔOTAREFSENS |  |  |  |
|  | 60 | G-FR1-A2-6 | -64.5 – ΔOTAREFSENS | -64.5 – ΔOTAREFSENS |  |  |  |
| 50 | 15 | G-FR1-A2-4 | -64.2 – ΔOTAREFSENS | -64.2 – ΔOTAREFSENS |  | -72.1 – ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-5 | -64.2 – ΔOTAREFSENS | -64.2 – ΔOTAREFSENS |  |  |  |
|  | 60 | G-FR1-A2-6 | -64.5 – ΔOTAREFSENS | -64.5 – ΔOTAREFSENS |  |  |  |
| 60 | 30 | G-FR1-A2-5 | -64.2 – ΔOTAREFSENS | -64.2 – ΔOTAREFSENS |  | -71.3 – ΔOTAREFSENS | AWGN |
|  | 60 | G-FR1-A2-6 | -64.5 – ΔOTAREFSENS | -64.5 – ΔOTAREFSENS |  |  |  |
| 70 | 30 | G-FR1-A2-5 | -64.2 – ΔOTAREFSENS | -64.2 – ΔOTAREFSENS |  | -70.7 – ΔOTAREFSENS | AWGN |
|  | 60 | G-FR1-A2-6 | -64.5 – ΔOTAREFSENS | -64.5 – ΔOTAREFSENS |  |  |  |
| 80 | 30 | G-FR1-A2-5 | -64.2 – ΔOTAREFSENS | -64.2 – ΔOTAREFSENS |  | -70.1 – ΔOTAREFSENS | AWGN |
|  | 60 | G-FR1-A2-6 | -64.5 – ΔOTAREFSENS | -64.5 – ΔOTAREFSENS |  |  |  |
| 90 | 30 | G-FR1-A2-5 | -64.2 – ΔOTAREFSENS | -64.2 – ΔOTAREFSENS |  | -69.5 – ΔOTAREFSENS | AWGN |
|  | 60 | G-FR1-A2-6 | -64.5 – ΔOTAREFSENS | -64.5 – ΔOTAREFSENS |  |  |  |
| 100 | 30 | G-FR1-A2-5 | -64.2 – ΔOTAREFSENS | -64.2 – ΔOTAREFSENS |  | -69.1 – ΔOTAREFSENS | AWGN |
|  | 60 | G-FR1-A2-6 | -64.5 – ΔOTAREFSENS | -64.5 – ΔOTAREFSENS |  |  |  |
| NOTE: The wanted signal mean power is the power level of a single instance of the corresponding reference measurement channel. This requirement shall be met for each consecutive application of a single instance of the reference measurement channel mapped to disjoint frequency ranges with a width corresponding to the number of resource blocks of the reference measurement channel each, except for one instance that might overlap one other instance to cover the full BS channel bandwidth. |  |  |  |  |  |  |  |

Table 7.4.5.3-2: Medium Range BS dynamic range

| BS channel bandwidth | Subcarrier spacing (kHz) | Reference measurement channel | Wanted signal mean power (dBm) |  |  | Interfering signal mean power | Type of interfering signal |
| --- | --- | --- | --- | --- | --- | --- | --- |
| (MHz) |  |  | f ≤ 3.0 GHz | 3.0 GHz < f ≤ 4.2 GHz |  | (dBm) / BWConfig |  |
| 5 | 15 | G-FR1-A2-1 | -65.4 – ΔOTAREFSENS | -65.4 – ΔOTAREFSENS |  | -77.5 – ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-2 | -66.1 – ΔOTAREFSENS | -66.1 – ΔOTAREFSENS |  |  |  |
| 10 | 15 | G-FR1-A2-1 | -65.4 – ΔOTAREFSENS | -65.4 – ΔOTAREFSENS |  | -74.3 – ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-2 | -66.1 – ΔOTAREFSENS | -66.1 – ΔOTAREFSENS |  |  |  |
|  | 60 | G-FR1-A2-3 | -63.1 – ΔOTAREFSENS | -63.1 – ΔOTAREFSENS |  |  |  |
| 15 | 15 | G-FR1-A2-1 | -65.4 – ΔOTAREFSENS | -65.4 – ΔOTAREFSENS |  | -72.5 – ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-2 | -66.1 – ΔOTAREFSENS | -66.1 – ΔOTAREFSENS |  |  |  |
|  | 60 | G-FR1-A2-3 | -63.1 – ΔOTAREFSENS | -63.1 – ΔOTAREFSENS |  |  |  |
| 20 | 15 | G-FR1-A2-4 | -59.2 – ΔOTAREFSENS | -59.2 – ΔOTAREFSENS |  | -71.2 – ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-5 | -59.2 – ΔOTAREFSENS | -59.2 – ΔOTAREFSENS |  |  |  |
|  | 60 | G-FR1-A2-6 | -59.5 – ΔOTAREFSENS | -59.5 – ΔOTAREFSENS |  |  |  |
| 25 | 15 | G-FR1-A2-4 | -59.2 – ΔOTAREFSENS | -59.2 – ΔOTAREFSENS |  | -70.2 – ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-5 | -59.2 – ΔOTAREFSENS | -59.2 – ΔOTAREFSENS |  |  |  |
|  | 60 | G-FR1-A2-6 | -59.5 – ΔOTAREFSENS | -59.5 – ΔOTAREFSENS |  |  |  |
| 30 | 15 | G-FR1-A2-4 | -59.2 – ΔOTAREFSENS | -59.2 – ΔOTAREFSENS |  | -69.4 – ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-5 | -59.2 – ΔOTAREFSENS | -59.2 – ΔOTAREFSENS |  |  |  |
|  | 60 | G-FR1-A2-6 | -59.5 – ΔOTAREFSENS | -59.5 – ΔOTAREFSENS |  |  |  |
| 40 | 15 | G-FR1-A2-4 | -59.2 – ΔOTAREFSENS | -59.2 – ΔOTAREFSENS |  | -68.1 – ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-5 | -59.2 – ΔOTAREFSENS | -59.2 – ΔOTAREFSENS |  |  |  |
|  | 60 | G-FR1-A2-6 | -59.5 – ΔOTAREFSENS | -59.5 – ΔOTAREFSENS |  |  |  |
| 50 | 15 | G-FR1-A2-4 | -59.2 – ΔOTAREFSENS | -59.2 – ΔOTAREFSENS |  | -67.1 – ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-5 | --59.2 – ΔOTAREFSENS | --59.2 – ΔOTAREFSENS |  |  |  |
|  | 60 | G-FR1-A2-6 | -59.5 – ΔOTAREFSENS | -59.5 – ΔOTAREFSENS |  |  |  |
| 60 | 30 | G-FR1-A2-5 | -59.2 – ΔOTAREFSENS | -59.2 – ΔOTAREFSENS |  | -66.3 – ΔOTAREFSENS | AWGN |
|  | 60 | G-FR1-A2-6 | -59.5 – ΔOTAREFSENS | -59.5 – ΔOTAREFSENS |  |  |  |
| 70 | 30 | G-FR1-A2-5 | -59.2 – ΔOTAREFSENS | -59.2 – ΔOTAREFSENS |  | -65.7 – ΔOTAREFSENS | AWGN |
|  | 60 | G-FR1-A2-6 | -59.5 – ΔOTAREFSENS | -59.5 – ΔOTAREFSENS |  |  |  |
| 80 | 30 | G-FR1-A2-5 | -59.2 – ΔOTAREFSENS | -59.2 – ΔOTAREFSENS |  | -65.1 – ΔOTAREFSENS | AWGN |
|  | 60 | G-FR1-A2-6 | -59.5 – ΔOTAREFSENS | -59.5 – ΔOTAREFSENS |  |  |  |
| 90 | 30 | G-FR1-A2-5 | -59.2 – ΔOTAREFSENS | -59.2 – ΔOTAREFSENS |  | -64.5 – ΔOTAREFSENS | AWGN |
|  | 60 | G-FR1-A2-6 | -59.5 – ΔOTAREFSENS | -59.5 – ΔOTAREFSENS |  |  |  |
| 100 | 30 | G-FR1-A2-5 | -59.2 – ΔOTAREFSENS | -59.2 – ΔOTAREFSENS |  | -64.1 – ΔOTAREFSENS | AWGN |
|  | 60 | G-FR1-A2-6 | -59.5 – ΔOTAREFSENS | -59.5 – ΔOTAREFSENS |  |  |  |
| NOTE: The wanted signal mean power is the power level of a single instance of the corresponding reference measurement channel. This requirement shall be met for each consecutive application of a single instance of the reference measurement channel mapped to disjoint frequency ranges with a width corresponding to the number of resource blocks of the reference measurement channel each, except for one instance that might overlap one other instance to cover the full BS channel bandwidth. |  |  |  |  |  |  |  |

Table 7.4.5.3-3: Local Area BS dynamic range

| BS channel bandwidth | Subcarrier spacing (kHz) | Reference measurement channel | Wanted signal mean power (dBm) |  |  | Interfering signal mean power | Type of interfering signal |
| --- | --- | --- | --- | --- | --- | --- | --- |
| (MHz) |  |  | f ≤ 3.0 GHz | 3.0 GHz < f ≤ 4.2 GHz |  | (dBm) / BWConfig |  |
| 5 | 15 | G-FR1-A2-1 | -62.4 – ΔOTAREFSENS | -62.4 – ΔOTAREFSENS |  | -74.5 – ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-2 | -63.1 – ΔOTAREFSENS | -63.1 – ΔOTAREFSENS |  |  |  |
| 10 | 15 | G-FR1-A2-1 | -62.4 – ΔOTAREFSENS | -62.4 – ΔOTAREFSENS |  | -71.3 – ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-2 | -63.1 – ΔOTAREFSENS | -63.1 – ΔOTAREFSENS |  |  |  |
|  | 60 | G-FR1-A2-3 | -60.1 – ΔOTAREFSENS | -60.1 – ΔOTAREFSENS |  |  |  |
| 15 | 15 | G-FR1-A2-1 | -62.4 – ΔOTAREFSENS | -62.4 – ΔOTAREFSENS |  | -69.5 – ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-2 | -63.1 – ΔOTAREFSENS | -63.1 – ΔOTAREFSENS |  |  |  |
|  | 60 | G-FR1-A2-3 | -60.1 – ΔOTAREFSENS | -60.1 – ΔOTAREFSENS |  |  |  |
| 20 | 15 | G-FR1-A2-4 | -56.2 – ΔOTAREFSENS | -56.2 – ΔOTAREFSENS |  | -68.2 – ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-5 | -56.2 – ΔOTAREFSENS | -56.2 – ΔOTAREFSENS |  |  |  |
|  | 60 | G-FR1-A2-6 | -56.5 – ΔOTAREFSENS | -56.5 – ΔOTAREFSENS |  |  |  |
| 25 | 15 | G-FR1-A2-4 | -56.2 – ΔOTAREFSENS | -56.2 – ΔOTAREFSENS |  | -67.2 – ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-5 | -56.2 – ΔOTAREFSENS | -56.2 – ΔOTAREFSENS |  |  |  |
|  | 60 | G-FR1-A2-6 | -56.5 – ΔOTAREFSENS | -56.5 – ΔOTAREFSENS |  |  |  |
| 30 | 15 | G-FR1-A2-4 | -56.2 – ΔOTAREFSENS | -56.2 – ΔOTAREFSENS |  | -66.4 – ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-5 | -56.2 – ΔOTAREFSENS | -56.2 – ΔOTAREFSENS |  |  |  |
|  | 60 | G-FR1-A2-6 | -56.5 – ΔOTAREFSENS | -56.5 – ΔOTAREFSENS |  |  |  |
| 40 | 15 | G-FR1-A2-4 | -56.2 – ΔOTAREFSENS | -56.2 – ΔOTAREFSENS |  | -65.1 – ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-5 | -56.2 – ΔOTAREFSENS | -56.2 – ΔOTAREFSENS |  |  |  |
|  | 60 | G-FR1-A2-6 | -56.5 – ΔOTAREFSENS | -56.5 – ΔOTAREFSENS |  |  |  |
| 50 | 15 | G-FR1-A2-4 | -56.2 – ΔOTAREFSENS | -56.2 – ΔOTAREFSENS |  | -64.1 – ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-5 | -56.2 – ΔOTAREFSENS | -56.2 – ΔOTAREFSENS |  |  |  |
|  | 60 | G-FR1-A2-6 | -56.5 – ΔOTAREFSENS | -56.5 – ΔOTAREFSENS |  |  |  |
| 60 | 30 | G-FR1-A2-5 | -56.2 – ΔOTAREFSENS | -56.2 – ΔOTAREFSENS |  | -63.3 – ΔOTAREFSENS | AWGN |
|  | 60 | G-FR1-A2-6 | -56.5 – ΔOTAREFSENS | -56.5 – ΔOTAREFSENS |  |  |  |
| 70 | 30 | G-FR1-A2-5 | -56.2 – ΔOTAREFSENS | -56.2 – ΔOTAREFSENS |  | -62.7 – ΔOTAREFSENS | AWGN |
|  | 60 | G-FR1-A2-6 | -56.5 – ΔOTAREFSENS | -56.5 – ΔOTAREFSENS |  |  |  |
| 80 | 30 | G-FR1-A2-5 | -56.2 – ΔOTAREFSENS | -56.2 – ΔOTAREFSENS |  | -62.1 – ΔOTAREFSENS | AWGN |
|  | 60 | G-FR1-A2-6 | -56.5 – ΔOTAREFSENS | -56.5 – ΔOTAREFSENS |  |  |  |
| 90 | 30 | G-FR1-A2-5 | -56.2 – ΔOTAREFSENS | -56.2 – ΔOTAREFSENS |  | -61.5 – ΔOTAREFSENS | AWGN |
|  | 60 | G-FR1-A2-6 | -56.5 – ΔOTAREFSENS | -56.5 – ΔOTAREFSENS |  |  |  |
| 100 | 30 | G-FR1-A2-5 | -56.2 – ΔOTAREFSENS | -56.2 – ΔOTAREFSENS |  | -61.1 – ΔOTAREFSENS | AWGN |
|  | 60 | G-FR1-A2-6 | -56.5 – ΔOTAREFSENS | -56.5 – ΔOTAREFSENS |  |  |  |
| NOTE: The wanted signal mean power is the power level of a single instance of the corresponding reference measurement channel. This requirement shall be met for each consecutive application of a single instance of the reference measurement channel mapped to disjoint frequency ranges with a width corresponding to the number of resource blocks of the reference measurement channel each, except for one instance that might overlap one other instance to cover the full BS channel bandwidth. |  |  |  |  |  |  |  |

------------------------------ Next modified section ------------------------------

##### 7.5.5.1.3	Additional BC3 blocking test requirement

The interfering signal is a 1,28Mcps UTRA TDD modulated signal as specified in clause A.2 in TS 37.141 [13].

The requirement is applicable outside the Base Station RF Bandwidth or Maximum Radio Bandwidth. The interfering signal offset is defined relative to the Base Station RF Bandwidth edges or Maximum Radio Bandwidth edges.

For multi-band RIBs, the requirement applies in addition inside any Inter RF Bandwidth gap, in case the gap size is at least 4.8 MHz. The interfering signal offset is defined relative to the Base Station RF Bandwidth edges inside the Inter RF Bandwidth gap.

For the wanted and interfering signal coupled to the RIB, using the parameters in table 7.5.5.1.3-1, the following requirements shall be met:

-	For any measured E-UTRA carrier, the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel defined in clause 7.2.5.

Table 7.5.5.1.3-1: Additional blocking requirement for Band Category 3

| Operating Band | Centre Frequency of Interfering Signal [MHz] | Interfering Signal mean power [dBm] | Wanted Signal mean power [dBm] (NOTE) | Interfering signal centre frequency minimum frequency offset from the Base Station RF Bandwidth edge [MHz] |
| --- | --- | --- | --- | --- |
| 33 - 40 | (FUL_low - 20) to (FUL_high + 20) | -40 - ΔOTAREFSENS (NOTE 2) | EISREFSENS + 6 dB (NOTE 2) | ±2,4 |
|  |  | -40 – ΔminSENS (NOTE 3) | EISminSENS + 6 dB (NOTE 3) |  |
| NOTE 1: EISREFSENS and EISminSENS depend on the RAT, the BS class and on the channel bandwidth, see clauses 7.3 and 7.2. NOTE 2: This test requirement is only applied in the OTA REFSENS conformance test directions. NOTE 3: This test requirement is only applied in the OTA minSENS receiver target reference direction. |  |  |  |  |

------------------------------ Next modified section ------------------------------

#### 7.5.5.3	Single RAT E-UTRA operation

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel.

For E-UTRA Wide Area BS, the wanted and the interfering signal coupled to the BS antenna input are specified in tables 7.5.5.3-1 and 7.5.5.3-2 for narrowband blocking and 7.5.5.3-3 for ACS. The reference measurement channel for the wanted signal is identified in table 7.3.5.3-1 for each channel bandwidth and further specified in TS 36.104 [4] Annex A.

For E-UTRA Medium Range BS, the wanted and the interfering signal coupled to the BS antenna input are specified in tables 7.5.5.3-1 and 7.5.5.3-2 for narrowband blocking and in table 7.5.5.3-5 for ACS. Narrowband blocking requirements are not applied for Band 46. The reference measurement channel for the wanted signal is identified in table 7.3.5.3-3 for each channel bandwidth and further specified in TS 36.104 [4] Annex A.

For E-UTRA Local Area BS, the wanted and the interfering signal coupled to the BS antenna input are specified in tables 7.5.5.3-1 and 7.5.5.3-2 for narrowband blocking and 7.5.5.3-4 for ACS. Narrowband blocking requirements are not applied for Band 46. The reference measurement channel for the wanted signal is identified in table 7.3.5.3-2 for each channel bandwidth and further specified in TS 36.104 [4] Annex A.

For narrowband blocking the OTA levels are applied referenced to 2 antenna gain offsets ΔOTAREFSENS and ΔminSENS.

For ACS the OTA levels are applied referenced to ΔminSENS.

The ACS and narrowband blocking requirement is applicable outside the Base Station RF Bandwidth or Radio Bandwidth. The interfering signal offset is defined relative to the Base station RF Bandwidth edges or Radio Bandwidth edges.

For RIBs supporting operation in non-contiguous spectrum within any operating band, the ACS requirement applies in addition inside any sub-block gap, in case the sub-block gap size is at least as wide as the E-UTRA interfering signal in tables 7.5.5.3-3, 7.5.5.3-4 and 7.5.5.3-5. The interfering signal offset is defined relative to the sub-block edges inside the sub-block gap

For multi-band RIBs, the ACS requirement applies in addition inside any Inter RF Bandwidth gap at the RIB, in case the gap size is at least as wide as the E-UTRA interfering signal in tables 7.5.5.3-3, 7.5.5.3-4 and 7.5.5.3-5. The interfering signal offset is defined relative to the Base Station RF Bandwidth edges inside the Inter RF Bandwidth gap.

For a RIBs operating in non-contiguous spectrum within any operating band, the narrowband blocking requirement applies in addition inside any sub-block gap, in case the sub-block gap size is at least as wide as the channel bandwidth of the E-UTRA interfering signal in table 7.5.5.3-2. The interfering signal offset is defined relative to the sub-block edges inside the sub-block gap.

For multi-band RIBs,, the narrowband blocking requirement applies in addition inside any Inter RF Bandwidth gap, in case the Inter RF Bandwidth gap size is at least as wide as the E-UTRA interfering signal in table 7.5.5.3-2. The interfering signal offset is defined relative to the Base Station RF Bandwidth edges inside the Inter RF Bandwidth gap.

------------------------------ Next modified section ------------------------------

#### 7.9.5.2	NR test requirement

The requirement shall apply at the RIB when the AoA of the incident wave of the received signal and the interfering signal are the same direction and are within the minSENS RoAoA

The wanted and interfering signals applies to all supported polarizations, under the assumption of polarization matching.

Details of the reference measurement channels can be found in TS 38.141-2 [34] annex A.

Table 7.9.5.2-1: Wide Area BS in-channel selectivity

| BS channel bandwidth (MHz) | Subcarrier spacing (kHz) | Reference measurement channel | Wanted signal mean power (dBm) |  |  | Interfering signal mean power (dBm) | Type of interfering signal |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | f ≤ 3.0 GHz | 3.0 GHz < f ≤ 4.2 GHz |  |  |  |
| 5 | 15 | G-FR1-A1-7 | -98.9-ΔminSENS | -98.5-ΔminSENS |  | -81.4 - ΔminSENS | DFT-s-OFDM NR signal, 15 kHz SCS, 10 PRBs |
| 10, 15, 20, 25, 30 | 15 | G-FR1-A1-1 | -97-ΔminSENS | -96.6-ΔminSENS |  | -77.4 - ΔminSEN | DFT-s-OFDM NR signal, 15 kHz SCS, 25 PRBs |
| 40, 50 | 15 | G-FR1-A1-4 | -90.6-ΔminSENS | -90.2-ΔminSENS |  | -71.4 - ΔminSEN | DFT-s-OFDM NR signal, 15 kHz SCS, 100 PRBs |
| 5 | 30 | G-FR1-A1-8 | -99.6-ΔminSENS | -99.2-ΔminSENS |  | -81.4 - ΔminSEN | DFT-s-OFDM NR signal, 30 kHz SCS, 5 PRBs |
| 10, 15, 20, 25, 30 | 30 | G-FR1-A1-2 | -97.1-ΔminSENS | -96.7-ΔminSENS |  | -78.4 - ΔminSEN | DFT-s-OFDM NR signal, 30 kHz SCS, 10 PRBs |
| 40, 50, 60, 70, 80, 90, 100 | 30 | G-FR1-A1-5 | -90.9-ΔminSENS | -90.5-ΔminSENS |  | -71.4 - ΔminSEN | DFT-s-OFDM NR signal, 30 kHz SCS, 50 PRBs |
| 10, 15, 20, 25, 30 | 60 | G-FR1-A1-9 | -96.5-ΔminSENS | -96.1-ΔminSENS |  | -78.4 - ΔminSEN | DFT-s-OFDM NR signal, 60 kHz SCS, 5 PRBs |
| 40, 50, 60, 70, 80, 90, 100 | 60 | G-FR1-A1-6 | -91-ΔminSENS | -90.6-ΔminSENS |  | -71.6 - ΔminSEN | DFT-s-OFDM NR signal, 60 kHz SCS, 24 PRBs |
| NOTE: Wanted and interfering signal are placed adjacently around Fc, where the Fc is defined for BS channel bandwidth of the wanted signal according to the table 5.4.2.2-1 in TS 38.104 [37].The aggregated wanted and interferer signal shall be centred in the BS channel bandwidth of the wanted signal. |  |  |  |  |  |  |  |

Table 7.9.5.2-2: Medium Range BS in-channel selectivity

| BS channel bandwidth (MHz) | Subcarrier spacing (kHz) | Reference measurement channel | Wanted signal mean power (dBm) |  |  | Interfering signal mean power (dBm) | Type of interfering signal |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | f ≤ 3.0 GHz | 3.0 GHz < f ≤ 4.2 GHz |  |  |  |
| 5 | 15 | G-FR1-A1-7 | -93.9-ΔminSENS | -93.5-ΔminSENS |  | -76.4 - ΔminSENS | DFT-s-OFDM NR signal, 15 kHz SCS, 10 PRBs |
| 10, 15, 20, 25, 30 | 15 | G-FR1-A1-1 | -92-ΔminSENS | -91.6-ΔminSENS |  | -72.4 - ΔminSENS | DFT-s-OFDM NR signal, 15 kHz SCS, 25 PRBs |
| 40, 50 | 15 | G-FR1-A1-4 | -85.6-ΔminSENS | -85.2-ΔminSENS |  | -66.4 - ΔminSENS | DFT-s-OFDM NR signal, 15 kHz SCS, 100 PRBs |
| 5 | 30 | G-FR1-A1-8 | -94.6-ΔminSENS | -94.2-ΔminSENS |  | -76.4 - ΔminSENS | DFT-s-OFDM NR signal, 30 kHz SCS, 5 PRBs |
| 10, 15, 20, 25, 30 | 30 | G-FR1-A1-2 | -92.1-ΔminSENS | -91.7-ΔminSENS |  | -73.4 - ΔminSENS | DFT-s-OFDM NR signal, 30 kHz SCS, 10 PRBs |
| 40, 50, 60, 70, 80, 90, 100 | 30 | G-FR1-A1-5 | -85.9-ΔminSENS | -85.5-ΔminSENS |  | -66.4 - ΔminSENS | DFT-s-OFDM NR signal, 30 kHz SCS, 50 PRBs |
| 10, 15, 20, 25, 30 | 60 | G-FR1-A1-9 | -91.5-ΔminSENS | -91.1-ΔminSENS |  | -73.4 - ΔminSENS | DFT-s-OFDM NR signal, 60 kHz SCS, 5 PRBs |
| 40, 50, 60, 70, 80, 90, 100 | 60 | G-FR1-A1-6 | -86-ΔminSENS | -85.6-ΔminSENS |  | -66.6 - ΔminSENS | DFT-s-OFDM NR signal, 60 kHz SCS, 24 PRBs |
| NOTE: Wanted and interfering signal are placed adjacently around Fc, where the Fc is defined for BS channel bandwidth of the wanted signal according to the table 5.4.2.2-1 in TS 38.104 [37].The aggregated wanted and interferer signal shall be centred in the BS channel bandwidth of the wanted signal. |  |  |  |  |  |  |  |

Table 7.9.5.2-3: Local area BS in-channel selectivity

| BS channel bandwidth (MHz) | Subcarrier spacing (kHz) | Reference measurement channel | Wanted signal mean power (dBm) |  |  | Interfering signal mean power (dBm) | Type of interfering signal |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | f ≤ 3.0 GHz | 3.0 GHz < f ≤ 4.2 GHz |  |  |  |
| 5 | 15 | G-FR1-A1-7 | -90.9-ΔminSENS | -90.5-ΔminSENS |  | -73.4 - ΔminSENS | DFT-s-OFDM NR signal, 15 kHz SCS, 10 PRBs |
| 10, 15, 20, 25, 30 | 15 | G-FR1-A1-1 | -89-ΔminSENS | -88.6-ΔminSENS |  | -69.4 - ΔminSENS | DFT-s-OFDM NR signal, 15 kHz SCS, 25 PRBs |
| 40, 50 | 15 | G-FR1-A1-4 | -82.6-ΔminSENS | -82.2-ΔminSENS |  | -63.4 - ΔminSENS | DFT-s-OFDM NR signal, 15 kHz SCS, 100 PRBs |
| 5 | 30 | G-FR1-A1-8 | -91.6-ΔminSENS | -91.2-ΔminSENS |  | -73.4 - ΔminSENS | DFT-s-OFDM NR signal, 30 kHz SCS, 5 PRBs |
| 10, 15, 20, 25, 30 | 30 | G-FR1-A1-2 | -89.1-ΔminSENS | -88.7-ΔminSENS |  | -70.4 - ΔminSENS | DFT-s-OFDM NR signal, 30 kHz SCS, 10 PRBs |
| 40, 50, 60, 70, 80, 90, 100 | 30 | G-FR1-A1-5 | -82.9-ΔminSENS | -82.5-ΔminSENS |  | -63.4 - ΔminSENS | DFT-s-OFDM NR signal, 30 kHz SCS, 50 PRBs |
| 10, 15, 20, 25, 30 | 60 | G-FR1-A1-9 | -88.5-ΔminSENS | -88.1-ΔminSENS |  | -70.4 - ΔminSENS | DFT-s-OFDM NR signal, 60 kHz SCS, 5 PRBs |
| 40, 50, 60, 70, 80, 90, 100 | 60 | G-FR1-A1-6 | -83-ΔminSENS | -82.6-ΔminSENS |  | -63.6 - ΔminSENS | DFT-s-OFDM NR signal, 60 kHz SCS, 24 PRBs |
| NOTE: Wanted and interfering signal are placed adjacently around Fc, where the Fc is defined for BS channel bandwidth of the wanted signal according to the table 5.4.2.2-1 in TS 38.104 [37].The aggregated wanted and interferer signal shall be centred in the BS channel bandwidth of the wanted signal. |  |  |  |  |  |  |  |

------------------------------ Next modified section ------------------------------

------------------------------ Next modified section ------------------------------

------------------------------ End of modified section ------------------------------