3GPP TSG-RAN WG4 Meeting #98-e	R4-2103818

# Electronic Meeting,  Jan. 25th  – Feb. 5th, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-4 | CR | <CR#> | rev |  | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR: Introduce power imbalance requirements for NR V2X. |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL-Perf |  |  |  |  |  | Date: |  |  | 2021-1-4 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | According to the work plan of V2X demodulation, companies should submit draft CRs in RAN 4 98-e meeting and RAN 4 has agree to introduce power imbalance with two links test |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the requirments for power imbalance with two links. Add the RMC for power imbalance with two links. Add the resource pool. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The performance requirements will still be incompleted. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.1.X, A.x, A,y |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of change>

11.1.6	Power imbalance performance with two links

11.1.6.1   2RX requirements

11.1.6.1.1   Minimum requirements

The purpose of this test is to check the demodulation performance when receiving PSSCH transmissions from two Sidelink UEs with power imbalance in one slot.

The minimum requirements are specified in Table 11.1.X-1 with the test parameters specified in Table 11.1.X-1. The Sidelink UE 1 and 2 are synchronized to GNSS or GNSS-equivalent synchronization reference.

Table 11.1.6.1.1-1:  Test Parameters

| Parameter |  |  | Unit | Test 1 |
| --- | --- | --- | --- | --- |
| Communication resource pool configuration |  |  |  | xxx |
| Active cell(s) |  |  |  | None |
| Active Sidelink UE(s) |  |  |  | Sidelink UE 1, Sidelink UE 2 |
| Sidelink UE 1 | Sidelink Transmissions |  |  | PSCCH + PSSCH |
|  | PSSCH RMC |  |  | xxx |
|  | RB allocation |  |  | PRB (0~9) |
|  | Time offset (Note 1) |  | s | 0 |
|  | Frequency offset (Note 2) |  | Hz | 0 |
|  | Propagation Channel |  |  | AWGN |
|  | Antenna configuration |  |  | 1x2 |
|  | SCI format 2-A configuration | Payloads | Bits | 35 |
|  |  | α |  | 1 |
|  |  | βoffset |  | 3.5 |
|  | PSFCH periodicity |  | Slots | 4 |
|  | MintimeGapPSFCH |  | Slots | 3 |
| Sidelink UE 2 | Sidelink Transmissions |  |  | PSCCH + PSSCH |
|  | PSSCH RMC |  |  | xxx |
|  | RB allocation |  |  | PRB (30~39) |
|  | Time offset (Note 1) |  | s | 0 |
|  | Frequency offset (Note 2) |  | Hz | 0 |
|  | Propagation Channel |  |  | AWGN |
|  | Antenna configuration |  |  | 1x2 |
|  | SCI format 2-A configuration | Payloads | Bits | 35 |
|  |  | α |  | 1 |
|  |  | βoffset |  | 3.5 |
|  | PSFCH periodicity |  | Slots | 4 |
|  | MintimeGapPSFCH |  | Slots | 3 |
| Note 1: Time offset of Sidelink UE receive signal with respect to GNSS reference timing. Note 2: Frequency offset of Sidelink UE with respect to GNSS reference frequency. |  |  |  |  |

Table 11.1.6.1.1-2: Minimum performance

| Test number | Bandwidth | Sidelink UE | PSSCH Reference channel | Reference value |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | PSSCH BLER (%) | SNR (dB) of PSSCH |
| 1 | 20MHz | 1 | R.PSSCH.x | (Note 1) | TBD |
|  |  | 2 | R.PSSCH.x | TBD | TBD |
| Note 1: There is no BLER requirement for Sidelink UE 1. |  |  |  |  |  |

<End of change>

A.6  SL reference measurement channels

A.6.1 General

A.6.6 Reference measurement channels for power imbalance performance with two links

A.6.6.1 Reference measurement channels for SCS 15 kHz FR1

A.6.6.2 Reference measurement channels for SCS 30 kHz FR1

Table A.6.6.2-1: Reference measurement channels for PSSCH performance requirements

| Parameters | Unit | Value |
| --- | --- | --- |
| Reference channel |  | R.PSSCH.x |
| Synchronization source |  | GNSS |
| Channel bandwidth | MHz | 20 |
| Allocated resource blocks | RB | 10 |
| Subcarrier spacing | kHz | 30 |
| CP-OFDM symbols for slot with PSFCH (Note 2) |  | 10 |
| CP-OFDM symbols for slot without PSFCH (Note3) |  | 13 |
| DMRS symbols for slot with PSFCH |  | 2 |
| DMRS symbols for slot without PSFCH |  | 3 |
| Modulation order |  | 2 |
| MCS index |  | 4 (308/1024) |
| Transport Block Size for slot with PSFCH | Bits | 208 |
| Transport Block Size for slot without PSFCH | Bits | 432 |
| Transport block CRC | Bits | 24 |
| Maximum number of HARQ transmissions |  | 1 |
| Binary Channel Bits for slots with PSFCH |  | 744 |
| Binary Channel Bits for slots without PSFCH | Bits | 1464 |
| Antenna configuration |  | 1x2 Low |
| Performance metric |  | SNR@10% BLER of PSSCH |
| Note 1: The first OFDM symbol of a PSSCH and its associated PSCCH is duplicated as described in clauses 8.3.1.5 and 8.3.2.3 of TS 38.211 and not used for demodulation. The first OFDM symbol of a PSFCH is duplicated as described in clause 8.3.4.2.2 of TS 38.211 Note 2: Guard OFDM symbols as described in clause 8.2.1 of TS 38.211 and OFDM symbols used for PSFCH are not included. Note 3: Last OFDM symbol used for guard symbol as described in clause 8.2.1 of TS 38.211 is not included. |  |  |

A.7	SL reference resource pool configurations

Table A.7-7 V2X sidelink communication resource pool for power imbalance test

| Information Element |  |  | Value |
| --- | --- | --- | --- |
|  |  |  | xxx |
| SL-ResourcePool-r16 | sl-PSCCH-Config-r16 | sl-TimeResourcePSCCH-r16 | 2 |
|  |  | sl-FreqResourcePSCCH-r16 | n10 |
|  | sl-SyncAllowed-r16 |  | gnss-Sync-r16 |
|  | sl-SubchannelSize-r16 |  | 10 |
|  | sl-TimeResource-r16 |  | 160 |
|  | sl-StartRB-Subchannel-r16 |  | 0 |
|  | sl-NumSubchannel-r16 |  | 5 |
|  | sl-Additional-MCS-Table-r16 |  | Not presented |
|  | sl-PTRS-Config-r16 |  | Not presented |
|  | sl-RB-Number-r16 |  | 51 |
|  | sl-X-Overhead-r16 |  | n0 |