3GPP TSG-RAN WG4 Meeting #98-e	R4-2103820

# Electronic Meeting, Jan. 25th – Feb. 5th, 2021

Title: 	Updated simulation assumptions for NR V2X multiple link test cases

Source: 	Huawei, HiSilicon

Agenda item:	7.3.6.3

Document for:	Discussion

Introduction

In this contribution, we provide the updated simulation assumptions for NR V2X demodulation requirements as per the latest agreements. Companies are encouraged to submit the simulation results for next meetings. The values with different options or [ ] need be updated in next RAN4 meeting based on further discussions.

Simulation assumptions

Power imbalance test

- Test purpose: The purpose of this test is to check the demodulation performance when receiving PSSCH transmissions from two sidelink UEs with power imbalance in one slot.

- Methodology for power imbalance test:

  - The target requirement ICS = -27 dBc

  - Select SINR2 from simulation results for SNR@10% BLER point based on simulation assumptions in Table 2.1-2.

  - Select SNR2 such that SNR2 = SINR2+5dB

  - Compute SNR1 = 30.35dB from the relation: SINR2 = SNR2 – 10*log10(10^((SNR1 + ICS)/10)+1).

- The test setup is shown as Table 2.1-1:

Table 2.1-1: Test assumptions

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Channel bandwidth |  | MHz | 20 |
| Subcarrier spacing |  | kHz | 30 |
| Active Sidelink UE(s) |  |  | Sidelink UE 1, Sidelink UE 2 |
| Sidelink UE 1 | Sidelink Transmissions |  | PSCCH + PSSCH |
|  | PSSCH RB allocation |  | PRB index: 0~9 |
|  | PSSCH RMC |  | Refer to Table 2.1-2 |
|  | PSCCH allocation |  | 10 RBs and 3 OFDM symbols (including first symbol used for AGC) |
|  | Time offset (Note 1) | s | 0 |
|  | Frequency offset (Note 2) | Hz | 0 |
|  | Propagation Channel |  | AWGN |
|  | Antenna configuration |  | 1x2 Low |
| Sidelink UE 2 | Sidelink Transmissions |  | PSCCH + PSSCH |
|  | PSSCH RB allocation |  | PRB index: 30~39 |
|  | PSSCH RMC |  | Refer to Table 2.1-2 |
|  | PSCCH RB allocation |  | 10 RBs and 3 OFDM symbols (including first symbol used for AGC) |
|  | Time offset (Note 1) | s | 0 |
|  | Frequency offset (Note 2) | Hz | 0 |
|  | Propagation Channel |  | AWGN |
|  | Antenna configuration |  | 1x2 Low |
| Note 1: Time offset of Sidelink UE receive signal with respect to GNSS reference timing. Note 2: Frequency offset of Sidelink UE with respect to GNSS reference frequency. |  |  |  |

- Companies are encouraged to provide simulation results for SINR2 for Sidelink UE 2 based on Table 2.1-2:

Table 2.1-2: Simulation assumptions and FRC for derivation of SINR2

| Parameters |  | Unit | Value |
| --- | --- | --- | --- |
| Synchronization source |  |  | GNSS |
| Channel bandwidth |  | MHz | 20 |
| Allocated resource blocks |  | RB | 10 |
| Subcarrier spacing |  | kHz | 30 |
| CP-OFDM symbols for slot with PSFCH (Note 2) |  |  | 10 |
| CP-OFDM symbols for slot without PSFCH (Note3) |  |  | 13 |
| DMRS symbols for slot with PSFCH |  |  | 2 |
| DMRS symbols for slot without PSFCH |  |  | 3 |
| Modulation order |  |  | 2 |
| MCS index |  |  | 4 (308/1024) |
| SCI format 2-A configuration | Payloads | Bits | 35 |
|  | α |  | 1 |
|  | βoffset |  | 3.5 |
| Transport Block Size for slot with PSFCH |  | Bits | 208 |
| Transport Block Size for slot without PSFCH |  | Bits | 432 |
| Transport block CRC |  | Bits | 24 |
| Maximum number of HARQ transmissions |  |  | 1 |
| Binary Channel Bits for slots with PSFCH |  |  | 744 |
| Binary Channel Bits for slots without PSFCH |  | Bits | 1464 |
| PSFCH resource periodicity |  | Slot | 4 |
| MinTimeGapPSFCH |  | Slot | 2 |
| Performance metric |  |  | SNR@10% BLER of PSSCH |
| Note 1: The first OFDM symbol of a PSSCH and its associated PSCCH is duplicated as described in clauses 8.3.1.5 and 8.3.2.3 of TS 38.211 and not used for demodulation. The first OFDM symbol of a PSFCH is duplicated as described in clause 8.3.4.2.2 of TS 38.211 Note 2: Guard OFDM symbols as described in clause 8.2.1 of TS 38.211 and OFDM symbols used for PSFCH are not included. Note 3: Last OFDM symbol used for guard symbol as described in clause 8.2.1 of TS 38.211 is not included. Note 4: 10 RBs and 3 OFDM symbols including first symbol for AGC are used for PSCCH transmission. |  |  |  |

Soft buffer combing test

- Test purpose: This test is to verify soft buffer combining capability of UE and maximum number of receiving HARQ processes supported by UE per TTI

- Test design:

  - For verification of harq-RxProcessSidelink = n16 or n24, n16 or n24 UEs transmit signal one by one circularly for every slot and in the first subchannel.

  - For verification of harq-RxProcessSidelink = n32, the first 31 UEs transmit signal one by one circularly for every slot and in the first subchannel. The 32nd UE transmit signal in the same slot as the first UE but in the second subchannel.

  - For verification of harq-RxProcessSidelink = n48, the first 31 UEs transmit signal one by one circularly for every slot and in the first subchannel. The next 17 UEs transmit signal one by one circularly for every slot and in the second subchannel”

  - For verification of harq-RxProcessSidelink = n64, first 31 UEs transmit signal one by one circularly for every slot and in the first subchannel, the next 31 UEs transmit signal one by one circularly for every slot and in the second subchannel, the last 2 UEs transmit signal in the same slot as the first two UEs but in the third subchannel.

- According to the agreement of RAN4#98-e meeting, RAN4 will only define one set of performance requirements based on the results of harq-RxProcessSidelink = n64. Interested companies are encouraged to provide simulation results in next meeting based on the simulation assumptions given in Table 2.2-1 and Table 2.2-2.

Table 2.2-1： Simulation assumptions for soft buffer test

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Active cell(s) |  |  | None |
| Bandwidth |  | MHz | 20 |
| SCS |  | kHz | 30 |
| Subchannel size |  | RBs | 10 |
| Number of symbols for PSCCH |  | Symbol | 2 |
| Number of RBs for PSCCH |  | RBs | 10 |
| Active Sidelink UE(s) |  |  | 64 |
| Sidelink UE i, 0 ≤ i ≤ 63 | Sidelink Transmissions |  | First 31 UEs transmit PSCCH + PSSCH one by one circularly for every slot and in the first subchannel, the next 31 UEs transmit PSCCH + PSSCH one by one circularly for every slot and in the second subchannel, the last 2 UEs transmit PSCCH + PSSCH in the same slot as the first two UEs but in the third subchannel. |
|  | Time gap between initial transmission and retransmission | Slots | 31 |
|  | Timing offset (Note 1) | s | 0 |
|  | Frequency offset (Note 2) | Hz | 0 |
|  | Synchronization source |  | GNSS or GNSS-equivalent |
|  | Antenna configuration |  | 1x2 Low |
| Timing offset from receiving side (Note 3) |  | s | Option 1: 0 Option 2: 12Ts |
| Frequency offset from receiving side (Note 4) |  | Hz | Option 1: 0 Option 2: 600 |
| Propagation condition |  |  | AWGN |
| Test metric |  |  | SNR@5% BLER of PSSCH |
| Note 1: Time offset of Sidelink UE receive signal with respect to GNSS reference timing. Note 2: Frequency offset of Sidelink UE with respect to GNSS reference frequency. Note 3: Timing offset from receiving side is with respect to GNSS reference timing. Note 4: Frequency offset from receiving side is with respect to GNSS reference frequency. |  |  |  |

Table 2.2-2： FRC for PSSCH

| Parameters |  | Unit | Value |
| --- | --- | --- | --- |
| Test |  |  | SCH_Test1 |
| Synchronization source |  |  | GNSS |
| Channel bandwidth |  | MHz | 20 |
| Allocated resource blocks |  | RB | 10 |
| Subcarrier spacing |  | kHz | 30 |
| CP-OFDM symbols (Note 1) |  |  | 10 |
| DMRS symbols |  |  | 2 |
| MCS index |  |  | MCS 28 |
| SCI format 2-A configuration | Payloads | Bits | 35 |
|  | α |  | 1 |
|  | βoffset |  | 2.5 |
| Transport Block Size for slots |  | Bits | 6528 |
| Transport block CRC |  | Bits | 24 |
| Binary Channel Bits for slots |  | Bits | 2544 |
| PSFCH resource periodicity |  | Slot | 1 |
| MinTimeGapPSFCH |  | Slot | 3 |
| Note 1: OFDM symbols are for PSCCH/PSSCH transmission not including guard symbols and PSFCH symbols. Note 2: 10 RBs and 3 OFDM symbols including first symbol for AGC are used for PSCCH transmission. |  |  |  |

PSCCH/PSSCH decoding capability test

The purpose of this test is to verify the maximum number of Sidelink processes supported by the V2X UE.

- Option 1: Use 40MHz bandwidth and AT command to calculate BLER based on Table 2.3-1.

Table 2.3-1 Simulation assumptions for PSCCH decoding demodulation performance

| Parameter |  | Unit | value |
| --- | --- | --- | --- |
| Channel bandwidth |  | MHz/30kHz SCS | 40 |
| Subchannel size |  | RBs | 10 |
| PSFCH periodicity |  | slots | 0 |
| MCS |  |  | MCS 4 |
| Sidelink UE i, 0 ≤ i ≤ 9 (Note 4) | Sidelink Transmissions |  | PSCCH + PSSCH |
|  | Timing offset (Note 1) | s | 0 |
|  | Frequency offset (Note 2) | Hz | 0 |
|  | Synchronization source |  | GNSS |
|  | Propagation Channel |  | Static propagation condition without external noise |
|  | Antenna configuration |  | 1x2 Low |
|  | PSSCH RMC |  | Reuse RMC from single-link PSSCH test with 500km/h. |
|  | PSCCH RMC (Note 3) |  | Reuse RMC from single-link PSCCH test |
| Target requirement |  | 1% BLER of PSCCH |  |
| Note 1: Time offset of Sidelink UE received signal with respect to GNSS reference timing. Note 2: Frequency offset of Sidelink UE with respect to GNSS reference frequency. Note 3: OCC index (in TS 38.211) for PSCCH DMRS is randomly selected between {0, 1, 2} for each PSCCH transmission. Note 4: Each UE occupies one sub-channel, hence all the sub-channels are filled |  |  |  |

- Option 2: Using PSFCH with PSFCH with periodicity 1 and flexibly CBW depending on psfch-TxNumber capability based on Table 2.3-2:

- Table 2.3-2 Simulation assumptions for PSCCH decoding demodulation performance

| Parameter |  | Unit | value |
| --- | --- | --- | --- |
| Channel bandwidth |  | MHz/30kHz SCS | 40 for the UE supporting psfch-TxNumber =n16, 20 for the UE supporting psfch-TxNumber =n8 10 for the UE supporting psfch-TxNumber =n4. |
| Subchannel size |  | RBs | 10 |
| PSFCH period |  | slots | 1 |
| MCS |  |  | MCS 4 |
| Sidelink UE i, 0 ≤ i ≤ N (Note 4) | Sidelink Transmissions |  | PSCCH + PSSCH |
|  | Timing offset (Note 1) | s | 0 |
|  | Frequency offset (Note 2) | Hz | 0 |
|  | Synchronization source |  | GNSS |
|  | Propagation Channel |  | Static propagation condition without external noise |
|  | Antenna configuration |  | 1x2 Low |
|  | PSSCH RMC |  | Reuse RMC from single-link PSSCH test with 500km/h. |
|  | PSCCH RMC (Note 3) |  | Reuse RMC from single-link PSCCH test |
| Target requirement |  | 1% BLER of PSCCH |  |
| Note 1: Time offset of Sidelink UE received signal with respect to GNSS reference timing. Note 2: Frequency offset of Sidelink UE with respect to GNSS reference frequency. Note 3: OCC index (in TS 38.211) for PSCCH DMRS is randomly selected between {0, 1, 2} for each PSCCH transmission. Note 4: N=9 for 40MHz channel bandwidth, N=4 for 20MHz channel bandwidth and N=1 for 10MHz channel bandwidth ; Each UE occupies one sub-channel, hence all the sub-channels are filled |  |  |  |

- Option3: Use following methodology based on Table 2.3-1 with the modification for PSFCH periodicity from 0 to 1.

  - TE sets PSSCH priority (in PSCCH) when PSFCH Tx capability < 10, x PSSCH with higher priority, 10-x PSSCH with lower priority. Note that PSFCH is selected according to PSSCH priority, hence if PSFCH Tx capability = x, the x feedback corresponding to x high priority PSSCH is transmitted to TE.

  - TE can verify UE PSCCH decoding success or failure by checking whether all the higher priority PSSCH feedback is received. In order to always feedback all the high priority SCHs, UE has to decode all CCH to know the priority.

  - To avoid UE cheating, TE can randomize the location of higher priority PSSCHs

PSFCH decoding capability test

- The purpose of this test is to verify the maximum decoding PSFCHs with ACK/NACK feedback per slot.

| Parameter |  | Unit | Test 1 |  |
| --- | --- | --- | --- | --- |
| Active cell(s) |  |  | None |  |
| Bandwidth |  | MHz | 40 |  |
| SCS |  | kHz | 30 |  |
| Subchannel size |  | RBs | 10 |  |
| PSFCH periodicity |  | slots | 1 |  |
| Sidelink UE i, 0 ≤ i ≤ N-1 (Note 3) | Sidelink transmissions |  | PSFCH |  |
|  | Timing offset (Note 1) | s | 0 |  |
|  | Frequency offset (Note 2) | Hz | 0 |  |
|  | Synchronization source |  | GNSS or GNSS-equivalent |  |
|  | Propagation Channel |  | Static propagation condition Without external noise |  |
|  | Antenna configuration |  | 1x2 Low |  |
|  | PSFCH resource allocation |  | RB index | CS pair index |
|  |  |  | i | 0 |
|  | Test metric |  | Pr(ACK miss) <1% and Pr(DTX to ACK) <1% |  |
| Note 1: Time offset of Sidelink UE receive signal with respect to GNSS reference timing. Note 2: Frequency offset of Sidelink UE with respect to GNSS reference frequency. Note 3: N UEs transmit PSFCH one by one following procedure from Section 16.3 of TS 38.1213. N is {5, 15, 25, 32, 35, 45, 50, 64} depending on UE capability |  |  |  |  |