3GPP TSG-RAN WG4 Meeting#98-e	R4- 2103832

Online, 25th Jan – 5th Feb, 2021

Source: 	Qualcomm Incorporated

Title: 	Simulation results for single and multi-DCI based transmission scheme

Agenda item:	7.9.4.1

Document for:	Discussion

# Introduction

In this paper, we provide our simulation results for multi-TRP schemes based on simulation assumptions in [1].

# Simulation Results

We have below simulation results based on simulation assumptions in [1] and offline email discussion.

| Schemes/Reference point | Simulation cases | Alignment SNR (dB) | Impairment SNR (dB) |
| --- | --- | --- | --- |
| Multi-DCI SDM scheme SNR [dB] at 70% TP | 1-1 (FDD 2Rx) | 17.7 | 20.2 |
|  | 1-2 (FDD 4Rx) | 12.16 | 14.66 |
|  | 2-1 (TDD 2Rx) | 18 | 20.5 |
|  | 2-2 (TDD 4Rx) | 12.16 | 14.66 |
| Single-DCI SDM scheme (Test 1a) SNR [dB] at 70% TP | 3-1 (FDD 2Rx) | 17.71 | 20.21 |
|  | 3-2 (FDD 4Rx) | 12.29 | 14.79 |
|  | 4-1 (TDD 2Rx) | 17.38 | 19.88 |
|  | 4-2 (TDD 4Rx) | 12.09 | 14.59 |
| Single-DCI SDM scheme (Test 1b) SNR [dB] at 70% TP | 3-1 (FDD 2Rx) | 17.21 | 19.71 |
|  | 3-2 (FDD 4Rx) | 11.79 | 14.29 |
|  | 4-1 (TDD 2Rx) | 17.28 | 19.78 |
|  | 4-2 (TDD 4Rx) | 11.73 | 14.23 |
| Single-DCI based FDM Scheme A SNR [dB] at 70% TP | 5-1 (FDD 2Rx) | 15.68 | 18.18 |
|  | 5-2 (FDD 4Rx) | 9.38 | 11.88 |
|  | 6-1 (TDD 2Rx) | 16.6 | 19.1 |
|  | 6-2 (TDD 4Rx) | 9.1 | 11.6 |
| Single-DCI based inter-slot TDM Schemes SNR [dB] with 1% BLER | 7-1 (FDD 2Rx) | 1.3 | 3.8 |
|  | 7-2 (FDD 4Rx) | -2.3 | 0.2 |
|  | 8-1 (TDD 2Rx) | 1.4 | 3.9 |
|  | 8-2 (TDD 4Rx) | -2.4 | 0.1 |

# Conclusions

This paper provides simulation results based on simulation assumptions in [1].

# References

[1] R4-2017530, “Simulation assumption for PDSCH requirement with single-DCI and Multi-DCI transmission schemes”, 3GPP TSG RAN WG4 Meeting #97-e, Online, Nov 2 - Nov 13, 2020.