3GPP TSG-RAN WG4 Meeting #98-e	R4-2103838

# E-meeting, 25th January – 5th February 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-4 | CR | 0141 | rev | 1 | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on applicability rules and FRC for FR2 DL 256QAM CQI requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_DL256QAM_FR2-Perf |  |  |  |  |  | Date: |  |  | 2021-01-15 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Definition of FR2 DL 256QAM CQI requirements |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Added applicability rules - Added FRC |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | FR2 DL 256QAM CQI requirements are not defined |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.1.3, A.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-4 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R4-2101253 |  |  |  |  |  |  |  |  |

START OF CHANGE

#### 8.1.1.3	Applicability of requirements for optional UE features

The performance requirements in Table 8.1.1.3-1 shall apply for UEs which support optional UE features only.

Table 8.1.1.3-1: Requirements applicability for optional UE features

| UE feature/capability [14] | Test type |  | Test list | Applicability notes |
| --- | --- | --- | --- | --- |
| 256QAM modulation scheme for PDSCH for FR2 (pdsch-256QAM-FR2) | FR2 TDD | CQI | Clause 8.2.2.2.1.1 (Tests 3 and 4) | The test coverage can be considered fulfilled without executing of Test 1 and 2 from Clause 8.2.2.2.1.1 if UE passes Test 3 and 4 from Clause 8.2.2.2.1.1 |

<SKIP UNCHANGED PART>

# A.4	CSI reference measurement channels

<SKIP UNCHANGED PART>

Table A.4-2: Mapping of CQI Index to Information Bit payload (CQI table 2)

| TBS Scheme |  |  |  |  |  |  |  | TBS.2-1 | TBS.2-2 | TBS.2-3 | TBS.2-4 | TBS.2-5 | TBS.2-6 | TBS.2-7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MCS table |  |  |  | 256QAM |  |  |  |  |  |  |
| Number of allocated PDSCH resource blocks |  |  |  |  |  |  |  | 52 | 52 | 106 | 106 | 8 | 16 | 32 |
| Number of consecutive PDSCH symbols |  |  |  |  |  |  |  | 12 | 12 | 12 | 12 | 12 | 12 | 12 |
| Number of PDSCH MIMO layers |  |  |  |  |  |  |  | 1 | 2 | 1 | 2 | 1 | 1 | 1 |
| Number of DMRS REs (Note 1) |  |  |  |  |  |  |  | 24 | 24 | 24 | 24 | 24 | 24 | 24 |
| Overhead for TBS determination |  |  |  |  |  |  |  | 0 | 0 | 0 | 0 | 0 | 0 | 6 |
| Available RE-s for PDSCH |  |  |  |  |  |  |  | 6240 | 6240 | 12720 | 12720 | 960 | 1920 | 3680 |
| CQI index | Spectral efficiency | MCS index | Modulation | Information Bit Payload per Slot |  |  |  |  |  |  |
| 0 |  | OOR |  | OOR |  | OOR |  | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| 1 |  | 0.1523 |  | 0 |  | QPSK |  | 1480 | 2976 | 2976 | 5896 | 224 | 456 | 848 |
| 2 |  | 0.3770 |  | 1 |  |  |  | 2408 | 4744 | 4744 | 9480 | 368 | 736 | 1416 |
| 3 |  | 0.8770 |  | 3 |  |  |  | 5504 | 11016 | 11016 | 22536 | 848 | 1736 | 3240 |
| 4 |  | 1.4766 |  | 5 |  | 16QAM |  | 9224 | 18432 | 18960 | 37896 | 1416 | 2856 | 5376 |
| 5 |  | 1.9141 |  | 7 |  |  |  | 12040 | 24072 | 24576 | 49176 | 1864 | 3752 | 6912 |
| 6 |  | 2.4063 |  | 9 |  |  |  | 15112 | 30216 | 30728 | 61480 | 2408 | 4608 | 8712 |
| 7 |  | 2.7305 |  | 11 |  | 64QAM |  | 16896 | 33816 | 34816 | 69672 | 2600 | 5248 | 9992 |
| 8 |  | 3.3223 |  | 13 |  |  |  | 20496 | 40976 | 42016 | 83976 | 3240 | 6400 | 12040 |
| 9 |  | 3.9023 |  | 15 |  |  |  | 24576 | 49176 | 49176 | 98376 | 3752 | 7424 | 14344 |
| 10 |  | 4.5234 |  | 17 |  |  |  | 28168 | 56368 | 57376 | 114776 | 4352 | 8712 | 16392 |
| 11 |  | 5.1152 |  | 19 |  |  |  | 31752 | 63528 | 65576 | 131176 | 4864 | 9736 | 18432 |
| 12 |  | 5.5547 |  | 21 |  | 256QAM |  | 34816 | 69672 | 69672 | 139376 | 5248 | 10760 | 20496 |
| 13 |  | 6.2266 |  | 23 |  |  |  | 38936 | 77896 | 79896 | 159880 | 6016 | 12040 | 22536 |
| 14 |  | 6.9141 |  | 25 |  |  |  | 43032 | 86040 | 88064 | 176208 | 6656 | 13320 | 25104 |
| 15 |  | 7.4063 |  | 27 |  |  |  | 46104 | 92200 | 94248 | 188576 | 7040 | 14088 | 27144 |
| Note 1: Number of DMRS REs includes the overhead of the DM-RS CDM groups without data Note 2: PDSCH is not scheduled on slots containing CSI-RS or slots which are not full DL Note 3: PDSCH is not scheduled on slots containing PBCH, i.e. slot#0 per 20ms periodicity |  |  |  |  |  |  |  |  |  |  |

END OF CHANGE