TSG-RAN Working Group 4 (Radio) meeting #98-E	R4-2103853

Electronic Meeting, 25th Jan - 5th Feb 2020

Source:	Ericsson

Title:	WF on IAB-MT test environment

Agenda item:	7.4.3

Document for:	Approval

# Introduction

During  RAN4#98-E meeting a way-forward for IAB-MT terst envriroment is created based on the discussion [1].

# Way-Forward

Below is the GTW session agreements[1]:

Issue 1-1-1: Agreement:

Using same BS approach (no detailed synchronization configuration in conformance specifications; meanwhile add a note in conformance specs to clarify (IAB-MT sync with IAB-DU with DL signal configuration not precluded).

Issue 1-1-2:

Agreement:

Two-way communication is not specified for RF conformance tests, specification shall not preclude DL signals to be used e.g. for timing and frequency reference purposes during the test.

Companies further work on the clarification notes to conformance specifications for topic 1-1.

Issue 1-1-3:

Agreements: Option 1: Flexibility in connection / measurement setup is allowed by keeping the specified setup informative

Issue 1-1-4:

Choosing higher values of MU/TT among available methods only regarding the individual contribution of system simulator as generic approach:

- some exceptions not excluded considering regulatory requirements impact.

According to above agreements, the below aspects could be discussed further to give clear guidance on TP drafting.

Issue 1-1-1, 1-1-2 and 1-1-3:

- Annex D in conducted IAB conformance test specification:

  - Informative

  - For IAB-MT RF conducted TX/RX test setup, both Annex D.3 and D.4 in TS 38.141-1 and Annex A3.1 and A3.2 in TS38.508-1 could be referenced.

- Annex E in radiated IAB conformance test specification:

  - Informative

  - For IAB-MT RF radiated TX/RX test setup, both Annex E.1 and E.2 in TS 38.141-2 and Annex A3.3 and A3.4 in TS 38.508-1 could be referenced.

Issue 1-1-4:

- Chapter 4.1.2 in IAB conducted test specification:

  - the MU clause 4.1.2.1, 4.1.2.2 and 4.1.2.3 in TS38.141-1 are the baseline for MU of IAB-MT

  - Further modification of the MU considering the UE System simulator is no excluded

  - Side condition to modify the MU considering UE system simulator is FFS

- Annex C Test tolerances and derivation of test requirements in IAB conducted test specification:

  - Informative

  - The Annex C.1 and C.2 in TS 38.141-1 is the baseline for TT of IAB-MT for respective test

  - Further modification of the TT considering the UE System simulator is no excluded

  - Side condition to modify the TT considering UE system simulator is FFS

  - TT related to regulation should be defined separately. (e.g 0 dB)

- Chapter 4.1.2 in IAB radiated test specification:

  - the MU clause 4.1.2.1, 4.1.2.2 and 4.1.2.3 in TS38.141-2 are the baseline for MU of IAB-MT type 1-O and type 2-O.

  - Further modification of the MU considering the UE System simulator is no excluded

  - Side condition to modify the MU considering UE system simulator is FFS

- Annex C Test tolerances and derivation of test requirements in IAB radiated test specification:

  - Informative

  - The Annex C.1 and C.2 in TS 38.141-2 is the baseline for TT of IAB-MT for respective test

  - Further modification of the TT considering the UE System simulator is no excluded

  - Side condition to modify the TT considering UE system simulator is FFS

  - TT related to regulation should be defined separately. (e.g 0 dB)

# References

- R4-2103745, Email discussion summary for [98e][306] NR_IAB_Conformance_Part1, Nokia