3GPP TSG-RAN WG4 Meeting #98-e	R4-2103942

Electronic Meeting, 25 Jan - 5 Feb, 2021

Agenda item:			7.16

Source:	Moderator (China Telecom)

Title:	Email discussion summary for [98e][326] NR_perf_enh_Demod

Document for:	Information

Introduction

This email thread discusses the NR Rel-16 demodulation performance requirements in agenda 7.16. Note that no tdoc has been submitted for BS demodulation in agenda 7.16.2 in this meeting.

List of candidate target of email discussion for 1st round and 2nd round:

- 1st round: Invite companies to review the recommended WF in section 1~5, and provide comments (if any) in section 1.3, 2.3, 3.3, 4.3 and 5.3.

- 2nd round:  Focus on the CRs to finish the WI in this meeting. There will be 6 sub-threads:

  - [98e][326] NR_perf_enh_Demod - CR on release independence (led by China Telecom)

  - [98e][326] NR_perf_enh_Demod - CR on CA normal applicability rule (led by Intel)

  - [98e][326] NR_perf_enh_Demod - Section numbering for CA requirements (led by Ericsson)

  - Quick check if any company has concern on the proposed Option 1 and/or have different suffixes for CA power imbalance and CA normal, by Tuesday 6am UTC, Feb. 2.

  - [98e][326] NR_perf_enh_Demod - CR on PMI applicability rule (led by Huawei)

  - [98e][326] NR_perf_enh_Demod - CR on type II PMI requirements (led by Samsung)

  - Tentative gamma requirements will be discussed and decided in this sub-thread.

  - [98e][326] NR_perf_enh_Demod - CR on EN-DC power imbalance (led by Huawei)

Topic #1: Release independent aspect

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2100787 | China Telecom | CR for TS 38.307 on UE demodulation performance requirements (Rel-15) |
| R4-2100788 | China Telecom | CR for TS 38.307 on UE demodulation performance requirements (Rel-16) |
| R4-2100789 | China Telecom | CR for TS 38.307 on UE demodulation performance requirements (Rel-17) |

Open issues summary

No open issue.

Companies views’ collection for 1st round

CRs/TPs comments collection

| CR/TP number | Comments collection |
| --- | --- |
| R4-2100787, Rel-15 38.307 CR, Cat. B, CTC | Company A: |
|  | Company B: |
| R4-2100788, Rel-16 38.307 CR, Cat. B, CTC | docomo: In terms of the NOTE 1 in Table B.3.2-1 and Table B.3.2-2, we probably need to change Section 9.1.1.1 to Section 9.1.1. ZTE: Perhaps a more generic feature “Precoding matrix indicator (PMI) reporting requirements including Type I and Type II codebook” with a description of supporting up to 32Tx could be another option. In this way, the number of total Tx can be changed in different future release, while keeping the feature name unchanged, a similar way as other release-independence features in TS 38.307. |
|  | Ericsson: In Table B.3.1-1, the section/clause numbering need to be changed according to the conclusion of Issue 2-1. In Table B.3.1-1, the description of 7.2A.2 is ‘PDSCH 2RX demodulation requirements for NR FR2 CA configurations with FDL_high not exceeding 40000 MHz’. Since RAN4 is discussing the UE demodulation requirements for 47GHz (n262), we propose to remove ‘with FDL_high not exceeding 40000MHz’ to avoid the maintenance work in the future. |
|  | ZTE: Similar comments as above. |
| R4-2100789, Rel-17 38.307 CR, Cat. A, CTC (Not uploaded yet) | Company A: |

Note: To save time on typing the comments one by one, companies can also directly revise the draft CR and upload the revision in the draft inbox.

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |

Recommendations on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |
| R4-2100787, Rel-15 38.307 CR, Cat. B, CTC | to be revised |
| R4-2100788, Rel-16 38.307 CR, Cat. B, CTC | to be revised |

Discussion on 2nd round

R4-2103839	CR for TS 38.307 on UE demodulation performance requirements (Rel-15)

Type: CR		For: Agreement
					38.307 v15.7.0	  CR-0043  Cat: B (Rel-15)

					Source: China Telecom

Discussion:

[report of discussion]

New changes by ‘China Telecom_0201’ were made according to the comments in round 1, which are also highlighted by yellow.

Recommendation:		Agreeable

R4-2103840	CR for TS 38.307 on UE demodulation performance requirements (Rel-16)

Type: CR		For: Agreement
					38.307 v16.5.0	  CR-0044  Cat: B (Rel-16)

					Source: China Telecom

Discussion:

[report of discussion]

Decision:		Agreeable

Summary on 2nd round

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| XXX | Based on 2nd round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |
| R4-2103839, Rel-15 38.307 CR, Cat. B, CTC | agreeable |
| R4-2103840, Rel-16 38.307 CR, Cat. B, CTC | Agreeable |
| R4-2100789, Rel-17 38.307 CR, Cat. A, CTC | Agreeable |

Topic #2: UE	CA PDSCH requirements

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2100786 | China Telecom | Proposal 1: Use option 1 for the antenna connection for FR1 CA tests with 4 RX. Observation 1: The agreed procedure for selecting CA configuration(s) and CBW combination can be used to find the test scenario where UE support at least 2 layer on each CC. |
| R4-2100816 | CMCC | CR for NR PDSCH FR1 CA 2Rx performance requirements |
| R4-2100822 | CMCC | Issue1: Antenna connection for CA tests with 4 RX Proposal 1: We support to reuse the same antenna connection for CA tests with 4RX in LTE - If any of the Pcell and/or the Scells is a 4 RX supported RF band, all 4 RX should be connected with data source from system simulator. - If any of the Pcell and/or the Scells is a 2 RX supported RF band, 2 out of the 4 RX should be connected with data source from system simulator, and the other 2 RX are connected with zero input. Issue2: How to test the UE which does not support 2-layer transmission on all CCs for all supported CA configurations. In our point of view, the case that UE does not support 2-layer transmission on all CCs for all supported CA configurations is a conner case, no further discussion is needed in this stage. |
| R4-2101254 | Intel Corporation | Proposal 1: Reuse LTE applicability rule and antenna connection approach defined in 8.1.2.6.5 of 36.101 for NR CA testing of 4 Rx capable Ues. Proposal 2: Add the following clarification to NR CA applicability rules: “Verify Ues only on CCs, for which the supported maximum number of MIMO layers is not lower than 2. OCNG pattern is used for CCs, for which the supported maximum number of MIMO layers is 1, and performance is not verified on these CCs”. |
| R4-2101255 | Intel Corporation | CR on applicability rules for Normal NR CA requirements |
| R4-2101365 | Huawei, HiSilicon | Proposal 1: Reuse the test applicability rule defined for single carrier for CA tests with 2Rx and 4Rx, i.e. - Only conducted CA tests with 2Rx for UE only supports 2Rx - Only conducted CA tests with 4Rx for UE only supports 4Rx - Only conducted CA tests with 4Rx for UE supports both 2Rx and 4Rx - Not conducted CA test with 4Rx for UE only supports 2Rx - Proposal 2: No further discussion on how to test UE that does not support 2-layer transmission on all CCs for all supported CA configurations is needed. |
| R4-2101434 | Ericsson | Draft CR: Section numbering for PDSCH CA demodulation requirements It is important that the clause numberings are aligned between RAN5 conformance test specification and RAN4 specification at least on top test case title level. Also RAN5 test cases are common for 2Rx and 4Rx. However the current RAN4 PDSCH CA requirement specification structure is not aligned with other requirements, that is, the PDSCH CA requrements and power imbalance requirements are inside the 2Rx and 4Rx sections. For efficient RAN5 operation maintaining the clause number alignment, it is desirable that the 2Rx/4Rx requirements are added in the lowest clause number level, and any new features are added independently on the antenna number in a separate clause. |
| R4-2102818 | Qualcomm Incorporated | Proposal 1: Implement clause restructure of the CA PDSCH Demodulation and CA CQI reporting test cases in 38.101-4 spec as per option 1. If option 1 is not possible due to TS drafting rules, implement option 2. Option 1: 5.2A PDSCH Demodulation requirements for CA 5.2A.1 Minimum requirements (normal PDSCH) 5.2A.1.1 1RX requirements (Void) 5.2A.1.2 2RX requirements 5.2A.1.3 4RX requirements 5.2A.2 Minimum requirements for carrier aggregation with power imbalance 5.2A.2.1 1RX requirements (Void) 5.2A.2.2 2RX requirements 5.2A.2.3 4RX requirements Option 2: If option 1 is not possible due to TS drafting rules, this could be another option 5.2A PDSCH Demodulation requirements for CA 5.2A.1 Void 5.2A.2 Void 5.2A.3 Void 5.2A.4 Minimum requirements (normal PDSCH) 5.2A.4.1 1RX requirements (Void) 5.2A.4.2 2RX requirements 5.2A.4.3 4RX requirements 5.2A.5 Minimum requirements for carrier aggregation with power imbalance 5.2A.5.1 1RX requirements (Void) 5.2A.5.2 2RX requirements 5.2A.5.3 4RX requirements |

Open issues summary

Sub-topic 2-1: Specification section numbering

Issue 2-1: Section numbering for PDSCH CA demodulation requirements

- Current section numbering:

5.2A             PDSCH Demodulation requirements for CA

5.2A.1          1RX requirements (Void)

5.2A.2          2RX requirements

5.2A.2.1       Minimum requirements (normal PDSCH)

5.2A.2.2       Minimum requirements for carrier aggregation with power imbalance

5.2A.3          4RX requirements

5.2A.3.1       Minimum requirements (normal PDSCH)

5.2A.3.2       Minimum requirements for carrier aggregation with power imbalance

7.2A             PDSCH Demodulation requirements for CA

7.2A.1          1RX requirements (Void)

7.2A.2          2RX requirements

7.2A.2.1       Minimum requirements (normal PDSCH)

- Issues with the current section numbering (E///, QC)

  - It is important that the clause numberings are aligned between RAN5 conformance test specification and RAN4 specification at least on top test case title level. Also RAN5 test cases are common for 2Rx and 4Rx.

  - However the current RAN4 PDSCH CA requirement specification structure is not aligned with other requirements, that is, the PDSCH CA requrements and power imbalance requirements are inside the 2Rx and 4Rx sections.

  - For efficient RAN5 operation maintaining the clause number alignment, it is desirable that the 2Rx/4Rx requirements are added in the lowest clause number level, and any new features are added independently on the antenna number in a separate clause.

- Proposals

  - Option 1 (E///, QC)

5.2A             PDSCH Demodulation requirements for CA

5.2A.1          Void

5.2A.2          Void

5.2A.3          Void

5.2A.4          Minimum requirements

5.2A.4.1       1RX requirements (Void)

5.2A.4.2       2RX requirements

5.2A.4.3       4RX requirements

5.2A.5          Minimum requirements for carrier aggregation with power imbalance

5.2A.5.1       1RX requirements (Void)

5.2A.5.2       2RX requirements

5.2A.5.3       4RX requirements

7.2A             PDSCH Demodulation requirements for CA

7.2A.1          Void

7.2A.2          Void

7.2A.3          Minimum requirements

7.2A.3.1       1RX requirements (Void)

7.2A.3.2       2RX requirements

  - Option 2 (if allowed by TS drafting rules):

5.2A                   PDSCH Demodulation requirements for CA

5.2A.1                Minimum requirements (normal PDSCH)

5.2A.1.1            1RX requirements (Void)

5.2A.1.2            2RX requirements

5.2A.1.3            4RX requirements

5.2A.2                Minimum requirements for carrier aggregation with power imbalance

5.2A.2.1            1RX requirements (Void)

5.2A.2.2            2RX requirements

5.2A.2.3            4RX requirements

- Recommended WF

  - Encourage feedback from more companies.

  - If the proposed option 1 or option 2 is agreeable, suggest to prepare formal CR in this RAN4 meeting, to facilitate the RAN5 work in RAN5 #90e meeting. In addition, the section numbering for CA CQI needs to be updated accordingly.

Sub-topic 2-2: PDSCH CA test applicability

Issue 2-2-1: Antenna connection for CA tests with 4 RX

- Agreement in RAN4 #97e (R4-2017561, WF)

  - Option 1:

    - If any of the Pcell and/or the Scells is a 4 RX supported RF band, all 4 RX should be connected with data source from system simulator.

    - If any of the Pcell and/or the Scells is a 2 RX supported RF band, 2 out of the 4 RX should be connected with data source from system simulator, and the other 2 RX are connected with zero input.

  - Other options are not precluded

- Proposals

  - Option 1, i.e., reuse the same antenna connection for CA tests with 4RX in LTE (CTC, CMCC, Intel)

    - CTC: With option 1, the antenna connections for different scenarios (including all bands with 2RX, all bands with 4RX, 2RX bands + 4RX bands) are clear.

    - Intel: Two types of 4 Rx capable Ues may exist in the field: Type 1 (Ues only support 2Rx in certain bands and support 4Rx in the other bands) and Type 2 (Ues support 4Rx in all the bands). For Type 1 4 Rx capable Ues, test can be applied for CA configuration with mix of 2 and 4 Rx support.

  - Option 2: Reuse the test applicability rule defined for single carrier for CA tests with 2Rx and 4Rx, i.e. (HW)

    - Only conducted CA tests with 2Rx for UE only supports 2Rx

    - Only conducted CA tests with 4Rx for UE only supports 4Rx

    - Only conducted CA tests with 4Rx for UE supports both 2Rx and 4Rx

    - Not conducted CA test with 4Rx for UE only supports 2Rx

- Recommended WF

  - The main difference is on how to test UE supporting 2Rx in certain bands and supporting 4Rx in the other bands.

  - Considering the majority companies’ view, can we go with option 1?

Issue 2-2-2: Applicability for Ues not supporting 2-layer transmission on all CCs

- Proposals

  - Option 1: No further discussion is needed (CTC, CMCC, HW)

    - CTC: The agreed procedure for selecting CA configuration(s) and CBW combination can be used to find the test scenario where UE support at least 2 layer on each CC.

    - CMCC: the case that UE does not support 2-layer transmission on all CCs for all supported CA configurations is a conner case.

    - HW: Based on our understanding on the test applicability rule, for the selected CA configuration for test, only CCs supporting maximum number of MIMO layers not lower than 2 will be selected for tests, other CCs of the selected CA configuration not support maximum number of MIMO layers not lower than 2 will not be selected at all.

  - Option 2: Add the following clarification to NR CA applicability rules (Intel)

    - “Verify Ues only on CCs, for which the supported maximum number of MIMO layers is not lower than 2. OCNG pattern is used for CCs, for which the supported maximum number of MIMO layers is 1, and performance is not verified on these CCs”.

- Recommended WF

  - Can we go with option 1? Is the clarification in option 2 necessary?

Companies views’ collection for 1st round

Open issues

| Company | Comments |
| --- | --- |
| Company A | Issue 2-1: Section numbering for PDSCH CA demodulation requirements Issue 2-2-1: Antenna connection for CA tests with 4 RX Issue 2-2-2: Applicability for Ues not supporting 2-layer transmission on all CCs |
| Company B | Issue 2-1: Section numbering for PDSCH CA demodulation requirements Issue 2-2-1: Antenna connection for CA tests with 4 RX Issue 2-2-2: Applicability for Ues not supporting 2-layer transmission on all CCs |
| CMCC | Issue 2-2-1: Antenna connection for CA tests with 4 RX Support recommended WF to go with Option1 Issue 2-2-2: Applicability for Ues not supporting 2-layer transmission on all CCs Support Option1 |
| China Telecom | Issue 2-1: Section numbering for PDSCH CA demodulation requirements In our understanding, the motivation of the discussion is that: in the past, RAN5 always align their spec structure with 38.101-4. But this time, it is challenging for RAN5 to follow the current section numbering for PDSCH normal CA in 38.101-4, considering the Ues with different Rx antenna numbers (2Rx or 4Rx) in different bands. Meanwhile, we have to say that re-number the section in RAN4 is a big change to the RAN4 spec, resulting in several sections and sub-sections with “Void”. So, we are neutral on this issue. One point is that if it is agreeable to all companies in RAN4 to re-number the sections in 38.101-4, we should do it in this meeting but not in the future meeting, to minimize the impact due to potential cross-reference. Issue 2-2-1: Antenna connection for CA tests with 4 RX Option 1. CA test is different with single carrier test, for CA both 2Rx and 4Rx bands should be tested for Ues with 2RX bands + 4RX bands. Issue 2-2-2: Applicability for Ues not supporting 2-layer transmission on all CCs Option 1. The additional clarification in option 2 would not be needed, since the current test procedure is clear enough. |
| Intel | Issue 2-1: Section numbering for PDSCH CA demodulation requirements Based on our understanding, Option 2 is not possible based on TS drafting rules. Therefore, we can go only with Option 1 in case it will be agreed to make such changes. We think that proposed sections structure (i.e. Option 1) looks slightly better than existing one. Same time, such changes will lead to many “Void” in the TS. Therefore, we are fine with changes proposed in Option 1 or keep TS structure as it is. Issue 2-2-1: Antenna connection for CA tests with 4 RX Support Option 1. Based on our understanding, in NR we have 4 RX bands for which 4 RX processing is mandatory and for which it is optional. Therefore, for some 4 Rx bands 4 RX processing is up to UE implementation. We can face the situation where scenario with mix of 2 Rx and 4 Rx processing will be selected for testing. We think that it is better to verify performance for hall scenarios (i.e. 2 Rx CCs + 4Rx CCs) rather than only 4 Rx CCs. Issue 2-2-2: Applicability for Ues not supporting 2-layer transmission on all CCs Option 1 is fine for us. Intention of our proposal (i.e. Option 2) was to resolve the issue raised in the previous meeting. However, in this meeting, interested companies are fine with existing methodology. Therefore, we don’t need any clarification. |
| Qualcomm | Issue 2-1: Section numbering for PDSCH CA demodulation requirements We support Option 1 as discussed in our paper. Issue 2-2-1: Antenna connection for CA tests with 4 RX We have a few questions for clarification for Option 1: - When a UE supports 4Rx on one CC and 2Rx on another CC, then does this option mean that 4Rx will be connected for 1st CC and only 2Rx will be connected for 2nd CC? - When it says that “other 2Rx are connected with zero input”, does this mean that UE will actually receive zeros + some garbage noise and UE will process it or does this mean that UE will not receive anything on other 2Rx? Issue 2-2-2: Applicability for Ues not supporting 2-layer transmission on all CCs Ok with Option 1. |
| China Telecom 2 | Issue 2-2-1: Antenna connection for CA tests with 4 RX Reply to QC: To make it a little clearer, we copied the connection diagram from RAN5 LTE spec (TS 36.508) below (since the approach is reused from LTE CA normal PDSCH test): Figure A.94: Connection Diagram to enable Carrier Aggregation tests (with 2x4 channel) on a 4Rx-capable UE with CCs on both 4Rx-supported RF bands and 2Rx-supported bands (from RAN5 LTE spec TS 36.508) |
| Ericsson | Issue 2-1: Section numbering for PDSCH CA demodulation requirements We support Option 1 as explained the motivation in Qualcomm paper and our CR. Comments to China Telecom: our intension is to change the spec structure only for PDSCH CA part. We don’t see any issues for other requirements in TS38.101-4 V16.3.0. We don’t think it is a big change if you observe our draft CR R4-2101434. We agree we should fix it in this meeting to minimize the impact due to potential cross-reference. Issue 2-2-2: Applicability for Ues not supporting 2-layer transmission on all CCs We support Option 1. |
| ZTE | Issue 2-1: Section numbering for PDSCH CA demodulation requirements We are fine with one option following TS drafting rules of these two options. Issue 2-2-1: Antenna connection for CA tests with 4 RX Option 1 with the clarification on the question raised by QC according to the above figure. Issue 2-2-2: Applicability for Ues not supporting 2-layer transmission on all CCs Ok for Option 1. |
| Huawei, HiSilicon | Issue 2-1: Section numbering for PDSCH CA demodulation requirements Usually RAN5 should follow RAN4 specification structure, but now it is different. Actually from our point of view, we did not observe any issues for the current RAN4 existing section numbering by categorizing the performance requirements from the supported number of Rx antenna that is aligned with the original RAN4 agreement for the specification structure. Also like pointed out by other companies, many “Void” sections will be created that will make RAN4 specifications very ugly. We are not sure how much trouble will be brought to RAN5 work by following RAN4 current specification structure. Issue 2-2-1: Antenna connection for CA tests with 4 RX Here we would like to share some stories about LTE CA antenna connections: LTE demodulation performance requirements were defined based on 2Rx as baseline at first, in later release, RAN4 introduced performance requirements for 4Rx by a new WI, considering too heavy workload to define 4Rx related performance requirements for all scenarios considered for 2Rx, RAN4 agreed to define limited 4Rx performance requirements on top of 2Rx requirements, but UE supporting 4Rx should conduct the performance requirements defined for both 2Rx and 4Rx to ensure the test coverage. How to conduct those 2Rx related performance requirements for 4Rx capable UE is an issue, so related applicability rule and antenna connection are defined. But NR defined demodulation performance requirements with 4Rx as baseline in Rel-15, both 2Rx and 4Rx performance requirements are defined, performing separate 2Rx and 4Rx performance tests are enough to verify UE demodulation performance, it is not necessary to conduct the mixed 2Rx and 4Rx performance requirements considering the demodulation performance requirements are band agnostic, Issue 2-2-2: Applicability for Ues not supporting 2-layer transmission on all CCs Option 1. |

CRs/TPs comments collection

| CR/TP number | Comments collection |
| --- | --- |
| R4-2100816, CMCC, FR1 CA 2Rx | Company A: |
|  | Company B: |
| R4-2101255, Intel, applicability rules | CMCC: Since the sub-clause 5.1.1.7.3 in CR “Applicability rule and antenna connection for CA tests with 4 RX” is for CA tests, we propose to modify the description like below: Within the CA/DC configuration if any of the PCell and/or the SCells/PSCell is a 2Rx supported RF band, 2 out of the 4Rx should be connected with data source from system simulator, and the other 2Rx are connected with zero input, depending on UE’s declaration and AP configuration. Requirements from Clause 5.2A.2.1 are applied. Within the CA/DC configuration if any of the PCell and/or the SCells is a 4Rx supported RF band, all 4Rx should be connected with data source from system simulator. Requirements from Clause5.2A.3.1 are applied. |
|  | China Telecom: 1) The added note to Table 5.1.1.7.2-1 and Table 7.1.1.5.2-1 is pending on the conclusion in Issue 2-2-2. 2) Agree with CMCC’s comment and “PSCell” needs also to be removed. |
|  | Intel: Thank you for comments. We are fine with suggested proposals. We will revise our CR after collection of comments from all interested companies in the first round. |
|  | Qualcomm: It may have to be modified based on outcome of Issue 2-2-1. |
|  | ZTE: Another sentence under the first two sub-clauses may be required to clarify the situation when both sub-clauses are met. |
| R4-2101434, Ericsson, Draft CR: Section numbering for PDSCH CA | Company A: |
|  | Company B: |

Summary for 1st round

Open issues

|  | Status summary |
| --- | --- |
| Topic #2: UE CA PDSCH requirements | Issue 2-1: Section numbering for PDSCH CA demodulation requirements Candidate options: - Option 1: Update the section numbering for PDSCH CA normal requirements, with “void” for the originally used sections (Intel, QC, E///) - Option 2: Update the section numbering for PDSCH CA normal requirements, by changing the originally used sections - Option 3: Keep TS structure as it is. (Intel, HW) Recommendations: - Option 2 is precluded since it is not allowed by TS drafting rule. - Decide whether to go with Option 1 or Option 3 in the GTW. GTW Discussion on Jan. 28th: Samsung (spec editor of 38.101-4): From spec structure consistent aspect, we prefer to maintain the section structure alignment as much as possible, unless we see some specific technical reason, we can consider some exception cases carefully. QC: When we start with inter-working scenarios, these structure maybe not feasible. There are UEs equipped with 2Rx, and 4Rx on different CCs, from single test aspect, cross different sections needed. We usually focused on feature by features from RAN4, for CA test cases, readability aspect for CA, placed 2Rx, 4Rx in low level sections. For single CC test cases, only 2Rx or 4Rx need to be verified. Huawei: Share similar view as Samsung, in NR Rel-15, we spent lots of time to define the structure with feature basis vs Rx antennas as 3nd level sections; in the end we come up existing structures considering features themselves not stable and involved release by release which make specification not stable and uncontrollable. E///: For CA, power imbalance test cases, with mixed sections. We also think “void sections“not good from specification aspect. FFS for 2nd round discussion, if no consensus reached in this meeting and then no further discussion for renumbering issue. Recommendations for 2nd round: Further check if any company has concern on the Option 1 by Tuesday 6am UTC, Feb. 2. Issue 2-2-1: Antenna connection for CA tests with 4 RX Candidate options: - Option 1, i.e., reuse the same antenna connection for CA tests with 4RX in LTE (CMCC, CTC, Intel, ZTE) - If any of the Pcell and/or the Scells is a 4 RX supported RF band, all 4 RX should be connected with data source from system simulator. - If any of the Pcell and/or the Scells is a 2 RX supported RF band, 2 out of the 4 RX should be connected with data source from system simulator, and the other the corresponding 2 RX are connected based on UE declaration with zero input. - Option 2: Reuse the test applicability rule defined for single carrier for CA tests with 2Rx and 4Rx, i.e. (HW) - Only conducted CA tests with 2Rx for UE only supports 2Rx - Only conducted CA tests with 4Rx for UE only supports 4Rx - Only conducted CA tests with 4Rx for UE supports both 2Rx and 4Rx - Not conducted CA test with 4Rx for UE only supports 2Rx Recommendations: - The main difference between the two options is on how to test UE supporting 2Rx in some bands and supporting 4Rx in the other bands (see the 3rd bullet in option 2). - Further check can we go with Option 1 by following majority companies’ view? Any additional clarification questions to Option 1? GTW Discussion on Jan. 28th: QC: For op1, why do we need to test UE with 4Rx for 2Rx test set-up? Huawei: For op1 with 2nd bullet, it’s misunderstanding. If UE support 2Rx and 4Rx in different CCs, 2Rx CC pass 2Rx and 4Rx CC pass 4Rx requirements. Intel: how to apply for UE under 2Rx CC? our understanding as captured in option1. China Telecomm: CA test cases different with single CC cases. Both 2Rx CC and 4Rx CC need to be verified with corresponding requirements. Huawei: Need to improve the wording for option 1. Question: How to apply test set-up for UE which support 2Rx and 4Rx in different CCs? QC: Agreements: - Reuse the same antenna connection for CA tests with 4RX in LTE (CMCC, CTC, Intel, ZTE) - If any of the Pcell and/or the Scells is a 4 RX supported RF band, all 4 RX should be connected with data source from system simulator. - If any of the Pcell and/or the Scells is a 2 RX supported RF band, the corresponding 2 RX are connected based on UE declaration Further work the details texts into specification for above agreements. Recommendations for 2nd round: Work on the CR revision based on the GTW agreement. Issue 2-2-2: Applicability for UEs not supporting 2-layer transmission on all CCs - Option 1: No further discussion is needed (CMCC, CTC, Intel, QC, E///, ZTE, HW) - Option 2: Add the following clarification to NR CA applicability rules - “Verify UEs only on CCs, for which the supported maximum number of MIMO layers is not lower than 2. OCNG pattern is used for CCs, for which the supported maximum number of MIMO layers is 1, and performance is not verified on these CCs”. Tentative agreement: - Agree on Option 1, i.e., no further discussion is needed |

Suggestion on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |
| R4-2100816, CMCC, FR1 CA 2Rx | agreeable |
| R4-2101255, Intel, applicability rules | revised |
| R4-2101434, Ericsson, Draft CR: Section numbering for PDSCH CA | Not pursued. (Note: this draft CR is recommended as not pursued. Based on the further discussion, a formal CR can be requested in the 2nd round if needed. ) |

Discussion on 2nd round

R4-2103841	CR on applicability rules for Normal NR CA requirements

Type: CR		For: Agreement
					38.101-4 v16.3.0	  CR-0142  Cat: B (Rel-16)

					Source: Intel Corporation

Discussion:

[report of discussion]

Intel: Note: Wording “2 out of the 4Rx” was kept in comparison to version from GTW “the corresponding 2 Rx”, because, based on our view, it provides the clearer antenna connection procedure. Same time, please fell free to provide any suggestion on wording change.

Recommendation:		Agreeable

[98e][326] NR_perf_enh_Demod - Section numbering for CA requirements (led by Ericsson)

Issue 2-1: Section numbering for PDSCH CA demodulation requirements

Candidate options:

- Option 1: Update the section numbering for PDSCH CA normal requirements, with “void” for the originally used sections

- Option 3: Keep TS structure as it is.

| Company | 2nd round comments |
| --- | --- |
| Ericsson | We prefer Option 1. The reason RAN5 want to combine 2Rx and 4Rx into the same test case for CA is because the number of combinations of 2Rx and 4Rx (4Rx might only be on some of the CCs) would be very large if we don’t combine. In RAN4 performance core spec (TS38.101-4), PDSCH CA requirements specify the SNR requirements per CC/CBW/SCS, and the actual number of combinations is specified in the applicability rule in 5.1.1.7. On the other hand, RAN5 spec need to specify for each number of CCs, that is, 2CC, 3CC, 4CC,… Since RAN5 spec tries to align the spec structure with RAN4 spec, if RAN5 follow the existing RAN4 spec structure, the RAN5 spec for 2Rx becomes as below, and RAN5 need to copy almost same contents for 4Rx. X.Y 2Rx requirements X.Y.1_1 PDSCH for CA (2CC) X.Y.1_2 PDSCH for CA (3CC) X.Y.1_3 PDSCH for CA (4CC) X.Y.1_4 PDSCH for CA (5CC) X.Y.2_1 PDSCH for CA power imbalance (2CC) X.Y.2_2 PDSCH for CA power imbalance (3CC) X.Y.2_3 PDSCH for CA power imbalance (4CC) X.Y.2_4 PDSCH for CA power imbalance (5CC) X.Z 4Rx requirements X.Z.1_1 PDSCH for CA (2CC) … X.Z.2_1 PDSCH for CA power imbalance (2CC) …. Since the contents of CA configuration are not small, we would like to separate PDSCH CA requirements and CA power imbalance requirements to avoid the duplication of the same contents. |
| Intel | We prefer Option 3 at current stage to keep RAN4 specification clearer. Same time, based on our understanding, the following structure will be in RAN5 specification in case Option 1 will be adopted: X.Y PDSCH for CA X.Y.1_1 2Rx requirements (2CC) X.Y.1_2 2Rx requirements (3CC) X.Y.1_3 2Rx requirements (4CC) X.Y.1_4 2Rx requirements (5CC) X.Y.2_1 4Rx requirements (2CC) X.Y.2_2 4Rx requirements (3CC) X.Y.2_3 4Rx requirements (4CC) X.Y.2_4 4Rx requirements (5CC) X.Z PDSCH for CA power imbalance X.Z.1_1 2Rx requirements (2CC) … X.Z.2_1 4Rx requirements (2CC) …. If our understanding is correct then we would to double check what is the fundamental difference between Option 1 and 3, because we assume that content under Sections X.Y.1_1, X.Y.1_2 etc will be the same or we missed something? |
| Qualcomm | We prefer Option 1. We had discussed with our RAN5 colleagues and below are further clarifications from our side. If we go with Option 3, 4Rx requirements will be referenced in 2Rx clause and vice-versa which doesn’t make logical sense. Also, a lot of content in 2Rx section will have to be copied to 4Rx sections. RAN5 spec structure will look like below. Furthermore, they will have 2 sections for a single test because UE can support 2Rx on one CC and 4Rx on other, which doesn’t make sense for RAN5 spec. X.Y 2Rx requirements X.Y.1_1 PDSCH for CA (2CC) - Cross reference 4Rx requirements under 2Rx clause since UE can support 2Rx on one CC and 4Rx on another X.Y.1_2 PDSCH for CA (3CC) X.Y.1_3 PDSCH for CA (4CC) X.Y.1_4 PDSCH for CA (5CC) X.Y.2_1 PDSCH for CA power imbalance (2CC) X.Y.2_2 PDSCH for CA power imbalance (3CC) X.Y.2_3 PDSCH for CA power imbalance (4CC) X.Y.2_4 PDSCH for CA power imbalance (5CC) X.Z 4Rx requirements X.Z.1_1 PDSCH for CA (2CC) - Cross reference 2Rx requirements under 4Rx clause since UE can support 4Rx on one CC and 2Rx on another … X.Z.2_1 PDSCH for CA power imbalance (2CC) …. To Intel: If we go with Option 1, RAN5 spec structure will look like below. So, if any changes or additions are made to, for example, normal CA requirements, then RAN5 can still keep referring to Section X.Y for CA requirements and they will have less maintenance issues. With Option 3, changes will be made under 2Rx and 4Rx sections and will require maintenance in more sections. X.Y PDSCH for CA X.Y_1 2CC requirements X.Y_1.0 Minimum Requirements <No further subsections separating 2Rx/4Rx> … X.Y_2 3CC requirements X.Y_3 4CC requirements … X.Z PDSCH for CA power imbalance X.Z_1 2CC requirements … X.Z_2 3CC requirements …. |
| Samsung | Our preference is option 3, keep the spec as it is. What I would like to highlight, the CR already agreed in last RAN4 meeting and incorporated in the latest 101-4 specifications. Which means this test cases already completed, any additional modifications to the spec should be consensus basis. Regarding spec section renumbering, keep spec structure consistent and in a future proof manner quite important. Current RAN4 101-4 structure following such orders: 1. Split based on radiated, conducted, interworking , basically it’s aligned with the split for 101-1, 101-2,101-3 UE RF core specification. 2. Second level, Demod based on physical channel, CSI based on reporting contents 3. Third level/fourth level, number of Rx -> Duplex mode Based on companies’ comment, the only reason they want to change the section numbering was to facilitate RAN5 specification. Typically RAN5 will follow RAN4 specification structure, but not really necessary to strictly align two specifications across two WGs in one by one mapping manner. How they introduce RAN5 specification is up to RAN5 experts and spec editors based on RAN5 expertise. RAN4 and RAN5 are separate working groups, they have different work areas and different tasks i.e. RAN4 aims to define proper requirements to ensure sys, BS, terminal performance and RAN5 aims to define UE conformance test cases. Given the different work areas and different work expertise, RAN4 generate specifications including structure based on RAN4 experts’ input with considering many factors to decide what’s the most reasonable way (of course, RAN5 impact also considered when decided the structure, but this is not the only factor we consider, also be sure not the most factor we considered). From requirements aspect, clearly 2Rx and 4Rx have different performance, that’s the reason we differentiate UE performance with number of Rx in separate sections. I still didn’t see clear difference among these options, also no matter which option we always need to differentiate number of Rx antennas. Meanwhile with option 1, we will introduce “void” section in the spec and broke the consistent spec structure without clear benefits. Also there are several CRs pending on the decision of this, for sake of progress we proposed below solution: 1. In RAN4, take as it is since we didn’t have consensus and move forward. 2. For RAN5 issue, we can leave it to RAN5, I believe RAN5 experts can handle this quite well and make refinement in RAN5 specifications if necessary. |

Moderator: Based on the feedback, it looks consensus on re-numbering the sections for CA requirements cannot be reached, so let us keep the spec structure as it is.

Summary on 2nd round

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| XXX | Based on 2nd round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |
| R4-2103841, Intel, applicability rules | agreeable |

Topic #3: UE	PMI reporting requirements with larger number of Tx ports

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2100216 | Apple | Proposal #1: Define PMI reporting requirements with Type II codebook at 90% Max TP. |
| R4-2100897 | Samsung | Wrong document submitted, revised to R4-2102939. |
| R4-2100902 | Samsung | Simulation results for Type II and Type I codebook based on the agreements in RAN4 #97e meeting. Observation 1-MIMO Correlation: It is observed that the performance gain with Type II compared to Type I codebook in XP medium MIMO correlation is larger than in Custom Low correlation cases. Especially for 16x2 ‘Custom Low’, there is marginal gain for Type II codebook. Observation 2-Test Metric: As shown in table 2.1~2.3, the TP ratios are reasonable and the SNR points are workable under the “following Type II PMI vs. random Type I PMI” test metrics. Observation 3-Test point: The performance gap between following Type II and Type I is more obvious under 70% relative TP point than 90%, and 95% points. Observation 4- Performance gap between Type II and Type (XP-Medium with 70% relative test point): 2.5dB around SNR gap for 2Rx cases, 1.5dB around SNR gap for 4Rx cases Proposal 1: Introduce Type II codebook test cases - SU-MIMO set-up - MIMO correlation: XP Medium MIMO correlation - Test metric: Relative TP between following Type-II/random PMI with Type I codebook - Test point: relative 70% TP with following PMI |
| R4-2101317 | Huawei, HiSilicon | Simulation results for Type II codebook PMI reporting test |
| R4-2101318 | Huawei, HiSilicon | Proposal 1: Use 95% maximum throughput to be the test point for eType II codebook |
| R4-2101322 | Huawei, HiSilicon | CR for 38.101-4: Applicability for NR PMI requirements with Tx ports larger than 8 and up to 32 |
| R4-2101435 | Ericsson | Simulation results. |
| R4-2101436 | Ericsson | Proposal: RAN4 revisit the metric of Rel-15 Type-II PMI reporting test to ensure the UE reporting Type-I cannot pass the tests, e.g., following Type-II PMI over following Type-I PMI. |
| R4-2101437 | Ericsson | CR Correction of title on 16Tx port subband PMI reporting |
| R4-2102821 | Qualcomm Incorporated | Proposal 1: Use 90% test point for defining Type II PMI reporting test cases. Proposal 2: Use the test metric of throughput ratio between following Type II and random Type I for defining Type II PMI reporting tests and no need to check whether UE reported codebook is not only within Type I codebook set. Proposal 3: Use XP Medium correlation for defining Type II PMI reporting tests. |
| R4-2102939 | Samsung | CR Introduction of PMI test cases with Rel-15 Type II codebook |

Open issues summary

Issue 3-1: Test metric for type II codebook

- Agreement in RAN4 #97e (R4-2017681, WF)

  - Test metric:

    - Following PMI (Type II)/Random PMI (Type I codebook) (gamma values) based on the assumption that there are ensure enough performance difference over than Type I i.e., UE which employ Type I reporting will fail in the test case

      - This test metric applied to UE which support Type II codebook feedback irrespective whether supporting Type I codebook feedback or not

    - FFS: Whether to check UE reported codebook not only within Type I codebook set

- Proposals

  - Option 1: Following PMI (Type II)/Random PMI (Type I codebook) (gamma values) (Samsung, QC, [Apple], [HW])

    - Samsung: TP ratios are reasonable and the SNR points are workable under the “following Type II PMI vs. random Type I PMI” test metrics.

  - Option 2: Following Type-II PMI / following Type-I PMI (E///)

    - E///: There is less performance difference or almost no performance difference in terms of gamma value between the Type-I PMI reporting and Type-II PMI reporting.

- Recommended WF

  - Considering this is the last meeting for the WI, can we go with option 1 based on majority’s view? Any additional measure point to address E///’s concern?

Issue 3-2: SNR point for type II codebook

- Agreement in RAN4 #97e (R4-2017681, WF)

  - Test point

    - Option 1: 70%

    - Option 2: 90% (baseline)

    - Option 3: 95%

    - Other options not excluded

- Proposals

  - Option 1: 70% (Samsung)

    - Samsung: Performance gap between following Type II and Type I is more obvious under 70% relative TP point than 90%, and 95% points.

  - Option 2: 90% (Apple, QC)

    - Apple: At 90% max TP, we have 3 dB gain in performance and considerable difference in TP gain. Using a lower test point like 80% or 70% max TP would result in larger delta between Type II and Type I results.

    - QC: Link adaptation algorithms try to keep the UE PDSCH BLER closer to 10%.

  - Option 3: 95% (HW)

    - HW: The ratio between Type II follow PMI and Type I random PMI on 70% and 90% of maximum throughput is too large to be set as a proper test metric.

- Recommended WF

  - 4 companies proposed 3 different options due to different observations from the simulation results.

    - In general, in all companies’ results, with higher percentage of TP point, smaller performance gap between following Type II and random Type I is observed; but the amount of the gap is quite different in different companies’ simulation results.

  - In the 1st round, encourage companies to double check the simulation results, and check if there is another acceptable option in addition to the favourite option?

Issue 3-3: MIMO correlation for type II codebook

- Agreement in RAN4 #97e (R4-2017681, WF)

  - MIMO correlation

    - XP Medium as Baseline

    - XP (custom) Low only can be considered if XP medium not workable

- Proposals

  - Option 1: XP Medium (Samsung, QC)

    - Samsung: The performance gain with Type II compared to Type I codebook in XP medium MIMO correlation is larger than in Custom Low correlation cases. Especially for 16x2 ‘Custom Low’, there is marginal gain for Type II codebook.

- Recommended WF

  - Use XP Medium.

Issue 3-4: Simulation results and gamma value

- Summary of simulation results for following Type II/Random Type I

Summary of FDD simulation results

| Antenna configuration | Correlation | Company | SNR Point [dB 95% max TP] | Gamma [95% max TP] | SNR Point [dB 90% max TP] | Gamma [90% max TP] | SNR Point [dB 70% max TP] | Gamma [70% max TP] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 16Tx / 2Rx | Medium | E/// | 14.5 | 1.6 | 12.4 | 1.9 | 7.8 | 2.2 |
|  |  | Apple |  |  | 10.56 | 2.92 |  |  |
|  |  | Samsung | 10.8 | 2.88 | 9.5 | 3.18 | 6.8 | 4.18 |
|  |  | Huawei |  |  | 9.82 |  |  |  |
|  | Custom Low | E/// | 12.8 | 1.7 | 10.9 | 1.8 | 7.2 | 2.1 |
|  |  | Samsung | 9.9 | 2.65 | 9.0 | 2.87 | 6.6 | 3.72 |
| 16Tx / 4Rx | Medium | E/// | 7.6 | 2.1 | 6.8 | 2.1 | 4.4 | 2.2 |
|  |  | Apple |  |  | 7.06 | 2.83 |  |  |
|  |  | Samsung | 8.6 | 2.01 | 7.6 | 2.24 | 5.2 | 2.51 |
|  |  | Huawei |  |  | 5.9 |  |  |  |
|  | Custom Low | E/// | 7.8 | 1.9 | 6.7 | 1.8 | 4.3 | 1.9 |
|  |  | Samsung | 6.9 | 2.27 | 5.8 | 2.38 | 4.0 | 2.41 |

Summary of TDD simulation results

| Antenna configuration | Correlation | Company | SNR Point [dB 95% max TP] | Gamma [95% max TP] | SNR Point [dB 90% max TP] | Gamma [90% max TP] | SNR Point [dB 70% max TP] | Gamma [70% max TP] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 16Tx / 2Rx | Medium | E/// | 10.3 | 2.2 | 8.8 | 2.7 | 5.7 | 3.1 |
|  |  | Apple |  |  | 11.25 | 2.82 |  |  |
|  |  | Samsung | 13.3 | 1.87 | 11.3 | 2.12 | 7.8 | 2.43 |
|  |  | Huawei |  |  | 9.82 |  |  |  |
|  | Custom Low | E/// | 10.0 | 2.0 | 7.9 | 2.3 | 5.4 | 2.8 |
|  |  | Samsung | 10.8 | 2.03 | 9.3 | 2.23 | 6.4 | 2.51 |
| 16Tx / 4Rx | Medium | E/// | 5.3 | 2.4 | 4.5 | 2.5 | 2.5 | 2.6 |
|  |  | Apple |  |  | 7.15 | 2.81 |  |  |
|  |  | Samsung | 9.6 | 1.81 | 8.2 | 1.91 | 5.7 | 1.96 |
|  |  | Huawei |  |  | 5.9 |  |  |  |
|  | Custom Low | E/// | 5.4 | 1.9 | 4.6 | 2.0 | 2.5 | 1.9 |
|  |  | Samsung | 9.3 | 1.64 | 7.8 | 1.76 | 4.8 | 1.90 |

- Observation

  - For the baseline parameter combination, i.e., Medium correlation + 90% max TP, 4 companies provided simulation results, and  the SPAN of the SNR point is quite large for some cases, e.g., 3.7 for TDD 16T4R, 2.5 for TDD 16T2R, 2.9 for FDD 16T2R.

- Recommended WF

  - In the 1st round, encourage companies to double check the simulation results

Companies views’ collection for 1st round

Open issues

| Company | Comments |
| --- | --- |
| Company A | Issue 3-1: Test metric for type II codebook Issue 3-2: SNR point for type II PMI codebook Issue 3-3: MIMO correlation for type II codebook Issue 3-4: Simulation results and gamma value Others |
| Apple | Issue 3-1: Test metric for type II codebook Results we presented show delta between Type II and Type I performance. We support recommended WF. Issue 3-2: SNR point for type II PMI codebook We see reasonable delta at 90% Max TP. With lower for e.g. 70% Max TP, we expect to see higher TP gain and more delta compared to Type I performance. We are fine with going with 70% max TP if that’s the majority view. Issue 3-3: MIMO correlation for type II codebook We support recommended WF. Issue 3-4: Simulation results and gamma value Others |
| China Telecom | Issue 3-1: Test metric for type II codebook Support option 1 for the timely completion of the WI. Issue 3-2: SNR point for type II PMI codebook Slightly prefer option 2, which is the baseline and with a wider support from companies. Issue 3-3: MIMO correlation for type II codebook Support the Recommended WF. Issue 3-4: Simulation results and gamma value Others |
| Qualcomm | Issue 3-1: Test metric for type II codebook We support Option1. In some cases, E///’s SNR required at test point is an outlier compared to other companies’ results. So, it will be good if they could double check their simulations during the meeting. Issue 3-2: SNR point for type II PMI codebook Prefer Option 2. Issue 3-3: MIMO correlation for type II codebook Ok with recommended WF. Issue 3-4: Simulation results and gamma value Others |
| Samsung | Issue 3-1: Test metric for type II codebook We support option 1 and recommend WF. Based on the most of companies results, TP ratios are reasonable and the SNR points are workable under the “following Type II PMI vs. random Type I PMI” test metrics Issue 3-2: SNR point for type II PMI codebook From our results, we can observe with 70% relative TP point can achieve better performance gap between Type II and Type I compared with 90% relative TP point. We are also OK with 90% relative TP point with majority review. Issue 3-3: MIMO correlation for type II codebook We are ok with option 1 and recommended WF. Issue 3-4: Simulation results and gamma value Our results will be updated slightly during this meeting. |
| Ericsson | Issue 3-1: Test metric for type II codebook We are generally fine with Option 1, if the final gamma of Rel-15 Type-II (TP ratio of following Rel-15 Type-II and random Type-I) becomes tighter than the TP ratio of following Type-I and random Type-I). Issue 3-2: SNR point for type II PMI codebook We propose to review the simulation summary after the 1st round. If we observe the gamma with Rel-15 Type-II is tighter than gamma with Type-I, we are also fine to set SNR to 90% of the maximum throughput with the followed Rel-15 Type-II PMI. Issue 3-3: MIMO correlation for type II codebook We are fine with the recommended WF, XP medium. Issue 3-4: Simulation results and gamma value |
| Huawei, HiSilicon | Issue 3-1: Test metric for type II codebook We prefer option 1. We have observed performance difference between eType II and Type I based on the agreed simulation assumption and test metric. In the meanwhile, candidate test metric: follow eType II TP over random eType II TP suffers from the uncertainty of random eType II implementation among companies. Another candidate test metric: follow eType II TP over follow Type I TP will mix two optional feature into one test. Thus, we choose to stick to the agreement. Issue 3-2: SNR point for type II PMI codebook Option 2 and option 3 are both fine for us. We have updated our simulation results for 90% maximum TP. Option 2 is also fine for us since there is reasonable gap between two performances. Issue 3-3: MIMO correlation for type II codebook Prefer XP medium. Since there is no issue has been found yet using XP medium, we prefer to stick to the previous agreement. Issue 3-4: Simulation results and gamma value We have updated our simulation results in the table. Besides, we propose to set the required throughput ratio at least larger than γType-I. |

CRs/TPs comments collection

| CR/TP number | Comments collection |
| --- | --- |
| R4-2101322, HW, CR for Applicability | Apple: Agree that new table with applicability for optional features should be introduced. The applicability rule suggests that same UE feature is used to indicate support of both Type I with >8TX and Type II. In addition to support of 16TX for Type II, other UE features should also be included. |
|  | China Telecom: Share the similar view with Apple on the optional support of the whole type II codebook feature. In addition, the section title for 6.1.1.3 is duplicated. |
|  | Intel: 1) Based on 38.822 “Supported max # of configured NZP-CSI-RS resources per CC” (part of 2-33) is mandatory with capability upports feature. Therefore, the original version for Type I PMI is correct. Based on our understanding, same feature cannot be captured in table with optional and mandatory features. Same time, we probably need to define applicability for features/capabilities 2-36 and 2-41 (independent from 2-33) which inform about supported CSI-RS configuration and number of Tx ports for each codebook configuration. 2) Based on our review, Sections 6.3.2.1.5, 6.3.3.1.5, 6.3.2.2.5, 6.3.3.2.5 are not available in Table 6.1.1.4-1 of TS 38.101-1 v16.3.0. Therefore, removing of these sections from Table 6.1.1.4-1 is impossible. |
| R4-2101437, E///, Correction of title on 16Tx port subband PMI | apple: Sub-section heading cannot be changed based on our understanding of TS drafting rules. We are not sure why 16TX Type I PMI reporting is multiple PMI |
|  | China Telecom: Support the change from the technical point of view, i.e., multiple PMI for subband reporting, and single PMI for wideband reporting. |
|  | Ericsson: To Apple, the 16TX Type-I PMI reporting test uses sub-band PMI reporting. This is the reason the title should be ‘Multiple-PMI’. For the sub-section heading, our understanding is we can modify the heading, although we cannot remove section (should be set to ‘Void’). |
| R4-2102939, Samsung, Introduction of PMI test cases with Rel-15 Type II codebook | Moderator’s note: Endorsed draft CR R4-2017569 with updates to the notes in Table 6.3.2.1.5-1, Table 6.3.2.2.5-1, Table 6.3.3.1.5-1 and Table 6.3.3.2.5-1. |
|  | Company A |
|  | Company B |

Note: To save time on typing the comments one by one, companies can also directly revise the draft CR and upload the revision in the draft inbox.

Summary for 1st round

Open issues

|  | Status summary |
| --- | --- |
|  | Issue 3-1: Test metric for type II codebook Tentative agreement: - Following PMI (Type II)/Random PMI (Type I codebook) (gamma values) (Samsung, QC, Apple, HW, CTC, E/// - fine if tighter gamma compared to Type I codebook) - E///: Generally fine with Option 1, if the final gamma of Rel-15 Type-II (TP ratio of following Rel-15 Type-II and random Type-I) becomes tighter than the TP ratio of following Type-I and random Type-I) GTW Agreements: Following PMI (Type II)/Random PMI (Type I codebook) Issue 3-2: SNR point for type II PMI codebook Tentative agreement: - 90% max TP (Apple, QC, CTC, Samsung, E/// - fine if tighter gamma compared to Type I codebook) - E///: Propose to review the simulation summary after the 1st round. If we observe the gamma with Rel-15 Type-II is tighter than gamma with Type-I, we are also fine to set SNR to 90%. GTW Agreements: 90% max TP Issue 3-3: MIMO correlation for type II codebook Tentative agreement: - XP Medium (Samsung, QC, Apple, CTC, E///, HW) GTW Agreements: XP Medium Issue 3-4: Simulation results and gamma value - Companies’ simulation results (TP ratio with following Type II / random Type I) under XP medium and 90% max TP TxRx; Duplex; BW; SCS; E///; Apple; Samsung; QC; HW. 16x2; FDD; 10; 15; 1.9; 2.92; 3.1; 3.62; 3.8. 16x4; 2.1; 2.83; 2.92; 2.77; 3.2. 16x2; TDD; 40; 30; 2.7; 2.82; 2.12; 3.35; 3.9. 16x4; 2.5; 2.81; 1.9; 2.63; 3.1. - Companies’ simulation results (TP ratio with following Type I / random Type I) under XP medium and 90% max TP TxRx; Duplex; BW; SCS; E///; Apple; Samsung; QC; HW. 16x2; FDD; 10; 15; 1.5; 2.06; 2.1; 2.27; 2.1. 16x4; 1.7; 2.26; 2.3; 2.14; 2.12. 16x2; TDD; 40; 30; 2.1; 2.02; 1.83; 2.17; 2.2. 16x4; 2.3; 2.3; 1.75; 2.08; 2.3. Recommendations for 2nd round: With the intention to tighten the gamma requirements whenever possible, discuss and decide the tentative gamma requirements in the sub-thread for the CR revision. |

Suggestion on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| #1 | Simulation assumptions for NR PMI reporting requirements for more than 8 Tx ports (Note: Update the simulation assumptions to capture the GTW agreements) | Ericsson |
| #2 | Summary of simulation results of Rel-15 Type-II UE PMI test | Ericsson |

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |
| R4-2101322, HW, CR for Applicability | to be revised |
| R4-2101437, E///, Correction of title on 16Tx port subband PMI | agreeable |
| R4-2102939, Samsung, Introduction of PMI test cases with Rel-15 Type II codebook | to be revised (In the 2nd round, discuss the gamma requirements in the sub-thread for this CR revision) |
| R4-2100902, Samsung, Discussion and simulation results for Type II codebook | to be revised (The simulation results need to be updated) |

Discussion on 2nd round

R4-2103847	Summary of simulation results of Rel-15 Type-II UE PMI test

Type: other		For: Information
					Source: Ericsson

Abstract:

Discussion:

Recommendation:		Noted

R4-2103846	Simulation assumptions for NR PMI reporting requirements for more than 8 Tx ports

Type: other		For: Approval
					Source: Ericsson

Abstract:

Discussion:

CTC: The simulation assumption provided by Ericsson looks OK for us.

Recommendation:		Agreeable

R4-2103842	CR for 38.101-4 Applicablity of PMI reporting test with Tx ports larger than 8 and up to 32

Type: CR		For: Agreement
					38.101-4 v16.3.0	  CR-0149  Cat: B (Rel-16)

					Source: Huawei, HiSilicon

Discussion:

[report of discussion]

HW: Based on the comments, I’ve made following changes:

- Feature of 2-36 for Type I single panel has been added

- Whole part of feature 2-41 has been added

- Remove the duplicated chapter name’6.1.1.3 ……’

- Remove the tentative delete on 6.3.2.1.5, 6.3.3.1.5, 6.3.2.2.5, 6.3.3.2.5

Besides, there are two things I would like to know if companies have common understanding:

- Regarding the specific feature for Type I single panel with Tx ports larger than 8, there is no ‘optional with capability…’ feature for it in 38.822. So is it possible to first reuse the feature of 2-33 and also add 2-36 for Type I single panel with Tx ports larger than 8?

- Is it necessary to add feature 2-36(Type I single panel) into the table 6.1.1.4-1?

Intel: We have the following comments:

- Type I codebooks with Tx ports larger than 8 is mandatory with capability feature. Therefore, clauses 6.3.2.1.3, 6.3.2.1.4, 6.3.3.1.3, 6.3.3.1.4, 6.3.2.2.3, 6.3.2.2.4, 6.3.3.2.3, 6.3.3.2.4 should be captured in section 6.1.1.4.

- Supported codebook configuration is reported in the following format based on 38.331

CodebookParameters ::= SEQUENCE { type1 SEQUENCE { singlePanel SEQUENCE { supportedCSI-RS-ResourceList SEQUENCE (SIZE (1.. maxNrofCSI-RS-Resources)) OF SupportedCSI-RS-Resource, modes ENUMERATED {mode1, mode1andMode2}, maxNumberCSI-RS-PerResourceSet INTEGER (1..8) }, multiPanel SEQUENCE { supportedCSI-RS-ResourceList SEQUENCE (SIZE (1.. maxNrofCSI-RS-Resources)) OF SupportedCSI-RS-Resource, modes ENUMERATED {mode1, mode2, both}, nrofPanels ENUMERATED {n2, n4}, maxNumberCSI-RS-PerResourceSet INTEGER (1..8) } OPTIONAL }, type2 SEQUENCE { supportedCSI-RS-ResourceList SEQUENCE (SIZE (1.. maxNrofCSI-RS-Resources)) OF SupportedCSI-RS-Resource, parameterLx INTEGER (2..4), amplitudeScalingType ENUMERATED {wideband, widebandAndSubband}, amplitudeSubsetRestriction ENUMERATED {supported} OPTIONAL } OPTIONAL, type2-PortSelection SEQUENCE { supportedCSI-RS-ResourceList SEQUENCE (SIZE (1.. maxNrofCSI-RS-Resources)) OF SupportedCSI-RS-Resource, parameterLx INTEGER (2..4), amplitudeScalingType ENUMERATED {wideband, widebandAndSubband} } OPTIONAL }

Therefore, probably, to simplify applicability description, we can consider the following approach:

Table 6.1.1.3-1: Requirements applicability for optional UE features

| UE feature/capability [14] | Test type |  | Test list | Applicability notes |
| --- | --- | --- | --- | --- |
| Support of Type II single panel codebook (type2 is part of CodebookParameters) | FR1 FDD | PMI | Clause 6.3.2.1.5 Clause 6.3.3.1.5 |  |
|  | FR1 TDD | PMI | Clause 6.3.2.2.5 Clause 6.3.3.2.5 |  |

Table 6.1.1.4-1: Requirements applicability for mandatory features with UE capability signalling

| UE feature/capability [14] | Test type |  | Test list | Applicability notes |
| --- | --- | --- | --- | --- |
| Support of Type I single panel codebook (type1 and singlePanel are part of CodebookParameters) | FR1 FDD | PMI | Clause 6.3.2.1.1 Clause 6.3.2.1.2 Clause 6.3.2.1.3 Clause 6.3.2.1.4 Clause 6.3.3.1.1 Clause 6.3.3.1.2 Clause 6.3.3.1.3 Clause 6.3.3.1.4 |  |
|  | FR1 TDD | PMI | Clause 6.3.2.2.1 Clause 6.3.2.2.2 Clause 6.3.2.2.3 Clause 6.3.2.2.4 Clause 6.3.3.2.1 Clause 6.3.3.2.2 Clause 6.3.3.2.3 Clause 6.3.3.2.4 |  |

- All PMI test cases (Type I and Type II) can be included in the test list for the following feature: Supported maximum number of ports across all configured NZP-CSI-RS resources per CC (maxConfigNumberPortsAcrossNZP-CSI-RS-PerCC), because, based on our understanding, this feature is related to all scenarios with number of CSI-RS ports higher than 2.

HW: Your proposed approach looks much concise

Based on your approach, we have the following suggestions:

Table 6.1.1.4-1: Requirements applicability for mandatory features with UE capability signalling

| UE feature/capability [14] | Test type |  | Test list | Applicability notes |
| --- | --- | --- | --- | --- |
| Support of Type I single panel codebook (supportedCSI-RS-ResourceList) (modes) (maxNumberCSI-RS-PerResourceSet) | FR1 FDD | PMI | Clause 6.3.2.1.1 Clause 6.3.2.1.2 Clause 6.3.2.1.3 Clause 6.3.2.1.4 Clause 6.3.3.1.1 Clause 6.3.3.1.2 Clause 6.3.3.1.3 Clause 6.3.3.1.4 |  |
|  | FR1 TDD | PMI | Clause 6.3.2.2.1 Clause 6.3.2.2.2 Clause 6.3.2.2.3 Clause 6.3.2.2.4 Clause 6.3.3.2.1 Clause 6.3.3.2.2 Clause 6.3.3.2.3 Clause 6.3.3.2.4 |  |

And for Type II:

Table 6.1.1.3-1: Requirements applicability for optional UE features

| UE feature/capability [14] | Test type |  | Test list | Applicability notes |
| --- | --- | --- | --- | --- |
| Support of Type II single panel codebook (supportedCSI-RS-ResourceList) (parameterLx) (amplitudeScalingType) (amplitudeSubsetRestriction) | FR1 FDD | PMI | Clause 6.3.2.1.5 Clause 6.3.3.1.5 |  |
|  | FR1 TDD | PMI | Clause 6.3.2.2.5 Clause 6.3.3.2.5 |  |

Intel:

We think that information about configuring of “type1” and “singlePanel” or “type2” in “CodebookParameters” provides the better definition of each feature.

Probably we can consider the following definition:

- Support of Type I single panel codebook (CodebookParameters contains type1, singlePanel, supportedCSI-RS-ResourceList, modes, maxNumberCSI-RS-PerResourceSet)

- Support of Type II single panel codebook (CodebookParameters contains type2, supportedCSI-RS-ResourceList, parameterLx, amplitudeScalingType, amplitudeSubsetRestriction)

Another comment: definition of “Support of Type I single panel codebook” feature affects Rel-15 requirements. Therefore, probably we need to define this feature in the next meeting for both Rel-15 and Rel-16 requirements. And in this meeting we can define only applicability rules for Type II requirements.

China Telecom: As for the definition of feature, both Intel’s and Huawei’s version are acceptable for us.

Same time, we think Intel’s version is clearer and is aligned with the capability definition in TS38.306.

HW: In this revision:

- Features for Type II codebook is described as Intel’s comments with slightly modified ‘Support of Type II single panel codebook…’ as ‘Support of Type II codebook…’

- Remove the intention of any change on Table 6.1.1.4-1: Requirements applicability for mandatory features with UE capability signalling

We will come back in the next meeting with the revision of the applicability for R15/R16(with larger Tx ports) Type I single panel.

Recommendation:		Agreeable

R4-2103843	Introduction of PMI test cases with Rel-15 Type II codebook

Type: CR		For: Agreement
					38.101-4 v16.3.0	  CR-0136  Cat: B (Rel-16)

					Source: Samsung

Abstract:

Introduction of Type II codebook PMI test cases

Discussion:

[report of discussion]

Moderator’s note: With the intention to tighten the gamma requirements whenever possible, discuss and decide the tentative gamma requirements in the sub-thread for the CR revision.

Samsung: During the 1st round discussion, 5 companies provided results. Based on the simulation results summary, the span of the SNR point is quite large for some cases.

Companies are encouraged to further check their results and updated if any.

If no more results updated before deadline (Wed, 11pm UTC, Feb. 3), considering the targeting of this meeting is to finalize the requirement of Rel-16 NR performance enhancement WI,  my suggestion is to introduce the requirement with SNR in [] in this meeting, and further update the results and the values in future RAN4 meeting.

The following are simulation summary based on the agreed channel MIMO correlation (XP medium) and test point (90% TP)

| Cases | TP ratio (Following PMI/Rand PMI) | Samsung | Qualcomm | Huawei | Ericsson | Apple |
| --- | --- | --- | --- | --- | --- | --- |
| FDD 16x2 | Type II/ Random Type I at 90% point | 3.1 | 3.62 | 3.8 | 1.9 | 2.92 |
|  | Type I/ Random Type I at 90% point | 2.1 | 2.27 | 2.1 | 1.5 | 2.06 |
| FDD 16x4 | Type II/ Random Type I at 90% point | 2.92 | 2.77 | 3.2 | 2.1 | 2.83 |
|  | Type I/ Random Type I at 90% point | 2.3 | 2.14 | 2.12 | 1.7 | 2.26 |
| TDD 16x2 | Type II/ Random Type I at 90% point | 2.12 | 3.35 | 3.9 | 2.7 | 2.82 |
|  | Type I/ Random Type I at 90% point | 1.83 | 2.17 | 2.2 | 2.1 | 2.02 |
| TDD 16x4 | Type II/ Random Type I at 90% point | 1.9 | 2.63 | 3.1 | 2.5 | 2.81 |
|  | Type I/ Random Type I at 90% point | 1.75 | 2.08 | 2.3 | 2.3 | 2.3 |

Based on the summary, we suggest the tentative gamma value as

·         FDD

-         2Rx: [1.8]

-         4Rx: [2.0]

·         TDD

-         2Rx: [2.0]

-         4Rx: [1.8]

We would like to check whether the tentative gamma value is acceptable for companies?

Apple: We are fine with the proposed gamma values.

HW: Based on the simulation results summary, we have the following observations:

- For FDD, one companies’ results of Type II codebook have higher span compared with others

- For TDD, one companies’ results of Type II codebook have higher span compared with others

Because of these four results, the tentative gamma value have to be a little bit lower, which makes each of tentative gamma value reachable by every last companies’ results of Type I.

e.g. Four companies’ results of Type I are higher than tentative 2Rx [1.8].

| Cases | TP ratio (Following PMI/Rand PMI) | Samsung | Qualcomm | Huawei | Ericsson | Apple |
| --- | --- | --- | --- | --- | --- | --- |
| FDD 16x2 | Type II/ Random Type I at 90% point | 3.1 | 3.62 | 3.8 | 1.9 | 2.92 |
|  | Type I/ Random Type I at 90% point | 2.1 | 2.27 | 2.1 | 1.5 | 2.06 |
| FDD 16x4 | Type II/ Random Type I at 90% point | 2.92 | 2.77 | 3.2 | 2.1 | 2.83 |
|  | Type I/ Random Type I at 90% point | 2.3 | 2.14 | 2.12 | 1.7 | 2.26 |
| TDD 16x2 | Type II/ Random Type I at 90% point | 2.12 | 3.35 | 3.9 | 2.7 | 2.82 |
|  | Type I/ Random Type I at 90% point | 1.83 | 2.17 | 2.2 | 2.1 | 2.02 |
| TDD 16x4 | Type II/ Random Type I at 90% point | 1.9 | 2.63 | 3.1 | 2.5 | 2.81 |
|  | Type I/ Random Type I at 90% point | 1.75 | 2.08 | 2.3 | 2.3 | 2.3 |

So in this way, we can’t ensure what we agreed on ‘enough performance difference’ in the following agreement:

Therefore, we have the following suggestions and proposal:

- Introduce the requirement with [SNR] at least a little bit larger than Type I

- Encourage companies to further check and update their results

- Proposed gamma value:

  - FDD

    - 2Rx: [2.5]

    - 4Rx: [2.4]

  - TDD

    - 2Rx: [2.3]

    - 4Rx: [2.4]

CTC: As for the gamma values for Type II, we generally share same view with Huawei. The gamma value for Type II needs to be tighter, so as to ensure performance difference between Type I and Type II.

As for the specific gamma value, for simplification, we propose to set same value for each duplex mode and antenna combination, i.e.,

·         FDD

-         2Rx: [2.4]

-         4Rx: [2.4]

·         TDD

-         2Rx: [2.4]

-         4Rx: [2.4]

E///: We think it is also important to consider the requirements for eType-II PMI reporting. So far we are going to set 2.2 for eType-II PMI reporting requirements regardless of FDD/TDD and 2Rx/4Rx.

According to the simulation summary, we observe the gamma of eType-II is better than Type-II. So gamma for Type-II should be less than 2.2. Otherwise we need to set larger value for eType-II.

|  |  | FDD 2Rx | FDD 4Rx | TDD 2Rx | TDD 4Rx |
| --- | --- | --- | --- | --- | --- |
| eType-II PMI (eMIMO) | Reference | [2.2] | [2.2] | [2.2] | [2.2] |
| Type-II PMI (Perf Enh) | Option 1 | 1.8 | 2.0 | 2.0 | 1.8 |
| Type-II PMI (Perf Enh) | Option 2 | 2.4 | 2.4 | 2.4 | 2.4 |

If we want to set the same gamma regardless of FDD/TDD and 2Rx/4Rx, we propose to set 2.0.

Samsung: In general, it is difficult to align the simulation results for PMI requirement due to different UE implementation. Therefore, the TP ratio of PMI requirement is set based on the minimum value of companies results to fulfill the UE can pass the PMI test, same approach has already applied in the previous PMI test.

During PMI test, the phase error is existed. Therefore, some margin should be reserved based on ideal simulation results,

The following are ideal simulation results summary with including both type II and eType II.

| Cases | TP ratio (Following PMI/Rand PMI) | Samsung | Qualcomm | Huawei | Ericsson | Apple |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| FDD 16x2 | eType II/ Random Type I at 90% point | 6.5 | 6.16 | 7.4 | 2.3 | 3.41 |  |
|  | Type II/ Random Type I at 90% point | 3.1 | 3.62 | 3.8 | 1.9 | 2.92 |  |
|  | Type I/ Random Type I at 90% point | 2.1 | 2.19 | 2.1 | 1.5 | 2.01 |  |
| FDD 16x4 | eType II/ Random Type I at 90% point | 6.05 | 3.69 | 7.1 | 2.3 | 2.85 |  |
|  | Type II/ Random Type I at 90% point | 2.92 | 2.77 | 3.2 | 2.1 | 2.83 |  |
|  | Type I/ Random Type I at 90% point | 2.01 | 2.12 | 2.12 | 1.7 | 2.11 |  |
| TDD 16x2 | eType II/ Random Type I at 90% point | 4.05 | 4.18 | 5.82 | 3 | 3.49 |  |
|  | Type II/ Random Type I at 90% point | 2.12 | 3.35 | 3.9 | 2.7 | 2.82 |  |
|  | Type I/ Random Type I at 90% point | 1.83 | 2.05 | 2.2 | 2.2 | 2.03 |  |
| TDD 16x4 | eType II/ Random Type I at 90% point | 4.15 | 3.1 | 5.73 | 2.4 | 3 |  |
|  | Type II/ Random Type I at 90% point | 1.9 | 2.63 | 3.1 | 2.5 | 2.81 |  |
|  | Type I/ Random Type I at 90% point | 1.76 | 2.01 | 2.3 | 2.2 | 2.31 |  |

Based on the simulation results submitted by companies, all companies results can show large gain can be achieved between eType II and type II.

Also, from the absolute throughput perspective, we can also see the performance gain compared with type II for eType II codebook based on our contribution

Therefore, it is not realistic to set the gamma value for type II larger than eType II.

Based on the GTW agreement, [2.2] is set as the gamma value for eType II, where the minimum of TP ratio (2.3) is considered to set the requirement with considering some margin as 0.1

Based on above analysis, we can not accept the proposed gamma value by companies.

Therefore, we prefer to apply the same approach to set the gamma value for type II. Among all the companies, 1.9 is minimum value for the TP ratio of type II. For simplification, we prefer to set the same value for each duplex mode and antenna combination with considering margin as

         FDD

-         2Rx: [1.8]

-         4Rx: [1.8]

         TDD

-         2Rx: [1.8]

-         4Rx: [1.8]

QC: Thank you for providing the CR. While we are still discussing the requirements, we have a minor comment. TDoc number should be R4-2103843 instead of R4-2103834 based on Chairman notes.

Recommendation:		Agreeable

Summary on 2nd round

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| XXX | Based on 2nd round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |
| R4-2103847 Summary of simulation results of Rel-15 Type-II UE PMI test | Noted |
| R4-2103846, Simulation assumptions for PMI | agreeable |
| R4-2103842, HW, CR for Applicability | agreeable |
| R4-2103843, Samsung, Introduction of PMI test cases with Rel-15 Type II codebook | agreeable |

Topic #4: UE	power imbalance requirements

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2101366 | Huawei, HiSilicon | Proposal 1: Add the following general rules in the specification and update the test applicability as shown in Table 1: - For Ues supporting FR1 intra-band contiguous and non-contiguous EN-DC and inter-band EN-DC, where the frequency range of the LTE band is a subset of the frequency range of the NR band, the requirements applicability is specified in Table 9.1.1-3 - For UE only supporting contiguous EN-DC, only performance requirements for contiguous EN-DC in clause 9.5B.1.1 are applicable - For UE only supporting non-contiguous EN-DC, only performance requirements for non-contiguous EN-DC in 9.5B.1.2 are applicable - For UE supporting both contiguous and non-contiguous EN-DC, only performance requirements for contiguous EN-DC in 9.5B.1.1 are applicable |
| R4-2101367 | Huawei, HiSilicon | CR: Updates to power imbalance for CA |

Open issues summary

Sub-topic 4-1: Test applicability rule for UE power imbalance for EN-DC

Issue 4-1: Test applicability rule for UE power imbalance for EN-DC

- Proposals

  - Proposal 1: Add the following general rules in the specification and update the test applicability as shown in Table 1:

    - For Ues supporting FR1 intra-band contiguous and non-contiguous EN-DC and inter-band EN-DC, where the frequency range of the LTE band is a subset of the frequency range of the NR band, the requirements applicability is specified in Table 9.1.1-3

      - For UE only supporting contiguous EN-DC, only performance requirements for contiguous EN-DC in clause 9.5B.1.1 are applicable

      - For UE only supporting non-contiguous EN-DC, only performance requirements for non-contiguous EN-DC in 9.5B.1.2 are applicable

      - For UE supporting both contiguous and non-contiguous EN-DC, only performance requirements for contiguous EN-DC in 9.5B.1.1 are applicable

- Recommended WF

  - Encourage feedback on proposal 1.

Companies views’ collection for 1st round

Open issues

| Company | Comments |
| --- | --- |
| Company A | Issue 4-1: Test applicability rule for UE power imbalance for EN-DC |
| Company B | Issue 4-1: Test applicability rule for UE power imbalance for EN-DC |
| CMCC | Issue 4-1: Test applicability rule for UE power imbalance for EN-DC In our views, there is no need to add the general rules in the specification since the rules has been included in the Table 9.1.1-3 implicitly. The update of test applicability is as below for reference: Inter-band scenarios are not upports; UE indicates “interBandContiguousMRDC” i.e. support intra-band contiguous EN-DC requirements for supported inter-band EN-DC combinations (Note 1); UE does not indicate “interBandContiguousMRDC” i.e. support intra-band non-contiguous EN-DC requirements for supported inter-band EN-DC combinations (Note 1). Intra-band scenarios are not supported; N/A; Clause 9.5B.1.1 is executed for inter-band EN-DC scenarios; Clause 9.5B.1.2 is executed for inter-band EN-DC scenarios. UE does not indicate “intraBandENDC-Support” or UE indicates “both” in “intraBandENDC-Support”, i.e. supports intra-band contiguous or both intra-band contiguous and non-contiguous EN-DC for supported intra-band EN-DC combinations; Clause 9.5B.1.1 is only executed for intra-band EN-DC scenarios; Clause 9.5B.1.1 is executed for both intra-band and inter-band EN-DC scenarios; Clause 9.5B.1.1 is only executed for intra-band EN-DC scenarios. UE indicates “non-contiguous” in “intraBandENDC-Support”, i.e. supports only intra-band non-contiguous EN-DC for supported intra-band EN-DC combinations; Clause 9.5B.1.2 is only executed for intra-band EN-DC scenarios; Clause 9.5B.1.1 is executed for inter-band EN-DC scenarios; Clause 9.5B.1.2 is executed for both intra-band and inter-band EN-DC scenarios. We are Ok to merge the third row and the fourth row of the original table, but we think it is not necessary to add the “i.e.” to the UE indication. We prefer to modify the table like below and also open to further discuss: Inter-band scenarios are not upports; UE indicates “interBandContiguousMRDC” (Note 1); UE does not indicate “interBandContiguousMRDC”(Note 1). Intra-band scenarios are not supported; N/A; Clause 9.5B.1.1 is executed for inter-band EN-DC scenarios; Clause 9.5B.1.2 is executed for inter-band EN-DC scenarios. UE does not indicate “intraBandENDC-Support” or UE indicates “both” in “intraBandENDC-Support”; Clause 9.5B.1.1 is only executed for intra-band EN-DC scenarios; Clause 9.5B.1.1 is executed for both intra-band and inter-band EN-DC scenarios; Clause 9.5B.1.1 is only executed for intra-band EN-DC scenarios. UE indicates “non-contiguous” in “intraBandENDC-Support”; Clause 9.5B.1.2 is only executed for intra-band EN-DC scenarios; Clause 9.5B.1.1 is executed for inter-band EN-DC scenarios; Clause 9.5B.1.2 is executed for both intra-band and inter-band EN-DC scenarios. |
| Intel | Option 1 from HW is fine for us. Same time, proposal from CMCC is also fine. |
| Qualcomm | The last two bullets in the proposal are already captured in the table 9.1.1-3. We are ok to capture the 1st bullet in the table as well. |
| Docomo | We slightly prefer CMCC’s proposal, but HW’s proposal is also fine. |
| Ericsson | We are fine with the proposal by Huawei. We think the descriptions after ‘i.e.’ is also good to understand the scenario. Maybe we put it in the notes in the footer as follows (based on CMCC version): Inter-band scenarios are not upports; UE indicates “interBandContiguousMRDC” (Note 1, Note 2); UE does not indicate “interBandContiguousMRDC” (Note 1, Note 3). Intra-band scenarios are not supported; N/A; Clause 9.5B.1.1 is executed for inter-band EN-DC scenarios; Clause 9.5B.1.2 is executed for inter-band EN-DC scenarios. UE does not indicate “intraBandENDC-Support” or UE indicates “both” in “intraBandENDC-Support” (Note 4); Clause 9.5B.1.1 is only executed for intra-band EN-DC scenarios; Clause 9.5B.1.1 is executed for both intra-band and inter-band EN-DC scenarios; Clause 9.5B.1.1 is only executed for intra-band EN-DC scenarios. UE indicates “non-contiguous” in “intraBandENDC-Support” (Note 5); Clause 9.5B.1.2 is only executed for intra-band EN-DC scenarios; Clause 9.5B.1.1 is executed for inter-band EN-DC scenarios; Clause 9.5B.1.2 is executed for both intra-band and inter-band EN-DC scenarios. Note 1: Requirements are applicable to intra-band scenarios and only inter-band scenarios from Table 5.5B.4.1-1 of TS 38.101-3 [8] for which Note 4 is applied. Note 2: UE supports intra-band contiguous EN-DC requirements for supported inter-band EN-DC combinations Note 3: UE supports intra-band non-contiguous EN-DC requirements for supported inter-band EN-DC combinations Note 4: UE supports intra-band contiguous EN-DC, or both intra-band contiguous and non-contiguous EN-DC for supported intra-band EN-DC combinations Note 5: UE supports only intra-band non-contiguous EN-DC for supported intra-band EN-DC combinations. |
| Huawei, HiSilicon | As CMCC indicated, all test applicability rules are implicitly included in the table, also it is the reason that we propose to add the explicitly test applicability rules to facilitate to understand the specification quickly and clearly, especially for readers not involved this discussion, it is very hard to figure out the background logic for that created table by checking all implicitly included possible scenarios. The proposal from Ericsson to add note for those “i.e” contents is fine for us. |
| docomo2 | I have a comment on the interpretation of "UE indicates “interBandContiguousMRDC". This IE can show the following patterns. (R2-2002350) ・UE indicates “interBandContiguousMRDC” → UE supports both intra-band contiguous and non-contiguous EN-DC ・UE does not indicate “interBandContiguousMRDC” → UE supports intra-band non-contiguous EN-DC Thus, Note 2 should be changed from "UE supports intra-band contiguous EN-DC" to "UE supports both intra-band contiguous and non-contiguous EN-DC". |

CRs/TPs comments collection

| CR/TP number | Comments collection |
| --- | --- |
| R4-2101367, HW, CR: Updates to UE power imbalance for CA | Intel: Corrections is this CR are fine for us. Same time, we think that coordination between changes in this CR and CR R4-2101434 should be further discussed in case changes from CR R4-2101434, which affect section numbering, will be approved. |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic 4-1: Test applicability rule for UE power imbalance for EN-DC | Issue 4-1: Test applicability rule for UE power imbalance for EN-DC GTW agreement: Inter-band scenarios are not supported; UE indicates “interBandContiguousMRDC” (Note 1, Note 2); UE does not indicate “interBandContiguousMRDC” (Note 1, Note 3). Intra-band scenarios are not supported; N/A; Clause 9.5B.1.1 is executed for inter-band EN-DC scenarios; Clause 9.5B.1.2 is executed for inter-band EN-DC scenarios. UE does not indicate “intraBandENDC-Support” or UE indicates “both” in “intraBandENDC-Support” (Note 4); Clause 9.5B.1.1 is only executed for intra-band EN-DC scenarios; Clause 9.5B.1.1 is executed for both intra-band and inter-band EN-DC scenarios; Clause 9.5B.1.1 is only executed for intra-band EN-DC scenarios. UE indicates “non-contiguous” in “intraBandENDC-Support” (Note 5); Clause 9.5B.1.2 is only executed for intra-band EN-DC scenarios; Clause 9.5B.1.1 is executed for inter-band EN-DC scenarios; Clause 9.5B.1.2 is executed for both intra-band and inter-band EN-DC scenarios. Note 1: Requirements are applicable to intra-band scenarios and only inter-band scenarios from Table 5.5B.4.1-1 of TS 38.101-3 [8] for which Note 4 is applied. Note 2: UE supports both intra-band contiguous and non-contiguous EN-DC requirements for supported inter-band EN-DC combinations Note 3: UE supports intra-band non-contiguous EN-DC requirements for supported inter-band EN-DC combinations Note 4: UE supports intra-band contiguous EN-DC, or both intra-band contiguous and non-contiguous EN-DC for supported intra-band EN-DC combinations Note 5: UE supports only intra-band non-contiguous EN-DC for supported intra-band EN-DC combinations. |

Suggestion on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |
| R4-2101367, HW, CR: Updates to UE power imbalance for CA | Return to Pending on the discussion on the section number for CA normal PDSCH, this CR can be agreeable or needs to be revised. |
| CR: Update on test applicability rule for EN-DC power imbalance | New CR to implement the GTW agreement (Source: Huawei, HiSilicon; 38.101-4 v16.3.0; Cat F; WI code: NR_perf_enh-Perf) (will request the CR number from MCC after getting the tdoc number from Session Chair) |

Discussion on 2nd round

R4-2101367	CR: Updates to power imbalance for CA

Type: CR		For: Agreement
					38.101-4 v16.3.0	  CR-0153  Cat: F (Rel-16)

					Source: Huawei, HiSilicon

Discussion:

[report of discussion]

Moderator’s note: Pending on the discussion on the section number for CA normal PDSCH, this CR can be agreeable or needs to be revised.

Recommendation:		Agreeable

R4-2103845	CR: Update on test applicability rule for EN-DC power imbalance

Type: CR		For: Agreement
					38.101-4 v16.3.0	  CR-?  Cat: F (Rel-16)

					Source: Huawei, HiSilicon

Abstract:

Session Chair: Contact with MCC to get CR number

Discussion:

Moderator’s note: Please CR proponent to ask for the CR number from MCC as suggested by Session Chair.

Intel: I’ve double check about interBandContiguousMRDC capability in the latest version of TS 38.306 and found the following description.

interBandContiguousMRDC Indicates for an inter-band (NG)EN-DC/NE-DC combination, where the frequency range of the E-UTRA band is a subset of the frequency range of the NR band (as specified in Table 5.5B.4.1-1 of TS 38.101-3 [4]), that the UE supports intra-band contiguous (NG)EN-DC/NE-DC requirements (see TS 38.101-3 [4]). If the field is absent for such an inter-band (NG)EN-DC/NE-DC combination, the UE supports intra-band non-contiguous (NG)EN-DC/NE-DC requirements.

Based on this version, Note 2 should be “UE supports both intra-band contiguous and non-contiguous EN-DC requirements for supported inter-band EN-DC combinations.”

Could you or someone else clarify where does another interpretation of this capability come from?

HW: During the discussion paper preparation before the meeting, I also checked TS 38.306 and have the same understanding as you. During the 1st round discussion, NTT DOCOMO raised this interpretation by referring to RAN2 contribution R2-2002350.

Intel: We’ve also checked the content of R2-2002350. This CR is from March 2020. Based on our review, content of this RAN2 CR is aligned with the latest version of 38.306. Therefore, probably we can check if DCM have another understanding of this feature.

HW: After further checked the core specification TS 38.101-3:

NOTE 4:    The minimum requirements for intra-band contiguous or non-contiguous EN-DC apply. The intra-band requirements also apply for these carriers when applicable EN-DC configuration is a subset of a higher order EN-DC configuration.

It clearly states that intra-band contiguous or non-contiguous EN-DC requirements applied, not both requirements. From RAN4 RF point of view, any EN-DC combinations are optional for UE to support.

I also checked with our RAN2 colleague, the final core specification should be referred to instead of the coversheet explanation, so based on the RAN2 specification TS 38.306, we have the similar understanding with Intel:

interBandContiguousMRDC Indicates for an inter-band (NG)EN-DC/NE-DC combination, where the frequency range of the E-UTRA band is a subset of the frequency range of the NR band (as specified in Table 5.5B.4.1-1 of TS 38.101-3 [4]), that the UE supports intra-band contiguous (NG)EN-DC/NE-DC requirements (see TS 38.101-3 [4]). If the field is absent for such an inter-band (NG)EN-DC/NE-DC combination, the UE supports intra-band non-contiguous (NG)EN-DC/NE-DC requirements.

DCM: We checked the related description in TS 38.306 and TS 38.101-3 and we agree that it is very unclear, so let me explain our understanding:

=================================

Our understanding is:

- UE supporting an inter-band EN-DC which follows intra-band EN-DC requirements such as DC_42_n77 shall supports intra-band non-contiguous EN-DC requirements without signaling interBandContiguousMRDC.

- If UE support intra-band contiguous EN-DC requirements, UE shall report the interBandContiguousMRDC

Reasons:

- The approved documents clearly states it.

  - In chairman report of RAN4#92(R4-1909063)

    - Agreement: When a UE supports DC_42_n78 and/or DC_42_n77, the band combinations shall support intra band non-contiguous EN-DC without signalling intra band EN-DC support capability.

  - In approved LS: R4-1913130

    - In addition, for the above configurations, RAN4 agreed that intra-band non-contiguous EN-DC requirements for these inter-band EN-DC configurations applies as mandatory but it is optional to apply intra-band contiguous EN-DC requirements for these inter-band EN-DC configurations.

  - In cover sheet of agreed CR: R2-2002350

    - In this case, RAN4 also agreed that non-contigous CC allocation is mandatory, whereas contiguous CC allocation is optional.

- We need to allow UE supporting both intra-band contiguous and non-contiguous EN-DC requirements to signal its capability to NW.

  - If we follows the rules of our understanding, UE can just signal interBandContiguousMRDC to indicate the supportiveness of both contiguous and non-contiguous.

    - Note that this is a similar manner to that of intraBandENDC-Support

  - However, if we follows the rules where NW assumes that UE signalling interBandContiguousMRDC supports only contiguous requirements, how can UE supporting both contiguous and non-contiguous signal its capability? Actually, this is a concern from our side.

==================================

We would to note that we originally had proposed that DC_42_n77 and DC_42_n78 shall support both contiguous and non-contiguous, but there were concerns on mandatory support of contiguous case. This is a background why interBandContiguousMRDC was introduced.

Now, we also agree that the current description is unclear. So we guess it would be better to modify the description in Table 5.5B.4.1-1 in TS 38.101-3, which is referred by TS 38.306.

We would appreciate if you could take care about our understanding and concerns.

HW: Thanks for your detailed explanation on the background information for interBandContiguousMRDC.

Currently based on the understanding about the core specifications of both TS 38.306 and TS 38.101-3, the latest wording in CR R4-2103845 is correct. As you provided, if some alignments ever happened for capturing the RAN4 RF agreements both in RAN4 RF and RAN2 specifications, but follow the 3GPP work procedure, the performance requirements can only be defined based on the core specifications. From our point of view, the best way is to change the core specification firstly before we change the performance requirements if the provided information from you is the common understanding for all companies.

For the UE capability of intraBandENDC-Support in TS 38.306:

intraBandENDC-Support Indicates whether the UE supports intra-band (NG)EN-DC with only non-contiguous spectrum, or with both contiguous and non-contiguous spectrum for the (NG)EN-DC combination as specified in TS 38.101-3 [4]. If the UE does not include this field for an intra-band (NG)EN-DC combination the UE only supports the contiguous spectrum for the intra-band (NG)EN-DC combination.

For the definition in TS 38.331:

intraBandENDC-Support               ENUMERATED {non-contiguous, both}   OPTIONAL,

It is very clearly to distinguish three kinds of UE capability.

DCM: Thanks for your explanation.

We, of course, understand that the latest wording in CR R4-2103845 about NOTE 2 is correct.

However, the related description in TS 38.306 and TS 38.101-3 is very unclear, so we can also understand that UE supports both intra-band contiguous and non-contiguous EN-DC requirements.

In this sense, we prefer to add the description of NOTE 2 after discussing the interpretation of “interBandContiguousMRDC” at RF discussion.

Therefore, we guess it would be better to delete the NOTE 2 from CR R4-2103845 at present.

In addition, another reason for our proposal is that we believe TS 38.101-4 can function without adding a NOTE since the purpose of this CR modification is to clarify the test applicability rule.

We would appreciate it if you would consider our proposal.

CTC: With the explanation from DCM, would it be feasible to delete the Note 2 for now?

Meanwhile, if needed, in the summary document we can suggest to add the following note in the Chairman Notes:

One additional note to Table 9.1.1-3 might be added in the future based on further discussion on the interpretation of “interBandContiguousMRDC” in RF session.

Intel: Form our point of view, it will be rather strange if we define clarification notes for all cases except one.

We have two suggestions:

- Option 1: Remove all Notes and come back once we will have stable situation for “interBandContiguousMRDC” capability.

- Option 2: Keep it as it is, because these Notes are correct in case we rely on the latest version of TS 38.306. Same time, we can add note in the Chairman Notes: “Note 2 in Table 9.1.1-3 might be updated in the future based on further discussion on the interpretation of “interBandContiguousMRDC” in RF session.”

Both options are fine for us. Option 2 is slightly preferred.

CTC: How about the option 3 below? I just checked the latest 38.101-4, there are a few TBDs here, so I guess it is feasible.

- Option 3: leave Note 2 as TBD. Same time, we can add note in the Chairman Notes: “Note 2 in Table 9.1.1-3 will be updated in the future based on further discussion on the interpretation of “interBandContiguousMRDC” in RF session.”

Intel: Option 3 is also fine for us.

HW: Thanks for the discussion, from my personal point of view, I prefer Option 2. Because currently it is the correct core specification interpretation before any further discussion in RF session. But considering the concerns from DCM, one chairman note as suggested by Intel can address DCM’s concern.

For the sake of progress, we can accept Option 3.

DCM: Thanks for the suggestion.

The updated CR (Option 3) are fine for us.

Thank you very much for your compromise.

Recommendation:		Agreeable

Summary on 2nd round

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| XXX | Based on 2nd round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |
| R4-2101367, CR: Updates to UE power imbalance for CA | agreeable |
| R4-2103845, CR: Update on test applicability rule for EN-DC power imbalance | agreeable In addition, add the following note in the Chairman Notes: Note 2 in Table 9.1.1-3 will be updated in the future based on further discussion on the interpretation of “interBandContiguousMRDC” in RF session.” |

Topic #5: NR CA CQI reporting requirements

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2100886 | China Telecom | CR: Adding applicability and requirements for FR1 and FR2 CA CQI reporting test |

Open issues summary

No open issue.

Companies views’ collection for 1st round

Open issues

CRs/TPs comments collection

| CR/TP number | Comments collection |
| --- | --- |
| R4-2100886, CTC, CR on applicaability and requiremets | Intel: Section numbering procedure probably should be updated based on discussion of Issue 2-1 and CR R4-2101434 |
|  | Qualcomm: Measurement channels should point to CSI RMCs. Section numbering may have to be modified based on the discussion on Spec structure for CA. |
|  | China Telecom 2: For the section numbering, pending agreements in Issue 2-1. @Qualcomm: For the measurement channel configuration, our purpose is to avoid introducing too many new FRC tables in clauseA.4 for each possible bandwidth and CQI index combination, and the same way is utilized in the reference channel configuration for CA power imbalance, e.g., Table 5.2A.2.2-3 in 38.101-4. |

Summary for 1st round

Open issues

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |
| R4-2100886, CTC, CR on applicaability and requiremets | Return to Pending on the discussion on the section number for CA normal PDSCH, this CR can be agreeable or needs to be revised. |

Discussion on 2nd round

R4-2100886	CR: Adding applicability and requirements for FR1 and FR2 CA CQI reporting test

Type: CR		For: Agreement
					38.101-4 v16.3.0	  CR-0134  Cat: B (Rel-16)

					Source: China Telecom

Discussion:

[report of discussion]

Moderator’s note: Pending on the discussion on the section number for CA normal PDSCH, this CR can be agreeable or needs to be revised.

Recommendation:		Agreeable

Summary on 2nd round

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| XXX | Based on 2nd round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |
| R4-2100886, CTC, CR on CA CQI applicaability and requiremets | agreeable |