3GPP TSG-RAN WG4 Meeting #98e                                              R4-2103977

Electronic Meeting, 25th Jan– 5th Feb, 2021

Agenda Item:   7.4.3.3, 7.4.3.4

Source:        Moderator(Samsung)

Title:           WF on Testing aspect on IAB-MT dynamic range, power control and frequency error

Document for:  Agreement

Introduction

According to 1st round discussion on [98e][307] NR_IAB_Conformance_Part2, the candidate agreement captured in moderator summary R4-2103746.

This way forward is to confirm the tentative agreement and possible more agreement during 2nd round.

Discussion

2.1 Dynamic range for IAB-MT

Regarding IAB-MT dynamic range test points on 26th Jan GTW session two options provided online as below:

Option 1(Samsung, Nokia, CATT, ZTE): - Test points 1: Maximum output power with full RB allocation and maximum output power - Test points 2: single RB allocation with 5/10 dB lower PSD as used in test point 1) - Test point 1- test point 2 = X+Y （+/- uncertainty FFS ） Option 2 (E///, ZTE): - Test points 1: Maximum output power with full RB allocation and maximum output power - Test points 3: Minimum output power (as set by 5/10 dB dynamic range requirement) with full RB allocation. - Test point 4: Sing PRB transmission with same PSD as test point 1 - Test point 1- test point3 = X (+/- uncertainty FFS) - Test point 1 – test point4 =Y (+/- uncertainty FFS) FFS for test applicability among with other conformance requirements

Conclusion during 2nd round: companies agreed to compromise as option 1 and the concern on power accuracy can be FFS.

Agreement: Test point on power control requirement for IAB-MT is agreed as:

    - Test points 1: Maximum output power with full RB allocation and maximum output power

    - Test points 2: single RB allocation with 5/10 dB lower PSD as used in test point 1)

    - Test point 1- test point 2 =  X+Y （+/- uncertainty FFS ）

2.2 WF on local area IAB-MT power control

Agreement for testing point for power accuracy on 29TH GTW meeting:

For relative power control accuracy Agreements:

Option 3: Partial PRB allocation to be considered in Test model design if to reuse the similar test configuration as UE.”

For aggregated power control accuracy agreements:

NO detailed conformance test cases for this requirement, FFS whether can be jointly verified or covered by dynamic range conformance test cases.

Further Way forward

According to feedback from companies during 2nd round the preference is both power control accuracy requirements can be verified together with dynamic range. However there is no extensive discussion on that. Hence it is suggested

- Investigate further whether based current test points agreed for dynamic range whether the test purpose for both relative power control accuracy and aggregated power control accuracy can be covered.

- Proposal on test point/configuration for relative power accuracy is not precluded.

WF on two-way signal: below agreement applied for power control requirement.

Issue 1-1-2: Two-way communication in IAB-MT tests in [306] Two-way communication is not specified for RF conformance tests, specification shall not preclude DL signals to be used e.g. for timing and frequency reference purposes during the test. Companies further work on the clarification notes to conformance specifications for topic 1-1.

2.3 WF on frequency error

During Jan 29th GTW session it is agreed that

Topics from email thread [306] Issue 1-1-1: Synchronization Agreement: Using same BS approach (no detailed synchronization configuration in conformance specifications; meanwhile add a note in conformance specs to clarify (IAB-MT sync with IAB-DU with DL signal configuration not precluded). Issue 1-1-3: Description of connection/measurement setup in specification annex Agreements: Option 1: Flexibility in connection / measurement setup is allowed by keeping the specified setup informative

Agreements: Above agreement from [306] applied for frequency error test as well.

With this agreement frequency error can be verified with flexibility. RAN4 will work on specification draft detail to ensure both verification options on frequency error can be enabled.

- Option 1: Verification can be done together with EVM and allow the alternative to apply BS test equipment

- Option 2: Verify by UE approach including test set-up and test environment.