3GPP TSG-RAN WG4 Meeting #98-e	R4-2103982

# Electronic Meeting,  Jan. 25th  – Feb. 5th, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-4 | CR | <CR#> | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR: Introduce PSCCH/PSSCH decoding capability requirement for NR V2X. |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL-Perf |  |  |  |  |  | Date: |  |  | 2021-1-4 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | According to the work plan of V2X demodulation, companies should submit draft CRs in RAN 4 98-e meeting and RAN 4 has agree to introduce PSCCH/PSSCH decoding capability requirement |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the requirments for PSCCH/PSSCH decoding capability. Add the RMC for PSCCH/PSSCH decoding capability test. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The performance requirements will still be incompleted. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.1.8, A.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of change>

11.1.8	PSSCH/PSCCH decoding capability test

11.1.8.1   2RX requirements

11.1.8.1.1   Minimum requirements

The purpose of this test is to verify the maximum number of Sidelink processes per TTI supported by the V2X UE.

The minimum requirements are specified in Table 11.1.6.1.1-2 with the test parameters specified in Table 11.1.6.1.1-1

Table 11.1.8.1.1-1:  Test Parameters_

| Parameter |  | Unit | value |
| --- | --- | --- | --- |
| Channel bandwidth |  | MHz | TBD |
| Sidelink UE i, 0 ≤ i ≤ TBD | Sidelink Transmissions |  | PSCCH + PSSCH |
|  | Timing offset (Note 1) | s | 0 |
|  | Frequency offset (Note 2) | Hz | 0 |
|  | Synchronization source |  | GNSS |
|  | Propagation Channel |  | Static propagation condition No external noise sources are applied |
|  | Antenna configuration |  | 1x2 |
|  | PSSCH RMC |  | R.PSSCH.2-x-y |
|  | PSCCH RMC (Note 3) |  | R.PSCCH.2-x-y |
|  | PSFCH periodicity | Slots | 1 |
|  | MintimeGapPSFCH | Slots | 3 |
| Note 1: Time offset of Sidelink UE received signal with respect to GNSS reference timing. Note 2: Frequency offset of Sidelink UE with respect to GNSS reference frequency. Note 3: OCC index (in TS 38.211) for PSCCH DMRS is randomly selected between {0, 1, 2} for each PSCCH transmission. |  |  |  |

Table 11.1.8.1.1-2: Minimum performance

| Bandwidth | PSCCH Reference channel | Reference value |
| --- | --- | --- |
|  |  | Probability of missed PSCCH (%) |
| [20] MHz | R.PSCCH.x | 1 |

<End of change>