3GPP TSG-RAN4 Meeting #98-e			R4-2100020

Electronic Meeting, 25 January – 5 February, 2021

3GPP TSG-RAN5 Meeting #89-e			R5-206676

Electronic Meeting, 9 – 20 November, 2020

Title:	LS on ambiguity in deciding TL,C

Response to:

Release:	Release 15

Work Item:	5GS_NR_LTE-UEConTest

Source:	TSG RAN WG5

To:	TSG RAN WG4

Cc:

Contact Person:

#### Name:	Gu, Chunying

Tel. Number:

E-mail Address:	guchunying@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1. Overall Description:

In TS 38.101-1, the tolerance for configured output power is specified in section 6.2.4 as below:

The measured configured maximum output power PUMAX,f,c shall be within the following bounds:

PCMAX_L,f,c  –  MAX{TL,c, T(PCMAX_L,f,c)}  ≤  PUMAX,f,c  ≤  PCMAX_H,f,c  +  T(PCMAX_H,f,c).

where the tolerance T(PCMAX,f,c) for applicable values of PCMAX,f,c is specified in Table 6.2.4-1. The tolerance TL,c is the absolute value of the lower tolerance for the applicable operating band as specified in Table 6.2.1-1.

The tolerance TL,C refers back directly to table 6.2.1-1, where exists a NOTE 3 further reducing the lower tolerance limit by 1.5dB if RB allocation is at the band edge.

NOTE 3:	Refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB.

However, when calculating PCMAX_L,f,c, the 1.5dB relaxation is already considered in the format of ∆TC,c

PCMAX_L,f,c = MIN {PEMAX,c– ∆TC,c,  (PPowerClass – ΔPPowerClass) – MAX(MAX(MPRc+∆MPRc, A-MPRc)+ ΔTIB,c + ∆TC,c + ∆TRxSRS, P-MPRc) }

where

…

∆TC,c = 1.5dB when NOTE 3 in Table 6.2.1-1 in 38.101-1 applies for a serving cell c, otherwise ∆TC,c = 0 dB ;

Regarding above core requirements, RAN5 has two different understandings:

- The source of ∆TC,c  is the same as NOTE 3 in table 6.2.1-1, therefore the 1.5dB relaxation shouldn’t be considered again when deciding TL,C.

- Strictly following above core requirements, the 1.5dB relaxation should be considered twice when deciding ∆TC,c  and TL,C.

The numeric example of understanding 1 and 2 can been seen below. With understanding 2 the lower limit of Pumax is further relaxed by 1.5dB.

| Understanding | PPowerClass (dBm) | MPR (dB) | ΔTC,c (dB) | PCMAX_L,f,c (dBm) | T(PCMAX_L,f,c) (dB) | TL,c (dB) | Lower limit (dBm) PCMAX_L,f,c – MAX(T(PCMAX_L,f,c), TL,c) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0 | 1.5 | 21.5 | 2.0 | 2 | 19.5 |
| 2 | 23 | 0 | 1.5 | 21.5 | 2.0 | 3.5 | 18 |

Please note both understanding have been adopted in TS 38.521-1 for different bands respectively, therefore it’s important that RAN4 provides clear guidance on which understanding is correct.

2. Actions:

To RAN4 group.

ACTION: 	RAN5 kindly asks RAN4 group to confirm which one of the above understandings is correct.

3. Date of Next TSG-RAN WG5 Meetings:

TSG-RAN5 Meeting#90 	 22nd Feb – 5th Mar 2021	Electronic Meeting

TSG-RAN5 Meeting#91 	 17th – 28th May 2021	Electronic Meeting