R4-2107960

3GPP TSG-RAN WG4 Meeting # 99-e

Electronic Meeting, 19th – 27th May 2021

Agenda item:			10.2

Source:	Moderator (Ericsson)

Title:	Email discussion summary for [99-e][150] FS_NR_eff_BW_util

Document for:	Information

# Introduction

This email discussion is for FS_NR_eff_BW_util study item.  The main objective of the study is on efficient utilization of licensed spectrum that is not aligned with existing NR channel bandwidth.  The following is the agreed agenda:

- Study on Efficient utilization of licensed spectrum that is not aligned with existing NR channel bandwidths

  - General and work plan

  - Evaluation of use of larger channel bandwidths than operator licensed bandwidth

  - Evaluation of use of overlapping UE channel bandwidths

  - Others

The following topics are discussed in this email thread:

Topic #1: General TR Update

Topic #2: Evaluation of Use of Larger Channel Bandwidth

Topic #3: Evaluation of Use of Overlapping UE Channel Bandwidths

# Topic #1: General TR Update

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2110487 | Ericsson | Updated draft TR 38.844 after RAN4#98bis-e |

## Companies views’ collection for 1st round

    - TPs comments collection

For close-to-finalize WIs and maintenance work, comments collections can be arranged for TPs and CRs. For ongoing WIs, suggest to focus on open issues discussion on 1st round

| CR/TP number | Comments collection |
| --- | --- |
| R4-2110487 | Company A |
|  | Company B |

  - Summary for 1st round

    - Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2110487 | No comments during first round. Therefore Agreeable. |

# Topic #2: Evaluation of use of larger channel bandwidths

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2109427 | ZTE Wistron Telecom AB | Observation 1: UL transmission within the irregular bandwidth will not cause any co-existence issue. Observation 2: In DL reception, the number of significance bits of ADC output will be reduced due to the interference of blanked PRBs from the other operator. Proposal 1: RAN4 needs to specify the number of blanked PRBs for specifying RF requirements corresponding to the irregular bandwidth. Proposal 2: RAN4 to define new REFSENS associated with a certain number of blanked PRBs for the use of WiderCBW. |
| R4-2110661 | Huawei, HiSilicon | Observation 1: Without introduction of new channel filters, WiderCHBW might not be applicable for non-collocated scenarios. Observation 2: To meet the TX emission and RX ACS/blocking, new channel filters are needed for the gNB which is not prioritized as stated in the SID. Observation 3: If no dedicated channel filter is assumed, the requirements are not clear for the UE with capability to receiver the entire spectrum block. |
| R4-2111147 | Ericsson | Observation 1: For irregular bandwidths between 5 and 10MHz the overlapping UE channel bandwith solution is not preffered since the minimum bandwith of CORESET#0. Hence the method of using immediate wider channel bandwidht is suggested. Observation 2: A “legacy” UE not indicating ensured support for unwanted emission while blanking will still be able to attach to the NW but be configured with a smaller UE CHBW providing lower SU. Observation 3: Additional UE Emission requirements might need to be developed (in a possible W.I phase) Observation 4: The blanking method will need implementation changes for both the NW/base station as well as a UE. Observation 5: A reduced set of requirements for the irregular bandwidth, only regulatory emissions requirements in UL, are required for irregular bandwidths if next largest standardized bandwidth is supported. Observation 6: A “Fall back” mode to the next smaller regular BW can be used and handled by NW implementation (deployment or steering of BWP) for scenarios where near-far problem occurs. Proposal 1: Agree to further extend the wider CHBW (blanking) approach for irregular bandwidths also larger than 10 MHz. Proposal 2: Agree to define only smaller CHBW used by the UE in UL and further develop the usage of full irregular CHBW in UL in later releases. Proposal 3: Investigate specification (RAN1, RAN2, RAN4) impact of asymmetric configurated BWP within an operating band that is/is not asymmetric defined. Proposal 4: Agree to further develop the “immediate wider channel bandwidth” method (a.k.a blanking) for irregular BW’s between 5 and 10MHz. Since this method provides highest possible SU, less complaxity on BS (and possibly UE), shared implementation burden between NW and UE. Proposal 5: Adding UE capability/capabilities that indicate the UEs support for irregular bandwidths if to be supported in UL. |
| R4-2111148 | Ericsson | Draft TP to TR38.844 on wider channel BW method Moderator: Comments on specifics for the TP should be captured in Section 2.3.2 |
| R4-2109245 | Intel Inc | Proposal 2: Use the WiderCBW method for irregular CBW < 10MHz. |
| R4-2109435 | Apple | TP on using next larger channel bandwidth solution Moderator: Comments on specifics for the TP should be captured in Section 2.3.2 |

## Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

    - Sub-topic 2-1

General aspects such as applicibility of widerCHBW approach and DL/UL

Issue 2-1: Applicability of widerCBW approach

- Proposals

  - Option 1: Use the WiderCBW method for irregular CBW < 10MHz only (Intel, ZTE, T-Mobile USA)

  - Option 2: Apply the WiderCBW method as a general approach for all irregular CBWs (Ericsson, Qualcomm, Apple, Skyworks, CMCC, China Telecom)

    - Need further discussion (ZTE, Nokia, Huawei)

- Recommended WF

  - TBA

- Notes after GTW on May 20th

  - The proponents think the issue of out-band emission is more bad for narrow bandwidth compared to wider bandwidth, while other companies disagreed and would like to have more analysis.

  - More discussion is needed.

Issue 2-2: irregularBW in UL

- Proposals

  - Option 1: Adding UE capability/capabilities that indicate the UEs support for irregular bandwidths if to be supported in UL.

  - Option 2: Agree to define only smaller CHBW used by the UE in UL and further develop the usage of full irregular CHBW in UL in later releases.

- Recommended WF

  - WF (R4-2103263) Agreement on DL only shall be upheld.  Option 3 should be discussed for DL (widerCHBW) and UL (smallerCHBW) where the DL utilizes irregularBW.  To support UL for irregularBW in future releases is FFS.

- Agreement of GTW on May 20th

  - For all the potential approaches, to support UL for irregularBW from UE perspective needs further discussion in future release,

  - For widerCHBW approach, to support irregular CBW for DL only the following discussions are needed

    - Need further discussion on support of asymmetric bandwidths

    - Need further discussion on support of TX-RX separation

Issue 2-3:Asymmetric Configured BWP

- Proposals

  - Option 1: Investigate specification (RAN1, RAN2, RAN4) impact of asymmetric configurated BWP within an operating band that is/is not asymmetric defined.

  - Option 2: No impact to specification (RAN1, RAN2, RAN4) with respect to asymmetric configured BWP

- Recommended WF

  - TBA

    - Sub-topic 2-2

BS TX requirements.

Issue 2-3: BS TX Emissions

- Proposals

  - Option 1: new TX emissions mask needs to be defined associated with a certain number of blanked PRBs for the use of WiderCBW

  - Option 2: no new requirements are needed

- Recommended WF

  - gNB TX emissions requirements are needed for irregularBW definition due to regulatory requirements

    - Sub-topic 2-3

UE RX requirements.  .

Issue 2-4: UE RX ACS/Blocking

- Proposals

  - Option 1: “Fall back” mode can be applied therefore no specific UE ACS/blocking for irregularBW is needed

  - Option 2: new UE RX ACS/blocking requirement needs to be defined

  - Option 3: UE RX ACS/blocking is not an issue.

- Recommended WF

  - TBA

Issue 2-5: UE REFSENS

- Proposals

  - Option 1: new REFSENS needs to be defined associated with a certain number of blanked PRBs for the use of WiderCBW

  - Option 2: no new REFSENS requirements are needed

- Recommended WF

  - TBA

Issue 2-6: UE Filter

- Proposals

  - Option 1: no dedicated channel filter is assumed for irregularBW only widerCHBW is needed

  - Option 2: dedicated channel filter is required for irregularBW

- Recommended WF

  - TBA

## Companies views’ collection for 1st round

## Open issues

Sub topic 2-1

| Company | Comments |
| --- | --- |
| Intel | Issue 2.1 Applicability of WiderCBW Approach Option 1 – Use WiderCBW < 10MHz and Overlapping BW for > 10MHz. The WiderCBW approach is weaker with degraded ACS / reduced REFSENSE and should not be used above 10MHz as there are better solutions. Issue 2.2 IrregularBW in UL Agree with WF. UL FFS only. Issue 2.3 Asymmetric Configured BWP Not clear that asymmetric BWP needed |
| Ericsson | Issue 2-1: WiderCBW approach can be applicable for all irregularBWs Issue 2-2: Prioritize for DL as per WF. UL is feasible with WiderCBW approach and could be studied at a later stage. Issue 2-3: For DL as irregularBW and UL in smallerCHBW this is asymmetric and is supported by specifications. Band specific requirements. |
| ZTE | Issue 2.1 Applicability of WiderCBW Approach Option 1. We cannot conclude a generic principle applicable to all irregular bandwidths without sufficient study. Issue 2.2 IrregularBW in UL We are fine with the WF. For Option 1, a UE supports IrregularBW in UL via WiderCBW approach, and if the WiderCBW is mandatory for the UE, then there is no capability needed, and otherwise if WiderCBW is optional, then there is already a signaling indicating its support. So in any case, no new capability signaling is needed. Issue 2.3 Asymmetric Configured BWP Our understanding is Option 2. In current specs, DL BWP and UL BWP are configured separately, implying that asymmetric configured BWP is already supported. |
| Qualcomm | Issue 2-1: Option 2. this should be conditioned on using the approach in the first place which we do not actually think it is a good idea. our preference would be not to employ this method at all. Issue 2-2: WE agree with the recommended WF Issue 2-3: Option 1. even though this asymmetric configured channel BW is used in some bands, implications should be studied. Also, increase in testing burden should be considered. |
| Nokia | In general, there are still many open issues for this method which need to be evaluated first. It should be clarified how many PRBs need to be blanked for BS (to verify SU efficiency) to meet co-existence and emissions requirements assuming no new gNB channel filters are used. Furthermore, UE co-existence need to be addressed for blanked part (e.g. interfering signal rejection) of the spectrum which is used by other operator. Would it require new UE capability with flexible filter to address issues on Rx co-existence? Issue 2-2: Option 3 needs to be further clarified. What would be the Tx-Rx separation, would new UE capability be needed for DL due to variable duplex? Issue 2-3: Impact to be checked by RAN1/2. |
| After GTW on May 20 |  |
| T-Mobile USA | Issue 2-1: We have concerns about blocking from the adjacent operator if the base stations are not co-located. But since the majority of companies seem to be interested in applying the widerCBW approach to all irregular channel BWs, we can accept that. However, we also want to evaluate the overlapping approaches. Issue 2-3: Asymmetric channel BWs are already supported. |
| China Telecom | Issue 2-1: We support Option 2. Single-carrier approach is preferred. At least 11MHz CBW should be included. Issue 2-3: Option 2. Same as ZTE, our understanding is that the asymmetric configured BWP has already been supported. |
| CMCC | Issue 2-1: Option2 Issue 2-3: Option 3. We think asymmetric configured BWP is already supported. |
| Apple | Issue 2-1: Option 2 Issue 2-2: UL is feasible with using next larger channel bandwidth and can be studied later. Issue 2-3: Option 2, our understanding is that asymmetric BPW is already supported. |

Sub topic 2-2

| Company | Comments |
| --- | --- |
| Intel | Issue 2-3 BS Tx Emissions the existing Tx emissions masks are sufficient and must still be met. No need to define additional masks for every single irregularBW combination. |
| Huawei | Option2. Support of UL will need to define new filter and associated RF requirements. |
| Ericsson | Option 2 |
| ZTE | Issue 2-3 BS TX Emissions Option 2. Blanked PRBs are actually within the filter bandwidth, thus its “in-band emission” level should be less than the regulatory requirements for the “blanked frequency trunk” immediately outside the irregular bandwidth, so no new requirements are needed. |
| Qualcomm | Issue 2-3: Option 2. Our understanding is that if Option 1 is chosen, then this is equivalent to defining a new channel BW so the method would fall under a different category. |
| Nokia | Option 2 is not clear, does it mean to apply existing requirements? How about the transmission BW and the minimum guard band? |

Sub topic 2-3

| Company | Comments |
| --- | --- |
| Intel | Issue 2-4 UE RX ACS/Blocking ACS / blocking is certainly an issue. Issue 2-5 UE REFSENS Possibly need REFSENS spec to clearly indicate the potential reduction in sensitivity due to lack of filter coverage in BW with blanked PRBs Issue 2-6 UE Filter No dedicated channel filter should be required as this becomes an overly complicated requirement for hardware to support each newly introduced mode. A key premise of the WiderCHBW approach is to re-use existing hardware |
| Huawei | For the wider CBW approach, the baseline question should be clarified that whether dedicated channel filter is assumed for the irregularBW? What is the granularity |
| Ericsson | Issue 2-4: Fall back mode can be applied Issue 2-5: Support Option 2. For example if 5 MHz REFSENS is met and 10 MHz for the irregularBW of 7 MHz then this would provide sufficient coverage. Issue 2-6: Support Option 1. |
| ZTE | Issue 2-4 UE RX ACS/Blocking Option 2. Interference from the actual user of the blanked PRBs will impact UE Rx ACS/Blocking, and REFSENS. Issue 2-5 UE REFSENS Option 1. As seen in the above. Issue 2-6 UE Filter Option 1 as stated in the SID. |
| Qualcomm | Issue 2-4: Option 1. our understanding of this method is that there would be no impact to specs/design and a wider channel BW would be configured. Option 2 would actually mean that we are defining a new channel BW. Issue 2-5: Opiton 1. see also our previous commentary. there should be no spec impact form this scheme Issue 2-6: Option 1. if Option 2 is chosen, this means defining a new CHBW. |
| Nokia | Issue 2-4: Operator deployments are not necessarily coordinated with each other. Option 1 works only if the adjacent channel is used by the same operator. For option 2, co-existence solution needs to be addressed. Option 3: definitely this is an issue. Issue 2-5: Option 1 – it is not a REFSENS but blocking issue. Issue 2-6: None of two options work, option 1 ignores the co-existence issues (ACS/blocking), option 2 is against the purpose of using wider regular channel BW. |
| Apple | Issue 2-4: Option 1 can be always used by the network to change the UE operating channel to a smaller one. Option 2 is effectively introduction of a new channel bandwidth. Option 3 needs more analysis for particular cases and scenarios. Our general view is that some degradation might happen, but it depends a lot on the actual channel size, configured channel size is, location of a blocker, etc. Issue 2-5: Option 2 Issue 2-6: Option 1 |

## CRs/TPs comments collection

Major close to finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2111148 | Nokia: as commented above and in GTW, there are still many open issues for this method which need to be addressed by proper analysis. Quantitative statements about the performance gap in terms of ACS and in-band blocking and the number of PRBs that need to be left unused would be needed. “In UL the UE will be configured with the next smaller BW (5 MHz)” can the UL BW be independently chosen and is the 5MHz position individually configurable? Why asymmetric BW DL/UL configuration needs to be studied for all methods, e.g. for overlapping channel BW from network perspective? “RAN4 needs to further investigate if there is a need to add the operating band in the table(s) for Asymmetric channel bandwidths clause 5.3.6 of TS38.101-1” – would that be backward compatible change? “A cell could indicate in SIB1 that the carrier bandwidth is 7 MHz” - this is different from the previous section reading "wider CHBW of 10MHz indicated in carrierBandwidth in SIB1". “dedicated BWP that covers the entire 7 MHz” – is 7MHz (36 PRBs) possible also with all legacy UEs? |
| R4-2109435 | Ericsson: Strive for merging with R4-2111148 during meeting week. |
|  | Nokia: On top of general issues addressed in R4-2111148:. “For instance, the number of available RBs can be calculated by taking the actual spectrum allocation size and guard bands from the next lager standard channel” - Yes, this is an example but no technical motivation is provided to back this approach. Is the objective to achieve as good filtering as for today's CBWs? The difference between the guard band of - the next narrower and - the next wider CBW is motivated by the typically higher sampling rate of the next wider CBW, resulting at the same number of filter taps in a wider transition between the filter's passband and stopband. However, this filter will anyway not fit to the irregular CBW. Hence the iFFT's/FFT's filtering properties will have to be considered, but this consideration – combined with some objective for the stopband attenuation in the transmitter and the receiver – is missing. Table X-1: as commented in GTW, what are the assumptions for the Tx channel filter to end up with indicated “channel guard bands” and “channel Nrb” (assuming relevant Tx requirements are met outside the studied channel/spectrum block)? Do existing PRB positions need to be taken into account to calculate “channel guard bands”? “Theoretically speaking” - yes, this paragraph is theory. Simulation results showing that the performance requirements are still met for the irregular CBWs in question would be more convincing. Hence UE manufacturers are welcome to provide such simulations. “mask will scale to the used bandwidth” - the edge of the mask must be aligned with the irregular BW. Aspects of the mask which do not depend on the CBW (e.g. lines about ΔfOOB of ± 1-5 and ± 5-6 in TS 38.101-1 table 6.5.2.2-1) must be rather shifted than scaled? |

## Summary for 1st round

#### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Topic #2 | Issue 2-1: Applicability of widerCBW approach - Proposals - Option 1: Use the WiderCBW method for irregular CBW < 10MHz only (Intel, ZTE) - Option 2: Apply the WiderCBW method as a general approach for all irregular CBWs (Ericsson, Qualcomm, Apple, Skyworks, CMCC, China Telecom) - Need further discussion (ZTE, Nokia, Huawei) - Concerns about blocking from adjacent operator if BS are not co-located (T-Mobile US) - Notes after GTW on May 20th - The proponents think the issue of out-band emission is more bad for narrow bandwidth compared to wider bandwidth, while other companies disagreed and would like to have more analysis. - More discussion is needed. Recommendations for 2nd round: Continue to discuss the need to limit applicability of widerCBW approach during second round. If Option 2 is selected this does not preclude RAN4 from studying the use of overlapping channel bandwidth approach. Issue 2-2: irregularBW in UL - Proposals - Option 1: Adding UE capability/capabilities that indicate the UEs support for irregular bandwidths if to be supported in UL. - Option 2: Agree to define only smaller CHBW used by the UE in UL and further develop the usage of full irregular CHBW in UL in later releases. - Recommended WF - WF (R4-2103263) Agreement on DL only shall be upheld. Option 3 should be discussed for DL (widerCHBW) and UL (smallerCHBW) where the DL utilizes irregularBW. To support UL for irregularBW in future releases is FFS. - Agreement of GTW on May 20th - For all the potential approaches, to support UL for irregularBW from UE perspective needs further discussion in future release, - For widerCHBW approach, to support irregular CBW for DL only the following discussions are needed - Need further discussion on support of asymmetric bandwidths - Need further discussion on support of TX-RX separation Recommendations for 2nd round: Additional comments from companies after GTW that asymmetric bandwidths is already supported. Confirm understanding during second round. |

## CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2111148 | To be revised with comments taken into account from 1st round. Revisions shall be discussed during GTW (Monday) |
| R4-2109435 | To be revised with comments taken into account from 1st round. Revisions shall be discussed during GTW (Monday) |

## Discussion on 2nd round (if applicable)

Moderator can provide summary of 2nd round here. Note that recommended decisions on tdocs should be provided in the section titled ”Recommendations for Tdocs”.

    - Sub-topic 2-1

General aspects such as applicibility of widerCHBW approach and DL/UL

Issue 2-1: Applicability of widerCBW approach

- Proposals

  - Option 1: Use the WiderCBW method for irregular CBW < 10MHz only (Intel, ZTE)

  - Option 2: Apply the WiderCBW method as a general approach for all irregular CBWs (Ericsson, Qualcomm, Apple, Skyworks, CMCC, China Telecom)

    - Need further discussion (ZTE, Nokia, Huawei)

    - Concerns about blocking from adjacent operator if BS are not co-located (T-Mobile US)

- Notes after GTW on May 20th

  - The proponents think the issue of out-band emission is more bad for narrow bandwidth compared to wider bandwidth, while other companies disagreed and would like to have more analysis.

  - More discussion is needed.

Issue 2-2: irregularBW in UL

- Proposals

  - Option 1: Adding UE capability/capabilities that indicate the UEs support for irregular bandwidths if to be supported in UL.

  - Option 2: Agree to define only smaller CHBW used by the UE in UL and further develop the usage of full irregular CHBW in UL in later releases.

- Agreement of GTW on May 20th

  - For all the potential approaches, to support UL for irregularBW from UE perspective needs further discussion in future release,

  - For widerCHBW approach, to support irregular CBW for DL only the following discussions are needed

    - Need further discussion on support of asymmetric bandwidths

    - Need further discussion on support of TX-RX separation

Moderator recommendation for 2nd round: Additional comments from companies after GTW that asymmetric bandwidths is already supported.  Confirm understanding during second round.

## Companies views’ collection for 2nd round

| Company | Comments |
| --- | --- |
| Qualcomm | Issue 2-1: Option 2. Whether this method is usable or not and performance impact will depend on the deployment scenario and adjacent systems. |
| Skyworks | Issue 2-1: Option 2. Also note that when the irregular BW is on the band edge (quite some case in the SI), the lack of selectivity can be partially mitigated. Also this method can complementary to the overlapping methods |

## CRs/TPs comments collection

Comments on revised TPs.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2107885 | Huawei: is there any UE capability defined for the UE which can support the whole spectrum block? And what is the requirements should be applied? And for BS, how many RB should be blanked and what’s the BS filter assumption for the SEM and ACS requirements? |
|  | Intel: The TP needs to discuss the ACS degradation due to lack of filtering in blanked RBs. This is the primary trade-off of this approach. |
|  | Nokia: Further changes are needed: not clear why certain text was put in []? As proposed before, 6.1.1.1 should be not included at this moment, to be confirmed by RAN1/2 first. “WiderCHBW in DL may be applicable to co-located adjacent channels deployment” should not be included as commented in the 2nd round. There were also comments from Dish which are not reflected in this TP. |

# Topic #3: Evaluation of use of overlapping UE channel bandwidths

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2109245 | Intel Inc | Observation 1: For all CBW > 10MHz, the minimum overlap always exceeds 5MHz, so there is sufficient overlap for the SSB on any Sync raster and CoreSet0. Observation 2: It is worth noting that the Overlapping carrier approach for CBW > 10MHz, can be implemented with no hardware changes to existing NR compliant UEs. Observation 3: All of the proposed methods represent a significant SU improvement compared to Legacy mode SU and the difference is small. Other factors such as hardware feasiblity should take precedent over optimizing the last 1% SU. Observation 4: It is better to pursue solutions with RB-alignment between the two CCs since the SU benefit of No-RB alignment is minimal, 0.6% on average. Proposal 1: Utilize the Overlapping carrier methods with RB-alignment and 100kHz raster alignment between the two carriers for new irregular CBW > 10MHz. |
| R4-2109426 | ZTE Wistron Telecom AB | Observation 1: Non-aligned PRB grid may result in up to 11 subcarriers loss and even more for the case of PRB bundling. Observation 2: A UE supporting DL overlapping CA is required to support asymmetric UL and DL channel bandwidth for the concerned band. Observation 3: For combined UE channel bandwidth, legacy UEs are either restricted to use only one of frequency trunks, or to suffer from a potential NBC issue, and asymmetric UL and DL channel bandwidth support may be required for new UEs. Observation 4: For overlapping UE channel bandwidth from network perspective, legacy UEs are either restricted to use only one of frequency trunks, or to suffer from a potential NBC issue. Proposal 1: PRBs from overlapped CCs should be aligned in order to avoid unnecessary resource loss. Proposal 2: Asymmetric UL and DL channel bandwidth support should be enabled accordingly in the specs for the band where only DL overlapping CA is operating. |
| R4-2109484 | CMCC | Observation 1: Option 3 (Overlapping UE CBW) cannot achieve 90% spectrum utilization of irregular bandwidth from UE perspective and requires new gNB irregular bandwidths. Observation 2: Compared to option 2 (Combined UE CBW), option1 (Overlapping CA) provides more flexibility on SSB configurations and allows legacy UEs operate on both carriers. Observation 3: Except for the UE capabilities, no RAN1/2 impacts are foreseen for option1 (Overlapping CA) and option 2 (Combined UE CBW). Proposal: it is proposed to consider option1 (Overlapping CA) as baseline, and further study option 2 (Combined UE CBW). |
| R4-2109579 | Qualcomm Incorporated | Based on our analysis, the scheme “combined UE CBW” proposed in [2] has the following issues: - RAN1/2 impact on spec changes and behavior clarifications - RAN4 impact on spec changes to clarify requirements and possible behavior - RAN4 impact on performance requirements - UE implementation complexity - New UE performance tests and interop testing |
| R4-2110488 | Ericsson | Overlapping CA Approach requires the largest amount of implementation complexity and only RAN2 UE capability signalling is required For Combined UE Channel Approach an extensive change is required for RAN2 (TS 38.331) for UE to differentiate the need to union the additional PRBs with the main carrier described in SIB1. Overlapping UE Channel Bandwidth approach provides minimal specification changes |
| R4-2110662 | Huawei, HiSilicon | Observation 1: for channel bandwidths less than 50 MHz, integer-multiples of 5MHz channel bandwidths are supported/will be supported in BS/UE specifications. Proposal 1: New dedicated channel bandwidths are not considered for both BS and UE. Observation 2: The configuration of SSB can be left to network and no specification impact is needed. Observation 3: The impact to RF core requirements is very limited to support overlapping CA. Observation 4: there is no impact on RAN1 and RAN2 of intra-band overlapping CA to support the irregular channel bandwidth except for UE capability signalling. |
| R4-2111219 | Nokia, Nokia Shanghai Bell | Proposal 1: In order to maximize the spectrum utilization while keeping the PRB grid alignment between the main and the additional RF carrier, an alignment of the additional RF carrier with the 100 kHz channel raster is not required. Observation 1: The proposed method does not have impact to existing RAN2 signalling or to RAN1 specifications. RAN4 may consider LS to RAN1/2 to confirm this observation. Proposal 2: The study of overlapping channel bandwidths from UE perspective, according to objective 3 of the SID, shall include an approach with a single carrier from baseband perspective, allowing for a single BWP to cover the combined channel bandwidths in RRC_CONNECTED. Proposal 3: The PRB grid alignment is mandatory among overlapping channel bandwidths. Observation 2: Overlapping carriers with two SSBs are less spectrum efficient due to redundant radio resource allocations for common channels and signals. Furthermore, the scheduling of those resources is complicated. Proposal 4: For spectrum efficiency, solutions with only a single SSB are considered with higher priority than solutions needing a second SSB. Feedback from operators is desired on whether it is sufficient to serve all legacy UEs on the same side of a spectrum block if it is smaller than 10 MHz (e.g. in the main RF carrier's 5 MHz on the left-hand side of figure 2). Proposal 5: It is proposed to agree on the text proposal below Moderator: Comments on specifics for the TP should be captured in Section 3.3.2 |
| R4-2109587 | Qualcomm Incorporated | Comparison of Different Schemes |
| R4-2109436 | Apple, Ericsson, Skyworks Solutions Inc. | TP on using overlapping channels from the network perspective solution Moderator: Comments on specifics for the TP should be captured in Section 3.3.2 |

## Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

Each sub-topic is divided to capture the discussion on each approach for overlapping.  First issue will be used to capture the feasibility issues (if any).  Second issue will be related to specification changes needed to facility the approach (either RAN1, 2 or 4 specs).  Third issue will be used to capture the benefits, draw backs and implementation complexities.  Headings for each approach description (and their reference) is taken from R4-2105419, “Way forward on Evaluation of IrregularBW Approaches” also R4-2105419 is used as a basis for comparison in this meeting.  Please provide your comments on each “Approach aspect” and if it is not correct assessment of the approach please indicate why.

    - Sub-topic 3-1

Sub-topic description: Overlapping CA (two cells) (R4-2106486)

Sub-topic 3-1 Issue 3-1: Feasibility

- Proposals

  - Approach aspect 1: Legacy UE can operate in either carrier if two SSBs are configured.

  - Approach aspect 2: UE perspective, overlapping channels supported in DL only

  - Approach aspect 3: RB alignment is needed (without alignment there is no spectral efficiency gain)

  - Approach aspect 4: UE testing for irregularBW is needed. The CA framework can be reused.

Notes of GTW on May 20 (Yellow means more discussion is needed. Blue seems agreeable but confirmation is needed)

- Moderator summary: Proposals

  - Approach aspect 1: Legacy UE can operate in either carrier if two SSBs are configured.

  - Nokia: not always possible to have 2 SSBs for <10MHz in the frequency domain

  - Approach aspect 2: UE perspective, overlapping channels supported in DL only

  - Nokia: new type of CA BW classes would be needed with less than minimum channel spacing, would UEs support every CHBW with 1MHz step or new class for each irregular BW would be needed?

  - Approach aspect 3: RB alignment is needed (without alignment there is no spectral efficiency gain)

  - Huawei: RB alignment is really not a must for CA. It is a subset and can be configured if needed.

  - Approach aspect 4: UE testing for irregularBW is needed. The CA framework can be reused.

Chair: Please prepare the WF to try to capture the conclusions on feasibility, specification impacts, and benefits, drawbacks and implementation complexity for each candidate solution. Based on WF, we could prepare/update the corresponding TPs.

Sub- topic 3-1 Issue 3-2: Specification impacts (RAN1, 2, and/or 4)

- Proposals

  - Approach aspect 1: UE capability of supporting overlapping CA is needed.  New feature introduction.

  - Approach aspect 2: Define CA combinations (irregularBW + regular NR BW)

  - Approach aspect 3: Clarify which/how CA requirements apply in this case (channel spacing, emissions, etc.)

Notes of GTW on May 20 (Yellow means more discussion is needed. Blue seems agreeable but confirmation is needed)

Moderator summary:

- Proposals

  - Approach aspect 1: UE capability of supporting overlapping CA is needed.  New feature introduction.

  - Approach aspect 2: Define CA combinations (irregularBW + regular NR BW)

  - Should not support (or N/A)

  - Approach aspect 3: Clarify which/how CA requirements apply in this case (channel spacing, emissions, etc.)

  - Nokia: How does UE report CSI for the overlapped part? What about SRS, can there be SRS carrier switching between the carriers? SCell has either PDCCH or is cross-carrier scheduled, how would that work in overlapping CA case?

Sub- topic 3-1 Issue 3-3: Benefits, draw backs and implementation complexities

- Proposals

  - Approach aspect 1: No new channel filters for UE is needed. No new UE CBW is needed

  - Approach aspect 2: No new gNB CBW is required. Need further check how the regulatory requirements should be defined

  - Approach aspect 3: Asymmetric UL and DL channel bandwidth support should be enabled accordingly in the specs for the band where only DL overlapping CA is operating.

Notes of GTW on May 20 (Yellow means more discussion is needed. Blue seems agreeable but confirmation is needed)

Moderator summary:

- Proposals

  - Approach aspect 1: No new channel filters for UE is needed. No new UE CBW is needed

  - Approach aspect 2: No new gNB CBW is required. Need further check how the regulatory requirements should be defined

  - Huawei: BS out of band requirements are defined according to the edge carrier

  - Approach aspect 3: Asymmetric UL and DL channel bandwidth support should be enabled accordingly in the specs for the band where only DL overlapping CA is operating.

  - Different company understandings on symmetry of DL/UL

    - Sub-topic 3-2

Sub-topic description: Combined UE CBW (One cell) (R4-2107040)

Sub- topic 3-2 Issue 3-1: Feasibility

- Proposals

  - Approach aspect 1: from UE perspective, overlapping channels supported in DL only

  - Approach aspect 2: Legacy UE would operate in smallerCHBW carrier

Notes of GTW on May 20 (Yellow means more discussion is needed. Blue seems agreeable but confirmation is needed)

Moderator summary:

- Proposals

  - Approach aspect 1: from UE perspective, overlapping channels supported in DL only

  - Approach aspect 2: Legacy UE would operate in smallerCHBW carrier

  - Qualcomm: all legacy UEs would have to be in the same spectrum chunk

  - Skyworks: UE possible in one channel only

Sub- topic 3-2 Issue 3-2: Specification impacts (RAN1, 2, and/or 4)

- Proposals

  - Approach aspect 1: network will broadcast smallerCHBW in SIB1 and reconfigure UEs supporting this feature in connected mode to use wider BWP (to cover combined channel bandwidths)

  - Approach aspect 2: Impact to RAN1 and RAN2 to change the BWP constraints and clarify UE configuration/behavior with the new channel and BWP configuration.

  - Approach aspect 3: no change to RAN1 and RAN2 existing signalling

  - Approach aspect 4: New UE capability signaling needed

  - Approach aspect 5: BS requirement clarification needed main and additional RF carrier would re-use existing RF requirements

Notes of GTW on May 20 (Yellow means more discussion is needed. Blue seems agreeable but confirmation is needed)

Moderator summary:

- Moderator: seems different understanding on if RAN1/2 spec impact.  Different understanding on whether RAN1/2 spec needs changing.

Sub- topic 3-2 Issue 3-3: Benefits, draw backs and implementation complexities

- Proposals

  - Approach aspect 1: if UEs handle two carriers in separate FFT, then this will be similar with “overlapping CA” approach from UE perspective.

  - Approach aspect 2: No new gNB CBW is required. Need further check how the regulatory requirements should be defined.

  - Approach aspect 4: Increased implementation complexity at BS and UE

  - Approach aspect 5: new RAN4 performance requirements needed

  - Approach aspect 6: Solution can be applied to any bandwidth

Notes of GTW on May 20 (Yellow means more discussion is needed. Blue seems agreeable but confirmation is needed)

Moderator summary:

- Proposals

  - Approach aspect 1: if UEs handle two carriers in separate FFT, then this will be similar with “overlapping CA” approach from UE perspective.

- Qualcomm: Disagree. the UE processing on the baseband physical layer is totally different compared to CA. CA works independently(separate codewords, HARQ processes, etc). this method implies processing the 2 chunks together as a single channel.

  - Approach aspect 2: No new gNB CBW is required. Need further check how the regulatory requirements should be defined.

  - Approach aspect 4: Increased implementation complexity at BS and UE

  - Approach aspect 5: new RAN4 performance requirements needed

  - Approach aspect 6: Solution can be applied to any bandwidth

    - Sub-topic 3-3

Sub-topic description: Overlapping UE CBW from network perspective (One cell) (R4-2106689) (R4-2104887)

Sub-topic 3-3 Issue 3-1: Feasibility

- Proposals

  - Approach aspect 1: Only the smallerCHBW is used for DL and UL.

  - Approach aspect 2: No change is needed from UE perspective.

Notes of GTW on May 20 (Yellow means more discussion is needed. Blue seems agreeable but confirmation is needed)

Moderator summary:

- Proposals

  - Approach aspect 1: Only the smallerCHBW is used for DL and UL.

  - Approach aspect 2: No change is needed from UE perspective.

Sub- topic 3-3 Issue 3-2: Specification impacts (RAN1, 2, and/or 4)

- Proposals

  - Approach aspect 1: No impacts on other WGs are foreseen.

  - Approach aspect 2: new BS requirements for the irregularBW

Notes of GTW on May 20 (Yellow means more discussion is needed. Blue seems agreeable but confirmation is needed)

Moderator summary:

- Proposals

  - Approach aspect 1: No impacts on other WGs are foreseen.

  - Approach aspect 2: new BS requirements for the irregularBW

  - Ericsson, Qualcomm: Minimum set of requirements needed

Sub- topic 3-3 Issue 3-3: Benefits, draw backs and implementation complexities

- Proposals

  - Approach aspect 1: No UE impact, fully backwards compatible

  - Approach aspect 2: Symmetric UL/DL operation

  - Approach aspect 3: Simplest approach however for less than 10 MHz irregularBW complexity increases due to SSB multiplexed in time

Notes of GTW on May 20 (Yellow means more discussion is needed. Blue seems agreeable but confirmation is needed)

Moderator summary:

- Proposals

  - Approach aspect 1: No UE impact, fully backwards compatible

  - Approach aspect 2: Symmetric UL/DL operation

  - ZTE: From UE perspective

  - Huawei: Only smaller channel bandwidth is used for UE

  - Approach aspect 3: Simplest approach however for less than 10 MHz irregularBW complexity increases due to SSB multiplexed in time

  - Nokia: This solution is putting all complexity (e.g. may require duplicated SSBs/other radio resources which may conflict in frequency domain -> SSB to be transmitted in a staggered manner in time domain which has impact to scheduler complexity) in BS implementation as there is no UE impact. There is no gain from UE perspective since they would use smaller channel BW.

  - Qualcomm: Disagree. complexity increases compared to which scheme? overlapping CA has the same issue and even for combined CHBW, in order to be able to put legacy UEs anywhere in the spectrum block, multiple SSBs could be needed.

## Companies views’ collection for 1st round

    - Open issues

Sub topic 3-1 – Overlapping CA

| Company | Comments |
| --- | --- |
| Intel | Issue 3-1 Feasibility Aspect – 1,3,4. RB alignment is needed. Legacy UE can operate in either carrier. UE testing can reuse CA framework, FFS Issue 3-2 Spec Impacts (RAN1, 2, 4) Aspect – 1, While existing CA structure is mostly sufficient, overlapping CA should be introduced as a new feature and seek buy-in from RAN1, 2 Issue 3-3 Benefits, drawback and implementation complexities No new channel filters for UE are needed. No new CBW is needed for UE or gNB |
| Huawei | Feasibility: RB alignment is really not a must for CA. It is a subset and can be configured if needed. Specification impacts: - IrregularBW should be avoided hence there is no scenario to define irregularBW+regularBW - The CA framework can be reused, i.e. with the overlapping CA is configured, it is measured carrier by carrier and the relevant requirement per carrier can be reused. Benefits, draw backs and implementation complexities: 1. the BS out of band requirements are defined according to the edge carrier. Hence the existing requirements can be reused directly for regulatory requirements. 2. No asymmetric UL and DL channel bandwidth support is needed for overlapping CA. It is the same case as the normal 2 DL CA and 1UL. |
| Ericsson | Issue 3-1: Aspect 1: Agree. Aspect 2: Agree. Aspect 3: For irregularBW < 10 MHz as example CORESET#0 would need to be TDM anything other than RB alignment would be complex. Aspect 4: CA framework can be used but CA with irregularBW + regular NR BW would require at least 3 carriers. Issue 3-2: Aspect 1: Agree (UE behaviour for overlapping CBW is unpredictable) Aspect 2: Agree. Since UE capability in Aspect 1 is per CA combination. Issue 3-3: Aspect 1: Agree. Aspect 2: Agree Aspect 3: Not needed. UL/DL asymmetry is inherently supported by intra-band CA. |
| ZTE | Sub-topic 3-1 Issue 3-1: Feasibility o Approach aspect 1: Legacy UE can operate in either carrier if two SSBs are configured. Agree. o Approach aspect 2: UE perspective, overlapping channels supported in DL only Agree. o Approach aspect 3: RB alignment is needed (without alignment there is no spectral efficiency gain) Agree, otherwise there will be unnecessary spectrum efficiency. o Approach aspect 4: UE testing for irregularBW is needed. The CA framework can be reused. Agree. Sub- topic 3-1 Issue 3-2: Specification impacts (RAN1, 2, and/or 4) o Approach aspect 1: UE capability of supporting overlapping CA is needed. New feature introduction. Treating overlapping CA as a new feature is an over-kill since most of core requirements and tests can refer to normal CA. o Approach aspect 2: Define CA combinations (irregularBW + regular NR BW) If IrregularBW is not a defined CBW, this approach (Defining CA combos) is not appliable. o Approach aspect 3: Clarify which/how CA requirements apply in this case (channel spacing, emissions, etc.) Agree. This would have minimized specs impact. Sub- topic 3-1 Issue 3-3: Benefits, draw backs and implementation complexities o Approach aspect 1: No new channel filters for UE is needed. No new UE CBW is needed This is agreed in the SID. o Approach aspect 2: No new gNB CBW is required. Need further check how the regulatory requirements should be defined This is not encouraged in the SID. o Approach aspect 3: Asymmetric UL and DL channel bandwidth support should be enabled accordingly in the specs for the band where only DL overlapping CA is operating. Of course single UL CC is supported by default. However, enabling asymmetric UL and DL support with 2 UL CCs can increase UL spectrum usage from UE perspective. |
| Qualcomm | Issue 3-1: Feasibility Approach aspect 1: Agree Approach aspect 2: Agree Approach aspect 3: Agree Approach aspect 4: Agree. some kind of testing will be needed to check that the UE works as expected Issue 3-2: Specification impact Approach aspect 1: Agree Approach aspect 2: Agree Approach aspect 3: Agree Issue 3-3: Benefist, drawbacks and implementation complexities Approach aspect 1: Agree Approach aspect 2: Agree Approach aspect 3: Disagree. our understanding is that the overlapping channels that are aggregated are from the existing set and would be symmetric. |
| Nokia | Issue 3-1: aspect 1: not always possible to have 2 SSBs for <10MHz in the frequency domain Issue 3-2: aspect 2: new type of CA BW classes would be needed with less than minimum channel spacing, would UEs support every CHBW with 1MHz step or new class for each irregular BW would be needed? Issue 3-3: aspect 3: would that require additional signalling overhead? Specification impact: On top of RAN4, impact to RAN1/2 needs to be evaluated, e.g. since CA is considered, there will be BSR per serving cell – how would this be handled when these are overlapped? How does UE report CSI for the overlapped part? What about SRS, can there be SRS carrier switching between the carriers? SCell has either PDCCH or is cross-carrier scheduled, how would that work in overlapping CA case? Benefits, draw backs and implementation complexities: Peak data rate reduced comparing to sub-topic 3-2 (single cell) due to CA overhead and two SSBs. Without RB alignment SU is further reduced. May require duplicated SSBs/other radio resources which may conflict in frequency domain -> SSB to be transmitted in a staggered manner in time domain which has impact to scheduler complexity. |
| After GTW on May 20th |  |
| T-Mobile USA | Issue 3-1 comment on aspect 3: If RB alignment is needed, then it seems like the carriers can only be offset in increments of 900 kHz for 100 kHz raster (least common denominator of 180 kHz and 100 kHz). If subcarrier alignment is needed, then 300 kHz offset steps are possible for 100 kHz raster. If not alignment is needed, then 100 kHz offset steps are possible with the 100 kHz raster. But, it is not clear if RAN1 and RAN2 specs support this. |
| China Telecom | Issue 3-1: Feasibility: Whether the overlapping UE channel bandwidth approach can be used for small CBW, e.g., CBW <10MHz need to be confirmed. Issue 3-2: Specification impacts (RAN1, 2, and/or 4) Agree aspect 1 and 3. Issue 3-3: Benefits, draw backs and implementation complexities Agree aspect 1. For aspect 3 on whether it is symmetric or asymmetric, it seems we need to decouple the discussion for single carrier CBW, and the number of carriers in DL and UL. |
| CMCC | Issue 3-1: Aspect 1~2, 4: agree, Aspect 3: It would be more flexible to not restrict RB alignment between two carriers. Issue 3-2: Aspect 1~3: Agree Issue 3-3: Aspect 1~3: Agree |

Sub topic 3-2 – Combined UE CBW

| Company | Comments |
| --- | --- |
| Intel | Issue 3-1 Feasibility Feasibility is unclear to us how this option is intended to work for the UE. Perhaps an additional block diagram showing how UE signal combining might work could be provided Issue 3-2 Spec Impacts (RAN1, 2, 4) Issue 3-3 Benefits, drawback and implementation complexities Aspect 1: without better understanding the proposed block diagram, it appears that the only WF is to use separate FFT for each UE which is similar to “overlappling CA” approach. It is probably not feasible to require UEs to implement a new architecture to support combining spectrum before FFT |
| Huawei | Specification impacts: There is RAN1/RAN2 impact, e.g. the BWP can’t be outside of the resource grid. Benefits, draw backs and implementation complexities: The major complexity is that the existing implementation is that the baseband channel and RF channel are one to one mapping. Hence the approach with single BB channel mapping to two RF channel will need significant implementation complexities. |
| Ericsson | Issue 3-1: Aspect 1: Not feasible since the cell-specific carrierBandwidth cannot be changed in dedicated signaling (ServingCellConfigCommon). A BWP cannot be configured outside the carrierBandwidth. However, a UE dedicated BW (location of RF filter) can be configured outside the carrierBandwidth Aspect 2: Agree. Issue 3-2: Major change to RAN2 spec. |
| ZTE | Sub- topic 3-2 Issue 3-1: Feasibility o Approach aspect 1: from UE perspective, overlapping channels supported in DL only Agree as an previous agreement. o Approach aspect 2: Legacy UE would operate in smallerCHBW carrier Agree. Sub- topic 3-2 Issue 3-2: Specification impacts (RAN1, 2, and/or 4) o Approach aspect 1: network will broadcast smallerCHBW in SIB1 and reconfigure UEs supporting this feature in connected mode to use wider BWP (to cover combined channel bandwidths) Reconfigured DL BWP after connection should be restricted within the broadcast BS channel bandwidth. o Approach aspect 2: Impact to RAN1 and RAN2 to change the BWP constraints and clarify UE configuration/behavior with the new channel and BWP configuration. Yes agree. o Approach aspect 3: no change to RAN1 and RAN2 existing signalling No, RAN1/RAN2 change is needed. o Approach aspect 3: New UE capability signaling needed Needs to check further. o Approach aspect 4: BS requirement clarification needed main and additional RF carrier would re-use existing RF requirements Agree. Sub- topic 3-2 Issue 3-3: Benefits, draw backs and implementation complexities o Approach aspect 1: if UEs handle two carriers in separate FFT, then this will be similar with “overlapping CA” approach from UE perspective. Agree. o Approach aspect 2: No new gNB CBW is required. Need further check how the regulatory requirements should be defined. Agree. |
| Qualcomm | Issue 3-1: Feasibility Approach aspect 1: Agree.However, implementation is difficult Approach aspect 2: Agree.However, all legacy UEs would have to be in the same spectrum chunk. Issue 3-2: Specification impact(RAN1,2, and/or 4) Approach aspect 1: Agree Approach aspect 2: Agree Approach aspect 3: Basically agree. However, this would have to be checked by RAN1 and RAN2 to make sure. Approach aspect 4: Agree Approach aspect 5: Agree. there might be other impications also that cannot be assessed now before the method is fully understood Issue 3-3: Benefits, draw backs and implementation complexities Approach aspect 1: Disagree. the UE processing on the baseband physical layer is totally different compared to CA. CA works independently(separate codewords, HARQ processes, etc). this method implies processing the 2 chunks together as a single channel. Approach aspect 2: Agree Approach aspect 4: Agree Approach aspect 5: agree Approach aspect 6: Agree |
| Nokia | Issue 3-2: aspect 2: there is no impact to RAN1 and existing RAN2 signalling (both BWP and CHBW (UE specific) are reconfigured for some UEs in connected mode), details are provided in R4-2111219. Aspect 4: similar changes are expected as for sub-topic 3-1 (based on non-contiguous intra-band CA but without the gap). Specification impacts: No impact to RAN1 and existing RAN2 signalling. Limited impact to RAN4 since most requirements are re-used to guarantee co-existence. Benefits, draw backs and implementation complexities: Highest SU for both gNB and UE with one SSB only and no CA overhead. Future proof solution, solution can be applied to any spectrum block. No additional channel filters for UE and gNB. Increased complexity but this applies to other methods as well. |
| After GTW on May 20th |  |
| CMCC | Issue 3-1: Agree with aspect 1 and 2 for combined UE CBW Issue 3-2: Aspect 1: Network cannot configure BWP larger than channel bandwidth. In our view, network need to reconfigure UE supporting this feature with a new wider bandwidth first, then wider BWP can be configured within the wider channel bandwidth. Aspect 2: Clarification in RAN1/2 may be needed. According to our understanding of aspect 1, there are no BWP constraints. Aspect 3~5: Agree Issue 3-3: |
| Apple | Feasibility Agree to all aspects Specification impacts (RAN1, 2, and/or 4) Aspect 1-3: Our understanding is that BWP must be within the channel bandwidth broadcast in SIB1. Configuring the channel bandwidth outside that range might have an impact on other WGs, even though signalling supports it. Aspect 4-5: Agree Benefits, draw backs and implementation complexities Aspect 1: Disagree. From the UE perspective it does not look like the CA design because two different RF front ends are required, but at the same time one baseband is assumed. This approach might require further changes in the UE design, so we cannot assume that if a UE supports a certain CA combination for a particular band, then overlapping channels from the UE perspective are also possible. Aspect 4-6: Agree. |

Sub topic 3-3 – Overlapping UE CBW from network perspective

| Company | Comments |
| --- | --- |
| Intel | Issue 3-1 Feasibility Approach 2, No hardware change should be required Issue 3-2 Spec Impacts (RAN1, 2, 4) Ideally, minimal impacts on other WGs for CBW > 10MHz. Inevitably there will be some signaling changes or notifications required Issue 3-3 Benefits, drawback and implementation complexities This solution is feasible for >10MHz because there are no required changes in SSB timing. For < 10MHz, the required additional modes for time staggered SSBs is likely too complex and would require significant updates to the SSB methodology from RAN1, 2 |
| Huawei | Benefits, draw backs and implementation complexities - New BS channel bandwidths are needed which is not prioritized as stated in the SID. - Only smaller channel bandwidth is used for UE |
| Ericsson | Issue 3-1: Agree to both Aspect 1 and 2. Issue 3-2: Aspect 1: Agree. Aspect 2: Minimum set of (regulatory) requires are needed. Issue 3-3: Aspect 1: Agree Aspect 2: Agree. Aspect 3: Agree. It would be good to consider this approach only for > 10 MHz irregularBW |
| ZTE | Sub-topic 3-3 Issue 3-1: Feasibility o Approach aspect 1: Only the smallerCHBW is used for DL and UL. Agree. o Approach aspect 2: No change is needed from UE perspective. Agree. Sub- topic 3-3 Issue 3-2: Specification impacts (RAN1, 2, and/or 4) o Approach aspect 1: No impacts on other WGs are foreseen. Agree. o Approach aspect 2: new BS requirements for the irregularBW Agree. Sub- topic 3-3 Issue 3-3: Benefits, draw backs and implementation complexities o Approach aspect 1: No UE impact, fully backwards compatible Agree. o Approach aspect 2: Symmetric UL/DL operation Agree from UE perspective. o Approach aspect 3: Simplest approach however for less than 10 MHz irregularBW complexity increases due to SSB multiplexed in time Yes from BS perspective. |
| Qualcomm | Issue 3-1: Feasibility Approach aspect 1: Agree Approach aspect 2: Agree Issue 3-2: Specification impacts Approach aspect 1: Agree Approach aspect 2: Agree. the specs can be written in such a way that implementation impact can be minimized and an implementation on the BS sides as proposed for Combined CHBW is possible Issue 3-3: Benefits, drawbacks Approach aspect 1: Agree Approach aspect 2: Agree Approach aspect 3: Disagree. complexity increases compared to which scheme? overlapping CA has the same issue and even for combined CHBW, in order to be able to put legacy UEs anywhere in the spectrum block, multiple SSBs could be needed. |
| Nokia | This solution is putting all complexity (e.g. may require duplicated SSBs/other radio resources which may conflict in frequency domain -> SSB to be transmitted in a staggered manner in time domain which has impact to scheduler complexity) in BS implementation as there is no UE impact. There is no gain from UE perspective since they would use smaller channel BW. |
| After GTW on May 20th |  |
| DISH | Issue 3-3. Even for <10MHz this method should be used. This is the ONLY method which allows to use whole BS channel for UL also for legacy UE’s on system level. For instance on low band, having 10RB more system capacity on DL is nice, but having the same for UL is extremely important. Low band UL RB’s are the “gems” of this option. |
| China Telecom | Feasibility Agree to both aspects Specification impacts (RAN1, 2, and/or 4) Agree to both aspects. Benefits, draw backs and implementation complexities Agree to the three aspects. Legacy UE access is an important aspect to be considered. |
| CMCC | Issue 3-1: It seems this is already supported if only the smallerCHBW is used for DL and UL. And there is no Issue 3-2: Aspect 1: Agree Aspect 2: Agree are needed. Issue 3-3: Aspect 3: There is no throughput gain from UE side, since only smallerCHBW is used for DL and UL. Compared to other two solutions, this option provides lowest benefits. So it is not fair to say this is the simplest approach. |
| Apple | Feasibility: Agree to all aspects Specification impact: Agree with aspect 1. As for the aspect 2, we are not in a position to comment on the BS side, but we believe that the BS specification impact, if any, can be minimized. Benefits, draw backs and implementation complexities Agree with a remark on the increased complexity. As also noted by Qualcomm, multiple SSB are needed in the overlapping CA approach. In fact, multiple SSBs could be needed even in the overlapping channels from the UE perspective. In that sense we cannot much of the “increased complexity”. |

    - CRs/TPs comments collection

Major close to finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2111219 | Ericsson: We suggest to copy the table of SU % into the WF of evaluations of approaches to collect tables on SU for each approach into one place. Clause 6.5 “ no impact to RAN1 and existing RAN2 signalling” is not agreeable based upon discussion in Topic #3 of email summary. |
| R4-2109436 | Nokia: On top of few editorial issues we have the following comments: “this approach may require 3GPP to specify the new bandwidths for the BS side” – it should be noted BS may support irregular bandwidth by two overlapping carriers, introduction of new BWs for BS is not a generic solution and would have significant impact on specification changes “a subset of requirements tested for 13 MHz” – it is not clear which subset is tested having in mind requirements could be met for other regular CHBWs? How this would work if BS would use two overlapping carriers (e.g. 10 MHz and 5 MHz)? “From the BS perspective, it is a 6MHz channel with 30RBs and 300kHz guard bands” – this seems to overlook how the UEs' 100 kHz raster fits to the frequency position of the operator's licensed spectrum! Table X-1, “channel guard bands” – numbers are not correct, PRBs are symmetrically placed in irregular BW but legacy 5/10MHz UEs may be not at 100kHz raster “most likely need to broadcast two separate SSBs” – irregular CBWs >10 MHz should allow for a common initial BWP between the carrier that a UE finds and the shifted carrier that a UE may be commanded to use by dedicated signalling |

## Summary for 1st round

    - Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

Recommendations for 2nd round: Continue to discuss blue aspects that seem agreeable.

|  | Status summary |
| --- | --- |
| Sub-topic#3-1 | Sub-topic 3-1 Issue 3-1: Feasibility Notes of GTW on May 20 (Yellow means more discussion is needed. Blue seems agreeable but confirmation is needed) - Moderator summary: Proposals - Approach aspect 1: Legacy UE can operate in either carrier if two SSBs are configured. - Nokia: not always possible to have 2 SSBs for <10MHz in the frequency domain - Approach aspect 2: UE perspective, overlapping channels supported in DL only - Nokia: new type of CA BW classes would be needed with less than minimum channel spacing, would UEs support every CHBW with 1MHz step or new class for each irregular BW would be needed? - Approach aspect 3: RB alignment is needed (without alignment there is no spectral efficiency gain) - Huawei: RB alignment is really not a must for CA. It is a subset and can be configured if needed. - Approach aspect 4: UE testing for irregularBW is needed. The CA framework can be reused. Sub- topic 3-1 Issue 3-2: Specification impacts (RAN1, 2, and/or 4) Notes of GTW on May 20 (Yellow means more discussion is needed. Blue seems agreeable but confirmation is needed) Moderator summary: - Proposals - Approach aspect 1: UE capability of supporting overlapping CA is needed. New feature introduction. - Approach aspect 2: Define CA combinations (irregularBW + regular NR BW) - Should not support (or N/A) - Approach aspect 3: Clarify which/how CA requirements apply in this case (channel spacing, emissions, etc.) - Nokia: How does UE report CSI for the overlapped part? What about SRS, can there be SRS carrier switching between the carriers? SCell has either PDCCH or is cross-carrier scheduled, how would that work in overlapping CA case? Sub- topic 3-1 Issue 3-3: Benefits, draw backs and implementation complexities Notes of GTW on May 20 (Yellow means more discussion is needed. Blue seems agreeable but confirmation is needed) Moderator summary: - Proposals - Approach aspect 1: No new channel filters for UE is needed. No new UE CBW is needed - Approach aspect 2: No new gNB CBW is required. Need further check how the regulatory requirements should be defined - Huawei: BS out of band requirements are defined according to the edge carrier - Approach aspect 3: Asymmetric UL and DL channel bandwidth support should be enabled accordingly in the specs for the band where only DL overlapping CA is operating. - Different company understandings on symmetry of DL/UL |
| Sub-topic#3-2 | Sub- topic 3-2 Issue 3-1: Feasibility Notes of GTW on May 20 (Yellow means more discussion is needed. Blue seems agreeable but confirmation is needed) Moderator summary: - Proposals - Approach aspect 1: from UE perspective, overlapping channels supported in DL only - Approach aspect 2: Legacy UE would operate in smallerCHBW carrier - Qualcomm: all legacy UEs would have to be in the same spectrum chunk - Skyworks: UE possible in one channel only Sub- topic 3-2 Issue 3-2: Specification impacts (RAN1, 2, and/or 4) Notes of GTW on May 20 (Yellow means more discussion is needed. Blue seems agreeable but confirmation is needed) Moderator summary: - Moderator: seems different understanding on if RAN1/2 spec impact. Different understanding on whether RAN1/2 spec needs changing. Sub- topic 3-2 Issue 3-3: Benefits, draw backs and implementation complexities Notes of GTW on May 20 (Yellow means more discussion is needed. Blue seems agreeable but confirmation is needed) Moderator summary: - Proposals - Approach aspect 1: if UEs handle two carriers in separate FFT, then this will be similar with “overlapping CA” approach from UE perspective. - Qualcomm: Disagree. the UE processing on the baseband physical layer is totally different compared to CA. CA works independently(separate codewords, HARQ processes, etc). this method implies processing the 2 chunks together as a single channel. - Approach aspect 2: No new gNB CBW is required. Need further check how the regulatory requirements should be defined. - Approach aspect 4: Increased implementation complexity at BS and UE - Approach aspect 5: new RAN4 performance requirements needed - Approach aspect 6: Solution can be applied to any bandwidth |
| Sub-topic#3-3 | Sub-topic 3-3 Issue 3-1: Feasibility Notes of GTW on May 20 (Yellow means more discussion is needed. Blue seems agreeable but confirmation is needed) Moderator summary: - Proposals - Approach aspect 1: Only the smallerCHBW is used for DL and UL. - Approach aspect 2: No change is needed from UE perspective. Sub- topic 3-3 Issue 3-2: Specification impacts (RAN1, 2, and/or 4) Notes of GTW on May 20 (Yellow means more discussion is needed. Blue seems agreeable but confirmation is needed) Moderator summary: - Proposals - Approach aspect 1: No impacts on other WGs are foreseen. - Approach aspect 2: new BS requirements for the irregularBW - Ericsson, Qualcomm: Minimum set of requirements needed Sub- topic 3-3 Issue 3-3: Benefits, draw backs and implementation complexities Notes of GTW on May 20 (Yellow means more discussion is needed. Blue seems agreeable but confirmation is needed) Moderator summary: - Proposals - Approach aspect 1: No UE impact, fully backwards compatible - Approach aspect 2: Symmetric UL/DL operation - ZTE: From UE perspective - Huawei: Only smaller channel bandwidth is used for UE - Approach aspect 3: Simplest approach however for less than 10 MHz irregularBW complexity increases due to SSB multiplexed in time - Nokia: This solution is putting all complexity (e.g. may require duplicated SSBs/other radio resources which may conflict in frequency domain -> SSB to be transmitted in a staggered manner in time domain which has impact to scheduler complexity) in BS implementation as there is no UE impact. There is no gain from UE perspective since they would use smaller channel BW. - Qualcomm: Disagree. complexity increases compared to which scheme? overlapping CA has the same issue and even for combined CHBW, in order to be able to put legacy UEs anywhere in the spectrum block, multiple SSBs could be needed. |

    - CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2111219 | To be revised with comments taken into account from 1st round. Revisions shall be discussed during GTW (Monday) |
| R4-2109436 | To be revised with comments taken into account from 1st round. Revisions shall be discussed during GTW (Monday) |

## Discussion on 2nd round (if applicable)

Moderator can provide summary of 2nd round here. Note that recommended decisions on tdocs should be provided in the section titled ”Recommendations for Tdocs”.

    - Sub-topic 3-1

Sub-topic description: Overlapping UE CBW from network perspective (One cell) (R4-2106689) (R4-2104887)

Sub-topic 3-1 Issue 3-1: Feasibility

Notes of GTW on May 20 (Yellow means more discussion is needed. Blue seems agreeable but confirmation is needed)

- Moderator summary: Proposals

  - Approach aspect 1: Legacy UE can operate in either carrier if two SSBs are configured.

  - Nokia: not always possible to have 2 SSBs for <10MHz in the frequency domain

  - Approach aspect 2: UE perspective, overlapping channels supported in DL only

  - Nokia: new type of CA BW classes would be needed with less than minimum channel spacing, would UEs support every CHBW with 1MHz step or new class for each irregular BW would be needed?

  - Approach aspect 3: RB alignment is needed (without alignment there is no spectral efficiency gain)

  - Huawei: RB alignment is really not a must for CA. It is a subset and can be configured if needed.

  - Approach aspect 4: UE testing for irregularBW is needed. The CA framework can be reused.

Sub- topic 3-1 Issue 3-2: Specification impacts (RAN1, 2, and/or 4)

Notes of GTW on May 20 (Yellow means more discussion is needed. Blue seems agreeable but confirmation is needed)

Moderator summary:

- Proposals

  - Approach aspect 1: UE capability of supporting overlapping CA is needed.  New feature introduction.

  - Approach aspect 2: Define CA combinations (irregularBW + regular NR BW)

  - Should not support (or N/A)

  - Approach aspect 3: Clarify which/how CA requirements apply in this case (channel spacing, emissions, etc.)

  - Nokia: How does UE report CSI for the overlapped part? What about SRS, can there be SRS carrier switching between the carriers? SCell has either PDCCH or is cross-carrier scheduled, how would that work in overlapping CA case?

Sub- topic 3-1 Issue 3-3: Benefits, draw backs and implementation complexities

Notes of GTW on May 20 (Yellow means more discussion is needed. Blue seems agreeable but confirmation is needed)

Moderator summary:

- Proposals

  - Approach aspect 1: No new channel filters for UE is needed. No new UE CBW is needed

  - Approach aspect 2: No new gNB CBW is required. Need further check how the regulatory requirements should be defined

  - Huawei: BS out of band requirements are defined according to the edge carrier

  - Approach aspect 3: Asymmetric UL and DL channel bandwidth support should be enabled accordingly in the specs for the band where only DL overlapping CA is operating.

  - Different company understandings on symmetry of DL/UL

    - Sub-topic 3-2

Sub- topic 3-2 Issue 3-1: Feasibility

Notes of GTW on May 20 (Yellow means more discussion is needed. Blue seems agreeable but confirmation is needed)

Moderator summary:

- Proposals

  - Approach aspect 1: from UE perspective, overlapping channels supported in DL only

  - Approach aspect 2: Legacy UE would operate in smallerCHBW carrier

  - Qualcomm: all legacy UEs would have to be in the same spectrum chunk

  - Skyworks: UE possible in one channel only

Sub- topic 3-2 Issue 3-2: Specification impacts (RAN1, 2, and/or 4)

Notes of GTW on May 20 (Yellow means more discussion is needed. Blue seems agreeable but confirmation is needed)

Moderator summary:

- Moderator: seems different understanding on if RAN1/2 spec impact.  Different understanding on whether RAN1/2 spec needs changing.

Sub- topic 3-2 Issue 3-3: Benefits, draw backs and implementation complexities

Notes of GTW on May 20 (Yellow means more discussion is needed. Blue seems agreeable but confirmation is needed)

Moderator summary:

- Proposals

  - Approach aspect 1: if UEs handle two carriers in separate FFT, then this will be similar with “overlapping CA” approach from UE perspective.

- Qualcomm: Disagree. the UE processing on the baseband physical layer is totally different compared to CA. CA works independently(separate codewords, HARQ processes, etc). this method implies processing the 2 chunks together as a single channel.

  - Approach aspect 2: No new gNB CBW is required. Need further check how the regulatory requirements should be defined.

  - Approach aspect 4: Increased implementation complexity at BS and UE

  - Approach aspect 5: new RAN4 performance requirements needed

  - Approach aspect 6: Solution can be applied to any bandwidth

    - Sub-topic 3-3

Sub-topic 3-3 Issue 3-1: Feasibility

Notes of GTW on May 20 (Yellow means more discussion is needed. Blue seems agreeable but confirmation is needed)

Moderator summary:

- Proposals

  - Approach aspect 1: Only the smallerCHBW is used for DL and UL.

  - Approach aspect 2: No change is needed from UE perspective.

Sub- topic 3-3 Issue 3-2: Specification impacts (RAN1, 2, and/or 4)

Notes of GTW on May 20 (Yellow means more discussion is needed. Blue seems agreeable but confirmation is needed)

Moderator summary:

- Proposals

  - Approach aspect 1: No impacts on other WGs are foreseen.

  - Approach aspect 2: new BS requirements for the irregularBW

  - Ericsson, Qualcomm: Minimum set of requirements needed

Sub- topic 3-3 Issue 3-3: Benefits, draw backs and implementation complexities

Notes of GTW on May 20 (Yellow means more discussion is needed. Blue seems agreeable but confirmation is needed)

Moderator summary:

- Proposals

  - Approach aspect 1: No UE impact, fully backwards compatible

  - Approach aspect 2: Symmetric UL/DL operation

  - ZTE: From UE perspective

  - Huawei: Only smaller channel bandwidth is used for UE

  - Approach aspect 3: Simplest approach however for less than 10 MHz irregularBW complexity increases due to SSB multiplexed in time

  - Nokia: This solution is putting all complexity (e.g. may require duplicated SSBs/other radio resources which may conflict in frequency domain -> SSB to be transmitted in a staggered manner in time domain which has impact to scheduler complexity) in BS implementation as there is no UE impact. There is no gain from UE perspective since they would use smaller channel BW.

  - Qualcomm: Disagree. complexity increases compared to which scheme? overlapping CA has the same issue and even for combined CHBW, in order to be able to put legacy UEs anywhere in the spectrum block, multiple SSBs could be needed.

## Companies views’ collection for 2nd round

Sub topic 3-1 – Overlapping CA

| Company | Comments |
| --- | --- |
| Qualcomm | Issue 3-1: Approach aspect 1: SSBs can be TDM-ed in time or different SSB beam indexes can be used. Approach aspect 3: RB alignment is needed. without RB alignment there is no increase in spectral efficience because partial RB alocation does not exist and having partial overlap will not enable the same RB to be scheduled to multiple UEs at the same time. Sub-topic 3-2: Issue 3-2: even if current signaling can be reused, the UE behavior is different compared to legacy UEs so there will be a need to make changes to the specs. |
| Skyworks | Issue 3-1: RB alignment needed also to enable complementing with the use of LargerCBW. If SSB is in the overlapping region RB alignement will be needed anyhow. |

Sub topic 3-2 – Combined UE CBW

| Company | Comments |
| --- | --- |
| MediaTek | It is very difficult to understand how this scheme is supposed to be working considering the lack of architectural diagrams. In the document R4-2107886, the TP suggests that everything is perfect, 1 RB granularity, no ACS or blocking issues at all. We have doubts that this is the case in practice, but as Qualcomm mentioned there needs to be more detail provided. |
| Nokia | we do not see any issues in ACS/blocking since existing carriers are re-used (to guarantee co-existence). This is well explained in our paper, can Mediatek clarify what is the issue with ACS/blocking? |

Sub topic 3-3 – Overlapping UE CBW from network perspective

| Company | Comments |
| --- | --- |

## CRs/TPs comments collection

Comments on revised TPs.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2107886 | Huawei: as many companies comment, the following aspects are not addressed. - There is RAN1/RAN2 impact, e.g. the BWP can’t be outside of the resource grid. - The major complexity is that the existing implementation is that the baseband channel and RF channel are one to one mapping. Hence the approach with single BB channel mapping to two RF channel will need significant implementation complexities. - In a long term, the majority is legacy UEs and the two channel should be on the channel raster. |
|  | MediaTek: There needs to be much more clarity on how the Combined UE Channel Bandwidth is supposed to work. This would help us to understand better whether the claims of no ACS or blocking issues coupled with a 1 RB granularity of channel bandwidth is really achievable. We have some doubts about that. As such we cannot agree to the TP that analyses the benefits and drawbacks until there is a clearer understanding provided. |
|  | Intel: The proposed TP is unfeasible as it requires phase alignment each Rx path in the UE. This is not supported by many existing UEs. |
|  | Apple: Firstly, our apologies to provide comments after the deadline (we mistakenly assumed that the deadline was not applicable to TPs). As for the comments for the TP: Our understanding is that proponents should capture all the relevant information to the corresponding sections dedicated to a particular solution. This TP also adds text into sub-clauses 6.3 and 6.4, but we cannot accept it as the consolidated agreement from RAN4. Thus, all the relevant information shall be moved to sub-clause 6.2. We agree with the comment from other companies that little information is captured on the potential impact on the UE design pretending that this is an easy approach. There should be some block diagram showing existing CA architecture (two RF and two baseband blocks) and a new diagram showing that this solution requires a different internal UE design; In the following sentence, “very efficient from spectrum utilization point of view...”, how do we define “very efficient”? In fact, referring to the SU numbers in the same TP we cannot see much difference between this and other solutions. So it is better to clarify the wording accordingly. What does the following sentence mean, “additional RF carrier” not to be on the channel raster to increase spectrum utilization (up to 2 PRBs)"? If this assumes that we explore an option of using new channel raster points, then all the pros and cons should be explained accordingly. It effectively means that legacy UEs will not be able to use the second carrier and thus the overall system capacity will decline a lot, as opposed to e.g. overlapping channels from the network perspective. We can accept the TP if the aforementioned issue are clarified/fixed. |
|  | Nokia: in the e-mail discussion, Intel was concerned about the two RF carriers' phase alignment in the UE because they assumed the use of separate antennas, RF paths etc. However, we assume a multi-carrier receiver with a sufficiently high ADC sampling rate. Two RF carriers at different frequencies in the same operating band are possible with one and the same antenna and analogue RX path. In the digital domain, the RF carriers are separated by NCOs and digital filters. After the separate FFTs of the two RF carriers, the subcarriers are merged into a single BB carrier. Because of the RF carriers' overlap, there are subcarriers which are part of both RF carriers and thus part of both FFTs. By comparing the phases of the two RF carriers' overlapping subcarriers, the phase difference between the RF carriers can be determined and compensated. This requires extra effort but compared with the other receiver algorithms (such as interference cancellation and soft decoding), these additional steps are neither complicated nor require a lot of computational effort. To Mediatek: we do not see any issues in ACS/blocking since existing carriers are re-used (to guarantee co-existence). This is well explained in our paper, can Mediatek clarify what is the issue with ACS/blocking? To Apple: if you are fine to copy exactly the same text from 6.3, 6.4. and 6.6 then we are fine to include that in 6.2. I do not understand what is the problem with “very efficient”, it is clear this method provides the largest number of RBs (for both UE and gNB) comparing to other methods. It is clarified in the text already “additional RF carrier” would be used only by UEs which supports this solution so I do not see a point on legacy UEs. |
| R4-2107887 | Huawei: in clause 6.2.1, it says “To avoid specifying a new channel bandwidth at the BS side ”, but it is still unclear how it can avoid to define a new BS CBW? In our understanding the approach itself is to introduce new BS CBW. |
|  | Intel: It would help to clarify how multiple Rx chains in the UE will know to respond to separate SSBs and then combine RBs from each chain |
|  | Apple: Referring to the comment from Intel, a UE will act only upon one SSB in the same as it works in the legacy deployments when there are two cells and one SSB per each cell. In other words we do not put any new requirement on reading/following two SSBs as it is not needed. |
|  | Nokia: as suggested by Dish in the e-mail reflector, text should be changed to “To support two overlapping carriers, the network broadcasts two separate SSBs at least for BW <10MHz”” |
| R4-2108015 | The LS scope is too broad. We effectively ask RAN1/RAN2 to look at all the solutions we have, delve into the details (some of which are still missing), and provide a thorough answer. So, it is almost like opening a new SI in RAN1/RAN2 |
|  | Nokia: The intention is to ask from RAN1/2 as there were comments for each method and that is one of SI objectives. If there are no issues for specific methods, then no issues will be reported. Why try to hide that fact and rather put it out open? What if RAN4 thinks it works and later on it turns out that doesn’t work in practice? We are worried if LS is not sent in this meeting, it will have impact on SI completion in the near future. |
|  | Ericsson: We do not agree to send an LS on all methods to RAN1/2. As stated above they will need to investigate and understand the details of each method something RAN4 not yet fully do, hence we would not get a valid answer within the near future. We should pinpoint the questions raised by companies per method in detail in a potential LS out. |

# Recommendations for Tdocs

## 1st round

New tdocs

| Title | Source | Comments |
| --- | --- | --- |
| WF on … | YYY |  |
| LS on … | ZZZ | To: RAN_X; Cc: RAN_Y |

Existing tdocs

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-2110487 | Updated draft TR 38.844 after RAN4#98bis-e | Ericsson | Approve |  |
| R4-2111148 | Draft TP to TR38.844 on wider channel BW method | Ericsson | To be Revised | Aim to capture any agreeable high level aspects of approach into TR |
| R4-2109435 | TP on using next larger channel bandwidth solution | Apple | Noted. | Content to be merged into Revision of R4-2111148 |
| R4-2111219 | On the use of overlapping channel bandwidths from UE perspective | Nokia, Nokia Shanghai Bell | To be Revised | Aim to capture any agreeable high level aspects of approach into TR |
| R4-2109436 | TP on using overlapping channels from the network perspective solution | Apple, Ericsson, Skyworks Solutions Inc. | To be Revised | Aim to capture any agreeable high level aspects of approach into TR |

## 2nd round

New tdocs

| Title | Source | Comments |
| --- | --- | --- |
| R4-2108015 LS on specification impact for methods on efficient utilization of licensed spectrum that is not aligned with existing NR channel bandwidths | Nokia, Nokia Shanghai Bell | Noted |

Existing tdocs

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-2107886 | On the use of overlapping channel bandwidths from UE perspective | Nokia, Nokia Shanghai Bell | Revise after GTW. | Revision of R4-2111219 |
| R4-2108021 | TP on the use of overlapping channel bandwidths from UE perspective | Nokia, Nokia Shanghai Bell | To be agreed | Revision of R4-2107886 |
| R4-2107885 | TP to TR38.844 on wider channel BW method | Ericsson, Apple, Skyworks | Revise after GTW. | Combined revision of R4-2111148 and R4-2109435 |
| R4-2108022 |  |  | To be agreed |  |
| R4-2107887 | TP on using overlapping channels from the network perspective solution | Apple, Ericsson, Skyworks Solutions Inc | Revise after GTW. | Revision of R4-2109436 |
| R4-2111540 |  |  | To be agreed |  |